1) acq_car_gen_mix_gp2015_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_SIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity acq_car_gen_mix_gp2015_e is

Port (  Sys_CLK_is : in  STD_LOGIC;

POR_RST_is : in  STD_LOGIC;

Incoming_data_is : in  STD_LOGIC_VECTOR (5 downto 0);

car_mix_ROM_ena_is : IN STD_LOGIC;

car_mix_ROM_wea_is : IN STD_LOGIC_VECTOR(0 DOWNTO 0);	car_mix_ROM_addra_is : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

car_mix_ROM_dina_is : IN STD_LOGIC_VECTOR(19 DOWNTO 0);

Sat_load_is : in std_logic;

IF_Freq_input_is : in STD_LOGIC_VECTOR (31 downto 0);

IP_mixout_os : out  STD_LOGIC_VECTOR (6 downto 0);

QP_mixout_os : out  STD_LOGIC_VECTOR (6 downto 0));

end acq_car_gen_mix_gp2015_e;

architecture acq_car_gen_mix_gp2015_a of acq_car_gen_mix_gp2015_e is

signal IF_freq_carr_phase_s : std_logic_vector(31 downto 0);

signal IF_freq_s : std_logic_vector(31 downto 0);

signal addrb_s : std_logic_vector(9 downto 0);

signal doutb_s : std_logic_vector(19 downto 0);

signal edac_out_s : std_logic_vector(13 downto 0);

COMPONENT carrier_mix_ROM

PORT (

clka : IN STD_LOGIC;  ena : IN STD_LOGIC;

wea : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

addra : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

dina : IN STD_LOGIC_VECTOR(19 DOWNTO 0);

clkb : IN STD_LOGIC;

addrb : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

doutb : OUT STD_LOGIC_VECTOR(19 DOWNTO 0) );

END COMPONENT carrier_mix_ROM;

COMPONENT edac_14_dec1_e

Port (

rd_en_is       : in  STD_LOGIC;  -- EDAC Decoder  Read enable

data_in_is     : in  STD_LOGIC_VECTOR (13 downto 0);   -- Input Information bits to EDAC Decoder

par_in_is      : in  STD_LOGIC_VECTOR (5 downto 0);   -- Input Parity bits to EDAC Decoder

data_out_os    : out  STD_LOGIC_VECTOR (13 downto 0));  -- Output information bits after correction

end COMPONENT edac_14_dec1_e;

begin

carmixrom : carrier_mix_ROM

PORT MAP (

clka => Sys_CLK_is,     ena => car_mix_ROM_ena_is,

wea => car_mix_ROM_wea_is,     addra => car_mix_ROM_addra_is,        dina => car_mix_ROM_dina_is,  clkb => Sys_CLK_is,

addrb => addrb_s,      doutb => doutb_s   );

EDAC_mixROM: edac_14_dec1_e

port map (

rd_en_is => '1', data_in_is => doutb_s(13 downto 0)  ,par_in_is =>  doutb_s(19 downto 14) ,data_out_os => edac_out_s );

P_IQ_multi: process(Sys_CLK_is,POR_RST_is )

begin

if POR_RST_is='1' then

IF_freq_carr_phase_s <= (others=>'0');

IF_freq_s <= x"02B020C5"; --0.42e6 at 40M

IP_mixout_os<= (others=>'0');

QP_mixout_os<= (others=>'0');

addrb_s<= (others=>'0');

elsif Sys_CLK_is='1' and Sys_CLK_is'event then

IP_mixout_os <=   edac_out_s(6 downto 0);

QP_mixout_os <=   edac_out_s(13 downto 7);

addrb_s<=Incoming_data_is(5 downto 0)& IF_freq_carr_phase_s(31 downto 28) ;

if(Sat_load_is = '1') then

IF_freq_s <= IF_Freq_input_is;

end if;

IF_freq_carr_phase_s <= IF_freq_carr_phase_s + IF_freq_s;

end if;--clk event with POR

end process P_IQ_multi;

end acq_car_gen_mix_gp2015_a;

2) acq_code_generator_gp2015_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity acq_code_generator_gp2015_e is

Port ( Sys_CLK_40M_is : in  STD_LOGIC;

POR_RST_is : in STD_LOGIC;  Load_is : in  STD_LOGIC;

Acq_Svidin_is : in  STD_LOGIC_VECTOR (5 downto 0);

g2_reg_input_is : in  STD_LOGIC_VECTOR (9 downto 0);

Codeclk_2p046M_os : out STD_LOGIC;   Codeout_os : out  STD_LOGIC );end acq_code_generator_gp2015_e;

architecture acq_code_generator_gp2015_a of acq_code_generator_gp2015_e is

signal g1_reg_s : std_logic_vector(9 downto 0);

signal g2_reg_s : std_logic_vector(9 downto 0);

signal Codeclk_1p023M_s: std_logic;

signal Codeclk_1p023M_latched_s: std_logic;

signal Code_freq_phase_s : std_logic_vector(31 downto 0);

begin

P_acq_code_Gen:process(Sys_CLK_40M_is,POR_RST_is,g2_reg_input_is )

begin

if POR_RST_is ='1' then

g1_reg_s <= (others => '1') ;

Codeout_os <= '0';

Codeclk_1p023M_latched_s <= '0';

Codeclk_1p023M_s <= '0';

Code_freq_phase_s <=  (others => '0') ;

Codeclk_2p046M_os <= '0';

g2_reg_s <= g2_reg_input_is;

elsif Sys_CLK_40M_is='1' and Sys_CLK_40M_is'event then

if Load_is ='1' then

g1_reg_s <= (others => '1') ;

Codeout_os <= '0';

Codeclk_1p023M_latched_s <= '0';

Codeclk_1p023M_s <= '0';

Code_freq_phase_s <= (others => '0') ;

Codeclk_2p046M_os <= '0';

C_SVID_decoder: case Acq_Svidin_is(5 downto 0) is

when "000000" => g2_reg_s <= "1111101100"; --GPS SVID 1

when "000001" => g2_reg_s <= "1111011000"; --GPS SVID 2

when "000010" => g2_reg_s <= "1110110000"; --GPS SVID 3

when "000011" => g2_reg_s <= "1101100000"; --GPS SVID 4

when "000100" => g2_reg_s <= "0010010110"; --GPS SVID 5

when "000101" => g2_reg_s <= "0100101100"; --GPS SVID 6

when "000110" => g2_reg_s <= "0110010110"; --GPS SVID 7

when "000111" => g2_reg_s <= "1100101100"; --GPS SVID 8

when "001000" => g2_reg_s <= "1110010111"; -- NAVIC-1A

when "001001" => g2_reg_s <= "0110010000"; -- NAVIC-1B

when "001010" => g2_reg_s <= "0010110001"; -- NAVIC-1C

when "001011" => g2_reg_s <= "0100111010"; -- NAVIC-1D

when "001100" => g2_reg_s <= "0001010010";--0450  PRN-138

when "001101" => g2_reg_s <= "1000010101";--1241  PRN-121

when "001110" => g2_reg_s <= "1110110100";--0267  PRN-122

when "001111" => g2_reg_s <= "0101100100";--0232  PRN-123

-- first 8 are GPS 9- 16  next 4 NavIC 5-8  next SBAS 5-8

when "010000" => g2_reg_s <= "1001011000"; --GPS SVID 9

when "010001" => g2_reg_s <= "1101110100"; --GPS SVID 10

when "010010" => g2_reg_s <= "1011101000"; --GPS SVID 11

when "010011" => g2_reg_s <= "1110100000"; --GPS SVID 12

when "010100" => g2_reg_s <= "1101000000"; --GPS SVID 13

when "010101" => g2_reg_s <= "1010000000"; --GPS SVID 14

when "010110" => g2_reg_s <= "0100000000"; --GPS SVID 15

when "010111" => g2_reg_s <= "1000000000"; --GPS SVID 16

when "011000" => g2_reg_s <= "0000110111"; -- NAVIC-1E

when "011001" => g2_reg_s <= "1101011000"; -- NAVIC-1F

when "011010" => g2_reg_s <= "0010100000"; -- NAVIC-1G

when "011011" => g2_reg_s <= "0000110010"; -- NAVIC-1H

when "011100" => g2_reg_s <= "1011011100"; -- 0355  PRN 147

when "011101" => g2_reg_s <= "0111110001"; -- 1076  PRN 125

when "011110" => g2_reg_s <= "0010111111"; -- 1764  PRN 126

when "011111" => g2_reg_s <= "1111001110"; -- 0717  PRN 127

-- first 8 are GPS 17- 24 next 4 NavIC 9-12  next SBAS 9 - 12

when "100000" => g2_reg_s <= "1000100110"; --GPS SVID 17

when "100001" => g2_reg_s <= "0001001100"; --GPS SVID 18

when "100010" => g2_reg_s <= "0010011000"; --GPS SVID 19

when "100011" => g2_reg_s <= "0100110000"; --GPS SVID 20

when "100100" => g2_reg_s <= "1001100000"; --GPS SVID 21

when "100101" => g2_reg_s <= "0011000000"; --GPS SVID 22

when "100110" => g2_reg_s <= "0011001110"; --GPS SVID 23

when "100111" => g2_reg_s <= "1001110000"; --GPS SVID 24

when "101000" => g2_reg_s <= "0001100100"; -- NAVIC-1I

when "101001" => g2_reg_s <= "0010011011"; -- NAVIC-1J

when "101010" => g2_reg_s <= "0011001000"; -- NAVIC-1K

when "101011" => g2_reg_s <= "0011111011"; -- NAVIC-1L

when "101100" => g2_reg_s <= "0101101011";  --1532  PRN 128

when "101101" => g2_reg_s <= "0001010101";  --1250  PRN 129

when "101110" => g2_reg_s <= "1000011100";  --0341  PRN 130

when "101111" => g2_reg_s <= "1001011010";  --0551  PRN 131

-- first 8 are GPS 25- 32 next 4 NavIC 13-14  next SBAS 13 - 18

when "110000" => g2_reg_s <= "0011100000"; --GPS SVID 25

when "110001" => g2_reg_s <= "0111000000"; --GPS SVID 26

when "110010" => g2_reg_s <= "1110000000"; --GPS SVID 27

when "110011" => g2_reg_s <= "1100000000"; --GPS SVID 28

when "110100" => g2_reg_s <= "0001010110"; --GPS SVID 29

when "110101" => g2_reg_s <= "0010101100"; --GPS SVID 30

when "110110" => g2_reg_s <= "0101011000"; --GPS SVID 31

when "110111" => g2_reg_s <= "1010110000"; --GPS SVID 32

when "111000" => g2_reg_s <= "0100101101"; -- NAVIC-1M

when "111001" => g2_reg_s <= "0101011110"; -- NAVIC-1N

when "111010" => g2_reg_s <= "0000101010";  --0520  PRN 132

when "111011" => g2_reg_s <= "1001101111";  --1731  PRN 133

when "111100" => g2_reg_s <= "0110001110";  --0706  PRN 134

when "111101" => g2_reg_s <= "0111000101";  --1216  PRN 135

when "111110" => g2_reg_s <= "0000011110";  --0740  PRN 136

when others =>   g2_reg_s <= "1110000001";  --1007  PRN 137

end case C_SVID_decoder;   else

Code_freq_phase_s <= Code_freq_phase_s + "00000110100011000001010101001101";--"00101101110101001001010100011000" ;

Codeclk_2p046M_os <= not Code_freq_phase_s(30);

if(Codeclk_1p023M_s= '1') and (Codeclk_1p023M_latched_s = '0')  then

Codeout_os <= g1_reg_s(0) xor g2_reg_s(0);

g1_reg_s(8 downto 0) <= ( g1_reg_s(9 downto 1) );

g2_reg_s(8 downto 0) <= ( g2_reg_s(9 downto 1) );

g1_reg_s(9) <= g1_reg_s(7) xor g1_reg_s(0);

g2_reg_s(9) <= g2_reg_s(8) xor g2_reg_s(7) xor g2_reg_s(4) xor g2_reg_s(2) xor g2_reg_s(1) xor g2_reg_s(0) ;

Codeclk_1p023M_latched_s <= '1' ;

end if;

if(Codeclk_1p023M_s = '0') then

Codeclk_1p023M_latched_s <= '0';

end if;  end if; -- sync reset

end if; -- clk event

end process P_acq_code_Gen;

end acq_code_generator_gp2015_a;

3) acq_down_sampler_gp2015_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_SIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity acq_down_sampler_gp2015_e is

Port( Sys_CLK_40M_is : in  STD_LOGIC;

POR_RST_is : in std_logic;

Load_is : in  STD_LOGIC;

IP_mixout_is: in std_logic_vector(6 downto 0);

QP_mixout_is: in std_logic_vector(6 downto 0);

Completed_accumulation_bi_os : out std_logic;

codeout_is: in std_logic;

sig_accum_inphase_os: out  std_logic_vector(27 downto 0);

write_addr_os : out std_logic_vector(10 downto 0);

edac_wr_en_os:out std_logic);

end acq_down_sampler_gp2015_e;

architecture acq_down_sampler_gp2015_a of acq_down_sampler_gp2015_e is

signal Completed_accumulation_s :  std_logic;

signal sig_accum_inphase_s,sig_accum_qphase_s : std_logic_vector(13 downto 0);

signal intrpt_counter_s: std_logic_vector(8 downto 0);

signal Write_addr_s: std_logic_vector(10 downto 0);

signal wea_eadc_s: std_logic;

signal codemixed_inphase_s, codemixed_qphase_s : std_logic_vector(6 downto 0);

begin

Completed_accumulation_bi_os <= Completed_accumulation_s ;

edac_wr_en_os<= wea_eadc_s;

write_addr_os <= Write_addr_s;

ds_proc: process(Sys_CLK_40M_is,POR_RST_is)

begin

if(POR_RST_is='1') then

sig_accum_inphase_s <= (others=>'0');

sig_accum_qphase_s <=  (others=>'0');

sig_accum_inphase_os <= (others=>'0');

intrpt_counter_s <= (others=>'0');

Write_addr_s  <= (others=>'1');

Completed_accumulation_s <= '0';

wea_eadc_s <= '0';

codemixed_inphase_s <= (others=>'0');

codemixed_qphase_s <= (others=>'0');

elsif Sys_CLK_40M_is='1' and Sys_CLK_40M_is'event then

if(Load_is = '1') then

sig_accum_inphase_s  <= (others=>'0');

sig_accum_qphase_s   <=  (others=>'0');

sig_accum_inphase_os <= (others=>'0');

codemixed_inphase_s  <= (others=>'0');

codemixed_qphase_s   <= (others=>'0');

intrpt_counter_s <= (others=>'0');

Write_addr_s  <= (others=>'1');

Completed_accumulation_s <= '0';

wea_eadc_s <= '0';

else

if(codeout_is='1') then

codemixed_inphase_s <= IP_mixout_is;

codemixed_qphase_s <= QP_mixout_is;

else

codemixed_inphase_s <= -IP_mixout_is;

codemixed_qphase_s <= -QP_mixout_is;

end if;

if ( Completed_accumulation_s = '0') then			intrpt_counter_s <= intrpt_counter_s + 1;

if(intrpt_counter_s = "100101100" ) then --300 (301 = 43 x 7 )

intrpt_counter_s <= (others=>'0');

sig_accum_inphase_s <=   sxt(codemixed_inphase_s(6 downto 0), 14) ;

sig_accum_qphase_s <=     sxt(codemixed_qphase_s(6 downto 0), 14) ;

else

sig_accum_inphase_s <= sig_accum_inphase_s + codemixed_inphase_s;

sig_accum_qphase_s <= sig_accum_qphase_s + codemixed_qphase_s;

end if; end if;

if(intrpt_counter_s =   "100101100" ) then  --300  (total 301)

sig_accum_inphase_os <= sig_accum_inphase_s & sig_accum_qphase_s ;

wea_eadc_s <= '1';

Write_addr_s <= Write_addr_s + 1; else

wea_eadc_s <= '0';

end if;

if(wea_eadc_s = '1') then				if(Write_addr_s = "11111111111") then

Completed_accumulation_s <= '1';

end if;  end if;  end if; -- sync reset  end if; -- CLK Event.

end process ds_proc;

end acq_down_sampler_gp2015_a;

4) acq_fsm1_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

library UNISIM;

use UNISIM.VComponents.all;

entity acq_fsm1_e is

port(    clk_40M_is     : in std_logic; --40MHz Clock

reset_is       : in std_logic; --Synchronised Reset

sat_pr_is      : in std_logic; --Satellite presence after a search

acq_done_is    : in std_logic; --Acquisition search across multiple slews completed

I_data_ant1_is : in std_logic_vector(5 downto 0); -- Ant1 I-data

I_data_ant2_is : in std_logic_vector(5 downto 0); -- Ant2 I-data

I_data_ant3_is : in std_logic_vector(5 downto 0); -- Ant3 I-data

I_data_ant4_is : in std_logic_vector(5 downto 0); -- Ant4 I-data

DSP_SV_reg_ant1_is : in std_logic_vector(63 downto 0); -- Ant1 Almanac register

DSP_SV_reg_ant2_is : in std_logic_vector(63 downto 0); -- Ant2 Almanac register

DSP_SV_reg_ant3_is : in std_logic_vector(63 downto 0); -- Ant3 Almanac register

DSP_SV_reg_ant4_is : in std_logic_vector(63 downto 0); -- Ant4 Almanac register

FPGA_SV_register_is: in std_logic_vector(63 downto 0); -- FPGA Tracking status register

module_id_is       : in std_logic_vector(1 downto 0);  -- Acquisition module id (0/1/2/3)

fsm1_enable_is     : in std_logic;                     -- FSM1 enable

I_mux_out_os: out std_logic_vector(5 downto 0);      -- I-mux output based on antenna selection

svid_code_gen_os: out std_logic_vector(5 downto 0);  -- SVID for code generator module

load_svid_os: out std_logic;                         -- Load signal for rest of the acq mdoule

Antenna_output_os: out std_logic_vector(1 downto 0); -- Antenna output  navic_svid_indicator_os: out std_logic

);

end acq_fsm1_e;

architecture acq_fsm1_a of acq_fsm1_e is

type state_type is (s0,s1,s2,s3,s4,s5,s6,s7,s8,s9); --FSM State variable

signal fsm_state_s, prev_fsm_state_s: state_type;

signal ant_sel_s:std_logic_vector(1 downto 0); --Antenna Selection

signal svid_temp_s:std_logic_vector(5 downto 0);

attribute FSM_ENCODING : string;attribute FSM_ENCODING of fsm_state_s: signal is "ONE-HOT";

attribute SAFE_IMPLEMENTATION :string;

attribute SAFE_IMPLEMENTATION of fsm_state_s: signal is"yes";

begin

svid_code_gen_os  <= svid_temp_s;--module id and local svid is provided to code gen

Antenna_output_os <= ant_sel_s;

--------- FSM State Transition logic --------------

P_FSM_state: process(clk_40M_is,reset_is )

begin

if(reset_is = '1') then

fsm_state_s <= s0; --POR State/Local reset state

elsif(clk_40M_is='1') and (clk_40M_is'event) then

C_fsm_state_dec: case fsm_state_s is

When s0=>if fsm1_enable_is='1' and  FPGA_SV_register_is(conv_integer(svid_temp_s)) = '1'  thenfsm_state_s <= s9; -- Tracking successful, hence go to load state.

elsif  fsm1_enable_is='1' and   DSP_SV_reg_ant1_is(conv_integer(svid_temp_s))='1' then

fsm_state_s <= s1;

elsif  fsm1_enable_is='1' and DSP_SV_reg_ant1_is(conv_integer(svid_temp_s))='0' then

fsm_state_s <= s2;

end if;

when s1=>  if sat_pr_is='1'  then

fsm_state_s <= s9; --Acquisition  successful in Ant1 port go to load state

elsif acq_done_is='1' then

fsm_state_s <= s2;  --Acquisition not successful in Ant1 port

end if;

When s2=>  if fsm1_enable_is='1' and FPGA_SV_register_is(conv_integer(svid_temp_s)) = '1'  then

fsm_state_s <= s9; -- Tracking successful, hence go to load state.

elsif  fsm1_enable_is='1' and DSP_SV_reg_ant2_is(conv_integer(svid_temp_s))='1' then

fsm_state_s <= s3;

elsif fsm1_enable_is='1' and  DSP_SV_reg_ant2_is(conv_integer(svid_temp_s))='0' then

fsm_state_s <= s4;

end if;

when s3=>  if  sat_pr_is='1'  then

fsm_state_s <= s9; --Acquisition  successful in Ant2 port go to load state

elsif  acq_done_is='1' then

fsm_state_s <= s4;  --Acquisition not successful in Ant2 port

end if;

when s4=> if  fsm1_enable_is='1' and FPGA_SV_register_is(conv_integer(svid_temp_s)) = '1'  then

fsm_state_s <= s9; -- Tracking successful, hence go to load state.

elsif fsm1_enable_is='1' and   DSP_SV_reg_ant3_is(conv_integer(svid_temp_s))='1'  then

fsm_state_s <= s5;

elsif fsm1_enable_is='1' and DSP_SV_reg_ant3_is(conv_integer(svid_temp_s))='0' then

fsm_state_s <= s6;

end if;

when s5=>  if  sat_pr_is='1'  then

fsm_state_s <= s9; --Acquisition  successful in Ant2 port go to load state

elsif  acq_done_is='1' then

fsm_state_s <= s6;  --Acquisition not successful in Ant2 port

end if;

when s6=> if  fsm1_enable_is='1' and FPGA_SV_register_is(conv_integer(svid_temp_s)) = '1'  then

fsm_state_s <= s9; -- Tracking successful, hence go to load state.

elsif  fsm1_enable_is='1' and  DSP_SV_reg_ant4_is(conv_integer(svid_temp_s))='1' then

fsm_state_s <= s7;

elsif  fsm1_enable_is='1' and DSP_SV_reg_ant4_is(conv_integer(svid_temp_s))='0' then

fsm_state_s <= s8;

end if;

when s7=> if  sat_pr_is='1'  then

fsm_state_s <= s9; --Acquisition  successful in Ant2 port go to load state

elsif acq_done_is='1' then

fsm_state_s <= s8;  --Acquisition not successful in Ant2 port

end if;

when s8=>   fsm_state_s <= s9; --Goto state s9.

when s9 =>   fsm_state_s <= s0; --Reach initial state again

when others =>  fsm_state_s <= s0; --Reach initial state again

end case C_fsm_state_dec;

end if;

end process P_FSM_state;

--------SVID Increment Logic ---------------------

P_svid_inc: process(clk_40M_is,reset_is )

begin

if(reset_is = '1') then

svid_temp_s <= module_id_is & "0000" ;

elsif(clk_40M_is='1') and (clk_40M_is'event) then

if fsm_state_s=s9 then

svid_temp_s<=svid_temp_s+1;

end if ;       end if;

end process P_svid_inc;

--------Load and reset logic ---------------------

P_load_gen: process(clk_40M_is,reset_is )

begin

if(reset_is = '1') then

load_svid_os<='0';

prev_fsm_state_s<=s0;

elsif(clk_40M_is='1') and (clk_40M_is'event) then

prev_fsm_state_s<= fsm_state_s;

if fsm_state_s=s0 and prev_fsm_state_s=s9  then

load_svid_os<='1';

elsif fsm_state_s=s1 and prev_fsm_state_s=s0 then

load_svid_os<='1';

elsif fsm_state_s=s3 and prev_fsm_state_s=s2 then

load_svid_os<='1';

elsif fsm_state_s=s5 and prev_fsm_state_s=s4 then

load_svid_os<='1';

elsif fsm_state_s=s7 and prev_fsm_state_s=s6 then

load_svid_os<='1';

else

load_svid_os<='0';

end if;

end if;

end process P_load_gen;

--------Antenna Selection logic ----------------

P_antenna_sel: process(clk_40M_is,reset_is )

begin

if(reset_is = '1') then

ant_sel_s<="00";

elsif(clk_40M_is='1') and (clk_40M_is'event) then

if fsm_state_s=s0 or fsm_state_s=s1 then ant_sel_s<="00";

elsif fsm_state_s=s2 or fsm_state_s=s3 then ant_sel_s<="01";

elsif fsm_state_s=s4 or fsm_state_s=s5 then ant_sel_s<="10";

elsif fsm_state_s=s6 or fsm_state_s=s7 then ant_sel_s<="11";

end if;  end if;

end process P_antenna_sel;

-------- Navic SVID Indicator generation------------

P_NAVIC_ind: process( svid_temp_s)

begin

C_navid_ind_dec: case  svid_temp_s is

when "001000" => navic_svid_indicator_os<='1';

when "001001" => navic_svid_indicator_os<='1';

when "001010" => navic_svid_indicator_os<='1';

when "001011" => navic_svid_indicator_os<='1';

when "011000" => navic_svid_indicator_os<='1';

when "011001" => navic_svid_indicator_os<='1';

when "011010" => navic_svid_indicator_os<='1';

when "011011" => navic_svid_indicator_os<='1';

when "101000" => navic_svid_indicator_os<='1';

when "101001" => navic_svid_indicator_os<='1';

when "101010" => navic_svid_indicator_os<='1';

when "101011" => navic_svid_indicator_os<='1';

when "111000" => navic_svid_indicator_os<='1';

when "111001" => navic_svid_indicator_os<='1';

when  OTHERS  => navic_svid_indicator_os<='0';

end case C_navid_ind_dec;

end process P_NAVIC_ind;

--------- I Data Mux output based on Antenna selection logic----------

P_mux: process(ant_sel_s,I_data_ant1_is,I_data_ant2_is,I_data_ant3_is,I_data_ant4_is)

begin

C_antenna_sel_dec: case ant_sel_s is

when "00" =>  I_mux_out_os <= I_data_ant1_is;

when "01" =>  I_mux_out_os <= I_data_ant2_is;

when "10" =>  I_mux_out_os <= I_data_ant3_is;

when "11" =>  I_mux_out_os <= I_data_ant4_is;

when OTHERS => I_mux_out_os <= I_data_ant1_is;

end case C_antenna_sel_dec;

end process P_mux;

end acq_fsm1_a;

5) acq_fsm2_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

use WORK.signal_type_p.ALL;

library UNISIM;

use UNISIM.VComponents.all;

entity acq_fsm2_e is

port(  clk_40M_is     : in std_logic; --40MHz Clock

reset_is       : in std_logic; --Synchronised Reset

accum_over_is  : in std_logic; -- Accumulation Over for all 25 modules

--fft_edone_is   : in std_logic; -- FFT Early Done signal

fft_done_is    : in std_logic; -- FFT Done signal

fft_max_done_is : in std_logic; -- FFT Maxium computed signal

fsm2_enable_is  : in std_logic; load_svid_is       : in std_logic;

current_module_os: out std_logic_vector(5 downto 0); -- Current parallel module.

read_out_sig_os  : out std_logic_vector(acq_parallel_mods_c downto 0);

fft_start_os     : out std_logic;  -- Start signal to FFT Module.

fft_unload_os    : out std_logic;  -- unload signal to FFT module.

--  fft_enable_os    : out std_logic;  -- enable signal to FFT module

--fft_reset_os     : out std_logic;

completed_25mod_os : out std_logic;

edac_rd_en_os   : out std_logic;

acq_done_os      : out std_logic   );

end acq_fsm2_e;

architecture acq_fsm2_a of acq_fsm2_e is

type state_type is (s1,s2,s3,s4,s5); --FSM State type

signal fsm_state_s: state_type; --FSM State variable

signal prev_state_s: state_type; -- Previous State variable

constant slew_count_limit_c : std_logic_vector(6 downto 0) :=  "1110000";

signal current_module_s : integer range 0 to acq_parallel_mods_c; --can take value between 0 to acq_parallel_mods_c.

signal slew_count_s: std_logic_vector(6 downto 0);-- lies between 0 to 128

signal slew_count_en_s: std_logic;

signal completed_25mod_s,completed_25mod_out_s: std_logic;

signal e_done_counter_s : std_logic_vector(13 downto 0);

signal fft_edone_na_s  : std_logic; -- FFT edone not available.

signal fft_edone_na1_s : std_logic; -- FFT edone not available with 1 cycle dealy

signal start_edone_counter_s : std_logic;

signal fft_start_s,fft_start_reg_s: std_logic;

signal fft_unload_s,fft_unload_reg_s: std_logic;

attribute FSM_ENCODING : string;

attribute FSM_ENCODING of fsm_state_s: signal is "ONE-HOT";

attribute SAFE_IMPLEMENTATION :string;

attribute SAFE_IMPLEMENTATION of fsm_state_s: signal is"yes";

begin

current_module_os<= conv_std_logic_vector (current_module_s,6);

completed_25mod_os<= completed_25mod_out_s;

--fft_reset_os <= fft_reset_s;

fft_start_os  <= fft_start_s or fft_start_reg_s;

fft_unload_os <= fft_unload_s or fft_unload_reg_s;

--------- FSM State Transition logic --------------

P_FSM_state_gen: process(clk_40M_is,reset_is )

begin

if(reset_is = '1') then

fsm_state_s <= s1; --POR State/Local reset state.

elsif(clk_40M_is='1') and (clk_40M_is'event) then

C_fsm_state_dec: case fsm_state_s is

when s1=>  if fsm2_enable_is='1' and completed_25mod_s='0' then

fsm_state_s<=s2; end if;

when s2=>  if accum_over_is='1' then

fsm_state_s<=s3;   end if;

when s3=>  if fft_done_is ='1' or fft_edone_na_s='1' then

fsm_state_s<=s4;

elsif fsm2_enable_is='0' then fsm_state_s <=s1; end if;

when s4=>  if fft_max_done_is='1' or  fft_edone_na1_s='1' then

fsm_state_s<=s5; end if;

when s5=>  if completed_25mod_s='1' or fsm2_enable_is='0' then

fsm_state_s<=s1;

elsif completed_25mod_s='0' then fsm_state_s <=s2;

end if;

when others=> fsm_state_s <=s1;

end case C_fsm_state_dec; end if;

end process P_FSM_state_gen;

slew_count_en_s <='1' when prev_state_s=s5 and fsm_state_s=s1 else

'0';

--FSM Outputs based on Current state

P_output_gen: process(clk_40M_is,reset_is )

begin

if(reset_is = '1') then

current_module_s<=0; read_out_sig_os<=(others=>'0');

prev_state_s<= s1; slew_count_s<=(others=>'0');

acq_done_os<='0';  edac_rd_en_os <='0';

elsif clk_40M_is='1' and clk_40M_is'event then

prev_state_s<=fsm_state_s;

-- Increment current module.. from 0 to acq_parallel_mods_c.

if fsm_state_s=s1   then current_module_s<=0;

elsif fsm_state_s=s5 and prev_state_s=s4   then

if current_module_s=acq_parallel_mods_c then

current_module_s<=0;  else

current_module_s <= current_module_s+1;

end if ; end if;

--- Increment slew count.. rage 0 to 128.

if load_svid_is='1' then slew_count_s<=(others=>'0');

elsif slew_count_s >=slew_count_limit_c then

slew_count_s<=(others=>'0');

elsif slew_count_en_s='1'then

slew_count_s <= slew_count_s+1;  end if;

---- Generate Acquisition done signal

if slew_count_s >=slew_count_limit_c  then

acq_done_os<='1';  else

acq_done_os<='0'; end if;

-- Generate read out signal

if fsm_state_s=s3  then

read_out_sig_os(current_module_s)<='1';

edac_rd_en_os <='1'; else

read_out_sig_os(current_module_s)<='0';

edac_rd_en_os <='0'; end if;end if;

end process P_output_gen;

P_fft_sig_gen: process(clk_40M_is,reset_is )

begin

if(reset_is = '1') then

fft_unload_s<='0';fft_unload_reg_s <='0';

fft_start_s<='0';fft_start_reg_s <='0';

completed_25mod_s<='0';

completed_25mod_out_s<='0';

elsif(clk_40M_is='1') and (clk_40M_is'event) then

fft_unload_reg_s <= fft_unload_s;

fft_start_reg_s  <= fft_start_s;

--FFT 25 modules completed signal

if fsm_state_s=s5  then

if current_module_s=acq_parallel_mods_c then

completed_25mod_out_s<='1'; else

completed_25mod_out_s<='0'; end if;

elsif fsm_state_s=s1 then

completed_25mod_out_s<='0'; end if;

if fsm2_enable_is='1' and (fft_max_done_is='1' or  fft_edone_na1_s='1') then

if current_module_s=acq_parallel_mods_c then

completed_25mod_s<='1'; end if;

elsif fsm2_enable_is='0' then

completed_25mod_s<='0';        end if;

if fsm_state_s=s3 and prev_state_s=s2  then

fft_start_s<='1'; else

fft_start_s<='0'; end if;

-- FFT Unload signal generation -- one cycle high after edone detection.

if fsm_state_s=s4 and prev_state_s=s3 then fft_unload_s<='1';

else fft_unload_s<='0';  end if; end if;

end process P_fft_sig_gen;

-- Generating FFT edone not available signal.

P_done_gen: process(clk_40M_is,reset_is )

begin

if(reset_is = '1') then

e_done_counter_s <= (others => '0'); fft_edone_na_s <= '0';

fft_edone_na1_s <= '0';  start_edone_counter_s <= '0';

elsif(clk_40M_is='1') and (clk_40M_is'event) then

fft_edone_na1_s<= fft_edone_na_s;

if(fft_start_s = '1') then start_edone_counter_s <= '1';

elsif fft_done_is='1' then  start_edone_counter_s <= '0';

elsif(e_done_counter_s >= "11010101011011") then  --13659 cycles:

start_edone_counter_s <= '0';          end if;

if(start_edone_counter_s = '1') then

e_done_counter_s <= e_done_counter_s + 1; else

e_done_counter_s <= (others => '0');     end if; if(e_done_counter_s >= "11010101011010" ) then --13658 120 cycles more than conversion time of FFT

fft_edone_na_s <= '1'; else  fft_edone_na_s <= '0'; end if;

end if; end process P_done_gen; end acq_fsm2_a;

6) acq_memory_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity acq_memory_e is

Port( Sys_CLK_40M_is : in  STD_LOGIC;

ram_wr_en_is:in std_logic;

write_addr_is : in std_logic_vector(10 downto 0);

acq_bram_din_is : in std_logic_vector(104 downto 0);

enableb_is: in std_logic;

addressb_is: in std_logic_vector(10 downto 0);

sig_accum_inphase_ram_out_os: out  std_logic_vector(104 downto 0));

end acq_memory_e;

architecture acq_memory_a of acq_memory_e is

COMPONENT acq_bram1

PORT (

clka : IN STD_LOGIC;  ena : IN STD_LOGIC;

wea : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

addra : IN STD_LOGIC_VECTOR(10 DOWNTO 0);

dina : IN STD_LOGIC_VECTOR(104 downto 0);

clkb : IN STD_LOGIC;enb : IN STD_LOGIC;

addrb : IN STD_LOGIC_VECTOR(10 DOWNTO 0);

doutb : OUT STD_LOGIC_VECTOR(104 downto 0)

);

END COMPONENT;

signal edac_wr_en_in_temp: std_logic_vector(0 downto 0);

begin

edac_wr_en_in_temp(0)<=ram_wr_en_is;

downsampler_ram : acq_bram1

PORT MAP ( clka => Sys_CLK_40M_is,

ena => ram_wr_en_is,   wea => edac_wr_en_in_temp,

addra => write_addr_is,   dina => acq_bram_din_is ,

clkb => Sys_CLK_40M_is,  enb => enableb_is,

addrb => addressb_is,   doutb => sig_accum_inphase_ram_out_os

);

end acq_memory_a;

7) acqmod_almanc_reg_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

use WORK.signal_type_p.ALL;

entity acqmod_almanc_reg_e is

port ( clk_40M_is     : in std_logic; --40MHz Clock

reset_is       : in std_logic; --Synchronised Reset

alm_ant1_gps_reg_is  : in std_logic_vector(acq_modules_count_c downto 0); -- Ant-1 GPS

alm_ant2_gps_reg_is  : in std_logic_vector(acq_modules_count_c downto 0); -- Ant-2 GPS

alm_ant3_gps_reg_is  : in std_logic_vector(acq_modules_count_c downto 0); -- Ant-3 GPS

alm_ant4_gps_reg_is  : in std_logic_vector(acq_modules_count_c downto 0); -- Ant-4 GPS

alm_ant1_nav_reg_is  : in std_logic_vector(acq_modules_count_c downto 0); -- Ant-1 NAVIC

alm_ant2_nav_reg_is  : in std_logic_vector(acq_modules_count_c downto 0); -- Ant-2 NAVIC

alm_ant3_nav_reg_is  : in std_logic_vector(acq_modules_count_c downto 0); -- Ant-3 NAVIC

alm_ant4_nav_reg_is  : in std_logic_vector(acq_modules_count_c downto 0); -- Ant-4 NAVIC

alm_ant1_sbas_reg_is : in std_logic_vector(acq_modules_count_c downto 0); -- Ant-1 SBAS

alm_ant2_sbas_reg_is : in std_logic_vector(acq_modules_count_c downto 0); -- Ant-2 SBAS

alm_ant3_sbas_reg_is : in std_logic_vector(acq_modules_count_c downto 0); -- Ant-3 SBAS

alm_ant4_sbas_reg_is : in std_logic_vector(acq_modules_count_c downto 0); -- Ant-4 SBAS

data_processor_is    : in std_logic_vector(31 downto 0);

DSP_SV_register_ant1_os  : out sixtyfour_acq1_4_typ;

DSP_SV_register_ant2_os  : out sixtyfour_acq1_4_typ;

DSP_SV_register_ant3_os  : out sixtyfour_acq1_4_typ;

DSP_SV_register_ant4_os  : out sixtyfour_acq1_4_typ   );

end acqmod_almanc_reg_e;

architecture acqmod_almanc_reg_a of acqmod_almanc_reg_e is

signal DSP_SV_register_ant1_s, DSP_SV_register_ant2_s ,DSP_SV_register_ant3_s , DSP_SV_register_ant4_s : sixtyfour_acq1_4_typ;

begin

DSP_SV_register_ant1_os <= DSP_SV_register_ant1_s;

DSP_SV_register_ant2_os <= DSP_SV_register_ant2_s;

DSP_SV_register_ant3_os <= DSP_SV_register_ant3_s;

DSP_SV_register_ant4_os <= DSP_SV_register_ant4_s;

Process_p1: process(clk_40M_is, reset_is)

begin

if(reset_is = '1') then

-- Acquistion module-1, Antenna-1, GPS

DSP_SV_register_ant1_s(0)(15 downto 0)  <= "0000000011111111";  -- GPS 1-8

DSP_SV_register_ant1_s(0)(31 downto 16) <= "0000000000000000";

DSP_SV_register_ant1_s(0)(47 downto 32) <= "0000000000000000";

DSP_SV_register_ant1_s(0)(63 downto 48) <= "0000000000000000";

-- Acquistion module-1, Antenna-2, NAVIC

DSP_SV_register_ant2_s(0)(15 downto 0) <= "0000111100000000";   -- NAVIC 1-4

DSP_SV_register_ant2_s(0)(31 downto 16) <= "0000000000000000";

DSP_SV_register_ant2_s(0)(47 downto 32) <= "0000000000000000";

DSP_SV_register_ant2_s(0)(63 downto 48) <= "0000000000000000";

-- Acquistion module-1, Antenna-3, GPS

DSP_SV_register_ant3_s(0)(15 downto 0) <= "0000000011111111";   -- GPS 1-8

DSP_SV_register_ant3_s(0)(31 downto 16) <= "0000000000000000";

DSP_SV_register_ant3_s(0)(47 downto 32) <= "0000000000000000";

DSP_SV_register_ant3_s(0)(63 downto 48) <= "0000000000000000";

-- Acquistion module-1, Antenna-4, NAVIC

DSP_SV_register_ant4_s(0)(15 downto 0) <= "0000111100000000";   -- NAVIC 1-4

DSP_SV_register_ant4_s(0)(31 downto 16) <= "0000000000000000";

DSP_SV_register_ant4_s(0)(47 downto 32) <= "0000000000000000";

DSP_SV_register_ant4_s(0)(63 downto 48) <= "0000000000000000";

-- Acquistion module-2, Antenna-1, GPS

DSP_SV_register_ant1_s(1)(15 downto 0) <= "0000000000000000";

DSP_SV_register_ant1_s(1)(31 downto 16) <= "0000000011111111";  -- GPS 9-16

DSP_SV_register_ant1_s(1)(47 downto 32) <= "0000000000000000";

DSP_SV_register_ant1_s(1)(63 downto 48) <= "0000000000000000";

-- Acquistion module-2, Antenna-2, NAVIC

DSP_SV_register_ant2_s(1)(15 downto 0) <= "0000000000000000";

DSP_SV_register_ant2_s(1)(31 downto 16) <= "0000001100000000";

DSP_SV_register_ant2_s(1)(47 downto 32) <= "0000000000000000";

DSP_SV_register_ant2_s(1)(63 downto 48) <= "0000000000000000";

-- Acquistion module-2, Antenna-3, GPS

DSP_SV_register_ant3_s(1)(15 downto 0) <= "0000000000000000";

DSP_SV_register_ant3_s(1)(31 downto 16) <= "0000000011111111";  -- GPS 9-16

DSP_SV_register_ant3_s(1)(47 downto 32) <= "0000000000000000";

DSP_SV_register_ant3_s(1)(63 downto 48) <= "0000000000000000";

-- Acquistion module-2, Antenna-4, NAVIC

DSP_SV_register_ant4_s(1)(15 downto 0) <= "0000000000000000";

DSP_SV_register_ant4_s(1)(31 downto 16) <= "0000001100000000";

DSP_SV_register_ant4_s(1)(47 downto 32) <= "0000000000000000";

DSP_SV_register_ant4_s(1)(63 downto 48) <= "0000000000000000";

-- Acquistion module-3, Antenna-1, GPS

DSP_SV_register_ant1_s(2)(15 downto 0) <= "0000000000000000";

DSP_SV_register_ant1_s(2)(31 downto 16) <= "0000000000000000";

DSP_SV_register_ant1_s(2)(47 downto 32) <= "0000000011111111";  -- GPS 17-24

DSP_SV_register_ant1_s(2)(63 downto 48) <= "0000000000000000";

-- Acquistion module-3, Antenna-2, NAVIC

DSP_SV_register_ant2_s(2)(15 downto 0) <= "0000000000000000";

DSP_SV_register_ant2_s(2)(31 downto 16) <= "0000000000000000";

DSP_SV_register_ant2_s(2)(47 downto 32) <= "0000000100000000";

DSP_SV_register_ant2_s(2)(63 downto 48) <= "0000000000000000";

-- Acquistion module-3, Antenna-3, GPS

DSP_SV_register_ant3_s(2)(15 downto 0) <= "0000000000000000";

DSP_SV_register_ant3_s(2)(31 downto 16) <= "0000000000000000";

DSP_SV_register_ant3_s(2)(47 downto 32) <= "0000000011111111";  -- GPS 17-24

DSP_SV_register_ant3_s(2)(63 downto 48) <= "0000000000000000";

-- Acquistion module-3, Antenna-4, NAVIC

DSP_SV_register_ant4_s(2)(15 downto 0) <= "0000000000000000";

DSP_SV_register_ant4_s(2)(31 downto 16) <= "0000000000000000";

DSP_SV_register_ant4_s(2)(47 downto 32) <= "0000000100000000";

DSP_SV_register_ant4_s(2)(63 downto 48) <= "0000000000000000";

-- Acquistion module-4, Antenna-1, GPS

DSP_SV_register_ant1_s(3)(15 downto 0) <= "0000000000000000";

DSP_SV_register_ant1_s(3)(31 downto 16) <= "0000000000000000";

DSP_SV_register_ant1_s(3)(47 downto 32) <= "0000000000000000";

DSP_SV_register_ant1_s(3)(63 downto 48) <= "0000000011111111";  -- GPS 25-32

-- Acquistion module-4, Antenna-2, NAVIC

DSP_SV_register_ant2_s(3)(15 downto 0)  <= "0000000000000000";

DSP_SV_register_ant2_s(3)(31 downto 16) <= "0000000000000000";

DSP_SV_register_ant2_s(3)(47 downto 32) <= "0000000000000000";

DSP_SV_register_ant2_s(3)(63 downto 48) <= "0000000000000000";

-- Acquistion module-4, Antenna-3, GPS

DSP_SV_register_ant3_s(3)(15 downto 0) <= "0000000000000000";

DSP_SV_register_ant3_s(3)(31 downto 16) <= "0000000000000000";

DSP_SV_register_ant3_s(3)(47 downto 32) <= "0000000000000000";

DSP_SV_register_ant3_s(3)(63 downto 48) <= "0000000011111111";  -- GPS 25-32

-- Acquistion module-4, Antenna-4, NAVIC

DSP_SV_register_ant4_s(3)(15 downto 0) <= "0000000000000000";

DSP_SV_register_ant4_s(3)(31 downto 16) <= "0000000000000000";

DSP_SV_register_ant4_s(3)(47 downto 32) <= "0000000000000000";

DSP_SV_register_ant4_s(3)(63 downto 48) <= "0000000000000000";

elsif(clk_40M_is='1') and (clk_40M_is'event) then

-- Acquisition module-1.

-- Acquisition module-1, Antenna-1 DSP almanac register.

if alm_ant1_gps_reg_is(0) = '1' then

DSP_SV_register_ant1_s(0)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant1_s(0)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant1_s(0)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant1_s(0)(55 downto 48)  <= data_processor_is(31 downto 24);

end if;

if alm_ant1_nav_reg_is(0) = '1' then

DSP_SV_register_ant1_s(0)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant1_s(0)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant1_s(0)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant1_s(0)(57 downto 56) <= data_processor_is(13 downto 12); end if;

if alm_ant1_sbas_reg_is(0) = '1'  then

DSP_SV_register_ant1_s(0)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant1_s(0)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant1_s(0)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant1_s(0)(63 downto 58) <= data_processor_is(17 downto 12); end if;

-- Acquisition module-1, Antenna-2 DSP almanac register

if alm_ant2_gps_reg_is(0) = '1' then

DSP_SV_register_ant2_s(0)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant2_s(0)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant2_s(0)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant2_s(0)(55 downto 48)  <= data_processor_is(31 downto 24); end if;

if alm_ant2_nav_reg_is(0) = '1' then

DSP_SV_register_ant2_s(0)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant2_s(0)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant2_s(0)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant2_s(0)(57 downto 56) <= data_processor_is(13 downto 12); end if;

if alm_ant2_sbas_reg_is(0) = '1' then

DSP_SV_register_ant2_s(0)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant2_s(0)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant2_s(0)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant2_s(0)(63 downto 58) <= data_processor_is(17 downto 12);	end if;

-- Acquisition module-1, Antenna-3 DSP almanac register

if alm_ant3_gps_reg_is(0) = '1' then

DSP_SV_register_ant3_s(0)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant3_s(0)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant3_s(0)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant3_s(0)(55 downto 48)  <= data_processor_is(31 downto 24);	end if;

if alm_ant3_nav_reg_is(0) = '1' then

DSP_SV_register_ant3_s(0)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant3_s(0)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant3_s(0)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant3_s(0)(57 downto 56) <= data_processor_is(13 downto 12);	end if;

if alm_ant3_sbas_reg_is(0) = '1' then

DSP_SV_register_ant3_s(0)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant3_s(0)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant3_s(0)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant3_s(0)(63 downto 58) <= data_processor_is(17 downto 12);	end if;

-- Acquisition module-1, Antenna-4 DSP almanac register

if alm_ant4_gps_reg_is(0) = '1' then

DSP_SV_register_ant4_s(0)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant4_s(0)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant4_s(0)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant4_s(0)(55 downto 48)  <= data_processor_is(31 downto 24); end if;

if alm_ant4_nav_reg_is(0) = '1' then

DSP_SV_register_ant4_s(0)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant4_s(0)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant4_s(0)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant4_s(0)(57 downto 56) <= data_processor_is(13 downto 12);	end if;

if alm_ant4_sbas_reg_is(0) = '1' then

DSP_SV_register_ant4_s(0)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant4_s(0)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant4_s(0)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant4_s(0)(63 downto 58) <= data_processor_is(17 downto 12);	end if;

-- Acquisition module-2

-- Acquisition module-2, Antenna-1 DSP almanac register

if alm_ant1_gps_reg_is(1) = '1' then

DSP_SV_register_ant1_s(1)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant1_s(1)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant1_s(1)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant1_s(1)(55 downto 48)  <= data_processor_is(31 downto 24);	end if;

if alm_ant1_nav_reg_is(1) = '1' then

DSP_SV_register_ant1_s(1)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant1_s(1)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant1_s(1)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant1_s(1)(57 downto 56) <= data_processor_is(13 downto 12); end if;

if alm_ant1_sbas_reg_is(1) = '1'  then

DSP_SV_register_ant1_s(1)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant1_s(1)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant1_s(1)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant1_s(1)(63 downto 58) <= data_processor_is(17 downto 12);	end if;

-- Acquisition module-2, Antenna-2 DSP almanac register

if alm_ant2_gps_reg_is(1) = '1' then

DSP_SV_register_ant2_s(1)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant2_s(1)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant2_s(1)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant2_s(1)(55 downto 48)  <= data_processor_is(31 downto 24);	end if;

if alm_ant2_nav_reg_is(1) = '1' then

DSP_SV_register_ant2_s(1)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant2_s(1)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant2_s(1)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant2_s(1)(57 downto 56) <= data_processor_is(13 downto 12); end if;

if alm_ant2_sbas_reg_is(1) = '1' then

DSP_SV_register_ant2_s(1)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant2_s(1)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant2_s(1)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant2_s(1)(63 downto 58) <= data_processor_is(17 downto 12);	end if;

-- Acquisition module-2, Antenna-3 DSP almanac register

if alm_ant3_gps_reg_is(1) = '1' then

DSP_SV_register_ant3_s(1)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant3_s(1)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant3_s(1)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant3_s(1)(55 downto 48)  <= data_processor_is(31 downto 24);  end if;

if alm_ant3_nav_reg_is(1) = '1' then

DSP_SV_register_ant3_s(1)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant3_s(1)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant3_s(1)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant3_s(1)(57 downto 56) <= data_processor_is(13 downto 12);	end if;

if alm_ant3_sbas_reg_is(1) = '1' then

DSP_SV_register_ant3_s(1)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant3_s(1)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant3_s(1)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant3_s(1)(63 downto 58) <= data_processor_is(17 downto 12);	end if;

-- Acquisition module-2, Antenna-4 DSP almanac register

if alm_ant4_gps_reg_is(1) = '1' then

DSP_SV_register_ant4_s(1)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant4_s(1)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant4_s(1)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant4_s(1)(55 downto 48)  <= data_processor_is(31 downto 24);	end if;

if alm_ant4_nav_reg_is(1) = '1' then

DSP_SV_register_ant4_s(1)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant4_s(1)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant4_s(1)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant4_s(1)(57 downto 56) <= data_processor_is(13 downto 12);end if;

if alm_ant4_sbas_reg_is(1) = '1' then

DSP_SV_register_ant4_s(1)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant4_s(1)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant4_s(1)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant4_s(1)(63 downto 58) <= data_processor_is(17 downto 12);	end if;

-- Acquisition module-3

-- Acquisition module-3, Antenna-1 DSP almanac register

if alm_ant1_gps_reg_is(2) = '1' then

DSP_SV_register_ant1_s(2)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant1_s(2)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant1_s(2)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant1_s(2)(55 downto 48)  <= data_processor_is(31 downto 24);	end if;

if alm_ant1_nav_reg_is(2) = '1' then

DSP_SV_register_ant1_s(2)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant1_s(2)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant1_s(2)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant1_s(2)(57 downto 56) <= data_processor_is(13 downto 12); end if;

if alm_ant1_sbas_reg_is(2) = '1'  then

DSP_SV_register_ant1_s(2)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant1_s(2)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant1_s(2)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant1_s(2)(63 downto 58) <= data_processor_is(17 downto 12);  end if;

-- Acquisition module-3, Antenna-2 DSP almanac register

if alm_ant2_gps_reg_is(2) = '1' then

DSP_SV_register_ant2_s(2)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant2_s(2)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant2_s(2)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant2_s(2)(55 downto 48)  <= data_processor_is(31 downto 24); end if;

if alm_ant2_nav_reg_is(2) = '1' then

DSP_SV_register_ant2_s(2)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant2_s(2)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant2_s(2)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant2_s(2)(57 downto 56) <= data_processor_is(13 downto 12);  end if;

if alm_ant2_sbas_reg_is(2) = '1' then

DSP_SV_register_ant2_s(2)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant2_s(2)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant2_s(2)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant2_s(2)(63 downto 58) <= data_processor_is(17 downto 12);	end if;

-- Acquisition module-3, Antenna-3 DSP almanac register

if alm_ant3_gps_reg_is(2) = '1' then

DSP_SV_register_ant3_s(2)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant3_s(2)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant3_s(2)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant3_s(2)(55 downto 48)  <= data_processor_is(31 downto 24);end if;

if alm_ant3_nav_reg_is(2) = '1' then

DSP_SV_register_ant3_s(2)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant3_s(2)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant3_s(2)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant3_s(2)(57 downto 56) <= data_processor_is(13 downto 12);	end if;

if alm_ant3_sbas_reg_is(2) = '1' then

DSP_SV_register_ant3_s(2)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant3_s(2)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant3_s(2)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant3_s(2)(63 downto 58) <= data_processor_is(17 downto 12);	end if;

-- Acquisition module-3, Antenna-4 DSP almanac register

if alm_ant4_gps_reg_is(2) = '1' then

DSP_SV_register_ant4_s(2)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant4_s(2)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant4_s(2)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant4_s(2)(55 downto 48)  <= data_processor_is(31 downto 24);		end if;

if alm_ant4_nav_reg_is(2) = '1' then

DSP_SV_register_ant4_s(2)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant4_s(2)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant4_s(2)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant4_s(2)(57 downto 56) <= data_processor_is(13 downto 12);	end if;

if alm_ant4_sbas_reg_is(2) = '1' then

DSP_SV_register_ant4_s(2)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant4_s(2)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant4_s(2)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant4_s(2)(63 downto 58) <= data_processor_is(17 downto 12);		end if;

-- Acquisition module-4

-- Acquisition module-4, Antenna-1 DSP almanac register

if alm_ant1_gps_reg_is(3) = '1' then

DSP_SV_register_ant1_s(3)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant1_s(3)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant1_s(3)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant1_s(3)(55 downto 48)  <= data_processor_is(31 downto 24);  end if;

if alm_ant1_nav_reg_is(3) = '1' then

DSP_SV_register_ant1_s(3)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant1_s(3)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant1_s(3)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant1_s(3)(57 downto 56) <= data_processor_is(13 downto 12); end if;

if alm_ant1_sbas_reg_is(3) = '1'  then

DSP_SV_register_ant1_s(3)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant1_s(3)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant1_s(3)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant1_s(3)(63 downto 58) <= data_processor_is(17 downto 12);		end if;

-- Acquisition module-4, Antenna-2 DSP almanac register

if alm_ant2_gps_reg_is(3) = '1' then

DSP_SV_register_ant2_s(3)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant2_s(3)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant2_s(3)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant2_s(3)(55 downto 48)  <= data_processor_is(31 downto 24);	end if;

if alm_ant2_nav_reg_is(3) = '1' then

DSP_SV_register_ant2_s(3)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant2_s(3)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant2_s(3)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant2_s(3)(57 downto 56) <= data_processor_is(13 downto 12);  end if;

if alm_ant2_sbas_reg_is(3) = '1' then

DSP_SV_register_ant2_s(3)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant2_s(3)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant2_s(3)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant2_s(3)(63 downto 58) <= data_processor_is(17 downto 12);		end if;

-- Acquisition module-4, Antenna-3 DSP almanac register

if alm_ant3_gps_reg_is(3) = '1' then

DSP_SV_register_ant3_s(3)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant3_s(3)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant3_s(3)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant3_s(3)(55 downto 48)  <= data_processor_is(31 downto 24);	end if;

if alm_ant3_nav_reg_is(3) = '1' then

DSP_SV_register_ant3_s(3)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant3_s(3)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant3_s(3)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant3_s(3)(57 downto 56) <= data_processor_is(13 downto 12);		end if;

if alm_ant3_sbas_reg_is(3) = '1' then

DSP_SV_register_ant3_s(3)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant3_s(3)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant3_s(3)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant3_s(3)(63 downto 58) <= data_processor_is(17 downto 12);		end if;

-- Acquisition module-4, Antenna-4 DSP almanac register

if alm_ant4_gps_reg_is(3) = '1' then

DSP_SV_register_ant4_s(3)(7 downto 0)    <= data_processor_is(7 downto 0);

DSP_SV_register_ant4_s(3)(23 downto 16)  <= data_processor_is(15 downto 8);

DSP_SV_register_ant4_s(3)(39 downto 32)  <= data_processor_is(23 downto 16);

DSP_SV_register_ant4_s(3)(55 downto 48)  <= data_processor_is(31 downto 24);

end if;

if alm_ant4_nav_reg_is(3) = '1' then

DSP_SV_register_ant4_s(3)(11 downto 8) <= data_processor_is(3 downto 0);

DSP_SV_register_ant4_s(3)(27 downto 24) <= data_processor_is(7 downto 4);

DSP_SV_register_ant4_s(3)(43 downto 40) <= data_processor_is(11 downto 8);

DSP_SV_register_ant4_s(3)(57 downto 56) <= data_processor_is(13 downto 12);  end if;

if alm_ant4_sbas_reg_is(3) = '1' then

DSP_SV_register_ant4_s(3)(15 downto 12) <= data_processor_is(3 downto 0);

DSP_SV_register_ant4_s(3)(31 downto 28) <= data_processor_is(7 downto 4);

DSP_SV_register_ant4_s(3)(47 downto 44) <= data_processor_is(11 downto 8);

DSP_SV_register_ant4_s(3)(63 downto 58) <= data_processor_is(17 downto 12);	end if;   end if;

end process Process_p1;

end acqmod_almanc_reg_a;

8) acq_module0_controller_gp2015_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_SIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

use WORK.signal_type_p.ALL;

entity acq_module0_controller_gp2015_e is

port (

Sys_CLK_40M_is : in  STD_LOGIC;

POR_RST_is : in  STD_LOGIC;

Incoming_data_ant1_is : in std_logic_vector(5 downto 0);

Incoming_data_ant2_is : in std_logic_vector(5 downto 0);

Incoming_data_ant3_is : in std_logic_vector(5 downto 0);

Incoming_data_ant4_is : in std_logic_vector(5 downto 0);

DSP_SV_register_ant1_is : in std_logic_vector(63 downto 0);

DSP_SV_register_ant2_is : in std_logic_vector(63 downto 0);

DSP_SV_register_ant3_is : in std_logic_vector(63 downto 0);

DSP_SV_register_ant4_is : in std_logic_vector(63 downto 0);

FPGA_SV_register_is : in std_logic_vector(63 downto 0);

Module_input_is :in std_logic_vector(1 downto 0);

g2_reg_input_is : in std_logic_vector(9 downto 0);

Acquisition_threshold_is : in std_logic_vector(51 downto 0);

IF_Freq_GPS_is : in std_logic_vector(31 downto 0);

IF_Freq_NAVIC_is : in std_logic_vector(31 downto 0);

car_mix_ROM_ena_is : IN STD_LOGIC;

car_mix_ROM_wea_is : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

car_mix_ROM_addra_is : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

car_mix_ROM_dina_is : IN STD_LOGIC_VECTOR(19 DOWNTO 0);

fft_done_is : in std_logic;

xk_re1_is : in std_logic_vector(25 downto 0);

xk_im1_is : in std_logic_vector(25 downto 0);

xk_re2_is : in std_logic_vector(25 downto 0);

xk_im2_is : in std_logic_vector(25 downto 0);

xn_index_is : in std_logic_vector(10 downto 0);

set_delay_is : in std_logic;

SVID_acquired_os : out std_logic_vector(5 downto 0);

SVID_Searched_os : out std_logic_vector(5 downto 0);

Antenna_output_os : out std_logic_vector(1 downto 0);

Track_epoch_output_os : out std_logic;

SVID_loading_is : in std_logic;

car_freq_init_os : out std_logic_vector(31 downto 0);

code_freq_init_os : out std_logic_vector(31 downto 0);		order1_loop_init_os: out std_logic_vector(48 downto 0);

dll_order1_loop_init_os:out std_logic_vector(48 downto 0);

xn_re1_os : out std_logic_vector(13 downto 0);

xn_im1_os : out std_logic_vector(13 downto 0);

xn_re2_os : out std_logic_vector(13 downto 0);

xn_im2_os : out std_logic_vector(13 downto 0);

fft_start_os : out std_logic;

fft_unload_os : out std_logic;

fsm2_enable_os: out std_logic;

slew_done_os: out std_logic  );

end acq_module0_controller_gp2015_e;

architecture acq_module0_controller_gp2015_a of acq_module0_controller_gp2015_e is

constant acq_parallel_mods_c : integer :=23 ; --24 Parallel down sampler module.

constant acq_bram_count_c    : integer :=7 ;  --8 Block  RAM Modules.

component acq_car_gen_mix_gp2015_e

Port (  Sys_CLK_is : in  STD_LOGIC;

POR_RST_is : in  STD_LOGIC;

Incoming_data_is : in  STD_LOGIC_VECTOR (5 downto 0);	car_mix_ROM_ena_is : IN STD_LOGIC;

car_mix_ROM_wea_is : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

car_mix_ROM_addra_is : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

car_mix_ROM_dina_is : IN STD_LOGIC_VECTOR(19 DOWNTO 0);

Sat_load_is : in std_logic;

IF_Freq_input_is : in STD_LOGIC_VECTOR (31 downto 0);

IP_mixout_os : out  STD_LOGIC_VECTOR (6 downto 0);

QP_mixout_os : out  STD_LOGIC_VECTOR (6 downto 0));

end component;

COMPONENT acq_code_generator_gp2015_e

PORT(

Sys_CLK_40M_is : IN std_logic;

POR_RST_is : IN std_logic;

Load_is : IN std_logic;

Acq_Svidin_is : IN std_logic_vector(5 downto 0);

g2_reg_input_is : in  STD_LOGIC_VECTOR (9 downto 0);	Codeclk_2p046M_os : OUT std_logic;

Codeout_os : OUT std_logic  );

END COMPONENT;

component acq_down_sampler_gp2015_e

Port (  Sys_CLK_40M_is : in  STD_LOGIC;

POR_RST_is : in std_logic;

Load_is : in  STD_LOGIC;			IP_mixout_is: in std_logic_vector(6 downto 0);

QP_mixout_is: in std_logic_vector(6 downto 0);

Completed_accumulation_bi_os : out std_logic;

codeout_is: in std_logic;

sig_accum_inphase_os: out  std_logic_vector(27 downto 0);

write_addr_os : out std_logic_vector(10 downto 0);

edac_wr_en_os:out std_logic  );

end component;

COMPONENT acq_memory_e

PORT( Sys_CLK_40M_is : in  STD_LOGIC;

ram_wr_en_is:in std_logic;

write_addr_is : in std_logic_vector(10 downto 0);

acq_bram_din_is : in std_logic_vector(104 downto 0);

enableb_is: in std_logic;

addressb_is: in std_logic_vector(10 downto 0);	sig_accum_inphase_ram_out_os: out  std_logic_vector(104 downto 0));

END COMPONENT;

COMPONENT edac_top_e

PORT(sig_accum_inphase_enc_is : IN std_logic_vector(27 downto 0);

sig_accum_qphase_enc_is : IN std_logic_vector(27 downto 0);	sig_accum_cphase_enc_is : IN std_logic_vector(27 downto 0);

sig_accum_inphase_dec_is : IN std_logic_vector(34 downto 0);

edac_wr_en_is : IN std_logic;

edac_rd_en_is : IN std_logic;

sig_accum_inphase_enc_os : OUT std_logic_vector(34 downto 0);

sig_accum_qphase_enc_os : OUT std_logic_vector(34 downto 0);

sig_accum_cphase_enc_os : OUT std_logic_vector(34 downto 0);	sig_accum_inphase_dec_os : OUT std_logic_vector(27 downto 0)  );

END COMPONENT;

COMPONENT acq_fsm1_e

PORT(

clk_40M_is : IN std_logic;

reset_is : IN std_logic;

sat_pr_is : IN std_logic;

acq_done_is : IN std_logic;

I_data_ant1_is : IN std_logic_vector(5 downto 0);

I_data_ant2_is : IN std_logic_vector(5 downto 0);

I_data_ant3_is : IN std_logic_vector(5 downto 0);

I_data_ant4_is : IN std_logic_vector(5 downto 0);

DSP_SV_reg_ant1_is : IN std_logic_vector(63 downto 0);

DSP_SV_reg_ant2_is : IN std_logic_vector(63 downto 0);

DSP_SV_reg_ant3_is : IN std_logic_vector(63 downto 0);

DSP_SV_reg_ant4_is : IN std_logic_vector(63 downto 0);

FPGA_SV_register_is : IN std_logic_vector(63 downto 0);

module_id_is       : in std_logic_vector(1 downto 0);

fsm1_enable_is : IN std_logic;

I_mux_out_os : OUT std_logic_vector(5 downto 0);

svid_code_gen_os : OUT std_logic_vector(5 downto 0);

load_svid_os : OUT std_logic;

Antenna_output_os: out std_logic_vector(1 downto 0);

navic_svid_indicator_os: out std_logic );

END COMPONENT;

COMPONENT acq_fsm2_e

PORT(

clk_40M_is : IN std_logic;

reset_is : IN std_logic;

accum_over_is : IN std_logic;

--fft_edone_is : IN std_logic;

fft_done_is    : in std_logic;

fft_max_done_is : IN std_logic;

fsm2_enable_is : IN std_logic;

load_svid_is: IN std_logic;

current_module_os : OUT std_logic_vector(5 downto 0);

read_out_sig_os : OUT std_logic_vector(acq_parallel_mods_c downto 0);

fft_start_os : OUT std_logic;

fft_unload_os : OUT std_logic;

completed_25mod_os : OUT std_logic;

edac_rd_en_os : OUT std_logic;

acq_done_os: OUT std_logic  );

END COMPONENT;

signal Acquired_signal_latched_s : std_logic;

type aq_aray_25_len_1_typ is array (0 to acq_parallel_mods_c) of std_logic;

type aq_aray_25_len_9_typ is array (0 to acq_parallel_mods_c) of std_logic_vector(8 downto 0);

signal reset_for_acq_s : std_logic_vector(acq_parallel_mods_c downto 0);

signal Completed_accumulation_s : aq_aray_25_len_1_typ;

signal start_fft_s, unload_sig_s : std_logic;

signal done_s : std_logic;

signal reset_for_codegen_s : std_logic;

signal xn_index_fft_s : std_logic_vector(10 downto 0);

signal xn_index_fft1_s : std_logic_vector(11 downto 0);

signal current_module_s: std_logic_vector(5 downto 0);

signal Find_max_s: std_logic;

signal combined_power_s,Max_power_s,previous_max_power_s :  std_logic_vector(51 downto 0);

signal combined_power1_s, combined_power2_s : std_logic_vector(51 downto 0) ;

signal Real_out_fft1_s, Imag_out_fft1_s :  std_logic_vector(25 downto 0);

signal Real_out_fft2_s, Imag_out_fft2_s :  std_logic_vector(25 downto 0);

signal Max_computed_s : std_logic;

signal Reset_counter_s : std_logic_vector(20 downto 0);

signal SVID_input_s, svid_code_gen_s : std_logic_vector(5 downto 0);

signal read_out_sig_s :  std_logic_vector(acq_parallel_mods_c downto 0);

signal read_out_sig_mux_s : std_logic_vector(acq_bram_count_c downto 0);

signal Codeout_16mods_s : std_logic_vector(acq_parallel_mods_c downto 0);

signal Codeclk_2p046M_s : std_logic;

signal Codeclk_2p046M_latched_s : std_logic;

signal fft_count_s : std_logic_vector(11 downto 0);

signal reset_input_all_acq_s : std_logic;

signal Incoming_data_mux_out_s : std_logic_vector(5 downto 0);

signal inphase_in1_s, qphase_in1_s : std_logic_vector(13 downto 0);

signal inphase_in2_s, qphase_in2_s : std_logic_vector(13 downto 0);

signal load_and_reset_the_channel_s  : std_logic;

signal codeoutput_s : std_logic;

signal IP_mixout_s, QP_mixout_s : std_logic_vector(6 downto 0);

signal Track_epoch_counter_adjusted_rising_s : std_logic_vector(13 downto 0);

signal doppler_adjustment_s: std_logic_vector(13 downto 0);

signal doppler_bin_at_max_s : std_logic_vector(10 downto 0);

signal temp_code_init_s : std_logic_vector(46 downto 0);

signal delay_required_track_epoch_s: std_logic_vector(12 downto 0);

signal Navic_svid_indicator_s : std_logic;

signal acquired_counter_s : std_logic_vector(2 downto 0);

type aq_aray_25_len_28_typ is array (0 to acq_parallel_mods_c) of std_logic_vector(27 downto 0);

type aq_aray_8_len_105_typ is array (0 to acq_bram_count_c) of std_logic_vector(104 downto 0);

signal ram_wen_s:  std_logic_vector(acq_bram_count_c downto 0) ; --8 sets

signal edac_wr_en_s:  std_logic_vector(acq_parallel_mods_c downto 0) ; --24 modules

signal edac_wr_en_temp_s, edac_rd_en_temp_s : std_logic;

signal sig_accum_inphase_enc_s: std_logic_vector(104 downto 0) ;

signal sig_accum_inphase_s: aq_aray_25_len_28_typ;

signal sig_accum_inphase_ram_out_s: aq_aray_8_len_105_typ;

signal sig_accum_inphase_temp_s,sig_accum_qphase_temp_s,sig_accum_cphase_temp_s,sig_accum_inphase_dec_temp_s: std_logic_vector(27 downto 0);

signal sig_accum_inphase_ram_out_temp_s,sig_accum_inphase_enc_temp_s,sig_accum_qphase_enc_temp_s,sig_accum_cphase_enc_temp_s : std_logic_vector(34 downto 0);

signal edac_count_s:  std_logic_vector(4 downto 0) ;

signal edac_count_en_s: std_logic;

signal acq_done_s: std_logic;

signal accum_over_s: std_logic;

signal fsm1_enable_s,fsm2_enable_s: std_logic;

signal sat_present_s: std_logic;

signal Antenna_output_s: std_logic_vector(1 downto 0);

type aq_aray_25_len_11_typ is array (0 to acq_parallel_mods_c) of std_logic_vector(10 downto 0);

type aq_aray_8_len_11_typ is array (0 to acq_bram_count_c) of std_logic_vector(10 downto 0);

signal write_addr_mux_s: aq_aray_8_len_11_typ ;

signal write_addr_s: aq_aray_25_len_11_typ ;

signal search_enable_s, Navic_sat_indi_latched_during_acq_s: std_logic;

signal IF_Freq_s : std_logic_vector(31 downto 0);

constant  coh_int_timer1_s :  std_logic_vector(20 downto 0) := "011110100000010100100"; -- default value: 999588 for 24 modules - 10.5 chips, 24.989225 msec

constant  coh_int_timer2_s :  std_logic_vector(20 downto 0) := "011110100000010010000"; -- default value: 999568 24 modules - 10.5 chips,24.988725 msec

constant  nco_factor1_s :   std_logic_vector(18 downto 0)   := "0000001101100110111";   -- default value: 6967 =  64.887 * NCO (107)

constant  freq_factor_s :  std_logic_vector(37 downto 0)    := "00000000000000001000000111000110100110"; -- default value: 21,26,246

constant  code_freq_factor_s :   std_logic_vector(37 downto 0) := "00000000000000000000000000010101100101"; -- default value: 1381

constant  code_freq_nco_factor_s :  std_logic_vector(19 downto 0) := "00100100001100011001"; -- default value: 148249

constant  code_freq_factor_navic_s :   std_logic_vector(37 downto 0) := "00000000000000000000000000011100111001"; -- default value: 1849

constant  code_freq_nco_factor_navic_s :  std_logic_vector(19 downto 0) := "00110000011101111101"; -- default value: 198525

begin

slew_done_os <= completed_25mod_s;

fsm2_enable_os <= fsm2_enable_s ;

code_freq_init_os  <= temp_code_init_s(46 downto 15);

accum_over_s<=Completed_accumulation_s(0);

acqcode_gen : acq_code_generator_gp2015_e

port map (Sys_CLK_40M_is => Sys_CLK_40M_is,

POR_RST_is => POR_RST_is,

Load_is => reset_for_codegen_s,

Acq_Svidin_is => svid_code_gen_s,

g2_reg_input_is => g2_reg_input_is,

Codeclk_2p046M_os => Codeclk_2p046M_s,

Codeout_os =>codeoutput_s  );

acq_mix: acq_car_gen_mix_gp2015_e

port map(Sys_CLK_is => Sys_CLK_40M_is,

POR_RST_is => POR_RST_is,

Incoming_data_is => Incoming_data_mux_out_s, car_mix_ROM_ena_is => car_mix_ROM_ena_is,

car_mix_ROM_wea_is => car_mix_ROM_wea_is,

car_mix_ROM_addra_is => car_mix_ROM_addra_is,

car_mix_ROM_dina_is => car_mix_ROM_dina_is,		Sat_load_is => reset_for_codegen_s,

IF_Freq_input_is => IF_Freq_s,

IP_mixout_os => IP_mixout_s,

QP_mixout_os => QP_mixout_s );

gen_corr:

for i in 0 to acq_parallel_mods_c generate

corr0: acq_down_sampler_gp2015_e

port map(Sys_CLK_40M_is => Sys_CLK_40M_is,

POR_RST_is => POR_RST_is,

Load_is => reset_for_acq_s(i),

IP_mixout_is => IP_mixout_s,

QP_mixout_is => QP_mixout_s,

Completed_accumulation_bi_os =>Completed_accumulation_s(i),

codeout_is => Codeout_16mods_s(i),

sig_accum_inphase_os => sig_accum_inphase_s(i),

write_addr_os => write_addr_s(i),

edac_wr_en_os => edac_wr_en_s(i) );

end generate gen_corr;

acq_edac_top_e: edac_top_e PORT MAP(

sig_accum_inphase_enc_is => sig_accum_inphase_temp_s ,

sig_accum_qphase_enc_is =>  sig_accum_qphase_temp_s,

sig_accum_cphase_enc_is =>  sig_accum_cphase_temp_s,

sig_accum_inphase_dec_is => sig_accum_inphase_ram_out_temp_s,

edac_wr_en_is => edac_wr_en_temp_s,

edac_rd_en_is => edac_rd_en_temp_s,

sig_accum_inphase_enc_os => sig_accum_inphase_enc_temp_s,

sig_accum_qphase_enc_os =>  sig_accum_qphase_enc_temp_s,

sig_accum_cphase_enc_os =>  sig_accum_cphase_enc_temp_s,

sig_accum_inphase_dec_os => sig_accum_inphase_dec_temp_s

);

gen_acqram:

for i in 0 to acq_bram_count_c generate

acq_bram0: acq_memory_e

port map(

Sys_CLK_40M_is => Sys_CLK_40M_is,

ram_wr_en_is => ram_wen_s(i),

write_addr_is => write_addr_mux_s(i),			 acq_bram_din_is => sig_accum_inphase_enc_s,

enableb_is =>  read_out_sig_mux_s(i),

addressb_is => xn_index_fft_s,

sig_accum_inphase_ram_out_os => sig_accum_inphase_ram_out_s(i)

);

end generate gen_acqram;

Process_p1: process(Sys_CLK_40M_is, POR_RST_is)

begin

if POR_RST_is = '1' then

write_addr_mux_s(0)  <= (others=>'0');

write_addr_mux_s(1)  <= (others=>'0');

write_addr_mux_s(2)  <= (others=>'0');

write_addr_mux_s(3)  <= (others=>'0');

write_addr_mux_s(4)  <= (others=>'0');

write_addr_mux_s(5)  <= (others=>'0');

write_addr_mux_s(6)  <= (others=>'0');

write_addr_mux_s(7)  <= (others=>'0');

read_out_sig_mux_s   <=  (others=>'0');

elsif Sys_CLK_40M_is='1'  and  Sys_CLK_40M_is'event then

write_addr_mux_s(0)  <=  write_addr_s(2)  ;

write_addr_mux_s(1)  <=  write_addr_s(5)  ;

write_addr_mux_s(2)  <=  write_addr_s(8)  ;

write_addr_mux_s(3)  <=  write_addr_s(11)  ;

write_addr_mux_s(4)  <=  write_addr_s(14)  ;

write_addr_mux_s(5)  <=  write_addr_s(17) ;

write_addr_mux_s(6)  <=  write_addr_s(20) ;

write_addr_mux_s(7)  <=  write_addr_s(23) ;

read_out_sig_mux_s(0)  <= read_out_sig_s(0)   or read_out_sig_s(1)  or read_out_sig_s(2);

read_out_sig_mux_s(1)  <= read_out_sig_s(3)   or read_out_sig_s(4)  or read_out_sig_s(5) ;

read_out_sig_mux_s(2)  <= read_out_sig_s(6)   or read_out_sig_s(7)  or read_out_sig_s(8);

read_out_sig_mux_s(3)  <= read_out_sig_s(9)   or read_out_sig_s(10)  or read_out_sig_s(11);

read_out_sig_mux_s(4)  <= read_out_sig_s(12)  or read_out_sig_s(13)  or read_out_sig_s(14);

read_out_sig_mux_s(5)  <= read_out_sig_s(15)  or read_out_sig_s(16) or read_out_sig_s(17);

read_out_sig_mux_s(6)  <= read_out_sig_s(18)  or read_out_sig_s(19) or read_out_sig_s(20);

read_out_sig_mux_s(7)  <= read_out_sig_s(21)  or read_out_sig_s(22) or read_out_sig_s(23);

end if;

end process Process_p1;

P_EDAC_count: process(Sys_CLK_40M_is, POR_RST_is)

begin

if POR_RST_is = '1' then

edac_count_en_s<='0';

edac_count_s<= (OTHERS=>'0');

elsif Sys_CLK_40M_is='1' and Sys_CLK_40M_is'event then

if(edac_wr_en_s(2)='1' or edac_wr_en_s(acq_parallel_mods_c)='1') then

edac_count_en_s<='1';

elsif (edac_count_s >="01110") then --14

edac_count_en_s<='0';  end if;

if edac_count_en_s='1' then

edac_count_s<= edac_count_s+1;      else

edac_count_s<= (OTHERS=>'0');    end if;     end if;

end process P_EDAC_count;

P_edac_data: process(Sys_CLK_40M_is, POR_RST_is)

begin

if(POR_RST_is = '1') then

sig_accum_inphase_temp_s<= (OTHERS=>'0');

sig_accum_qphase_temp_s <= (OTHERS=>'0');

sig_accum_cphase_temp_s <= (OTHERS=>'0');

ram_wen_s<=(OTHERS=>'0');

edac_wr_en_temp_s <= '0';

sig_accum_inphase_enc_s<= (OTHERS=>'0');

elsif(Sys_CLK_40M_is='1') and (Sys_CLK_40M_is'event) then

if(edac_count_s="00001") then  --Set 1

edac_wr_en_temp_s<= '1';

sig_accum_inphase_temp_s <= sig_accum_inphase_s(0);

sig_accum_qphase_temp_s  <= sig_accum_inphase_s(1);

sig_accum_cphase_temp_s  <= sig_accum_inphase_s(2);

elsif(edac_count_s="00010") then --Set 2

edac_wr_en_temp_s<= '1';

sig_accum_inphase_temp_s <= sig_accum_inphase_s(3);

sig_accum_qphase_temp_s  <= sig_accum_inphase_s(4);

sig_accum_cphase_temp_s  <= sig_accum_inphase_s(5);

sig_accum_inphase_enc_s (104 downto 70) <= sig_accum_inphase_enc_temp_s;

sig_accum_inphase_enc_s (69 downto 35) <= sig_accum_qphase_enc_temp_s;

sig_accum_inphase_enc_s (34 downto 0)  <= sig_accum_cphase_enc_temp_s;

ram_wen_s<=ram_wen_s(acq_bram_count_c-1 downto 0) & '1';

elsif(edac_count_s="00011") then --Set 3

edac_wr_en_temp_s<= '1';

sig_accum_inphase_temp_s <= sig_accum_inphase_s(6);

sig_accum_qphase_temp_s  <= sig_accum_inphase_s(7);

sig_accum_cphase_temp_s  <= sig_accum_inphase_s(8);

sig_accum_inphase_enc_s (104 downto 70) <= sig_accum_inphase_enc_temp_s;

sig_accum_inphase_enc_s (69 downto 35) <= sig_accum_qphase_enc_temp_s;

sig_accum_inphase_enc_s (34 downto 0)  <= sig_accum_cphase_enc_temp_s;

ram_wen_s<=ram_wen_s(acq_bram_count_c-1 downto 0) & '0';

elsif(edac_count_s="00100") then  --Set 4

edac_wr_en_temp_s<= '1';

sig_accum_inphase_temp_s <= sig_accum_inphase_s(9);

sig_accum_qphase_temp_s  <= sig_accum_inphase_s(10);

sig_accum_cphase_temp_s  <= sig_accum_inphase_s(11);

sig_accum_inphase_enc_s (104 downto 70) <= sig_accum_inphase_enc_temp_s;

sig_accum_inphase_enc_s (69 downto 35) <= sig_accum_qphase_enc_temp_s;

sig_accum_inphase_enc_s (34 downto 0)  <= sig_accum_cphase_enc_temp_s;

ram_wen_s<=ram_wen_s(acq_bram_count_c-1 downto 0) & '0';

elsif(edac_count_s="00101") then --Set 5

edac_wr_en_temp_s<= '1';

sig_accum_inphase_temp_s <= sig_accum_inphase_s(12);

sig_accum_qphase_temp_s  <= sig_accum_inphase_s(13);

sig_accum_cphase_temp_s  <= sig_accum_inphase_s(14);

sig_accum_inphase_enc_s (104 downto 70) <= sig_accum_inphase_enc_temp_s;

sig_accum_inphase_enc_s (69 downto 35) <= sig_accum_qphase_enc_temp_s;

sig_accum_inphase_enc_s (34 downto 0)  <= sig_accum_cphase_enc_temp_s;

ram_wen_s<=ram_wen_s(acq_bram_count_c-1 downto 0) & '0';

elsif(edac_count_s="00110") then --Set 6

edac_wr_en_temp_s<= '1';

sig_accum_inphase_temp_s <= sig_accum_inphase_s(15);

sig_accum_qphase_temp_s  <= sig_accum_inphase_s(16);

sig_accum_cphase_temp_s  <= sig_accum_inphase_s(17);

sig_accum_inphase_enc_s (104 downto 70) <= sig_accum_inphase_enc_temp_s;

sig_accum_inphase_enc_s (69 downto 35) <= sig_accum_qphase_enc_temp_s;

sig_accum_inphase_enc_s (34 downto 0)  <= sig_accum_cphase_enc_temp_s;

ram_wen_s<=ram_wen_s(acq_bram_count_c-1 downto 0) & '0';

elsif(edac_count_s="00111") then --Set 7

edac_wr_en_temp_s<= '1';

sig_accum_inphase_temp_s <= sig_accum_inphase_s(18);

sig_accum_qphase_temp_s  <= sig_accum_inphase_s(19);

sig_accum_cphase_temp_s  <= sig_accum_inphase_s(20);

sig_accum_inphase_enc_s (104 downto 70) <= sig_accum_inphase_enc_temp_s;

sig_accum_inphase_enc_s (69 downto 35) <= sig_accum_qphase_enc_temp_s;

sig_accum_inphase_enc_s (34 downto 0)  <= sig_accum_cphase_enc_temp_s;

ram_wen_s<=ram_wen_s(acq_bram_count_c-1 downto 0) & '0';

elsif(edac_count_s="01000") then --Set 8

edac_wr_en_temp_s<= '1';

sig_accum_inphase_temp_s <= sig_accum_inphase_s(21);

sig_accum_qphase_temp_s  <= sig_accum_inphase_s(22);

sig_accum_cphase_temp_s  <= sig_accum_inphase_s(23);

sig_accum_inphase_enc_s (104 downto 70) <= sig_accum_inphase_enc_temp_s;

sig_accum_inphase_enc_s (69 downto 35) <= sig_accum_qphase_enc_temp_s;

sig_accum_inphase_enc_s (34 downto 0)  <= sig_accum_cphase_enc_temp_s;

ram_wen_s<=ram_wen_s(acq_bram_count_c-1 downto 0) & '0';

elsif(edac_count_s="01001") then – set 9

edac_wr_en_temp_s<= '1';

sig_accum_inphase_enc_s (104 downto 70) <= sig_accum_inphase_enc_temp_s;

sig_accum_inphase_enc_s (69 downto 35) <= sig_accum_qphase_enc_temp_s;

sig_accum_inphase_enc_s (34 downto 0)  <= sig_accum_cphase_enc_temp_s;

ram_wen_s<=ram_wen_s(acq_bram_count_c-1 downto 0) & '0';

else

edac_wr_en_temp_s<= '0';

sig_accum_inphase_enc_s<= (OTHERS=>'0');

ram_wen_s <= (OTHERS=>'0');

end if;   end if;

end process P_edac_data;

P_downsampling: process(Sys_CLK_40M_is, POR_RST_is)

begin

if(POR_RST_is = '1') then

sig_accum_inphase_ram_out_temp_s <= (OTHERS=>'0');

inphase_in1_s<= (OTHERS=>'0');

qphase_in1_s<= (OTHERS=>'0');

inphase_in2_s<= (OTHERS=>'0');

qphase_in2_s<= (OTHERS=>'0');

xn_index_fft1_s <= (OTHERS=>'0');

elsif(Sys_CLK_40M_is='1') and (Sys_CLK_40M_is'event) then

xn_index_fft1_s <= '0' & xn_index_fft_s;

inphase_in1_s <= sig_accum_inphase_dec_temp_s(27 downto 14);	qphase_in1_s  <= sig_accum_inphase_dec_temp_s(13 downto 0);

if xn_index_fft1_s >="010000000001" then --1025

inphase_in2_s <= not sig_accum_inphase_dec_temp_s(27 downto 14)  + "00000000000001" ;

qphase_in2_s  <= not sig_accum_inphase_dec_temp_s(13 downto 0)   + "00000000000001" ;

else

inphase_in2_s <= sig_accum_inphase_dec_temp_s(27 downto 14);

qphase_in2_s  <= sig_accum_inphase_dec_temp_s(13 downto 0);

end if;

Case1: case read_out_sig_s is

when   X"000001" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(0)(104 downto 70);

when   X"000002" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(0)(69 downto 35);

when   X"000004" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(0)(34 downto 0);

when   X"000008" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(1)(104 downto 70);

when   X"000010" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(1)(69 downto 35);

when   X"000020" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(1)(34 downto 0);

when   X"000040" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(2)(104 downto 70);

when   X"000080" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(2)(69 downto 35);

when   X"000100" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(2)(34 downto 0);

when   X"000200" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(3)(104 downto 70);

when   X"000400" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(3)(69 downto 35);

when   X"000800" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(3)(34 downto 0);

when   X"001000" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(4)(104 downto 70);

when   X"002000" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(4)(69 downto 35);

when   X"004000" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(4)(34 downto 0);

when   X"008000" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(5)(104 downto 70);

when   X"010000" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(5)(69 downto 35);

when   X"020000" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(5)(34 downto 0);

when   X"040000" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(6)(104 downto 70);

when   X"080000" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(6)(69 downto 35);

when   X"100000" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(6)(34 downto 0);

when   X"200000" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(7)(104 downto 70);

when   X"400000" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(7)(69 downto 35);

when   X"800000" => sig_accum_inphase_ram_out_temp_s  <= sig_accum_inphase_ram_out_s(7)(34 downto 0);

when others =>   sig_accum_inphase_ram_out_temp_s <= (OTHERS=>'0');

end case Case1;     end if;

end process P_downsampling;

xn_re1_os <= inphase_in1_s;

xn_im1_os <= qphase_in1_s ;

xn_re2_os <= inphase_in2_s;

xn_im2_os <= qphase_in2_s ;

fft_start_os  <= start_fft_s;

fft_unload_os <= unload_sig_s;

Real_out_fft1_s <= xk_re1_is;

Imag_out_fft1_s <= xk_im1_is;

Real_out_fft2_s <= xk_re2_is;

Imag_out_fft2_s <= xk_im2_is;

xn_index_fft_s <= xn_index_is;

done_s  <= fft_done_is;

SVID_Searched_os <= svid_code_gen_s  ; -- Acquistion SVID searched by the engine.

acq_fsm1: acq_fsm1_e PORT MAP(

clk_40M_is => Sys_CLK_40M_is ,

reset_is => POR_RST_is ,

sat_pr_is => sat_present_s ,

acq_done_is => acq_done_s ,

I_data_ant1_is => Incoming_data_ant1_is,

I_data_ant2_is => Incoming_data_ant2_is,

I_data_ant3_is => Incoming_data_ant3_is,

I_data_ant4_is => Incoming_data_ant4_is,

DSP_SV_reg_ant1_is => DSP_SV_register_ant1_is,

DSP_SV_reg_ant2_is => DSP_SV_register_ant2_is,

DSP_SV_reg_ant3_is => DSP_SV_register_ant3_is,

DSP_SV_reg_ant4_is => DSP_SV_register_ant4_is,

FPGA_SV_register_is => FPGA_SV_register_is,

module_id_is       => Module_input_is,

fsm1_enable_is => fsm1_enable_s,

I_mux_out_os => Incoming_data_mux_out_s,

svid_code_gen_os => SVID_input_s ,

load_svid_os => load_and_reset_the_channel_s ,

Antenna_output_os  => Antenna_output_s,

navic_svid_indicator_os=> Navic_svid_indicator_s

);

acq_fsm2: acq_fsm2_e PORT MAP(

clk_40M_is => Sys_CLK_40M_is ,

reset_is => POR_RST_is ,

accum_over_is => accum_over_s ,

--fft_edone_is => edone_s ,

fft_done_is => done_s ,

fft_max_done_is => Max_computed_s,

fsm2_enable_is => fsm2_enable_s,

load_svid_is => load_and_reset_the_channel_s,

current_module_os => current_module_s ,

read_out_sig_os => read_out_sig_s ,

fft_start_os => start_fft_s ,

fft_unload_os => unload_sig_s,

completed_25mod_os => completed_25mod_s,

edac_rd_en_os  => edac_rd_en_temp_s,

acq_done_os   => acq_done_s  );

P_power_comp: process(Sys_CLK_40M_is, POR_RST_is, Module_input_is )

begin

if(POR_RST_is = '1') then

reset_for_codegen_s <= '1'; -- Reset for code gen module.

reset_input_all_acq_s <= '1'; -- Reset for 25 down-sampler modules are kept high.

Reset_counter_s <= (OTHERS=>'0'); --initilized to 0

reset_for_acq_s <= (OTHERS=>'1');

fft_count_s <= (OTHERS=>'0'); --initilized to 0

Max_computed_s <= '0'; --initilized to 0

Find_max_s <= '0'; --initilized to 0

combined_power_s <=(OTHERS=>'0'); --initilized to 0

combined_power1_s <=(OTHERS=>'0'); --initilized to 0

combined_power2_s <=(OTHERS=>'0'); --initilized to 0

Max_power_s <= (OTHERS=>'0'); --initilized to 0

previous_max_power_s <= (OTHERS=>'0'); --initilized to 0

Codeout_16mods_s <= (OTHERS=>'0');

Codeclk_2p046M_latched_s <= '0';

Acquired_signal_latched_s <= '0';

SVID_acquired_os <=  "000000";

svid_code_gen_s<= Module_input_is & "0000";

Track_epoch_output_os <= '0';

Track_epoch_counter_adjusted_rising_s <= "00000101111111"; --383

doppler_adjustment_s <= (others => '0'); -- Initialsed to zero

doppler_bin_at_max_s <= (others => '0');-- Initialsed to zero

-- all the signals are initialized to 0

car_freq_init_os <= (others => '0');

order1_loop_init_os <= (others => '0');

dll_order1_loop_init_os <= (others => '0');

temp_code_init_s <= (others => '0');

acquired_counter_s <= (others => '0');

delay_required_track_epoch_s <= (others => '0');

fsm1_enable_s<='0';

fsm2_enable_s<='0';

-- Initialse to default values at power on reset.

sat_present_s<='0';

search_enable_s <= '1';

Navic_sat_indi_latched_during_acq_s <= '0';

Antenna_output_os<="00";

IF_Freq_s <=  x"02B020C5"; --0.42e6 at 40M

elsif(Sys_CLK_40M_is='1') and (Sys_CLK_40M_is'event) then

if(Codeclk_2p046M_s = '1') and (Codeclk_2p046M_latched_s = '0') thenCodeout_16mods_s(acq_parallel_mods_c downto 1) <= Codeout_16mods_s(acq_parallel_mods_c-1 downto 0);	Codeout_16mods_s(0) <= codeoutput_s;

reset_for_acq_s(acq_parallel_mods_c downto 1) <=reset_for_acq_s(acq_parallel_mods_c-1 downto 0);

reset_for_acq_s(0) <= reset_input_all_acq_s;		Codeclk_2p046M_latched_s <= '1';    end if;

if(Codeclk_2p046M_s = '0') then  -- to maitain 2.046MHz as clock enable.

Codeclk_2p046M_latched_s <= '0';  end if;

if set_delay_is='1' then

Reset_counter_s <=(others=>'0');

elsif(Reset_counter_s >= coh_int_timer1_s)  then  -- 319706 for 16 modules 319530 , 12 chips less than 8184 (old = 1001110000011011010 )

Reset_counter_s <=(OTHERS=>'0'); -- reset counter made 0 at exactly 8ms-7.5 chip

else

Reset_counter_s <= Reset_counter_s + 1;   end if;

if (Reset_counter_s >= "010010110100000100000") and  (Reset_counter_s <= coh_int_timer1_s)  then  -- 616480 i.e.,15.412 msec

fsm2_enable_s <= '1';   else

fsm2_enable_s <= '0';   end if;

if set_delay_is='1'then

reset_for_codegen_s <= '1';

elsif(Reset_counter_s >= coh_int_timer2_s ) and (Reset_counter_s <= coh_int_timer1_s)  then --(Reset_counter_s < "1001110000000101010") then --319530 (12 chip less than 8ms as only one module is there

reset_for_codegen_s <= '1';   else

reset_for_codegen_s <= '0';   end if;

if(Reset_counter_s >= "011110011111010100100") and (Reset_counter_s <= coh_int_timer1_s) then -- 999076

fsm1_enable_s<='1';    else

fsm1_enable_s<='0';    end if;

if set_delay_is='1'then

reset_input_all_acq_s <= '1';

elsif(Reset_counter_s >= coh_int_timer2_s) and (Reset_counter_s <= coh_int_timer1_s) then --(Reset_counter_s < "1001110000000101010") then --319530 (12 chip less than 8ms as only one module is there

reset_input_all_acq_s <= '1';

elsif(Codeclk_2p046M_s = '1') and (Codeclk_2p046M_latched_s = '0') then

reset_input_all_acq_s <= '0';   end if;

if(Navic_sat_indi_latched_during_acq_s= '1') then  --NAVIC

--Track_epoch_counter_adjusted_rising_s <= "00000110111100"  - doppler_adjustment_s + delay_required_track_epoch_s(12 downto 1) ;

Track_epoch_counter_adjusted_rising_s <= "00000101111111"  - doppler_adjustment_s + delay_required_track_epoch_s(12 downto 1) ;

else

Track_epoch_counter_adjusted_rising_s <= "00000101111111"  - doppler_adjustment_s + delay_required_track_epoch_s(12 downto 1) ;	end if;

if (Reset_counter_s =  "0000000" & Track_epoch_counter_adjusted_rising_s ) and (Acquired_signal_latched_s = '1')  then --300

Track_epoch_output_os <= '1';

elsif (SVID_loading_is = '1')   then  -- falling edge of svid loading is used to make the track epoch to zero

Track_epoch_output_os <= '0';   end if;

if done_s ='1' then

Find_max_s <= '1';

elsif  fft_count_s  = "100000001000" then

Find_max_s <= '0';    end if;

if Find_max_s = '1'  then

fft_count_s <= fft_count_s + 1;

if fft_count_s  = "100000001000" then --2054

fft_count_s <= (OTHERS=>'0');

Max_computed_s <= '1';   end if;

combined_power1_s  <=  Real_out_fft1_s * Real_out_fft1_s + Imag_out_fft1_s * Imag_out_fft1_s; -- To be updated

combined_power2_s  <=  Real_out_fft2_s * Real_out_fft2_s + Imag_out_fft2_s * Imag_out_fft2_s; -- To be updated

if combined_power1_s >= combined_power2_s then

combined_power_s <= combined_power1_s;  else

combined_power_s <= combined_power2_s;   end if;

if(combined_power_s >= Max_power_s) then

Max_power_s <= combined_power_s;

doppler_bin_at_max_s  <= fft_count_s(10 downto 0) + "11111111000"; --2042

end if;

else

Max_computed_s <= '0';

Max_power_s <=(OTHERS=>'0');

combined_power1_s<=(OTHERS=>'0');

combined_power2_s<=(OTHERS=>'0');

combined_power_s<=(OTHERS=>'0');  end if;

if FPGA_SV_register_is(conv_integer('0' & svid_code_gen_s(5 downto 0))) = '0' then

if DSP_SV_register_ant1_is(conv_integer('0' & svid_code_gen_s(5 downto 0)))='1'  then

search_enable_s <= '1';

elsif DSP_SV_register_ant2_is(conv_integer('0' & svid_code_gen_s(5 downto 0)))='1'  then

search_enable_s <= '1';

elsif DSP_SV_register_ant3_is(conv_integer('0' & svid_code_gen_s(5 downto 0)))='1'  then

search_enable_s <= '1';

elsif DSP_SV_register_ant4_is(conv_integer('0' & svid_code_gen_s(5 downto 0)))='1'  then

search_enable_s <= '1';  else

search_enable_s <= '0'; end if;

else

search_enable_s <= '0'; end if;

if Max_computed_s='1' and search_enable_s='1'  then

if (Max_power_s >=  Acquisition_threshold_is) and (Max_power_s >= previous_max_power_s) then

previous_max_power_s <= Max_power_s;

Antenna_output_os <= Antenna_output_s;

Acquired_signal_latched_s	<= '1';

delay_required_track_epoch_s <= current_module_s * "0100111";

if Navic_svid_indicator_s= '1'  then  --NAVIC

temp_code_init_s <=  "00000110100011000001010101001101000000000000000" + code_freq_nco_factor_navic_s * (doppler_bin_at_max_s) ;--temp_loading_var_s(29 downto 15);

dll_order1_loop_init_os <= doppler_bin_at_max_s * code_freq_factor_navic_s;

doppler_adjustment_s <= sxt(doppler_bin_at_max_s(10 downto 4),14);  -- navic factor divide by 18 (so 4 bits discarded)

car_freq_init_os <=  IF_Freq_NAVIC_is + doppler_bin_at_max_s * nco_factor1_s; --NCO_factor1_c;

else

temp_code_init_s <=  "00000110100011000001010101001101000000000000000" + code_freq_nco_factor_s * (doppler_bin_at_max_s) ;--temp_loading_var_s(29 downto 15);

dll_order1_loop_init_os <= doppler_bin_at_max_s * code_freq_factor_s;

doppler_adjustment_s <= sxt(doppler_bin_at_max_s(10 downto 5),14);  -- GPS factor divide by 24 (so 5 bits discarded)

car_freq_init_os <=  IF_Freq_GPS_is + doppler_bin_at_max_s * nco_factor1_s; --NCO_factor1_c;

end if;

order1_loop_init_os <= doppler_bin_at_max_s * freq_factor_s;--Freq_factor_c ;

Navic_sat_indi_latched_during_acq_s <= Navic_svid_indicator_s;

SVID_acquired_os <= svid_code_gen_s ;

end if;

elsif  SVID_loading_is = '1' then -- falling edge of svid loading is used for making acquisition latch zero.

Acquired_signal_latched_s <= '0';

elsif Acquired_signal_latched_s = '1' and  acquired_counter_s = "010" then -- even if SVID loading does not come also it will make it zero in next cycle Acquired_signal_latched_s <= '0'; end if;

if Reset_counter_s = "011110100000010000001" and Acquired_signal_latched_s = '1' then  --16 cycles prior to reset. this incremented.

acquired_counter_s <= acquired_counter_s + 1;

elsif Acquired_signal_latched_s = '0' then

acquired_counter_s <= (others => '0'); end if;

if Acquired_signal_latched_s='1' and completed_25mod_s='1' then

sat_present_s<='1';  else

sat_present_s<='0'; end if;

if load_and_reset_the_channel_s = '1' then

svid_code_gen_s <= SVID_input_s ;	-- Load SVID to Code generator.

Max_power_s     <= (others=>'0');

combined_power1_s<=(OTHERS=>'0');

combined_power2_s<=(OTHERS=>'0');

previous_max_power_s <= (others=>'0');

if(Navic_svid_indicator_s = '1') then

IF_Freq_s <= IF_Freq_NAVIC_is;   else

IF_Freq_s <= IF_Freq_GPS_is;	end if;

end if;   end if;

end process P_power_comp;

end acq_module0_controller_gp2015_a;

9) addr_gen_2048_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity addr_gen_2048_e  is

generic(

fft_size_log_g     : integer );

port(

clk_is: in std_logic;

reset_is : in std_logic;

start_is : in std_logic;  --Start signal from Acquisition FSM2.

unload_is : in std_logic; --Unload signal from Acquisition FSM2.

stage_count_is: in std_logic_vector(3 downto 0);

group_count_is: in std_logic_vector(fft_size_log_g-2 downto 0);

xn_index_os       : out std_logic_vector(fft_size_log_g-1 downto 0); -- xn index to be connected to Block RAM module.

xk_index_os       : out std_logic_vector(fft_size_log_g-1 downto 0); -- xk index for FFT output.

xk_index1_os      : out std_logic_vector(fft_size_log_g-1 downto 0);

xn_index1_os      : out std_logic_vector(fft_size_log_g-1 downto 0);

xk_count_en_os     : out std_logic ;

fft_ram1_raddr_os : out std_logic_vector(fft_size_log_g-1 downto 0); --FFT RAM1 Address.

fft_ram2_raddr_os : out std_logic_vector(fft_size_log_g-1 downto 0); --FFT RAM2 Address.

fft_rom_addr_os  : out std_logic_vector(fft_size_log_g-2 downto 0)   -- FFT ROM address  );

end addr_gen_2048_e;

architecture addr_gen_2048_a of addr_gen_2048_e is

signal twid_rom_addr_s,rom_addr_s : std_logic_vector(fft_size_log_g-2 downto 0);

signal shift_reg1_s, shift_reg2_s  : std_logic_vector(fft_size_log_g-1 downto 0);

signal ram1_addr_s , ram2_addr_s  : std_logic_vector(fft_size_log_g-1 downto 0);

signal xn_index_s,xn_index1_s, xk_index_s,xk_index1_s: std_logic_vector(fft_size_log_g-1  downto 0);

signal start_reg_s, unload_reg_s: std_logic_vector(3 downto 0);

signal xn_count_en_s, xn_count_en1_s,xk_count_en_s,xk_count_en1_s : std_logic;

begin

xk_count_en_os <= xk_count_en_s;

P_index_gen:	process(clk_is, reset_is)

begin

if reset_is='1' then

xn_count_en_s<='0';

xn_count_en1_s <='0';

xk_count_en_s <='0';

xk_count_en1_s <='0';

unload_reg_s <= (others=>'0');

start_reg_s  <= (others=>'0');

elsif  clk_is'event and  clk_is='1' then

start_reg_s(0)<= start_is;

start_reg_s(3 downto 1) <= start_reg_s(2 downto 0);

unload_reg_s(0) <= unload_is;

unload_reg_s(3 downto 1) <= unload_reg_s(2 downto 0);

if start_reg_s(2)='1' and start_is='0' then

xn_count_en_s<='1';

elsif xn_index_s >="11111111110" then

xn_count_en_s<='0'; end if;

if start_reg_s(3) ='1' and start_is='0' then

xn_count_en1_s <='1';

elsif xn_index1_s >="11111111110" then

xn_count_en1_s <='0';  end if;

if unload_reg_s(1)='1' and unload_is='0' then

xk_count_en1_s<='1';

elsif xk_index1_s >="11111111110" then

xk_count_en1_s<='0';  end if;

if unload_reg_s(3)='1' and unload_reg_s(1)='0' then -- unload pulse two cycles high.

xk_count_en_s<='1';

elsif xk_index_s >="11111111110" then

xk_count_en_s<='0'; end if;  end if;

end process P_index_gen;

P_xn_gen:    process(clk_is, reset_is)

begin

if reset_is='1' then

xn_index_s <= (others=>'0');

xn_index1_s <= (others=>'0');

elsif  clk_is'event and  clk_is='1' then

if xn_count_en_s='0' then

xn_index_s <= (others=>'0');   else

xn_index_s <= xn_index_s+1;  end if;

if xn_count_en1_s='0' then

xn_index1_s <= (others=>'0');   else

xn_index1_s <= xn_index1_s+1;  end if;  end if;

end process P_xn_gen;

P_xk_gen:	process(clk_is, reset_is)

begin

if reset_is='1' then

xk_index_s <= (others=>'0');

xk_index1_s <= (others=>'0');

elsif  clk_is'event and  clk_is='1' then

if xk_count_en_s='0' then

xk_index_s <= (others=>'0');   else

xk_index_s <= xk_index_s+1;  end if;

if xk_count_en1_s='0' then

xk_index1_s <= (others=>'0');   else

xk_index1_s <= xk_index1_s+1;   end if;  end if;

end process P_xk_gen;

xk_index_os  <= xk_index_s;

xk_index1_os <= xk_index1_s;

xn_index_os  <= xn_index_s;

xn_index1_os(i) <= xn_index1_s(fft_size_log_g-1-i);

end generate index_gen ;

P_butt_addr_gen:	 process(clk_is, reset_is)

begin

if reset_is='1' then

shift_reg1_s <= (others=>'0');

shift_reg2_s <= (others=>'0');

elsif  clk_is'event and  clk_is='1' then

shift_reg1_s <= group_count_is(fft_size_log_g-2 downto 0) & '0';

shift_reg2_s <= group_count_is(fft_size_log_g-2 downto 0) & '0' + "00000000001" ;   end if;

end process P_butt_addr_gen;

P_ram_addr_gen:	process(clk_is, reset_is)

begin

if reset_is='1' then

ram1_addr_s  <= (others=>'0');

ram2_addr_s  <= (others=>'0');

elsif  clk_is'event and  clk_is='1' then

C_stage_count_dec: case stage_count_is is

when "0000" =>    ram1_addr_s (10 downto 0) <=  shift_reg1_s(10 downto 0);

ram2_addr_s (10 downto 0) <=  shift_reg2_s(10 downto 0);

when "0001" =>    ram1_addr_s (10 downto 0) <=  shift_reg1_s(9 downto 0) & shift_reg1_s(10);

ram2_addr_s (10 downto 0) <=  shift_reg2_s(9 downto 0) & shift_reg2_s(10);

when "0010" =>    ram1_addr_s (10 downto 0) <=  shift_reg1_s(8 downto 0) & shift_reg1_s(10 downto 9);

ram2_addr_s (10 downto 0) <=  shift_reg2_s(8 downto 0) & shift_reg2_s(10 downto 9);

when "0011"=>     ram1_addr_s (10 downto 0) <=  shift_reg1_s(7 downto 0) & shift_reg1_s(10 downto 8);				ram2_addr_s (10 downto 0) <=  shift_reg2_s(7 downto 0) & shift_reg2_s(10 downto 8);

when "0100" =>    ram1_addr_s (10 downto 0) <=  shift_reg1_s(6 downto 0) & shift_reg1_s(10 downto 7);

ram2_addr_s (10 downto 0) <=  shift_reg2_s(6 downto 0) & shift_reg2_s(10 downto 7);

when "0101" =>    ram1_addr_s (10 downto 0) <=  shift_reg1_s(5 downto 0) & shift_reg1_s(10 downto 6);

ram2_addr_s (10 downto 0) <=  shift_reg2_s(5 downto 0) & shift_reg2_s(10 downto 6);

when "0110" =>    ram1_addr_s (10 downto 0) <=  shift_reg1_s(4 downto 0) & shift_reg1_s(10 downto 5);

ram2_addr_s (10 downto 0) <=  shift_reg2_s(4 downto 0) & shift_reg2_s(10 downto 5);

when "0111" =>    ram1_addr_s (10 downto 0) <=  shift_reg1_s(3 downto 0) & shift_reg1_s(10 downto 4);					                    ram2_addr_s (10 downto 0) <=  shift_reg2_s(3 downto 0) & shift_reg2_s(10 downto 4);

when "1000"=>     ram1_addr_s (10 downto 0) <=  shift_reg1_s(2 downto 0) & shift_reg1_s(10 downto 3);					                    ram2_addr_s (10 downto 0) <=  shift_reg2_s(2 downto 0) & shift_reg2_s(10 downto 3);

when "1001"=>     ram1_addr_s (10 downto 0) <=  shift_reg1_s(1 downto 0) & shift_reg1_s(10 downto 2);					                    ram2_addr_s (10 downto 0) <=  shift_reg2_s(1 downto 0) & shift_reg2_s(10 downto 2);

when "1010"=>     ram1_addr_s (10 downto 0) <=  shift_reg1_s(0)          & shift_reg1_s(10 downto 1);					                    ram2_addr_s (10 downto 0) <=  shift_reg2_s(0)          & shift_reg2_s(10 downto 1);

when  others=>    ram1_addr_s (10 downto 0)<=   shift_reg1_s(10 downto 0);					                    ram2_addr_s (10 downto 0)<=   shift_reg2_s(10 downto 0);

end case C_stage_count_dec;   end if;

end process P_ram_addr_gen;

fft_ram1_raddr_os  <= ram1_addr_s;

fft_ram2_raddr_os  <= ram2_addr_s;

rom_addr_s <= group_count_is; --

P_twid_rom_index_gen: process(clk_is, reset_is)

begin

if reset_is='1' then

twid_rom_addr_s<= (others=>'0');

elsif  clk_is'event and  clk_is='1' then

C_stage_dec: case stage_count_is is

when "0000" =>    twid_rom_addr_s (9 downto 0) <=  (others=>'0');

when "0001" =>    twid_rom_addr_s (9 downto 0) <=  rom_addr_s(9) & "000000000";

when "0010" =>    twid_rom_addr_s (9 downto 0) <=  rom_addr_s(9 downto 8) & "00000000" ;

when "0011"=>     twid_rom_addr_s (9 downto 0) <=  rom_addr_s(9 downto 7) & "0000000"  ;

when "0100" =>    twid_rom_addr_s (9 downto 0) <=  rom_addr_s(9 downto 6) & "000000"   ;

when "0101" =>    twid_rom_addr_s (9 downto 0) <=  rom_addr_s(9 downto 5) & "00000"    ;

when "0110" =>    twid_rom_addr_s (9 downto 0) <=  rom_addr_s(9 downto 4) & "0000"     ;

when "0111" =>    twid_rom_addr_s (9 downto 0) <=  rom_addr_s(9 downto 3) & "000"      ;

when "1000"=>     twid_rom_addr_s (9 downto 0) <=  rom_addr_s(9 downto 2) & "00" ;

when "1001"=>     twid_rom_addr_s (9 downto 0) <=  rom_addr_s(9 downto 1) & '0' ;

when "1010"=>     twid_rom_addr_s (9 downto 0) <=  rom_addr_s(9 downto 0);

when  others=>    twid_rom_addr_s (9 downto 0) <=  rom_addr_s(9 downto 0);

end case C_stage_dec;   end if;

end process P_twid_rom_index_gen;

fft_rom_addr_os  <= twid_rom_addr_s;

end addr_gen_2048_a;

10) address_decoder_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity address_decoder_e is

port( clk_40M_is : in std_logic;

reset_is : in std_logic;

up_address_is: in std_logic_vector(27 downto 2);

iosn_is : in std_logic;

data_from_correlator_is: in std_logic_vector(31 downto 0);

rt_address_is: in std_logic_vector(1 downto 0);

pr_read_data_from_rt1_ip_is: in std_logic_vector(15 downto 0);

pr_read_data_from_descr_ram1_is: in std_logic_vector(15 downto 0);

pr_read_data_from_comm_ram1_is:  in std_logic_vector(15 downto 0);

pr_read_data_from_data_ram1_is:  in std_logic_vector(15 downto 0);

pr_read_data_from_rt2_ip_is: in std_logic_vector(15 downto 0);

pr_read_data_from_descr_ram2_is: in std_logic_vector(15 downto 0);

pr_read_data_from_comm_ram2_is:  in std_logic_vector(15 downto 0);

pr_read_data_from_data_ram2_is:  in std_logic_vector(15 downto 0);

pr_read_data_from_rt3_ip_is: in std_logic_vector(15 downto 0);

pr_read_data_from_descr_ram3_is: in std_logic_vector(15 downto 0);

pr_read_data_from_comm_ram3_is:  in std_logic_vector(15 downto 0);

pr_read_data_from_data_ram3_is:  in std_logic_vector(15 downto 0);

correlator_cs_os: out std_logic;

mil_rt1_ip_csn_os,mil_rt2_ip_csn_os,mil_rt3_ip_csn_os : out std_logic;

mil_data_ram1_cs_os,mil_data_ram2_cs_os,mil_data_ram3_cs_os: out std_logic;

mil_command_ram1_cs_os,mil_command_ram2_cs_os,mil_command_ram3_cs_os: out std_logic;

mil_descriptor_ram1_cs_os,mil_descriptor_ram2_cs_os,mil_descriptor_ram3_cs_os : out std_logic;

fpga_data_out_to_up_os: out  std_logic_vector(31 downto 0);

rt_address_RT1_os: out std_logic_vector(5 downto 0);

rt_address_RT2_os: out std_logic_vector(5 downto 0);

rt_address_RT3_os: out std_logic_vector(5 downto 0);

prom_cs_is: in std_logic;

prom_data_to_up_is: in std_logic_vector(31 downto 0));

end address_decoder_e;

architecture address_decoder_a of address_decoder_e is

signal mil_command_ram1_cs_s,mil_command_ram2_cs_s,mil_command_ram3_cs_s: std_logic;

signal mil_data_ram1_cs_s,mil_data_ram2_cs_s,mil_data_ram3_cs_s: std_logic;

signal mil_descriptor_ram1_cs_s, mil_descriptor_ram2_cs_s,mil_descriptor_ram3_cs_s: std_logic;

signal mil_rt1_ip_csn_s,mil_rt2_ip_csn_s,mil_rt3_ip_csn_s: std_logic;

signal correlator_cs_s: std_logic;

signal mil_command_ram1_cs_reg_s,mil_command_ram2_cs_reg_s,mil_command_ram3_cs_reg_s: std_logic;

signal mil_data_ram1_cs_reg_s,mil_data_ram2_cs_reg_s,mil_data_ram3_cs_reg_s: std_logic;

signal mil_descriptor_ram1_cs_reg_s, mil_descriptor_ram2_cs_reg_s,mil_descriptor_ram3_cs_reg_s: std_logic;

signal mil_rt1_ip_csn_reg_s,mil_rt2_ip_csn_reg_s,mil_rt3_ip_csn_reg_s: std_logic;

signal correlator_cs_reg_s: std_logic;

begin

correlator_cs_s <= '1' when iosn_is = '0' and up_address_is(27 downto 14) = X"00"& "000000" else

'0';

mil_rt1_ip_csn_s <= '0' when iosn_is = '0' and up_address_is(27 downto 14) = X"00"&  "110000" else

'1';

mil_rt2_ip_csn_s <= '0' when iosn_is = '0' and up_address_is(27 downto 14) = X"00"& "110001" else

'1';

mil_rt3_ip_csn_s <= '0' when iosn_is = '0' and up_address_is(27 downto 14) = X"00"& "110010" else

'1';

mil_descriptor_ram1_cs_s <= '1' when iosn_is = '0' and up_address_is(27 downto 14) = X"00"&  "010000" and up_address_is(13 downto 12) = "00" else

'0';

mil_descriptor_ram2_cs_s <= '1' when iosn_is = '0' and up_address_is(27 downto 14) = X"00"& "010000" and up_address_is(13 downto 12) = "01" else

'0';

mil_descriptor_ram3_cs_s <= '1' when iosn_is = '0' and up_address_is(27 downto 14) = X"00"& "010000" and up_address_is(13 downto 12) = "10"  else

'0';					 mil_data_ram1_cs_s <= '1' when iosn_is = '0' and up_address_is(27 downto 14) = X"00"& "010001" and up_address_is(13 downto 12) = "00" else

'0';

mil_data_ram2_cs_s <= '1' when iosn_is = '0' and up_address_is(27 downto 14) = X"00"&  "010001" and up_address_is(13 downto 12) = "01" else

'0';

mil_data_ram3_cs_s <= '1' when iosn_is = '0' and up_address_is(27 downto 14) = X"00"& "010001" and up_address_is(13 downto 12) = "10" else

'0';			mil_command_ram1_cs_s <= '1' when iosn_is = '0' and up_address_is(27 downto 14) = X"00"&  "010010" and up_address_is(13 downto 12) = "00" else

'0';

mil_command_ram2_cs_s <= '1' when iosn_is = '0' and up_address_is(27 downto 14) = X"00"&  "010010" and up_address_is(13 downto 12) = "01" else

'0';

mil_command_ram3_cs_s <= '1' when iosn_is = '0' and up_address_is(27 downto 14) = X"00"&  "010010" and up_address_is(13 downto 12) = "10" else

'0';			fpga_data_out_to_up_os<=x"0000"&pr_read_data_from_rt1_ip_is when mil_rt1_ip_csn_reg_s = '0' else

x"0000" & pr_read_data_from_descr_ram1_is when mil_descriptor_ram1_cs_reg_s = '1' else

x"0000" & pr_read_data_from_comm_ram1_is when mil_command_ram1_cs_reg_s = '1' else

x"0000" & pr_read_data_from_data_ram1_is when mil_data_ram1_cs_reg_s = '1' else

x"0000" & pr_read_data_from_rt2_ip_is when mil_rt2_ip_csn_reg_s = '0' else

x"0000" & pr_read_data_from_descr_ram2_is when mil_descriptor_ram2_cs_reg_s = '1' else

x"0000" & pr_read_data_from_comm_ram2_is when mil_command_ram2_cs_reg_s = '1' else

x"0000" & pr_read_data_from_data_ram2_is when mil_data_ram2_cs_reg_s = '1' else

x"0000" & pr_read_data_from_rt3_ip_is when mil_rt3_ip_csn_reg_s = '0' else

x"0000" & pr_read_data_from_descr_ram3_is when mil_descriptor_ram3_cs_reg_s = '1' else

x"0000" & pr_read_data_from_comm_ram3_is when mil_command_ram3_cs_reg_s = '1' else

x"0000" & pr_read_data_from_data_ram3_is when mil_data_ram3_cs_reg_s = '1' else

prom_data_to_up_is when prom_cs_is = '0' else

data_from_correlator_is ;

P_cs_registers: process(clk_40M_is,reset_is )

begin

if reset_is = '1' then

mil_command_ram1_cs_reg_s<='0';

mil_command_ram2_cs_reg_s<='0';

mil_command_ram3_cs_reg_s<='0';

mil_descriptor_ram1_cs_reg_s<='0';

mil_descriptor_ram2_cs_reg_s<='0';

mil_descriptor_ram3_cs_reg_s<='0';

mil_data_ram1_cs_reg_s <='0';

mil_data_ram2_cs_reg_s <='0';

mil_data_ram3_cs_reg_s <='0';

mil_rt1_ip_csn_reg_s <='0';

mil_rt2_ip_csn_reg_s <='0';

mil_rt3_ip_csn_reg_s <='0';

correlator_cs_reg_s  <='0';

elsif clk_40M_is='1' and clk_40M_is'event then

mil_command_ram1_cs_reg_s<= mil_command_ram1_cs_s;

mil_command_ram2_cs_reg_s<= mil_command_ram2_cs_s;

mil_command_ram3_cs_reg_s<= mil_command_ram3_cs_s;

mil_descriptor_ram1_cs_reg_s<= mil_descriptor_ram1_cs_s;

mil_descriptor_ram2_cs_reg_s<= mil_descriptor_ram2_cs_s;

mil_descriptor_ram3_cs_reg_s<= mil_descriptor_ram3_cs_s;

mil_data_ram1_cs_reg_s <= mil_data_ram1_cs_s;

mil_data_ram2_cs_reg_s <= mil_data_ram2_cs_s;

mil_data_ram3_cs_reg_s <= mil_data_ram3_cs_s;

mil_rt1_ip_csn_reg_s <= mil_rt1_ip_csn_s;

mil_rt2_ip_csn_reg_s <= mil_rt2_ip_csn_s;

mil_rt3_ip_csn_reg_s <= mil_rt3_ip_csn_s;

correlator_cs_reg_s  <= correlator_cs_s;  end if;

end process P_cs_registers;

correlator_cs_os<=correlator_cs_reg_s;

mil_rt1_ip_csn_os <= mil_rt1_ip_csn_reg_s;

mil_rt2_ip_csn_os <= mil_rt2_ip_csn_reg_s;

mil_rt3_ip_csn_os <= mil_rt3_ip_csn_reg_s;

mil_data_ram1_cs_os <=  mil_data_ram1_cs_reg_s;

mil_data_ram2_cs_os <=  mil_data_ram2_cs_reg_s;

mil_data_ram3_cs_os <=  mil_data_ram3_cs_reg_s;

mil_command_ram1_cs_os <= mil_command_ram1_cs_reg_s;

mil_command_ram2_cs_os <= mil_command_ram2_cs_reg_s;

mil_command_ram3_cs_os <= mil_command_ram3_cs_reg_s;

mil_descriptor_ram1_cs_os <= mil_descriptor_ram1_cs_reg_s;

mil_descriptor_ram2_cs_os <= mil_descriptor_ram2_cs_reg_s;

mil_descriptor_ram3_cs_os <= mil_descriptor_ram3_cs_reg_s;

rt_address_RT1_os<= "111011" when  rt_address_is="00" else         -- RT Address 1B(hex), parity bit- '1',MSB

"110111" when  rt_address_is="01" else         -- RT Address 17(hex), parity bit- '1',MSB

"011010" when  rt_address_is="10" else         -- RT Address 1A(hex), parity bit- '0',MSB

"011100" ;                                     -- RT Address 1C(hex), parity bit- '0',MSB

rt_address_RT2_os<= "111011" when  rt_address_is="00" else         -- RT Address 1B(hex), parity bit- '1',MSB

"110111" when  rt_address_is="01" else         -- RT Address 17(hex), parity bit- '1',MSB

"011010" when  rt_address_is="10" else         -- RT Address 1A(hex), parity bit- '0',MSB

"011100" ;                                     -- RT Address 1C(hex), parity bit- '0',MSB

rt_address_RT3_os<= "111011" when  rt_address_is="00" else         -- RT Address 1B(hex), parity bit- '1',MSB

"110111" when  rt_address_is="01" else         -- RT Address 17(hex), parity bit- '1',MSB

"011010" when  rt_address_is="10" else         -- RT Address 1A(hex), parity bit- '0',MSB

"011100" ;                                     -- RT Address 1C(hex), parity bit- '0',MSB

end address_decoder_a;

11) bch_16_dec1_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity bch_16_dec1_e is

Port (

rd_en_is       : in STD_LOGIC;  -- EDAC Read enable

data_in_is     : in  STD_LOGIC_VECTOR (15 downto 0);   -- Input Information bits to EDAC

par_in_is      : in  STD_LOGIC_VECTOR (9 downto 0);   -- Input Parity bits to EDAC

data_out_os    : out  STD_LOGIC_VECTOR (15 downto 0)  -- Output information bits after correction );

end bch_16_dec1_e;

architecture bch_16_dec1_a of bch_16_dec1_e is

signal synd_s: STD_LOGIC_VECTOR(9 downto 0); -- Syndrome calculation.

signal error_s : STD_LOGIC_VECTOR(15 downto 0); -- Error locator.

begin

Process_p1: process(rd_en_is,data_in_is,par_in_is,synd_s,error_s)

begin

if(rd_en_is='1') then   -- when  read is enabled for EDAC module,

.		synd_s(9) <= data_in_is(15) xor  data_in_is(14)  xor   data_in_is(12) xor  data_in_is(10) xor  data_in_is(8)  xor data_in_is(7)  xor data_in_is(6)  xor data_in_is(5) xor data_in_is(2) xor par_in_is(9)  ;

synd_s(8) <= data_in_is(14) xor  data_in_is(13)  xor   data_in_is(11) xor  data_in_is(9)  xor  data_in_is(7)  xor data_in_is(6)  xor data_in_is(5)  xor data_in_is(4) xor data_in_is(1) xor par_in_is(8) ;

synd_s(7) <= data_in_is(13) xor  data_in_is(12)  xor   data_in_is(10) xor  data_in_is(8)  xor  data_in_is(6)  xor data_in_is(5)  xor data_in_is(4)  xor data_in_is(3) xor data_in_is(0) xor par_in_is(7) ;

synd_s(6) <= data_in_is(15) xor  data_in_is(14)  xor   data_in_is(11) xor  data_in_is(10) xor  data_in_is(9)  xor data_in_is(8)  xor data_in_is(6)  xor data_in_is(4) xor data_in_is(3) xor par_in_is(6) ;

synd_s(5) <= data_in_is(14) xor  data_in_is(13)  xor   data_in_is(10) xor  data_in_is(9)  xor  data_in_is(8)  xor data_in_is(7)  xor data_in_is(5)  xor data_in_is(3) xor data_in_is(2) xor par_in_is(5) ;

synd_s(4) <= data_in_is(15) xor  data_in_is(14)  xor   data_in_is(13) xor  data_in_is(10) xor  data_in_is(9)  xor data_in_is(5)  xor data_in_is(4)  xor data_in_is(1) xor par_in_is(4) ;

synd_s(3) <= data_in_is(15) xor  data_in_is(13)  xor   data_in_is(10) xor  data_in_is(9)  xor  data_in_is(7)  xor data_in_is(6)  xor data_in_is(5)  xor data_in_is(4) xor data_in_is(3) xor data_in_is(2) xor data_in_is(0) xor par_in_is(3) ;

synd_s(2) <= data_in_is(14) xor  data_in_is(12)  xor   data_in_is(9)  xor  data_in_is(8)  xor  data_in_is(6)  xor data_in_is(5)  xor data_in_is(4)  xor data_in_is(3) xor data_in_is(2) xor data_in_is(1) xor par_in_is(2);

synd_s(1) <= data_in_is(15) xor  data_in_is(14)  xor   data_in_is(13) xor  data_in_is(12) xor  data_in_is(11) xor data_in_is(10) xor data_in_is(6)  xor data_in_is(4) xor data_in_is(3) xor data_in_is(1) xor data_in_is(0) xor par_in_is(1) ;

synd_s(0) <= data_in_is(15) xor  data_in_is(13)  xor   data_in_is(11) xor  data_in_is(9)  xor  data_in_is(8)  xor data_in_is(7)  xor data_in_is(6)  xor data_in_is(3) xor data_in_is(0) xor par_in_is(0);

error_s(15) <=

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9))) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ));

error_s(14) <=

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9))) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) )) ;

error_s(13) <=

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ));

error_s(12) <=

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ));

error_s(11) <=

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) )) ;

error_s(10) <=

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ));

error_s(9)  <=

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ));

error_s(8)  <=

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ));

error_s(7)  <=

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ));

error_s(6)  <=

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) )) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ));

error_s(5)  <=

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ));

error_s(4)  <=

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ));

error_s(3)  <=

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) )) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) )) ;

error_s(2)  <=

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ));

error_s(1)  <=

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ));

error_s(0)  <=

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ) ) OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) )) OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( synd_s(9) ))   OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))  OR

(( NOT synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))  OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) )) OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) )) OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( NOT synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( NOT synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( NOT synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( NOT synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) ))OR

(( synd_s(0) )AND( synd_s(1) )AND( NOT synd_s(2) )AND( synd_s(3) )AND( NOT synd_s(4) )AND( NOT synd_s(5) )AND( NOT synd_s(6) )AND( synd_s(7) )AND( NOT synd_s(8) )AND( NOT synd_s(9) )) ;

data_out_os <= data_in_is xor error_s;    else

synd_s      <= (OTHERS=>'0');

error_s     <= (OTHERS=>'0');

data_out_os <= data_in_is; end if;

end process Process_p1;

end bch_16_dec1_a;

12) bit_sync_ch_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_SIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity bit_sync_ch_e is

PORT(

clk_40MHz_is : IN std_logic;

POR_is : IN std_logic;

start_is : IN std_logic;

one_ms_pulse_is: IN std_logic;

I_data_is, Q_data_is : IN std_logic_vector(22 downto 0);

lock_status_is : IN std_logic;

--reset_channel_is : IN std_logic;

navdata_read_mp_is : IN std_logic;

navdata_os : OUT std_logic;

newdata_indicator_os : OUT std_logic;

bit_sync_indicator_os : OUT std_logic;

bit_sync_found_pulse_os: OUT std_logic;

p_cndr_os : OUT std_logic_vector(27 downto 0);

q_cndr_os : OUT std_logic_vector(27 downto 0));

end bit_sync_ch_e;

architecture bit_sync_ch_a of bit_sync_ch_e is

signal bit_sync_indicator_s: std_logic;

type state_type is (s1,s2,s3,s4);

signal bit_sync_state_s : state_type;

attribute FSM_ENCODING : string;

attribute FSM_ENCODING of bit_sync_state_s: signal is "ONE-HOT";

attribute SAFE_IMPLEMENTATION :string;

attribute SAFE_IMPLEMENTATION of bit_sync_state_s: signal is"yes";

signal bit_sync_pulse_s,bit_xor_s : std_logic;

signal bit_sync_pulse_delay_s: std_logic;

signal bit_sync_counter_s: std_logic_vector(4 downto 0);

signal bit_sync_count_const_s: std_logic_vector(4 downto 0);

signal channel_oms_counter_s : std_logic_vector(10 downto 0);

signal channel_state_s : std_logic;

signal I_data_prev_s : std_logic;

signal p_cndr_temp_s, q_cndr_temp_s : std_logic_vector(27 downto 0);

signal one_ms_pulse_delay_s,one_ms_pulse_delay2_s: std_logic;

signal i_data_15_delay_s: std_logic;

signal bit_sync_pulse_delay2_s, data_bit_temp_s : std_logic;

begin

bit_sync_count_const_s <= "10100" ;

bit_sync_indicator_os <= bit_sync_indicator_s;

P_ch_oms_counter: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

channel_oms_counter_s <= (others => '0');

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if start_is = '1' then

channel_oms_counter_s <= (others => '0');

elsif one_ms_pulse_is = '1' then

channel_oms_counter_s <= channel_oms_counter_s + 1;

end if;  end if;

end process P_ch_oms_counter;

P_channel_state_reg: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

channel_state_s <= '0';

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if start_is = '1' then

channel_state_s <= '0';

elsif channel_oms_counter_s(10) = '1' then

channel_state_s <= '1';

end if;  end if;

end process P_channel_state_reg;

Process_p1: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

I_data_prev_s <= '0';

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

I_data_prev_s <= i_data_15_delay_s;

end if;

end process Process_p1;

Process_p2: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

i_data_15_delay_s <= '0';

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if one_ms_pulse_delay_s = '1' then

i_data_15_delay_s <= I_data_is(22);

end if;  end if;

end process Process_p2;

Process_p3: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

bit_sync_indicator_s <= '0' ;

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if start_is = '1' then

bit_sync_indicator_s <= '0' ;

elsif lock_status_is = '0' then

bit_sync_indicator_s <= '0' ;

elsif bit_sync_state_s = s4 then

bit_sync_indicator_s <= '1' ;

else

bit_sync_indicator_s <= '0' ;

end if;  end if;

end process Process_p3;

Process_p4: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

bit_sync_state_s <= s1 ;

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if start_is = '1' then

bit_sync_state_s <= s1;

elsif lock_status_is = '0' then

bit_sync_state_s <= s1;

elsif channel_state_s = '0' and channel_oms_counter_s(10) = '1' then

bit_sync_state_s <= s2;

elsif bit_sync_state_s = s2 and bit_xor_s = '1' and bit_sync_counter_s = bit_sync_count_const_s then

bit_sync_state_s <= s3;

elsif bit_sync_state_s = s3 and bit_xor_s = '1' and bit_sync_counter_s = bit_sync_count_const_s then

bit_sync_state_s <= s4;

elsif bit_sync_state_s = s3 and bit_xor_s = '1' then

bit_sync_state_s <= s2;

end if;  end if;

end process Process_p4;

P_new_bit_indicator: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

bit_sync_counter_s <= (others => '0');

bit_sync_pulse_delay_s <= '0';

bit_sync_pulse_delay2_s <= '0';

newdata_indicator_os <= '0';

navdata_os <= '0';

one_ms_pulse_delay_s <= '0';

one_ms_pulse_delay2_s <= '0';

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

one_ms_pulse_delay_s <= one_ms_pulse_is;

one_ms_pulse_delay2_s <= one_ms_pulse_delay_s;

if start_is = '1' then

bit_sync_counter_s <= (others => '0');

elsif channel_state_s = '0' and channel_oms_counter_s(10) = '1' then

bit_sync_counter_s <= (others => '0');

elsif bit_xor_s = '1' and bit_sync_state_s = s2 then

bit_sync_counter_s <= "00001" ;

elsif bit_xor_s = '1' and bit_sync_state_s = s3 then

bit_sync_counter_s <= "00001" ;

elsif bit_sync_pulse_s = '1' then

bit_sync_counter_s <= "00001" ;

elsif one_ms_pulse_delay2_s = '1' then

bit_sync_counter_s <= bit_sync_counter_s + 1;   end if;

if start_is = '1' then

bit_sync_pulse_delay_s <= '0';

bit_sync_pulse_delay2_s <= '0';

else

bit_sync_pulse_delay_s <= bit_sync_pulse_s ;

bit_sync_pulse_delay2_s <= bit_sync_pulse_delay_s;   end if;

if start_is = '1' then

navdata_os  <= '0';

elsif bit_sync_pulse_delay_s = '1' then

navdata_os <= data_bit_temp_s;    end if;

if start_is = '1' then

newdata_indicator_os <= '0';

elsif lock_status_is = '0' then					 newdata_indicator_os <= '0';

elsif lock_status_is = '1' and bit_sync_pulse_delay2_s = '1'  and bit_sync_indicator_s ='1' then

newdata_indicator_os <= '1';

elsif navdata_read_mp_is = '1' then

newdata_indicator_os <= '0';

end if;  end if;

end process P_new_bit_indicator;

P_CNDR_computation: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

p_cndr_temp_s <= (others => '0');

q_cndr_temp_s <= (others => '0');

p_cndr_os <= (others => '0');

q_cndr_os <= (others => '0');

data_bit_temp_s <= '0';

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if start_is = '1' then

p_cndr_temp_s <= (others => '0');

q_cndr_temp_s <= (others => '0');

elsif bit_sync_pulse_s = '1' then

p_cndr_temp_s <= sxt(I_data_is,28);

q_cndr_temp_s <= sxt(Q_data_is,28);

elsif one_ms_pulse_delay2_s = '1' then

p_cndr_temp_s <= p_cndr_temp_s + sxt(I_data_is,28);

q_cndr_temp_s <= q_cndr_temp_s + sxt(Q_data_is,28);

end if;

if start_is = '1' then

p_cndr_os <= (others => '0');

q_cndr_os <= (others => '0');

data_bit_temp_s <= '0';

elsif bit_sync_pulse_s = '1' then

p_cndr_os <= p_cndr_temp_s;

q_cndr_os <= q_cndr_temp_s;

data_bit_temp_s <= p_cndr_temp_s(27);

end if;  end if;

end process P_CNDR_computation;

bit_sync_found_pulse_os <= '1' when bit_sync_state_s = s3 and bit_sync_pulse_s = '1' and bit_xor_s = '1' else '0';

bit_sync_pulse_s <= '1' when bit_sync_counter_s = bit_sync_count_const_s and one_ms_pulse_delay2_s = '1' else

'0';

bit_xor_s <= i_data_15_delay_s xor I_data_prev_s;

end bit_sync_ch_a;

13) bit_sync_sbas_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_SIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity bit_sync_sbas_e is

PORT(

clk_40MHz_is : IN std_logic;

POR_is : IN std_logic;

start_is : IN std_logic;

one_ms_pulse_is: IN std_logic;

I_data_is, Q_data_is : IN std_logic_vector(22 downto 0);

lock_status_is : IN std_logic;

navdata_read_mp_is : IN std_logic;

sbas_enable_is : IN std_logic;

navdata_os : OUT std_logic;

newdata_indicator_os : OUT std_logic;

bit_sync_indicator_os : OUT std_logic;

bit_sync_found_pulse_os: OUT std_logic;

p_cndr_os : OUT std_logic_vector(27 downto 0);

q_cndr_os : OUT std_logic_vector(27 downto 0);

sbas_navdata_os : out std_logic_vector(5 downto 0)	);	end bit_sync_sbas_e;

architecture bit_sync_sbas_a of bit_sync_sbas_e is

signal bit_sync_indicator_s: std_logic;

type state_type is (s1,s2,s3,s4);

signal bit_sync_state_s : state_type;

attribute FSM_ENCODING : string;

attribute FSM_ENCODING of bit_sync_state_s: signal is "ONE-HOT";

attribute SAFE_IMPLEMENTATION :string;

attribute SAFE_IMPLEMENTATION of bit_sync_state_s: signal is"yes";

signal bit_sync_pulse_s,bit_xor_s : std_logic;

signal bit_sync_pulse_delay_s,bit_sync_pulse_delay2_s : std_logic;

signal bit_sync_counter_s: std_logic_vector(4 downto 0);

signal bit_sync_count_const_s: std_logic_vector(4 downto 0);

signal channel_oms_counter_s : std_logic_vector(10 downto 0);

signal channel_state_s : std_logic;

signal I_data_prev_s : std_logic;

signal p_cndr_temp_s, q_cndr_temp_s : std_logic_vector(27 downto 0);

signal one_ms_pulse_delay_s,one_ms_pulse_delay2_s,one_ms_pulse_delay3_s: std_logic;

signal i_data_15_delay_s,data_bit_temp_s: std_logic;

signal sbas_bit_counter_s : std_logic_vector(2 downto 0);

signal sbas_navdata_temp_s: std_logic_vector(5 downto 0);

signal sbas_idata_s :  std_logic_vector(22 downto 0);

begin

bit_sync_count_const_s <= "10100" when sbas_enable_is='0' else

"00010";

bit_sync_indicator_os <= bit_sync_indicator_s;

Process_p1: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

channel_oms_counter_s <= (others => '0');

sbas_idata_s <= (others => '0');

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if start_is = '1' then

channel_oms_counter_s <= (others => '0');

sbas_idata_s <= (others => '0');

elsif one_ms_pulse_is = '1' then

channel_oms_counter_s <= channel_oms_counter_s + 1;

sbas_idata_s <= abs(I_data_is);

end if;  end if;

end process Process_p1;

Process_p2: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

channel_state_s <= '0';

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if start_is = '1' then

channel_state_s <= '0';

elsif channel_oms_counter_s(10) = '1' then

channel_state_s <= '1';

end if;  end if;

end process Process_p2;

Process_p3: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

I_data_prev_s <= '0';

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

I_data_prev_s <= i_data_15_delay_s;

end if;

end process Process_p3;

Process_p4: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

i_data_15_delay_s <= '0';

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if one_ms_pulse_delay_s = '1' then

i_data_15_delay_s <= I_data_is(22);

end if;  end if;

end process Process_p4;

Process_p5: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

bit_sync_indicator_s <= '0' ;

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if start_is = '1' then

bit_sync_indicator_s <= '0' ;

elsif lock_status_is = '0' then

bit_sync_indicator_s <= '0' ;

elsif bit_sync_state_s = s4 then

bit_sync_indicator_s <= '1' ;  else

bit_sync_indicator_s <= '0' ;

end if;  end if;

end process Process_p5;

Process_p6: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

bit_sync_state_s <= s1 ;

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if start_is = '1' then

bit_sync_state_s <= s1;

elsif lock_status_is = '0' then

bit_sync_state_s <= s1;

elsif channel_state_s = '0' and channel_oms_counter_s(10) = '1' then

bit_sync_state_s <= s2;

elsif bit_sync_state_s = s2 and bit_xor_s = '1' and bit_sync_counter_s = bit_sync_count_const_s then

bit_sync_state_s <= s3;

elsif bit_sync_state_s = s3 and bit_xor_s = '1' and bit_sync_counter_s = bit_sync_count_const_s then

bit_sync_state_s <= s4;

elsif bit_sync_state_s = s3 and bit_xor_s = '1' then

bit_sync_state_s <= s2;

end if;   end if;

end process Process_p6;

process_p7: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

bit_sync_counter_s <= (others => '0');

bit_sync_pulse_delay_s <= '0';

bit_sync_pulse_delay2_s <= '0';

newdata_indicator_os <= '0';

navdata_os <= '0';

one_ms_pulse_delay_s <= '0';

one_ms_pulse_delay2_s <= '0';

one_ms_pulse_delay3_s <= '0';

sbas_navdata_os <= "000000";

sbas_bit_counter_s <= "000";

sbas_navdata_temp_s <= "000000";

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

one_ms_pulse_delay_s <= one_ms_pulse_is;

one_ms_pulse_delay2_s <= one_ms_pulse_delay_s;

one_ms_pulse_delay3_s <= one_ms_pulse_delay2_s;

if start_is = '1' then

bit_sync_counter_s <= (others => '0');

elsif channel_state_s = '0' and channel_oms_counter_s(10) = '1' then

bit_sync_counter_s <= (others => '0');

elsif bit_xor_s = '1' and bit_sync_state_s = s2 then

bit_sync_counter_s <= "00001" ;

elsif bit_xor_s = '1' and bit_sync_state_s = s3 then

bit_sync_counter_s <= "00001" ;

elsif bit_sync_pulse_s = '1' then

bit_sync_counter_s <= "00001" ;

elsif one_ms_pulse_delay2_s = '1' then

bit_sync_counter_s <= bit_sync_counter_s + 1;

end if;

if start_is = '1' then

sbas_bit_counter_s <= (others => '0');

elsif bit_sync_state_s = s2 or bit_sync_state_s = s3 then

sbas_bit_counter_s <= (others => '0');

elsif bit_sync_pulse_s = '1' then

sbas_bit_counter_s <= sbas_bit_counter_s + 1;

elsif sbas_bit_counter_s = "110" then

sbas_bit_counter_s <= (others => '0');

end if;

if start_is = '1' then

bit_sync_pulse_delay_s <= '0';

bit_sync_pulse_delay2_s <= '0';   else

bit_sync_pulse_delay_s <= bit_sync_pulse_s ;

bit_sync_pulse_delay2_s <= bit_sync_pulse_delay_s;

end if;

if start_is = '1' then

sbas_navdata_temp_s <= (others => '0');

elsif bit_sync_pulse_s = '1' then

sbas_navdata_temp_s(5 downto 0) <= sbas_navdata_temp_s(4 downto 0) & I_data_is(22);

end if;

if start_is = '1' then

sbas_navdata_os <= (others => '0');

elsif sbas_bit_counter_s = "110" then

sbas_navdata_os <= sbas_navdata_temp_s;

end if;

if start_is = '1' then

navdata_os  <= '0';

elsif bit_sync_pulse_delay_s = '1' then

navdata_os <= data_bit_temp_s;   end if;

if start_is = '1' then

newdata_indicator_os <= '0';

elsif lock_status_is = '0' then					 newdata_indicator_os <= '0';

elsif lock_status_is = '1' and sbas_bit_counter_s = "110" and bit_sync_indicator_s ='1' and sbas_enable_is = '1' then

newdata_indicator_os <= '1';

elsif lock_status_is = '1' and bit_sync_pulse_delay2_s = '1'  and bit_sync_indicator_s ='1' and sbas_enable_is = '0' then

newdata_indicator_os <= '1';

elsif navdata_read_mp_is = '1' then

newdata_indicator_os <= '0';  end if;  end if;

end process process_p7;

Process_p8: process(clk_40MHz_is, POR_is)

begin

if POR_is = '1' then

p_cndr_temp_s <= (others => '0');

q_cndr_temp_s <= (others => '0');

p_cndr_os <= (others => '0');

q_cndr_os <= (others => '0');

data_bit_temp_s <= '0';

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if start_is = '1' then

p_cndr_temp_s <= (others => '0');

q_cndr_temp_s <= (others => '0');

elsif bit_sync_pulse_s = '1' and sbas_enable_is = '0' then

p_cndr_temp_s <= sxt(I_data_is,28);

q_cndr_temp_s <= sxt(Q_data_is,28);

elsif sbas_bit_counter_s = "110" and sbas_enable_is = '1' then

p_cndr_temp_s <= sxt(sbas_idata_s,28);

q_cndr_temp_s <= sxt(Q_data_is,28);

elsif one_ms_pulse_delay2_s = '1' and sbas_enable_is = '0'  then

p_cndr_temp_s <= p_cndr_temp_s + sxt(I_data_is,28);

q_cndr_temp_s <= q_cndr_temp_s + sxt(Q_data_is,28);

elsif one_ms_pulse_delay3_s = '1' and sbas_enable_is = '1'  then

p_cndr_temp_s <= p_cndr_temp_s + sxt(sbas_idata_s,28);

q_cndr_temp_s <= q_cndr_temp_s + sxt(Q_data_is,28);

end if;

if start_is = '1' then

p_cndr_os <= (others => '0');

q_cndr_os <= (others => '0');

data_bit_temp_s <= '0';

elsif bit_sync_pulse_s = '1' then

p_cndr_os <= p_cndr_temp_s;

q_cndr_os <= q_cndr_temp_s;

data_bit_temp_s <= p_cndr_temp_s(27);

end if;   end if;

end process Process_p8;

bit_sync_found_pulse_os <= '1' when bit_sync_state_s = s3 and bit_sync_pulse_s = '1' and bit_xor_s = '1' else '0';

bit_sync_pulse_s <= '1' when bit_sync_counter_s = bit_sync_count_const_s and one_ms_pulse_delay2_s = '1' else

'0';

bit_xor_s <= i_data_15_delay_s xor I_data_prev_s;

end bit_sync_sbas_a;

14) blk_mem_gen_v6_2

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

LIBRARY XilinxCoreLib;

ENTITY blk_mem_gen_v6_2 IS

PORT (

clka : IN STD_LOGIC;

ena : IN STD_LOGIC;

wea : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

addra : IN STD_LOGIC_VECTOR(16 DOWNTO 0);

dina : IN STD_LOGIC_VECTOR(31 DOWNTO 0);

clkb : IN STD_LOGIC;

enb : IN STD_LOGIC;

addrb : IN STD_LOGIC_VECTOR(16 DOWNTO 0);

doutb : OUT STD_LOGIC_VECTOR(31 DOWNTO 0)  );

END blk_mem_gen_v6_2;

ARCHITECTURE blk_mem_gen_v6_2_a OF blk_mem_gen_v6_2 IS

COMPONENT wrapped_blk_mem_gen_v6_2

PORT ( clka : IN STD_LOGIC;

ena : IN STD_LOGIC;

wea : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

addra : IN STD_LOGIC_VECTOR(16 DOWNTO 0);

dina : IN STD_LOGIC_VECTOR(31 DOWNTO 0);

clkb : IN STD_LOGIC;

enb : IN STD_LOGIC;

addrb : IN STD_LOGIC_VECTOR(16 DOWNTO 0);

doutb : OUT STD_LOGIC_VECTOR(31 DOWNTO 0) );

END COMPONENT;

FOR ALL : wrapped_blk_mem_gen_v6_2 USE ENTITY XilinxCoreLib.blk_mem_gen_v6_2(behavioral)

GENERIC MAP (  c_addra_width => 17, c_addrb_width => 17,

c_algorithm => 1, c_axi_id_width => 4,

c_axi_slave_type => 0, c_axi_type => 1,

c_byte_size => 9,c_common_clk => 1,

c_default_data => "0", c_disable_warn_bhv_coll => 0,

c_disable_warn_bhv_range => 0, c_family => "virtex5",

c_has_axi_id => 0,   c_has_ena => 1,

c_has_enb => 1,  c_has_injecterr => 0,

c_has_mem_output_regs_a => 0,  c_has_mem_output_regs_b => 0,

c_has_mux_output_regs_a => 0, c_has_mux_output_regs_b => 0,

c_has_regcea => 0,  c_has_regceb => 0,

c_has_rsta => 0,  c_has_rstb => 0,

c_has_softecc_input_regs_a => 0,  c_has_softecc_output_regs_b => 0,

c_init_file_name => "blk_mem_gen_v6_2.mif",  c_inita_val => "0",

c_initb_val => "0",  c_interface_type => 0,

c_load_init_file => 1, c_mem_type => 1,

c_mux_pipeline_stages => 0,  c_prim_type => 1,

c_read_depth_a => 131072,  c_read_depth_b => 131072,

c_read_width_a => 32, c_read_width_b => 32,

c_rst_priority_a => "CE",  c_rst_priority_b => "CE",

c_rst_type => "SYNC",  c_rstram_a => 0,

c_rstram_b => 0,  c_sim_collision_check => "ALL",

c_use_byte_wea => 0,  c_use_byte_web => 0,

c_use_default_data => 0, c_use_ecc => 0,

c_use_softecc => 0,  c_wea_width => 1,

c_web_width => 1, c_write_depth_a => 131072,

c_write_depth_b => 131072,  c_write_mode_a => "WRITE_FIRST",

c_write_mode_b => "NO_CHANGE", c_write_width_a => 32,

c_write_width_b => 32, c_xdevicefamily => "virtex5" );

BEGIN

U0 : wrapped_blk_mem_gen_v6_2

PORT MAP ( clka => clka,  ena => ena,

wea => wea, addra => addra,

dina => dina, clkb => clkb,

enb => enb,addrb => addrb,

doutb => doutb );

END blk_mem_gen_v6_2_a;

15) BRM

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.NUMERIC_STD.all;

use work.convert.all;

entity BRM is

generic ( BCENABLE : INTEGER range 0 to 1 := 1;

RTENABLE : INTEGER range 0 to 1 := 1;

MTENABLE : INTEGER range 0 to 1 := 1;

LEGREGS  : INTEGER range 0 to 5 := 1;

ENHANCED : INTEGER range 0 to 1 := 1;

INITFREQ : INTEGER range 12 to 24  := 24;

BETIMING : INTEGER range 0 to 1    := 0);

port ( CLK        : in  std_logic;   TCLK       : in  std_logic;

RSTINn     : in  std_logic;   RSTOUTn    : out std_logic;

-- 1553B Interface

BUSAINEN   : out std_logic;   BUSAINP    : in  std_logic;

BUSAINN    : in  std_logic;   BUSBINEN   : out std_logic;

BUSBINP    : in  std_logic;    BUSBINN    : in  std_logic;

BUSAOUTIN  : out std_logic;   BUSAOUTP   : out std_logic;

BUSAOUTN   : out std_logic;   BUSBOUTIN  : out std_logic;

BUSBOUTP   : out std_logic;   BUSBOUTN   : out std_logic;

-- BRM Control Signals - should be tied high or low

CPUMEMEN   : in  std_logic;   ASYNCIF    : in  std_logic;

INTLEVEL   : in  std_logic;

-- Combined Status Signals

ACTIVE     : out std_logic;   BUSY       : out std_logic;

INTOUTH    : out std_logic;   INTOUTM    : out std_logic;

INTACKH    : in  std_logic;   INTACKM    : in  std_logic;

MEMFAIL    : out std_logic;  OPMODE     : out std_logic_vector(1 downto 0);

-- RT ADDRESS Setting

LOCKn      : in  std_logic;

RTADDRIN   : in  std_logic_vector(4 downto 0); RTADDRPIN  : in  std_logic;

RTADERR    : out std_logic;  MSELIN     : in  std_logic_vector(1 downto 0);

SSYSFn     : in  std_logic;   ABSTDIN    : in  std_logic;

-- RT Status Signals

MSGSTART   : out std_logic;   CMDSYNC    : out std_logic;

SYNCNOW    : out std_logic;   BUSRESET   : out std_logic;

-- RT Command Legalization Interface

CMDSTB     : out std_logic; CMDVAL     : out std_logic_vector(11 downto 0);

CMDOK      : in  std_logic;  CMDOKOUT   : out std_logic;

-- CPU Interface

CPUWRn     : in  std_logic_vector(1 downto 0); CPURDn     : in  std_logic;

CPUCSn     : in  std_logic;  CPUMEM     : in  std_logic;

CPUWAITn   : out std_logic;  CPUDEN     : out std_logic;

CPUADDR    : in  std_logic_vector(15 downto 0);

CPUDOUT    : out std_logic_vector(15 downto 0);

CPUDIN     : in  std_logic_vector(15 downto 0);

-- Backend Memory interface

MEMREQn    : out std_logic;  MEMGNTn    : in  std_logic;

MEMACCn    : out std_logic;MEMWRn     : out std_logic_vector(1 downto 0);

MEMRDn     : out std_logic;  MEMCSn     : out std_logic;

MEMWAITn   : in  std_logic;  MEMCEN     : out std_logic;

MEMDEN     : out std_logic;

MEMADDR    : out std_logic_vector(15 downto 0);

MEMDOUT    : out std_logic_vector(15 downto 0);

MEMDIN     : in  std_logic_vector(15 downto 0)  );

end BRM;

architecture RTL of BRM is

attribute KEEP_HIERARCHY : string;

attribute KEEP_HIERARCHY of RTL: architecture is "TRUE";

component BRMPC

generic ( BCENABLE : INTEGER range 0 to 1   :=  1;

RTENABLE : INTEGER range 0 to 1   :=  1;

MTENABLE : INTEGER range 0 to 1   :=  1;

LEGREGS  : INTEGER range 0 to 5   :=  1;

ENHANCED : INTEGER range 0 to 1   :=  1;

INITFREQ : INTEGER range 12 to 24 := 24);

port ( CLK            : in  std_logic;  TCLK           : in  std_logic;

RSTINn         : in  std_logic;   RSTOUTn        : out std_logic;

CLKSPD         : out std_logic_vector(1 downto 0);

LOCKn          : in  std_logic;

RTADDRIN       : in  std_logic_vector(4 downto 0);

RTADDRPIN      : in  std_logic;  RTADERR        : out std_logic;

MSELIN         : in  std_logic_vector(1 downto 0);  SSYSFn         : in  std_logic;

ABSTDIN        : in  std_logic;  SHUTDOWNA      : out std_logic;

SHUTDOWNB      : out std_logic;  RXBUSENA       : out std_logic;

RXBUSENB       : out std_logic;  BUSSEL         : out std_logic;

DECA_FROMUS    : in  std_logic;  DECA_STROBE    : in  std_logic;

DECA_DATAOUT   : in  std_logic_vector(15 downto 0);

DECA_CMDSTAT   : in  std_logic;  DECA_GOTSYNC   : in  std_logic;

DECA_PARERR    : in  std_logic;  DECA_MANERR    : in  std_logic;

DECA_ACTIVE    : in  std_logic;  DECA_CONTIG    : in  std_logic;

DECB_FROMUS    : in  std_logic;  DECB_STROBE    : in  std_logic;

DECB_DATAOUT   : in  std_logic_vector(15 downto 0);

DECB_CMDSTAT   : in  std_logic;  DECB_GOTSYNC   : in  std_logic;

DECB_PARERR    : in  std_logic;  DECB_MANERR    : in  std_logic;

DECB_ACTIVE    : in  std_logic;  DECB_CONTIG    : in  std_logic;

LOOPFAILA      : in  std_logic;  LOOPFAILB      : in  std_logic;

ENCSTROBE      : out std_logic;  ENCABORT       : out std_logic;

ENCDATA        : out std_logic_vector(15 downto 0);

ENCCMDSTAT     : out std_logic;         ENCREADY       : in  std_logic;

ENCBUSY        : in  std_logic;   ENCCLEAR       : out std_logic;

TXTIMEOUT      : in  std_logic;   BENDREQ        : out std_logic;

BENDBURST      : out std_logic;   BENDWRT        : out std_logic;

BENDBUSY       : in  std_logic;  BENDFAIL       : in  std_logic;

BENDADDR       : out std_logic_vector(15 downto 0);

BENDDIN        : in  std_logic_vector(15 downto 0);

BENDDOUT       : out std_logic_vector(15 downto 0);

CPUWR          : in  std_logic_vector(1 downto 0);   CPURD          : in  std_logic;

CPUADDR        : in  std_logic_vector(5 downto 0);

CPUDIN         : in  std_logic_vector(15 downto 0);

CPUDOUT        : out std_logic_vector(15 downto 0);

MEMFAIL        : out std_logic;   INTLEVEL       : in  std_logic;

INTOUTH        : out std_logic;   INTOUTM        : out std_logic;

INTACKH        : in  std_logic;   INTACKM        : in  std_logic;

ACTIVE         : out std_logic;   BUSY           : out std_logic;

OPMODE         : out std_logic_vector(1 downto 0);

MSGSTART       : out std_logic;  CMDSYNC        : out std_logic;

SYNCNOW        : out std_logic;   BUSRESET       : out std_logic;

CMDVAL         : out std_logic_vector(11 downto 0);

CMDSTB         : out std_logic;

CMDOK          : in  std_logic;   CMDOKOUT       : out std_logic;

LOOPBACK       : out std_logic );

end component;

component BRMENC

port ( CLK        : in  std_logic;  RSTn       : in  std_logic;

CLKSPD     : in  std_logic_vector(1 downto 0);     STROBE     : in  std_logic;

ABORTX     : in  std_logic;

DATAIN     : in  std_logic_vector(15 downto 0);

CMDSTAT    : in  std_logic;       TXENABLE   : out std_logic;

TXBUSY     : out std_logic;        TXREADY    : out std_logic;

DOUT       : out std_logic;          TXENA      : in  std_logic;

RXSTROBEA  : in  std_logic;    RXERRORA   : in  std_logic;

RXDATAA    : in  std_logic_vector(15 downto 0);

RXDATAAS   : in  std_logic;       LOOPFAILA  : out std_logic;

TXENB      : in  std_logic;         RXSTROBEB  : in  std_logic;

RXERRORB   : in  std_logic;

RXDATAB    : in  std_logic_vector(15 downto 0);

RXDATABS   : in  std_logic;     LOOPFAILB  : out std_logic;

CLRERRORS  : in  std_logic;     FSM_ERROR  : out std_logic );

end component;

component BRMDEC

port ( CLK       : in  std_logic;   RSTn      : in  std_logic;

CLKSPD    : in  std_logic_vector(1 downto 0);

DATAINP   : in  std_logic;  DATAINN   : in  std_logic;

TXENABLE  : in  std_logic;  FROMUS    : out std_logic;

STROBE    : out std_logic;   DATAOUT   : out std_logic_vector(15 downto 0);

CMDSTAT   : out std_logic;   GOTSYNC   : out std_logic;

PARERR    : out std_logic;   MANERR    : out std_logic;

ACTIVE    : out std_logic;   CONTIG    : out std_logic);

end component;

component BRMBEND

generic ( BETIMING : INTEGER range 0 to 1 );

port ( CLK       : in  std_logic;   RSTn      : in  std_logic;

CLKSPD    : in  std_logic_vector(1 downto 0);    ASYNCIF   : in  std_logic;

CPUMEMEN  : in  std_logic;   CPUREQ    : in  std_logic;

CPUWRT    : in  std_logic_vector(1 downto 0);    CPUWAIT   : out std_logic;

CPUADDR   : in  std_logic_vector(15 downto 0);

CPUDOUT   : in  std_logic_vector(15 downto 0);

CPUDIN    : out std_logic_vector(15 downto 0);   BENDREQ   : in  std_logic;

BENDWRT   : in  std_logic;   BENDBURST : in  std_logic;

BENDBUSY  : out std_logic;    BENDFAIL  : out std_logic;

BENDADDR  : in  std_logic_vector(15 downto 0);

BENDDOUT  : in  std_logic_vector(15 downto 0);

BENDDIN   : out std_logic_vector(15 downto 0);    MEMREQn   : out std_logic;

MEMGNTn   : in  std_logic;    MEMACCn   : out std_logic;

MEMWRn    : out std_logic_vector(1 downto 0);   MEMRDn    : out std_logic;

MEMCSn    : out std_logic;    MEMWAITn  : in  std_logic;

MEMCEN    : out std_logic;    MEMDEN    : out std_logic;

MEMADDR   : out std_logic_vector(15 downto 0);

MEMDOUT   : out std_logic_vector(15 downto 0);

MEMDIN    : in  std_logic_vector(15 downto 0));

end component;

-- Encoder Signals

signal  BUSOUT         : std_logic;    signal  BUSA_INUSE     : std_logic;

signal  BUSB_INUSE     : std_logic;    signal  BUSA_TXENABLE  : std_logic;

signal  BUSB_TXENABLE  : std_logic;

signal  ENC_DATA       : std_logic_vector(15 downto 0);

signal  ENC_STROBE     : std_logic;    signal  ENC_ABORT      : std_logic;

signal  ENC_CMDSTAT    : std_logic;    signal  ENC_READY      : std_logic;

signal  ENC_BUSY       : std_logic;      signal  ENC_CLEAR      : std_logic;

signal  TXTIMEOUT      : std_logic;    signal  TXT_ENABLE     : std_logic;

signal  SHUTDOWNA      : std_logic;    signal  SHUTDOWNB      : std_logic;

signal  RXBUSENA       : std_logic;     signal  RXBUSENB       : std_logic;

signal  LOOPFAILA      : std_logic;    signal  LOOPFAILB      : std_logic;

signal  HWTIMVALUE     : UNSIGNED(6 downto 0);

-- Decoder Signals

signal  BUSAINP_LP     : std_logic;    signal  BUSAINN_LP     : std_logic;

signal  BUSBINP_LP     : std_logic;    signal  BUSBINN_LP     : std_logic;

signal  BUSSEL         : std_logic;       signal  DECA_STROBE    : std_logic;

signal  DECA_ERROR     : std_logic;

signal  DECA_DATAOUT   : std_logic_vector(15 downto 0);

signal  DECA_CMDSTAT   : std_logic;    signal  DECA_GOTSYNC   : std_logic;

signal  DECA_PARERR    : std_logic;      signal  DECA_MANERR    : std_logic;

signal  DECA_ACTIVE    : std_logic;       signal  DECA_CONTIG    : std_logic;

signal  DECA_FROMUS    : std_logic;     signal  DECB_STROBE    : std_logic;

signal  DECB_ERROR     : std_logic;

signal  DECB_DATAOUT   : std_logic_vector(15 downto 0);

signal  DECB_CMDSTAT   : std_logic;    signal  DECB_GOTSYNC   : std_logic;

signal  DECB_PARERR    : std_logic;     signal  DECB_MANERR    : std_logic;

signal  DECB_ACTIVE    : std_logic;      signal  DECB_CONTIG    : std_logic;

signal  DECB_FROMUS    : std_logic;

-- CPU interface Signals

signal  CPURDREG       : std_logic;

signal  CPUWRREG       : std_logic_vector(1 downto 0);

signal  CPUREGOUT      : std_logic_vector(15 downto 0);

-- Backend Signals

signal  BCPUREQ        : std_logic;         signal  BCPUWAIT       : std_logic;

signal  BCPUWRT        : std_logic_vector(1 downto 0);

signal  BCPUADDR       : std_logic_vector(15 downto 0);

signal  BCPUDIN        : std_logic_vector(15 downto 0);

signal  BCPUDOUT       : std_logic_vector(15 downto 0);

signal  BENDREQ        : std_logic;     signal  BENDWRT        : std_logic;

signal  BENDBURST      : std_logic;    signal  BENDBUSY       : std_logic;

signal  BENDADDR       : std_logic_vector(15 downto 0);

signal  BENDDIN        : std_logic_vector(15 downto 0);

signal  BENDDOUT       : std_logic_vector(15 downto 0);

signal  BENDFAIL       : std_logic;

-- Misc Signals

signal  CLKSPD         : std_logic_vector(1 downto 0);

signal  iOPMODE        : std_logic_vector(1 downto 0);

signal  LOOPBACK       : std_logic;     signal  RSTn           : std_logic;

signal  Logic0         : std_logic;       signal  Logic1         : std_logic;

begin

Logic0 <= '0';

Logic1 <= '1';

BUSAENC: BRMENC

port map ( CLK        => CLK,   RSTn       => RSTn,

CLKSPD     => CLKSPD,   STROBE     => ENC_STROBE,

ABORTX     => ENC_ABORT,  DATAIN     => ENC_DATA,

CMDSTAT    => ENC_CMDSTAT,  TXREADY    => ENC_READY,

TXBUSY     => ENC_BUSY,  TXENABLE   => TXT_ENABLE,

DOUT       => BUSOUT,   TXENA      => BUSA_TXENABLE,

RXSTROBEA  => DECA_STROBE,   RXERRORA   => DECA_ERROR,

RXDATAA    => DECA_DATAOUT,   RXDATAAS   => DECA_CMDSTAT,

LOOPFAILA  => LOOPFAILA,   TXENB      => BUSB_TXENABLE,

RXSTROBEB  => DECB_STROBE,  RXERRORB   => DECB_ERROR,

RXDATAB    => DECB_DATAOUT,  RXDATABS   => DECB_CMDSTAT,

LOOPFAILB  => LOOPFAILB,   CLRERRORS  => ENC_CLEAR,

FSM_ERROR  => open   );

BUSA_INUSE <= not BUSSEL;

BUSB_INUSE <= BUSSEL;

BUSA_TXENABLE <= BUSA_INUSE and TXT_ENABLE;

BUSB_TXENABLE <= BUSB_INUSE and TXT_ENABLE;

-- Generate all the output signals

PTXT:process(CLK,RSTn)

begin

if RSTn='0' then

BUSAOUTIN <= '1';  BUSAOUTP  <= '1';

BUSAOUTN  <= '1';  BUSBOUTIN <= '1';

BUSBOUTP  <= '1';  BUSBOUTN  <= '1';

BUSAINEN  <= '0';   BUSBINEN  <= '0';

elsif CLK'event and CLK='1' then

BUSAOUTIN <= SHUTDOWNA;  BUSBOUTIN <= SHUTDOWNB;

BUSAINEN  <= RXBUSENA;   BUSBINEN  <= RXBUSENB;

BUSAOUTP  <= BUSOUT     or not BUSA_TXENABLE;

BUSAOUTN  <= not BUSOUT or not BUSA_TXENABLE;

BUSBOUTP  <= BUSOUT     or not BUSB_TXENABLE;

BUSBOUTN  <= not BUSOUT or not BUSB_TXENABLE;

if LOOPBACK='1' then   -- Dissable outputs to transceiver

BUSAOUTIN <= '1';   BUSBOUTIN <= '1';

BUSAINEN  <= '0';  BUSBINEN  <= '0';

BUSAOUTP  <= '1';  BUSAOUTN  <= '1';

BUSBOUTP  <= '1';  BUSBOUTN  <= '1';

end if;  end if;

end process;

process(CLKSPD)

begin

case CLKSPD is

when "00"   => HWTIMVALUE <= "0100001"; -- 12MHz

when "01"   => HWTIMVALUE <= "0101011"; -- 16MHz

when "10"   => HWTIMVALUE <= "0110110"; -- 20MHz

when others => HWTIMVALUE <= "1000001"; -- 24MHz

end case;

end process;

PTXTTIM:

process(CLK,RSTn)

variable COUNT : UNSIGNED(14 downto 0);

begin

if RSTn='0' then

COUNT     := ( others => '0');   TXTIMEOUT <= '0';

elsif CLK'event and CLK='1' then

TXTIMEOUT <= '0';

if ENC_BUSY='1' and TXTIMEOUT='0' then

COUNT := COUNT + 1;    else

COUNT := ( others => '0');    end if;

TXTIMEOUT <= to_logic( COUNT(14 downto 8) = HWTIMVALUE);

end if;

end process;

BUSAINP_LP  <= BUSAINP  when LOOPBACK='0' else (BUSOUT     or not BUSA_TXENABLE);

BUSAINN_LP  <= BUSAINN  when LOOPBACK='0' else (not BUSOUT or not BUSA_TXENABLE);

BUSBINP_LP  <= BUSBINP  when LOOPBACK='0' else (BUSOUT     or not BUSB_TXENABLE);

BUSBINN_LP  <= BUSBINN  when LOOPBACK='0' else (not BUSOUT or not BUSB_TXENABLE);

BUSADEC: BRMDEC

port map ( CLK       => CLK,                 RSTn      => RSTn,

CLKSPD    => CLKSPD,               DATAINP   => BUSAINP_LP,

DATAINN   => BUSAINN_LP,  TXENABLE  => BUSA_TXENABLE,

FROMUS    => DECA_FROMUS,   STROBE    => DECA_STROBE,

DATAOUT   => DECA_DATAOUT,        CMDSTAT   => DECA_CMDSTAT,

GOTSYNC   => DECA_GOTSYNC,        PARERR    => DECA_PARERR,

MANERR    => DECA_MANERR,         ACTIVE    => DECA_ACTIVE,

CONTIG    => DECA_CONTIG         );

BUSBDEC: BRMDEC

port map ( CLK       => CLK,                 RSTn      => RSTn,

CLKSPD    => CLKSPD,               DATAINP   => BUSBINP_LP,

DATAINN   => BUSBINN_LP,   TXENABLE  => BUSB_TXENABLE,

FROMUS    => DECB_FROMUS,  STROBE    => DECB_STROBE,

DATAOUT   => DECB_DATAOUT,        CMDSTAT   => DECB_CMDSTAT,

GOTSYNC   => DECB_GOTSYNC,        PARERR    => DECB_PARERR,

MANERR    => DECB_MANERR,         ACTIVE    => DECB_ACTIVE,

CONTIG    => DECB_CONTIG       );

DECA_ERROR <= DECA_PARERR or DECA_MANERR;

DECB_ERROR <= DECB_PARERR or DECB_MANERR;

UPC : BRMPC

generic map

( BCENABLE => BCENABLE,    RTENABLE => RTENABLE,

MTENABLE => MTENABLE,    LEGREGS  => LEGREGS,

ENHANCED => ENHANCED,  INITFREQ => INITFREQ   )

port map ( CLK            =>  CLK,  TCLK           =>  TCLK,

RSTINn         =>  RSTINn,   RSTOUTn        =>  RSTn,

CLKSPD         =>  CLKSPD,   LOCKn          =>  LOCKn,

RTADDRIN       =>  RTADDRIN,    RTADDRPIN      =>  RTADDRPIN,

RTADERR        =>  RTADERR,   MSELIN         =>  MSELIN,

SSYSFn         =>  SSYSFn,    ABSTDIN        =>  ABSTDIN,

SHUTDOWNA      =>  SHUTDOWNA,   SHUTDOWNB      =>  SHUTDOWNB,

RXBUSENA       =>  RXBUSENA,   RXBUSENB       =>  RXBUSENB,

BUSSEL         =>  BUSSEL,   DECA_FROMUS    =>  DECA_FROMUS,

DECA_STROBE    =>  DECA_STROBE,  DECA_DATAOUT   =>  DECA_DATAOUT,

DECA_CMDSTAT   =>  DECA_CMDSTAT,

DECA_GOTSYNC   =>  DECA_GOTSYNC,

DECA_PARERR    =>  DECA_PARERR,  DECA_MANERR    =>  DECA_MANERR,

DECA_ACTIVE    =>  DECA_ACTIVE,  DECA_CONTIG    =>  DECA_CONTIG,

DECB_FROMUS    =>  DECB_FROMUS,  DECB_STROBE    =>  DECB_STROBE,

DECB_DATAOUT   =>  DECB_DATAOUT,

DECB_CMDSTAT   =>  DECB_CMDSTAT,

DECB_GOTSYNC   =>  DECB_GOTSYNC,

DECB_PARERR    =>  DECB_PARERR,  DECB_MANERR    =>  DECB_MANERR,

DECB_ACTIVE    =>  DECB_ACTIVE,  DECB_CONTIG    =>  DECB_CONTIG,

LOOPFAILA      =>  LOOPFAILA,   LOOPFAILB      =>  LOOPFAILB,

ENCSTROBE      =>  ENC_STROBE, ENCABORT       =>  ENC_ABORT,

ENCDATA        =>  ENC_DATA,       ENCCMDSTAT     =>  ENC_CMDSTAT,

ENCREADY       =>  ENC_READY,  ENCBUSY        =>  ENC_BUSY,

ENCCLEAR       =>  ENC_CLEAR, TXTIMEOUT      =>  TXTIMEOUT,

BENDREQ        =>  BENDREQ,   BENDBURST      =>  BENDBURST,

BENDWRT        =>  BENDWRT,  BENDBUSY       =>  BENDBUSY,

BENDFAIL       =>  BENDFAIL,   BENDADDR       =>  BENDADDR,

BENDDIN        =>  BENDDIN,    BENDDOUT       =>  BENDDOUT,

CPUWR          =>  CPUWRREG,      CPURD          =>  CPURDREG,

CPUADDR        =>  CPUADDR(5 downto 0), CPUDIN         =>  CPUDIN,

CPUDOUT        =>  CPUREGOUT, MEMFAIL        =>  MEMFAIL,

INTLEVEL       =>  INTLEVEL,  INTOUTH        =>  INTOUTH,

INTOUTM        =>  INTOUTM,   INTACKH        =>  INTACKH,

INTACKM        =>  INTACKM,  BUSY           =>  BUSY,

ACTIVE         =>  ACTIVE,  OPMODE         =>  iOPMODE,

MSGSTART       =>  MSGSTART,  CMDSYNC        =>  CMDSYNC,

SYNCNOW        =>  SYNCNOW,   BUSRESET       =>  BUSRESET,

CMDVAL         =>  CMDVAL,    CMDOK          =>  CMDOK,

CMDSTB         =>  CMDSTB,     CMDOKOUT       =>  CMDOKOUT,

LOOPBACK       =>  LOOPBACK   );

OPMODE  <= iOPMODE;

RSTOUTn <= RSTn;

BCPUREQ     <=    (not CPUWRN(0) or not CPUWRN(1) or not CPURDN )

and not CPUCSN and CPUMEM and CPUMEMEN;

BCPUWRT(0)  <= not CPUWRN(0) and CPUMEMEN;

BCPUWRT(1)  <= not CPUWRN(1) and CPUMEMEN;

CPUWRREG(0) <= not CPUWRN(0) and not CPUCSN and not CPUMEM;

CPUWRREG(1) <= not CPUWRN(1) and not CPUCSN and not CPUMEM;

CPURDREG    <= not CPURDN and not CPUCSN and not CPUMEM;

BCPUADDR <= CPUADDR;

BCPUDIN  <= CPUDIN;

CPUDOUT  <= BCPUDOUT when CPUMEM='1' else CPUREGOUT;

CPUWAITN <= not (BCPUWAIT and BCPUREQ);

CPUDEN   <= not CPUCSN and not CPURDN;

UBEND:  BRMBEND

generic map ( BETIMING => BETIMING )

port map      ( CLK       =>  CLK,   RSTn      =>  RSTn,

CLKSPD    =>  CLKSPD,         ASYNCIF   =>  ASYNCIF,

CPUMEMEN  =>  CPUMEMEN,CPUREQ    =>  BCPUREQ,

CPUWRT    =>  BCPUWRT,  CPUWAIT   =>  BCPUWAIT,

CPUADDR   =>  BCPUADDR,

CPUDOUT   =>  BCPUDIN,      -- Input from CPU is output to RAM

CPUDIN    =>  BCPUDOUT,     -- and vise versa

BENDREQ   =>  BENDREQ,  BENDWRT   =>  BENDWRT,

BENDBURST =>  BENDBURST,   BENDBUSY  =>  BENDBUSY,

BENDFAIL  =>  BENDFAIL,  BENDADDR  =>  BENDADDR,

BENDDOUT  =>  BENDDOUT,   BENDDIN   =>  BENDDIN,

MEMREQn   =>  MEMREQn,    MEMGNTn   =>  MEMGNTn,

MEMACCn   =>  MEMACCn,   MEMWRn    =>  MEMWRn,

MEMRDn    =>  MEMRDn,    MEMCSn    =>  MEMCSn,

MEMWAITn  =>  MEMWAITn,

MEMCEN    =>  MEMCEN,  MEMDEN    =>  MEMDEN,

MEMADDR   =>  MEMADDR, MEMDOUT   =>  MEMDOUT,

MEMDIN    =>  MEMDIN );

end RTL;

16) BRMbend

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.NUMERIC_STD.all;

entity BRMbend is

generic ( BETIMING : INTEGER range 0 to 1 );

port ( CLK       : in  std_logic;   RSTn      : in  std_logic;

CLKSPD    : in  std_logic_vector(1 downto 0);

ASYNCIF   : in  std_logic;    CPUMEMEN  : in  std_logic;

-- Interface from the CPU Interface

CPUREQ    : in  std_logic;

CPUWRT    : in  std_logic_vector(1 downto 0);

CPUWAIT   : out std_logic;

CPUADDR   : in  std_logic_vector(15 downto 0);

CPUDOUT   : in  std_logic_vector(15 downto 0);

CPUDIN    : out std_logic_vector(15 downto 0);

-- Interface from the BC logic

BENDREQ   : in  std_logic;   BENDWRT   : in  std_logic;

BENDBURST : in  std_logic;   BENDBUSY  : out std_logic;

BENDFAIL  : out std_logic;

BENDADDR  : in  std_logic_vector(15 downto 0);

BENDDOUT  : in  std_logic_vector(15 downto 0);

BENDDIN   : out std_logic_vector(15 downto 0);

-- Backend Memory interface

MEMREQn   : out std_logic;   MEMGNTn   : in  std_logic;

MEMACCn   : out std_logic;

MEMWRn    : out std_logic_vector(1 downto 0);

MEMRDn    : out std_logic;     MEMCSn    : out std_logic;

MEMWAITn  : in  std_logic;    MEMCEN    : out std_logic;

MEMDEN    : out std_logic;

MEMADDR   : out std_logic_vector(15 downto 0);

MEMDOUT   : out std_logic_vector(15 downto 0);

MEMDIN    : in  std_logic_vector(15 downto 0)  );

end BRMbend;

architecture RTL of BRMbend is

type    T_MSTATE is   (IDLE,BREQ,READ1,WRITE1,IDLE1);

signal  MSTATE         : T_MSTATE;

signal  iBENDFAIL      : std_logic;   signal  iMEMREQn       : std_logic;

signal  iMEMACCn       : std_logic;  signal  iMEMCSn        : std_logic;

signal  iMEMWRn        : std_logic_vector(1 downto 0);

signal  iMEMRDn        : std_logic;   signal  iMEMCEN        : std_logic;

signal  iMEMDEN        : std_logic;

signal  MEMWRT         : std_logic_vector(1 downto 0);

signal  GNT            : std_logic;

signal  MEMTIMER       : UNSIGNED(7 downto 0);

signal  IDLETIMER      : UNSIGNED(2 downto 0);

signal  ACKVALUE       : INTEGER range 0 to 255;

signal  WAITVALUE      : INTEGER range 0 to 15;

signal  BENDPENDING    : std_logic;   signal  CPUACCESS      : std_logic;

signal  LASTCPUREQ     : std_logic;     signal  CPUNOREQ       : std_logic;

signal  iCPUWAIT       : std_logic;

begin

process(CLKSPD,CPUMEMEN)

variable muxsel : std_logic_vector(2 downto 0);

begin

muxsel := CPUMEMEN & CLKSPD;

case muxsel is

when "000"   => ACKVALUE <= 59;   -- 12MHz NO CPU memory access

when "001"   => ACKVALUE <= 81;   -- 16MHz NO CPU memory access

when "010"   => ACKVALUE <= 112;  -- 20MHz NO CPU memory access

when "011"   => ACKVALUE <= 150;  -- 24MHz NO CPU memory access

when "100"   => ACKVALUE <= 33;   -- 12MHz CPU memory access

when "101"   => ACKVALUE <= 45;   -- 16MHz CPU memory access

when "110"   => ACKVALUE <= 66;   -- 20MHz CPU memory access

when "111"   => ACKVALUE <= 91;   -- 24MHz CPU memory access

when others  => ACKVALUE <= 0;

end case;

if BETIMING=1 then  -- use alternate values

case muxsel is

when "000"   => ACKVALUE <= 48;  -- 12MHz NO CPU memory access

when "001"   => ACKVALUE <= 64;  -- 16MHz NO CPU memory access

when "010"   => ACKVALUE <= 85;  -- 20MHz NO CPU memory access

when "011"   => ACKVALUE <= 118; -- 24MHz NO CPU memory access

when "100"   => ACKVALUE <= 24;  -- 12MHz CPU memory access

when "101"   => ACKVALUE <= 32;  -- 16MHz CPU memory access

when "110"   => ACKVALUE <= 45;  -- 20MHz CPU memory access

when "111"   => ACKVALUE <= 66;  -- 24MHz CPU memory access

when others  => ACKVALUE <= 0;

end case;   end if;

end process;

process(CLKSPD,CPUMEMEN)

variable muxsel : std_logic_vector(1 downto 0);

begin

muxsel := CLKSPD;

case muxsel is

when "00"    => WAITVALUE <= 3+1;   --  12MHz

when "01"    => WAITVALUE <= 6+1;   --  16MHz

when "10"    => WAITVALUE <= 7+1;   --  20MHz

when "11"    => WAITVALUE <= 7+1;   --  24MHz

when others  => WAITVALUE <= 0;

end case;

if BETIMING=1 then -- use alternate values

case muxsel is

when "00"    => WAITVALUE <= 5+1;   --  12MHz

when "01"    => WAITVALUE <= 9+1;   --  16MHz

when "10"    => WAITVALUE <= 12+1;  --  20MHz

when "11"    => WAITVALUE <= 13+1;  --  24MHz

when others  => WAITVALUE <= 0;

end case;    end if;

end process;

GNT <= not MEMGNTn;

PMEMSM2:

process(CLK,RSTn)

begin

if RSTn='0' then

MSTATE      <= IDLE;    iMEMREQn    <= '1';

iMEMACCn    <= '1';    iMEMCSn     <= '1';

iMEMWRn     <= "11";    iMEMRDn     <= '1';

iMEMCEN     <= '0';     iMEMDEN     <= '0';

BENDDIN     <= ( others => '0');    CPUDIN      <= ( others => '0');

iCPUWAIT    <= '0';   BENDBUSY    <= '0';

iBENDFAIL   <= '0';   BENDPENDING <= '0';

CPUACCESS   <= '0';   LASTCPUREQ  <= '0';

CPUNOREQ    <= '0';    MEMADDR     <= ( others => '0');

MEMDOUT     <= ( others => '0');   MEMWRT      <= ( others => '0');

MEMTIMER    <= ( others => '0');    IDLETIMER   <= ( others => '1');

elsif CLK'event and CLK='1' then

LASTCPUREQ <= CPUREQ;    iBENDFAIL  <= '0';

if to_integer(MEMTIMER)=0 then

iBENDFAIL <= '1';   end if;

if BENDREQ='1' and CPUMEMEN='1' then

BENDBUSY    <= '1';    BENDPENDING <= '1';

end if;

if CPUREQ='0' then

CPUNOREQ <= '1';    iCPUWAIT <= '1';

end if;

MEMTIMER  <= MEMTIMER - 1;    IDLETIMER <= IDLETIMER -1;

case MSTATE is                                -- TX=1 memory reads

when IDLE =>  iMEMCSn   <= '1';

iMEMWRn   <= "11";    iMEMRDn   <= '1';

iMEMCEN   <= '0';    iMEMDEN   <= '0';

iMEMREQn  <= '1';   BENDBUSY  <= '0';

iBENDFAIL <= '0';                           -- not started so cant fail here

MEMTIMER  <= to_unsigned(ACKVALUE,8);       -- Memory already granted

if BENDREQ='1' or BENDPENDING='1' then

BENDPENDING <= '0';    BENDBUSY  <= '1';

iMEMREQn  <= not iMEMACCn;    MSTATE    <= BREQ;

CPUACCESS <= '0';    MEMADDR   <= BENDADDR;

MEMDOUT   <= BENDDOUT;

MEMWRT    <= BENDWRT & BENDWRT;

elsif CPUREQ='1' and CPUNOREQ='1' and CPUMEMEN='1' and iMEMACCn='1' then

CPUNOREQ  <= '0';    iMEMREQn  <= not iMEMACCn;

MSTATE    <= BREQ;    CPUACCESS <= '1';

MEMADDR   <= CPUADDR;    MEMDOUT   <= CPUDOUT;

MEMWRT    <= CPUWRT;

elsif IDLETIMER="000" then

iMEMACCn   <= '1';    end if;

when BREQ  => if iBENDFAIL='1' then

MSTATE  <= IDLE;

if CPUACCESS='1' then

iCPUWAIT <= '0';    end if;

elsif GNT='1' or iMEMACCn='0' then

iMEMACCn  <= '0';    iMEMCEN   <= '1';

iMEMCSn   <= '0';    iBENDFAIL <= '0';

MEMTIMER  <= to_unsigned(WAITVALUE,8);

if MEMWRT(0)='1' or MEMWRT(1)='1' then

MSTATE   <= WRITE1;

iMEMDEN  <= '1';

iMEMWRn(0)  <= not (MEMWRT(0) and not ASYNCIF);   -- set if SYNC mode

iMEMWRn(1)  <= not (MEMWRT(1) and not ASYNCIF);   -- set if SYNC mode

else

MSTATE  <= READ1;   iMEMRDn <= ASYNCIF;    -- set if SYNC mode

end if;     end if;

when WRITE1 => iMEMWRn  <= not MEMWRT;

iMEMREQn <= '1';

if ( MEMWAITn='1' or iBENDFAIL='1') and iMEMWRn/="11" then

iMEMWRn    <= "11";    MSTATE     <= IDLE1;

if CPUACCESS='1' then

iCPUWAIT <= '0';   end if;

iBENDFAIL  <= '0';                       -- complete so dont fail

end if;

when READ1  => iMEMREQn <= '1';

if ASYNCIF='1' then

iMEMRDn  <= '0';

if (MEMWAITn='1' or iBENDFAIL='1') and iMEMRDn='0' then

iMEMRDn    <= '1';

if CPUACCESS='1' then

CPUDIN   <= MEMDIN;      iCPUWAIT  <= '0';

else

BENDDIN  <= MEMDIN;    end if;

iBENDFAIL  <= '0';                      -- complete so dont fail

MSTATE     <= IDLE1;    end if;

else

if (MEMWAITn='1' or iBENDFAIL='1') then   -- if Wait delay input

iMEMRDn  <= '1';

end if;

if iMEMRDn='1' then

if CPUACCESS='0' then

BENDDIN  <= MEMDIN;

else

CPUDIN   <= MEMDIN;

iCPUWAIT <= '0';   end if;

iBENDFAIL  <= '0';                      -- complete so dont fail

MSTATE     <= IDLE1;

end if;   end if;

when IDLE1 => iMEMCSn  <= '1';

iMEMWRn  <= "11";   iMEMRDn  <= '1';

iMEMCEN  <= '0';    iMEMDEN  <= '0';

iMEMACCn <= not BENDBURST;

if CPUACCESS='0' then

BENDBUSY <= '0';  end if;

iBENDFAIL <= '0';                           -- complete so dont fail

IDLETIMER <= "111";          MSTATE    <= IDLE;

end case;

if iBENDFAIL='1' and CPUACCESS='0' then

BENDBUSY    <= '0';   BENDPENDING <= '0';

end if;      end if;

end process;

process(CLK,RSTn)

begin

if RSTn='0' then

CPUWAIT <= '1';

elsif CLK'event and CLK='0' then

CPUWAIT <= iCPUWAIT;

end if;

end process;

BENDFAIL <= iBENDFAIL;   MEMACCn <= iMEMACCn;

MEMREQn <= iMEMREQn;   MEMWRn  <= iMEMWRn;

MEMRDn  <= iMEMRDn;    MEMCEN  <= iMEMCEN;

MEMDEN  <= iMEMDEN;     MEMCSn  <= iMEMCSn;

end RTL;

17) BRMdec

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.NUMERIC_STD.all;

entity BRMdec is

port ( CLK       : in  std_logic;   RSTn      : in  std_logic;

CLKSPD    : in  std_logic_vector(1 downto 0);

DATAINP   : in  std_logic;   DATAINN   : in  std_logic;

TXENABLE  : in  std_logic;    FROMUS    : out std_logic;

STROBE    : out std_logic;

DATAOUT   : out std_logic_vector(15 downto 0);

CMDSTAT   : out std_logic;    GOTSYNC   : out std_logic;

PARERR    : out std_logic;    MANERR    : out std_logic;

ACTIVE    : out std_logic;    CONTIG    : out std_logic   );

end BRMdec;

architecture RTL of BRMdec is

signal  DINP       : std_logic;   signal  DINN       : std_logic;

signal  DIN        : std_logic;   signal  LASTDIN    : std_logic;

signal  DLLCNT     : UNSIGNED(3 downto 0);

signal  DLLLIM     : UNSIGNED(3 downto 0);

signal  DLLSMP     : UNSIGNED(3 downto 0);

signal  SAMPLE     : std_logic;   signal  SAMPLEX    : std_logic;

signal  SAMPDEL    : std_logic;   signal  SYNC       : std_logic;

signal  DECDATA    : std_logic;     signal  DECERR     : std_logic;

signal  SHIFTDATA  : std_logic_vector(5 downto 0);

signal  DATASYNC   : std_logic;     signal  PAR        : std_logic;

signal  WCNT       : UNSIGNED(4 downto 0);

signal  DWORD      : std_logic_vector(16 downto 0);

signal  FIRSTBIT   : std_logic;   signal  STARTED    : std_logic;

signal  WSTROBE    : std_logic;   signal  STBDONE    : std_logic;

signal  MANENCOK   : std_logic;

begin

process(CLKSPD)

begin

case CLKSPD is           -- FREQ/2 -1

when "00"   => DLLLIM <= "0101"; -- 12MHz

when "01"   => DLLLIM <= "0111"; -- 16MHz

when "10"   => DLLLIM <= "1001"; -- 20MHz

when others => DLLLIM <= "1011"; -- 24MHz

end case;

case CLKSPD is           -- Offset after edge to sample

when "00"   => DLLSMP <= "0001"; -- 12MHz

when "01"   => DLLSMP <= "0010"; -- 16MHz

when "10"   => DLLSMP <= "0011"; -- 20MHz

when others => DLLSMP <= "0100"; -- 24MHz

end case;

end process;

UDLL:

process(CLK,RSTn)

variable EDGE : std_logic;

begin

if RSTn='0' then

DIN     <= '0';   LASTDIN <= '0';

DLLCNT  <= "0000";   SAMPLEX <= '0';

DINP    <= '0';    DINN    <= '1';

elsif CLK'event and CLK='1' then

SAMPLEX <= '0';

DINP    <= DATAINP;                                  -- Sample incoming data stream

DINN    <= DATAINN;                                  -- Use Pos/Neg Data to clean signals

if DIN='1' and DINP='0' and DINN='1' then            -- Only change the data value if both

DIN <= '0';                                        -- switched

elsif DIN='0' and DINP='1' and DINN='0' then

DIN <= '1';   end if;

LASTDIN <= DIN;

EDGE   := DIN xor LASTDIN;

DLLCNT <= DLLCNT +1;                                 -- Divide by clkrate/2 counter

if DLLCNT=DLLSMP and EDGE='0' then                   -- Sample incoming data strobe

SAMPLEX <= '1';                                    -- if about tosync then dont sample

end if;

if EDGE='1' or DLLCNT=DLLLIM then                    -- if data edge resync clock divider

DLLCNT <="0000";

end if;  end if;

end process;

SAMPLE <= SAMPLEX and not (DIN xor LASTDIN);

UMAN:

process(CLK,RSTn)

begin

if RSTn='0' then

SYNC      <= '0';  DECDATA   <= '0';

DECERR    <= '0'; SHIFTDATA <= "000000";

DATASYNC  <= '0';   FIRSTBIT  <= '0';

SAMPDEL   <= '0';

elsif CLK'event and CLK='1' then

SYNC     <= '0';   DECDATA  <= '0';

DECERR   <= '0';DATASYNC <= '0';

SAMPDEL  <= SAMPLE;

if SAMPLE='1' then

SHIFTDATA <= SHIFTDATA(4 downto 0) & DIN;

end if;

if (SHIFTDATA="111000" or SHIFTDATA="000111") and SAMPDEL='1'then

DECDATA <= SHIFTDATA(5);  SYNC    <= '1';

end if;

if SYNC='1' then

FIRSTBIT<='0';

end if;

if SAMPDEL='1' then

if FIRSTBIT='0' then

FIRSTBIT <= '1';

else

FIRSTBIT <= '0';  DATASYNC <= '1';

case SHIFTDATA(1 downto 0) is

when "01"   => DECDATA <= '0';

when "10"   => DECDATA <= '1';

when others => DECERR  <= '1';

end case;  end if;   end if;   end if;

end process;

USTP:

process(CLK,RSTn)

begin

if RSTn='0' then

CMDSTAT    <= '0'; GOTSYNC    <= '0';

PAR        <= '0';  WCNT       <= "00000";

DWORD      <= ( others => '0');  WSTROBE    <= '0';

DATAOUT    <= ( others => '0');  MANENCOK   <= '0';

STARTED    <= '0';  CONTIG     <= '0';

STBDONE    <= '0';FROMUS     <= '0';

elsif CLK'event and CLK='1' then

GOTSYNC    <= '0';  WSTROBE    <= '0';

if SYNC='1' then

FROMUS   <= TXENABLE;  CMDSTAT  <= DECDATA;

PAR      <= '1';   WCNT     <= "00000";

STARTED  <= '1';            STBDONE  <= '0';

if WCNT="10011" and FIRSTBIT='0' then    -- SYNC contiguous to previous word

CONTIG   <= '1';

MANENCOK <= '1';                       -- contiguous words are okay

else

CONTIG   <= '0';

MANENCOK <= DECDATA;                   -- OK if a CW, DW's must be contiguous

end if;   else

if DATASYNC='1' and STARTED='1' then     -- Datasync is a free running 1us pulse

WCNT  <= WCNT+1;                       -- WCNT stops at 10101

DWORD <= DWORD(15 downto 0) & DECDATA;

PAR   <= PAR xor DECDATA;

if DECERR='1' then   MANENCOK <= '0';

if WCNT(4 downto 1)="0000" then      -- If either bits 0 or 1 are corrupt then assume it is

STARTED  <= '0';                   -- noise and terminate the decoder

CONTIG   <= '0';

end if;  end if;

if MANENCOK='1' and DECERR='0' and WCNT="00001" then

GOTSYNC  <= '1';                     -- SYNC and 2 databits validated

end if;  end if;

if WCNT="10001" and STBDONE='0' then

WSTROBE  <= '1';  STBDONE  <= '1';

DATAOUT  <= DWORD(16 downto 1);

end if;

if WCNT="10100" then               -- Goes In active 3us after Parity Bit

STARTED <= '0';                  -- No valid SYNC by this time

end if;                            -- If valid SYNC STARTED stays active

end if;  end if;

end process;

PARERR <= PAR;

MANERR <= not MANENCOK;

STROBE <= WSTROBE;

ACTIVE <= STARTED;

end RTL;

18) BRMenc

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.NUMERIC_STD.all;

entity BRMenc is

port ( CLK        : in  std_logic;     RSTn       : in  std_logic;

CLKSPD     : in  std_logic_vector(1 downto 0);

STROBE     : in  std_logic;       ABORTX     : in  std_logic;

DATAIN     : in  std_logic_vector(15 downto 0);

CMDSTAT    : in  std_logic;   TXENABLE   : out std_logic;

TXBUSY     : out std_logic;       TXREADY    : out std_logic;

DOUT       : out std_logic;          TXENA      : in  std_logic;

RXSTROBEA  : in  std_logic;   RXERRORA   : in  std_logic;

RXDATAA    : in  std_logic_vector(15 downto 0);

RXDATAAS   : in  std_logic;     LOOPFAILA  : out std_logic;

TXENB      : in  std_logic;         RXSTROBEB  : in  std_logic;

RXERRORB   : in  std_logic;

RXDATAB    : in  std_logic_vector(15 downto 0);

RXDATABS   : in  std_logic;     LOOPFAILB  : out std_logic;

CLRERRORS  : in  std_logic;      FSM_ERROR  : out std_logic  );

end BRMenc;

architecture RTL of BRMenc is

type TSTATE is (IDLE,SYNC1,SYNC2,DOUT1,DOUT2,NDATA);

signal  STATE     : TSTATE;

signal  CLKCNT    : unsigned(5 downto 0);

signal  DATATEMP  : std_logic_vector(15 downto 0);

signal  COMPDATA  : std_logic_vector(15 downto 0);

signal  DATACNT   : unsigned(4 downto 0);

signal  READY     : std_logic;    signal  BUSY      : std_logic;

signal  PAR       : std_logic;    signal  xDOUT     : std_logic;

signal  CHECKA    : std_logic;    signal  CHECKB    : std_logic;

signal  COMPSYNC  : std_logic;

signal  HOLDDATA  : std_logic_vector(15 downto 0);  -- Extra in BRM

signal  HOLDCMDS  : std_logic;                      -- Extra in BRM

signal  CLKVALUE1 : unsigned(5 downto 0);

signal  CLKVALUE2 : unsigned(5 downto 0);

signal  CLKVALUE3 : unsigned(5 downto 0);

signal  CLKVALUE4 : unsigned(5 downto 0);

signal  CLKVALUE5 : unsigned(5 downto 0);

signal  CLKVALUE6 : unsigned(5 downto 0);

begin

process(CLKSPD)

begin

case CLKSPD is

when "00"   => CLKVALUE1 <= "010001"; --12MHz        -- FREQ * 1.5 -1

CLKVALUE2 <= "010000";                -- FREQ * 1.5 -1

CLKVALUE3 <= "001011";                -- FREQ -1

CLKVALUE4 <= "001010";                -- FREQ -2

CLKVALUE5 <= "000101";                -- FREQ/2 -1

CLKVALUE6 <= "100011";                -- FREQ * 2.4 + 6

when "01"   => CLKVALUE1 <= "010111"; --16MHz

CLKVALUE2 <= "010110";

CLKVALUE3 <= "001111";

CLKVALUE4 <= "001110";

CLKVALUE5 <= "000111";

CLKVALUE6 <= "101101";

when "10"   => CLKVALUE1 <= "011101"; --20MHz

CLKVALUE2 <= "011100";

CLKVALUE3 <= "010011";

CLKVALUE4 <= "010010";

CLKVALUE5 <= "001001";

CLKVALUE6 <= "110110";

when others => CLKVALUE1 <= "100011"; --24MHz

CLKVALUE2 <= "100010";

CLKVALUE3 <= "010111";

CLKVALUE4 <= "010110";

CLKVALUE5 <= "001011";

CLKVALUE6 <= "111111";

end case;  end process;

UTX:

process(CLK,RSTn)

begin

if RSTn='0' then

CLKCNT    <= "000000";  STATE     <= IDLE;

xDOUT     <= '0';   BUSY      <= '0';

READY     <= '1';   DATATEMP  <= (others => '0');

DATACNT   <= "00000";   PAR       <= '0';

CHECKA    <= '0';   CHECKB    <= '0';

LOOPFAILA <= '0';   LOOPFAILB <= '0';

TXENABLE  <= '0';

COMPDATA  <= (others => '0');

COMPSYNC  <= '0';   FSM_ERROR <= '0';

HOLDDATA  <= (others => '0');

HOLDCMDS  <= '0';

elsif CLK'event and CLK='1' then

FSM_ERROR <= '0';

if STROBE='1' then

READY    <= '0';   HOLDDATA <= DATAIN;

HOLDCMDS <= CMDSTAT;

end if;

case STATE is

when IDLE  => CLKCNT   <= CLKCNT - 1;

TXENABLE <= '0';  BUSY     <= '0';

if  READY='0' then

STATE    <= SYNC1;    xDOUT    <= HOLDCMDS;

CLKCNT   <= CLKVALUE1;   BUSY     <= '1';

TXENABLE <= '1';    DATATEMP <= HOLDDATA;

DATACNT  <= "00000";   PAR      <= '1';

READY    <= '1';

end if;

if CLKCNT="000000" then

if CHECKA='1' then           -- No RX data after 2.4us, error

LOOPFAILA <= '1';          -- Previous data not received

end if;

if CHECKB='1' then

LOOPFAILB <= '1';          -- Previous data not received

end if;

CHECKA <= '0';  CHECKB <= '0';

end if;

when SYNC1 => CLKCNT   <= CLKCNT - 1;

if CLKCNT="000000" then

xDOUT   <= not xDOUT;   STATE   <= SYNC2;

CLKCNT  <= CLKVALUE2;

end if;

when SYNC2 => CLKCNT <= CLKCNT - 1;

if CLKCNT="000000" then

STATE  <= NDATA;

if CHECKA='1' then

LOOPFAILA <= '1';             -- Previous data not received

end if;                         -- end of SYNC pattern 3us later

if CHECKB='1' then

LOOPFAILB <= '1';             -- Previous data not received

end if;

COMPDATA <= DATATEMP;           -- Update data for compare

COMPSYNC <= not xDOUT;          -- Also the SYNC value

CHECKA   <= TXENA;              CHECKB   <= TXENB;

end if;

when DOUT2 => CLKCNT <= CLKCNT -1;

if CLKCNT="000000" then

if DATACNT="10001" then         -- Back to IDLE

STATE    <= IDLE;   CLKCNT   <= CLKVALUE6;

else

STATE <= NDATA;

end if;   end if;

when NDATA => if DATACNT(4)='1' then

xDOUT <= PAR;

else

xDOUT <= DATATEMP(15);

end if;

PAR      <= PAR xor DATATEMP(15);

DATATEMP <= DATATEMP(14 downto 0) & '0';

DATACNT  <= DATACNT +1;

CLKCNT   <= CLKVALUE4;   STATE    <= DOUT1;

when DOUT1 => CLKCNT   <= CLKCNT -1;

if CLKCNT=CLKVALUE5 then

xDOUT  <= not xDOUT;   STATE  <= DOUT2;

end if;

end case;

if RXSTROBEA='1' and CHECKA='1' then

CHECKA <= '0';

if COMPDATA/=RXDATAA or COMPSYNC/=RXDATAAS or RXERRORA='1' then

LOOPFAILA <= '1';

end if;   end if;

if RXSTROBEB='1' and CHECKB='1' then

CHECKB <= '0';

if COMPDATA/=RXDATAB or COMPSYNC/=RXDATABS or RXERRORB='1' then

LOOPFAILB <= '1';

end if;   end if;

if CLRERRORS='1' then

LOOPFAILA <= '0';  LOOPFAILB <= '0';

end if;

if ABORTX='1' then

STATE    <= IDLE;  READY    <= '1';

CLKCNT   <= "000000";  CHECKA   <= '0';

CHECKB   <= '0';  TXENABLE <= '0';

BUSY     <= '0';

end if; end if;end process;

TXREADY <= READY;

TXBUSY  <= BUSY or CHECKA or CHECKB;

DOUT <= xDOUT;

end RTL;

19) BRMMT

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.NUMERIC_STD.all;

use work.convert.all;

entity BRMMT is

generic ( MTENABLE : INTEGER range 0 to 1    :=  1 );

port ( CLK            : in  std_logic;   RSTn           : in  std_logic;

-- Control Signals

BCASTEN        : in  std_logic;   MIL1553A       : in  std_logic;

EXPECTSW       : in  std_logic;

-- Decoder Interface

DECA_STROBE    : in  std_logic;

DECA_DATAOUT   : in  std_logic_vector(15 downto 0);

DECA_CMDSTAT   : in  std_logic;   DECA_GOTSYNC   : in  std_logic;

DECA_PARERR    : in  std_logic;  DECA_MANERR    : in  std_logic;

DECA_ACTIVE    : in  std_logic;  DECA_CONTIG    : in  std_logic;

DECA_FROMUS    : in  std_logic;

DECB_STROBE    : in  std_logic;

DECB_DATAOUT   : in  std_logic_vector(15 downto 0);

DECB_CMDSTAT   : in  std_logic;  DECB_GOTSYNC   : in  std_logic;

DECB_PARERR    : in  std_logic;  DECB_MANERR    : in  std_logic;

DECB_ACTIVE    : in  std_logic;  DECB_CONTIG    : in  std_logic;

DECB_FROMUS    : in  std_logic;

-- Interface to/from Protocol Logic

MTMSG_BUSSEL   : out std_logic;  MTMSG_STBALL   : out std_logic;

MTMSG_START    : out std_logic;

MTMSG_CMDWORD  : out std_logic_vector(23 downto 0));

end BRMMT;

architecture RTL of BRMMT is

-- Bus Selection and Multiplexing Signals

signal  iBUSSEL        : std_logic;   signal  DEC_STROBE     : std_logic;

signal  DEC_DATAOUT    : std_logic_vector(15 downto 0);

signal  DEC_CMDSTAT    : std_logic;   signal  DEC_GOTSYNC    : std_logic;

signal  DEC_PARERR     : std_logic;   signal  DEC_MANERR     : std_logic;

signal  DEC_ACTIVE     : std_logic;   signal  DEC_CONTIG     : std_logic;

signal  DEC_FROMUS     : std_logic;   signal  DATASYNC       : std_logic;

-- Command Word Signals

signal  CW_STBALL       : std_logic;   signal  PENDING         : std_logic;

signal  STAMTMSG        : std_logic;

signal  CW_RTADDR       : std_logic_vector(4 downto 0);

signal  CW_TX           : std_logic;

signal  CW_SUBADR       : std_logic_vector(4 downto 0);

signal  CW_MCWC         : std_logic_vector(4 downto 0);

signal  CW_COUNT        : std_logic_vector(5 downto 0);

signal  CW_BCAST        : std_logic;   signal  CW_MCODE        : std_logic;

signal  CW_DATA         : std_logic;   signal  SW_STBOK        : std_logic;

signal  DW_STBOK        : std_logic;   signal  BURST           : std_logic;

-- Misc Signals

signal  Logic0          : std_logic;    signal  Logic1          : std_logic;

begin

-- Misc

Logic0 <= '0';   Logic1 <= '1';

-- Decoder Multiplexing logic

DEC_STROBE   <=  DECA_STROBE  when iBUSSEL='0' else DECB_STROBE;

DEC_DATAOUT  <=  DECA_DATAOUT when iBUSSEL='0' else DECB_DATAOUT;

DEC_CMDSTAT  <=  DECA_CMDSTAT when iBUSSEL='0' else DECB_CMDSTAT;

DEC_GOTSYNC  <=  DECA_GOTSYNC when iBUSSEL='0' else DECB_GOTSYNC;

DEC_PARERR   <=  DECA_PARERR  when iBUSSEL='0' else DECB_PARERR;

DEC_MANERR   <=  DECA_MANERR  when iBUSSEL='0' else DECB_MANERR;

DEC_ACTIVE   <=  DECA_ACTIVE  when iBUSSEL='0' else DECB_ACTIVE;

DEC_CONTIG   <=  DECA_CONTIG  when iBUSSEL='0' else DECB_CONTIG;

DEC_FROMUS   <=  DECA_FROMUS  when iBUSSEL='0' else DECB_FROMUS;

DATASYNC     <= DEC_GOTSYNC and not DEC_CMDSTAT;

BURST        <= DEC_CONTIG;

PCWD:

process(CLK,RSTn)

variable RTPAROK    : std_logic;  variable CWOKA      : std_logic;

variable CWOKB      : std_logic;   variable CWSTB      : std_logic;

variable ADDRA      : std_logic;   variable ADDRB      : std_logic;

variable SYNCS      : std_logic_vector(1 downto 0);

begin

if RSTn='0' then

CW_STBALL     <= '0';   iBUSSEL       <= '0';    -- default bus

DW_STBOK      <= '0';    SW_STBOK      <= '0';

elsif CLK'event and CLK='1' then

DW_STBOK    <= DEC_STROBE and not DEC_CMDSTAT and not DEC_MANERR and not DEC_PARERR and BURST;

SW_STBOK    <= DEC_STROBE and     DEC_CMDSTAT and not DEC_MANERR and not DEC_PARERR;

CWOKA := DECA_STROBE and DECA_CMDSTAT

and not DECA_PARERR and not DECA_MANERR;

CWOKB := DECB_STROBE and DECB_CMDSTAT

and not DECB_PARERR and not DECB_MANERR;

CWSTB := not EXPECTSW and (  (CWOKA and not DECA_CONTIG)

or (CWOKB and not DECB_CONTIG) );

CW_STBALL <= '0';

-- Switch the Buses when a Valid Command Received

if CWSTB='1' then

CW_STBALL <= '1';   SYNCS(1) := CWOKB;

SYNCS(0) := CWOKA;

case SYNCS is

when "01"   => iBUSSEL <= '0';  --Bus A

when "10"   => iBUSSEL <= '1';  --Bus B

when others =>

end case;  end if;   end if;

end process;

MTMSG_STBALL   <= CW_STBALL;

-- Command Word Register & Decode

PCW:

process(CLK,RSTn)

begin

if RSTn='0' then

CW_RTADDR <= "00000";   CW_TX     <= '0';

CW_SUBADR <= "00000";   CW_MCWC   <= "00000";

CW_COUNT  <= "000000";   CW_BCAST  <= '0';

CW_MCODE  <= '0';   STAMTMSG  <= '0';

PENDING   <= '0';

elsif CLK'event and CLK='1' then

STAMTMSG  <= '0';

if CW_STBALL='1' then

PENDING   <= '1';

CW_RTADDR <= DEC_DATAOUT(15 downto 11);

CW_TX     <= DEC_DATAOUT(10);

CW_SUBADR <= DEC_DATAOUT(9 downto 5);

CW_MCWC   <= DEC_DATAOUT(4 downto 0);

CW_BCAST  <= to_logic(DEC_DATAOUT(15 downto 11)="11111") and BCASTEN;

if (DEC_DATAOUT(9 downto 5)="00000") then

CW_MCODE  <= '1';

CW_COUNT  <= "00000" & DEC_DATAOUT(4);

elsif (MIL1553A='0' and DEC_DATAOUT(9 downto 5)="11111") then

CW_MCODE  <= '1';   CW_COUNT  <= "000000";

else

CW_MCODE  <= '0';

CW_COUNT  <= to_logic(DEC_DATAOUT(4 downto 0)="00000" ) & DEC_DATAOUT(4 downto 0);

end if;   end if;

if PENDING='1' and (DEC_ACTIVE='0' or DEC_GOTSYNC='1') then

STAMTMSG  <= '1';  PENDING   <= '0';

end if;  end if;

end process;

MTMSG_CMDWORD <= CW_BCAST & CW_MCODE & CW_COUNT & CW_RTADDR & CW_TX & CW_SUBADR & CW_MCWC

when MTENABLE=1

else ( others => '0');

MTMSG_BUSSEL  <= iBUSSEL   and to_logic(MTENABLE);

MTMSG_STBALL  <= CW_STBALL and to_logic(MTENABLE);

MTMSG_START   <= STAMTMSG  and to_logic(MTENABLE);

end RTL;

20)BRMpack

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.NUMERIC_STD.all;

package convert is

subtype WORD      is std_logic_vector (15 downto 0);

type WORD_ARRAY   is array ( INTEGER range <>) of WORD;

function to_std_logic( x: UNSIGNED ) return std_logic_vector;

function to_unsigned( x: std_logic_vector ) return UNSIGNED;

function to_logic( x: BOOLEAN) return std_logic;

function to_logic( x: INTEGER) return std_logic;

function to_integer( x: BOOLEAN) return integer;

end convert;

package body convert is

-- Handle UNSIGNED to std_logic_vector conversions

function to_std_logic( x: UNSIGNED ) return std_logic_vector is

variable y: std_logic_vector(x'range);

begin

for i in x'range loop

y(i) := x(i);

end loop;

return(y);

end to_std_logic;

function to_unsigned( x: std_logic_vector ) return UNSIGNED is

variable y: UNSIGNED(x'range);

begin

for i in x'range loop

y(i) := x(i);

end loop;

return(y);

end to_unsigned;

function to_logic( x: BOOLEAN) return std_logic is

variable y : std_logic;

begin

if x then y:='1';

else y:='0';

end if;

return(y);

end to_logic;

function to_logic( x: INTEGER) return std_logic is

variable y : std_logic;

begin

y:='0';

if x=1 then

y:='1';  end if;

return(y);

end to_logic;

function to_integer( x: BOOLEAN) return integer is

variable y : integer;

begin

if x then y:=1;

else y:=0;

end if;

return(y);

end to_integer;

end convert;

21) BRMPC

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.NUMERIC_STD.all;

use work.convert.all;

entity BRMPC is

generic ( BCENABLE : INTEGER range 0 to 1    :=  1;

RTENABLE : INTEGER range 0 to 1    :=  1;

MTENABLE : INTEGER range 0 to 1    :=  1;

LEGREGS  : INTEGER range 0 to 5    :=  1;

ENHANCED : INTEGER range 0 to 1    :=  1;

INITFREQ : INTEGER range 12 to 24  := 24 );

port ( CLK            : in  std_logic;    TCLK           : in  std_logic;

RSTINn         : in  std_logic;     RSTOUTn        : out std_logic;

CLKSPD         : out std_logic_vector(1 downto 0);

LOCKn          : in  std_logic;

RTADDRIN       : in  std_logic_vector(4 downto 0);

RTADDRPIN      : in  std_logic;   RTADERR        : out std_logic;

MSELIN         : in  std_logic_vector(1 downto 0);

SSYSFn         : in  std_logic;    ABSTDIN        : in  std_logic;

SHUTDOWNA      : out std_logic;    SHUTDOWNB      : out std_logic;

RXBUSENA       : out std_logic;    RXBUSENB       : out std_logic;

BUSSEL         : out std_logic;    DECA_FROMUS    : in  std_logic;

DECA_STROBE    : in  std_logic;

DECA_DATAOUT   : in  std_logic_vector(15 downto 0);

DECA_CMDSTAT   : in  std_logic;    DECA_GOTSYNC   : in  std_logic;

DECA_PARERR    : in  std_logic;    DECA_MANERR    : in  std_logic;

DECA_ACTIVE    : in  std_logic;    DECA_CONTIG    : in  std_logic;

DECB_FROMUS    : in  std_logic;    DECB_STROBE    : in  std_logic;

DECB_DATAOUT   : in  std_logic_vector(15 downto 0);

DECB_CMDSTAT   : in  std_logic;    DECB_GOTSYNC   : in  std_logic;

DECB_PARERR    : in  std_logic;    DECB_MANERR    : in  std_logic;

DECB_ACTIVE    : in  std_logic;    DECB_CONTIG    : in  std_logic;

LOOPFAILA      : in  std_logic;    LOOPFAILB      : in  std_logic;

ENCSTROBE      : out std_logic;    ENCABORT       : out std_logic;

ENCDATA        : out std_logic_vector(15 downto 0);

ENCCMDSTAT     : out std_logic;       ENCREADY       : in  std_logic;

ENCBUSY        : in  std_logic;    ENCCLEAR       : out std_logic;

TXTIMEOUT      : in  std_logic;    BENDREQ        : out std_logic;

BENDBURST      : out std_logic;    BENDWRT        : out std_logic;

BENDBUSY       : in  std_logic;    BENDFAIL       : in  std_logic;

BENDADDR       : out std_logic_vector(15 downto 0);

BENDDIN        : in  std_logic_vector(15 downto 0);

BENDDOUT       : out std_logic_vector(15 downto 0);

CPUWR          : in  std_logic_vector(1 downto 0);

CPURD          : in  std_logic;

CPUADDR        : in  std_logic_vector(5 downto 0);

CPUDIN         : in  std_logic_vector(15 downto 0);

CPUDOUT        : out std_logic_vector(15 downto 0);

MEMFAIL        : out std_logic;    INTLEVEL       : in  std_logic;

INTOUTH        : out std_logic;    INTOUTM        : out std_logic;

INTACKH        : in  std_logic;    INTACKM        : in  std_logic;

ACTIVE         : out std_logic;    BUSY           : out std_logic;

OPMODE         : out std_logic_vector(1 downto 0);

-- External Status Signals

MSGSTART       : out std_logic;    CMDSYNC        : out std_logic;

SYNCNOW        : out std_logic;    BUSRESET       : out std_logic;

-- Command Legalization Interface

CMDVAL         : out std_logic_vector(11 downto 0);

CMDOK          : in  std_logic;    CMDSTB         : out std_logic;

CMDOKOUT       : out std_logic;

-- Misc

LOOPBACK       : out std_logic );

end BRMPC;

architecture RTL of BRMPC is

component BRMREG

generic ( BCENABLE : INTEGER range 0 to 1   :=  1;

RTENABLE : INTEGER range 0 to 1   :=  1;

MTENABLE : INTEGER range 0 to 1   :=  1;

LEGREGS  : INTEGER range 0 to 5   :=  1;

ENHANCED : INTEGER range 0 to 1   :=  1;

INITFREQ : INTEGER range 12 to 24 := 24

);

port ( CLK       : in  std_logic;    RSTn      : in  std_logic;

CPUSEL    : in  std_logic;

CPUADDR   : in  std_logic_vector( 3 downto 0);

CPUADDRR  : in  std_logic_vector( 3 downto 0);

CPUDIN    : in  std_logic_vector(15 downto 0);

CPUDOUT   : out std_logic_vector(15 downto 0);

CPUWR     : in  std_logic_vector( 1 downto 0);

CPURD     : in  std_logic;   SMR0_STEX : in  std_logic;

CMDSTB    : in  std_logic;

BITSELECT : in  std_logic_vector(7 downto 0);

CMDOKREG  : out std_logic );

end component;

component  BRMRT

generic ( BCENABLE : INTEGER range 0 to 1   :=  1;

RTENABLE : INTEGER range 0 to 1   :=  1;

MTENABLE : INTEGER range 0 to 1   :=  1;

LEGREGS  : INTEGER range 0 to 5   :=  1;

ENHANCED : INTEGER range 0 to 1   :=  1;

INITFREQ : INTEGER range 12 to 24 := 24);

port ( CLK            : in  std_logic;

CLKSPD         : in  std_logic_vector(1 downto 0);

RSTn           : in  std_logic;    RSTREG         : out std_logic;

RTMODE         : in  std_logic;

RTADDR         : in  std_logic_vector(4 downto 0);

RTADDRP        : in  std_logic;   RTADERR        : out std_logic;

BCASTEN        : in  std_logic;

MIL1553A       : in  std_logic;

SMR0_EMODE     : in  std_logic_vector(1 downto 0);

SMR9_SB        : in  std_logic_vector(10 to 19);

DBA            : in  std_logic;   XMTSW          : in  std_logic;

SSYSFn         : in  std_logic;

CHAEN          : in  std_logic;

CHBEN          : in  std_logic;

RTBUSY         : out std_logic;

BUSSEL         : out std_logic;

BUSSELALL      : in  std_logic;

TXTIMEOUT      : in  std_logic;

DECA_FROMUS    : in  std_logic;

DECA_STROBE    : in  std_logic;

DECA_DATAOUT   : in  std_logic_vector(15 downto 0);

DECA_CMDSTAT   : in  std_logic;

DECA_GOTSYNC   : in  std_logic;

DECA_PARERR    : in  std_logic;

DECA_MANERR    : in  std_logic;

DECA_ACTIVE    : in  std_logic;

DECA_CONTIG    : in  std_logic;

DECB_FROMUS    : in  std_logic;

DECB_STROBE    : in  std_logic;

DECB_DATAOUT   : in  std_logic_vector(15 downto 0);

DECB_CMDSTAT   : in  std_logic;

DECB_GOTSYNC   : in  std_logic;

DECB_PARERR    : in  std_logic;

DECB_MANERR    : in  std_logic;

DECB_ACTIVE    : in  std_logic;

DECB_CONTIG    : in  std_logic;

LOOPFAILA      : in  std_logic;

LOOPFAILB      : in  std_logic;

SHUTDOWNRTA    : out std_logic;

SHUTDOWNRTB    : out std_logic;

LATCHSW        : in  std_logic;

WCNTTO         : in  std_logic;

WCNTOVR        : in  std_logic;

CMDDONE        : in  std_logic;

RTSMBUSY       : in  std_logic;

BENDFAIL       : in  std_logic;

RTMSG_STBME    : out std_logic;

RTMSG_START    : out std_logic;

RTMSG_CMDWORD  : out std_logic_vector(23 downto 0);

RTMSG_LEGAL    : out std_logic;

RTMSG_RTRT     : out std_logic;

RTMSG_RTRTOK   : out std_logic;

RTMSG_OKAY     : out std_logic;

RTMSG_GOTSW    : out std_logic;

CLR_SRQ        : out std_logic;

TIMER57US      : out std_logic;

MSGINFO        : out std_logic_vector(15 downto 0);

BITWORD        : in  std_logic_vector(15 downto 0);

TXT_CMDSTAT    : in  std_logic;

BENDDIN        : in  std_logic_vector(15 downto 0);

TXT_DATAIN     : out std_logic_vector(15 downto 0);

CMDSYNC        : out std_logic;

SYNCNOW        : out std_logic;

BITSELECT      : out std_logic_vector(7 downto 0);

CMDOKREG       : in  std_logic;

USEXTOK        : in  std_logic;

CMDVAL         : out std_logic_vector(11 downto 0);

CMDSTB         : out std_logic;

CMDOK          : in  std_logic;

CMDOKOUT       : out std_logic

);

end component;

component BRMMT

generic ( MTENABLE : INTEGER range 0 to 1    :=  1 );

port ( CLK            : in  std_logic;

RSTn           : in  std_logic;

BCASTEN        : in  std_logic;

MIL1553A       : in  std_logic;

EXPECTSW       : in  std_logic;

DECA_STROBE    : in  std_logic;

DECA_DATAOUT   : in  std_logic_vector(15 downto 0);

DECA_CMDSTAT   : in  std_logic;

DECA_GOTSYNC   : in  std_logic;

DECA_PARERR    : in  std_logic;

DECA_MANERR    : in  std_logic;

DECA_ACTIVE    : in  std_logic;

DECA_CONTIG    : in  std_logic;

DECA_FROMUS    : in  std_logic;

DECB_STROBE    : in  std_logic;

DECB_DATAOUT   : in  std_logic_vector(15 downto 0);

DECB_CMDSTAT   : in  std_logic;

DECB_GOTSYNC   : in  std_logic;

DECB_PARERR    : in  std_logic;

DECB_MANERR    : in  std_logic;

DECB_ACTIVE    : in  std_logic;

DECB_CONTIG    : in  std_logic;

DECB_FROMUS    : in  std_logic;

MTMSG_BUSSEL   : out std_logic;

MTMSG_STBALL   : out std_logic;

MTMSG_START    : out std_logic;

MTMSG_CMDWORD  : out std_logic_vector(23 downto 0)

);

end component;

component GLOBALBUFF

port ( A : in  std_logic;

Y : out std_logic

);

end component;

-- Core Operating Mode

signal iRTMODE      : std_logic;

signal iBCMODE      : std_logic;

signal iMTMODE      : std_logic;

-- Clock Dividers etc

signal CLKFREQ      : std_logic_vector(1 downto 0);

signal CLKSTB       : std_logic;

signal CLKSTB1US    : std_logic;

signal SCALE        : UNSIGNED(4 downto 0);

signal SCALER1      : UNSIGNED(4 downto 0);

signal SCALER2      : UNSIGNED(5 downto 0);

signal IMGVALUE     : UNSIGNED(4 downto 0);

signal RESPVALUE    : UNSIGNED(4 downto 0);

-- Registered CPU interface

signal CPUDINR      : std_logic_vector(15 downto 0);

signal CPUADDRR     : std_logic_vector( 5 downto 0);

signal CPUWRR       : std_logic_vector( 1 downto 0);

signal CPUWRRD      : std_logic_vector( 1 downto 0);

signal CPURDR       : std_logic;

signal CPUSTB       : std_logic;

signal CPUCLRINT    : std_logic;

signal CPUDOUT1     : std_logic_vector(15 downto 0);

signal CPUDOUT2     : std_logic_vector(15 downto 0);

-- CPU Registers

signal FORCEORUN    : std_logic;

signal ACT_ENHANCE  : std_logic;

-- Internal Status Flags etc

signal SW_ERROR     : std_logic;

signal SW_TFLAG     : std_logic;

signal SW_DBA       : std_logic;

signal SW_SSFLAG    : std_logic;

signal SW_BUSY      : std_logic;

signal SW_BCAST     : std_logic;

signal SW_SREQ      : std_logic;

signal SW_INSTRU    : std_logic;

signal SW_MSGERR    : std_logic;

signal RT_NORESP    : std_logic;

signal RT_ERROR     : std_logic;

signal EXPWCNT      : UNSIGNED(5 downto 0);

signal MSG_ERROR    : std_logic;

signal MSG_ERRORCC  : std_logic;

signal ABORT        : std_logic;

--Protocol BCSTATE Machine

signal DECSTB       : std_logic;

signal DECSTBDEL1   : std_logic;

signal DECSTBOKSW   : std_logic;

signal DECSTBOKSWA  : std_logic;

signal DECSTBOKDW   : std_logic;

signal DECSTBOKCW   : std_logic;

type TBCSTATE is ( BCIDLE , BCILIST, BCGETOPCODE ,BCLOADMMT, BCWAITMMT, BCGOTO, BCCALL,

BCLOADMFT, BCGETBR,

BCSTARTM, BCGETCW1, BCTXCW, BCCHECKCW, BCGETCW2, BCGETDPTR, BCWAITSW1, BCWRDATA,

BCRDDATA1, BCRDDATA2, BCWAITSW2, BCRETRY1,

BCWAITBQ, BCCHECK, BCWRTTSW, BCMSGDONE, BCDECOPCODE,

BCALLDONE, BCGENINT );

signal BCSTATE      : TBCSTATE;

signal CCFLAG       : std_logic;

signal RTTIMER      : UNSIGNED(4 downto 0);

signal BCAST        : std_logic;

signal MSGTYPE      : std_logic_vector(2 downto 0);

signal CWRT1        : std_logic_vector(4 downto 0);

signal CWRT2        : std_logic_vector(4 downto 0);

signal GOTSW1       : std_logic;

signal GOTSW2       : std_logic;

signal RTTIMEOUT    : std_logic;

signal IMGDONE      : std_logic;

signal WORDCOUNT    : UNSIGNED(5 downto 0);

signal iENCSTROBE   : std_logic;

signal iENCCMDSTAT  : std_logic;

signal iBENDREQ     : std_logic;

signal iBENDWRT     : std_logic;

signal iBENDBURST   : std_logic;

signal iBUSY        : std_logic;

signal iLOOPBACK    : std_logic;

-- RT Signals

signal  DECSTROBE     : std_logic;

signal  DECDATA       : std_logic_vector(15 downto 0);

signal  DECCMDSTAT    : std_logic;

signal  DECGOTSYNC    : std_logic;

signal  DECPARERR     : std_logic;

signal  DECMANERR     : std_logic;

signal  DECACTIVE     : std_logic;

signal  DECCONTIG     : std_logic;

signal  DECFROMUS     : std_logic;

signal  CW_BCAST      : std_logic;

signal  CW_DATA       : std_logic;

signal  CW_MCODE      : std_logic;

signal  CW_COUNT      : std_logic_vector(5 downto 0);

signal  CW_TX         : std_logic;

signal  CW_SUBADDR    : std_logic_vector(4 downto 0);

signal  CW_MCWC       : std_logic_vector(4 downto 0);

signal  CW_STBME      : std_logic;

signal  RTSTART       : std_logic;

type    T_RTSTATE is  (RTIDLE,RTINIT1,RTGETD1,RTGETD2,RTGETD3,RTGETD4,RTGETD5,

RTINIT2,RTREAD1,RTREAD2,RTWRITE1,

RTTXSTAT,RTSTATRX,

RTPUTCW,RTPUTTT,RTPUTD0,RTPUTD1,RTPUTD2,RTPUTD3,

RTMSGSTAT,RTMSGDONE,RTALLDONE);

signal  RTSTATE       : T_RTSTATE;

signal  PCONBUSY      : std_logic;

signal  RTCMDDONE     : std_logic;

signal  RTRTRT        : std_logic;

signal  RTRTOK        : std_logic;

signal  LEGAL         : std_logic;

signal  RTADDRERR     : std_logic;

signal  RTBUSY        : std_logic;

signal  iBUSSEL       : std_logic;

signal  RTBUSSEL      : std_logic;

signal  LATCHSW        :  std_logic;

signal  WCNTOVR        :  std_logic;

signal  WCNTTO         :  std_logic;

signal  RTMSG_STBME    :  std_logic;

signal  RTMSG_START    :  std_logic;

signal  RTMSG_CMDWORD  :  std_logic_vector(23 downto 0);

signal  RTMSG_RTRT     :  std_logic;

signal  RTMSG_RTRTOK   :  std_logic;

signal  RTMSG_LEGAL    :  std_logic;

signal  RTMSG_OKAY     :  std_logic;

signal  GOTSW          :  std_logic;

signal  TIMER57US      :  std_logic;

signal  SHUTDOWNRTA    :  std_logic;

signal  SHUTDOWNRTB    :  std_logic;

signal  RSTn           :  std_logic;

signal  RSTREG         :  std_logic;

signal  iBUSRESET      :  std_logic;

signal  CLRERR         :  std_logic;

signal  SWRESETX       :  std_logic;

signal  USEXTOK        :  std_logic;

signal  LOOPFAIL       :  std_logic;

signal  iSYNCNOW       :  std_logic;

signal  CMDOKREG       :  std_logic;

signal  iCMDSTB        :  std_logic;

signal  BITSELECT      :  std_logic_vector(7 downto 0);

-- Interrupt System

type TINSTATE is ( IDLE, WRTIIW, WRTIAW, DONE );

signal INTSTATE        : TINSTATE;

signal DOINT           : std_logic;

signal MTINT           : std_logic;

-- Monitor Signals

signal  SMR0_STEX     :  std_logic;

signal  SMR0_SBIT     :  std_logic;

signal  SMR0_SRST     :  std_logic;

signal  SMR0_CHAEN    :  std_logic;

signal  SMR0_CHBEN    :  std_logic;

signal  SMR0_ETCE     :  std_logic;

signal  SMR0_MTO      :  std_logic;

signal  SMR0_EMODE    :  std_logic_vector(1 downto 0);

signal  SMR0_BUFR     :  std_logic;

signal  SMR0_SMTC     :  std_logic;

signal  SMR0_BCEN     :  std_logic;

signal  SMR0_DYNBC    :  std_logic;

signal  SMR0_PPEN     :  std_logic;

signal  SMR0_PPON     :  std_logic;

signal  SMR0_INTEN    :  std_logic;

signal  SMR0_XMTSW    :  std_logic;

signal  SMR0_BIT9     :  std_logic;

signal  SMR1_RTA      :  std_logic_vector(4 downto 0);

signal  SMR1_RTPTY    :  std_logic;

signal  SMR1_MSEL     :  std_logic_vector(1 downto 0);

signal  SMR1_STDA     :  std_logic;

signal  SMR1_LOCK     :  std_logic;

signal  SMR1_AUTOEN   :  std_logic;

signal  SMR1_SSYSF    :  std_logic;

signal  SMR1_EX       :  std_logic;

signal  SMR1_TAPF     :  std_logic;

signal  SMR1_READY    :  std_logic;

signal  SMR1_TERACT   :  std_logic;

signal  SMR2_CC       :  std_logic_vector(15 downto 0);

signal  SMR3_INTM     :  std_logic_vector(15 downto 0);

signal  SMR4_INTR     :  std_logic_vector(15 downto 0);

signal  SMR4_INTRP    :  std_logic_vector(15 downto 0);

signal  SMR5_INTA     :  std_logic_vector(15 downto 1);

signal  SMR6_DMAF     :  std_logic;

signal  SMR6_WRAPF    :  std_logic;

signal  SMR6_TAPF     :  std_logic;

signal  SMR6_BITF     :  std_logic;

signal  SMR6_CHAF     :  std_logic;

signal  SMR6_CHBF     :  std_logic;

signal  SMR6_MSBF     :  std_logic;

signal  SMR6_LSBF     :  std_logic;

signal  SMR6_UDB      :  std_logic_vector(7 downto 0);

signal  SMR7_TT       :  UNSIGNED(15 downto 0);

signal  SMR8_PREG     :  UNSIGNED(15 downto 0);

signal  SMR9_IMCLR    :  std_logic;

signal  SMR9_SB       :  std_logic_vector(10 to 19);

signal  SMR11_MBA     :  UNSIGNED(15 downto 0);

signal  SMR12_MDA     :  UNSIGNED(15 downto 0);

signal  SMR13_MBC     :  UNSIGNED(15 downto 0);

signal  SMR14_MF      :  std_logic_vector(31 downto 0);

signal  SMR_REG0      :  std_logic_vector(15 downto 0);

signal  SMR_REG1      :  std_logic_vector(15 downto 0);

signal  SMR_REG2      :  std_logic_vector(15 downto 0);

signal  SMR_REG3      :  std_logic_vector(15 downto 0);

signal  SMR_REG4      :  std_logic_vector(15 downto 0);

signal  SMR_REG5      :  std_logic_vector(15 downto 0);

signal  SMR_REG6      :  std_logic_vector(15 downto 0);

signal  SMR_REG7      :  std_logic_vector(15 downto 0);

signal  SMR_REG8      :  std_logic_vector(15 downto 0);

signal  SMR_REG9      :  std_logic_vector(15 downto 0);

signal  SMR_REGA      :  std_logic_vector(15 downto 0);

signal  SMR_REGB      :  std_logic_vector(15 downto 0);

signal  SMR_REGC      :  std_logic_vector(15 downto 0);

signal  SMR_REGD      :  std_logic_vector(15 downto 0);

signal  SMR_REGE      :  std_logic_vector(15 downto 0);

signal  SMR_REGF      :  std_logic_vector(15 downto 0);

signal  SMR_REG20     :  std_logic_vector(15 downto 0);

signal  SMD_CONTROL   :  std_logic_vector(15 downto 0);

signal  SMD_DPTR1     :  UNSIGNED(15 downto 0);

signal  SMD_DPTR2     :  UNSIGNED(15 downto 0);

signal  SMD_DPTR3     :  UNSIGNED(15 downto 0);

signal  SMD_MSGINFO   :  std_logic_vector(15 downto 0);

signal  SMD_MIBCAF    :  UNSIGNED(15 downto 0);

signal  SMDC_INDX     :  UNSIGNED(7 downto 0);

signal  SMDC_INTX     :  std_logic;

signal  SMDC_IWA      :  std_logic;

signal  SMDC_IBRD     :  std_logic;

signal  SMDC_BAC      :  std_logic;

signal  SMDC_BUFA     :  std_logic;

signal  SMDC_BRD      :  std_logic;

signal  SMDC_NII      :  std_logic;

signal  SMR3_DMAF     :  std_logic;

signal  SMR3_WRAPF    :  std_logic;

signal  SMR3_TAPF     :  std_logic;

signal  SMR3_BITF     :  std_logic;

signal  SMR3_MERR     :  std_logic;

signal  SMR3_SUBAD    :  std_logic;

signal  SMR3_BDRCV    :  std_logic;

signal  SMR3_IXEQ0    :  std_logic;

signal  SMR3_ILLCMD7  :  std_logic;

signal  SMR3_EOL      :  std_logic;

signal  SMR3_ILLCMD4  :  std_logic;

signal  SMR3_ILLOP    :  std_logic;

signal  SMR3_RTF      :  std_logic;

signal  SMR3_CBA      :  std_logic;

signal  SMR3_MBC      :  std_logic;

signal  SMR4_DMAF     :  std_logic;

signal  SMR4_WRAPF    :  std_logic;

signal  SMR4_TAPF     :  std_logic;

signal  SMR4_BITF     :  std_logic;

signal  SMR4_MERR     :  std_logic;

signal  SMR4_SUBAD    :  std_logic;

signal  SMR4_BDRCV    :  std_logic;

signal  SMR4_IXEQ0    :  std_logic;

signal  SMR4_ILLCMD7  :  std_logic;

signal  SMR4_EOL      :  std_logic;

signal  SMR4_ILLCMD4  :  std_logic;

signal  SMR4_ILLOP    :  std_logic;

signal  SMR4_RTF      :  std_logic;

signal  SMR4_CBA      :  std_logic;

signal  SMR4_MBC      :  std_logic;

signal  DATAPTR       :  UNSIGNED(15 downto 0);

signal  BASEPTR       :  UNSIGNED(15 downto 0);

signal  DESCPTR       :  UNSIGNED(15 downto 0);

signal  CAF           :  UNSIGNED(7 downto 0);

signal  CAFPLUS2      :  UNSIGNED(7 downto 0);

signal  CAFOVERFLOW   :  std_logic;

signal  INTLOG        :  std_logic;

signal  INTMSG        :  std_logic;

signal  WINT          :  std_logic;

signal  SMCW_OPCODE   :  std_logic_vector(3 downto 0);

signal  SMCW_RETRY    :  std_logic_vector(1 downto 0);

signal  SMCW_CHA      :  std_logic;

signal  SMCW_RTRT     :  std_logic;

signal  SMCW_CCODE    :  std_logic_vector(7 downto 1);

signal  MODECODE      :  std_logic;

signal  TRANSMIT      :  std_logic;

signal  SA            :  std_logic_vector(4 downto 0);

signal  NODATA        :  std_logic;

signal  BCBUSSEL      :  std_logic;

signal  ATTEMPTS      :  UNSIGNED(2 downto 0);

signal  M2MTIMER      :  UNSIGNED(15 downto 0);

signal  CW_DOFFSET    :  std_logic_vector(6 downto 0);

signal  SYNC_MRST     :  std_logic;

signal  SYNC_MRST1    :  std_logic;

signal  RSTINTn       :  std_logic;

signal  TCLK_DEL1     :  std_logic;

signal  TCLK_DEL2     :  std_logic;

signal  TTZERO        :  std_logic;

signal  BCLOADTT      :  std_logic;

signal  BC_CMDWORD    :  std_logic_vector(15 downto 0);

signal  iENCDATA      :  std_logic_vector(15 downto 0);

signal  DECDATASYNC   :  std_logic;

signal  BITWORD       :  std_logic_vector(15 downto 0);

signal  MIL1553A      :  std_logic;

signal  MIL1553B      :  std_logic;

-- Monitor

type TMTSTATE is (MTIDLE, MTGTCW2, MTWRDP, MTWRTT, MTWRCW1, MTWRCW2, MTWRSW1,

MTWRDATA,MTWRTSW2,MTWRMINFO,MTMSGDONE,MTALLDONE);

signal MTSTATE :  TMTSTATE;

signal  MTMSG_BUSSEL  :  std_logic;

signal  MTMSG_STBALL  :  std_logic;

signal  MTMSG_START   :  std_logic;

signal  MTMSG_CMDWORD :  std_logic_vector(23 downto 0);

signal  MTDESCPTR     :  UNSIGNED(15 downto 0);

signal  MTDATAPTR     :  UNSIGNED(15 downto 0);

signal  MTCOUNT       :  UNSIGNED(15 downto 0);

signal  MTCOUNTHD     :  UNSIGNED(15 downto 0);

signal  MTWORDCOUNT   :  UNSIGNED(5 downto 0);

signal  MTEXPWCNT     :  UNSIGNED(5 downto 0);

signal  MTMI_WORD     :  std_logic_vector(15 downto 0);

signal  MTMI_CHA      :  std_logic;

signal  MTMI_RTRT     :  std_logic;

signal  MTMI_MSGERR   :  std_logic;

signal  MTMI_MCND     :  std_logic;

signal  MTMI_TOUT     :  std_logic;

signal  MTMI_OVR      :  std_logic;

signal  MTMI_PERR     :  std_logic;

signal  MTMI_MERR     :  std_logic;

signal  MTCW_BCAST    :  std_logic;

signal  MTCW_MCODE    :  std_logic;

signal  MTCW_DATA     :  std_logic;

signal  MTCW_COUNT    :  std_logic_vector(5 downto 0);

signal  MTCW_TX       :  std_logic;

signal  MTCW_SUBADDR  :  std_logic_vector(4 downto 0);

signal  MTCW_MCWC     :  std_logic_vector(4 downto 0);

signal  MONHIT        :  std_logic;

signal  RTACTIVE      :  std_logic;

signal  EXPECTSW      :  std_logic;

signal  INTREAD       :  std_logic;

signal  INTOUTHX      :  std_logic;

signal  IMCOUNT       :  UNSIGNED(1 downto 0);

signal  IHCOUNT       :  UNSIGNED(1 downto 0);

signal  CLR_SRQ       :  std_logic;

signal  FASTIMG       :  std_logic;

signal  CWRT1_LB      :  std_logic_vector(10 downto 0);

signal  CWRT2_LB      :  std_logic_vector(10 downto 0);

signal  TTTEMP        :  UNSIGNED(15 downto 0);

-- MISC

signal  Logic0        :  std_logic;

signal  Logic1        :  std_logic;

signal  ZERO          :  std_logic_vector(15 downto 0);

-- Stop the SWRESETX register from being replicated

attribute syn_replicate : boolean;

attribute syn_replicate of SWRESETX : signal is false;

begin

Logic0 <= '0';

Logic1 <= '1';

ZERO   <= ( others => '0');

iBUSRESET  <= RSTREG;          -- RT mode reset does not reset the whole core

BUSRESET   <= iBUSRESET;

RSTINTn   <= not ( not RSTINn or SWRESETX);

URST : GLOBALBUFF port map ( A => RSTINTn, Y => RSTn );

RSTOUTn   <= RSTn;      -- Only asserted by CPU or external reset

process(CLK,RSTINn)     -- This is only reset by the external reset

begin

if RSTINn='0' then

SWRESETX   <= '0';

elsif CLK'event and CLK='1' then

SWRESETX   <= SMR0_SRST;

end if;

end process;

process(CLK,RSTn)      -- This is reset by the internal reset, allows the RT address to be reloaded

begin

if RSTn='0' then

SYNC_MRST  <= '1';

SYNC_MRST1 <= '1';

elsif CLK'event and CLK='1' then

SYNC_MRST1 <= '0';

SYNC_MRST  <= SYNC_MRST1;

end if;

end process;

-- RT Command Word Decoding

URT:  BRMRT

generic map ( BCENABLE => BCENABLE,

RTENABLE => RTENABLE,

MTENABLE => MTENABLE,

LEGREGS  => LEGREGS,

ENHANCED => ENHANCED,

INITFREQ => INITFREQ )

port map

( CLK            =>  CLK,

CLKSPD         =>  CLKFREQ,

RSTn           =>  RSTn,

RSTREG         =>  RSTREG,

RTMODE         =>  iRTMODE,

RTADDR         =>  SMR1_RTA,

RTADDRP        =>  SMR1_RTPTY,

RTADERR        =>  RTADDRERR,

BCASTEN        =>  SMR0_BCEN,

MIL1553A       =>  SMR1_STDA,

SMR0_EMODE     =>  SMR0_EMODE,

SMR9_SB        =>  SMR9_SB,

DBA            =>  SMR0_DYNBC,

XMTSW          =>  SMR0_XMTSW,

CHAEN          =>  SMR0_CHAEN,

CHBEN          =>  SMR0_CHBEN,

SSYSFn         =>  SSYSFn,

RTBUSY         =>  RTBUSY,

SHUTDOWNRTA    =>  SHUTDOWNRTA,

SHUTDOWNRTB    =>  SHUTDOWNRTB,

BUSSEL         =>  RTBUSSEL,

BUSSELALL      =>  iBUSSEL,

TXTIMEOUT      =>  TXTIMEOUT,

DECA_FROMUS    =>  DECA_FROMUS,

DECA_STROBE    =>  DECA_STROBE,

DECA_DATAOUT   =>  DECA_DATAOUT,

DECA_CMDSTAT   =>  DECA_CMDSTAT,

DECA_GOTSYNC   =>  DECA_GOTSYNC,

DECA_PARERR    =>  DECA_PARERR,

DECA_MANERR    =>  DECA_MANERR,

DECA_ACTIVE    =>  DECA_ACTIVE,

DECA_CONTIG    =>  DECA_CONTIG,

DECB_FROMUS    =>  DECB_FROMUS,

DECB_STROBE    =>  DECB_STROBE,

DECB_DATAOUT   =>  DECB_DATAOUT,

DECB_CMDSTAT   =>  DECB_CMDSTAT,

DECB_GOTSYNC   =>  DECB_GOTSYNC,

DECB_PARERR    =>  DECB_PARERR,

DECB_MANERR    =>  DECB_MANERR,

DECB_ACTIVE    =>  DECB_ACTIVE,

DECB_CONTIG    =>  DECB_CONTIG,

LOOPFAILA      =>  LOOPFAILA,

LOOPFAILB      =>  LOOPFAILB,

LATCHSW        =>  LATCHSW,

WCNTTO         =>  WCNTTO,

WCNTOVR        =>  WCNTOVR,

CMDDONE        =>  RTCMDDONE,

RTSMBUSY       =>  PCONBUSY,

BENDFAIL       =>  BENDFAIL,

RTMSG_STBME    =>  RTMSG_STBME,

RTMSG_START    =>  RTMSG_START,

RTMSG_CMDWORD  =>  RTMSG_CMDWORD,

RTMSG_LEGAL    =>  RTMSG_LEGAL,

RTMSG_RTRT     =>  RTMSG_RTRT,

RTMSG_RTRTOK   =>  RTMSG_RTRTOK ,

RTMSG_OKAY     =>  RTMSG_OKAY,

RTMSG_GOTSW    =>  GOTSW,

CLR_SRQ        =>  CLR_SRQ,

TIMER57US      =>  TIMER57US,

MSGINFO        =>  SMD_MSGINFO,

BITWORD        =>  BITWORD,

TXT_CMDSTAT    =>  iENCCMDSTAT,

BENDDIN        =>  BENDDIN,

TXT_DATAIN     =>  iENCDATA,

CMDSYNC        =>  CMDSYNC,

SYNCNOW        =>  iSYNCNOW,

USEXTOK        =>  USEXTOK,

BITSELECT      =>  BITSELECT,

CMDOKREG       =>  CMDOKREG,

CMDVAL         =>  CMDVAL,

CMDOK          =>  CMDOK,

CMDSTB         =>  iCMDSTB,

CMDOKOUT       =>  CMDOKOUT

);

CMDSTB <= iCMDSTB;

ENCDATA    <= iENCDATA;

MSGSTART   <=  RTMSG_START;

DECFROMUS  <=  DECA_FROMUS  when iBUSSEL='0' else DECB_FROMUS;

DECSTROBE  <=  DECA_STROBE  when iBUSSEL='0' else DECB_STROBE;

DECDATA    <=  DECA_DATAOUT when iBUSSEL='0' else DECB_DATAOUT;

DECCMDSTAT <=  DECA_CMDSTAT when iBUSSEL='0' else DECB_CMDSTAT;

DECGOTSYNC <=  DECA_GOTSYNC when iBUSSEL='0' else DECB_GOTSYNC;

DECPARERR  <=  DECA_PARERR  when iBUSSEL='0' else DECB_PARERR;

DECMANERR  <=  DECA_MANERR  when iBUSSEL='0' else DECB_MANERR;

DECACTIVE  <=  DECA_ACTIVE  when iBUSSEL='0' else DECB_ACTIVE;

DECCONTIG  <=  DECA_CONTIG  when iBUSSEL='0' else DECB_CONTIG;

DECDATASYNC <= DECGOTSYNC and not DECCMDSTAT;

PBS:

process(iBCMODE,iMTMODE,SHUTDOWNRTA,SHUTDOWNRTB,RTBUSSEL,BCBUSSEL,

RTACTIVE,MTMSG_BUSSEL,RTADDRERR)

begin

if iBCMODE='1' then

SHUTDOWNA <=     BCBUSSEL;

SHUTDOWNB <= not BCBUSSEL;

RXBUSENA  <= not BCBUSSEL;

RXBUSENB  <=     BCBUSSEL;

iBUSSEL   <=     BCBUSSEL;

elsif RTACTIVE='1' then

SHUTDOWNA <= SHUTDOWNRTA;

SHUTDOWNB <= SHUTDOWNRTB;

RXBUSENA  <= not RTADDRERR;

RXBUSENB  <= not RTADDRERR;

iBUSSEL   <= RTBUSSEL;

elsif iMTMODE='1' then

SHUTDOWNA <= '0';

SHUTDOWNB <= '0';

RXBUSENA  <= '1';

RXBUSENB  <= '1';

iBUSSEL   <= MTMSG_BUSSEL;

else

SHUTDOWNA <= SHUTDOWNRTA;

SHUTDOWNB <= SHUTDOWNRTB;

RXBUSENA  <= not RTADDRERR;

RXBUSENB  <= not RTADDRERR;

iBUSSEL   <= RTBUSSEL;

end if;

end process;

LOOPFAIL   <=  LOOPFAILA or LOOPFAILB;

BUSSEL  <= iBUSSEL;

-- SYNC is active for RT SYNC modecode

SYNCNOW <= iSYNCNOW;

-- RT Command Word Decoding

CW_BCAST   <= RTMSG_CMDWORD(23);

CW_MCODE   <= RTMSG_CMDWORD(22);

CW_DATA    <= not RTMSG_CMDWORD(22);

CW_COUNT   <= RTMSG_CMDWORD(21 downto 16);

CW_TX      <= RTMSG_CMDWORD(10);

CW_SUBADDR <= RTMSG_CMDWORD(9 downto 5);

CW_MCWC    <= RTMSG_CMDWORD(4 downto 0);

CW_STBME   <= RTMSG_STBME;

RTSTART    <= RTMSG_START;

RTRTRT     <= RTMSG_RTRT;

RTRTOK     <= RTMSG_RTRTOK;

LEGAL      <= RTMSG_LEGAL;

process( CW_MCODE,CW_TX,CW_SUBADDR,CW_MCWC)

begin

if CW_MCODE='0' then

CW_DOFFSET <= '0' & CW_TX & CW_SUBADDR;

else

CW_DOFFSET <= '1' & CW_TX & CW_MCWC;

end if;

end process;

-- Monitor Command Word Decoding

UMT :  BRMMT

port map ( CLK           => CLK,

RSTn          => RSTn,

BCASTEN       => SMR0_BCEN,

MIL1553A      => SMR1_STDA,

EXPECTSW      => EXPECTSW,

DECA_STROBE   => DECA_STROBE,

DECA_DATAOUT  => DECA_DATAOUT,

DECA_CMDSTAT  => DECA_CMDSTAT,

DECA_GOTSYNC  => DECA_GOTSYNC,

DECA_PARERR   => DECA_PARERR,

DECA_MANERR   => DECA_MANERR,

DECA_ACTIVE   => DECA_ACTIVE,

DECA_CONTIG   => DECA_CONTIG,

DECA_FROMUS   => DECA_FROMUS,

DECB_STROBE   => DECB_STROBE,

DECB_DATAOUT  => DECB_DATAOUT,

DECB_CMDSTAT  => DECB_CMDSTAT,

DECB_GOTSYNC  => DECB_GOTSYNC,

DECB_PARERR   => DECB_PARERR,

DECB_MANERR   => DECB_MANERR,

DECB_ACTIVE   => DECB_ACTIVE,

DECB_CONTIG   => DECB_CONTIG,

DECB_FROMUS   => DECB_FROMUS,

MTMSG_BUSSEL  => MTMSG_BUSSEL,

MTMSG_STBALL  => MTMSG_STBALL,

MTMSG_START   => MTMSG_START,

MTMSG_CMDWORD => MTMSG_CMDWORD

);

MTCW_BCAST   <= MTMSG_CMDWORD(23);

MTCW_MCODE   <= MTMSG_CMDWORD(22);

MTCW_DATA    <= not MTMSG_CMDWORD(22);

MTCW_COUNT   <= MTMSG_CMDWORD(21 downto 16);

MTCW_TX      <= MTMSG_CMDWORD(10);

MTCW_SUBADDR <= MTMSG_CMDWORD(9 downto 5);

MTCW_MCWC    <= MTMSG_CMDWORD(4 downto 0);

-- Register Block

UREGS: BRMREG

generic map ( BCENABLE => BCENABLE,

RTENABLE => RTENABLE,

MTENABLE => MTENABLE,

LEGREGS  => LEGREGS,

ENHANCED => ENHANCED,

INITFREQ => INITFREQ )

port map    ( CLK       =>  CLK,

RSTn      =>  RSTn,

SMR0_STEX =>  SMR0_STEX,

CMDSTB    =>  iCMDSTB,

CPUSEL    =>  CPUADDRR(4),

CPUADDR   =>  CPUADDR(3 downto 0),

CPUADDRR  =>  CPUADDRR(3 downto 0),

CPUWR     =>  CPUWRR,

CPURD     =>  CPURDR,

CPUDIN    =>  CPUDINR,

CPUDOUT   =>  CPUDOUT2,

BITSELECT =>  BITSELECT,

CMDOKREG  =>  CMDOKREG

);

process(CPUADDR,SMR_REG0,SMR_REG1,SMR_REG2,SMR_REG3,

SMR_REG4,SMR_REG5,SMR_REG6,SMR_REG7,

SMR_REG8,SMR_REG9,SMR_REGA,SMR_REGB,

SMR_REGC,SMR_REGD,SMR_REGE,SMR_REGF )

begin

CPUDOUT1 <= ( others => '-');

case CPUADDR(3 downto 0) is

when "0000"  => CPUDOUT1 <= SMR_REG0;

when "0001"  => CPUDOUT1 <= SMR_REG1;

when "0010"  => CPUDOUT1 <= SMR_REG2;

when "0011"  => CPUDOUT1 <= SMR_REG3;

when "0100"  => CPUDOUT1 <= SMR_REG4;

when "0101"  => CPUDOUT1 <= SMR_REG5;

when "0110"  => CPUDOUT1 <= SMR_REG6;

when "0111"  => CPUDOUT1 <= SMR_REG7;

when "1000"  => if BCENABLE=1 or RTENABLE=1 then CPUDOUT1 <= SMR_REG8; end if;

when "1001"  => if RTENABLE=1 then CPUDOUT1 <= SMR_REG9; end if;

when "1010"  => if BCENABLE=1 then CPUDOUT1 <= SMR_REGA; end if;

when "1011"  => if MTENABLE=1 then CPUDOUT1 <= SMR_REGB; end if;

when "1100"  => if MTENABLE=1 then CPUDOUT1 <= SMR_REGC; end if;

when "1101"  => if MTENABLE=1 then CPUDOUT1 <= SMR_REGD; end if;

when "1110"  => if MTENABLE=1 then CPUDOUT1 <= SMR_REGE; end if;

when "1111"  => if MTENABLE=1 then CPUDOUT1 <= SMR_REGF; end if;

when others  => CPUDOUT1 <= ( others => '-');

end case;

end process;

process(CPUADDR,CPUDOUT1,CPUDOUT2,SMR_REG20)

begin

case CPUADDR(4) is

when '0'     => CPUDOUT <= CPUDOUT1;

when others  => CPUDOUT <= CPUDOUT2;

end case;

if ENHANCED=1 then

case CPUADDR(5 downto 4) is

when "00"   => CPUDOUT <= CPUDOUT1;

when "01"   => CPUDOUT <= CPUDOUT2;

when others => CPUDOUT <= SMR_REG20;

end case;

end if;

end process;

-- Register CPU inputs to simplify timing

process(CLK,RSTn)

begin

if RSTn='0' then

CPUDINR   <= ( others => '0');

CPUADDRR  <= "000000";

CPUWRR    <= "00";

CPURDR    <= '0';

CPUWRRD   <= "00";

CPUSTB    <= '0';

CPUCLRINT <= '0';

elsif CLK'event and CLK='1' then

CPUDINR   <= CPUDIN;

CPUADDRR  <= '0' & CPUADDR(4 downto 0);

if ENHANCED=1 then

CPUADDRR(5)  <= CPUADDR(5);

end if;

CPUWRR    <= CPUWR;

CPURDR    <= CPURD;

CPUWRRD   <= CPUWRR;

CPUSTB    <= CPURD or CPUWR(1) or CPUWR(0);

CPUCLRINT <= not (CPURD or CPUWR(1) or CPUWR(0)) and CPUSTB;

end if;

end process;

process(CLK,RSTINn)

begin

if RSTINn='0' then

FORCEORUN   <= '0';

USEXTOK     <= '0';

FASTIMG     <= '0';

CLKFREQ     <= to_std_logic(to_unsigned(((INITFREQ-12)/4),2)); -- convert 12-24 to 00-11

ACT_ENHANCE <= '0';

iLOOPBACK   <= '0';

elsif CLK'event and CLK='1' then

if CPUWRR(1)='1' and CPUADDRR="100000" and ENHANCED=1 then

CLKFREQ     <= CPUDINR(1 downto 0);

USEXTOK     <= CPUDINR(2);

FORCEORUN   <= CPUDINR(3);

FASTIMG     <= CPUDINR(4);

ACT_ENHANCE <= CPUDINR(5);

iLOOPBACK   <= CPUDINR(6);

end if;

end if;

end process;

LOOPBACK <= iLOOPBACK;

SMR_REG20 <= VERSION & '0' & iLOOPBACK & ACT_ENHANCE & FASTIMG & FORCEORUN & USEXTOK & CLKFREQ

when ENHANCED=1

else ( others => '-');

-- CPU Registers

SMR1_AUTOEN  <= '0';

SMR1_SSYSF   <= not SSYSFn;

SMR1_EX      <= iBUSY;

SMR1_TAPF    <= RTADDRERR;

SMR1_READY   <= '1';         -- since no BIT or Initilaisation is always ready

SMR1_TERACT  <= iBUSY;

iRTMODE <= to_logic(RTENABLE) and to_logic(SMR1_MSEL="01" or SMR1_MSEL="11");

iBCMODE <= to_logic(BCENABLE) and to_logic(SMR1_MSEL="00");

iMTMODE <= to_logic(MTENABLE) and to_logic(SMR1_MSEL="10" or SMR1_MSEL="11");

PREG01:

process(CLK,RSTn)

begin

if RSTn='0' then

SMR1_RTA      <= "00000";

SMR1_RTPTY    <= '1';     -- prevents a TAPF before loading the register from pads

SMR1_MSEL     <= "00";

SMR1_STDA     <= '0';

SMR1_LOCK     <= '1';

elsif CLK'event and CLK='1' then

if SYNC_MRST='1'  then

SMR1_RTA   <= RTADDRIN;

SMR1_RTPTY <= RTADDRPIN;

SMR1_MSEL  <= MSELIN;

SMR1_STDA  <= ABSTDIN;

SMR1_LOCK  <= not LOCKn;

elsif SMR1_LOCK='0' and SMR0_STEX='0' then      -- only load when inactive and not lock

if CPUWRR(1)='1' and CPUADDRR="000001" then

SMR1_RTA   <= CPUDINR(15 downto 11);

SMR1_RTPTY <= CPUDINR(10);

SMR1_MSEL  <= CPUDINR(9 downto 8);

end if;

if CPUWRR(0)='1' and CPUADDRR="000001" then

SMR1_STDA <= CPUDINR(7);

end if;

end if;

end if;

end process;

OPMODE <= SMR1_MSEL;

SMR0_BIT9 <= SMR0_MTO when iRTMODE='0' else SMR0_PPON;

SMR_REG0 <= SMR0_STEX & SMR0_SBIT & SMR0_SRST

& SMR0_CHAEN & SMR0_CHBEN & SMR0_ETCE  & SMR0_BIT9

& SMR0_EMODE & SMR0_BUFR  & SMR0_SMTC  & SMR0_BCEN

& SMR0_DYNBC & SMR0_PPEN  & SMR0_INTEN & SMR0_XMTSW;

SMR_REG1 <= SMR1_RTA & SMR1_RTPTY & SMR1_MSEL & SMR1_STDA

& SMR1_LOCK & SMR1_AUTOEN & SMR1_SSYSF & SMR1_EX & SMR1_TAPF

& SMR1_READY & SMR1_TERACT;

PMCCW:

process(CLK,RSTn)

begin

if RSTn='0' then

BC_CMDWORD <= ( others => '0');

elsif CLK'event and CLK='1' then

if iENCCMDSTAT='1' and iENCSTROBE='1' then

BC_CMDWORD <= iENCDATA;

end if;

end if;

end process;

PCCWMUX:

process(SMR1_MSEL,BC_CMDWORD,RTMSG_CMDWORD,MTMSG_CMDWORD)

begin

case SMR1_MSEL is

when "00"   => SMR2_CC <= BC_CMDWORD;

when "01"   => SMR2_CC <= RTMSG_CMDWORD(15 downto 0);

when "10"   => SMR2_CC <= MTMSG_CMDWORD(15 downto 0);

when others => SMR2_CC <= MTMSG_CMDWORD(15 downto 0);

end case;

end process;

SMR_REG2 <= SMR2_CC;

SMR3_INTM <=   SMR3_DMAF & SMR3_WRAPF & SMR3_TAPF & SMR3_BITF & SMR3_MERR

& SMR3_SUBAD & SMR3_BDRCV & SMR3_IXEQ0 & SMR3_ILLCMD7 & '0'

& SMR3_EOL & SMR3_ILLCMD4 & SMR3_ILLOP & SMR3_RTF

& SMR3_CBA & SMR3_MBC;

SMR4_INTRP<=   SMR4_DMAF & SMR4_WRAPF & SMR4_TAPF & SMR4_BITF

& SMR4_MERR & SMR4_SUBAD & SMR4_BDRCV & SMR4_IXEQ0

& SMR4_ILLCMD7 & '0' & SMR4_EOL & SMR4_ILLCMD4

& SMR4_ILLOP & SMR4_RTF & SMR4_CBA & SMR4_MBC;

SMR_REG3 <= SMR3_INTM;

SMR_REG4 <= SMR4_INTR;

SMR_REG5 <= SMR5_INTA & '0';

INTOUTHX <= SMR4_DMAF or SMR4_WRAPF or SMR4_TAPF or SMR4_BITF;

MEMFAIL  <= SMR4_DMAF;

-- Generates the  3 cycle pulse on INTOUTH or level

PINTOUT:

process(CLK,RSTn)

begin

if RSTn='0' then

IHCOUNT <= "00";

INTOUTH <= '0';

elsif CLK'event and CLK='1' then

case IHCOUNT is

when "00"   => if INTOUTHX='1' then

INTOUTH <= '1';

IHCOUNT <= "01";

end if;

when "01"   => IHCOUNT <= "10";

when "10"   => IHCOUNT <= "11";

when others => if INTLEVEL='0' then

INTOUTH <= '0';

end if;

if INTOUTHX='0' then

IHCOUNT <= "00";

end if;

end case;

if INTLEVEL='1' and INTACKH='1' then

INTOUTH <= '0';

end if;

end if;

end process;

INTMSG  <=    SMR4_MERR or SMR4_SUBAD or SMR4_BDRCV or SMR4_IXEQ0 or SMR4_ILLCMD7

or SMR4_EOL or SMR4_ILLCMD4 or SMR4_ILLOP or SMR4_RTF or SMR4_CBA or SMR4_MBC;

INTOUTM <= INTMSG when SMR0_INTEN='0' else INTLOG;

-- BIT Register

PREGRT6:

process(CLK,RSTn)

begin

if RSTn='0' then

SMR6_DMAF  <= '0';

SMR6_WRAPF <= '0';

SMR6_BITF  <= '0';

SMR6_TAPF  <= '0';

SMR6_CHAF  <= '0';

SMR6_CHBF  <= '0';

SMR6_MSBF  <= '0';

SMR6_LSBF  <= '0';

SMR6_UDB   <= ( others => '0');

elsif CLK'event and CLK='1' then

if CPUWRR(1)='1' and CPUADDRR="000110" then

SMR6_DMAF  <= CPUDINR(15);

SMR6_WRAPF <= CPUDINR(14);

SMR6_BITF  <= CPUDINR(12);

SMR6_TAPF  <= CPUDINR(13);

SMR6_CHAF  <= CPUDINR(11);

SMR6_CHBF  <= CPUDINR(10);

SMR6_MSBF  <= CPUDINR(9);

SMR6_LSBF  <= CPUDINR(8);

end if;

if CPUWRR(0)='1' and CPUADDRR="000110" then

SMR6_UDB <= CPUDINR(7 downto 0);

end if;

if BENDFAIL='1'  then

SMR6_DMAF <= '1';

end if;

if LOOPFAIL='1' then

SMR6_WRAPF <= '1';

end if;

if RTADDRERR='1' then

SMR6_TAPF <= '1';

end if;

if TXTIMEOUT='1' and iBUSSEL='0'  then

SMR6_CHAF <= '1';

end if;

if TXTIMEOUT='1' and iBUSSEL='1' then

SMR6_CHBF <= '1';

end if;

if SYNC_MRST='1' then

SMR6_UDB <= VERSION;

end if;

end if;

end process;

BITWORD <=   SMR6_DMAF & SMR6_WRAPF & SMR6_TAPF & SMR6_BITF

& SMR6_CHAF & SMR6_CHBF  & SMR6_MSBF & SMR6_LSBF

& SMR6_UDB;

SMR_REG6  <= BITWORD;

PREG7:

process(CLK,RSTn)

begin

if RSTn='0' then

SMR7_TT   <= ( others => '0');

TCLK_DEL1 <= '0';

TCLK_DEL2 <= '0';

elsif CLK'event and CLK='1' then

TCLK_DEL1 <= TCLK;

TCLK_DEL2 <= TCLK_DEL1;

-- Counting Control

if   (SMR0_ETCE='0' and CLKSTB='1')

or (SMR0_ETCE='1' and TCLK_DEL1='1' and TCLK_DEL2='0')

then

if iBCMODE='1' then

if TTZERO='0' then

SMR7_TT <= SMR7_TT - 1;

end if;

else

SMR7_TT <= SMR7_TT + 1;

end if;

end if;

-- Register is Read only in BC mode

if iBCMODE='0' and CPUWRR(1)='1' and CPUADDRR="000111" then

SMR7_TT(15 downto 8) <= to_unsigned(CPUDINR(15 downto 8));

end if;

if iBCMODE='0' and CPUWRR(0)='1' and CPUADDRR="000111" then

SMR7_TT(7 downto 0)  <= to_unsigned(CPUDINR(7 downto 0));

end if;

if (iSYNCNOW='1' or iBUSRESET='1') and iRTMODE='1' then

-- TIMETAG is loaded with data or reset

if SMR2_CC(4)='1' then

SMR7_TT <= to_unsigned(DECDATA);

else

SMR7_TT <= ( others => '0');

end if;

end if;

if (iBCMODE='1' and SMR0_STEX='0')  then

SMR7_TT <= ( others => '0');

end if;

if BCLOADTT='1' then

SMR7_TT <= to_unsigned(BENDDIN);

end if;

end if;

end process;

TTZERO <= to_logic( SMR7_TT = 0);

SMR_REG7 <= to_std_logic(SMR7_TT);

SMR_REG8 <= to_std_logic(SMR8_PREG);

PREGRT9:

process(CLK,RSTn)

begin

if RSTn='0' then

SMR9_IMCLR <= '0';

SMR9_SB    <= ( others => '0');

elsif CLK'event and CLK='1' then

if CPUWRR(1)='1' and CPUADDRR="001001" then

SMR9_IMCLR <= CPUDINR(15);

SMR9_SB(10 to 11) <= CPUDINR(9 downto 8);

end if;

if CPUWRR(0)='1' and CPUADDRR="001001" then

SMR9_SB(12 to 19) <= CPUDINR(7 downto 0);

end if;

if RTCMDDONE='1' and SMR9_IMCLR='1' then

SMR9_SB <= ( others => '0');

end if;

if CLR_SRQ='1' and SMR1_STDA='0' then  -- clear bit on VW if in BUSB mode

SMR9_SB(11) <= '0';

end if;

end if;

end process;

SMR_REG9 <= SMR9_IMCLR & "00000" & SMR9_SB;

PREGRT11:

process(CLK,RSTn)

begin

if RSTn='0' then

SMR11_MBA <= ( others => '0');

elsif CLK'event and CLK='1' then

if CPUWRR(1)='1' and CPUADDRR="001011" then

SMR11_MBA(15 downto 8) <= to_unsigned(CPUDINR(15 downto 8));

end if;

if CPUWRR(0)='1' and CPUADDRR="001011" then

SMR11_MBA(7 downto 0) <= to_unsigned(CPUDINR(7 downto 0));

end if;

end if;

end process;

SMR_REGB  <= to_std_logic(SMR11_MBA);

PREGRT12:

process(CLK,RSTn)

begin

if RSTn='0' then

SMR12_MDA <= ( others => '0');

elsif CLK'event and CLK='1' then

if CPUWRR(1)='1' and CPUADDRR="001100" then

SMR12_MDA(15 downto 8) <= to_unsigned(CPUDINR(15 downto 8));

end if;

if CPUWRR(0)='1' and CPUADDRR="001100" then

SMR12_MDA(7 downto 0) <= to_unsigned(CPUDINR(7 downto 0));

end if;

end if;

end process;

SMR_REGC  <= to_std_logic(SMR12_MDA);

SMR_REGD  <= to_std_logic(SMR13_MBC);

PREGRT14:

process(CLK,RSTn)

begin

if RSTn='0' then

SMR14_MF <= ( others => '0');

elsif CLK'event and CLK='1' then

if CPUWRR(1)='1' and CPUADDRR="001110" then

SMR14_MF(31 downto 24) <= CPUDINR(15 downto 8);

end if;

if CPUWRR(0)='1' and CPUADDRR="001110" then

SMR14_MF(23 downto 16) <= CPUDINR(7 downto 0);

end if;

if CPUWRR(1)='1' and CPUADDRR="001111" then

SMR14_MF(15 downto 8) <= CPUDINR(15 downto 8);

end if;

if CPUWRR(0)='1' and CPUADDRR="001111" then

SMR14_MF(7 downto 0) <= CPUDINR(7 downto 0);

end if;

end if;

end process;

SMR_REGE  <= SMR14_MF(31 downto 16);

SMR_REGF  <= SMR14_MF(15 downto 0);

SMR_REGA  <= ( others => '0');   -- BC Command Block Initialization

RTADERR  <= RTADDRERR;

-- Clock and TIMER Scaling

PSC1:

process(CLKFREQ)

begin

case CLKFREQ is

when "00"   => SCALE <= "01011"; -- 12MHz

when "01"   => SCALE <= "01111"; -- 16MHz

when "10"   => SCALE <= "10011"; -- 20MHz

when others => SCALE <= "10111"; -- 24MHz

end case;

end process;

CLKSPD <= CLKFREQ;

PSC2:

process(CLK,RSTn)

begin

if RSTn='0' then

CLKSTB    <= '0';

CLKSTB1US <= '0';

SCALER1   <= "00000";

SCALER2   <= "000000";

elsif CLK'event and CLK='1' then

CLKSTB  <= '0';

SCALER1 <= SCALER1 + 1;

if SCALER1=SCALE then

SCALER1 <= "00000";

end if;

CLKSTB1US <= to_logic(SCALER1="00000");

if CLKSTB1US='1' then

SCALER2 <= SCALER2 + 1;

CLKSTB  <= to_logic(SCALER2(5 downto 0) = "111111"); -- 64us

end if;

end if;

end process;

-- DECACTIVE stays Active for approx 5us after the last word if the bus is quiet

process(FASTIMG)

begin

if FASTIMG='1' then

IMGVALUE <=  "00001";   -- 6 us 5+1    We can go faster depending on memory

else

IMGVALUE <=  "10111";   -- 28 us 5+23  Summit has an IMG of 28us

end if;

end process;

-- DECACTIVE stays active for 5us after last WORD and takes 3us to reactivate

-- DEC_INACTIVE = RESPTIME - 2.0us

process(SMR1_STDA,SMR0_MTO)

variable muxsel : std_logic_vector(1 downto 0);

begin

muxsel := SMR0_MTO & SMR1_STDA;

case muxsel is

when "00" => RESPVALUE <= "01100";   -- B: 14 us   14-2=12

when "01" => RESPVALUE <= "00111";   -- A: 9  us    9-2=7

when "10" => RESPVALUE <= "11100";   -- B: 30 us   30-2=28

when "11" => RESPVALUE <= "10011";   -- A: 21 us   21-2=19

when others =>

end case;

end process;

-- BC SW Checking TSW Setting

MIL1553A <= SMR1_STDA;

MIL1553B <= not SMR1_STDA;

MSG_ERROR   <= SW_MSGERR or RT_NORESP or RT_ERROR or SW_ERROR;   -- Note SW_BUSY is not a message error

MSG_ERRORCC <= SW_MSGERR or RT_NORESP or RT_ERROR or SW_ERROR;

-- Condition Code Mux

PCCMUX:

process(SMCW_CCODE,MSG_ERRORCC,SW_MSGERR,SW_BUSY,SW_TFLAG,SW_SSFLAG,SW_INSTRU,SW_SREQ)

variable CC : std_logic_vector(7 downto 1);

begin

CC(7) := SMCW_CCODE(7) and MSG_ERRORCC;

CC(6) := SMCW_CCODE(6) and SW_MSGERR;

CC(5) := SMCW_CCODE(5) and SW_BUSY;

CC(4) := SMCW_CCODE(4) and SW_TFLAG;

CC(3) := SMCW_CCODE(3) and SW_SSFLAG;

CC(2) := SMCW_CCODE(2) and SW_INSTRU;

CC(1) := SMCW_CCODE(1) and SW_SREQ;

CCFLAG <= CC(7) or CC(6) or CC(5) or CC(4) or CC(3) or CC(2) or CC(1);

end process;

-- The Master STATE machine

PSTB:

process(CLK,RSTn)

begin

if RSTn='0' then

DECSTBDEL1  <= '0';

DECSTB      <= '0';

DECSTBOKSW  <= '0';

DECSTBOKSWA <= '0';

DECSTBOKDW  <= '0';

DECSTBOKCW  <= '0';

elsif CLK'event and CLK='1' then

DECSTBDEL1  <= DECSTROBE;

DECSTB      <= DECSTROBE and not DECFROMUS;

DECSTBOKCW  <= DECSTROBE and DECCMDSTAT and not DECFROMUS

and not DECPARERR and not DECMANERR;

DECSTBOKSW  <= DECSTROBE and DECCMDSTAT and not DECCONTIG and not DECFROMUS

and not DECPARERR and not DECMANERR;

DECSTBOKSWA <= DECSTROBE and DECCMDSTAT and not DECCONTIG

and not DECPARERR and not DECMANERR;

DECSTBOKDW  <= DECSTROBE and not DECCMDSTAT and DECCONTIG

and not DECPARERR and not DECMANERR;

end if;

end process;

PSM:

process(CLK,RSTn)

type TBUFMODE is ( INDEX,PPONG,CIRC1,CIRC2);

variable BENDWAIT   : std_logic;

variable MUXSEL     : std_logic_vector(3 downto 0);

variable RTMUXSEL   : std_logic_vector(2 downto 0);

variable MTMUXSEL   : std_logic_vector(2 downto 0);

variable RTBUFMODE  : std_logic_vector(2 downto 0);

variable BUFMODE    : TBUFMODE;

variable CALLOKAY   : std_logic;

begin

if RSTn='0' then

BCSTATE      <= BCIDLE;

ABORT        <= '0';

iENCSTROBE   <= '0';

iENCCMDSTAT  <= '0';

iBENDREQ     <= '0';

iBENDWRT     <= '0';

iBENDBURST   <= '0';

BENDADDR     <= ( others => '0');

BENDDOUT     <= ( others => '0');

RTTIMER      <= ( others => '0');

DATAPTR      <= ( others => '0');

BASEPTR      <= ( others => '0');

DESCPTR      <= ( others => '0');

BCAST        <= '0';

MSGTYPE      <= "000";

WORDCOUNT    <= "000000";

EXPWCNT      <= "000000";

CWRT1        <= "00000";

CWRT2        <= "00000";

IMGDONE      <= '1';

RTTIMEOUT    <= '0';

RT_NORESP    <= '0';

RT_ERROR     <= '0';

GOTSW1       <= '0';

GOTSW2       <= '0';

SW_ERROR     <= '0';

SW_TFLAG     <= '0';

SW_DBA       <= '0';

SW_SSFLAG    <= '0';

SW_BUSY      <= '0';

SW_BCAST     <= '0';

SW_SREQ      <= '0';

SW_INSTRU    <= '0';

SW_MSGERR    <= '0';

RTSTATE      <= RTIDLE;

WCNTTO       <= '0';

WCNTOVR      <= '0';

PCONBUSY     <= '0';

LATCHSW      <= '0';

RTCMDDONE    <= '0';

CLRERR       <= '0';

SMDC_INDX    <= ( others => '0');

SMDC_INTX    <= '0';

SMDC_IWA     <= '0';

SMDC_IBRD    <= '0';

SMDC_BAC     <= '0';

SMDC_BUFA    <= '0';

SMDC_BRD     <= '0';

SMDC_NII     <= '0';

SMD_DPTR1    <= ( others => '0');

SMD_DPTR2    <= ( others => '0');

SMD_DPTR3    <= ( others => '0');

SMR0_STEX    <= '0';

SMR0_SBIT    <= '0';

SMR0_SRST    <= '0';

SMR0_CHAEN   <= '0';

SMR0_CHBEN   <= '0';

SMR0_ETCE    <= '0';

SMR0_MTO     <= '0';

SMR0_EMODE   <= "00";

SMR0_BUFR    <= '0';

SMR0_SMTC    <= '0';

SMR0_BCEN    <= '0';

SMR0_DYNBC   <= '0';

SMR0_PPEN    <= '0';

SMR0_PPON    <= '0';

SMR0_INTEN   <= '0';

SMR0_XMTSW   <= '0';

SMR3_DMAF    <= '0';

SMR3_WRAPF   <= '0';

SMR3_TAPF    <= '0';

SMR3_BITF    <= '0';

SMR3_MERR    <= '0';

SMR3_SUBAD   <= '0';

SMR3_BDRCV   <= '0';

SMR3_IXEQ0   <= '0';

SMR3_EOL     <= '0';

SMR3_ILLCMD7 <= '0';

SMR3_EOL     <= '0';

SMR3_ILLCMD4 <= '0';

SMR3_ILLOP   <= '0';

SMR3_RTF     <= '0';

SMR3_CBA     <= '0';

SMR3_MBC     <= '0';

SMR4_DMAF    <= '0';

SMR4_WRAPF   <= '0';

SMR4_TAPF    <= '0';

SMR4_BITF    <= '0';

SMR4_MERR    <= '0';

SMR4_SUBAD   <= '0';

SMR4_BDRCV   <= '0';

SMR4_IXEQ0   <= '0';

SMR4_EOL     <= '0';

SMR4_ILLCMD7 <= '0';

SMR4_EOL     <= '0';

SMR4_ILLCMD4 <= '0';

SMR4_ILLOP   <= '0';

SMR4_RTF     <= '0';

SMR4_CBA     <= '0';

SMR4_MBC     <= '0';

SMR4_INTR    <= ( others => '0');

SMR5_INTA    <= ( others => '0');

SMR8_PREG    <= ( others => '0');

DATAPTR      <= ( others => '0');

BASEPTR      <= ( others => '0');

DESCPTR      <= ( others => '0');

CAF          <= ( others => '0');

WCNTOVR      <= '0';

WCNTTO       <= '0';

WINT         <= '0';

INTLOG       <= '0';

SMCW_OPCODE  <= "0000";

SMCW_RETRY   <= "00";

SMCW_CHA     <= '0';

SMCW_RTRT    <= '0';

SMCW_CCODE   <= "0000000";

ATTEMPTS     <= "000";

NODATA       <= '0';

MODECODE     <= '0';

TRANSMIT     <= '0';

BCBUSSEL     <= '0';

SA           <= "00000";

M2MTIMER     <= ( others => '0');

BCLOADTT     <= '0';

DOINT        <= '0';

MTSTATE      <= MTIDLE;

MTMI_TOUT    <= '0';

INTSTATE     <= IDLE;

MTDATAPTR    <= ( others => '0');

MTDESCPTR    <= ( others => '0');

MTWORDCOUNT  <= ( others => '0');

MTCOUNT      <= ( others => '0');

MTCOUNTHD    <= ( others => '0');

MTMI_OVR     <= '0';

MTMI_RTRT    <= '0';

MTEXPWCNT    <= ( others => '0');

RTACTIVE     <= '0';

MONHIT       <= '0';

EXPECTSW     <= '0';

INTREAD      <= '0';

IMCOUNT      <= "00";

MTINT        <= '0';

CWRT1_LB     <= ( others => '0');

CWRT2_LB     <= ( others => '0');

TTTEMP       <= ( others => '0');

elsif CLK'event and CLK='1' then

-- TIMER, Counts held when Core is inactive

if CLKSTB1US='1' and DECACTIVE='0' then

RTTIMER <= RTTIMER + 1;

end if;

if DECACTIVE='1' or iENCSTROBE='1' then

RTTIMER   <= "00000";

IMGDONE   <= '0';

RTTIMEOUT <= '0';

else

if RTTIMER=RESPVALUE then

RTTIMEOUT <= '1';

end if;

if RTTIMER=IMGVALUE then

IMGDONE <= '1';

end if;

end if;

-- Default Actions

iBENDREQ   <= '0';

BENDWAIT   := iBENDREQ or BENDBUSY;

iENCSTROBE <= '0';

RTCMDDONE  <= '0';

CLRERR     <= '0';

BCLOADTT   <= '0';

-- CPU Registers

if CPUWRR(1)='1' and CPUADDRR="000000"  then

SMR0_STEX     <= CPUDINR(15) and not CPUDINR(14);  -- BIT has priority

SMR0_SBIT     <= CPUDINR(14);                      -- these bits can be written when BUSY

SMR0_SRST     <= CPUDINR(13);

end if;

if CPUWRR(1)='1' and CPUADDRR="000000" and SMR0_STEX='0' then

SMR0_CHAEN    <= CPUDINR(12);               -- Must be inactive to change these bits

SMR0_CHBEN    <= CPUDINR(11);

SMR0_ETCE     <= CPUDINR(10);

SMR0_MTO      <= CPUDINR(9);

SMR0_EMODE(1) <= CPUDINR(8);

end if;

if CPUWRR(0)='1' and CPUADDRR="000000" and SMR0_STEX='0' then

SMR0_EMODE(0) <= CPUDINR(7);                -- Must be inactive to change these bits

SMR0_BUFR     <= CPUDINR(6);

SMR0_SMTC     <= CPUDINR(5);

SMR0_BCEN     <= CPUDINR(4);

SMR0_DYNBC    <= CPUDINR(3);

SMR0_PPEN     <= CPUDINR(2);

SMR0_PPON     <= CPUDINR(2);

SMR0_INTEN    <= CPUDINR(1);

SMR0_XMTSW    <= CPUDINR(0);

end if;

-- SAR52998  If started then set/reset the PPON bit on writes to the register instead of PPEN

-- if write pulse is multiple cycles initially this bit will be set with PPEN thats okay

if CPUWRR(0)='1' and CPUADDRR="000000" and SMR0_STEX='1' and SMR0_PPEN='1' then

SMR0_PPON     <= CPUDINR(2);

end if;

if iRTMODE='1' and RTADDRERR='1' then

SMR0_STEX <= '0';

end if;

if CPUWRR(1)='1' and CPUADDRR="000011" then

SMR3_DMAF  <= CPUDINR(15);

SMR3_WRAPF <= CPUDINR(14);

SMR3_TAPF  <= CPUDINR(13);

SMR3_BITF  <= CPUDINR(12);

SMR3_MERR  <= CPUDINR(11);

SMR3_SUBAD <= CPUDINR(10);

SMR3_BDRCV <= CPUDINR(9);

SMR3_IXEQ0 <= CPUDINR(8);

end if;

if CPUWRR(0)='1' and CPUADDRR="000011" then

SMR3_ILLCMD7 <= CPUDINR(7);

SMR3_EOL     <= CPUDINR(5);

SMR3_ILLCMD4 <= CPUDINR(4);

SMR3_ILLOP   <= CPUDINR(3);

SMR3_RTF     <= CPUDINR(2);

SMR3_CBA     <= CPUDINR(1);

SMR3_MBC     <= CPUDINR(0);

end if;

if CPURDR='1' and CPUADDRR="000100"  then

INTREAD <= '1';

elsif INTREAD='0' then

SMR4_INTR <= SMR4_INTRP;

end if;

if CPUCLRINT='1' and INTREAD='1' then

INTREAD      <= '0';

SMR4_DMAF    <= SMR4_DMAF    and not SMR4_INTR(15); -- clear if previously read

SMR4_WRAPF   <= SMR4_WRAPF   and not SMR4_INTR(14);

SMR4_TAPF    <= SMR4_TAPF    and not SMR4_INTR(13);

SMR4_BITF    <= SMR4_BITF    and not SMR4_INTR(12);

if SMR0_INTEN='0' then  -- If log enabled do not clear these bits

SMR4_MERR    <= SMR4_MERR    and not SMR4_INTR(11);

SMR4_SUBAD   <= SMR4_SUBAD   and not SMR4_INTR(10);

SMR4_BDRCV   <= SMR4_BDRCV   and not SMR4_INTR(9);

SMR4_IXEQ0   <= SMR4_IXEQ0   and not SMR4_INTR(8);

SMR4_ILLCMD7 <= SMR4_ILLCMD7 and not SMR4_INTR(7);

-- Bit 6 is not used

SMR4_EOL     <= SMR4_EOL     and not SMR4_INTR(5);

SMR4_ILLCMD4 <= SMR4_ILLCMD4 and not SMR4_INTR(4);

SMR4_ILLOP   <= SMR4_ILLOP   and not SMR4_INTR(3);

SMR4_RTF     <= SMR4_RTF     and not SMR4_INTR(2);

SMR4_CBA     <= SMR4_CBA     and not SMR4_INTR(1);

SMR4_MBC     <= SMR4_MBC     and not SMR4_INTR(0);

end if;

end if;

if CPUWRR(1)='1' and CPUADDRR="000101" then

SMR5_INTA(15 downto 8) <= CPUDINR(15 downto 8);

end if;

if CPUWRR(0)='1' and CPUADDRR="000101" then

SMR5_INTA(7 downto 1) <= CPUDINR(7 downto 1);

end if;

if CPUWRR(1)='1' and CPUADDRR="001000" then

SMR8_PREG(15 downto 8) <= to_unsigned(CPUDINR(15 downto 8));

end if;

if CPUWRR(0)='1' and CPUADDRR="001000" then

SMR8_PREG(7 downto 0)  <= to_unsigned(CPUDINR(7 downto 0));

end if;

if CPUWRR(1)='1' and CPUADDRR="001101" then

MTCOUNTHD(15 downto 8) <= to_unsigned(CPUDINR(15 downto 8));

MTCOUNT  (15 downto 8) <= to_unsigned(CPUDINR(15 downto 8));

end if;

if CPUWRR(0)='1' and CPUADDRR="001101" then

MTCOUNTHD(7 downto 0) <= to_unsigned(CPUDINR(7 downto 0));

MTCOUNT  (7 downto 0) <= to_unsigned(CPUDINR(7 downto 0));

end if;

-- External Interrupt Settings

if LOOPFAIL='1' then

RT_ERROR   <= '1';

SMR4_WRAPF <= SMR3_WRAPF;

end if;

if BENDFAIL='1' then

SMR4_DMAF <= SMR3_DMAF;

end if;

SMR4_TAPF <= SMR3_TAPF and RTADDRERR and iRTMODE;

SMR4_BITF <= '0';    -- BIT not implemented therefore always 0

-- Instruction State Machine Bus Controller

if ACT_ENHANCE='0' or SMCW_RETRY(1)='0' then

CALLOKAY := '1';

else

CALLOKAY := not SMCW_RETRY(0);

end if;

if iBCMODE='1' then

case BCSTATE is

when BCIDLE  =>      iBENDBURST <= '0';

PCONBUSY   <=  SMR0_STEX;

if SMR0_STEX='1' then

BCSTATE <= BCILIST;

DESCPTR <= SMR8_PREG;

end if;

when BCILIST =>      if BENDWAIT='0' then

iBENDREQ   <= '1';

iBENDWRT   <= '0';

iBENDBURST <= '1';

BENDADDR   <= to_std_logic(DESCPTR+0);

BCSTATE    <= BCGETOPCODE;

end if;

when BCGETOPCODE =>  if BENDWAIT='0' then

SMCW_OPCODE <= BENDDIN(15 downto 12);

SMCW_RETRY  <= BENDDIN(11 downto 10);

SMCW_CHA    <= BENDDIN(9);

SMCW_RTRT   <= BENDDIN(8);

SMCW_CCODE  <= BENDDIN(7 downto 1);

BCBUSSEL    <= not BENDDIN(9);

ATTEMPTS    <= to_unsigned( (not BENDDIN(11) and not BENDDIN(10)) & BENDDIN(11 downto 10));

iBENDREQ    <= '1';

iBENDWRT    <= '0';

iBENDBURST  <= '0';

BCSTATE     <= BCDECOPCODE;

end if;

when BCDECOPCODE =>  if BENDWAIT='0' then

SMR8_PREG  <= SMR8_PREG+8;

case SMCW_OPCODE is

when "0000" => -- End of List

BCSTATE   <= BCMSGDONE;

--SMR0_STEX <= '0';	  -- v2.16 moved to the end

SMR4_EOL  <= SMR3_EOL;

SMR8_PREG <= SMR8_PREG; -- dont increment

when "0001" => -- SKIP and wait for MMT

iBENDREQ   <= '1';

iBENDWRT   <= '0';

BENDADDR   <= to_std_logic(DESCPTR+7);

BCSTATE    <= BCLOADMMT;

when "0010" => -- GOTO Branch Address

if CALLOKAY='1' then

iBENDREQ   <= '1';

iBENDWRT   <= '0';

BENDADDR   <= to_std_logic(DESCPTR+6);

BCSTATE    <= BCGOTO;

else

BCSTATE    <= BCIDLE;

end if;

when "0011" => -- Build in Test

BCSTATE    <= BCMSGDONE;

CLRERR     <= '1';

when "0100" => -- Execute Block

BCSTATE <= BCSTARTM;

when "0101" => -- Execute Block and Branch

BCSTATE <= BCSTARTM;

when "0110" => -- Exceute Block and Branch on Conition

BCSTATE <= BCSTARTM;

when "0111" => -- Retry and Execute Block

BCSTATE <= BCSTARTM;

when "1000" => -- Retry on condition and Branch

BCSTATE <= BCSTARTM;

when "1001" => -- Retry on condition and Branch if failed

BCSTATE <= BCSTARTM;

when "1010" => -- Interrupt and Continue

SMR4_CBA  <= SMR3_CBA;

BCSTATE   <= BCMSGDONE;

when "1011" => -- Call

if CALLOKAY='1' then

iBENDREQ   <= '1';

iBENDWRT   <= '0';

BENDADDR   <= to_std_logic(DESCPTR+6);

BCSTATE    <= BCCALL;

else

BCSTATE    <= BCIDLE;

end if;

when "1100" => -- Return

SMR8_PREG  <= SMD_DPTR1;

BCSTATE    <= BCIDLE;

when "1110" => -- Load Minor Frame Timer

iBENDREQ   <= '1';

iBENDWRT   <= '0';

BENDADDR   <= to_std_logic(DESCPTR+7);

BCSTATE    <= BCLOADMFT;

when "1111" => -- Return to Branch Address

SMR8_PREG  <= SMD_DPTR2;

BCSTATE    <= BCIDLE;

when others => -- Illegal

SMR4_ILLOP <= SMR3_ILLOP;

SMR0_STEX  <= '0';

BCSTATE    <= BCMSGDONE;

end case;

end if;

when BCLOADMMT  =>  if BENDWAIT='0' then

M2MTIMER <= to_unsigned(BENDDIN);

BCSTATE  <= BCWAITMMT;

end if;

when BCWAITMMT  =>  M2MTIMER <= M2MTIMER -1;

if M2MTIMER=0 then

BCSTATE <= BCMSGDONE;

end if;

when BCGOTO     =>  if BENDWAIT='0' then

SMR8_PREG <= to_unsigned(BENDDIN);

if ACT_ENHANCE='1' then

BCSTATE       <= BCWRTTSW;

SMCW_RETRY(0) <= '1';

else

BCSTATE <= BCMSGDONE;

end if;

end if;

when BCCALL     =>  if BENDWAIT='0' then

SMD_DPTR1  <= SMR8_PREG;   -- share RT registers

SMR8_PREG  <= to_unsigned(BENDDIN);

if ACT_ENHANCE='1' then

BCSTATE       <= BCWRTTSW;

SMCW_RETRY(0) <= '1';

else

BCSTATE <= BCMSGDONE;

end if;

end if;

when BCLOADMFT  =>  if BENDWAIT='0' and TTZERO='1' then

BCLOADTT <= '1';

BCSTATE  <= BCMSGDONE;

end if;

when BCSTARTM   =>  GOTSW1      <= '0';

GOTSW2      <= '0';

WORDCOUNT   <= "000000";

iBENDREQ    <= '1';

iBENDWRT    <= '0';

iBENDBURST  <= '0';

BENDADDR    <= to_std_logic(DESCPTR+1);

iBENDREQ    <= '1';

iBENDWRT    <= '0';

RT_NORESP   <= '0';

RT_ERROR    <= '0';

SW_TFLAG    <= '0';

SW_DBA      <= '0';

SW_SSFLAG   <= '0';

SW_BUSY     <= '0';

SW_BCAST    <= '0';

SW_SREQ     <= '0';

SW_INSTRU   <= '0';

SW_MSGERR   <= '0';

SW_ERROR    <= '0';

BCSTATE     <= BCGETCW1;

when BCGETCW1   =>  if BENDWAIT='0' and IMGDONE='1' then

CWRT1  <= BENDDIN(15 downto 11);

CWRT2  <= BENDDIN(15 downto 11);

SA       <= BENDDIN(9 downto 5);

BCAST    <= to_logic( BENDDIN(15 downto 11) = "11111") and SMR0_BCEN;

TRANSMIT <= BENDDIN(10);

if (   BENDDIN(9 downto 5)="00000"

or (SMR1_STDA='0' and BENDDIN(9 downto 5)="11111"))  then

MODECODE <= '1';

if SMR1_STDA='0' then

NODATA   <= not BENDDIN(4);

EXPWCNT  <= "00000" & BENDDIN(4);

else

NODATA   <= '1';

EXPWCNT  <= "000000";

end if;

else

MODECODE <= '0';

NODATA   <= '0';

if BENDDIN(4 downto 0)="00000" then

EXPWCNT <= to_unsigned('1' & BENDDIN(4 downto 0));

else

EXPWCNT <= to_unsigned('0' & BENDDIN(4 downto 0));

end if;

end if;

if FORCEORUN='1' then

EXPWCNT(5) <= '1';

end if;

BCSTATE <= BCCHECKCW;

end if;

when BCCHECKCW   => if   (BCAST='1' and MODECODE='0' and TRANSMIT='1')

or (SMCW_RTRT='1' and TRANSMIT='1')

then

BCSTATE      <= BCMSGDONE;    -- Illegal Command Word

SMR0_STEX    <= '0';

SMR4_ILLCMD4 <= SMR3_ILLCMD4;

else

BCSTATE <= BCTXCW;

end if;

when BCTXCW      => iENCCMDSTAT <= '1';

iENCSTROBE  <= '1';

if SMCW_RTRT='1' then

iBENDREQ    <= '1';    -- Read CW2

iBENDWRT    <= '0';

BENDADDR    <= to_std_logic(DESCPTR+2);

BCSTATE     <= BCGETCW2;

elsif MODECODE='1' and NODATA='1' then

if BCAST='1' then

BCSTATE <= BCWAITBQ;

else

BCSTATE <= BCWAITSW2;

end if;

else

iBENDREQ  <= '1';    -- Read DPTR

iBENDWRT  <= '0';

BENDADDR  <= to_std_logic(DESCPTR+3);

BCSTATE   <= BCGETDPTR;

end if;

when BCGETCW2   => if BENDWAIT='0' then

CWRT1       <= BENDDIN(15 downto 11);

iENCCMDSTAT <= '1';

iENCSTROBE  <= '1';

iBENDREQ    <= '1';

iBENDWRT    <= '0';

BENDADDR    <= to_std_logic(DESCPTR+3);

BCSTATE     <= BCGETDPTR;

if BENDDIN(10)='0' or ( SMR0_BCEN='1' and BENDDIN(15 downto 11)="11111") then   -- check transmit bit is set

BCSTATE      <= BCMSGDONE;                               -- and not BCAST

SMR0_STEX    <= '0';

SMR4_ILLCMD4 <= SMR3_ILLCMD4;

end if;

end if;

when BCGETDPTR  => if BENDWAIT='0' then

DATAPTR <= to_unsigned(BENDDIN);

if TRANSMIT='0' and SMCW_RTRT='0' then

BCSTATE <= BCRDDATA1;

else

BCSTATE <= BCWAITSW1;

end if;

end if;

when BCWAITSW1   => if DECSTBOKSW='1' then

BCSTATE   <= BCWRDATA;

GOTSW1    <= '1';

iBENDREQ  <= '1';

iBENDWRT  <= '1';

BENDADDR  <= to_std_logic(DESCPTR+4);

BENDDOUT  <= DECDATA;

SW_TFLAG  <= SW_TFLAG  or DECDATA(0);

SW_DBA    <= SW_DBA    or DECDATA(1);

SW_SSFLAG <= SW_SSFLAG or DECDATA(2);

SW_BUSY   <= SW_BUSY   or DECDATA(3);

SW_BCAST  <= SW_BCAST  or DECDATA(4);

SW_SREQ   <= SW_SREQ   or DECDATA(8);

SW_INSTRU <= SW_INSTRU or DECDATA(9);

SW_MSGERR <= SW_MSGERR or DECDATA(10);

if DECDATA(15 downto 11) /= CWRT1 then

BCSTATE  <= BCCHECK;

SW_ERROR <= '1';

end if;

elsif DECSTB='1' then

BCSTATE   <= BCCHECK;

RT_ERROR  <= '1';

elsif DECACTIVE='0' and RTTIMEOUT='1' and iENCSTROBE='0' then

BCSTATE   <= BCCHECK;

RT_NORESP <= '1';

end if;

when BCWRDATA    => if DECSTBOKDW='1' and (WORDCOUNT<EXPWCNT) then

iBENDREQ  <= '1';

iBENDWRT  <= '1';

BENDADDR  <= to_std_logic(DATAPTR);

DATAPTR   <= DATAPTR+1;

BENDDOUT  <= DECDATA;

WORDCOUNT <= WORDCOUNT+ 1;

elsif DECSTB='1' then

BCSTATE   <= BCCHECK;

RT_ERROR  <= '1';

elsif DECACTIVE='0' then

if WORDCOUNT/=EXPWCNT then

RT_ERROR <= '1';

end if;

if SMCW_RTRT='1' and BCAST='0' then

BCSTATE <= BCWAITSW2;

else

BCSTATE <= BCWAITBQ;

end if;

end if;

when BCRDDATA1  =>  iBENDREQ  <= '1';

iBENDWRT  <= '0';

BENDADDR  <= to_std_logic(DATAPTR);

DATAPTR   <= DATAPTR+1;

BCSTATE   <= BCRDDATA2;

WORDCOUNT <= WORDCOUNT+1;

when BCRDDATA2   => if BENDWAIT='0' and ENCREADY='1' then

iENCCMDSTAT <= '0';

iENCSTROBE  <= '1';

if WORDCOUNT/=EXPWCNT then

BCSTATE <= BCRDDATA1;

else

if BCAST='1' then

BCSTATE <= BCWAITBQ;

else

BCSTATE <= BCWAITSW2;

end if;

end if;

end if;

when BCWAITSW2   => if DECSTBOKSW='1' then

BCSTATE   <= BCWAITBQ;

iBENDREQ  <= '1';

iBENDWRT  <= '1';

BENDDOUT  <= DECDATA;

SW_TFLAG  <= SW_TFLAG  or DECDATA(0);

SW_DBA    <= SW_DBA    or DECDATA(1);

SW_SSFLAG <= SW_SSFLAG or DECDATA(2);

SW_BUSY   <= SW_BUSY   or DECDATA(3);

SW_BCAST  <= SW_BCAST  or DECDATA(4);

SW_SREQ   <= SW_SREQ   or DECDATA(8);

SW_INSTRU <= SW_INSTRU or DECDATA(9);

SW_MSGERR <= SW_MSGERR or DECDATA(10);

if DECDATA(15 downto 11) /= CWRT2 then

SW_ERROR <= '1';

end if;

if SMCW_RTRT='0' then

GOTSW1    <= '1';

BENDADDR  <= to_std_logic(DESCPTR+4);

else

GOTSW2    <= '1';

BENDADDR  <= to_std_logic(DESCPTR+5);

end if;

elsif DECSTB='1' then

BCSTATE   <= BCCHECK;

RT_ERROR  <= '1';

elsif DECACTIVE='0' and RTTIMEOUT='1' and iENCSTROBE='0' then

BCSTATE    <= BCCHECK;

RT_NORESP  <= '1';

end if;

when BCWAITBQ    => if DECSTB='1' then     -- if any extra words discard and error

BCSTATE   <= BCCHECK;

RT_ERROR  <= '1';

elsif DECACTIVE='0' and ENCBUSY='0' then

BCSTATE <= BCCHECK;

end if;

when BCCHECK     => BCSTATE <= BCWRTTSW;

case SMCW_OPCODE is

when "0111" => -- Retry on Condition

if CCFLAG='1' then

BCSTATE <= BCRETRY1;

end if;

when "1000" => -- Retry on Condition & Branch

if CCFLAG='1' then

BCSTATE <= BCRETRY1;

end if;

when "1001" => -- Retry on Condition & Branch if fails

if CCFLAG='1' then

BCSTATE <= BCRETRY1;

end if;

when others => BCSTATE <= BCWRTTSW;

end case;

if MSG_ERROR='1' then

SMR4_MERR <= SMR3_MERR;

end if;

when BCRETRY1    => if ATTEMPTS="000" then

BCSTATE   <= BCWRTTSW;

if MSG_ERROR='1' then

SMR4_RTF  <= SMR3_RTF; -- Final retry failed so set the flag

end if;

else

ATTEMPTS <= ATTEMPTS - 1;

BCSTATE  <= BCSTARTM;

if SMR0_PPEN='1' then

BCBUSSEL <= not BCBUSSEL;

end if;

end if;

when BCWRTTSW    => if BENDWAIT='0' and ENCBUSY='0' then

iBENDREQ  <= '1';

iBENDWRT  <= '1';

BENDADDR  <= to_std_logic(DESCPTR);

BENDDOUT  <= SMCW_OPCODE & SMCW_RETRY & SMCW_CHA & SMCW_RTRT

& SMCW_CCODE & MSG_ERROR;

BCSTATE   <= BCMSGDONE;

end if;

when BCMSGDONE  => if BENDWAIT='0' and ENCBUSY='0' then

BCSTATE <= BCGENINT;

case SMCW_OPCODE is

when "0101" => -- Execute Block and Branch

iBENDREQ <= '1';

iBENDWRT <= '0';

BENDADDR <= to_std_logic(DESCPTR+6);

BCSTATE  <= BCGETBR;

when "0110" => -- Execute Block and Conditionally Branch

if CCFLAG='1' then

iBENDREQ <= '1';

iBENDWRT <= '0';

BENDADDR <= to_std_logic(DESCPTR+6);

BCSTATE  <= BCGETBR;

end if;

when "1000" => -- Retry on condition and Branch

iBENDREQ <= '1';

iBENDWRT <= '0';

BENDADDR <= to_std_logic(DESCPTR+6);

BCSTATE  <= BCGETBR;

when "1001" => -- Retry on condition and Branch if failed

if CCFLAG='1' then

iBENDREQ <= '1';

iBENDWRT <= '0';

BENDADDR <= to_std_logic(DESCPTR+6);

BCSTATE  <= BCGETBR;

end if;

when others => BCSTATE  <= BCGENINT;

end case;

end if;

when BCGETBR    =>  if BENDWAIT='0' then

SMD_DPTR2  <= SMR8_PREG;   -- share RT register

SMR8_PREG  <= to_unsigned(BENDDIN);

BCSTATE    <= BCGENINT;

end if;

when BCGENINT   =>  DOINT     <= '1';

BCSTATE   <= BCALLDONE;

RTCMDDONE <= '1';  -- clears LOOPFAIL in encoders

when BCALLDONE  =>  if BENDWAIT='0' and DOINT='0' then

BCSTATE    <= BCIDLE;

if SMCW_OPCODE="0000" then		-- v2.16

SMR0_STEX  <= '0';

end if;

end if;

end case;

if ABORT='1' or BENDFAIL='1' then

ABORT      <= '0';

BCSTATE    <= BCIDLE;

SMR0_STEX  <= '0';

RTCMDDONE  <= '1';  -- clears LOOPFAIL in encoders

iBENDBURST <= '0';

end if;

end if;

-- Some Variable Assignments

RTBUFMODE := ( SMR0_PPEN & SMR0_EMODE);

case RTBUFMODE is

when "100"  => BUFMODE := PPONG;

when "001"  => BUFMODE := INDEX;

when "101"  => BUFMODE := PPONG;

when "010"  => BUFMODE := CIRC1;

when "110"  => BUFMODE := CIRC1;

when "011"  => BUFMODE := CIRC2;

when "111"  => BUFMODE := CIRC2;

when others => BUFMODE := INDEX;

end case;

if iRTMODE='1' then

LATCHSW  <= '0';

ABORT    <= '0';

RTMUXSEL := CW_BCAST & CW_TX & CW_DATA;

if CW_STBME='1' then  -- Second command is received need to abort operation

RTSTATE <= RTIDLE;

ABORT   <= '1';

elsif RTSTART='1' and SMR0_STEX='1' then

RTSTATE <= RTINIT1;

TTTEMP  <= SMR7_TT;

else

case RTSTATE is

when RTIDLE =>  PCONBUSY   <= '0';

when RTINIT1 => -- Clear various things

INTSTATE  <= IDLE;        -- must abort any pending interrupt log write

if INTLOG='1' then        -- If interrupt log set make sure nothing left set from the previous message

SMR4_MERR    <= '0';   -- Clear Just the RT inturrupts

SMR4_SUBAD   <= '0';

SMR4_BDRCV   <= '0';

SMR4_IXEQ0   <= '0';

SMR4_ILLCMD7 <= '0';

end if;

DOINT     <= '0';

MTINT     <= '0';        -- if in RTMT mode and second message is RT clear MTINT request

PCONBUSY  <= '1';

WORDCOUNT <= "000000";

EXPWCNT   <= to_unsigned(CW_COUNT);

if FORCEORUN='1' then

EXPWCNT(5) <= '1';   -- force to many words

end if;

WINT    <= '0';

WCNTOVR <= '0';

WCNTTO  <= '0';

CLRERR  <= '1';

case RTMUXSEL is

when "000" => RTSTATE  <= RTGETD1;        -- RX Mode Code

when "010" => RTSTATE  <= RTTXSTAT;       -- TX Mode Code

when "001" => RTSTATE  <= RTGETD1;        -- RX Data Transfer

when "011" => RTSTATE  <= RTTXSTAT;       -- TX Data Transfer

when "100" => RTSTATE  <= RTGETD1;        -- Bcast RX Mode Code

when "110" => RTSTATE  <= RTGETD1;        -- Bcast TX Mode Code

LATCHSW  <= '1';

when "101" => RTSTATE  <= RTGETD1;        -- Bcast RX Data Transfer

when "111" => RTSTATE  <= RTGETD1;        -- Bcast TX Data Transfer

LATCHSW  <= '1';

when others =>

end case;

DESCPTR <= SMR8_PREG + to_unsigned(( ZERO(15 downto 9) & CW_DOFFSET & "00"));

DATAPTR <= SMR8_PREG + to_unsigned(( ZERO(15 downto 9) & CW_DOFFSET & "00"));

when RTTXSTAT=> if DECSTBDEL1='1' and (RTRTRT='0' or RTRTOK='0') then   -- if another word after TX Command then error

RTSTATE     <= RTGETD1;              -- need to get pointers before MSGSTAT

WCNTOVR     <= '1';

LATCHSW     <= '1';

elsif DECACTIVE='0' then

iENCSTROBE  <= '1';                  -- Transmit the SW

iENCCMDSTAT <= '1';

LATCHSW     <= '1';

RTSTATE     <= RTGETD1;

end if;

when RTGETD1=>  if BENDWAIT='0' then                   -- Read the Descriptor

iBENDREQ   <= '1';

iBENDWRT   <= '0';

BENDADDR   <= to_std_logic(DATAPTR);

DATAPTR    <= DATAPTR + 1;

iBENDBURST <= '1';

RTSTATE    <= RTGETD2;

end if;

when RTGETD2=>  if BENDWAIT='0' then

SMDC_INDX  <= to_unsigned(BENDDIN(15 downto 8));

SMDC_INTX  <= BENDDIN(7);

SMDC_IWA   <= BENDDIN(6);

SMDC_IBRD  <= BENDDIN(5);

SMDC_BAC   <= BENDDIN(4);

SMDC_BUFA  <= BENDDIN(2);

SMDC_BRD   <= BENDDIN(1);

SMDC_NII   <= BENDDIN(0);

iBENDREQ   <= '1';

iBENDWRT   <= '0';

BENDADDR   <= to_std_logic(DATAPTR);

DATAPTR    <= DATAPTR + 1;

iBENDBURST <= '1';

RTSTATE    <= RTGETD3;

end if;

when RTGETD3=>  if BENDWAIT='0' then

SMD_DPTR1  <= to_unsigned(BENDDIN);

iBENDREQ   <= '1';

iBENDWRT   <= '0';

BENDADDR   <= to_std_logic(DATAPTR);

DATAPTR    <= DATAPTR + 1;

iBENDBURST <= '1';

RTSTATE    <= RTGETD4;

end if;

when RTGETD4=>  if BENDWAIT='0' then

SMD_DPTR2  <= to_unsigned(BENDDIN);

iBENDREQ   <= '1';

iBENDWRT   <= '0';

BENDADDR   <= to_std_logic(DATAPTR);

DATAPTR    <= DATAPTR + 1;

iBENDBURST <= '0';

RTSTATE    <= RTGETD5;

end if;

when RTGETD5=>  if BENDWAIT='0' then

SMD_DPTR3  <= to_unsigned(BENDDIN);

RTSTATE    <= RTINIT2;

end if;

when RTINIT2 => case BUFMODE is

when PPONG => if SMDC_BUFA='1' then

BASEPTR <= SMD_DPTR2;

DATAPTR <= SMD_DPTR2+2;

else

BASEPTR <= SMD_DPTR1;

DATAPTR <= SMD_DPTR1+2;

end if;

if CW_BCAST='1' and SMDC_NII='1' then

BASEPTR <= SMD_DPTR3;

DATAPTR <= SMD_DPTR3+2;

end if;

when INDEX => if CW_BCAST='1' and SMDC_NII='1' then

BASEPTR <= SMD_DPTR3;

DATAPTR <= SMD_DPTR3+2;

else

BASEPTR <= SMD_DPTR1;

DATAPTR <= SMD_DPTR1+2;

end if;

when CIRC1 => BASEPTR <= SMD_DPTR2;

DATAPTR <= SMD_DPTR2+2;

when CIRC2 => BASEPTR <= SMD_DPTR3;

DATAPTR <= SMD_DPTR2;

CAF     <= SMD_DPTR3(7 downto 0);

end case;

case RTMUXSEL is

when "000" => RTSTATE  <= RTWRITE1;         -- RX Mode Code

when "010" =>                               -- TX Mode Code

if RTBUSY='1' or LEGAL='0' then         -- If BUSY just we send the SW

RTSTATE <= RTMSGSTAT;                 --  or command is illegal

else

RTSTATE <= RTREAD2;

end if;

when "001" => RTSTATE  <= RTWRITE1;                  -- RX Data Transfer

when "011" =>                                        -- TX Data Transfer

if RTBUSY='1' or LEGAL='0' then        -- If BUSY just we send the SW

RTSTATE <= RTMSGSTAT;                --  or command is illegal

else

RTSTATE <= RTREAD2;

end if;

when "100" => RTSTATE  <= RTWRITE1;       -- Bcast RX Mode Code

when "110" => RTSTATE  <= RTMSGSTAT;      -- Bcast TX Mode Code

when "101" => RTSTATE  <= RTWRITE1;       -- Bcast RX Data Transfer

when "111" => RTSTATE  <= RTMSGSTAT;      -- Bcast TX Data Transfer

when others =>

end case;

if WCNTOVR='1' then    --  TXSTAT failed we now have the pointers etc

RTSTATE <= RTMSGSTAT;

else

end if;

when RTREAD1 => if BENDWAIT='0' and ENCREADY='1' then

iENCCMDSTAT <= '0';                    -- Transmit a data word

iENCSTROBE  <= '1';

RTSTATE     <= RTREAD2;

end if;

when RTREAD2 => if WORDCOUNT=EXPWCNT then

RTSTATE <= RTMSGSTAT;

elsif BENDWAIT='0' then

RTSTATE  <= RTREAD1;

iBENDREQ <= LEGAL and not RTBUSY;

iBENDWRT <= '0';

BENDADDR <= to_std_logic(DATAPTR);

WORDCOUNT<= WORDCOUNT + 1;

DATAPTR  <= DATAPTR + 1;

end if;

when RTWRITE1=> if (RTRTRT='0' or GOTSW='1') then         -- must have the SW for RTRT

if DECACTIVE='0' then                   -- end of transmission

if WORDCOUNT > EXPWCNT  then         -- Check count

WCNTOVR <= '1';

RTSTATE <= RTMSGSTAT;

elsif WORDCOUNT < EXPWCNT  then      -- Check count

WCNTTO  <= '1';

RTSTATE <= RTMSGSTAT;

else

RTSTATE <= RTSTATRX;

end if;

LATCHSW <= '1';

elsif DECSTBOKDW='1' then

iBENDREQ    <= LEGAL and not RTBUSY;

iBENDWRT    <= '1';

BENDADDR    <= to_std_logic(DATAPTR);

BENDDOUT    <= DECDATA;

DATAPTR     <= DATAPTR+1;

WORDCOUNT   <= WORDCOUNT+1;

elsif DECSTBDEL1='1' then

RTSTATE  <= RTMSGSTAT;                  -- Bad Transfer

LATCHSW  <= '1';

WCNTTO   <= '1';

end if;

end if;

if TIMER57US='1'  then                       -- No data received at 67us

WCNTTO  <= '1';

LATCHSW <= '1';

RTSTATE <= RTMSGSTAT;                     -- Send the SW

end if;

when RTSTATRX =>LATCHSW <= '1';

if CW_BCAST='1'  then

RTSTATE  <= RTMSGSTAT;

elsif DECACTIVE='0' then

iENCCMDSTAT <= '1';                        -- transmit SW

iENCSTROBE  <= '1';

RTSTATE     <= RTMSGSTAT;

end if;

when RTMSGSTAT=>if ENCBUSY='0' and iENCSTROBE='0' then

RTSTATE <= RTPUTCW;

DATAPTR <= BASEPTR;

if RTMSG_OKAY='1' then

iENCCMDSTAT <= SMDC_INDX(0);

case BUFMODE is

when PPONG => if CW_BCAST='1' and SMDC_NII='1' then

-- if NII enabled do not flip buffer

elsif SMR0_PPON='1' then                  -- SA52998  assuming ping ponging not dissabled switch the buffer

SMDC_BUFA <= not SMDC_BUFA;

end if;

when INDEX => if CW_TX='0' then -- INDEX mode only on RX messages !

if SMDC_INDX/="0000000"  then

SMDC_INDX <= SMDC_INDX - 1;

end if;

if SMDC_INDX="00000001" then

WINT <= '1';

end if;

if CW_BCAST='1' and SMDC_NII='1' then

SMD_DPTR3   <= DATAPTR;

else

SMD_DPTR1   <= DATAPTR;

end if;

end if;

when CIRC1 => if DATAPTR>=SMD_DPTR3 then

SMD_DPTR2 <= SMD_DPTR1;

WINT <= '1';

else

SMD_DPTR2 <= DATAPTR;

end if;

when CIRC2 => CAF <= CAFPLUS2;

if CAFOVERFLOW='1' then

SMD_DPTR2 <= SMD_DPTR1;

SMD_DPTR3 <= SMD_MIBCAF;

WINT <= '1';

else

SMD_DPTR2 <= DATAPTR;

SMD_DPTR3 <= SMD_MIBCAF;

end if;

end case;

else

case BUFMODE is

-- in PPONG mode we flip the buffer even if errors

when PPONG => if CW_BCAST='1' and SMDC_NII='1' then

-- if NII enabled do not flip buffer

elsif SMR0_PPON='1' then               -- SAR52998 only if ping ponging still on

SMDC_BUFA <= not SMDC_BUFA;

end if;

when others =>

end case;

end if;

end if;

when RTPUTCW =>  if BENDWAIT='0' and ENCBUSY='0' and iENCSTROBE='0' then

iBENDREQ  <= '1';

iBENDWRT  <= '1';

iBENDBURST<= '1';                   -- Stays Active until RTPUTD3

BENDADDR  <= to_std_logic(DATAPTR);

BENDDOUT  <= SMD_MSGINFO;

RTSTATE   <= RTPUTTT;

DATAPTR   <= DATAPTR + 1;

end if;

when RTPUTTT=>   if BENDWAIT='0' then

iBENDREQ  <= '1';

iBENDWRT  <= '1';

BENDADDR  <= to_std_logic(DATAPTR);

BENDDOUT  <= to_std_logic(TTTEMP);

RTSTATE   <= RTPUTD0;

DATAPTR   <= DESCPTR;

end if;

when RTPUTD0=>   if BENDWAIT='0' then

iBENDREQ  <= '1';

iBENDWRT  <= '1';

BENDADDR  <= to_std_logic(DATAPTR);

DATAPTR   <= DATAPTR+1;

BENDDOUT  <= SMD_CONTROL;

RTSTATE   <= RTPUTD1;

end if;

when RTPUTD1=>   if BENDWAIT='0' then

iBENDREQ  <= to_logic(BUFMODE=INDEX and not(CW_BCAST='1' and SMDC_NII='1'));

iBENDWRT  <= '1';

BENDADDR  <= to_std_logic(DATAPTR);

DATAPTR   <= DATAPTR+1;

BENDDOUT  <= to_std_logic(SMD_DPTR1);

RTSTATE   <= RTPUTD2;

end if;

when RTPUTD2=>   if BENDWAIT='0' then

iBENDREQ  <= to_logic(BUFMODE=CIRC1 or BUFMODE=CIRC2);

iBENDWRT  <= '1';

BENDADDR  <= to_std_logic(DATAPTR);

DATAPTR   <= DATAPTR+1;

BENDDOUT  <= to_std_logic(SMD_DPTR2);

RTSTATE   <= RTPUTD3;

end if;

when RTPUTD3=>   if BENDWAIT='0' then

iBENDREQ  <= to_logic((BUFMODE=INDEX and CW_BCAST='1' and SMDC_NII='1') or BUFMODE=CIRC2);

iBENDWRT  <= '1';

iBENDBURST<= SMR0_INTEN;

BENDADDR  <= to_std_logic(DATAPTR);

DATAPTR   <= DATAPTR+1;

BENDDOUT  <= to_std_logic(SMD_DPTR3);

RTSTATE   <= RTMSGDONE;

end if;

when RTMSGDONE=> if RTMSG_OKAY='0' then

SMR4_MERR <= SMR3_MERR;

end if;

if SMDC_IWA='1' then

SMR4_SUBAD <= SMR3_SUBAD;

end if;

if SMDC_IBRD='1' and CW_BCAST='1' then

SMR4_BDRCV <= SMR3_BDRCV;

end if;

if WINT='1' then

SMR4_IXEQ0 <= SMR3_IXEQ0 and SMDC_INTX;

end if;

if LEGAL='0' then

SMR4_ILLCMD7 <= SMR3_ILLCMD7;

end if;

RTSTATE <= RTALLDONE;

DOINT   <= '1';

when RTALLDONE=> if BENDWAIT='0' and DOINT='0' then

RTSTATE    <= RTIDLE;

RTCMDDONE  <= '1';

end if;

end case;

if PCONBUSY='1' and (TXTIMEOUT='1' or BENDFAIL='1') then

RTSTATE  <= RTIDLE;

LATCHSW  <= '1';

WCNTTO   <= '1';

ABORT    <= '1';

iBENDBURST <= '0';

end if;

end if;

end if;

RTACTIVE <= not to_logic(RTSTATE = RTIDLE);

if iMTMODE='1' then

MTMUXSEL := MTCW_BCAST & MTCW_TX & MTCW_DATA;

if SMR0_STEX='0' then

MTDESCPTR <= SMR11_MBA;

MTDATAPTR <= SMR12_MDA;

end if;

if MTMSG_START='1' and SMR0_STEX='1' then

INTSTATE    <= IDLE;   -- Must abort any pending interrupt log write

DOINT       <= '0';

MTINT       <= '0';

PCONBUSY    <= '1';

PCONBUSY    <= '1';

MTWORDCOUNT <= "000000";

MTEXPWCNT   <= to_unsigned(MTCW_COUNT);

MTMI_OVR    <= '0';

MTMI_TOUT   <= '0';

MTMI_RTRT   <= '0';

EXPECTSW    <= '0';

MONHIT      <= '0';

CLRERR      <= '1';

CWRT1       <= DECDATA(15 downto 11);        -- Store the Command Word

CWRT1_LB    <= DECDATA(10 downto 0);

MTSTATE     <= MTGTCW2;

MONHIT      <= not SMR0_SMTC or SMR14_MF(to_integer(to_unsigned(DECDATA(15 downto 11))));

TTTEMP      <= SMR7_TT;

else

case MTSTATE is

when MTIDLE  =>   PCONBUSY  <=  RTACTIVE;                         -- if RT is active the activate PCONBUSY

EXPECTSW  <= '0';

MONHIT    <= '0';

when MTGTCW2 =>   INTSTATE   <= IDLE;       -- must abort interrupt log write if active

if INTLOG='1' then        -- If interrupt log set make sure nothing left set from the previous message

SMR4_MERR <= '0';      -- Clear Just the MT inturrupts

-- but allow MBC to be generated next time

end if;

if DECACTIVE='0' or DECCMDSTAT='0' then         -- or following word is data

case MTMUXSEL is

when "000" => MTSTATE  <= MTWRDP;           -- RX Mode Code - store DW

when "010" => MTSTATE  <= MTWRSW1;          -- TX Mode Code - store SW

when "001" => MTSTATE  <= MTWRDP;           -- RX Data Transfer

when "011" => MTSTATE  <= MTWRSW1;          -- TX Data Transfer

when "100" => MTSTATE  <= MTWRDP;           -- Bcast RX Mode Code

when "110" => MTSTATE  <= MTWRDP;           -- Bcast TX Mode Code

when "101" => MTSTATE  <= MTWRDP;           -- Bcast RX Data Transfer

when "111" => MTSTATE  <= MTWRDP;           -- Bcast TX Data Transfer

when others =>

end case;

elsif DECSTBOKCW='1'  then

MTSTATE   <= MTWRSW1;                        -- Store the 2nd CW

MTMI_RTRT <= '1';

CWRT2     <= DECDATA(15 downto 11);

CWRT2_LB  <= DECDATA(10 downto 0);

MONHIT    <= MONHIT or SMR14_MF(to_integer(to_unsigned(DECDATA(15 downto 11))));

elsif DECSTBDEL1='1' then

MTSTATE    <= MTWRMINFO;                     -- Bad Transfer

MTMI_TOUT  <= '1';

end if;

if not (RTSTATE=RTIDLE) then                    -- RT has started so disable the monitor

MONHIT  <= '0';

end if;

when MTWRSW1=>    EXPECTSW <= '1';

if not (RTSTATE=RTIDLE) then                    -- RT has started so disable the monitor

MONHIT  <= '0';                               -- This catches the RT-RT case

end if;

if DECSTBOKSWA='1' then

EXPECTSW <= '0';

if MONHIT='1' then

iBENDREQ <= '1';

iBENDWRT <= '1';

BENDADDR <= to_std_logic(MTDESCPTR+4);

BENDDOUT <= DECDATA;

end if;

MTSTATE  <= MTWRDP;

elsif DECSTBDEL1='1' or RTTIMEOUT='1' then

EXPECTSW  <= '0';

MTSTATE   <= MTWRMINFO;                       -- Bad Transfer

MTMI_TOUT <= '1';

end if;

when MTWRDP  =>   if BENDWAIT='0' or MONHIT='0' then

if MONHIT='1' then

iBENDREQ <= '1';

iBENDWRT <= '1';

BENDADDR <= to_std_logic(MTDESCPTR+3);

BENDDOUT <= to_std_logic(MTDATAPTR);

end if;

MTSTATE <= MTWRDATA;

end if;

when MTWRDATA =>  if DECACTIVE='0' then                         -- End of transmission

if MTWORDCOUNT > MTEXPWCNT  then           -- Check count

MTMI_OVR  <= '1';

MTSTATE   <= MTWRMINFO;

elsif MTWORDCOUNT < MTEXPWCNT  then        -- Check count

MTMI_TOUT <= '1';

MTSTATE   <= MTWRMINFO;

else

MTSTATE <= MTWRTSW2;

end if;

elsif DECSTBOKDW='1' then

if MONHIT='1' then

iBENDREQ   <= '1';

iBENDWRT   <= '1';

BENDADDR   <= to_std_logic(MTDATAPTR);

BENDDOUT   <= DECDATA;

MTDATAPTR  <= MTDATAPTR+1;

end if;

MTWORDCOUNT <= MTWORDCOUNT+1;

elsif DECSTBDEL1='1' then

MTSTATE   <= MTWRMINFO;                          -- Bad Transfer

MTMI_TOUT <= '1';

end if;

when MTWRTSW2=>   EXPECTSW <= '1';

if MTCW_BCAST='1' or (MTMI_RTRT='0' and MTCW_TX='1') then

EXPECTSW <= '0';         -- No SW at the end

MTSTATE  <= MTWRMINFO;

elsif DECSTBOKSWA='1' then

MTSTATE  <= MTWRMINFO;

EXPECTSW <= '0';

if MONHIT='1' then

iBENDREQ <= '1';

iBENDWRT <= '1';

if MTMI_RTRT='1' then

BENDADDR <= to_std_logic(MTDESCPTR+5);

else

BENDADDR <= to_std_logic(MTDESCPTR+4);

end if;

BENDDOUT <= DECDATA;

end if;

elsif DECSTBDEL1='1' or RTTIMEOUT='1' then

EXPECTSW   <= '0';

MTSTATE    <= MTWRMINFO;                          -- Bad Transfer

MTMI_TOUT  <= '1';

end if;

when MTWRMINFO=>  EXPECTSW  <= '0';

if MONHIT='0' then

MTSTATE <= MTIDLE;

elsif BENDWAIT='0' then

iBENDREQ   <= '1';

iBENDWRT   <= '1';

iBENDBURST <= '1';                                -- Write MINFO

BENDADDR   <= to_std_logic(MTDESCPTR+0);

BENDDOUT   <= MTMI_WORD;

MTSTATE    <= MTWRCW1;

end if;

when MTWRCW1  =>   if BENDWAIT='0' then

iBENDREQ   <= '1';

iBENDWRT   <= '1';

BENDADDR   <= to_std_logic(MTDESCPTR+1);

BENDDOUT   <= CWRT1 & CWRT1_LB;

MTSTATE    <= MTWRCW2;

end if;

when MTWRCW2  =>  if BENDWAIT='0' then

iBENDREQ   <= MTMI_RTRT;

iBENDWRT   <= '1';

BENDADDR   <= to_std_logic(MTDESCPTR+2);

BENDDOUT   <= CWRT2 & CWRT2_LB;

MTSTATE    <= MTWRTT;

end if;

when MTWRTT  =>   if BENDWAIT='0' then

iBENDREQ   <= '1';

iBENDWRT   <= '1';

iBENDBURST <= '0';

BENDADDR   <= to_std_logic(MTDESCPTR+6);

BENDDOUT   <= to_std_logic(TTTEMP);

MTSTATE    <= MTMSGDONE;

end if;

when MTMSGDONE=>  if MTMI_MSGERR='1' then

SMR4_MERR <= SMR3_MERR;

end if;

MTSTATE <= MTALLDONE;

if MTCOUNT=1 then

SMR4_MBC  <= SMR3_MBC;

end if;

DOINT   <= '1';

MTINT   <= '1';

when MTALLDONE=>  if BENDWAIT='0' and  DOINT='0' then

MTCOUNT    <= MTCOUNT - 1;

MTDESCPTR  <= MTDESCPTR + 8;

if MTCOUNT=0 then          -- This mimics the Summit device

MTDESCPTR <= SMR11_MBA;  -- It actually captures 1 more block than requested

MTDATAPTR <= SMR12_MDA;  -- after the interrupt is generated

MTCOUNT   <= MTCOUNTHD;

end if;

MTSTATE    <= MTIDLE;

RTCMDDONE  <= '1';         -- Clears error detectors

end if;

end case;

if PCONBUSY='1' and  (ABORT='1' or BENDFAIL='1') then

MTSTATE   <= MTIDLE;

iBENDBURST <= '0';

end if;

end if;

end if;

-- Write the Interrupt to the Log if enabled

if  (INTACKM='1' and INTLEVEL='1')

or (IMCOUNT="11" and INTLEVEL='0') then

INTLOG <= '0';

end if;

if IMCOUNT/="00" then

IMCOUNT <= IMCOUNT+1;

end if;

case INTSTATE is

when IDLE =>   if DOINT='1' and INTMSG='1' and SMR0_INTEN='1' then

INTSTATE <= WRTIIW;   -- Process using the Log

elsif DOINT='1' then

DOINT <= '0';         -- No processing required

MTINT <= '0';

end if;

when WRTIIW=> if BENDWAIT='0' then

iBENDREQ  <= '1';

iBENDWRT  <= '1';

iBENDBURST<= '1';

BENDADDR  <= SMR5_INTA & '0';

BENDDOUT  <= "0000" & SMR4_INTR(11 downto 0);

INTSTATE  <= WRTIAW;

end if;

when WRTIAW=> if BENDWAIT='0' then

iBENDREQ  <= '1';

iBENDWRT  <= '1';

iBENDBURST<= '0';

BENDADDR  <= SMR5_INTA & '1';

if MTINT='1' then  -- If a Monitor Interrupt then put the count there

BENDDOUT  <= to_std_logic(MTDESCPTR);

else

BENDDOUT  <= to_std_logic(DESCPTR);

end if;    -- Increment by 2

INTSTATE  <= DONE;

end if;

when DONE   => if BENDWAIT='0' then

-- Dont update the Internal interrupt pointer until we know both locations written

SMR5_INTA(4 downto 1) <= to_std_logic(to_unsigned(SMR5_INTA(4 downto 1))+1);

INTSTATE     <= IDLE;

INTLOG       <= '1';

IMCOUNT      <= "01";

DOINT        <= '0';

MTINT        <= '0';

SMR4_MERR    <= '0';   -- Clear the Interrupts

SMR4_SUBAD   <= '0';   -- Since now in the log

SMR4_BDRCV   <= '0';

SMR4_IXEQ0   <= '0';

SMR4_ILLCMD7 <= '0';

SMR4_EOL     <= '0';

SMR4_ILLCMD4 <= '0';

SMR4_ILLOP   <= '0';

SMR4_RTF     <= '0';

SMR4_CBA     <= '0';

SMR4_MBC     <= '0';

end if;

end case;

if INTLOG='1' and SMR0_STEX='0' then

MTINT        <= '0';

SMR4_MERR    <= '0';   -- Make sure that interrupt log will be clean when started

SMR4_SUBAD   <= '0';   -- for instance if stopped mid RT message could leave some

SMR4_BDRCV   <= '0';   -- RT  bits set, if restarted as BC then RT bits would be active

SMR4_IXEQ0   <= '0';

SMR4_ILLCMD7 <= '0';

SMR4_EOL     <= '0';

SMR4_ILLCMD4 <= '0';

SMR4_ILLOP   <= '0';

SMR4_RTF     <= '0';

SMR4_CBA     <= '0';

SMR4_MBC     <= '0';

end if;  end if;

end process;

MTMI_MCND <= MTCW_MCODE and not MTCW_COUNT(0);

MTMI_WORD <= "0100" & "00" & MTMI_CHA & MTMI_RTRT

& MTMI_MSGERR & MTMI_MCND & MTCW_BCAST & '0'

& MTMI_TOUT & MTMI_OVR & MTMI_PERR & MTMI_MERR;

MTMI_CHA    <= not MTMSG_BUSSEL;

MTMI_PERR   <= SMD_MSGINFO(1);

MTMI_MERR   <= SMD_MSGINFO(0);

MTMI_MSGERR <= MTMI_TOUT or MTMI_OVR or MTMI_PERR or MTMI_MERR;

SMD_CONTROL <= to_std_logic(SMDC_INDX) & SMDC_INTX & SMDC_IWA & SMDC_IBRD & '1'

& '0' & SMDC_BUFA & CW_BCAST & SMDC_NII;

CAFPLUS2 <= CAF + 2;

process(CAF,SMDC_INDX,SMD_DPTR3,CAFPLUS2)

variable CAFMASK : std_logic_vector(7 downto 0);

begin

CAFMASK := to_std_logic(CAF) or not to_std_logic(SMDC_INDX);

if CAFMASK(7 downto 1)="1111111" then

CAFOVERFLOW <= '1';

else

CAFOVERFLOW <= '0';

end if;

SMD_MIBCAF <= SMD_DPTR3;

for i in 0 to 7 loop

if SMDC_INDX(i)='1' then

SMD_MIBCAF(i) <= CAFPLUS2(i);

end if;

end loop;

end process;

SMR13_MBC <= MTCOUNT;

ENCCLEAR <= RTCMDDONE;

iBUSY  <= PCONBUSY;

BUSY   <= PCONBUSY;

ACTIVE <= SMR0_STEX;

ENCSTROBE   <= iENCSTROBE;

ENCABORT    <= ABORT;

ENCCMDSTAT  <= iENCCMDSTAT;

BENDREQ     <= iBENDREQ;

BENDWRT     <= iBENDWRT;

BENDBURST   <= iBENDBURST;

end RTL;

22) BRMreg

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.NUMERIC_STD.all;

use work.convert.all;

entity BRMreg is

generic ( BCENABLE : INTEGER range 0 to 1   :=  1;

RTENABLE : INTEGER range 0 to 1   :=  1;

MTENABLE : INTEGER range 0 to 1   :=  1;

LEGREGS  : INTEGER range 0 to 5   :=  2;

ENHANCED : INTEGER range 0 to 1   :=  1;

INITFREQ : INTEGER range 12 to 24 := 24

);

port ( CLK       : in  std_logic;

RSTn      : in  std_logic;

CPUSEL    : in  std_logic;

CPUADDR   : in  std_logic_vector( 3 downto 0);

CPUADDRR  : in  std_logic_vector( 3 downto 0);

CPUDIN    : in  std_logic_vector(15 downto 0);

CPUDOUT   : out std_logic_vector(15 downto 0);

CPUWR     : in  std_logic_vector( 1 downto 0);

CPURD     : in  std_logic;

SMR0_STEX : in  std_logic;

CMDSTB    : in  std_logic;

BITSELECT : in  std_logic_vector(7 downto 0);

CMDOKREG  : out std_logic

);

end BRMreg;

architecture RTL of BRMreg is

component ram16x16_ax

port( WClock   : in  std_logic;

RClock   : in  std_logic;

WE       : in  std_logic;

WAddress : in  std_logic_vector(3 downto 0);

RAddress : in  std_logic_vector(3 downto 0);

Data     : in  std_logic_vector(15 downto 0);

Q        : out std_logic_vector(15 downto 0)

);

end component;

component ram16x16_apa

port(RCLOCK : in std_logic;

WCLOCK : in std_logic;

WRB    : in std_logic_vector(1 downto 0);

RDB    : in std_logic;

WADDR  : in std_logic_vector (3 downto 0);

RADDR  : in std_logic_vector (3 downto 0);

DI     : in std_logic_vector (15 downto 0);

DO     : out std_logic_vector (15 downto 0)

);

end component;

component ram16x16_pa3

port( CLOCK    : in   std_logic;

RESET    : in   std_logic;

WE       : in   std_logic;

RE       : in   std_logic;

WADDRESS : in   std_logic_vector(3 downto 0);

RADDRESS : in   std_logic_vector(3 downto 0);

DATA     : in   std_logic_vector(15 downto 0);

Q        : out  std_logic_vector(15 downto 0)

) ;

end component;

component ram16x16_edac

port(WDATA      : in std_logic_vector(15 downto 0);

WADDR      : in std_logic_vector(7 downto 0);

RADDR      : in std_logic_vector(7 downto 0);

WE         : in std_logic;

RE         : in std_logic;

RSTN       : in std_logic;

CLK        : in std_logic;

RDATA      : out std_logic_vector(15 downto 0));

end component;

signal  MEM        : WORD_ARRAY( 0 to 15);

signal  REGS       : std_logic_vector(255 downto 0);

signal  RAMWR      : std_logic;

signal  RAMWR10    : std_logic_vector(1 downto 0);

signal  RAMRD      : std_logic;

signal  RAMADDR    : std_logic_vector(3 downto 0);

signal  RAMDATA    : std_logic_vector(15 downto 0);

signal  CMDSTBD    : std_logic;

signal  EDACWADDR  : std_logic_vector(7 downto 0);

signal  EDACRADDR  : std_logic_vector(7 downto 0);

signal Logic0      : std_logic;

signal Logic1      : std_logic;

begin

Logic0 <= '0';

Logic1 <= '1';

--  Legalisation registers not implemented

REGS0:

if ( RTENABLE=0 or LEGREGS=0 ) generate

CPUDOUT  <= ( others => '0');

CMDOKREG <= '0';

end generate;

-- Legalisation registers  implemented  in registers

REGS1:

if (RTENABLE>0 and LEGREGS=1) generate

PMEM:

process(CLK,RSTn)

variable ADDR : INTEGER range 0 to 15;

begin

if RSTn='0' then                   -- NOTE when 1 dissables i.e ILLEGAL

--FEDCBA9876543210     -- Reset Values to set up the RT !

MEM(0)  <= "0000000000000000";   -- Receive  SA 15:0                0000

MEM(1)  <= "0000000000000000";   -- Receive  SA 31:16               0000

MEM(2)  <= "0000000000000000";   -- Transmit SA 15:0                0000

MEM(3)  <= "0000000000000000";   -- Transmit SA 31:16               0000

MEM(4)  <= "0000000000000000";   -- Bcast Receive  SA 15:0          0000

MEM(5)  <= "0000000000000000";   -- Bcast Receive  SA 31:16         0000

MEM(6)  <= "1111111111111111";   -- Bcast Transmit SA 15:0          FFFF

MEM(7)  <= "1111111111111111";   -- Bcast Transmit SA 31:16         FFFF

MEM(8)  <= "1111111111111111";   -- Mode Code Receive  15:0         FFFF

MEM(9)  <= "1111111111111101";   -- Mode Code Receive  31:16        FFFD

MEM(10) <= "1111111000000001";   -- Mode Code Transmit 15:0         FE01

MEM(11) <= "1111111111110010";   -- Mode Code Transmit 31:16        FFF2 MEM(12) <= "1111111111111111";   -- Bcast Mode Code Receive  15:0   FFFF

MEM(13) <= "1111111111111101";   -- Bcast Mode Code Receive  31:16  FFFD

MEM(14) <= "1111111000000101";   -- Bcast Mode Code Transmit 15:0   FE05

MEM(15) <= "1111111111111111";   -- Bcast Mode Code Transmit 31:16  FFFF

elsif CLK'event and CLK='1' then

if CPUWR(0)='1' and CPUSEL='1' then

ADDR := to_integer(to_unsigned(CPUADDRR));

MEM(ADDR)(7 downto 0) <= CPUDIN(7 downto 0);

end if;

if CPUWR(1)='1' and CPUSEL='1' then

ADDR := to_integer(to_unsigned(CPUADDRR));

MEM(ADDR)(15 downto 8) <= CPUDIN(15 downto 8);

end if;

end if;

end process;

PMRD:

process(MEM,CPUADDR)

begin

case CPUADDR is

when "0000" => CPUDOUT <= MEM(0)  ;

when "0001" => CPUDOUT <= MEM(1)  ;

when "0010" => CPUDOUT <= MEM(2)  ;

when "0011" => CPUDOUT <= MEM(3)  ;

when "0100" => CPUDOUT <= MEM(4)  ;

when "0101" => CPUDOUT <= MEM(5)  ;

when "0110" => CPUDOUT <= MEM(6)  ;

when "0111" => CPUDOUT <= MEM(7)  ;

when "1000" => CPUDOUT <= MEM(8)  ;

when "1001" => CPUDOUT <= MEM(9)  ;

when "1010" => CPUDOUT <= MEM(10) ;

when "1011" => CPUDOUT <= MEM(11) ;

when "1100" => CPUDOUT <= MEM(12) ;

when "1101" => CPUDOUT <= MEM(13) ;

when "1110" => CPUDOUT <= MEM(14) ;

when "1111" => CPUDOUT <= MEM(15) ;

when others => CPUDOUT <= ( others => '-');

end case;

end process;

-- Create the look up array , memory being 1 illegalizes the SA

PREGS:

process(MEM)

begin

REGS <= ( others => '0');

for i in 0 to 15 loop

REGS(i*16+15 downto i*16) <= MEM(i);

end loop;

end process;

CMDOKREG <= not REGS(to_integer(to_unsigned(BITSELECT)));

end generate;

-- Legalisation registers implemented using AX RAM

REGS2:

if (RTENABLE>0 and LEGREGS=2) generate

RAMWR   <= CPUWR(0) and CPUWR(1) and CPUSEL;

RAMADDR <= CPUADDR when SMR0_STEX='0' else BITSELECT(7 downto 4);

URAM:  ram16x16_ax port map

( WClock   => CLK,

RClock   => CLK,

WE       => RAMWR,

WAddress => CPUADDRR,

RAddress => RAMADDR,

Data     => CPUDIN,

Q        => RAMDATA

);

CPUDOUT  <= RAMDATA;

CMDOKREG <= not RAMDATA(to_integer(to_unsigned(BITSELECT(3 downto 0))));

end generate;

-- Legalisation registers implemented using APA RAM

REGS3:

if (RTENABLE>0 and LEGREGS=3) generate

RAMWR10(1)   <= not ( CPUWR(1) and CPUSEL);

RAMWR10(0)   <= not ( CPUWR(0) and CPUSEL);

RAMADDR <= CPUADDR when SMR0_STEX='0' else BITSELECT(7 downto 4);

URAM:  ram16x16_apa port map

( RCLOCK => CLK,

WCLOCK => CLK,

WRB    => RAMWR10,

RDB    => Logic0,

WADDR  => CPUADDRR,

RADDR  => RAMADDR,

DI     => CPUDIN,

DO     => RAMDATA

);

CPUDOUT  <= RAMDATA;

CMDOKREG <= not RAMDATA(to_integer(to_unsigned(BITSELECT(3 downto 0))));

end generate;

-- Legalisation registers implemented using AX EDAC RAM

REGS4:

if (RTENABLE>0 and LEGREGS=4) generate

RAMWR   <= CPUWR(0) and CPUWR(1) and CPUSEL;

RAMADDR <= CPUADDR when SMR0_STEX='0' else BITSELECT(7 downto 4);

RAMRD   <= CMDSTB or CPURD;

EDACWADDR <= "0000" & CPUADDRR;

EDACRADDR <= "0000" & RAMADDR;

URAM: ram16x16_edac port map

(WDATA      => CPUDIN,

WADDR      => EDACWADDR,

RADDR      => EDACRADDR,

WE         => RAMWR,

RE         => RAMRD,

RSTN       => RSTn,

CLK        => CLK,

RDATA      => RAMDATA

);

process(CLK,RSTn)

begin

if RSTn='0' then

CMDOKREG <= '0';

CMDSTBD  <= '0';

elsif CLK'event and CLK='1' then

CMDSTBD <= CMDSTB;

if CMDSTBD='1' then

CMDOKREG <= not RAMDATA(to_integer(to_unsigned(BITSELECT(3 downto 0))));

end if; end if;

end process;

CPUDOUT  <= RAMDATA;

end generate;

-- Legalisation registers implemented using PA3 RAM

REGS5:

if (RTENABLE>0 and LEGREGS=5) generate

RAMWR   <= ( CPUWR(0) and CPUWR(1) and CPUSEL);

RAMADDR <= CPUADDR when SMR0_STEX='0' else BITSELECT(7 downto 4);

URAM:  ram16x16_pa3 port map

(  CLOCK     => CLK,

RESET     => RSTn,

WE        => RAMWR,

RE        => Logic1,

WADDRESS  => CPUADDRR,

RADDRESS  => RAMADDR,

DATA      => CPUDIN,

Q         => RAMDATA      );

CPUDOUT  <= RAMDATA;

CMDOKREG <= not RAMDATA(to_integer(to_unsigned(BITSELECT(3 downto 0))));

end generate;

end RTL;

23) BRMRT

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.NUMERIC_STD.all;

use work.convert.all;

entity BRMRT is

generic ( BCENABLE : INTEGER range 0 to 1    :=  1;

RTENABLE : INTEGER range 0 to 1    :=  1;

MTENABLE : INTEGER range 0 to 1    :=  1;

LEGREGS  : INTEGER range 0 to 5    :=  1;

ENHANCED : INTEGER range 0 to 1    :=  1;

INITFREQ : INTEGER range 12 to 24  := 24

);

port ( CLK            : in  std_logic;

CLKSPD         : in  std_logic_vector(1 downto 0);

RSTn           : in  std_logic;

RSTREG         : out std_logic;

RTMODE         : in  std_logic;

-- 1553B Interface

RTADDR         : in  std_logic_vector(4 downto 0);

RTADDRP        : in  std_logic;

RTADERR        : out std_logic;

-- Control Signals

BCASTEN        : in  std_logic;

MIL1553A       : in  std_logic;

SMR0_EMODE     : in  std_logic_vector(1 downto 0);

SMR9_SB        : in  std_logic_vector(10 to 19);

DBA            : in  std_logic;

XMTSW          : in  std_logic;

SSYSFn         : in  std_logic;

CHAEN          : in  std_logic;

CHBEN          : in  std_logic;

RTBUSY         : out std_logic;

-- Decoder Interface

BUSSEL         : out std_logic;

BUSSELALL      : in  std_logic;

TXTIMEOUT      : in  std_logic;

DECA_FROMUS    : in  std_logic;

DECA_STROBE    : in  std_logic;

DECA_DATAOUT   : in  std_logic_vector(15 downto 0);

DECA_CMDSTAT   : in  std_logic;

DECA_GOTSYNC   : in  std_logic;

DECA_PARERR    : in  std_logic;

DECA_MANERR    : in  std_logic;

DECA_ACTIVE    : in  std_logic;

DECA_CONTIG    : in  std_logic;

DECB_FROMUS    : in  std_logic;

DECB_STROBE    : in  std_logic;

DECB_DATAOUT   : in  std_logic_vector(15 downto 0);

DECB_CMDSTAT   : in  std_logic;

DECB_GOTSYNC   : in  std_logic;

DECB_PARERR    : in  std_logic;

DECB_MANERR    : in  std_logic;

DECB_ACTIVE    : in  std_logic;

DECB_CONTIG    : in  std_logic;

LOOPFAILA      : in  std_logic;

LOOPFAILB      : in  std_logic;

SHUTDOWNRTA    : out std_logic;

SHUTDOWNRTB    : out std_logic;

-- Interface to/from Protocol Logic

LATCHSW        : in  std_logic;

WCNTTO         : in  std_logic;

WCNTOVR        : in  std_logic;

CMDDONE        : in  std_logic;

RTSMBUSY       : in  std_logic;

BENDFAIL       : in  std_logic;

RTMSG_STBME    : out std_logic;

RTMSG_START    : out std_logic;

RTMSG_CMDWORD  : out std_logic_vector(23 downto 0);

RTMSG_LEGAL    : out std_logic;

RTMSG_RTRT     : out std_logic;

RTMSG_RTRTOK   : out std_logic;

RTMSG_OKAY     : out std_logic;

RTMSG_GOTSW    : out std_logic;

TIMER57US      : out std_logic;

MSGINFO        : out std_logic_vector(15 downto 0);

BITWORD        : in  std_logic_vector(15 downto 0);

CLR_SRQ        : out std_logic;

-- RT Transmiter Data

TXT_CMDSTAT    : in  std_logic;

BENDDIN        : in  std_logic_vector(15 downto 0);

TXT_DATAIN     : out std_logic_vector(15 downto 0);

-- External Status Signals

CMDSYNC        : out std_logic;

SYNCNOW        : out std_logic;

-- Command Legalization Interface

BITSELECT      : out std_logic_vector(7 downto 0);

CMDOKREG       : in  std_logic;

USEXTOK        : in  std_logic;

CMDVAL         : out std_logic_vector(11 downto 0);

CMDOK          : in  std_logic;

CMDSTB         : out std_logic;

CMDOKOUT       : out std_logic );

end BRMRT;

architecture RTL of BRMRT is

-- Bus Enable Signals

signal  MCSHUTDOWNA    : std_logic;

signal  MCSHUTDOWNB    : std_logic;

signal  SHUTDOWNA      : std_logic;

signal  SHUTDOWNB      : std_logic;

-- Bus Selection and Multiplexing Signals

signal  iBUSSEL        : std_logic;

signal  DEC_STROBE     : std_logic;

signal  DEC_STBDEL1    : std_logic;

signal  DEC_DATAOUT    : std_logic_vector(15 downto 0);

signal  DEC_CMDSTAT    : std_logic;

signal  DEC_GOTSYNC    : std_logic;

signal  DEC_PARERR     : std_logic;

signal  DEC_MANERR     : std_logic;

signal  DEC_ACTIVE     : std_logic;

signal  DEC_CONTIG     : std_logic;

signal  DEC_FROMUS     : std_logic;

signal  DATASYNC       : std_logic;

-- Timer Signals

signal  TIMERRTRT       : UNSIGNED(10 downto 0);

signal  RTTIMVALUE      : UNSIGNED(6 downto 0);

signal  iTIMER57US      : std_logic;

-- Command Word Signals

signal  RTADERROR       : std_logic;

signal  CW_STBME        : std_logic;

signal  PENDING         : std_logic;

signal  STARTMSG        : std_logic;

signal  CW_RTADDR       : std_logic_vector(4 downto 0);

signal  CW_TX           : std_logic;

signal  CW_TXX          : std_logic;

signal  CW_SUBADR       : std_logic_vector(4 downto 0);

signal  CW_MCWC         : std_logic_vector(4 downto 0);

signal  CW_COUNT        : std_logic_vector(5 downto 0);

signal  CW_BCAST        : std_logic;

signal  CW_MCODE        : std_logic;

signal  CW_DATA         : std_logic;

signal  SW_STBOK        : std_logic;

signal  DW_STBOK        : std_logic;

signal  BURST           : std_logic;

signal  RTRT            : std_logic;

signal  RTRTOK          : std_logic;

signal  RTRTTXADDR      : std_logic_vector(4 downto 0);

-- Status Word Signals

signal  SW_MSGERR       : std_logic;

signal  STATUS          : std_logic_vector(15 downto 0);

signal  MSGOKAY         : std_logic;

signal  GOTSW           : std_logic;

signal  WCNTERR         : std_logic;

signal  iRTBUSY         : std_logic;

-- Mode Code Signals

signal  CMDOKAY         : std_logic;

signal  CMDVALINT       : std_logic_vector(11 downto 0);

signal  CMDOKAYR        : std_logic;

signal  HOLDCW          : std_logic;

signal  LASTSW          : std_logic;

signal  MASKTFLAG       : std_logic;

signal  DORESET         : std_logic;

signal  LastCommandWord : std_logic_vector(15 downto 0);

signal  LastStatusWord  : std_logic_vector(15 downto 0);

signal  MODEDATA        : std_logic_vector(15 downto 0);

signal  MCDMUX          : std_logic_vector(1 downto 0);

signal  DYNBUSOK        : std_logic;

-- MSGINFO & BIT Signals

signal  MSGI_ME         : std_logic;

signal  MSGI_MAN        : std_logic;

signal  MSGI_PRTY       : std_logic;

signal  MSGI_ILL        : std_logic;

signal  MSGI_TO         : std_logic;

signal  MSGI_OVR        : std_logic;

signal  MSGI_RTRT       : std_logic;

signal  MSGI_CHA        : std_logic;

signal  MSGI_WC         : std_logic_vector(4 downto 0);

signal  MSGI_MEMFAIL    : std_logic;

signal  MSGI_BC         : std_logic;

-- Misc Signals

signal  Logic0          : std_logic;

signal  Logic1          : std_logic;

constant BOTHSYNCS : boolean := FALSE;

begin

-- Misc

Logic0 <= '0';

Logic1 <= '1';

PTOA:

process(CLK,RSTn)

begin

if RSTn='0' then

SHUTDOWNA <= '0';

elsif CLK'event and CLK='1' then

if (TXTIMEOUT='1' and iBUSSEL='0') then        -- Transmission to long

SHUTDOWNA <= '1';

end if;

if  (STARTMSG='1' and iBUSSEL='0') then         -- valid command so clear

SHUTDOWNA <= '0';

end if;

end if;

end process;

PTOB:

process(CLK,RSTn)

begin

if RSTn='0' then

SHUTDOWNB <= '0';

elsif CLK'event and CLK='1' then

if (TXTIMEOUT='1' and iBUSSEL='1') then        -- Transmission to long

SHUTDOWNB <= '1';

end if;

if  (STARTMSG='1' and iBUSSEL='1') then         -- valid command so clear

SHUTDOWNB <= '0';

end if;

end if;

end process;

SHUTDOWNRTA <= SHUTDOWNA;

SHUTDOWNRTB <= SHUTDOWNB;

-- Decoder Multiplexing logic

-- SAR57312 - this now uses the top level bus select signal

DEC_FROMUS   <=  DECA_FROMUS  when BUSSELALL='0' else DECB_FROMUS;

DEC_STROBE   <=  DECA_STROBE  when BUSSELALL='0' else DECB_STROBE;

DEC_DATAOUT  <=  DECA_DATAOUT when BUSSELALL='0' else DECB_DATAOUT;

DEC_CMDSTAT  <=  DECA_CMDSTAT when BUSSELALL='0' else DECB_CMDSTAT;

DEC_GOTSYNC  <=  DECA_GOTSYNC when BUSSELALL='0' else DECB_GOTSYNC;

DEC_PARERR   <=  DECA_PARERR  when BUSSELALL='0' else DECB_PARERR;

DEC_MANERR   <=  DECA_MANERR  when BUSSELALL='0' else DECB_MANERR;

DEC_ACTIVE   <=  DECA_ACTIVE  when BUSSELALL='0' else DECB_ACTIVE;

DEC_CONTIG   <=  DECA_CONTIG  when BUSSELALL='0' else DECB_CONTIG;

DATASYNC     <= DEC_GOTSYNC and not DEC_CMDSTAT;

BURST        <= DEC_CONTIG;

PCWD:

process(CLK,RSTn)

variable RTPAROK    : std_logic;

variable CWOKA      : std_logic;

variable CWOKB      : std_logic;

variable CW_SECOND  : std_logic;

variable MYADDRA    : std_logic;

variable MYADDRB    : std_logic;

variable BCASTADDRA : std_logic;

variable BCASTADDRB : std_logic;

variable SYNCS      : std_logic_vector(1 downto 0);

begin

if RSTn='0' then

CW_STBME      <= '0';

iBUSSEL       <= '0';    -- default bus

RTRT          <= '0';

RTRTOK        <= '0';

RTRTTXADDR    <= "00000";

RTADERROR     <= '0';

DEC_STBDEL1   <= '0';

DW_STBOK      <= '0';

SW_STBOK      <= '0';

elsif CLK'event and CLK='1' then

DEC_STBDEL1 <= DEC_STROBE;

DW_STBOK    <= DEC_STROBE and not DEC_CMDSTAT and not DEC_MANERR and not DEC_PARERR and BURST;

SW_STBOK    <= DEC_STROBE and     DEC_CMDSTAT and not DEC_MANERR and not DEC_PARERR;

RTPAROK    := RTADDRP xor RTADDR(4) xor RTADDR(3) xor RTADDR(2) xor RTADDR(1) xor RTADDR(0);

RTADERROR  <= not RTPAROK or (to_logic(RTADDR="11111") and BCASTEN);

BCASTADDRA := to_logic(DECA_DATAOUT(15 downto 11)="11111") and BCASTEN;

BCASTADDRB := to_logic(DECB_DATAOUT(15 downto 11)="11111") and BCASTEN;

MYADDRA    := to_logic(DECA_DATAOUT(15 downto 11)=RTADDR) or BCASTADDRA;

MYADDRB    := to_logic(DECB_DATAOUT(15 downto 11)=RTADDR) or BCASTADDRB;

CWOKA := DECA_STROBE and DECA_CMDSTAT and not DECA_FROMUS

and not DECA_PARERR and not DECA_MANERR and not MCSHUTDOWNA and CHAEN;

CWOKB := DECB_STROBE and DECB_CMDSTAT and not DECB_FROMUS

and not DECB_PARERR and not DECB_MANERR and not MCSHUTDOWNB and CHBEN;

CW_SECOND := BURST and ((CWOKA and not iBUSSEL) or (CWOKB and iBUSSEL));

CW_STBME    <= not RTADERROR and ((MYADDRA and CWOKA) or (MYADDRB and CWOKB));

-- Switch the Buses when a Valid Command Received for me

SYNCS(1) := MYADDRB and CWOKB;

SYNCS(0) := MYADDRA and CWOKA;

case SYNCS is

when "01"   => iBUSSEL <= '0';  --Bus A

when "10"   => iBUSSEL <= '1';  --Bus B

when others =>

end case;

-- Check for RTRT to RT Transmitting RTRT does not need to know its RTRT

--  RXCW TXCW .... TXSW DW DW DW .. RXSW

if CW_STBME='1' and BURST='0' then       -- RX RT First CW

RTRT   <= '0';

RTRTOK <= not DEC_DATAOUT(10);         -- should be a RX Command

elsif CW_SECOND='1' and RTSMBUSY='1' then   -- Second command word and we are active

if RTRT='0' then

RTRT       <= '1';

RTRTOK     <= RTRTOK and DEC_DATAOUT(10) and not to_logic(DEC_DATAOUT(15 downto 11)="11111");

RTRTTXADDR <= DEC_DATAOUT(15 downto 11);

else -- third command word !

RTRTOK     <= '0';

end if;

end if;

end if;

end process;

BUSSEL         <= iBUSSEL;

RTMSG_RTRT     <= RTRT;

RTMSG_RTRTOK   <= RTRTOK;

RTMSG_STBME    <= CW_STBME;

RTADERR <= RTADERROR;

-- Command Word Register & Decode

PCW:

process(CLK,RSTn)

begin

if RSTn='0' then

CW_RTADDR  <= "00000";

CW_TX      <= '0';

CW_SUBADR  <= "00000";

CW_MCWC    <= "00000";

CW_COUNT   <= "000000";

CW_BCAST   <= '0';

CW_MCODE   <= '0';

STARTMSG   <= '0';

PENDING    <= '0';

CMDSTB     <= '0';

elsif CLK'event and CLK='1' then

STARTMSG  <= '0';

if CW_STBME='1' then

PENDING   <= '1';

CW_RTADDR <= DEC_DATAOUT(15 downto 11);

CW_TX     <= DEC_DATAOUT(10);

CW_SUBADR <= DEC_DATAOUT(9 downto 5);

CW_MCWC   <= DEC_DATAOUT(4 downto 0);

CW_BCAST  <= to_logic(DEC_DATAOUT(15 downto 11)="11111") and BCASTEN;

if MIL1553A='1' then

if DEC_DATAOUT(9 downto 5)="00000" then

CW_MCODE  <= '1';

CW_COUNT  <= "000000";

else

CW_MCODE  <= '0';

CW_COUNT  <= to_logic(DEC_DATAOUT(4 downto 0)="00000" ) & DEC_DATAOUT(4 downto 0);

end if;

else

if (DEC_DATAOUT(9 downto 5)="00000" or DEC_DATAOUT(9 downto 5)="11111") then

CW_MCODE  <= '1';

CW_COUNT  <= "00000" & DEC_DATAOUT(4);

else

CW_MCODE  <= '0';

CW_COUNT  <= to_logic(DEC_DATAOUT(4 downto 0)="00000" ) & DEC_DATAOUT(4 downto 0);

end if;

end if;

end if;

if PENDING='1' and (DEC_ACTIVE='0' or DEC_GOTSYNC='1') then

STARTMSG  <= '1';

PENDING   <= '0';

end if;

CMDSTB <= CW_STBME;

end if;

end process;

-- In 1553A Mode TX bit is ignored on mode codes and treated as TX

CW_TXX <=  '1' when (MIL1553A='1' and CW_MCODE='1') else CW_TX;

RTMSG_CMDWORD <= CW_BCAST & CW_MCODE & CW_COUNT & CW_RTADDR & CW_TXX & CW_SUBADR & CW_MCWC;

CW_DATA  <= not CW_MCODE;

-- Command Legallity Block

CMDVALINT <= CW_BCAST & CW_TX & CW_SUBADR & CW_MCWC;

-- External Command Word checker

CMDVAL  <= CMDVALINT;

-- Internal Checker,

process(CMDVALINT,MIL1553A)

variable BCAST : std_logic;

variable TX    : std_logic;

variable SA    : std_logic_vector(4 downto 0);

variable WCMC  : std_logic_vector(4 downto 0);

variable MCODE : std_logic;

begin

BCAST   := CMDVALINT(11);

TX      := CMDVALINT(10);

SA      := CMDVALINT(9 downto 5);

WCMC    := CMDVALINT(4 downto 0);

if ( SA="00000" or (SA="11111" and MIL1553A='0') ) then

-- Mode Code

BITSELECT  <= '1' & BCAST & TX & WCMC(4) & WCMC(3 downto 0);

else

-- Data

BITSELECT  <= '0' & BCAST & TX & SA(4) & SA(3 downto 0);

end if;

end process;

-- Select Internal or External Checker, also latch the input signal as we

-- start processing the message

PCMDOK:

process(CLK,RSTn)

begin

if RSTn='0' then

CMDOKAY <= '0';

elsif CLK'event and CLK='1' then

if USEXTOK='1' or LEGREGS=0 then

CMDOKAY <= CMDOK;

else

CMDOKAY <= CMDOKREG;

end if;

end if;

end process;

CMDOKOUT     <= CMDOKAY;

RTMSG_LEGAL  <= CMDOKAY;

PMCODE:

process(CLK,RSTn)

variable MUXSEL : std_logic_vector(5 downto 0);

variable WRITE  : std_logic;

begin

if RSTn='0' then

SYNCNOW      <= '0';

LASTSW       <= '0';

MASKTFLAG    <= '0';

DORESET      <= '0';

MCDMUX       <= "00";

MCSHUTDOWNA  <= '0';

MCSHUTDOWNB  <= '0';

elsif CLK'event and CLK='1' then

SYNCNOW      <= '0';

LASTSW       <= '0';

DORESET      <= '0';

MCDMUX       <= "00";

MUXSEL       := CW_TX & CW_MCWC;

WRITE        := STARTMSG and CMDOKAY;

if MIL1553A='1' then    -- No special action is taken on any mode codes, they are simply stored

if ( CW_MCODE='1' and CW_MCWC="00010" and XMTSW='1') then   -- XMTSW enabled so do auto return the SW

LASTSW   <= CMDOKAY;

end if;

else

if ( CW_MCODE='1' ) then                              -- Mode functions

case MUXSEL is

when "100000" =>

when "100001" =>  SYNCNOW  <= WRITE;                             --Synchronise

when "100010" =>  LASTSW   <= CMDOKAY;                  --Transmit Status Word

when "100100" =>  if WRITE='1' and iBUSSEL='1' then              --Transmitter ShutDown

MCSHUTDOWNA <= '1';

end if;

if WRITE='1' and iBUSSEL='0' then

MCSHUTDOWNB <= '1';

end if;

when "100101" =>  if WRITE='1' and iBUSSEL='1' then              --Override Transmitter Shutdown

MCSHUTDOWNA <= '0';

end if;

if WRITE='1' and iBUSSEL='0' then

MCSHUTDOWNB <= '0';

end if;

when "100110" =>  if WRITE='1' then

MASKTFLAG  <= '1';                           --Inhibit terminal flag

end if;

when "100111" =>  if WRITE='1' then               --Override inhibit terminal flag

MASKTFLAG  <= '0';

end if;

when "101000" =>  if CMDDONE='1' and CMDOKAY='1' then

DORESET     <= '1';                          --Reset Remote Terminal

MCSHUTDOWNA <= '0';                          -- Also clear as per summit

MCSHUTDOWNB <= '0';

MASKTFLAG   <= '0';

end if;

when "110000" =>  MCDMUX  <= "01";                         --Transmit Vector Word

when "010001" =>  SYNCNOW <= DW_STBOK and CMDOKAY and RTSMBUSY;  --Synchronise with data

when "110010" =>  LASTSW  <= CMDOKAY;               --Transmit last command

MCDMUX  <= "10";

when "110011" =>  MCDMUX  <= "11";                      --Transmit bit word

when others   =>

end case; end if;

end if;  end if;

end process;

RSTREG   <= DORESET;

DYNBUSOK <= DBA and not CW_BCAST and CW_MCODE and CW_TX and to_logic(CW_MCWC="00000");

CLR_SRQ  <= STARTMSG and not CW_BCAST and CW_MCODE and CW_TX and to_logic(CW_MCWC="10000");

HOLDCW   <= to_logic(MCDMUX="10");

-- Force 4-1 Mux for datapath

PMCMUX:

process(MCDMUX,LastCommandWord,BITWord)

begin

case MCDMUX(0) is  -- synopsys infer_mux

when '0'    => MODEDATA <= LastCommandWord;

when '1'    => MODEDATA <= BITWord;

when others =>

end case;

end process;

-- Error Control and Clearing

PERRLAT:

process(CLK,RSTn)

begin

if RSTn='0' then

MSGI_MAN     <= '0';

MSGI_PRTY    <= '0';

MSGI_MEMFAIL <= '0';

elsif CLK'event and CLK='1' then

if DEC_STROBE='1' and DEC_MANERR='1' and RTSMBUSY='1' then

MSGI_MAN <= '1';

end if;

if DEC_STROBE='1' and DEC_PARERR='1' and RTSMBUSY='1' then

MSGI_PRTY <= '1';

end if;

if CMDDONE='1' then

MSGI_MAN  <= '0';

MSGI_PRTY <= '0';

end if;

if BENDFAIL='1' then

MSGI_MEMFAIL <= '1';

end if;

if STARTMSG='1' then

MSGI_MEMFAIL <= '0';

end if;

end if;

end process;

-- Status Word Creation

SW_MSGERR  <= not CMDOKAY or MSGI_MAN or MSGI_PRTY or WCNTERR or MSGI_MEMFAIL;

iRTBUSY    <= SMR9_SB(16) and not MIL1553A;

RTBUSY     <= iRTBUSY;

PSWCR:

process(RTADDR,SW_MSGERR,SMR9_SB,SSYSFn,DBA,DYNBUSOK,MASKTFLAG,CW_BCAST,MIL1553A)

begin

STATUS(15 downto 11) <= RTADDR;

STATUS(10)           <= SW_MSGERR;

STATUS(9 downto 0)   <= SMR9_SB;

if MIL1553A='0' then                -- modify the B bit settings

STATUS(7 downto 5) <= "000";

STATUS(4) <= CW_BCAST;

STATUS(2) <= SMR9_SB(17) or not SSYSFn;

STATUS(1) <= DBA and DYNBUSOK;

STATUS(0) <= SMR9_SB(19) and not MASKTFLAG;

end if;

end process;

-- Message Info Word Creation, Written to RAM

WCNTERR <= WCNTOVR or WCNTTO;

MSGOKAY    <= not (MSGI_MEMFAIL or MSGI_MAN or WCNTERR or MSGI_PRTY

or LOOPFAILB or LOOPFAILA or not CMDOKAY or iRTBUSY);

RTMSG_OKAY <= MSGOKAY;

MSGI_ILL  <= not CMDOKAY;

MSGI_TO   <= WCNTTO;

MSGI_OVR  <= WCNTOVR;

MSGI_RTRT <= RTRT and RTRTOK;

MSGI_CHA  <= not iBUSSEL;

MSGI_WC   <= CW_MCWC;

MSGI_BC   <= CW_BCAST; --  and (SMR0_EMODE(1) or CW_MCODE);   -- undocumented bit 5 is BCAST in enhanced mode !

-- it is also set for MCODE messages

MSGINFO <= MSGI_WC & '0' & MSGI_CHA & MSGI_RTRT & MSGI_ME & "0" & MSGI_BC &

MSGI_ILL & MSGI_TO & MSGI_OVR & MSGI_PRTY & MSGI_MAN;

MSGI_ME <= not CMDOKAY or WCNTTO or WCNTOVR or MSGI_PRTY or MSGI_MAN;

-- Last Command and Status Word Store

PLCSW:

process(CLK)

begin

if CLK'event and CLK='1' then

if CMDDONE='1' and (HOLDCW='0' or CW_BCAST='1') then

LastCommandWord <= CW_RTADDR & CW_TX & CW_SUBADR & CW_MCWC;

end if;

if LATCHSW='1' and LASTSW='0' then

LastStatusWord <= STATUS;

end if;

end if;

end process;

PDM1:

process(LASTSW,TXT_CMDSTAT,CW_MCODE,BENDDIN,MODEDATA,STATUS,LastStatusWord,

MCDMUX,RTMODE)

variable MUXSELC : std_logic_vector(2 downto 0);

variable MUXSELD : std_logic_vector(1 downto 0);

begin

MUXSELC := LASTSW & TXT_CMDSTAT & CW_MCODE;

case MUXSELC is

when "000"  => MUXSELD := "00";

when "001"  => MUXSELD := "01";

when "010"  => MUXSELD := "10";

when "011"  => MUXSELD := "10";

when "101"  => MUXSELD := "01";

when "111"  => MUXSELD := "11";

when others => MUXSELD := ( others => '-');

end case;

if RTMODE='0' or (MCDMUX(1)='0' and CW_MCODE='1' and TXT_CMDSTAT='0') then

MUXSELD := "00";

end if;

case MUXSELD is -- synopsys infer_mux

when "00"   => TXT_DATAIN <= BENDDIN;

when "01"   => TXT_DATAIN <= MODEDATA;

when "10"   => TXT_DATAIN <= STATUS;

when others => TXT_DATAIN <= LastStatusWord;

end case;

end process;

-- Core Status Signals

CMDSYNC     <= (DECA_GOTSYNC or DECB_GOTSYNC )

when BOTHSYNCS else

((DECA_GOTSYNC and DECA_CMDSTAT) or (DECB_GOTSYNC and DECB_CMDSTAT));

RTMSG_START <= STARTMSG;

process(CLKSPD)

begin

case CLKSPD is           -- 60 * FREQ/16

when "00"   => RTTIMVALUE <= "0101101"; -- 12MHz  45

when "01"   => RTTIMVALUE <= "0111100"; -- 16MHz  60

when "10"   => RTTIMVALUE <= "1001011"; -- 20MHz  75

when others => RTTIMVALUE <= "1011010"; -- 24MHz  90

end case;

end process;

UTIMER:

process(CLK,RSTn)

begin

if RSTn='0' then

TIMERRTRT  <= "00000000000";

iTIMER57US <= '0';

GOTSW      <= '0';

elsif CLK'event and CLK='1' then

if DEC_ACTIVE='0' then

GOTSW <= '0';

elsif SW_STBOK='1' and BURST='0' and RTRT='1' and CW_STBME='0' then

-- Got the Status Word, check if okay

if (RTRTTXADDR=DEC_DATAOUT(15 downto 11) or (RTRTTXADDR="11111" and BCASTEN='1'))

and DEC_DATAOUT(10)='0' and DEC_DATAOUT(3)='0'

then

GOTSW <= '1';

else

iTIMER57US <= '1'; -- bad SW force RT to abort command

end if;  end if;

if CW_STBME='1' or DATASYNC='1' then

TIMERRTRT <= "00000000000";

iTIMER57US <= '0';

else

TIMERRTRT <= TIMERRTRT + 1;

if TIMERRTRT(10 downto 4) = RTTIMVALUE then

iTIMER57US <= '1';

end if;   end if;

end if;

end process;

TIMER57US    <= iTIMER57US;

RTMSG_GOTSW  <= GOTSW;

end RTL;

24) bus_interface_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

Library UNISIM;

use UNISIM.vcomponents.all;

entity bus_interface_e is

port(   data_bus_up_bi: inout std_logic_vector(31 downto 0);

fpga_data_out_to_up_is: in std_logic_vector(31 downto 0);

fpga_data_in_from_up_os: out std_logic_vector(31 downto 0);

oen_is : in std_logic ;

iosn_is : in std_logic;

prom_cs_is : in std_logic);

end bus_interface_e;

architecture bus_interface_a of bus_interface_e is

signal trien_up_bus_s: std_logic;

begin

trien_up_bus_s <= '0' when oen_is = '0' and iosn_is = '0' else

'0' when oen_is = '0' and prom_cs_is = '0' else

'1';

g1: for i in 0 to 31 generate

IOBUF_inst : IOBUF

generic map (

DRIVE => 12,

IOSTANDARD => "LVCMOS33",

SLEW => "SLOW"

)

port map (

I => fpga_data_out_to_up_is(i),     -- Buffer input

O => fpga_data_in_from_up_os(i),     -- Buffer output

IO => data_bus_up_bi(i),   -- Buffer inout port (connect directly to top-level port)

T => trien_up_bus_s

);      -- 3-state enable input

end generate g1;

end bus_interface_a;

25) channel_ff_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity channel_ff_e is

port( clk_is : in std_logic;

POR_is: in std_logic;

channel_load_is: in std_logic;

channel_clear_is: in std_logic;

channel_ff_os: out std_logic

);

end channel_ff_e;

architecture channel_ff_a of channel_ff_e is

begin

P_channel_register: process(clk_is, POR_is)

begin

if POR_is = '1' then

channel_ff_os <= '0';

elsif clk_is'event and clk_is = '1' then

if channel_load_is = '1' then

channel_ff_os <= '1';

elsif channel_clear_is = '1' then

channel_ff_os <= '0';

end if;end if;

end process P_channel_register;

end channel_ff_a;

26) clk_gen_e

library ieee;

use ieee.std_logic_1164.all;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

Library UNISIM;

use UNISIM.vcomponents.all;

entity clk_gen_e is

port(-- Inputs

clk_40MHz_in_p_is : in std_logic;

clk_40MHz_in_n_is : in std_logic;

reset_is: in std_logic;

-- Outputs

clk_40MHz_os: out std_logic;

clk_40MHz_pr_out_os: out std_logic;

clk_20MHz_os: out std_logic);

end clk_gen_e;

architecture clk_gen_a of clk_gen_e is

signal mil_clk_s : std_logic;

signal clk_20MHz_s, clk_20MHz_temp_s: std_logic;

signal clk_40MHz_in_s,clk_40MHz_s : std_logic;

signal clk_40MHz_not_s : std_logic;

begin

-- 40MHz differential clock to single-ended clock.

Clk_40MHz_BUFGDS: IBUFGDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => clk_40MHz_in_s,  -- Clock buffer output

I => clk_40MHz_in_p_is,  -- Diff_p clock buffer input

IB => clk_40MHz_in_n_is -- Diff_n clock buffer input);

-- 40MHz single-ended clock routed through global buffer.

Clk_40MHz_BUFG : BUFG

port map (O => clk_40MHz_s,     -- Clock buffer output

I => clk_40MHz_not_s      -- Clock buffer input);

clk_40MHz_not_s <= clk_40MHz_in_s;

clk_40MHz_os<=clk_40MHz_s;

-- 40MHz clock to LEON-3 processor

clk_40MHz_pr_out_os<= clk_40MHz_s;

-- 20MHz clock generation

P_20MHz_clk_gen: process(clk_40MHz_s,reset_is)

begin

if reset_is='1' then

mil_clk_s<='0';

elsif clk_40MHz_s'event and clk_40MHz_s = '1' then

mil_clk_s<=  not mil_clk_s ;				end if;

end process P_20MHz_clk_gen;

clk_20MHz_temp_s<= mil_clk_s;

--20MHz clock to Global buffer

BUFG_Mil : BUFG

port map (

I => clk_20MHz_temp_s,      -- Clock buffer input

O => clk_20MHz_s          -- Clock buffer output );

clk_20MHz_os <= clk_20MHz_s;

end clk_gen_a;

27) cmd_deglitch_e

library ieee;

use ieee.std_logic_1164.all;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity cmd_deglitch_e is

port(clk_40MHz_is : in std_logic;

reset_is: in std_logic;

cmd_in_is: in std_logic;

cmd_deg_os : out std_logic);

end cmd_deglitch_e;

architecture cmd_deglitch_a of cmd_deglitch_e is

signal counter_logic1_s, counter_logic2_s : std_logic_vector(5 downto 0);

signal enable_ff_logic1_s, enable_ff_logic2_s: std_logic;

signal enable_ff_s : std_logic;

begin

Process_p1: process(clk_40MHz_is,reset_is)

begin

if reset_is = '1' then

counter_logic1_s <= (others => '0');

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if cmd_in_is = '0' then

counter_logic1_s <= (others => '0');

else

counter_logic1_s <= counter_logic1_s + 1;

end if;  end if;

if reset_is = '1' then

counter_logic2_s <= (others => '0');

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if cmd_in_is = '1' then

counter_logic2_s <= (others => '0');

else

counter_logic2_s <= counter_logic2_s + 1;

end if;  end if;

if reset_is = '1' then

enable_ff_s <= '0';

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if enable_ff_logic1_s = '1' then

enable_ff_s <= '1';

elsif enable_ff_logic2_s = '1' then

enable_ff_s <= '0';

end if;  end if;

end process Process_p1;

enable_ff_logic1_s <= '1' when counter_logic1_s = "111111" else '0';

enable_ff_logic2_s <= '1' when counter_logic2_s = "111111" else '0';

cmd_deg_os <= enable_ff_s;

end cmd_deglitch_a;

28) cmd_synchronizer_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

Library UNISIM;

use UNISIM.vcomponents.all;

entity cmd_synchronizer_e is

port(

clk_40MHz_is: in std_logic;

POR_G_is: in std_logic;

signal_in_is: in std_logic;

signal_sync_os : out std_logic );

end cmd_synchronizer_e;

architecture cmd_synchronizer_a of cmd_synchronizer_e is

signal sync_ff1_s,sync_ff2_s,sync_ff3_s : std_logic;

attribute TIG: string;

attribute ASYNC_REG: string;

attribute SHIFT_EXTRACT: string;

attribute ASYNC_REG of sync_ff1_s: signal is "TRUE";

attribute TIG of signal_in_is: signal is "TRUE";

attribute SHIFT_EXTRACT of sync_ff1_s: signal is "NO";

attribute SHIFT_EXTRACT of sync_ff2_s: signal is "NO";

attribute SHIFT_EXTRACT of sync_ff3_s: signal is "NO";

begin

P_synchronizer: process(clk_40MHz_is, POR_G_is)

begin

if POR_G_is = '1' then

sync_ff1_s <= '0'; --Initialized  to zero

sync_ff2_s <= '0'; --Initialized  to zero

sync_ff3_s <= '0'; --Initialized  to zero

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

sync_ff1_s <= signal_in_is;

sync_ff2_s <= sync_ff1_s;

sync_ff3_s <= sync_ff2_s;

end if;

end process P_synchronizer;

signal_sync_os <= sync_ff3_s;

end cmd_synchronizer_a;

29) code_bram_ip

LIBRARY ieee;

USE ieee.std_logic_1164.ALL;

-- synthesis translate_off

LIBRARY XilinxCoreLib;

-- synthesis translate_on

ENTITY code_bram_ip IS

PORT (

clka : IN STD_LOGIC;

ena : IN STD_LOGIC;

addra : IN STD_LOGIC_VECTOR(16 DOWNTO 0);

douta : OUT STD_LOGIC_VECTOR(51 DOWNTO 0)  );

END code_bram_ip;

ARCHITECTURE code_bram_ip_a OF code_bram_ip IS

-- synthesis translate_off

COMPONENT wrapped_code_bram_ip

PORT (

clka : IN STD_LOGIC;

ena : IN STD_LOGIC;

addra : IN STD_LOGIC_VECTOR(16 DOWNTO 0);

douta : OUT STD_LOGIC_VECTOR(51 DOWNTO 0));

END COMPONENT;

-- Configuration specification

FOR ALL : wrapped_code_bram_ip USE ENTITY XilinxCoreLib.blk_mem_gen_v6_2(behavioral)

GENERIC MAP (

c_addra_width => 17,

c_addrb_width => 17,

c_algorithm => 1,

c_axi_id_width => 4,

c_axi_slave_type => 0,

c_axi_type => 1,

c_byte_size => 9,

c_common_clk => 0,

c_default_data => "0",

c_disable_warn_bhv_coll => 0,

c_disable_warn_bhv_range => 0,

c_family => "virtex5",

c_has_axi_id => 0,

c_has_ena => 1,

c_has_enb => 0,

c_has_injecterr => 0,

c_has_mem_output_regs_a => 0,

c_has_mem_output_regs_b => 0,

c_has_mux_output_regs_a => 0,

c_has_mux_output_regs_b => 0,

c_has_regcea => 0,

c_has_regceb => 0,

c_has_rsta => 0,

c_has_rstb => 0,

c_has_softecc_input_regs_a => 0,

c_has_softecc_output_regs_b => 0,

c_init_file_name => "no_coe_file_loaded",

c_inita_val => "0",

c_initb_val => "0",

c_interface_type => 0,

c_load_init_file => 0,

c_mem_type => 3,

c_mux_pipeline_stages => 0,

c_prim_type => 1,

c_read_depth_a => 81920,

c_read_depth_b => 81920,

c_read_width_a => 52,

c_read_width_b => 52,

c_rst_priority_a => "CE",

c_rst_priority_b => "CE",

c_rst_type => "SYNC",

c_rstram_a => 0,

c_rstram_b => 0,

c_sim_collision_check => "ALL",

c_use_byte_wea => 0,

c_use_byte_web => 0,

c_use_default_data => 0,

c_use_ecc => 0,

c_use_softecc => 0,

c_wea_width => 1,

c_web_width => 1,

c_write_depth_a => 81920,

c_write_depth_b => 81920,

c_write_mode_a => "WRITE_FIRST",

c_write_mode_b => "WRITE_FIRST",

c_write_width_a => 52,

c_write_width_b => 52,

c_xdevicefamily => "virtex5"

);

-- synthesis translate_on

BEGIN

-- synthesis translate_off

U0 : wrapped_code_bram_ip

PORT MAP (

clka => clka,

ena => ena,

addra => addra,

douta => douta );

-- synthesis translate_on

END code_bram_ip_a;

30) config_crc_e

library ieee;

use ieee.std_logic_1164.all;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

Library UNISIM;

use UNISIM.vcomponents.all;

entity config_crc_e is

port(-- Inputs

clk_40MHz_is : in std_logic;

POR_in_is    : in std_logic;

-- Outputs

crc_error_os: out std_logic;

ecc_error_os: out std_logic;

syndrome_valid_os: out std_logic;

icap_busy_os: out std_logic;

syndrome_os  : out std_logic_vector(11 downto 0);

synd_valid_count_os: out std_logic_vector(7 downto 0) );

end config_crc_e;

architecture config_crc_a of config_crc_e is

signal syndrome_valid_s: std_logic;

signal synd_valid_count_s: std_logic_vector(7 downto 0);

begin

-- Built-in FRAME_ECC Instantiation.

FRAME_ECC_VIRTEX5_inst : FRAME_ECC_VIRTEX5

port map (

CRCERROR => crc_error_os,   -- 1-bit output indicating a CRC error

ECCERROR => ecc_error_os,   -- 1-bit output indicating an ECC error

SYNDROME => syndrome_os,    -- 12-bit output location of erroneous bit

SYNDROMEVALID => syndrome_valid_s  -- 1-bit output indicating the syndrome valid status.                                     -- SYNDROME output is valid

);

syndrome_valid_os <= syndrome_valid_s;

-- Built-in ICAP Instantiation.

ICAP_VIRTEX5_inst : ICAP_VIRTEX5

generic map (

ICAP_WIDTH => "X8")                      -- "X8", "X16" or "X32"

port map (

BUSY => icap_busy_os,  -- High during read, Low during write.

O => open,            -- 8-bit data output.

CE => '0',            -- Active-low: Chip-Enable, similiar to CS_B in SelectMAP.

CLK => clk_40MHz_is,  -- 40MHz Clock input.

I => (others=>'0'),   -- 8-bit data input.

WRITE => '1'          -- 1: Read , 0: Write.

);

-- Count syndrome valid pulses in the

P_synd_valid_counter: process(clk_40MHz_is,POR_in_is)

begin

if POR_in_is='1' then

synd_valid_count_s <=(others=>'0');

elsif clk_40MHz_is'event and  clk_40MHz_is='1' then

if syndrome_valid_s='1' then

synd_valid_count_s<=synd_valid_count_s+1;

end if;  end if;

end process P_synd_valid_counter;

synd_valid_count_os <= synd_valid_count_s;

end config_crc_a;

31) corr_data_out_mux_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use WORK.signal_type_p.ALL;

entity corr_data_out_mux_e is

port( --Inputs

svid_acq_is: in six_acq1_4_typ;

nav_data_reg_ch_is : in eight_ch1_18_typ;

sat_id_is : in six_ch1_18_typ;

code_cyle_count_value_ch_is: in ten_ch1_18_typ;

one_ms_counter_value_ch_is : in five_ch1_18_typ;

twenty_ms_counter_value_ch_is: in ten_ch1_18_typ;

code_phase_count_ch_is : in thirtytwo_ch1_18_typ;

car_cycle_count_ch_is : in thirtytwo_ch1_18_typ;

car_phase_count_ch_is : thirtytwo_ch1_18_typ;

p_cndr_is,q_cndr_is : in twentyeight_ch1_18_typ;

pll_order1_is, pll_order2_is, pll_order3_is: in thirtytwo_ch1_18_typ;

sbas_navdata_is : in std_logic_vector(5 downto 0);

rt_address_RT1_is,rt_address_RT2_is ,rt_address_RT3_is : in std_logic_vector(5 downto 0);

lock_detector_ant1_is,lock_detector_ant2_is : in std_logic;

lock_detector_ant3_is,lock_detector_ant4_is : in std_logic;

wdt_cmd_status_is : in std_logic_vector(1 downto 0);

up_address_is : in std_logic_vector(11 downto 0);

hdwt_reset_counter_is : in std_logic_vector(7 downto 0);

hwdt_reset_status_ff_is: in std_logic;

channel_status_is : in std_logic_vector(channels_count_c downto 0);

sat_status_is : in std_logic_vector(63 downto 0);

TSM_data_RT1_is, TSM_data_RT2_is, TSM_data_RT3_is : in std_logic_vector(31 downto 0);

soft_reset_flag_is : in std_logic;

upconv1_spi_gpio4_out_is : in std_logic;

upconv2_spi_gpio4_out_is : in std_logic;

ant1_2015_upst_cntr_is,ant2_2015_upst_cntr_is: in std_logic_vector(7 downto 0);

ant3_2015_upst_cntr_is,ant4_2015_upst_cntr_is: in std_logic_vector(7 downto 0);

acq_threshold_LSW_is :in std_logic_vector(31 downto 0);

acq_threshold_MSW_is :in std_logic_vector(31 downto 0);

trk_threshold_is :in std_logic_vector(31 downto 0);

acq1_search_reg_is   : in std_logic_vector(63 downto 0);

acq2_search_reg_is   : in std_logic_vector(63 downto 0);

acq3_search_reg_is   : in std_logic_vector(63 downto 0);

acq4_search_reg_is   : in std_logic_vector(63 downto 0);

fft_status_is       : in std_logic_vector(31 downto 0);

launch_orbit_phase_is: in std_logic;

carr_aided_loop_is   : in std_logic;

fll_zero_en_dis_is   : in std_logic;

config_crc_err_is: in std_logic;

config_ecc_err_is: in std_logic;

config_synd_valid_is: in std_logic;

icap_busy_is      : in std_logic;

syndrome_is      : in std_logic_vector(11 downto 0);

synd_valid_count_is: in std_logic_vector(7 downto 0);

rt_adddr_sel_is    : in std_logic_vector(1 downto 0);

antenna_combining_control_is : in std_logic_vector(3 downto 0);

lock_detector_threshold1_is: in std_logic_vector(9 downto 0);

lock_detector_threshold2_is: in std_logic_vector(9 downto 0);

mil_1553_interrupt_RT1_is: in std_logic;

mil_1553_interrupt_RT2_is: in std_logic;

mil_1553_interrupt_RT3_is: in std_logic;

SY_FE_write_count_is : in std_logic_vector(3 downto 0);

reg_val_read_is : in std_logic_vector(7 downto 0);

latched_incoming_inp1_countp7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp1_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp1_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp1_countp1_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp1_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp1_countm5_is : in std_logic_vector(25 downto 0);

latched_incoming_inp1_countm3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp1_countm1_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp1_countp7_is : in std_logic_vector(25 downto 0);

latched_incoming_qp1_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp1_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_qp1_countp1_is : in std_logic_vector(25 downto 0);

latched_incoming_qp1_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp1_countm5_is: in std_logic_vector(25 downto 0);

latched_incoming_qp1_countm3_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp1_countm1_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countp7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countp1_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countm5_is : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countm3_is : in std_logic_vector(25 downto 0);

atched_incoming_inp2_countm1_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp2_countp7_is : in std_logic_vector(25 downto 0);

latched_incoming_qp2_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp2_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_qp2_countp1_is : in std_logic_vector(25 downto 0);

latched_incoming_qp2_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp2_countm5_is: in std_logic_vector(25 downto 0);

latched_incoming_qp2_countm3_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp2_countm1_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countp7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countp1_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countm5_is : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countm3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countm1_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp3_countp7_is : in std_logic_vector(25 downto 0);

latched_incoming_qp3_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp3_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_qp3_countp1_is : in std_logic_vector(25 downto 0);

latched_incoming_qp3_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp3_countm5_is: in std_logic_vector(25 downto 0);

latched_incoming_qp3_countm3_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp3_countm1_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countp7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countp1_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countm5_is : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countm3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countm1_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp4_countp7_is : in std_logic_vector(25 downto 0);

latched_incoming_qp4_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp4_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_qp4_countp1_is : in std_logic_vector(25 downto 0);

latched_incoming_qp4_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp4_countm5_is: in std_logic_vector(25 downto 0);

latched_incoming_qp4_countm3_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp4_countm1_is  : in std_logic_vector(25 downto 0);

-- Outputs

data_to_processor_os : out std_logic_vector(31 downto 0);

uart_txd_is: in std_logic;

rwe3_is    : in std_logic;

wdt_l_is : in std_logic;

bexc_l_is: in std_logic);

end corr_data_out_mux_e;

architecture corr_data_out_mux_a of corr_data_out_mux_e is

begin

data_to_processor_os <=

x"000000" & "00" & svid_acq_is(0) when up_address_is = x"410" else

x"000000" & "00" & svid_acq_is(1) when up_address_is = x"411" else

x"000000" & "00" & svid_acq_is(2) when up_address_is = x"412" else

x"000000" & "00" & svid_acq_is(3) when up_address_is = x"413" else  					--channel 1

x"000000" & nav_data_reg_ch_is(0) when up_address_is = x"000" else

x"000000" & "00" & sat_id_is(0) when up_address_is = x"001" else

x"0000" & "000000" & code_cyle_count_value_ch_is(0) when up_address_is = x"004" else

x"000000" & "000" & one_ms_counter_value_ch_is(0) when up_address_is = x"005" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(0) when up_address_is = x"006" else

code_phase_count_ch_is(0) when up_address_is = x"007" else

car_cycle_count_ch_is(0) when up_address_is = x"008" else

car_phase_count_ch_is(0) when up_address_is = x"009" else		 pll_order1_is(0) when up_address_is = x"00a" else

pll_order2_is(0) when up_address_is = x"00b" else

pll_order3_is(0) when up_address_is = x"00c" else

--                               dll_order1_is(0) when up_address_is = x"00d" else

--                               carr_nco_val_is(0) when up_address_is = x"00e" else

--                               code_nco_val_is(0) when up_address_is = x"00f" else

x"0" & p_cndr_is(0) when up_address_is = x"017" else

x"0" & q_cndr_is(0) when up_address_is = x"018" else

-- channel 2

x"000000" & nav_data_reg_ch_is(1) when up_address_is = x"020" else

x"000000" & "00" & sat_id_is(1) when up_address_is = x"021" else

x"0000" & "000000" & code_cyle_count_value_ch_is(1) when up_address_is = x"024" else

x"000000" & "000" & one_ms_counter_value_ch_is(1) when up_address_is = x"025" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(1) when up_address_is = x"026" else

code_phase_count_ch_is(1) when up_address_is = x"027" else

car_cycle_count_ch_is(1) when up_address_is = x"028" else

car_phase_count_ch_is(1) when up_address_is = x"029" else

pll_order1_is(1) when up_address_is = x"02a" else

pll_order2_is(1) when up_address_is = x"02b" else

pll_order3_is(1) when up_address_is = x"02c" else

--                               dll_order1_is(1) when up_address_is = x"02d" else

--                               carr_nco_val_is(1) when up_address_is = x"02e" else

--                               code_nco_val_is(1) when up_address_is = x"02f" else

x"0" & p_cndr_is(1) when up_address_is = x"037" else

x"0" & q_cndr_is(1) when up_address_is = x"038" else

-- channel 3

x"000000" & nav_data_reg_ch_is(2) when up_address_is = x"040" else

x"000000" & "00" & sat_id_is(2) when up_address_is = x"041" else

x"0000" & "000000" & code_cyle_count_value_ch_is(2) when up_address_is = x"044" else

x"000000" & "000" & one_ms_counter_value_ch_is(2) when up_address_is = x"045" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(2) when up_address_is = x"046" else

code_phase_count_ch_is(2) when up_address_is = x"047" else

car_cycle_count_ch_is(2) when up_address_is = x"048" else

car_phase_count_ch_is(2) when up_address_is = x"049" else

pll_order1_is(2) when up_address_is = x"04a" else

pll_order2_is(2) when up_address_is = x"04b" else

pll_order3_is(2) when up_address_is = x"04c" else

--                               dll_order1_is(2) when up_address_is = x"04d" else

--                               carr_nco_val_is(2) when up_address_is = x"04e" else

--                               code_nco_val_is(2) when up_address_is = x"04f" else

x"0" & p_cndr_is(2) when up_address_is = x"057" else

x"0" & q_cndr_is(2) when up_address_is = x"058" else

-- channel 4

x"000000" & nav_data_reg_ch_is(3) when up_address_is = x"060" else

x"000000" & "00" & sat_id_is(3) when up_address_is = x"061" else

x"0000" & "000000" & code_cyle_count_value_ch_is(3) when up_address_is = x"064" else

x"000000" & "000" & one_ms_counter_value_ch_is(3) when up_address_is = x"065" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(3) when up_address_is = x"066" else

code_phase_count_ch_is(3) when up_address_is = x"067" else

car_cycle_count_ch_is(3) when up_address_is = x"068" else

car_phase_count_ch_is(3) when up_address_is = x"069" else

pll_order1_is(3) when up_address_is = x"06a" else

pll_order2_is(3) when up_address_is = x"06b" else

pll_order3_is(3) when up_address_is = x"06c" else

--                               dll_order1_is(3) when up_address_is = x"06d" else

--                               carr_nco_val_is(3) when up_address_is = x"06e" else

--                               code_nco_val_is(3) when up_address_is = x"06f" else

x"0" & p_cndr_is(3) when up_address_is = x"077" else

x"0" & q_cndr_is(3) when up_address_is = x"078" else

-- channel 5

x"000000" & nav_data_reg_ch_is(4) when up_address_is = x"080" else

x"000000" & "00" & sat_id_is(4) when up_address_is = x"081" else

x"0000" & "000000" & code_cyle_count_value_ch_is(4) when up_address_is = x"084" else

x"000000" & "000" & one_ms_counter_value_ch_is(4) when up_address_is = x"085" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(4) when up_address_is = x"086" else

code_phase_count_ch_is(4) when up_address_is = x"087" else

car_cycle_count_ch_is(4) when up_address_is = x"088" else

car_phase_count_ch_is(4) when up_address_is = x"089" else

pll_order1_is(4) when up_address_is = x"08a" else

pll_order2_is(4) when up_address_is = x"08b" else

pll_order3_is(4) when up_address_is = x"08c" else

--                               dll_order1_is(4) when up_address_is = x"08d" else

--                               carr_nco_val_is(4) when up_address_is = x"08e" else

--                               code_nco_val_is(4) when up_address_is = x"08f" else

x"0" & p_cndr_is(4) when up_address_is = x"097" else

x"0" & q_cndr_is(4) when up_address_is = x"098" else

-- channel 6

x"000000" & nav_data_reg_ch_is(5) when up_address_is = x"0a0" else

x"000000" & "00" & sat_id_is(5) when up_address_is = x"0a1" else		 x"0000" & "000000" & code_cyle_count_value_ch_is(5) when up_address_is = x"0a4" else

x"000000" & "000" & one_ms_counter_value_ch_is(5) when up_address_is = x"0a5" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(5) when up_address_is = x"0a6" else

code_phase_count_ch_is(5) when up_address_is = x"0a7" else

car_cycle_count_ch_is(5) when up_address_is = x"0a8" else

car_phase_count_ch_is(5) when up_address_is = x"0a9" else

pll_order1_is(5) when up_address_is = x"0aa" else

pll_order2_is(5) when up_address_is = x"0ab" else

pll_order3_is(5) when up_address_is = x"0ac" else

--                               dll_order1_is(5) when up_address_is = x"0ad" else

--                               carr_nco_val_is(5) when up_address_is = x"0ae" else

--                               code_nco_val_is(5) when up_address_is = x"0af" else

x"0" & p_cndr_is(5) when up_address_is = x"0b7" else

x"0" & q_cndr_is(5) when up_address_is = x"0b8" else

-- channel 7

x"000000" & nav_data_reg_ch_is(6) when up_address_is = x"0c0" else

x"000000" & "00" & sat_id_is(6) when up_address_is = x"0c1" else

x"0000" & "000000" & code_cyle_count_value_ch_is(6) when up_address_is = x"0c4" else

x"000000" & "000" & one_ms_counter_value_ch_is(6) when up_address_is = x"0c5" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(6) when up_address_is = x"0c6" else

code_phase_count_ch_is(6) when up_address_is = x"0c7" else

car_cycle_count_ch_is(6) when up_address_is = x"0c8" else

car_phase_count_ch_is(6) when up_address_is = x"0c9" else

pll_order1_is(6) when up_address_is = x"0ca" else

pll_order2_is(6) when up_address_is = x"0cb" else

pll_order3_is(6) when up_address_is = x"0cc" else

--                               dll_order1_is(6) when up_address_is = x"0cd" else

--                               carr_nco_val_is(6) when up_address_is = x"0ce" else

--                               code_nco_val_is(6) when up_address_is = x"0cf" else

x"0" & p_cndr_is(6) when up_address_is = x"0d7" else

x"0" & q_cndr_is(6) when up_address_is = x"0d8" else

-- channel 8

x"000000" & nav_data_reg_ch_is(7) when up_address_is = x"0e0" els          x"000000" & "00" & sat_id_is(7) when up_address_is = x"0e1" else

x"0000" & "000000" & code_cyle_count_value_ch_is(7) when up_address_is = x"0e4" else

x"000000" & "000" & one_ms_counter_value_ch_is(7) when up_address_is = x"0e5" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(7) when up_address_is = x"0e6" else

code_phase_count_ch_is(7) when up_address_is = x"0e7" else

car_cycle_count_ch_is(7) when up_address_is = x"0e8" else

car_phase_count_ch_is(7) when up_address_is = x"0e9" else

pll_order1_is(7) when up_address_is = x"0ea" else

pll_order2_is(7) when up_address_is = x"0eb" else

pll_order3_is(7) when up_address_is = x"0ec" else

--                               dll_order1_is(7) when up_address_is = x"0ed" else

--                               carr_nco_val_is(7) when up_address_is = x"0ee" else

--                               code_nco_val_is(7) when up_address_is = x"0ef" else

x"0" & p_cndr_is(7) when up_address_is = x"0f7" else

x"0" & q_cndr_is(7) when up_address_is = x"0f8" else

-- channel 9

x"000000" & nav_data_reg_ch_is(8) when up_address_is = x"100" else

x"000000" & "00" & sat_id_is(8) when up_address_is = x"101" else

x"0000" & "000000" & code_cyle_count_value_ch_is(8) when up_address_is = x"104" else

x"000000" & "000" & one_ms_counter_value_ch_is(8) when up_address_is = x"105" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(8) when up_address_is = x"106" else

code_phase_count_ch_is(8) when up_address_is = x"107" else

car_cycle_count_ch_is(8) when up_address_is = x"108" else

car_phase_count_ch_is(8) when up_address_is = x"109" else

pll_order1_is(8) when up_address_is = x"10a" else

pll_order2_is(8) when up_address_is = x"10b" else

pll_order3_is(8) when up_address_is = x"10c" else

--                               dll_order1_is(8) when up_address_is = x"10d" else

--                               carr_nco_val_is(8) when up_address_is = x"10e" else

--                               code_nco_val_is(8) when up_address_is = x"10f" else

x"0" & p_cndr_is(8) when up_address_is = x"117" else

x"0" & q_cndr_is(8) when up_address_is = x"118" else

-- channel 10

x"000000" & nav_data_reg_ch_is(9) when up_address_is = x"120" else

x"000000" & "00" & sat_id_is(9) when up_address_is = x"121" else

x"0000" & "000000" & code_cyle_count_value_ch_is(9) when up_address_is = x"124" else

x"000000" & "000" & one_ms_counter_value_ch_is(9) when up_address_is = x"125" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(9) when up_address_is = x"126" else

code_phase_count_ch_is(9) when up_address_is = x"127" else

car_cycle_count_ch_is(9) when up_address_is = x"128" else

car_phase_count_ch_is(9) when up_address_is = x"129" else

pll_order1_is(9) when up_address_is = x"12a" else

pll_order2_is(9) when up_address_is = x"12b" else

pll_order3_is(9) when up_address_is = x"12c" else

--                               dll_order1_is(9) when up_address_is = x"12d" else

--                               carr_nco_val_is(9) when up_address_is = x"12e" else

--                               code_nco_val_is(9) when up_address_is = x"12f" else

x"0" & p_cndr_is(9) when up_address_is = x"137" else

x"0" & q_cndr_is(9) when up_address_is = x"138" else

-- channel 11

x"000000" & nav_data_reg_ch_is(10) when up_address_is = x"140" else

x"000000" & "00" & sat_id_is(10) when up_address_is = x"141" else

x"0000" & "000000" & code_cyle_count_value_ch_is(10) when up_address_is = x"144" else

x"000000" & "000" & one_ms_counter_value_ch_is  (10) when up_address_is = x"145" else

x"00000" & "00" & twenty_ms_counter_value_ch_is (10) when up_address_is = x"146" else

code_phase_count_ch_is(10) when up_address_is = x"147" else

car_cycle_count_ch_is (10) when up_address_is = x"148" else

car_phase_count_ch_is (10) when up_address_is = x"149" else

pll_order1_is(10) when up_address_is = x"14a" else

pll_order2_is(10) when up_address_is = x"14b" else

pll_order3_is(10) when up_address_is = x"14c" else

--                               dll_order1_is(10) when up_address_is = x"14d" else

--                               carr_nco_val_is(10) when up_address_is = x"14e" else

--                               code_nco_val_is(10) when up_address_is = x"14f" else

x"0" & p_cndr_is(10) when up_address_is = x"157" else

x"0" & q_cndr_is(10) when up_address_is = x"158" else

-- channel 12

x"000000" & nav_data_reg_ch_is(11) when up_address_is = x"160" else

x"000000" & "00" & sat_id_is (11) when up_address_is = x"161" else

x"0000" & "000000" & code_cyle_count_value_ch_is(11) when up_address_is = x"164" else

x"000000" & "000" & one_ms_counter_value_ch_is(11) when up_address_is = x"165" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(11) when up_address_is = x"166" else

code_phase_count_ch_is(11) when up_address_is = x"167" else

car_cycle_count_ch_is(11) when up_address_is = x"168" else

car_phase_count_ch_is(11) when up_address_is = x"169" else

pll_order1_is(11) when up_address_is = x"16a" else

pll_order2_is(11) when up_address_is = x"16b" else

pll_order3_is(11) when up_address_is = x"16c" else

--                               dll_order1_is(11) when up_address_is = x"16d" else

--                               carr_nco_val_is(11) when up_address_is = x"16e" else

--                               code_nco_val_is(11) when up_address_is = x"16f" else

x"0" & p_cndr_is(11) when up_address_is = x"177" else

x"0" & q_cndr_is(11) when up_address_is = x"178" else

-- channel 13

x"000000" & nav_data_reg_ch_is(12) when up_address_is = x"180" else

x"000000" & "00" & sat_id_is(12) when up_address_is = x"181" else

x"0000" & "000000" & code_cyle_count_value_ch_is(12) when up_address_is = x"184" else

x"000000" & "000" & one_ms_counter_value_ch_is(12) when up_address_is = x"185" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(12) when up_address_is = x"186" else

code_phase_count_ch_is(12) when up_address_is = x"187" else

car_cycle_count_ch_is(12) when up_address_is = x"188" else

car_phase_count_ch_is(12) when up_address_is = x"189" else

pll_order1_is(12) when up_address_is = x"18a" else

pll_order2_is(12) when up_address_is = x"18b" else

pll_order3_is(12) when up_address_is = x"18c" else

--                               dll_order1_is(12) when up_address_is = x"18d" else

--                               carr_nco_val_is(12) when up_address_is = x"18e" else

--                               code_nco_val_is(12) when up_address_is = x"18f" else

x"0" & p_cndr_is(12) when up_address_is = x"197" else

x"0" & q_cndr_is(12) when up_address_is = x"198" else-- channel 14

x"000000" & nav_data_reg_ch_is(13) when up_address_is = x"1A0" else

x"000000" & "00" & sat_id_is(13) when up_address_is = x"1A1" else

x"0000" & "000000" & code_cyle_count_value_ch_is(13) when up_address_is = x"1A4" else

x"000000" & "000" & one_ms_counter_value_ch_is(13) when up_address_is = x"1A5" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(13) when up_address_is = x"1A6" else

code_phase_count_ch_is(13) when up_address_is = x"1A7" else

car_cycle_count_ch_is(13) when up_address_is = x"1A8" else

car_phase_count_ch_is(13) when up_address_is = x"1A9" else

pll_order1_is(13) when up_address_is = x"1aa" else

pll_order2_is(13) when up_address_is = x"1ab" else

pll_order3_is(13) when up_address_is = x"1ac" else

--                               dll_order1_is(13) when up_address_is = x"1ad" else

--                               carr_nco_val_is(13) when up_address_is = x"1ae" else

--                               code_nco_val_is(13) when up_address_is = x"1af" else

x"0" & p_cndr_is(13) when up_address_is = x"1B7" else

x"0" & q_cndr_is(13) when up_address_is = x"1B8" else	-- channel 15

x"000000" & nav_data_reg_ch_is(14) when up_address_is = x"1C0" else

x"000000" & "00" & sat_id_is(14) when up_address_is = x"1C1" else

x"0000" & "000000" & code_cyle_count_value_ch_is(14) when up_address_is = x"1C4" else

x"000000" & "000" & one_ms_counter_value_ch_is(14) when up_address_is = x"1C5" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(14) when up_address_is = x"1C6" else

code_phase_count_ch_is(14) when up_address_is = x"1C7" else

car_cycle_count_ch_is(14) when up_address_is = x"1C8" else

car_phase_count_ch_is(14) when up_address_is = x"1C9" else

pll_order1_is(14) when up_address_is = x"1ca" else

pll_order2_is(14) when up_address_is = x"1cb" else

pll_order3_is(14) when up_address_is = x"1cc" else

--                               dll_order1_is(14) when up_address_is = x"1cd" else

--                               carr_nco_val_is(14) when up_address_is = x"1ce" else

--                               code_nco_val_is(14) when up_address_is = x"1cf" else

x"0" & p_cndr_is(14) when up_address_is = x"1D7" else

x"0" & q_cndr_is(14) when up_address_is = x"1D8" else	-- channel 16

x"000000" & nav_data_reg_ch_is(15) when up_address_is = x"1E0" else

x"000000" & "00" & sat_id_is(15) when up_address_is = x"1E1" else

x"0000" & "000000" & code_cyle_count_value_ch_is(15) when up_address_is = x"1E4" else

x"000000" & "000" & one_ms_counter_value_ch_is(15) when up_address_is = x"1E5" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(15) when up_address_is = x"1E6" else

code_phase_count_ch_is(15) when up_address_is = x"1E7" else

car_cycle_count_ch_is(15) when up_address_is = x"1E8" else

car_phase_count_ch_is(15) when up_address_is = x"1E9" else

pll_order1_is(15) when up_address_is = x"1ea" else

pll_order2_is(15) when up_address_is = x"1eb" else

pll_order3_is(15) when up_address_is = x"1ec" else

--                               dll_order1_is(15) when up_address_is = x"1ed" else

--                               carr_nco_val_is(15) when up_address_is = x"1ee" else

--                               code_nco_val_is(15) when up_address_is = x"1ef" else

x"0" & p_cndr_is(15) when up_address_is = x"1F7" else

x"0" & q_cndr_is(15) when up_address_is = x"1F8" else	-- channel 17

x"000000" & nav_data_reg_ch_is(16) when up_address_is = x"200" else

x"000000" & "00" & sat_id_is(16) when up_address_is = x"201" else

x"0000" & "000000" & code_cyle_count_value_ch_is(16) when up_address_is = x"204" else

x"000000" & "000" & one_ms_counter_value_ch_is(16) when up_address_is = x"205" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(16) when up_address_is = x"206" else

code_phase_count_ch_is(16) when up_address_is = x"207" else

car_cycle_count_ch_is(16) when up_address_is = x"208" else

car_phase_count_ch_is(16) when up_address_is = x"209" else

pll_order1_is(16) when up_address_is = x"20a" else

pll_order2_is(16) when up_address_is = x"20b" else

pll_order3_is(16) when up_address_is = x"20c" else

--                               dll_order1_is(16) when up_address_is = x"20d" else

--                               carr_nco_val_is(16) when up_address_is = x"20e" else

--                               code_nco_val_is(16) when up_address_is = x"20f" else

x"0" & p_cndr_is(16) when up_address_is = x"217" else

x"0" & q_cndr_is(16) when up_address_is = x"218" else	-- channel 18

x"000000" & nav_data_reg_ch_is(17) when up_address_is = x"220" else

x"000000" & "00" & sat_id_is(17) when up_address_is = x"221" else

-- "000000" & car_nco_is(17) when up_address_is = x"222" else

-- "000000" & code_nco_is(17) when up_address_is = x"223" else

x"0000" & "000000" & code_cyle_count_value_ch_is(17) when up_address_is = x"224" else

x"000000" & "000" & one_ms_counter_value_ch_is(17) when up_address_is = x"225" else

x"00000" & "00" & twenty_ms_counter_value_ch_is(17) when up_address_is = x"226" else

code_phase_count_ch_is(17) when up_address_is = x"227" else

car_cycle_count_ch_is(17) when up_address_is = x"228" else

car_phase_count_ch_is(17) when up_address_is = x"229" else

pll_order1_is(17) when up_address_is = x"22a" else

pll_order2_is(17) when up_address_is = x"22b" else

pll_order3_is(17) when up_address_is = x"22c" else

--                               dll_order1_is(17) when up_address_is = x"22d" else

--                               carr_nco_val_is(17) when up_address_is = x"22e" else

--                               code_nco_val_is(17) when up_address_is = x"22f" else

x"0" & p_cndr_is(17) when up_address_is = x"237" else

x"0" & q_cndr_is(17) when up_address_is = x"238" else

x"000000" & "00" & sbas_navdata_is(5 downto 0) when up_address_is = x"23D" else -- Channel 18 for SBAS 							   		x"0" & mil_1553_interrupt_RT3_is & mil_1553_interrupt_RT2_is & mil_1553_interrupt_RT1_is & lock_detector_ant4_is & lock_detector_ant3_is  &lock_detector_ant2_is & lock_detector_ant1_is &

rt_address_RT3_is &  rt_address_RT2_is & rt_address_RT1_is & '0'& wdt_cmd_status_is when up_address_is = x"4C0" else

x"000000" & hdwt_reset_counter_is when up_address_is = x"4C3" else

x"000000" & "0000000" & hwdt_reset_status_ff_is when up_address_is = x"4C5" else

"00000000000000" & channel_status_is when up_address_is = x"414" else -- To be updated while changing number of channels

sat_status_is(31 downto 0) when up_address_is   = x"415" else

sat_status_is(63 downto 32) when up_address_is  = x"416" else

TSM_data_RT1_is when up_address_is = x"4C7" else		TSM_data_RT2_is when up_address_is = x"4D1" else

TSM_data_RT3_is when up_address_is = x"4D4" else

x"0000000" & "000" & soft_reset_flag_is when up_address_is = x"4CA" else

x"0000000" & "000" & upconv1_spi_gpio4_out_is when up_address_is = x"4CC" else

x"0000000" & "000" & upconv2_spi_gpio4_out_is when up_address_is = x"4D0" else

acq_threshold_LSW_is when up_address_is = x"4D7" else

acq_threshold_MSW_is when up_address_is = x"4D8" else

trk_threshold_is when up_address_is = x"4D9" else

ant4_2015_upst_cntr_is(7 downto 0)  & ant3_2015_upst_cntr_is(7 downto 0)  & ant2_2015_upst_cntr_is(7 downto 0)  & ant1_2015_upst_cntr_is(7 downto 0) when up_address_is = x"4DA" else

acq1_search_reg_is(31 downto 0)  when up_address_is = x"4DB" else

acq1_search_reg_is(63 downto 32) when up_address_is = x"4DC" else

acq2_search_reg_is(31 downto 0)  when up_address_is = x"4DD" else					acq2_search_reg_is(63 downto 32) when up_address_is = x"4DE" else

acq3_search_reg_is(31 downto 0)  when up_address_is = x"4DF" else

acq3_search_reg_is(63 downto 32) when up_address_is = x"4E0" else					acq4_search_reg_is(31 downto 0)  when up_address_is = x"4E1" else

acq4_search_reg_is(63 downto 32) when up_address_is = x"4E2" else

fft_status_is when up_address_is = x"4E3" else

X"0" & lock_detector_threshold2_is & lock_detector_threshold1_is & antenna_combining_control_is &'0'& fll_zero_en_dis_is  & carr_aided_loop_is  & launch_orbit_phase_is when up_address_is = x"4E7" else

"00" & bexc_l_is & wdt_l_is & rwe3_is & uart_txd_is & rt_adddr_sel_is & syndrome_is & synd_valid_count_is& icap_busy_is & config_synd_valid_is & config_ecc_err_is & config_crc_err_is when up_address_is = x"4E8" else

x"00000" & SY_FE_write_count_is & reg_val_read_is when up_address_is = x"4E9" else

"000000" &  latched_incoming_inp1_countp7_is when up_address_is = x"500" else

"000000" &  latched_incoming_inp1_countp5_is when up_address_is = x"501" else

"000000" &  latched_incoming_inp1_countp3_is when up_address_is = x"502" else

"000000" &  latched_incoming_inp1_countp1_is when up_address_is = x"503" else

"000000" &  latched_incoming_inp1_countm7_is when up_address_is = x"504" else

"000000" &  latched_incoming_inp1_countm5_is when up_address_is = x"505" else

"000000" &  latched_incoming_inp1_countm3_is when up_address_is = x"506" else

"000000" &  latched_incoming_inp1_countm1_is when up_address_is = x"507" else

"000000" &  latched_incoming_qp1_countp7_is when up_address_is = x"508" else

"000000" &  latched_incoming_qp1_countp5_is when up_address_is = x"509" else

"000000" &  latched_incoming_qp1_countp3_is when up_address_is = x"50A" else

"000000" &  latched_incoming_qp1_countp1_is when up_address_is = x"50B" else

"000000" &  latched_incoming_qp1_countm7_is when up_address_is = x"50C" else

"000000" &  latched_incoming_qp1_countm5_is when up_address_is = x"50D" else

"000000" &  latched_incoming_qp1_countm3_is when up_address_is = x"50E" else

"000000" &  latched_incoming_qp1_countm1_is when up_address_is = x"50F" else

"000000" &  latched_incoming_inp2_countp7_is when up_address_is = x"510" else

"000000" &  latched_incoming_inp2_countp5_is when up_address_is = x"511" else

"000000" &  latched_incoming_inp2_countp3_is when up_address_is = x"512" else

"000000" &  latched_incoming_inp2_countp1_is when up_address_is = x"513" else

"000000" &  latched_incoming_inp2_countm7_is when up_address_is = x"514" else

"000000" &  latched_incoming_inp2_countm5_is when up_address_is = x"515" else

"000000" &  latched_incoming_inp2_countm3_is when up_address_is = x"516" else

"000000" &  latched_incoming_inp2_countm1_is when up_address_is = x"517" else

"000000" &  latched_incoming_qp2_countp7_is when up_address_is = x"518" else

"000000" &  latched_incoming_qp2_countp5_is when up_address_is = x"519" else

"000000" &  latched_incoming_qp2_countp3_is when up_address_is = x"51A" else

"000000" &  latched_incoming_qp2_countp1_is when up_address_is = x"51B" else

"000000" &  latched_incoming_qp2_countm7_is when up_address_is = x"51C" else

"000000" &  latched_incoming_qp2_countm5_is when up_address_is = x"51D" else

"000000" &  latched_incoming_qp2_countm3_is when up_address_is = x"51E" else

"000000" &  latched_incoming_qp2_countm1_is when up_address_is = x"51F" else

"000000" &  latched_incoming_inp3_countp7_is when up_address_is = x"520" else

"000000" &  latched_incoming_inp3_countp5_is when up_address_is = x"521" else

"000000" &  latched_incoming_inp3_countp3_is when up_address_is = x"522" else

"000000" &  latched_incoming_inp3_countp1_is when up_address_is = x"523" else

"000000" &  latched_incoming_inp3_countm7_is when up_address_is = x"524" else

"000000" &  latched_incoming_inp3_countm5_is when up_address_is = x"525" else

"000000" &  latched_incoming_inp3_countm3_is when up_address_is = x"526" else

"000000" &  latched_incoming_inp3_countm1_is when up_address_is = x"527" else

"000000" &  latched_incoming_qp3_countp7_is when up_address_is = x"528" else

"000000" &  latched_incoming_qp3_countp5_is when up_address_is = x"529" else

"000000" &  latched_incoming_qp3_countp3_is when up_address_is = x"52A" else

"000000" &  latched_incoming_qp3_countp1_is when up_address_is = x"52B" else

"000000" &  latched_incoming_qp3_countm7_is when up_address_is = x"52C" else

"000000" &  latched_incoming_qp3_countm5_is when up_address_is = x"52D" else

"000000" &  latched_incoming_qp3_countm3_is when up_address_is = x"52E" else

"000000" &  latched_incoming_qp3_countm1_is when up_address_is = x"52F" else

"000000" &  latched_incoming_inp4_countp7_is when up_address_is = x"530" else

"000000" &  latched_incoming_inp4_countp5_is when up_address_is = x"531" else

"000000" &  latched_incoming_inp4_countp3_is when up_address_is = x"532" else

"000000" &  latched_incoming_inp4_countp1_is when up_address_is = x"533" else

"000000" &  latched_incoming_inp4_countm7_is when up_address_is = x"534" else

"000000" &  latched_incoming_inp4_countm5_is when up_address_is = x"535" else

"000000" &  latched_incoming_inp4_countm3_is when up_address_is = x"536" else

"000000" &  latched_incoming_inp4_countm1_is when up_address_is = x"537" else

"000000" &  latched_incoming_qp4_countp7_is when up_address_is = x"538" else

"000000" &  latched_incoming_qp4_countp5_is when up_address_is = x"539" else

"000000" &  latched_incoming_qp4_countp3_is when up_address_is = x"53A" else

"000000" &  latched_incoming_qp4_countp1_is when up_address_is = x"53B" else

"000000" &  latched_incoming_qp4_countm7_is when up_address_is = x"53C" else

"000000" &  latched_incoming_qp4_countm5_is when up_address_is = x"53D" else

"000000" &  latched_incoming_qp4_countm3_is when up_address_is = x"53E" else

"000000" &  latched_incoming_qp4_countm1_is when up_address_is = x"53F" else

(others => '0');

end corr_data_out_mux_a;

32) corr_reg_address_decoder_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use WORK.signal_type_p.ALL;

entity corr_reg_address_decoder_e is

port( -- Inputs

wr_pr_is,cs_correlator_is: in std_logic;

up_address_is : in std_logic_vector(11 downto 0);

-- Outputs

upconv1_spi_reg_wr_sel_os : out std_logic;

upconv2_spi_reg_wr_sel_os : out std_logic;

PR_to_FPGA_reset_reg_sel_os : out std_logic;

sbas_enable_reg_wr_sel_os   : out std_logic;

navic_cmd_enable_reg_wr_sel_os : out std_logic;

wdt_enable_pr_wr_enable_os : out std_logic;

clear_wdt_pr_os : out std_logic;

-- MIL interface signals

reset_tsm_interrupt_RT1_os : out std_logic;

reset_tsm_interrupt_RT2_os : out std_logic;

reset_tsm_interrupt_RT3_os : out std_logic;

reset_pps_interrupt_RT1_os : out std_logic;

reset_pps_interrupt_RT2_os : out std_logic;

reset_pps_interrupt_RT3_os : out std_logic;

track_th_sel_os : out std_logic;

hwdt_reset_clear_pr_wr_enable_os : out std_logic;

--  TIC Registers

ten_ms_reg_os: out std_logic;

TIC_intrpt_clear_os: out std_logic;

TIC_wr_enable_os: out std_logic;

clear_TIC_counter_os: out std_logic;

TIC_drift_wr_enable_os: out std_logic;

-- Antenna ,GPS/NAVIC almanac registers

ant1_GPS_wr_enable_os   : out std_logic_vector(acq_modules_count_c downto 0);

ant1_NavIC_wr_enable_os : out std_logic_vector(acq_modules_count_c downto 0);

ant1_SBAS_wr_enable_os  : out std_logic_vector(acq_modules_count_c downto 0);

ant2_GPS_wr_enable_os   : out std_logic_vector(acq_modules_count_c downto 0);

ant2_NavIC_wr_enable_os : out std_logic_vector(acq_modules_count_c downto 0);

ant2_SBAS_wr_enable_os  : out std_logic_vector(acq_modules_count_c downto 0);

ant3_GPS_wr_enable_os   : out std_logic_vector(acq_modules_count_c downto 0);

ant3_NavIC_wr_enable_os : out std_logic_vector(acq_modules_count_c downto 0);

ant3_SBAS_wr_enable_os  : out std_logic_vector(acq_modules_count_c downto 0);

ant4_GPS_wr_enable_os   : out std_logic_vector(acq_modules_count_c downto 0);

ant4_NavIC_wr_enable_os : out std_logic_vector(acq_modules_count_c downto 0);

ant4_SBAS_wr_enable_os  : out std_logic_vector(acq_modules_count_c downto 0);

reset_channel_en_os : out std_logic_vector(channels_count_c downto 0);

nav_data_read_over_ch_os: out std_logic_vector(channels_count_c downto 0);

reg_wr_procr_tms_ch_os : out std_logic_vector(channels_count_c downto 0);

comparator_th_lsw_sel_os : out std_logic;

comparator_th_msw_sel_os : out std_logic;

pps_pulse_count_data_en_os : out std_logic;

pps_on_off_sel_os : out std_logic;

-- PLL, FLL and DLL parameters

-- PLL, Set1, Launch phase, enable registers

pll_par1_set1_launch_enable_os: out std_logic ;

pll_par2_set1_launch_enable_os: out std_logic ;

pll_par3_set1_launch_enable_os: out std_logic ;

pll_par4_set1_launch_enable_os: out std_logic ;

-- FLL, Set1, Launch phase, enable registers

fll_par1_set1_launch_enable_os: out std_logic ;

fll_par2_set1_launch_enable_os: out std_logic ;

fll_par3_set1_launch_enable_os: out std_logic ;

-- DLL, Set1, Launch phase, enable registers

dll_par1_set1_launch_enable_os: out std_logic ;

dll_par2_set1_launch_enable_os: out std_logic ;

dll_par3_set1_launch_enable_os: out std_logic ;

-- PLL, Set2, Launch phase, enable registers

pll_par1_set2_launch_enable_os: out std_logic ;

pll_par2_set2_launch_enable_os: out std_logic ;

pll_par3_set2_launch_enable_os: out std_logic ;

pll_par4_set2_launch_enable_os: out std_logic ;

-- FLL, Set2, Launch phase, enable registers

fll_par1_set2_launch_enable_os: out std_logic ;

fll_par2_set2_launch_enable_os: out std_logic ;

fll_par3_set2_launch_enable_os: out std_logic ;

-- DLL, Set2, Launch phase, enable registers

dll_par1_set2_launch_enable_os: out std_logic ;

dll_par2_set2_launch_enable_os: out std_logic ;

dll_par3_set2_launch_enable_os: out std_logic ;

-- PLL, Set3, Launch phase, enable registers

pll_par1_set3_launch_enable_os: out std_logic ;

pll_par2_set3_launch_enable_os: out std_logic ;

pll_par3_set3_launch_enable_os: out std_logic ;

pll_par4_set3_launch_enable_os: out std_logic ;

-- FLL, Set3, Launch phase, enable registers

fll_par1_set3_launch_enable_os: out std_logic ;

fll_par2_set3_launch_enable_os: out std_logic ;

fll_par3_set3_launch_enable_os: out std_logic ;

-- DLL, Set3, Launch phase, enable registers

dll_par1_set3_launch_enable_os: out std_logic ;

dll_par2_set3_launch_enable_os: out std_logic ;

dll_par3_set3_launch_enable_os: out std_logic ;

-- PLL, Set4, Launch phase, enable registers

pll_par1_set4_launch_enable_os: out std_logic ;

pll_par2_set4_launch_enable_os: out std_logic ;

pll_par3_set4_launch_enable_os: out std_logic ;

pll_par4_set4_launch_enable_os: out std_logic ;

-- FLL, Set4, Launch phase, enable registers

fll_par1_set4_launch_enable_os: out std_logic ;

fll_par2_set4_launch_enable_os: out std_logic ;

fll_par3_set4_launch_enable_os: out std_logic ;

-- DLL, Set4, Launch phase, enable registers

dll_par1_set4_launch_enable_os: out std_logic ;

dll_par2_set4_launch_enable_os: out std_logic ;

dll_par3_set4_launch_enable_os: out std_logic ;

-- PLL, Set5, Launch phase, enable registers

pll_par1_set5_launch_enable_os: out std_logic ;

pll_par2_set5_launch_enable_os: out std_logic ;

pll_par3_set5_launch_enable_os: out std_logic ;

pll_par4_set5_launch_enable_os: out std_logic ;

-- FLL, Set5, Launch phase, enable registers

fll_par1_set5_launch_enable_os: out std_logic ;

fll_par2_set5_launch_enable_os: out std_logic ;

fll_par3_set5_launch_enable_os: out std_logic ;

-- DLL, Set5, Launch phase, enable registers

dll_par1_set5_launch_enable_os: out std_logic ;

dll_par2_set5_launch_enable_os: out std_logic ;

dll_par3_set5_launch_enable_os: out std_logic ;

cdll_par1_set1_launch_enable_os: out std_logic ;

--

launch_orbit_phase_sel_os      : out std_logic;

carr_aided_loop_sel_os         : out std_logic;

-- Timer for launch pghase

set1_to_set2_timer_launch_enable_os   : out std_logic;

set2_to_set3_timer_launch_enable_os   : out std_logic;

set3_to_set4_timer_launch_enable_os   : out std_logic;

set4_to_set5_timer_launch_enable_os   : out std_logic;

Narrow_corr_step1_timer_sel_os  : out  std_logic;

Narrow_corr_step2_timer_sel_os   : out  std_logic;

Narrow_correlator_step1_enable_sel_os  : out   std_logic;

Narrow_correlator_step2_enable_sel_os  : out  std_logic;

fll_timer_enable_os            : out std_logic;

fll_zero_en_dis_sel_os          : out std_logic;

loop_par_status1_enable_os      : out std_logic;

-- Miscellaneous

All_Timers_updated_enable_os   : out std_logic;

Antenna_combining_sel_enable_os : out std_logic;

lock_detector_threshold_sel_os : out std_logic;

FLL_data_check_counter_enable_os : out std_logic;

reset_mil_interrupt_RT1_os: out std_logic;

reset_mil_interrupt_RT2_os: out std_logic;

reset_mil_interrupt_RT3_os: out std_logic;

IF_FREQ_GPS_acq_sel_os: out std_logic;

IF_FREQ_NAVIC_acq_sel_os: out std_logic;

IF_FREQ_GPS_trk_sel_os: out std_logic;

IF_FREQ_NAVIC_trk_sel_os: out std_logic;

-------------saphyrion spi reg--------

reprogram_SY_cmd_sel_os  : out std_logic;

SY17_PLL_val_sel_os  : out std_logic;

SY17_AGC_val_sel_os  : out std_logic;

SYFE_PM_val_sel_os  : out std_logic;

SYFE_RFPLL_val_sel_os  : out std_logic;

SYFE_IFPLL_val_sel_os  : out std_logic

);

end corr_reg_address_decoder_e;

architecture corr_reg_address_decoder_a of corr_reg_address_decoder_e is

begin

PR_to_FPGA_reset_reg_sel_os   <= '1' when up_address_is = x"4CA" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

upconv1_spi_reg_wr_sel_os     <= '1' when up_address_is = x"4CB" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';  			sbas_enable_reg_wr_sel_os     <= '1' when up_address_is = x"4CD" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

navic_cmd_enable_reg_wr_sel_os <= '1' when up_address_is = x"4CE" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

upconv2_spi_reg_wr_sel_os            <= '1' when up_address_is = x"4CF" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0'; 							 				wdt_enable_pr_wr_enable_os <= '1' when up_address_is = x"4C1" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

clear_wdt_pr_os <= '1' when up_address_is = x"4C2" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_tsm_interrupt_RT1_os <= '1' when up_address_is = x"4C8" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_tsm_interrupt_RT2_os <= '1' when up_address_is = x"4D2" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_tsm_interrupt_RT3_os <= '1' when up_address_is = x"4D5" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_pps_interrupt_RT1_os <= '1' when up_address_is = x"4C9" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_pps_interrupt_RT2_os <= '1' when up_address_is = x"4D3" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_pps_interrupt_RT3_os <= '1' when up_address_is = x"4D6" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';				 		 hwdt_reset_clear_pr_wr_enable_os <=  '1' when up_address_is = x"4C6" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

comparator_th_lsw_sel_os <= '1' when up_address_is = x"420" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

comparator_th_msw_sel_os <= '1' when up_address_is = x"421" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ten_ms_reg_os <= '1' when up_address_is = x"400" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

TIC_intrpt_clear_os <= '1' when up_address_is = x"401" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

TIC_wr_enable_os <= '1' when up_address_is = x"402" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

clear_TIC_counter_os <= '1' when up_address_is = x"403" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

track_th_sel_os <=  '1' when up_address_is = x"404" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

pps_pulse_count_data_en_os <= '1' when up_address_is = x"405" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

pps_on_off_sel_os <= '1' when up_address_is = x"406" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

TIC_drift_wr_enable_os <= '1' when up_address_is = x"407" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

-- Acquisition module-1 almanace registers

ant1_GPS_wr_enable_os(0)    <= '1' when up_address_is = x"422" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant1_NavIC_wr_enable_os(0)  <= '1' when up_address_is = x"423" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant1_SBAS_wr_enable_os(0)   <= '1' when up_address_is = x"424" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant2_GPS_wr_enable_os(0)    <= '1' when up_address_is = x"425" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant2_NavIC_wr_enable_os(0)  <= '1' when up_address_is = x"426" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant2_SBAS_wr_enable_os(0)   <= '1' when up_address_is = x"427" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant3_GPS_wr_enable_os(0)    <= '1' when up_address_is = x"428" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant3_NavIC_wr_enable_os(0)   <= '1' when up_address_is = x"429" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant3_SBAS_wr_enable_os(0)   <= '1' when up_address_is = x"42A" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant4_GPS_wr_enable_os(0)    <= '1' when up_address_is = x"42B" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant4_NavIC_wr_enable_os(0)  <= '1' when up_address_is = x"42C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant4_SBAS_wr_enable_os(0)   <= '1' when up_address_is = x"42D" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

-- Acquisition module-2 almanace registers

ant1_GPS_wr_enable_os(1)    <= '1' when up_address_is = x"42E" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant1_NavIC_wr_enable_os(1)  <= '1' when up_address_is = x"42F" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant1_SBAS_wr_enable_os(1)   <= '1' when up_address_is = x"430" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant2_GPS_wr_enable_os(1)    <= '1' when up_address_is = x"431" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant2_NavIC_wr_enable_os(1)  <= '1' when up_address_is = x"432" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant2_SBAS_wr_enable_os(1)   <= '1' when up_address_is = x"433" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant3_GPS_wr_enable_os(1)    <= '1' when up_address_is = x"434" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant3_NavIC_wr_enable_os(1)   <= '1' when up_address_is = x"435" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant3_SBAS_wr_enable_os(1)   <= '1' when up_address_is = x"436" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant4_GPS_wr_enable_os(1)    <= '1' when up_address_is = x"437" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant4_NavIC_wr_enable_os(1)  <= '1' when up_address_is = x"438" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant4_SBAS_wr_enable_os(1)   <= '1' when up_address_is = x"439" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

-- Acquisition module-3 almanace registers

ant1_GPS_wr_enable_os(2)    <= '1' when up_address_is = x"43A" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant1_NavIC_wr_enable_os(2)  <= '1' when up_address_is = x"43B" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant1_SBAS_wr_enable_os(2)   <= '1' when up_address_is = x"43C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant2_GPS_wr_enable_os(2)    <= '1' when up_address_is = x"43D" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant2_NavIC_wr_enable_os(2)  <= '1' when up_address_is = x"43E" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant2_SBAS_wr_enable_os(2)   <= '1' when up_address_is = x"43F" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant3_GPS_wr_enable_os(2)    <= '1' when up_address_is = x"500" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant3_NavIC_wr_enable_os(2)   <= '1' when up_address_is = x"501" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant3_SBAS_wr_enable_os(2)   <= '1' when up_address_is = x"502" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant4_GPS_wr_enable_os(2)    <= '1' when up_address_is = x"503" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant4_NavIC_wr_enable_os(2)  <= '1' when up_address_is = x"504" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant4_SBAS_wr_enable_os(2)   <= '1' when up_address_is = x"505" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

-- Acquisition module-4 almanace registers

ant1_GPS_wr_enable_os(3)    <= '1' when up_address_is = x"506" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant1_NavIC_wr_enable_os(3)  <= '1' when up_address_is = x"507" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant1_SBAS_wr_enable_os(3)   <= '1' when up_address_is = x"508" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant2_GPS_wr_enable_os(3)    <= '1' when up_address_is = x"509" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant2_NavIC_wr_enable_os(3)  <= '1' when up_address_is = x"50A" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant2_SBAS_wr_enable_os(3)   <= '1' when up_address_is = x"50B" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant3_GPS_wr_enable_os(3)    <= '1' when up_address_is = x"50C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant3_NavIC_wr_enable_os(3)   <= '1' when up_address_is = x"50D" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant3_SBAS_wr_enable_os(3)   <= '1' when up_address_is = x"50E" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant4_GPS_wr_enable_os(3)    <= '1' when up_address_is = x"50F" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant4_NavIC_wr_enable_os(3)  <= '1' when up_address_is = x"510" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

ant4_SBAS_wr_enable_os(3)   <= '1' when up_address_is = x"511" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

-- Clearing tracking channels.

reset_channel_en_os(0) <= '1' when up_address_is = x"01C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(1) <= '1' when up_address_is = x"03C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(2) <= '1' when up_address_is = x"05C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(3) <= '1' when up_address_is = x"07C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(4) <= '1' when up_address_is = x"09C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(5) <= '1' when up_address_is = x"0BC" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(6) <= '1' when up_address_is = x"0DC" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(7) <= '1' when up_address_is = x"0FC" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(8) <= '1' when up_address_is = x"11C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(9) <= '1' when up_address_is = x"13C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(10) <='1' when up_address_is = x"15C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(11) <= '1' when up_address_is = x"17C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(12) <='1' when up_address_is = x"19C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(13) <='1' when up_address_is = x"1BC" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(14) <='1' when up_address_is = x"1DC" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(15) <='1' when up_address_is = x"1FC" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(16) <='1' when up_address_is = x"21C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reset_channel_en_os(17) <='1' when up_address_is = x"23C" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(0) <= '1' when up_address_is = x"000" and cs_correlator_is = '1'  and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(1) <= '1' when up_address_is = x"020" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(2) <= '1' when up_address_is = x"040" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(3) <= '1' when up_address_is = x"060" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(4) <= '1' when up_address_is = x"080" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(5) <= '1' when up_address_is = x"0A0" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(6) <= '1' when up_address_is = x"0C0" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(7) <= '1' when up_address_is = x"0E0" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(8) <= '1' when up_address_is = x"100" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(9) <= '1' when up_address_is = x"120" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(10) <='1' when up_address_is = x"140" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(11) <= '1' when up_address_is = x"160" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(12) <= '1' when up_address_is = x"180" and cs_correlator_is = '1'  and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(13) <= '1' when up_address_is = x"1A0" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(14) <= '1' when up_address_is = x"1C0" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(15) <= '1' when up_address_is = x"1E0" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(16) <= '1' when up_address_is = x"200" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

nav_data_read_over_ch_os(17) <= '1' when up_address_is = x"220" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(0) <= '1' when up_address_is = x"006" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(1) <= '1' when up_address_is = x"026" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(2) <= '1' when up_address_is = x"046" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(3) <= '1' when up_address_is = x"066" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(4) <= '1' when up_address_is = x"086" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(5) <= '1' when up_address_is = x"0A6" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(6) <= '1' when up_address_is = x"0C6" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(7) <= '1' when up_address_is = x"0E6" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(8) <= '1' when up_address_is = x"106" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(9) <= '1' when up_address_is = x"126" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(10) <='1' when up_address_is = x"146" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(11) <='1' when up_address_is = x"166" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(12) <='1' when up_address_is = x"186" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(13) <='1' when up_address_is = x"1A6" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(14) <= '1' when up_address_is = x"1C6" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(15) <= '1' when up_address_is = x"1E6" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(16) <= '1' when up_address_is = x"206" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

reg_wr_procr_tms_ch_os(17) <= '1' when up_address_is = x"226" and cs_correlator_is = '1' and wr_pr_is = '1' else

'0';

pll_par1_set1_launch_enable_os <= '1' when up_address_is = x"440" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par2_set1_launch_enable_os <= '1' when up_address_is = x"441" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par3_set1_launch_enable_os <= '1' when up_address_is = x"442" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par4_set1_launch_enable_os <= '1' when up_address_is = x"443" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par1_set1_launch_enable_os <= '1' when up_address_is = x"444" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par2_set1_launch_enable_os <= '1' when up_address_is = x"445" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par3_set1_launch_enable_os <= '1' when up_address_is = x"446" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par1_set1_launch_enable_os <= '1' when up_address_is = x"447" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par2_set1_launch_enable_os <= '1' when up_address_is = x"448" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par3_set1_launch_enable_os <= '1' when up_address_is = x"449" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par1_set2_launch_enable_os <= '1' when up_address_is = x"44A" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par2_set2_launch_enable_os <= '1' when up_address_is = x"44B" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par3_set2_launch_enable_os <= '1' when up_address_is = x"44C" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par4_set2_launch_enable_os <= '1' when up_address_is = x"44D" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par1_set2_launch_enable_os <= '1' when up_address_is = x"44E" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par2_set2_launch_enable_os <= '1' when up_address_is = x"44F" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par3_set2_launch_enable_os <= '1' when up_address_is = x"450" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par1_set2_launch_enable_os <= '1' when up_address_is = x"451" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par2_set2_launch_enable_os <= '1' when up_address_is = x"452" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par3_set2_launch_enable_os <= '1' when up_address_is = x"453" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par1_set3_launch_enable_os <= '1' when up_address_is = x"454" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par2_set3_launch_enable_os <= '1' when up_address_is = x"455" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par3_set3_launch_enable_os <= '1' when up_address_is = x"456" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par4_set3_launch_enable_os <= '1' when up_address_is = x"457" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par1_set3_launch_enable_os <= '1' when up_address_is = x"458" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par2_set3_launch_enable_os <= '1' when up_address_is = x"459" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par3_set3_launch_enable_os <= '1' when up_address_is = x"45A" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par1_set3_launch_enable_os <= '1' when up_address_is = x"45B" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par2_set3_launch_enable_os <= '1' when up_address_is = x"45C" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par3_set3_launch_enable_os <= '1' when up_address_is = x"45D" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par1_set4_launch_enable_os <= '1' when up_address_is = x"45E" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par2_set4_launch_enable_os <= '1' when up_address_is = x"45F" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par3_set4_launch_enable_os <= '1' when up_address_is = x"460" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par4_set4_launch_enable_os <= '1' when up_address_is = x"461" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par1_set4_launch_enable_os<= '1' when up_address_is = x"462" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par2_set4_launch_enable_os<= '1' when up_address_is = x"463" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par3_set4_launch_enable_os<= '1' when up_address_is = x"464" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par1_set4_launch_enable_os<= '1' when up_address_is = x"465" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par2_set4_launch_enable_os<= '1' when up_address_is = x"466" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par3_set4_launch_enable_os<= '1' when up_address_is = x"467" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

cdll_par1_set1_launch_enable_os<= '1' when up_address_is = x"468" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

launch_orbit_phase_sel_os      <= '1' when up_address_is = x"49C" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

carr_aided_loop_sel_os         <= '1' when up_address_is = x"49D" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

set1_to_set2_timer_launch_enable_os    <= '1' when up_address_is = x"49E" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

set2_to_set3_timer_launch_enable_os    <= '1' when up_address_is = x"49F" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

set3_to_set4_timer_launch_enable_os    <= '1' when up_address_is = x"4A0" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

set4_to_set5_timer_launch_enable_os    <= '1' when up_address_is = x"4A1" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_timer_enable_os             <= '1' when up_address_is = x"4A6" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_zero_en_dis_sel_os          <= '1' when up_address_is = x"4A7" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

loop_par_status1_enable_os       <= '1' when up_address_is = x"4A8" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par1_set5_launch_enable_os <= '1' when up_address_is = x"4AA" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par2_set5_launch_enable_os <= '1' when up_address_is = x"4AB" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par3_set5_launch_enable_os <= '1' when up_address_is = x"4AC" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

pll_par4_set5_launch_enable_os <= '1' when up_address_is = x"4AD" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par1_set5_launch_enable_os <= '1' when up_address_is = x"4AE" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par2_set5_launch_enable_os <= '1' when up_address_is = x"4AF" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

fll_par3_set5_launch_enable_os <= '1' when up_address_is = x"4B0" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par1_set5_launch_enable_os <= '1' when up_address_is = x"4B1" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par2_set5_launch_enable_os <= '1' when up_address_is = x"4B2" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

dll_par3_set5_launch_enable_os <= '1' when up_address_is = x"4B3" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

All_Timers_updated_enable_os    <= '1' when up_address_is = x"4BE" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

FLL_data_check_counter_enable_os <= '1' when up_address_is = x"4BF" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

Antenna_combining_sel_enable_os <= '1' when up_address_is = x"4D4" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

lock_detector_threshold_sel_os <= '1' when up_address_is = x"4D7" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

reset_mil_interrupt_RT1_os <='1' when up_address_is = x"4D8" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

reset_mil_interrupt_RT2_os <='1' when up_address_is = x"4D9" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

reset_mil_interrupt_RT3_os <='1' when up_address_is = x"4DA" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

IF_FREQ_GPS_acq_sel_os<='1' when up_address_is = x"4DB" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

IF_FREQ_NAVIC_acq_sel_os<='1' when up_address_is = x"4DC" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

IF_FREQ_GPS_trk_sel_os<='1' when up_address_is = x"4DD" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

IF_FREQ_NAVIC_trk_sel_os<='1' when up_address_is = x"4DE" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

Narrow_corr_step1_timer_sel_os  <= '1' when up_address_is = x"4E0" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

Narrow_corr_step2_timer_sel_os <= '1' when up_address_is = x"4E1" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

Narrow_correlator_step1_enable_sel_os  <= '1' when up_address_is = x"4E2" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

Narrow_correlator_step2_enable_sel_os <= '1' when up_address_is = x"4E3" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

-------------Saphyrion spi reg val--------------

reprogram_SY_cmd_sel_os <= '1' when up_address_is = x"4E4" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

SY17_PLL_val_sel_os <= '1' when up_address_is = x"4E5" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

SY17_AGC_val_sel_os <= '1' when up_address_is = x"4E6" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

SYFE_PM_val_sel_os <= '1' when up_address_is = x"4E7" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

SYFE_RFPLL_val_sel_os <= '1' when up_address_is = x"4E8" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

SYFE_IFPLL_val_sel_os <= '1' when up_address_is = x"4E9" and cs_correlator_is = '1' and wr_pr_is = '1' else '0';

end corr_reg_address_decoder_a;

33) edac_14_dec1_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity edac_14_dec1_e is

Port ( -- Input signals

rd_en_is       : in  STD_LOGIC;  -- EDAC Decoder  Read enable

data_in_is     : in  STD_LOGIC_VECTOR (13 downto 0);   -- Input Information bits to EDAC Decoder

par_in_is      : in  STD_LOGIC_VECTOR (5 downto 0);   -- Input Parity bits to EDAC Decoder

-- Output signals

data_out_os    : out  STD_LOGIC_VECTOR (13 downto 0)  -- Output information bits after correction );

end edac_14_dec1_e;

architecture edac_14_dec1_a of edac_14_dec1_e is

signal synd_s: STD_LOGIC_VECTOR(5 downto 0); -- Syndrome calculation

begin

process(rd_en_is,data_in_is,par_in_is,synd_s)

begin

if(rd_en_is='1') then   -- when  read is enabled for EDAC module,

-- syndrome computations, error detection and correction are carried out.

-- Syndrome computation

synd_s(5) <= data_in_is(13) xor  data_in_is(12) xor   data_in_is(11) xor  data_in_is(10) xor  data_in_is(5) xor data_in_is(4) xor data_in_is(1)  xor  par_in_is(5) ;

synd_s(4) <= data_in_is(13) xor  data_in_is(12) xor   data_in_is(11) xor  data_in_is(10) xor  data_in_is(3) xor data_in_is(2) xor data_in_is(0)  xor  par_in_is(4) ;

synd_s(3) <= data_in_is(13) xor  data_in_is(9)  xor   data_in_is(7)  xor  data_in_is(6)  xor  data_in_is(3) xor data_in_is(2) xor data_in_is(1)  xor  par_in_is(3) ;

synd_s(2) <= data_in_is(12) xor  data_in_is(9)  xor   data_in_is(8)  xor  data_in_is(6)  xor  data_in_is(5) xor data_in_is(2) xor data_in_is(1)  xor  par_in_is(2) ;

synd_s(1) <= data_in_is(11) xor  data_in_is(9)  xor   data_in_is(8)  xor  data_in_is(7)  xor  data_in_is(4) xor data_in_is(3) xor data_in_is(0)  xor  par_in_is(1) ;

synd_s(0) <= data_in_is(10)  xor data_in_is(8)  xor   data_in_is(7)  xor  data_in_is(6)  xor  data_in_is(5) xor data_in_is(4) xor data_in_is(0)  xor  par_in_is(0) ;

-- Error detection and correction based on syndrome

case  synd_s is

-- No errors in information bits

when "000000" =>    data_out_os<=data_in_is;

when "100000" =>    data_out_os<=data_in_is;

when "010000" =>    data_out_os<=data_in_is;

when "001000" =>    data_out_os<=data_in_is;

when "000100" =>    data_out_os<=data_in_is;

when "000010" =>    data_out_os<=data_in_is;

when "000001" =>    data_out_os<=data_in_is;

-- Single error correction

when "111000" =>   	data_out_os<= not data_in_is(13) & data_in_is(12 downto 0);

when "110100" =>   	data_out_os<=  data_in_is(13)&  not data_in_is(12) & data_in_is(11 downto 0);

when "110010" =>   	data_out_os<=  data_in_is(13 downto 12 )&  not data_in_is(11) & data_in_is(10 downto 0);

when "110001" =>   	data_out_os<=  data_in_is(13 downto 11 )&  not data_in_is(10) & data_in_is(9 downto 0);

when "001110" =>   	data_out_os<=  data_in_is(13 downto 10 )&  not data_in_is(9) & data_in_is(8 downto 0);

when "000111" =>   	data_out_os<=  data_in_is(13 downto 9 )&  not data_in_is(8) & data_in_is(7 downto 0);

when "001011" =>   	data_out_os<=  data_in_is(13 downto 8 )&  not data_in_is(7) & data_in_is(6 downto 0);

when "001101" =>   	data_out_os<=  data_in_is(13 downto 7)&   not data_in_is(6) & data_in_is(5 downto 0);

when "100101" =>   	data_out_os<=  data_in_is(13 downto 6)&  not data_in_is(5)& data_in_is(4 downto 0);

when "100011" =>  	data_out_os<=  data_in_is(13 downto 5)& not data_in_is(4)& data_in_is(3 downto 0);

when "011010" =>  	data_out_os<=  data_in_is(13 downto 4)&  not data_in_is(3) & data_in_is(2 downto 0);

when "011100" =>  	data_out_os<=  data_in_is(13 downto 3)&  not data_in_is(2) & data_in_is(1 downto 0) ;

when "101100" =>  	data_out_os<=  data_in_is(13 downto 2)&  not data_in_is(1) & data_in_is(0);

when "010011" =>  	data_out_os<=  data_in_is(13 downto 1)&  not data_in_is(0);

when OTHERS =>    	data_out_os<=	data_in_is;

end case;

else

synd_s<=(OTHERS=>'0');

data_out_os<=data_in_is;

end if;

end process;

end edac_14_dec1_a;

34) edac_14_enc1_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity edac_14_enc1_e is

Port (  -- Input signals

wr_en_is       : in  STD_LOGIC;  -- EDAC Encoder Write enable

data_in_is     : in  STD_LOGIC_VECTOR (13 downto 0);   -- Input Information bits to EDAC Ecnoder

-- Output signals

data_out_os    : out  STD_LOGIC_VECTOR (13 downto 0);  -- Output information bits after correction

par_out_os     : out  STD_LOGIC_VECTOR (5 downto 0)  -- Output parity bits

);

end edac_14_enc1_e;

architecture edac_14_enc1_a of edac_14_enc1_e is

begin

process(wr_en_is,data_in_is)

begin

if(wr_en_is='1') then  -- Enable writing into EDAC and calculate parity bits

-- Parity bit computation

par_out_os(5) <= data_in_is(13) xor  data_in_is(12) xor   data_in_is(11) xor  data_in_is(10) xor  data_in_is(5) xor data_in_is(4) xor data_in_is(1) ;

par_out_os(4) <= data_in_is(13) xor  data_in_is(12) xor   data_in_is(11) xor  data_in_is(10) xor  data_in_is(3) xor data_in_is(2) xor data_in_is(0) ;

par_out_os(3) <= data_in_is(13) xor  data_in_is(9)  xor   data_in_is(7)  xor  data_in_is(6)  xor  data_in_is(3) xor data_in_is(2) xor data_in_is(1) ;

par_out_os(2) <= data_in_is(12) xor  data_in_is(9)  xor   data_in_is(8)  xor  data_in_is(6)  xor  data_in_is(5) xor data_in_is(2) xor data_in_is(1) ;

par_out_os(1) <= data_in_is(11) xor  data_in_is(9)  xor   data_in_is(8)  xor  data_in_is(7)  xor  data_in_is(4) xor data_in_is(3) xor data_in_is(0) ;

par_out_os(0) <= data_in_is(10) xor data_in_is(8)   xor   data_in_is(7)  xor  data_in_is(6)  xor  data_in_is(5) xor data_in_is(4) xor data_in_is(0) ;

data_out_os<=data_in_is;

else

par_out_os<=(OTHERS=>'0');

data_out_os<=data_in_is;

end if;

end process;

end edac_14_enc1_a;

35) edac_16_dec1_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity edac_16_dec1_e is

Port (-- Input signals

rd_en_is       : in STD_LOGIC;  -- EDAC Read enable

data_in_is     : in  STD_LOGIC_VECTOR (15 downto 0);   -- Input Information bits to EDAC

par_in_is      : in  STD_LOGIC_VECTOR (5 downto 0);   -- Input Parity bits to EDAC

-- Output signals

data_out_os    : out  STD_LOGIC_VECTOR (15 downto 0)  -- Output information bits after correction

);

end edac_16_dec1_e;

architecture edac_16_dec1_a of edac_16_dec1_e is

signal synd_s: STD_LOGIC_VECTOR(5 downto 0); -- Syndrome calculation

begin

P_decoder:   process(rd_en_is,data_in_is,par_in_is,synd_s)

begin

if(rd_en_is='1') then   -- when  read is enabled for EDAC module,

-- syndrome computations, error detection and correction are carried out.

-- Syndrome computation

synd_s(5) <= data_in_is(15) xor  data_in_is(14)  xor   data_in_is(13) xor  data_in_is(12) xor  data_in_is(7)  xor data_in_is(6)  xor data_in_is(3)  xor data_in_is(1)  xor  par_in_is(5) ;

synd_s(4) <= data_in_is(15) xor  data_in_is(14)  xor   data_in_is(13) xor  data_in_is(12) xor  data_in_is(5)  xor data_in_is(4)  xor data_in_is(2)  xor data_in_is(0)  xor  par_in_is(4) ;

synd_s(3) <= data_in_is(15) xor  data_in_is(11)  xor   data_in_is(9)  xor  data_in_is(8)  xor  data_in_is(5)  xor data_in_is(4)  xor data_in_is(3)  xor data_in_is(1)  xor  par_in_is(3) ;

synd_s(2) <= data_in_is(14) xor  data_in_is(11)  xor   data_in_is(10) xor  data_in_is(8)  xor  data_in_is(7)  xor data_in_is(4)  xor data_in_is(3)  xor data_in_is(0)  xor  par_in_is(2) ;

synd_s(1) <= data_in_is(13) xor  data_in_is(11)  xor   data_in_is(10) xor  data_in_is(9)  xor  data_in_is(6)  xor data_in_is(5)  xor data_in_is(2)  xor data_in_is(1)  xor  par_in_is(1) ;

synd_s(0) <= data_in_is(12) xor  data_in_is(10)  xor   data_in_is(9)  xor  data_in_is(8)  xor  data_in_is(7)  xor data_in_is(6)  xor data_in_is(2)  xor data_in_is(0)  xor  par_in_is(0) ;

-- Error detection and correction based on syndrome

C_decoder: case  synd_s is

-- No errors in information bits

when "000000" =>   data_out_os<=data_in_is;

when "100000" =>   data_out_os<=data_in_is;

when "010000" =>   data_out_os<=data_in_is;

when "001000" =>   data_out_os<=data_in_is;

when "000100" =>   data_out_os<=data_in_is;

when "000010" =>   data_out_os<=data_in_is;

when "000001" =>   data_out_os<=data_in_is;

-- Single error correction

when "111000" =>   data_out_os<= not data_in_is(15) & data_in_is(14 downto 0);

when "110100" =>   data_out_os<=  data_in_is(15)&  not data_in_is(14) & data_in_is(13 downto 0);

when "110010" =>   data_out_os<=  data_in_is(15 downto 14)&  not data_in_is(13) & data_in_is(12 downto 0);

when "110001" =>   data_out_os<=  data_in_is(15 downto 13)&  not data_in_is(12) & data_in_is(11 downto 0);

when "001110" =>   data_out_os<=  data_in_is(15 downto 12)&  not data_in_is(11) & data_in_is(10 downto 0);

when "000111" =>   data_out_os<=  data_in_is(15 downto 11)&  not data_in_is(10) & data_in_is(9 downto 0);

when "001011" =>   data_out_os<=  data_in_is(15 downto 10)&  not data_in_is(9) & data_in_is(8 downto 0);

when "001101" =>   data_out_os<=  data_in_is(15 downto 9)&   not data_in_is(8) & data_in_is(7 downto 0);

when "100101" =>   data_out_os<=  data_in_is(15 downto 8)&  not data_in_is(7)& data_in_is(6 downto 0);

when  "100011" =>  data_out_os<=  data_in_is(15 downto 7)& not data_in_is(6)& data_in_is(5 downto 0);

when  "011010" =>  data_out_os<=  data_in_is(15 downto 6)&  not data_in_is(5) & data_in_is(4 downto 0);

when  "011100" =>  data_out_os<=  data_in_is(15 downto 5)&  not data_in_is(4) & data_in_is(3 downto 0) ;

when  "101100" =>  data_out_os<=  data_in_is(15 downto 4)&  not data_in_is(3) & data_in_is(2 downto 0);

when  "010011" =>  data_out_os<=  data_in_is(15 downto 3)&  not data_in_is(2) & data_in_is(1 downto 0);

when  "101010" =>  data_out_os<=  data_in_is(15 downto 2)&  not data_in_is(1) & data_in_is(0);

when  "010101" =>  data_out_os<=  data_in_is(15 downto 1)&  not data_in_is(0);

-- Multiple error detection data valid is asserted low.

when OTHERS =>

data_out_os<=data_in_is;

end case C_decoder;

else

synd_s<=(OTHERS=>'0');

data_out_os<=data_in_is;

end if;

end process P_decoder;

end edac_16_dec1_a;

36) edac_16_enc1_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity edac_16_enc1_e is

Port (-- Input signals

wr_en_is       : in STD_LOGIC;  -- EDAC Write enable

data_in_is     : in  STD_LOGIC_VECTOR (15 downto 0);   -- Input Information bits to EDAC

-- Output signals

data_out_os    : out  STD_LOGIC_VECTOR (15 downto 0);  -- Output information bits after correction

par_out_os     : out  STD_LOGIC_VECTOR (5 downto 0)  -- Output parity bits

);

end edac_16_enc1_e;

architecture edac_16_enc1_a of edac_16_enc1_e is

begin

P_encoding:   process(wr_en_is,data_in_is)

begin

if(wr_en_is='1') then  -- Enable writing into EDAC and calculate parity bits

-- Parity bit computation

par_out_os(5) <= data_in_is(15) xor  data_in_is(14)  xor   data_in_is(13) xor  data_in_is(12) xor  data_in_is(7)  xor data_in_is(6)  xor data_in_is(3)  xor data_in_is(1);

par_out_os(4) <= data_in_is(15) xor  data_in_is(14)  xor   data_in_is(13) xor  data_in_is(12) xor  data_in_is(5)  xor data_in_is(4)  xor data_in_is(2)  xor data_in_is(0);

par_out_os(3) <= data_in_is(15) xor  data_in_is(11)  xor   data_in_is(9)  xor  data_in_is(8)  xor  data_in_is(5)  xor data_in_is(4)  xor data_in_is(3)  xor data_in_is(1);

par_out_os(2) <= data_in_is(14) xor  data_in_is(11)  xor   data_in_is(10) xor  data_in_is(8)  xor  data_in_is(7)  xor data_in_is(4)  xor data_in_is(3)  xor data_in_is(0);

par_out_os(1) <= data_in_is(13) xor  data_in_is(11)  xor   data_in_is(10) xor  data_in_is(9)  xor  data_in_is(6)  xor data_in_is(5)  xor data_in_is(2)  xor data_in_is(1);

par_out_os(0) <= data_in_is(12) xor  data_in_is(10)  xor   data_in_is(9)  xor  data_in_is(8)  xor  data_in_is(7)  xor data_in_is(6)  xor data_in_is(2)  xor data_in_is(0);

data_out_os<=data_in_is;

else

par_out_os<=(OTHERS=>'0');

data_out_os<=data_in_is;

end if;

end process P_encoding;

end edac_16_enc1_a;

37) edac_28_dec1_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity edac_28_dec1_e is

Port ( -- Input signals

rd_en_is       : in  STD_LOGIC;  -- EDAC Read enable

data_in_is     : in  STD_LOGIC_VECTOR (27 downto 0);   -- Input Information bits to EDAC

par_in_is      : in  STD_LOGIC_VECTOR (6 downto 0);   -- Input Parity bits to EDAC

-- Output signals

data_out_os    : out  STD_LOGIC_VECTOR (27 downto 0) );

end edac_28_dec1_e;

architecture edac_28_dec1_a of edac_28_dec1_e is

signal synd_s: STD_LOGIC_VECTOR(6 downto 0); -- Syndrome calculation

begin

process(rd_en_is,data_in_is,par_in_is,synd_s)

begin

if(rd_en_is='1') then   -- when  read is enabled for EDAC module,

-- Error detection and correction based on syndrome

synd_s(6)<=data_in_is(27) xor data_in_is(26) xor data_in_is(25) xor data_in_is(24) xor data_in_is(23) xor data_in_is(22) xor data_in_is(21) xor data_in_is(20) xor data_in_is(19) xor data_in_is(18) xor data_in_is(17) xor data_in_is(16) xor data_in_is(15) xor data_in_is(14) xor data_in_is(13) xor par_in_is(6) ;

synd_s(5)<=data_in_is(27) xor data_in_is(26) xor data_in_is(25) xor data_in_is(24) xor data_in_is(23) xor data_in_is(12) xor data_in_is(11) xor data_in_is(10) xor data_in_is(9) xor data_in_is(8) xor data_in_is(7) xor data_in_is(6) xor data_in_is(5) xor data_in_is(4) xor data_in_is(3) xor par_in_is(5) ;

synd_s(4)<=data_in_is(27) xor data_in_is(22) xor data_in_is(21) xor data_in_is(20) xor data_in_is(19) xor data_in_is(6) xor data_in_is(5) xor data_in_is(4) xor data_in_is(3) xor data_in_is(2) xor data_in_is(1) xor data_in_is(0) xor par_in_is(4) ;

synd_s(3)<=data_in_is(26) xor data_in_is(22) xor data_in_is(18) xor data_in_is(17) xor data_in_is(16) xor data_in_is(9) xor data_in_is(8) xor data_in_is(7) xor data_in_is(3) xor par_in_is(3) ;

synd_s(2)<=data_in_is(25) xor data_in_is(21) xor data_in_is(18) xor data_in_is(15) xor data_in_is(14) xor data_in_is(11) xor data_in_is(10) xor data_in_is(7) xor data_in_is(4) xor data_in_is(1) xor data_in_is(0) xor par_in_is(2) ;

synd_s(1)<=data_in_is(24) xor data_in_is(20) xor data_in_is(17) xor data_in_is(15) xor data_in_is(13) xor data_in_is(12) xor data_in_is(10) xor data_in_is(8) xor data_in_is(5) xor data_in_is(2) xor data_in_is(0) xor par_in_is(1) ;

synd_s(0)<=data_in_is(23) xor data_in_is(19) xor data_in_is(16) xor data_in_is(14) xor data_in_is(13) xor data_in_is(12) xor data_in_is(11) xor data_in_is(9) xor data_in_is(6) xor data_in_is(2) xor data_in_is(1) xor par_in_is(0) ;

case  synd_s is

-- No errors in information bits

When "0000000" => 	data_out_os<=data_in_is;

When "1000000" => 	data_out_os<=data_in_is;

When "0100000" => 	data_out_os<=data_in_is;

When "0010000" => 	data_out_os<=data_in_is;

When "0001000" => 	data_out_os<=data_in_is;

When "0000100" => 	data_out_os<=data_in_is;

When "0000010" => 	data_out_os<=data_in_is;

When "0000001" => 	data_out_os<=data_in_is;

When "1110000"  => 	data_out_os<= not data_in_is(27) & data_in_is(26 downto 0);

When "1101000"  => 	data_out_os<= data_in_is(27)& not data_in_is(26) & data_in_is(25 downto 0);

When "1100100"  => 	data_out_os<= data_in_is(27 downto 26 ) &  not data_in_is(25) & data_in_is(24 downto 0);

When "1100010"  => 	data_out_os<= data_in_is(27 downto 25 ) &  not data_in_is(24) & data_in_is(23 downto 0);

When "1100001"  => 	data_out_os<= data_in_is(27 downto 24 ) &  not data_in_is(23) & data_in_is(22 downto 0);

When "1011000"  => 	data_out_os<= data_in_is(27 downto 23 ) &  not data_in_is(22) & data_in_is(21 downto 0);

When "1010100"  => 	data_out_os<= data_in_is(27 downto 22 ) &  not data_in_is(21) & data_in_is(20 downto 0);

When "1010010"  => 	data_out_os<= data_in_is(27 downto 21 ) &  not data_in_is(20) & data_in_is(19 downto 0);

When "1010001"  => 	data_out_os<= data_in_is(27 downto 20 ) &  not data_in_is(19) & data_in_is(18 downto 0);

When "1001100"  => 	data_out_os<= data_in_is(27 downto 19 ) &  not data_in_is(18) & data_in_is(17 downto 0);

When "1001010"  => 	data_out_os<= data_in_is(27 downto 18 ) &  not data_in_is(17) & data_in_is(16 downto 0);

When "1001001"  => 	data_out_os<= data_in_is(27 downto 17 ) &  not data_in_is(16) & data_in_is(15 downto 0);

When "1000110"  => 	data_out_os<= data_in_is(27 downto 16 ) &  not data_in_is(15) & data_in_is(14 downto 0);

When "1000101"  => 	data_out_os<= data_in_is(27 downto 15 ) &  not data_in_is(14) & data_in_is(13 downto 0);

When "1000011"  => 	data_out_os<= data_in_is(27 downto 14 ) &  not data_in_is(13) & data_in_is(12 downto 0);

When "0100011"  => 	data_out_os<= data_in_is(27 downto 13 ) &  not data_in_is(12) & data_in_is(11 downto 0);

When "0100101"  => 	data_out_os<= data_in_is(27 downto 12 ) &  not data_in_is(11) & data_in_is(10 downto 0);

When "0100110"  => 	data_out_os<= data_in_is(27 downto 11 ) &  not data_in_is(10) & data_in_is(9 downto 0);

When "0101001"  => 	data_out_os<= data_in_is(27 downto 10 ) &  not data_in_is(9) & data_in_is(8 downto 0);

When "0101010"  => 	data_out_os<= data_in_is(27 downto 9 ) &  not data_in_is(8) & data_in_is(7 downto 0);

When "0101100"  => 	data_out_os<= data_in_is(27 downto 8 ) &  not data_in_is(7) & data_in_is(6 downto 0);

When "0110001"  => 	data_out_os<= data_in_is(27 downto 7 ) &  not data_in_is(6) & data_in_is(5 downto 0);

When "0110010"  => 	data_out_os<= data_in_is(27 downto 6 ) &  not data_in_is(5) & data_in_is(4 downto 0);

When "0110100"  => 	data_out_os<= data_in_is(27 downto 5 ) &  not data_in_is(4) & data_in_is(3 downto 0);

When "0111000"  => 	data_out_os<= data_in_is(27 downto 4 ) &  not data_in_is(3) & data_in_is(2 downto 0);

When "0010011"  => 	data_out_os<= data_in_is(27 downto 3 ) &  not data_in_is(2) & data_in_is(1 downto 0);

When "0010101"  => 	data_out_os<= data_in_is(27 downto 2 ) &  not data_in_is(1) & data_in_is(0 downto 0);

When "0010110"  => 	data_out_os<= data_in_is(27 downto 1) & not data_in_is(0);

when OTHERS =>      data_out_os<= data_in_is;

end case;

else

synd_s<=(OTHERS=>'0');

data_out_os<=data_in_is;

end if;

end process;

end edac_28_dec1_a;

38) edac_28_enc1_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity edac_28_enc1_e is

Port ( -- Input signals

wr_en_is       : in  STD_LOGIC;  -- EDAC Write enable

data_in_is     : in  STD_LOGIC_VECTOR  (27 downto 0);   -- Input Information bits to EDAC

-- Output signals

data_out_os    : out  STD_LOGIC_VECTOR (27 downto 0);  -- Output information bits after correction

par_out_os     : out  STD_LOGIC_VECTOR (6 downto 0)    -- Output parity bits

);

end edac_28_enc1_e;

architecture edac_28_enc1_a of edac_28_enc1_e is

begin

process(wr_en_is,data_in_is)

begin

if(wr_en_is='1') then  -- Enable writing into EDAC and calculate parity bits

par_out_os(6)<=data_in_is(27) xor data_in_is(26) xor data_in_is(25) xor data_in_is(24) xor data_in_is(23) xor data_in_is(22) xor data_in_is(21) xor data_in_is(20) xor data_in_is(19) xor data_in_is(18) xor data_in_is(17) xor data_in_is(16) xor data_in_is(15) xor data_in_is(14) xor data_in_is(13) ;

par_out_os(5)<=data_in_is(27) xor data_in_is(26) xor data_in_is(25) xor data_in_is(24) xor data_in_is(23) xor data_in_is(12) xor data_in_is(11) xor data_in_is(10) xor data_in_is(9) xor data_in_is(8) xor data_in_is(7) xor data_in_is(6) xor data_in_is(5) xor data_in_is(4) xor data_in_is(3) ;

par_out_os(4)<=data_in_is(27) xor data_in_is(22) xor data_in_is(21) xor data_in_is(20) xor data_in_is(19) xor data_in_is(6) xor data_in_is(5) xor data_in_is(4) xor data_in_is(3) xor data_in_is(2) xor data_in_is(1) xor data_in_is(0) ;

par_out_os(3)<=data_in_is(26) xor data_in_is(22) xor data_in_is(18) xor data_in_is(17) xor data_in_is(16) xor data_in_is(9) xor data_in_is(8) xor data_in_is(7) xor data_in_is(3) ;

par_out_os(2)<=data_in_is(25) xor data_in_is(21) xor data_in_is(18) xor data_in_is(15) xor data_in_is(14) xor data_in_is(11) xor data_in_is(10) xor data_in_is(7) xor data_in_is(4) xor data_in_is(1) xor data_in_is(0) ;

par_out_os(1)<=data_in_is(24) xor data_in_is(20) xor data_in_is(17) xor data_in_is(15) xor data_in_is(13) xor data_in_is(12) xor data_in_is(10) xor data_in_is(8) xor data_in_is(5) xor data_in_is(2) xor data_in_is(0) ;

par_out_os(0)<=data_in_is(23) xor data_in_is(19) xor data_in_is(16) xor data_in_is(14) xor data_in_is(13) xor data_in_is(12) xor data_in_is(11) xor data_in_is(9) xor data_in_is(6) xor data_in_is(2) xor data_in_is(1) ;

data_out_os<=data_in_is;

else

par_out_os<=(OTHERS=>'0');

data_out_os<=data_in_is;

end if;

end process;

end edac_28_enc1_a;

39) edac_top_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.NUMERIC_STD.ALL;

entity edac_top_e is

Port(  -- Inputs signals

sig_accum_inphase_enc_is:in  std_logic_vector(27 downto 0);

sig_accum_qphase_enc_is:in  std_logic_vector(27 downto 0);

sig_accum_cphase_enc_is:in  std_logic_vector(27 downto 0);

sig_accum_inphase_dec_is:in  std_logic_vector(34 downto 0);

edac_wr_en_is: in std_logic;

edac_rd_en_is: in std_logic;

--Output signals

sig_accum_inphase_enc_os:out  std_logic_vector(34 downto 0);

sig_accum_qphase_enc_os:out  std_logic_vector(34 downto 0);

sig_accum_cphase_enc_os:out  std_logic_vector(34 downto 0);

sig_accum_inphase_dec_os:out  std_logic_vector(27 downto 0)

);

end edac_top_e;

architecture edac_top_a of edac_top_e is

--18-bit EDAC Encoder component declaration

COMPONENT edac_28_enc1_e

PORT(

wr_en_is : IN std_logic;

data_in_is : IN std_logic_vector(27 downto 0);

data_out_os : OUT std_logic_vector(27 downto 0);

par_out_os : OUT std_logic_vector(6 downto 0)

);

END COMPONENT;

--18-bit EDAC Decoder component declaration

COMPONENT edac_28_dec1_e

PORT(

rd_en_is : IN std_logic;

data_in_is : IN std_logic_vector(27 downto 0);

par_in_is : IN std_logic_vector(6 downto 0);

data_out_os : OUT std_logic_vector(27 downto 0)

);

END COMPONENT;

begin

-- Instatntiation of EDAC Encoder for inphase data

inphase_edac_28_enc1_e: edac_28_enc1_e

PORT MAP(

wr_en_is => edac_wr_en_is,

data_in_is => sig_accum_inphase_enc_is ,

data_out_os => sig_accum_inphase_enc_os(34 downto 7),

par_out_os => sig_accum_inphase_enc_os(6 downto 0)

);

-- Instatntiation of EDAC Decoder for inphase data

inphase_edac_28_dec1_e: edac_28_dec1_e

PORT MAP(

rd_en_is => edac_rd_en_is ,

data_in_is => sig_accum_inphase_dec_is(34 downto 7),

par_in_is => sig_accum_inphase_dec_is(6 downto 0),

data_out_os => sig_accum_inphase_dec_os

);

-- Instatntiation of EDAC Encoder for qphase data

qphase_edac_28_enc1_e: edac_28_enc1_e

PORT MAP(

wr_en_is => edac_wr_en_is,

data_in_is => sig_accum_qphase_enc_is ,

data_out_os => sig_accum_qphase_enc_os(34 downto 7),

par_out_os => sig_accum_qphase_enc_os(6 downto 0)

);

-- Instatntiation of EDAC Encoder for cphase data

cphase_edac_28_enc1_e: edac_28_enc1_e

PORT MAP(

wr_en_is => edac_wr_en_is,

data_in_is => sig_accum_cphase_enc_is ,

data_out_os => sig_accum_cphase_enc_os(34 downto 7),

par_out_os => sig_accum_cphase_enc_os(6 downto 0)

);

end edac_top_a;

40) fft_core_2048_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity fft_core_2048_e is

generic(input_width_g      : integer  := 14;   -- input data width.

fft_size_log_g     : integer  := 11;  -- log2(transform size).

trunc_g            : integer  := 11;   --

radix2_size1_g     : integer  := 37;  -- input_width_g+trunc_g +fft_size_log_g+1.

radix2_size2_g     : integer  := 53   -- radix2_size1_g+16.);

port (

clk_is: in std_logic;

reset_is: in std_logic;

start_is : in std_logic;

unload_is : in std_logic;

xn_re_is: in std_logic_vector(input_width_g-1 downto 0); --Real input data for FFT Engine.

xn_im_is: in std_logic_vector(input_width_g-1 downto 0); --Imaginary input data for FFT Engine.

xn_index_os: out  std_logic_vector(fft_size_log_g-1 downto 0); -- FFT input data index..0 to 255.

-- edone_os: out std_logic;

done_os:  out std_logic;

fft_status_os : out std_logic_vector(7 downto 0);

xk_index_os: out std_logic_vector(fft_size_log_g-1 downto 0); --FFT output index.

xk_re_os: out std_logic_vector(input_width_g+fft_size_log_g downto 0); --FFT output- real data.

xk_im_os: out std_logic_vector(input_width_g+fft_size_log_g downto 0) --FFT output- imaginary data.

);

end fft_core_2048_e ;

architecture fft_core_2048_a of fft_core_2048_e is

component radix2_2048pt_e

generic(input_width_g      : integer  ;

fft_size_log_g     : integer  ;

radix2_size1_g     : integer  ;

radix2_size2_g     : integer  );

port(-- Clock and reset inputs.

clk_is: in std_logic;

reset_is: in std_logic;

-- Radix-2 butterfly inputs

x0_re_is: in std_logic_vector (radix2_size1_g-1 downto 0);

x0_im_is: in std_logic_vector (radix2_size1_g-1 downto 0);

x1_re_is: in std_logic_vector (radix2_size1_g-1 downto 0);

x1_im_is: in std_logic_vector (radix2_size1_g-1 downto 0);

twiddle_re_is: in std_logic_vector(15 downto 0); --16-bit twiddle/phase factor: real input

twiddle_im_is: in std_logic_vector(15 downto 0); --16-bit twiddle/phase factor: imaginary input

-- Radix-2 butterfly outputs

Y0_RE_OS: out std_logic_vector(radix2_size1_g-1 downto 0);

Y0_IM_OS: out std_logic_vector(radix2_size1_g-1 downto 0);

Y1_RE_OS: out std_logic_vector(radix2_size1_g-1 downto 0);

Y1_IM_OS: out std_logic_vector(radix2_size1_g-1 downto 0));

end component;

-- Address generation component instantiaiton

COMPONENT addr_gen_2048_e

GENERIC(fft_size_log_g     : integer  );

PORT(clk_is : IN std_logic;

reset_is : IN std_logic;

start_is : IN std_logic;

unload_is : IN std_logic;

stage_count_is : IN std_logic_vector(3 downto 0);

group_count_is : IN std_logic_vector(fft_size_log_g-2 downto 0);

xn_index_os : OUT std_logic_vector(fft_size_log_g-1 downto 0);

xk_index_os : OUT std_logic_vector(fft_size_log_g-1 downto 0);

xk_index1_os      :OUT  std_logic_vector(fft_size_log_g-1 downto 0);

xn_index1_os : OUT std_logic_vector(fft_size_log_g-1 downto 0);

xk_count_en_os : OUT std_logic;

fft_ram1_raddr_os : OUT std_logic_vector(fft_size_log_g-1 downto 0);

fft_ram2_raddr_os : OUT std_logic_vector(fft_size_log_g-1 downto 0);

fft_rom_addr_os : OUT std_logic_vector(fft_size_log_g-2 downto 0));

END COMPONENT;

--fsm signals

type state_type is (s0,s1,s2,s3,s4,s5); --FSM State type

signal fsm_state_s: state_type; --FSM State variable

signal prev_state_s: state_type; -- Previous State variable

signal stage_done_s: std_logic;

signal group_count_s : std_logic_vector(fft_size_log_g-2 downto 0);

signal stage_count_s : std_logic_vector(3 downto 0);

signal group_count_en_s: std_logic;

signal stage_count_en_s: std_logic;

signal done_s: std_logic;

--signal edone_s: std_logic;

signal fft_done_s : std_logic;

signal xn_index1_s: std_logic_vector(fft_size_log_g-1 downto 0);

signal ram1_addra_s,ram2_addra_s  : std_logic_vector(fft_size_log_g-1 downto 0);

signal iram1_addrb_s,iram2_addrb_s  : std_logic_vector(fft_size_log_g-1 downto 0);

signal qram1_addrb_s,qram2_addrb_s  : std_logic_vector(fft_size_log_g-1 downto 0);

signal iram1_dina_s,iram2_dina_s   : std_logic_vector(radix2_size1_g-1 downto 0);

signal qram1_dina_s,qram2_dina_s   : std_logic_vector(radix2_size1_g-1 downto 0);

signal iram1_doutb_s,iram2_doutb_s  : std_logic_vector(radix2_size1_g-1 downto 0);

signal qram1_doutb_s,qram2_doutb_s  : std_logic_vector(radix2_size1_g-1 downto 0);

signal iram1_wea_s, iram2_wea_s  : std_logic_vector(0 downto 0);

signal iram1_re_s, iram2_re_s    : std_logic;

signal qram1_wea_s, qram2_wea_s  : std_logic_vector(0 downto 0);

signal qram1_re_s,  qram2_re_s   : std_logic;

signal ram1_rd_addr_s, ram2_rd_addr_s : std_logic_vector(fft_size_log_g-1 downto 0);

signal ram1_wr_addr_reg_s, ram2_wr_addr_reg_s : std_logic_vector(fft_size_log_g-1 downto 0);

signal iram1_addra_reg_s, iram2_addra_reg_s : std_logic_vector(fft_size_log_g-1 downto 0);

signal iram1_addra_reg1_s, iram2_addra_reg1_s : std_logic_vector(fft_size_log_g-1 downto 0);

signal qram1_addra_reg_s, qram2_addra_reg_s : std_logic_vector(fft_size_log_g-1 downto 0);

signal Y0_IM_S,Y0_RE_S: std_logic_vector(radix2_size1_g-1 downto 0);

signal Y1_IM_S,Y1_RE_S: std_logic_vector(radix2_size1_g-1 downto 0);

signal load_done_s : std_logic;

signal fft_rom_addr_s : std_logic_vector (fft_size_log_g-2 downto 0);

signal iram1_ena_reg_s, iram2_ena_reg_s : std_logic;

signal iram1_ena_s, iram2_ena_s : std_logic;

signal qram1_ena_reg_s, qram2_ena_reg_s : std_logic;

signal x0_re_s, x0_im_s : std_logic_vector(radix2_size1_g-1 downto 0);

signal x1_re_s, x1_im_s : std_logic_vector(radix2_size1_g-1 downto 0);

COMPONENT fft_bram1_2048

PORT (

clka : IN STD_LOGIC;

ena : IN STD_LOGIC;

wea : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

addra : IN STD_LOGIC_VECTOR(fft_size_log_g-1 DOWNTO 0);

dina : IN STD_LOGIC_VECTOR (radix2_size1_g-1 downto 0);

clkb : IN STD_LOGIC;

enb : IN STD_LOGIC;

addrb : IN STD_LOGIC_VECTOR (fft_size_log_g-1 DOWNTO 0);

doutb : OUT STD_LOGIC_VECTOR(radix2_size1_g-1 downto 0)

);

END COMPONENT;

-- Twiddle factor Memory  instantiaiton

COMPONENT twid_ram_2048pt_e

GENERIC( fft_size_log_g     : integer );

PORT(clk_is: in std_logic;

reset_is : in std_logic;

fft_rom_re_is : in std_logic;

twid_refresh_is : in std_logic;

fft_rom_addr_is : in std_logic_vector(fft_size_log_g-2 downto 0);

twiddle_re_os  : out std_logic_vector(15 downto 0);

twiddle_im_os  : out std_logic_vector(15 downto 0));

END COMPONENT;

signal fft_rom_re_s: std_logic;

signal twiddle_re_s, twiddle_im_s : std_logic_vector(15 downto 0);

signal unload_over_s: std_logic;

signal xk_index_s,xk_index1_s: std_logic_vector(fft_size_log_g-1 downto 0);

signal ram1_wea_s, ram2_wea_s : std_logic;

signal par_ram1_ra_s, par_ram2_ra_s: std_logic;

signal par_ram1_wa_s, par_ram2_wa_s: std_logic;

signal par_ram1_ra_reg1_s, par_ram2_ra_reg1_s: std_logic;

signal par_ram1_ra_reg2_s, par_ram2_ra_reg2_s: std_logic;

signal half_over_s: std_logic;

signal zeros_s  :std_logic_vector(trunc_g -1 downto 0);

signal unload_wdt_s : std_logic;

signal wdt_count_s  : std_logic_vector(4 downto 0);

signal wdt_occur_s: std_logic_vector(3 downto 0);

signal xk_count_en_s: std_logic;

signal start_sync_ff1_s  , start_sync_ff2_s : std_logic;

attribute FSM_ENCODING : string;

attribute FSM_ENCODING of fsm_state_s: signal is "ONE-HOT";

attribute SAFE_IMPLEMENTATION :string;

attribute SAFE_IMPLEMENTATION of fsm_state_s: signal is"yes";

begin

P_FSM_state : process(clk_is,reset_is )

begin

if reset_is = '1' then

fsm_state_s  <= s0; --POR State.

prev_state_s <= s0;

elsif clk_is'event and clk_is='1' then

prev_state_s<= fsm_state_s;

C_fsm_state_dec: case fsm_state_s is

when s0=>  if start_sync_ff2_s='1'  then

fsm_state_s<=s1;

end if;

when s1=>  if load_done_s='1'  then

fsm_state_s<=s2;

end if;

when s2=>  if stage_done_s='1' then

fsm_state_s<=s3;

end if;

when s3=>  if fft_done_s='1' then

fsm_state_s<=s4;

else

fsm_state_s<=s2;

end if;

when s4 =>  if unload_over_s ='1' or unload_wdt_s='1' then

fsm_state_s<=s5;

end if;

when s5=>   fsm_state_s<=s0;

when others=> fsm_state_s<=s0;

end case C_fsm_state_dec;

end if;

end process P_FSM_state;

group_count_en_s <= '1' when fsm_state_s=s2 else

'0';

stage_count_en_s <='1' when fsm_state_s=s3 and prev_state_s=s2 else

'0';

zeros_s       <= (others=>'0');

fft_status_os <= stage_count_s & wdt_occur_s;

P_fft_control_sig_gen: process(clk_is,reset_is)

begin

if reset_is = '1' then

group_count_s<=(others=>'0');

stage_count_s<=(others=>'0');

fft_done_s   <='0';

done_s       <= '0';

--edone_s      <= '0';

unload_over_s <='0';

wdt_count_s <= (others=>'0');

wdt_occur_s <= (others=>'0');

unload_wdt_s <='0';

start_sync_ff1_s  <='0';

start_sync_ff2_s  <='0';

elsif clk_is'event and clk_is='1' then

if fsm_state_s=s1 or fsm_state_s=s3 or fsm_state_s=s5 then

group_count_s<=(others=>'0');

elsif group_count_en_s='1' then

group_count_s<= group_count_s +1;

end if;

if fsm_state_s=s0 then

stage_count_s<=(others=>'0');

elsif stage_count_en_s='1' then

stage_count_s<= stage_count_s +1;

end if;

if stage_count_s >="1010" then -- 10 stages

fft_done_s<='1';

else

fft_done_s <='0';

end if;

if stage_count_s >="1010" and fsm_state_s=s3 then

done_s <='1';

elsif fsm_state_s=s4 and prev_state_s=s3 then

done_s<='1';

else

done_s<='0';

end if;

if xk_index_s ="11111111111" then --

unload_over_s <='1';

else

unload_over_s <='0';

end if;

if xk_count_en_s ='1' or fsm_state_s=s5 then

wdt_count_s <= (others=>'0');

elsif fsm_state_s=s4 then

wdt_count_s <= wdt_count_s +1;

end if;

if wdt_count_s ="11111" then --31

unload_wdt_s <='1';

else

unload_wdt_s <='0';

end if;

if unload_wdt_s='1' then

wdt_occur_s <= wdt_occur_s+1;

end if;

start_sync_ff1_s<=start_is;

start_sync_ff2_s<=start_sync_ff1_s;-- Double flop synchroiser for start pulse.

end if;

end process P_fft_control_sig_gen;

-- edone_os <= edone_s;

done_os  <= done_s;

load_done_s <='1' when fsm_state_s=s1 and iram1_addra_reg_s="11111111111" else --

'0';

stage_done_s<='1' when fsm_state_s=s2 and iram2_addra_reg_s="11111111111" else --

'1' when fsm_state_s=s2 and iram1_addra_reg_s="11111111111"else  --

'0';

--Radix-2 butterfly instantiation

butterfly_radix2: radix2_2048pt_e

GENERIC MAP(input_width_g      => input_width_g,

fft_size_log_g     => fft_size_log_g,

radix2_size1_g     => radix2_size1_g,

radix2_size2_g     => radix2_size2_g

)

PORT MAP(

clk_is => clk_is,

reset_is => reset_is,

x0_re_is => x0_re_s,

x0_im_is => x0_im_s,

x1_re_is => x1_re_s,

x1_im_is => x1_im_s,

twiddle_re_is => twiddle_re_s,

twiddle_im_is => twiddle_im_s,

Y0_RE_OS => Y0_RE_S,

Y0_IM_OS => Y0_IM_S,

Y1_RE_OS => Y1_RE_S,

Y1_IM_OS => Y1_IM_S

);

x0_re_s <= iram1_doutb_s when par_ram1_ra_reg1_s='1' else

iram2_doutb_s;

x0_im_s <= qram1_doutb_s when par_ram1_ra_reg1_s='1' else

qram2_doutb_s ;

x1_re_s <= iram1_doutb_s when par_ram2_ra_reg1_s='1' else

iram2_doutb_s ;

x1_im_s <= qram1_doutb_s when par_ram2_ra_reg1_s='1' else

qram2_doutb_s ;

address_gen: addr_gen_2048_e

GENERIC MAP (

fft_size_log_g => fft_size_log_g )

PORT MAP(

clk_is => clk_is ,

reset_is => reset_is ,

start_is => start_is ,

unload_is => unload_is ,

stage_count_is =>  stage_count_s,

group_count_is =>  group_count_s,

xn_index_os => xn_index_os,

xk_index_os => xk_index_s,

xk_index1_os => xk_index1_s,

xn_index1_os => xn_index1_s,

xk_count_en_os => xk_count_en_s,

fft_ram1_raddr_os => ram1_rd_addr_s,

fft_ram2_raddr_os => ram2_rd_addr_s,

fft_rom_addr_os => fft_rom_addr_s

);

xk_index_os<= xk_index_s;

-- Parity computation for RAM1 and RAM2 read address.

par_ram1_ra_s <= ram1_rd_addr_s(0) xor ram1_rd_addr_s(1) xor ram1_rd_addr_s(2) xor ram1_rd_addr_s(3) xor ram1_rd_addr_s(4) xor

ram1_rd_addr_s(5) xor ram1_rd_addr_s(6) xor ram1_rd_addr_s(7) xor ram1_rd_addr_s(8) xor ram1_rd_addr_s(9) xor ram1_rd_addr_s(10);

par_ram2_ra_s <= ram2_rd_addr_s(0) xor ram2_rd_addr_s(1) xor ram2_rd_addr_s(2) xor ram2_rd_addr_s(3) xor ram2_rd_addr_s(4) xor

ram2_rd_addr_s(5) xor ram2_rd_addr_s(6) xor ram2_rd_addr_s(7) xor ram2_rd_addr_s(8) xor ram2_rd_addr_s(9) xor ram2_rd_addr_s(10);

P_parity_gen: process(clk_is,reset_is)

begin

if reset_is='1' then

par_ram1_wa_s<='0';

par_ram2_wa_s<='0';

par_ram1_ra_reg1_s<='0';

par_ram2_ra_reg1_s<='0';

par_ram1_ra_reg2_s<='0';

par_ram2_ra_reg2_s<='0';

elsif clk_is'event and clk_is='1' then

par_ram1_ra_reg1_s <= par_ram1_ra_s;

par_ram2_ra_reg1_s <= par_ram2_ra_s;

par_ram1_ra_reg2_s <= par_ram1_ra_reg1_s;

par_ram2_ra_reg2_s <= par_ram2_ra_reg1_s;

par_ram1_wa_s      <= par_ram1_ra_reg2_s;

par_ram2_wa_s      <= par_ram2_ra_reg2_s;

end if;

end process P_parity_gen;

Twiddle_ram_2048pt: twid_ram_2048pt_e

GENERIC MAP (

fft_size_log_g => fft_size_log_g )

PORT MAP (

clk_is => clk_is,

reset_is => reset_is ,

fft_rom_re_is => fft_rom_re_s ,

twid_refresh_is => start_sync_ff2_s ,

fft_rom_addr_is => fft_rom_addr_s ,

twiddle_re_os => twiddle_re_s ,

twiddle_im_os => twiddle_im_s

);

fft_rom_re_s <= '1' when fsm_state_s = s2 else

'0';

P_ram_rd_wr_gen: process(clk_is,reset_is )

begin

if reset_is = '1' then

ram1_wr_addr_reg_s<=(others=>'0');

ram2_wr_addr_reg_s<=(others=>'0');

ram1_wea_s <='0';

ram2_wea_s <='0';

iram1_re_s <='0';

iram2_re_s <='0';

elsif clk_is'event and clk_is='1' then

if fsm_state_s=s2 and stage_count_s="1010" then

ram1_wr_addr_reg_s <= ram2_rd_addr_s;

ram2_wr_addr_reg_s <= ram1_rd_addr_s;

else

ram1_wr_addr_reg_s <= iram1_addrb_s;

ram2_wr_addr_reg_s <= iram2_addrb_s;

end if;

if fsm_state_s=s2 and group_count_s >="0000000011" then

ram1_wea_s <='1';

elsif  fsm_state_s=s3 then

ram1_wea_s <='0';

end if;

if fsm_state_s=s2 and group_count_s >="0000000011" then

ram2_wea_s <='1';

elsif  fsm_state_s=s3 then

ram2_wea_s <='0';

end if;

if fsm_state_s=s2 or fsm_state_s=s4  then

iram1_re_s <='1';

iram2_re_s <='1';

else

iram1_re_s <='0';

iram2_re_s <='0';

end if;

end if;

end process P_ram_rd_wr_gen;

-- RAM1 and RAM2 Data selection.

iram1_dina_s  <=  sxt(xn_re_is,input_width_g+fft_size_log_g+1)  & zeros_s  when fsm_state_s=s1 else

Y1_RE_S   when fsm_state_s=s2 and stage_count_s >="1010" else --

Y0_RE_S   when fsm_state_s=s2 and par_ram1_wa_s='1' else

Y1_RE_S   when fsm_state_s=s2 and par_ram2_wa_s='1' else

(others=>'0');

qram1_dina_s  <=  sxt(xn_im_is,input_width_g+fft_size_log_g+1) & zeros_s  when fsm_state_s=s1 else

Y1_IM_S   when fsm_state_s=s2 and stage_count_s >="1010" else --

Y0_IM_S   when fsm_state_s=s2 and par_ram1_wa_s='1' else

Y1_IM_S   when fsm_state_s=s2 and par_ram2_wa_s='1' else

(others=>'0');

iram2_dina_s  <=  sxt(xn_re_is,input_width_g+fft_size_log_g+1) & zeros_s  when fsm_state_s=s1 else

Y0_RE_S   when fsm_state_s=s2 and stage_count_s >="1010" else --

Y0_RE_S   when fsm_state_s=s2 and par_ram1_wa_s='0' else

Y1_RE_S   when fsm_state_s=s2 and par_ram2_wa_s='0' else

(others=>'0');

qram2_dina_s   <= sxt(xn_im_is,input_width_g+fft_size_log_g+1) & zeros_s  when fsm_state_s=s1  else

Y0_IM_S   when fsm_state_s=s2 and stage_count_s >="1010" else  --

Y0_IM_S   when fsm_state_s=s2 and par_ram1_wa_s='0' else

Y1_IM_S   when fsm_state_s=s2 and par_ram2_wa_s='0' else

(others=>'0');

-- RAM1 and RAM2 Address selection

ram1_addra_s <=  xn_index1_s when fsm_state_s=s1 else

ram1_wr_addr_reg_s when fsm_state_s=s2 else

(others=>'0');	iram1_addrb_s <= ram1_rd_addr_s   when  fsm_state_s=s2 and par_ram1_ra_s='1' else

ram2_rd_addr_s   when  fsm_state_s=s2 and par_ram2_ra_s='1' else

xk_index1_s       when  fsm_state_s=s4 else

(others=>'0');

qram1_addrb_s <= iram1_addrb_s;

ram2_addra_s <=  xn_index1_s when fsm_state_s=s1 else

ram2_wr_addr_reg_s when fsm_state_s=s2 else

(others=>'0');					 iram2_addrb_s <= ram1_rd_addr_s   when fsm_state_s=s2 and par_ram1_ra_s='0' else

ram2_rd_addr_s   when fsm_state_s=s2 and par_ram2_ra_s='0' else

xk_index1_s       when fsm_state_s=s4 else

(others=>'0');

qram2_addrb_s <= iram2_addrb_s;

P_ram_addra_gen: process(clk_is,reset_is )

begin

if reset_is = '1' then

iram1_addra_reg_s<=(others=>'0');

iram2_addra_reg_s<=(others=>'0');

iram1_addra_reg1_s<=(others=>'0');

iram2_addra_reg1_s<=(others=>'0');

elsif clk_is'event and clk_is='1' then

iram1_addra_reg1_s<=ram1_addra_s;

iram2_addra_reg1_s<=ram2_addra_s;

iram1_addra_reg_s<=iram1_addra_reg1_s;

iram2_addra_reg_s<=iram2_addra_reg1_s;

end if;

end process P_ram_addra_gen;

qram1_addra_reg_s<=iram1_addra_reg_s;

qram2_addra_reg_s<=iram2_addra_reg_s;

-- RAM1 and RAM2 Write enable selection.

iram1_wea_s(0) <= iram1_ena_s;

iram1_ena_s  <= '1' when fsm_state_s=s1  else

ram1_wea_s when fsm_state_s=s2   else  '0';

iram2_wea_s(0) <= iram2_ena_s;

iram2_ena_s  <= '1' when fsm_state_s=s1 else

ram2_wea_s when fsm_state_s=s2 else  '0';

qram1_wea_s(0) <= iram1_ena_s;

qram1_re_s     <=  iram1_re_s;

qram2_wea_s(0) <= iram2_ena_s;

qram2_re_s     <= iram2_re_s;

P_iram2_gen: process(clk_is,reset_is )

begin

if reset_is = '1' then

iram1_ena_reg_s <='0';

iram2_ena_reg_s <='0';

elsif clk_is'event and clk_is='1' then

iram1_ena_reg_s <= iram1_ena_s;

iram2_ena_reg_s <= iram2_ena_s;			end if;

end process P_iram2_gen;

qram1_ena_reg_s <= iram1_ena_reg_s;

qram2_ena_reg_s <= iram2_ena_reg_s;

--FFT memory Bank1 RAM instantiation 									fft_re_ram1 : fft_bram1_2048

PORT MAP (

clka   => clk_is,

ena     => iram1_ena_reg_s,

wea    => iram1_wea_s,

addra  => iram1_addra_reg_s,

dina   => iram1_dina_s,

clkb   => clk_is,

enb    => iram1_re_s,

addrb  => iram1_addrb_s,

doutb  => iram1_doutb_s

);

fft_im_ram1 : fft_bram1_2048

PORT MAP (

clka   => clk_is,

ena     => qram1_ena_reg_s,

wea    => qram1_wea_s,

addra  => qram1_addra_reg_s,

dina   => qram1_dina_s,

clkb   => clk_is,

enb    => qram1_re_s,

addrb  => qram1_addrb_s,

doutb  => qram1_doutb_s

);

fft_re_ram2 : fft_bram1_2048

PORT MAP (

clka   => clk_is,

ena    => iram2_ena_reg_s,

wea    => iram2_wea_s,

addra  => iram2_addra_reg_s,

dina   => iram2_dina_s,

clkb   => clk_is,

enb    => iram2_re_s,

addrb  => iram2_addrb_s,

doutb  => iram2_doutb_s

);

fft_im_ram2 : fft_bram1_2048

PORT MAP (

clka   => clk_is,

ena     => qram2_ena_reg_s,

wea    => qram2_wea_s,

addra  => qram2_addra_reg_s,

dina   => qram2_dina_s,

clkb   => clk_is,

enb    => qram2_re_s,

addrb  => qram2_addrb_s,

doutb  => qram2_doutb_s

);

-- Load the FFT computation results to output.

half_over_s <= '1' when fsm_state_s=s4 and xk_index_s(10 downto 0)>="01111111111" else --1023

'0' when fsm_state_s=s5 else

'0';

P_xk_gen: process(clk_is,reset_is )

begin

if reset_is = '1' then

xk_re_os <= (others=>'0'); -- Initialized to zero

xk_im_os <= (others=>'0'); -- Initialized to zero

elsif clk_is'event and clk_is='1' then

if half_over_s='0' then

xk_re_os<= iram2_doutb_s(radix2_size1_g-1 downto trunc_g );

xk_im_os<= qram2_doutb_s(radix2_size1_g-1 downto trunc_g );

else

xk_re_os<= iram1_doutb_s(radix2_size1_g-1 downto trunc_g ) ;

xk_im_os<= qram1_doutb_s(radix2_size1_g-1 downto trunc_g ) ;

end if;

end if;

end process P_xk_gen;

end fft_core_2048_a;

41) fft_top_cont_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity fft_top_cont_e is

generic(input_width_g          : integer  := 14;  -- FFT input width

fft_size_g             : integer  := 11;  -- 2048pt FFT

fft_output_width_g     : integer  := 26   -- input_width_g+fft_size_g+1 (11+11+1).

);

port(    --Inputs---

clk_40M_is     : in std_logic; --40MHz Clock

reset_is       : in std_logic; --Synchronised Reset

xn_re1_is      : in std_logic_vector(input_width_g-1 downto 0); -- Real input for module1

xn_re2_is      : in std_logic_vector(input_width_g-1 downto 0); -- Real input for module2

xn_im1_is      : in std_logic_vector(input_width_g-1 downto 0); -- Imag input for module1

xn_im2_is      : in std_logic_vector(input_width_g-1 downto 0); -- Imag input for module2

fft_start1_is   : in std_logic;

fft_start2_is   : in std_logic;

fft_unload1_is  : in std_logic;

fft_unload2_is  : in std_logic;

sel_is          : in std_logic;

-- Outputs --

fft_done1_os    : out  std_logic; -- FFT Done signal for Module1.

fft_done2_os    : out  std_logic; -- FFT Done signal for Module2.

--  fft_edone1_os    : out  std_logic; -- FFT eDone signal for Module1.

--  fft_edone2_os    : out  std_logic; -- FFT eDone signal for Module2.

fft_status_os    : out  std_logic_vector(7 downto 0);

xk_re1_os       : out  std_logic_vector(fft_output_width_g-1 downto 0); -- Real Output  for module1.

xk_re2_os       : out  std_logic_vector(fft_output_width_g-1 downto 0); -- Real Output  for module2.

xk_im1_os       : out  std_logic_vector(fft_output_width_g-1 downto 0); -- Imag Output  for module1.

xk_im2_os       : out  std_logic_vector(fft_output_width_g-1 downto 0); -- Imag Output  for module2.

xn_index1_os    : out  std_logic_vector(fft_size_g-1 downto 0); -- Module1 Xn index.

xn_index2_os    : out  std_logic_vector(fft_size_g-1 downto 0) -- Module2 Xn index.

--  xk_index1_os    : out  std_logic_vector(10 downto 0); -- Module1 Xk index.

--  xk_index2_os    : out  std_logic_vector(10 downto 0); -- Module2 Xk index.

);

end fft_top_cont_e;

architecture fft_top_cont_a of fft_top_cont_e is

-- FFT module component declaration.

COMPONENT fft_core_2048_e

PORT(

clk_is : IN std_logic;

reset_is : IN std_logic;

start_is : IN std_logic;

unload_is : IN std_logic;

xn_re_is : IN std_logic_vector(input_width_g-1 downto 0);

xn_im_is : IN std_logic_vector(input_width_g-1 downto 0);

xn_index_os : OUT std_logic_vector(fft_size_g-1 downto 0);

-- edone_os : OUT std_logic;

done_os : OUT std_logic;

fft_status_os: OUT std_logic_vector(7 downto 0);

xk_index_os : OUT std_logic_vector(fft_size_g-1 downto 0);

xk_re_os : OUT std_logic_vector(fft_output_width_g-1 downto 0);

xk_im_os : OUT std_logic_vector(fft_output_width_g-1 downto 0)

);

END COMPONENT;

signal start_fft_s,unload_sig_s : std_logic;

signal inphase_in_s,qphase_in_s: std_logic_vector(input_width_g-1 downto 0);

signal xn_index_fft_s, xk_index_fft_s: std_logic_vector(fft_size_g-1 downto 0);

signal Real_out_fft_s,Imag_out_fft_s : std_logic_vector(fft_output_width_g-1 downto 0);

signal done_s : std_logic;

signal start_fft_reg_s : std_logic;

signal fft_status_s: std_logic_vector(7 downto 0);

begin

fft_status_os <= fft_status_s;

fft_engine: fft_core_2048_e PORT MAP(

clk_is => clk_40M_is,

reset_is => reset_is,

start_is => start_fft_reg_s,

unload_is => unload_sig_s,

xn_re_is => inphase_in_s,

xn_im_is => qphase_in_s,

xn_index_os => xn_index_fft_s ,

--edone_os => edone_s ,

done_os => done_s,

fft_status_os => fft_status_s,

xk_index_os => xk_index_fft_s,

xk_re_os => Real_out_fft_s ,

xk_im_os => Imag_out_fft_s

);

inphase_in_s<= xn_re1_is when sel_is='0' else

xn_re2_is when sel_is='1';

qphase_in_s<=  xn_im1_is when sel_is='0' else

xn_im2_is when sel_is='1';

start_fft_s <= fft_start1_is when sel_is='0' else

fft_start2_is when sel_is='1' ;

unload_sig_s <= fft_unload1_is when sel_is='0' else

fft_unload2_is when sel_is='1' ;

xn_index1_os <= xn_index_fft_s when sel_is='0' else

(others=>'0');

xn_index2_os <= xn_index_fft_s when sel_is='1' else

(others=>'0');

xk_re1_os <= Real_out_fft_s when sel_is='0' else

(others=>'0');

xk_im1_os <= Imag_out_fft_s when sel_is='0' else

(others=>'0');

xk_re2_os <= Real_out_fft_s when sel_is='1' else

(others=>'0');

xk_im2_os <= Imag_out_fft_s when sel_is='1' else

(others=>'0');

fft_done1_os <= done_s when sel_is='0' else

'0';

fft_done2_os <= done_s when sel_is='1' else

'0';

-- fft_edone1_os <= edone_s when sel_is='0' else

-- '0';

-- fft_edone2_os <= edone_s when sel_is='1' else

-- '0';

Process_p1: process(clk_40M_is,reset_is )

begin

if reset_is = '1' then

start_fft_reg_s <= '0';

elsif clk_40M_is'event and clk_40M_is='1' then

start_fft_reg_s <= start_fft_s;

end if;

end process Process_p1;

end fft_top_cont_a;

42) GLOBALBUFF

library IEEE;

use IEEE.STD_LOGIC_1164.all;

entity GLOBALBUFF is

port ( A : in  std_logic;

Y : out std_logic

);

end GLOBALBUFF;

architecture RTL of GLOBALBUFF is

begin

Y <= A;

end RTL;

43) gnss_correlator_top_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_SIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

use WORK.signal_type_p.ALL;

entity gnss_correlator_top_e is

port(clk_is,POR_G_is,POR_in_is: in std_logic;

ant1_data_is : in std_logic_vector(5 downto 0);

ant2_data_is : in std_logic_vector(5 downto 0);

ant3_data_is : in std_logic_vector(5 downto 0);

ant4_data_is : in std_logic_vector(5 downto 0);

soft_reset_os: out std_logic;

rt_address_RT1_is: in std_logic_vector(5 downto 0);

rt_address_RT2_is: in std_logic_vector(5 downto 0);

rt_address_RT3_is: in std_logic_vector(5 downto 0);

hwdt_cmd_is, swdt_cmd_is: in std_logic;

data_processor_is : in std_logic_vector(31 downto 0);

wr_pr_is: in std_logic;

cs_correlator_is : in std_logic;

up_address_is: in std_logic_vector(11 downto 0);

ms_10_interrupt_os, ms_500_interrupt_os,reset_processor_os : out std_logic;

data_to_processor_os: out std_logic_vector(31 downto 0);		  reset_tsm_interrupt_RT1_os : out std_logic;		TSM_interrupt_RT1_is : in std_logic;

TSM_flag_out_RT1_os,PPS_flag_out_RT1_os : out std_logic;

reset_tsm_interrupt_RT2_os : out std_logic;			TSM_interrupt_RT2_is : in std_logic;		  TSM_flag_out_RT2_os,PPS_flag_out_RT2_os : out std_logic;

reset_tsm_interrupt_RT3_os : out std_logic;			 TSM_interrupt_RT3_is : in std_logic;	TSM_flag_out_RT3_os,PPS_flag_out_RT3_os : out std_logic;

reset_mil_interrupt_RT1_os: out std_logic;

reset_mil_interrupt_RT2_os: out std_logic;

reset_mil_interrupt_RT3_os: out std_logic;

mil_1553_interrupt_RT1_is: in std_logic;

mil_1553_interrupt_RT2_is: in std_logic;

mil_1553_interrupt_RT3_is: in std_logic;

upconv1_spi_reg_os, upconv2_spi_reg_os : out std_logic_vector(4 downto 0);

upconv1_spi_gpio4_is, upconv2_spi_gpio4_is : in std_logic;

lock_detector_ant1_is,lock_detector_ant2_is : in std_logic;

lock_detector_ant3_is,lock_detector_ant4_is : in std_logic;

pps_signal_os: out std_logic_vector(5 downto 0);

config_crc_err_is: in std_logic;

config_ecc_err_is: in std_logic;

config_synd_valid_is: in std_logic;

icap_busy_is : in std_logic;

syndrome_is: in std_logic_vector(11 downto 0);

synd_valid_count_is: in std_logic_vector(7 downto 0);

rt_adddr_sel_is : in std_logic_vector(1 downto 0) ;

uart_txd_is: in std_logic;

rwe3_is    : in std_logic;

wdt_l_is : in std_logic;

bexc_l_is: in std_logic;

spi_clk_SY_is: in std_logic;

spi_en_SY_os: OUT std_logic;

spi_mosi_SY_os: OUT std_logic;

spi_miso_SY_is: in std_logic );

end gnss_correlator_top_e;

architecture gnss_correlator_top_a of gnss_correlator_top_e is

COMPONENT trk1_e

PORT(clk_is: in std_logic; -- 40 MHz clock

POR_is : in std_logic; -- POR and BMU reset combined

start_is: in std_logic;  -- to start the channel comes from top fsm when a satellite is found and this channel is free

-- samp_clk_enable_is : in std_logic;  -- sampling clk signal

channel_par_load_is : in std_logic;	  -- comes from descriminator module to update the nco values and store the loop register values

sat_id_in_is: in std_logic_vector(5 downto 0); -- comes from acquisition module

ant1_ant2_sel_ch_is: in std_logic_vector(1 downto 0);  -- Antenna selection comes from Acquistion module

car_freq_init_is,code_freq_init_is : in std_logic_vector(31 downto 0); -- comes from acquistion module for initializing the NCOs

order1_loop_init_is, dll_order1_loop_init_is : in std_logic_vector(48 downto 0); -- comes from acquisition module for initializing the loops

indata_ant1_is,indata_ant2_is,indata_ant3_is,indata_ant4_is: in std_logic_vector(5 downto 0);  -- data from different antennas

car_mix_ROM_ena_is : IN STD_LOGIC;

car_mix_ROM_wea_is : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

car_mix_ROM_addra_is : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

car_mix_ROM_dina_is : IN STD_LOGIC_VECTOR(19 DOWNTO 0);

sat_id_out_os : out std_logic_vector(5 downto 0);  -- satellite id of this channel

p_i_os,p_q_os,e_i_os,e_q_os,l_i_os,l_q_os : out std_logic_vector(22 downto 0);  -- A& D outputs

lock_status_out_os : out std_logic; -- goes to descriminator modulefor the top lock detector module..

lock_status_in_is: in std_logic; -- comes from descriminator module  for individual storage

loop_start_os: out std_logic;  -- mentioning A&D are ready and descriminator has to work and update

stage_control_os : out std_logic_vector(4 downto 0);  -- fll_start signal, when FLL descriminator to be operated

FLL_data_check_counter_is : in std_logic_vector(31 downto 0);

fll_zero_os : out std_logic;

NAVIC_indicator_os : out std_logic;

Data_boundary_output_os : out std_logic;

fll_data_check_os : out std_logic; -- fll data check for +/- 0.25 rad jump.

-- input to the tracking channels regarding the previous Loop filter values

Prev_phase_error_input_is : in std_logic_vector(16 downto 0);				Order1_input_is,Order2_input_is,Order3_input_is : in std_logic_vector(48 downto 0);

DLL_Order1_input_is,DLL_Order2_input_is : in std_logic_vector(48 downto 0);

-- output to the tracking channels regarding the previous Loop filter values

Prev_phase_error_os : out std_logic_vector(16 downto 0);			Order1_out_os,Order2_out_os,Order3_out_os : out std_logic_vector(48 downto 0);

DLL_Order1_out_os,DLL_Order2_out_os : out std_logic_vector(48 downto 0);

car_corr_freq_in_is: in std_logic_vector(31 downto 0); -- comes from descriminator module after loop filter, the required NCO value.

code_corr_freq_in_is: in std_logic_vector(31 downto 0);  ----comes from descriminator module after loop filter, the required NCO value.

navdata_read_mp_is : in std_logic; -- new data bit is resetted, comes from processor

navdata_reg_os : out std_logic_vector(7 downto 0);  -- register 0 contains navdata, new bit etc..

p_cndr_out_os,q_cndr_out_os : out std_logic_vector(27 downto 0); -- this channel p and q cndr register to compute cndr by processor

car_cycle_count_os: out std_logic_vector(31 downto 0); -- at meas epoch latched

car_phase_count_os : out std_logic_vector(31 downto 0); -- at  meas epoch latched

one_ms_counter_value_os: out std_logic_vector(4 downto 0); -- at meas  epoch latched

twenty_ms_counter_value_os: out std_logic_vector(9 downto 0); -- at meas  epoch latched

code_cyle_count_value_os: out std_logic_vector(9 downto 0); -- at meas  epoch latched

code_phase_count_os : out std_logic_vector(31 downto 0); -- at meas  epoch latched

reg_wr_processor_tms_is : in std_logic; -- to clear twenty ms counter by processor

epoch_latch_pulse_is: in std_logic;	-- epoch pulse to latch the measurements

set1_to_set2_timer_Launch_is : in std_logic_vector(31 downto 0);

set2_to_set3_timer_Launch_is : in std_logic_vector(31 downto 0);

set3_to_set4_timer_Launch_is : in std_logic_vector(31 downto 0);

set4_to_set5_timer_Launch_is : in std_logic_vector(31 downto 0);

Narrow_corr_step1_timer_is : in std_logic_vector(31 downto 0);

Narrow_corr_step2_timer_is : in std_logic_vector(31 downto 0);

Narrow_correlator_step1_enable_is: in std_logic;

Narrow_correlator_step2_enable_is: in std_logic;

fll_timer_is :  in std_logic_vector(31 downto 0);

antenna_combining_control_is : in std_logic_vector(3 downto 0);

lock_counter_threshold1_is : in std_logic_vector(9 downto 0);	lock_counter_threshold2_is : in std_logic_vector(9 downto 0);	reset_channel_en_pr_is : in std_logic -- force reset the channel by processor

);

END COMPONENT trk1_e;

COMPONENT trk2_e

PORT(clk_is: in std_logic; -- 40 MHz clock

POR_is : in std_logic; -- POR and BMU reset combined

start_is: in std_logic;  -- to start the channel comes from top fsm when a satellite is found and this channel is free

--samp_clk_enable_is : in std_logic;  -- sampling clk signal

channel_par_load_is : in std_logic;	  -- comes from descriminator module to update the nco values and store the loop register values

sat_id_in_is: in std_logic_vector(5 downto 0); -- comes from acquisition module

ant1_ant2_sel_ch_is: in std_logic_vector(1 downto 0);  -- Antenna selection comes from Acquistion module

car_freq_init_is,code_freq_init_is : in std_logic_vector(31 downto 0); -- comes from acquistion module for initializing the NCOs

order1_loop_init_is, dll_order1_loop_init_is : in std_logic_vector(48 downto 0); -- comes from acquisition module for initializing the loops

indata_ant1_is,indata_ant2_is,indata_ant3_is,indata_ant4_is: in std_logic_vector(5 downto 0);  -- data from different antennas

car_mix_ROM_ena_is : IN STD_LOGIC;

car_mix_ROM_wea_is : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

car_mix_ROM_addra_is : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

car_mix_ROM_dina_is : IN STD_LOGIC_VECTOR(19 DOWNTO 0);

sat_id_out_os : out std_logic_vector(5 downto 0);  -- satellite id of this channel

p_i_os,p_q_os,e_i_os,e_q_os,l_i_os,l_q_os : out std_logic_vector(22 downto 0);  -- A& D outputs

lock_status_out_os : out std_logic; -- goes to descriminator modulefor the top lock detector module..

lock_status_in_is: in std_logic; -- comes from descriminator module  for individual storage

loop_start_os: out std_logic;  -- mentioning A&D are ready and descriminator has to work and update

stage_control_os : out std_logic_vector(4 downto 0);  -- fll_start signal, when FLL descriminator to be operated

FLL_data_check_counter_is : in std_logic_vector(31 downto 0);

fll_zero_os : out std_logic;

NAVIC_indicator_os : out std_logic;

Data_boundary_output_os : out std_logic;

fll_data_check_os : out std_logic; -- fll data check for +/- 0.25 rad jump.

-- input to the tracking channels regarding the previous Loop filter values

Prev_phase_error_input_is : in std_logic_vector(16 downto 0);Order1_input_is,Order2_input_is,Order3_input_is : in std_logic_vector(48 downto 0);

DLL_Order1_input_is,DLL_Order2_input_is : in std_logic_vector(48 downto 0);

-- output to the tracking channels regarding the previous Loop filter values

Prev_phase_error_os : out std_logic_vector(16 downto 0);		Order1_out_os,Order2_out_os,Order3_out_os : out std_logic_vector(48 downto 0);

DLL_Order1_out_os,DLL_Order2_out_os : out std_logic_vector(48 downto 0);

car_corr_freq_in_is: in std_logic_vector(31 downto 0); -- comes from descriminator module after loop filter, the required NCO value.

code_corr_freq_in_is: in std_logic_vector(31 downto 0);  ----comes from descriminator module after loop filter, the required NCO value.

navdata_read_mp_is : in std_logic; -- new data bit is resetted, comes from processor

navdata_reg_os : out std_logic_vector(7 downto 0);  -- register 0 contains navdata, new bit etc..

p_cndr_out_os,q_cndr_out_os : out std_logic_vector(27 downto 0); -- this channel p and q cndr register to compute cndr by processor

car_cycle_count_os: out std_logic_vector(31 downto 0); -- at meas epoch latched

car_phase_count_os : out std_logic_vector(31 downto 0); -- at  meas epoch latched

one_ms_counter_value_os: out std_logic_vector(4 downto 0); -- at meas  epoch latched

twenty_ms_counter_value_os: out std_logic_vector(9 downto 0); -- at meas  epoch latched

code_cyle_count_value_os: out std_logic_vector(9 downto 0); -- at meas  epoch latched

code_phase_count_os : out std_logic_vector(31 downto 0); -- at meas  epoch latched

reg_wr_processor_tms_is : in std_logic; -- to clear twenty ms counter by processor

epoch_latch_pulse_is: in std_logic;	-- epoch pulse to latch the measurements

set1_to_set2_timer_Launch_is : in std_logic_vector(31 downto 0);

set2_to_set3_timer_Launch_is : in std_logic_vector(31 downto 0);

set3_to_set4_timer_Launch_is : in std_logic_vector(31 downto 0);

set4_to_set5_timer_Launch_is : in std_logic_vector(31 downto 0);

Narrow_corr_step1_timer_is : in std_logic_vector(31 downto 0);

Narrow_corr_step2_timer_is : in std_logic_vector(31 downto 0);

Narrow_correlator_step1_enable_is: in std_logic;

Narrow_correlator_step2_enable_is: in std_logic;

fll_timer_is :  in std_logic_vector(31 downto 0);

antenna_combining_control_is : in std_logic_vector(3 downto 0);

reset_channel_en_pr_is : in std_logic; -- force reset the channel by processor

lock_counter_threshold1_is : in std_logic_vector(9 downto 0);

lock_counter_threshold2_is : in std_logic_vector(9 downto 0);

-- these are for channel 18 only

sbas_enable_is : in std_logic; -- comes from processor if this channel needs to be tracked for SBAS

sbas_navdata_os : out std_logic_vector(5 downto 0) -- This channel SBAS data

);

END COMPONENT trk2_e;

component top_discriminator_e

port(	clk_is,POR_is: in std_logic;

p_i_cndr_is: in twentyeight_ch1_18_typ;  -- promt inphase for cndr

p_q_cndr_is: in twentyeight_ch1_18_typ;  -- promt inphase for cndr

p_i_is: in twentythree_ch1_18_typ;

p_q_is: in twentythree_ch1_18_typ;

e_i_is: in twentythree_ch1_18_typ;

e_q_is: in twentythree_ch1_18_typ;

l_i_is: in twentythree_ch1_18_typ;

l_q_is: in twentythree_ch1_18_typ;		lock_counter_enable_os : out std_logic_vector(channels_count_c downto 0);

channel_par_load_output_os: out std_logic_vector(channels_count_c downto 0);

car_corr_freq_word_out_os:  out std_logic_vector(31 downto 0);--thirtytwo_ch1_12_typ;

code_corr_freq_word_out_os : out std_logic_vector(31 downto 0);--thirtytwo_ch1_12_typ;

Prev_phase_error_input_is : in seventeen_ch1_18_typ;				Order1_input_is,Order2_input_is,Order3_input_is : in fortynine_ch1_18_typ;

DLL_Order1_input_is,DLL_Order2_input_is : in  fortynine_ch1_18_typ;

Prev_phase_error_output_os : out std_logic_vector(16 downto 0);--seventeen_ch1_12_typ;

Order1_output_os,Order2_output_os,Order3_output_os : out  std_logic_vector(48 downto 0);--fortynine_ch1_12_typ;

DLL_Order1_output_os,DLL_Order2_output_os : out std_logic_vector(48 downto 0);--out  fortynine_ch1_12_typ;

loop_start_input_is : in std_logic_vector(channels_count_c downto 0);

stage_control_is : in five_ch1_18_typ;

fll_zero_is : in std_logic_vector(channels_count_c downto 0);

NAVIC_indicator_is :  in std_logic_vector(channels_count_c downto 0);

Data_boundary_input_is  :  in std_logic_vector(channels_count_c downto 0);

fll_data_check_input_is : in std_logic_vector(channels_count_c downto 0);

--DLL_3rd_order_1st_order_is : in std_logic;

track_th_sel_is : in std_logic;

up_data_is : in std_logic_vector(31 downto 0);

pll_par1_set1_launch_enable_is : IN std_logic;

pll_par2_set1_launch_enable_is : IN std_logic;

pll_par3_set1_launch_enable_is : IN std_logic;

pll_par4_set1_launch_enable_is : IN std_logic;

-- FLL, Set1, Launch phase, enable registers

fll_par1_set1_launch_enable_is : IN std_logic;

fll_par2_set1_launch_enable_is : IN std_logic;

fll_par3_set1_launch_enable_is : IN std_logic;

-- DLL, Set1, Launch phase, enable registers

dll_par1_set1_launch_enable_is : IN std_logic;

dll_par2_set1_launch_enable_is : IN std_logic;

dll_par3_set1_launch_enable_is : IN std_logic;

-- PLL, Set2, Launch phase, enable registers

pll_par1_set2_launch_enable_is : IN std_logic;

pll_par2_set2_launch_enable_is : IN std_logic;

pll_par3_set2_launch_enable_is : IN std_logic;

pll_par4_set2_launch_enable_is : IN std_logic;

-- FLL, Set2, Launch phase, enable registers

fll_par1_set2_launch_enable_is : IN std_logic;

fll_par2_set2_launch_enable_is : IN std_logic;

fll_par3_set2_launch_enable_is : IN std_logic;

-- DLL, Set2, Launch phase, enable registers

dll_par1_set2_launch_enable_is : IN std_logic;

dll_par2_set2_launch_enable_is : IN std_logic;

dll_par3_set2_launch_enable_is : IN std_logic;

-- PLL, Set3, Launch phase, enable registers

pll_par1_set3_launch_enable_is : IN std_logic;

pll_par2_set3_launch_enable_is : IN std_logic;

pll_par3_set3_launch_enable_is : IN std_logic;

pll_par4_set3_launch_enable_is : IN std_logic;

-- FLL, Set3, Launch phase, enable registers

fll_par1_set3_launch_enable_is : IN std_logic;

fll_par2_set3_launch_enable_is : IN std_logic;

fll_par3_set3_launch_enable_is : IN std_logic;

-- DLL, Set3, Launch phase, enable registers

dll_par1_set3_launch_enable_is : IN std_logic;

dll_par2_set3_launch_enable_is : IN std_logic;

dll_par3_set3_launch_enable_is : IN std_logic;

-- PLL, Set4, Launch phase, enable registers

pll_par1_set4_launch_enable_is : IN std_logic;

pll_par2_set4_launch_enable_is : IN std_logic;

pll_par3_set4_launch_enable_is : IN std_logic;

pll_par4_set4_launch_enable_is : IN std_logic;

-- FLL, Set4, Launch phase, enable registers

fll_par1_set4_launch_enable_is : IN std_logic;

fll_par2_set4_launch_enable_is : IN std_logic;

fll_par3_set4_launch_enable_is : IN std_logic;

-- DLL, Set4, Launch phase, enable registers

dll_par1_set4_launch_enable_is : IN std_logic;

dll_par2_set4_launch_enable_is : IN std_logic;

dll_par3_set4_launch_enable_is : IN std_logic;

cdll_par1_set1_launch_enable_is: IN std_logic;

-- PLL, Set5, Launch phase, enable registers

pll_par1_set5_launch_enable_is : IN std_logic;

pll_par2_set5_launch_enable_is : IN std_logic;

pll_par3_set5_launch_enable_is : IN std_logic;

pll_par4_set5_launch_enable_is : IN std_logic;

-- FLL, Set5, Launch phase, enable registers

fll_par1_set5_launch_enable_is : IN std_logic;

fll_par2_set5_launch_enable_is : IN std_logic;

fll_par3_set5_launch_enable_is : IN std_logic;

-- DLL, Set5, Launch phase, enable registers

dll_par1_set5_launch_enable_is : IN std_logic;

dll_par2_set5_launch_enable_is : IN std_logic;

dll_par3_set5_launch_enable_is : IN std_logic;

carr_aided_loop_sel_is         : IN std_logic;

fll_zero_en_dis_sel_is         : IN std_logic;

loop_par_status1_enable_is      : IN std_logic;

IF_FREQ_TRK_GPS_sel_is        : IN std_logic;

IF_FREQ_TRK_NAVIC_sel_is        : IN std_logic;

trk_threshold_os                : OUT std_logic_vector(31 downto 0);

carr_aided_loop_os             : out std_logic ;

fll_zero_en_dis_os             : out std_logic

);

end component top_discriminator_e;

component Top_acquisition_e is

port (Sys_CLK_40M_is : in  STD_LOGIC;

POR_RST_is : in  STD_LOGIC;

Incoming_data_ant1_is : in std_logic_vector(5 downto 0);

Incoming_data_ant2_is : in std_logic_vector(5 downto 0);

Incoming_data_ant3_is : in std_logic_vector(5 downto 0);

Incoming_data_ant4_is : in std_logic_vector(5 downto 0);

IF_Freq_GPS_sel_is : in std_logic;

IF_Freq_NAVIC_sel_is : in std_logic;

car_mix_ROM_ena_is : IN STD_LOGIC;

car_mix_ROM_wea_is : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

car_mix_ROM_addra_is : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

car_mix_ROM_dina_is : IN STD_LOGIC_VECTOR(19 DOWNTO 0);

FPGA_Tracking_register1_is : in std_logic_vector(31 downto 0);

FPGA_Tracking_register2_is : in std_logic_vector(31 downto 0);

data_processor_is : in std_logic_vector(31 downto 0);

Acquisition_threshold_LSW_sel_is : in std_logic;

Acquisition_threshold_MSW_sel_is : in std_logic;

alm_ant1_gps_reg_is, alm_ant2_gps_reg_is, alm_ant3_gps_reg_is, alm_ant4_gps_reg_is : in std_logic_vector(acq_modules_count_c downto 0);

alm_ant1_nav_reg_is, alm_ant2_nav_reg_is, alm_ant3_nav_reg_is, alm_ant4_nav_reg_is : in std_logic_vector(acq_modules_count_c downto 0);

alm_ant1_sbas_reg_is, alm_ant2_sbas_reg_is, alm_ant3_sbas_reg_is, alm_ant4_sbas_reg_is : in std_logic_vector(acq_modules_count_c downto 0);

SVID_loading_is : in std_logic_vector(acq_modules_count_c downto 0);

SVID_acquired_os : out six_acq1_4_typ;

SVID_Searched_os : out six_acq1_4_typ;

Antenna_output_os : out two_acq1_4_typ;

Track_epoch_output_pulse_os : out std_logic_vector(acq_modules_count_c downto 0);

car_freq_init_os , code_freq_init_os: out thirtytwo_acq1_4_typ;

order1_loop_init_os, dll_order1_loop_init_os : out fortynine_acq1_4_typ;

acq_threshold_LSW_os : out std_logic_vector(31 downto 0);

acq_threshold_MSW_os : out std_logic_vector(31 downto 0);

fft_status_os : out std_logic_vector(31 downto 0);

acq1_search_reg_os   : out std_logic_vector(63 downto 0);

acq2_search_reg_os   : out std_logic_vector(63 downto 0);

acq3_search_reg_os   : out std_logic_vector(63 downto 0);

acq4_search_reg_os   : out std_logic_vector(63 downto 0)

);

end component Top_acquisition_e;

component ms_500_module_e is

port(clk_is,POR_G_is: in std_logic;

--    clk_enable_is : in std_logic;

data_processor_is: in std_logic_vector(31 downto 0);

TIC_intrpt_clear_is: in std_logic;

TIC_wr_enable_is : in std_logic;

clear_TIC_counter_is: in std_logic;

TIC_drift_wr_enable_is: in std_logic;

epoch_latch_pulse_os : out std_logic;

ms_500_interrupt_os: out std_logic;

--RT1

TSM_interrupt_RT1_is : in std_logic;

reset_pps_interrupt_RT1_is : in std_logic;

TSM_flag_RT1_out_os,PPS_flag_RT1_out_os : out std_logic;

TSM_data_RT1_os : out std_logic_vector(31 downto 0);

--RT2

TSM_interrupt_RT2_is : in std_logic;

reset_pps_interrupt_RT2_is : in std_logic;

TSM_flag_RT2_out_os,PPS_flag_RT2_out_os : out std_logic;

TSM_data_RT2_os : out std_logic_vector(31 downto 0);

--RT3

TSM_interrupt_RT3_is : in std_logic;

reset_pps_interrupt_RT3_is : in std_logic;

TSM_flag_RT3_out_os,PPS_flag_RT3_out_os : out std_logic;

TSM_data_RT3_os : out std_logic_vector(31 downto 0); pps_pulse_count_data_en_is: in std_logic;

pps_on_off_is: in std_logic;

pps_signal_os: out std_logic_vector(5 downto 0));

end component ms_500_module_e;

component wdt_logic_e is

port(clk_is,POR_G_is, POR_in_is: in std_logic;

hwdt_cmd_is, swdt_cmd_is,hwdt_reset_clear_pr_wr_enable_is: in std_logic;

up_data_is: in std_logic;

wdt_enable_pr_wr_enable_is, clear_wdt_pr_is: in std_logic;

reset_processor_os: out std_logic;

wdt_cmd_status_os: out std_logic_vector(1 downto 0);

hwdt_reset_status_ff_os : out std_logic;

hwdt_reset_counter_os: out std_logic_vector(7 downto 0);

PR_to_FPGA_reset_reg_sel_is: in std_logic;

soft_reset_os: out std_logic;

soft_reset_flag_os : out std_logic);

end component wdt_logic_e;

component total_ch_fsm_e is

port(clk_is,POR_is: in std_logic;

channel_lock_status_is: in std_logic_vector(channels_count_c downto 0); acq_interrupt_is  : in std_logic_vector(acq_modules_count_c downto 0);

ant1_ant2_sel_is : in two_acq1_4_typ;

svid_acq_is : in six_acq1_4_typ;

car_freq_init_is,code_freq_init_is : in thirtytwo_acq1_4_typ;

order1_loop_init_is, dll_order1_loop_init_is : in fortynine_acq1_4_typ;

svid_channel_is: in six_ch1_18_typ; 			 svid_channel_load_os: out std_logic_vector(5 downto 0);

car_freq_init_os,code_freq_init_os: out std_logic_vector(31 downto 0);

order1_loop_init_os, dll_order1_loop_init_os : out std_logic_vector(48 downto 0);

ant1_ant2_sel_ch_os: out std_logic_vector(1 downto 0);

channel_start_os : out std_logic_vector(channels_count_c downto 0);

acq_start_os : out std_logic_vector(acq_modules_count_c downto 0);

channel_status_os: out std_logic_vector(channels_count_c downto 0);

sat_current_status_os: out std_logic_vector(63 downto 0);

sbas_enable_is: in std_logic;

navic_cmd_enable_is : in std_logic_vector(13 downto 0)

);

end component total_ch_fsm_e;

component corr_reg_address_decoder_e is

PORT(	wr_pr_is : IN std_logic;

cs_correlator_is : IN std_logic;

up_address_is : IN std_logic_vector(11 downto 0);

upconv1_spi_reg_wr_sel_os : out std_logic;

upconv2_spi_reg_wr_sel_os : out std_logic;

PR_to_FPGA_reset_reg_sel_os : OUT std_logic;

sbas_enable_reg_wr_sel_os : OUT std_logic;

navic_cmd_enable_reg_wr_sel_os : OUT std_logic;

wdt_enable_pr_wr_enable_os : OUT std_logic;

clear_wdt_pr_os : OUT std_logic;

reset_tsm_interrupt_RT1_os : OUT std_logic;

reset_tsm_interrupt_RT2_os : OUT std_logic;

reset_tsm_interrupt_RT3_os : OUT std_logic;

reset_pps_interrupt_RT1_os : OUT std_logic;

reset_pps_interrupt_RT2_os : OUT std_logic;

reset_pps_interrupt_RT3_os : OUT std_logic;

track_th_sel_os : OUT std_logic;

hwdt_reset_clear_pr_wr_enable_os : OUT std_logic;

ten_ms_reg_os: out std_logic;

TIC_intrpt_clear_os: out std_logic;

TIC_wr_enable_os: out std_logic;

clear_TIC_counter_os: out std_logic;

TIC_drift_wr_enable_os: out std_logic;

ant1_GPS_wr_enable_os : OUT std_logic_vector(acq_modules_count_c downto 0);

ant1_NavIC_wr_enable_os : OUT std_logic_vector(acq_modules_count_c downto 0);

ant1_SBAS_wr_enable_os : OUT std_logic_vector(acq_modules_count_c downto 0);

ant2_GPS_wr_enable_os : OUT std_logic_vector(acq_modules_count_c downto 0);

ant2_NavIC_wr_enable_os : OUT std_logic_vector(acq_modules_count_c downto 0);

ant2_SBAS_wr_enable_os : OUT std_logic_vector(acq_modules_count_c downto 0);

ant3_GPS_wr_enable_os : OUT std_logic_vector(acq_modules_count_c downto 0);

ant3_NavIC_wr_enable_os : OUT std_logic_vector(acq_modules_count_c downto 0);

ant3_SBAS_wr_enable_os : OUT std_logic_vector(acq_modules_count_c downto 0);

ant4_GPS_wr_enable_os : OUT std_logic_vector(acq_modules_count_c downto 0);

ant4_NavIC_wr_enable_os : OUT std_logic_vector(acq_modules_count_c downto 0);

ant4_SBAS_wr_enable_os : OUT std_logic_vector(acq_modules_count_c downto 0);

reset_channel_en_os : OUT std_logic_vector(channels_count_c downto 0);

nav_data_read_over_ch_os : OUT std_logic_vector(channels_count_c downto 0);

reg_wr_procr_tms_ch_os : OUT std_logic_vector(channels_count_c downto 0);

comparator_th_lsw_sel_os : OUT std_logic;

comparator_th_msw_sel_os : OUT std_logic;

pps_pulse_count_data_en_os : OUT std_logic;

pps_on_off_sel_os : OUT std_logic;

pll_par1_set1_launch_enable_os : OUT std_logic;

pll_par2_set1_launch_enable_os : OUT std_logic;

pll_par3_set1_launch_enable_os : OUT std_logic;

pll_par4_set1_launch_enable_os : OUT std_logic;

fll_par1_set1_launch_enable_os : OUT std_logic;

fll_par2_set1_launch_enable_os : OUT std_logic;

fll_par3_set1_launch_enable_os : OUT std_logic;

dll_par1_set1_launch_enable_os : OUT std_logic;

dll_par2_set1_launch_enable_os : OUT std_logic;

dll_par3_set1_launch_enable_os : OUT std_logic;

pll_par1_set2_launch_enable_os : OUT std_logic;

pll_par2_set2_launch_enable_os : OUT std_logic;

pll_par3_set2_launch_enable_os : OUT std_logic;

pll_par4_set2_launch_enable_os : OUT std_logic;

fll_par1_set2_launch_enable_os : OUT std_logic;

fll_par2_set2_launch_enable_os : OUT std_logic;

fll_par3_set2_launch_enable_os : OUT std_logic;

dll_par1_set2_launch_enable_os : OUT std_logic;

dll_par2_set2_launch_enable_os : OUT std_logic;

dll_par3_set2_launch_enable_os : OUT std_logic;

pll_par1_set3_launch_enable_os : OUT std_logic;

pll_par2_set3_launch_enable_os : OUT std_logic;

pll_par3_set3_launch_enable_os : OUT std_logic;

pll_par4_set3_launch_enable_os : OUT std_logic;

fll_par1_set3_launch_enable_os : OUT std_logic;

fll_par2_set3_launch_enable_os : OUT std_logic;

fll_par3_set3_launch_enable_os : OUT std_logic;

dll_par1_set3_launch_enable_os : OUT std_logic;

dll_par2_set3_launch_enable_os : OUT std_logic;

dll_par3_set3_launch_enable_os : OUT std_logic;

pll_par1_set4_launch_enable_os : OUT std_logic;

pll_par2_set4_launch_enable_os : OUT std_logic;

pll_par3_set4_launch_enable_os : OUT std_logic;

pll_par4_set4_launch_enable_os : OUT std_logic;

fll_par1_set4_launch_enable_os : OUT std_logic;

fll_par2_set4_launch_enable_os : OUT std_logic;

fll_par3_set4_launch_enable_os : OUT std_logic;

dll_par1_set4_launch_enable_os : OUT std_logic;

dll_par2_set4_launch_enable_os : OUT std_logic;

dll_par3_set4_launch_enable_os : OUT std_logic;

pll_par1_set5_launch_enable_os: out std_logic ;

pll_par2_set5_launch_enable_os: out std_logic ;

pll_par3_set5_launch_enable_os: out std_logic ;

pll_par4_set5_launch_enable_os: out std_logic ;

fll_par1_set5_launch_enable_os: out std_logic ;

fll_par2_set5_launch_enable_os: out std_logic ;

fll_par3_set5_launch_enable_os: out std_logic ;

dll_par1_set5_launch_enable_os: out std_logic ;

dll_par2_set5_launch_enable_os: out std_logic ;

dll_par3_set5_launch_enable_os: out std_logic ;

cdll_par1_set1_launch_enable_os : OUT std_logic;

launch_orbit_phase_sel_os : OUT std_logic;

carr_aided_loop_sel_os : OUT std_logic;

set1_to_set2_timer_launch_enable_os   : out std_logic;

set2_to_set3_timer_launch_enable_os   : out std_logic;

set3_to_set4_timer_launch_enable_os   : out std_logic;

set4_to_set5_timer_launch_enable_os   : out std_logic;

Narrow_corr_step1_timer_sel_os  : out  std_logic;

Narrow_corr_step2_timer_sel_os   : out  std_logic;

Narrow_correlator_step1_enable_sel_os  : out   std_logic;

Narrow_correlator_step2_enable_sel_os  : out  std_logic;

fll_timer_enable_os : OUT std_logic;

fll_zero_en_dis_sel_os : OUT std_logic;

loop_par_status1_enable_os : OUT std_logic;

All_Timers_updated_enable_os   : out std_logic;

Antenna_combining_sel_enable_os : out std_logic;

lock_detector_threshold_sel_os : out std_logic;

FLL_data_check_counter_enable_os : out std_logic ;

reset_mil_interrupt_RT1_os: out std_logic;

reset_mil_interrupt_RT2_os: out std_logic;

reset_mil_interrupt_RT3_os: out std_logic;

IF_FREQ_GPS_acq_sel_os: out std_logic;

IF_FREQ_NAVIC_acq_sel_os: out std_logic;

IF_FREQ_GPS_trk_sel_os: out std_logic;

IF_FREQ_NAVIC_trk_sel_os: out std_logic;

-------------saphyrion spi reg--------

reprogram_SY_cmd_sel_os  : out std_logic;

SY17_PLL_val_sel_os  : out std_logic;

SY17_AGC_val_sel_os  : out std_logic;

SYFE_PM_val_sel_os  : out std_logic;

SYFE_RFPLL_val_sel_os  : out std_logic;

SYFE_IFPLL_val_sel_os  : out std_logic

);

END COMPONENT corr_reg_address_decoder_e;

component corr_data_out_mux_e is

port(svid_acq_is: in six_acq1_4_typ;

nav_data_reg_ch_is : in eight_ch1_18_typ;

sat_id_is : in six_ch1_18_typ;

code_cyle_count_value_ch_is: in ten_ch1_18_typ;

one_ms_counter_value_ch_is : in five_ch1_18_typ;

twenty_ms_counter_value_ch_is: in ten_ch1_18_typ;

code_phase_count_ch_is : in thirtytwo_ch1_18_typ;

car_cycle_count_ch_is : in thirtytwo_ch1_18_typ;

car_phase_count_ch_is : thirtytwo_ch1_18_typ;

p_cndr_is,q_cndr_is : in twentyeight_ch1_18_typ;

pll_order1_is, pll_order2_is, pll_order3_is: in thirtytwo_ch1_18_typ;

sbas_navdata_is : in std_logic_vector(5 downto 0);

rt_address_RT1_is,rt_address_RT2_is,rt_address_RT3_is : in std_logic_vector(5 downto 0);

lock_detector_ant1_is,lock_detector_ant2_is : in std_logic;

lock_detector_ant3_is,lock_detector_ant4_is : in std_logic;

wdt_cmd_status_is : in std_logic_vector(1 downto 0);

up_address_is : in std_logic_vector(11 downto 0);

hdwt_reset_counter_is : in std_logic_vector(7 downto 0);

hwdt_reset_status_ff_is: in std_logic;	 channel_status_is : in std_logic_vector(channels_count_c downto 0);

sat_status_is : in std_logic_vector(63 downto 0);

TSM_data_RT1_is, TSM_data_RT2_is ,TSM_data_RT3_is : in std_logic_vector(31 downto 0);

soft_reset_flag_is : in std_logic;

upconv1_spi_gpio4_out_is : in std_logic;

upconv2_spi_gpio4_out_is : in std_logic;

ant1_2015_upst_cntr_is,ant2_2015_upst_cntr_is: in std_logic_vector(7 downto 0);

ant3_2015_upst_cntr_is,ant4_2015_upst_cntr_is: in std_logic_vector(7 downto 0);

acq_threshold_LSW_is :in std_logic_vector(31 downto 0);

acq_threshold_MSW_is :in std_logic_vector(31 downto 0);

trk_threshold_is  :in std_logic_vector(31 downto 0);

acq1_search_reg_is   : in std_logic_vector(63 downto 0);

acq2_search_reg_is   : in std_logic_vector(63 downto 0);

acq3_search_reg_is   : in std_logic_vector(63 downto 0);

acq4_search_reg_is   : in std_logic_vector(63 downto 0);

fft_status_is : in std_logic_vector(31 downto 0);

launch_orbit_phase_is: in std_logic;

carr_aided_loop_is   : in std_logic;

fll_zero_en_dis_is   : in std_logic;

config_crc_err_is: in std_logic;

config_ecc_err_is: in std_logic;

config_synd_valid_is: in std_logic;

icap_busy_is: in std_logic;

syndrome_is      : in std_logic_vector(11 downto 0);

synd_valid_count_is: in std_logic_vector(7 downto 0);

rt_adddr_sel_is    : in std_logic_vector(1 downto 0);

mil_1553_interrupt_RT1_is: in std_logic;

mil_1553_interrupt_RT2_is: in std_logic;

mil_1553_interrupt_RT3_is: in std_logic;

antenna_combining_control_is : in std_logic_vector(3 downto 0);

lock_detector_threshold1_is : in std_logic_vector(9 downto 0);

lock_detector_threshold2_is : in std_logic_vector(9 downto 0);

SY_FE_write_count_is : in std_logic_vector(3 downto 0);

reg_val_read_is : in std_logic_vector(7 downto 0);

latched_incoming_inp1_countp7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp1_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp1_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp1_countp1_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp1_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp1_countm5_is : in std_logic_vector(25 downto 0);

latched_incoming_inp1_countm3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp1_countm1_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp1_countp7_is : in std_logic_vector(25 downto 0);

latched_incoming_qp1_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp1_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_qp1_countp1_is : in std_logic_vector(25 downto 0);

latched_incoming_qp1_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp1_countm5_is: in std_logic_vector(25 downto 0);

latched_incoming_qp1_countm3_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp1_countm1_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countp7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countp1_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countm5_is : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countm3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp2_countm1_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp2_countp7_is : in std_logic_vector(25 downto 0);

latched_incoming_qp2_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp2_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_qp2_countp1_is : in std_logic_vector(25 downto 0);

latched_incoming_qp2_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp2_countm5_is: in std_logic_vector(25 downto 0);

latched_incoming_qp2_countm3_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp2_countm1_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countp7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countp1_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countm5_is : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countm3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp3_countm1_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp3_countp7_is : in std_logic_vector(25 downto 0);

latched_incoming_qp3_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp3_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_qp3_countp1_is : in std_logic_vector(25 downto 0);

latched_incoming_qp3_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp3_countm5_is: in std_logic_vector(25 downto 0);

latched_incoming_qp3_countm3_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp3_countm1_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countp7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countp1_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countm5_is : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countm3_is : in std_logic_vector(25 downto 0);

latched_incoming_inp4_countm1_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp4_countp7_is : in std_logic_vector(25 downto 0);

latched_incoming_qp4_countp5_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp4_countp3_is : in std_logic_vector(25 downto 0);

latched_incoming_qp4_countp1_is : in std_logic_vector(25 downto 0);

latched_incoming_qp4_countm7_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp4_countm5_is: in std_logic_vector(25 downto 0);

latched_incoming_qp4_countm3_is  : in std_logic_vector(25 downto 0);

latched_incoming_qp4_countm1_is  : in std_logic_vector(25 downto 0);

-- Outputs

data_to_processor_os : out std_logic_vector(31 downto 0);

uart_txd_is: in std_logic;

rwe3_is    : in std_logic;

wdt_l_is : in std_logic;

bexc_l_is: in std_logic);

end component corr_data_out_mux_e;

COMPONENT edac_14_enc1_e     Port (  -- Input signals

wr_en_is       : in  STD_LOGIC;  -- EDAC Encoder Write enable

data_in_is     : in  STD_LOGIC_VECTOR (13 downto 0);   -- Input Information bits to EDAC Ecnoder

-- Output signals

data_out_os    : out  STD_LOGIC_VECTOR (13 downto 0);  -- Output information bits after correction

par_out_os     : out  STD_LOGIC_VECTOR (5 downto 0)  -- Output parity bits

);

end COMPONENT edac_14_enc1_e;

component SPI_code_sy

port (

reset_is : IN std_logic;

start_prog_flag_SY_is : IN std_logic;

SY17_PLL_val_is : IN std_logic_vector (15 downto 0) ;

SY17_AGC_val_is : IN std_logic_vector (15 downto 0) ;

SYFE_PM_val_is : IN std_logic_vector (15 downto 0) ;

SYFE_RFPLL_val_is : IN std_logic_vector (15 downto 0) ;

SYFE_IFPLL_val_is : IN std_logic_vector (15 downto 0) ;

read_SPI_SY_is : IN std_logic;

done_wr_SY_os: OUT std_logic;

read_error_SY_os: OUT std_logic;

spi_clk_SY_is: in std_logic;

spi_en_SY_os: OUT std_logic;

spi_mosi_SY_os: OUT std_logic;

spi_miso_SY_is: in std_logic;

reg_val_read_os: OUT std_logic_vector (7 downto 0)

);

end component SPI_code_sy;

component SPI_control_SY

Port ( clk_is : in  STD_LOGIC;

reset_is : in  STD_LOGIC;

reprogram_SY_cmd_flag_is : in  STD_LOGIC;

done_wr_SY_is : in  STD_LOGIC;

read_error_SY_is : in  STD_LOGIC;

start_prog_flag_SY_os : out  STD_LOGIC;

read_SPI_SY_os : out  STD_LOGIC;

SY_FE_write_count_os :out std_logic_vector(3 downto 0)

);

end component SPI_control_SY;

--	signal clk_enable_s: std_logic;

signal svid_channel_ch_s: six_ch1_18_typ;

signal svid_channel_in_s : std_logic_vector(5 downto 0);

signal sat_pr_interrupt_s : std_logic_vector(acq_modules_count_c downto 0);

signal svid_acq_s,svid_searched_s : six_acq1_4_typ ;

signal ant1_ant2_sel_s: two_acq1_4_typ;

signal start_aquisition_s: std_logic_vector(acq_modules_count_c downto 0);

signal sat_status_s : std_logic_vector(63 downto 0);

signal wdt_cmd_status_s:  std_logic_vector(1 downto 0);

signal hdwt_reset_counter_s: std_logic_vector(7 downto 0);

signal epoch_latch_pulse_s: std_logic;

signal channel_status_s: std_logic_vector(channels_count_c downto 0);

signal wdt_enable_pr_wr_enable_s,clear_wdt_pr_s: std_logic;

signal hwdt_reset_clear_pr_wr_enable_s,hwdt_reset_status_ff_s: std_logic;

signal ten_ms_counter1_18_prev_s,count_enable1_s,clear_ten_ms_counter1_ceil_s: std_logic;

signal set_ce1_s,clr_ce1_s: std_logic;

signal ten_ms_counter1_s : std_logic_vector(18 downto 0);

signal epoch_latch_pulse_temp_s: std_logic;

signal track_th_sel_s : std_logic;

signal reset_pps_interrupt_RT1_s, reset_pps_interrupt_RT2_s,reset_pps_interrupt_RT3_s : std_logic;

signal TSM_data_RT1_s, TSM_data_RT2_s ,TSM_data_RT3_s  :  std_logic_vector(31 downto 0);

signal car_doppler_ch_s,code_doppler_ch_s: std_logic_vector(31 downto 0);

signal PR_to_FPGA_reset_reg_sel_s,soft_reset_flag_s: std_logic;

signal reset_channel_en_s: std_logic_vector(channels_count_c downto 0);

signal upconv1_spi_gpio4_out_s,upconv2_spi_gpio4_out_s,upconv1_spi_reg_wr_sel_s,upconv2_spi_reg_wr_sel_s : std_logic;

signal sbas_enable_s,sbas_enable_reg_wr_sel_s,navic_cmd_enable_reg_wr_sel_s : std_logic;

signal navic_cmd_enable_s : std_logic_vector(13 downto 0);

signal sbas_navdata_s: std_logic_vector(5 downto 0);

signal ant1_ant2_sel_ch_s: std_logic_vector(1 downto 0);

signal pps_on_off_s,pps_on_off_sel_s: std_logic;

signal data_to_processor_old_s : std_logic_vector(31 downto 0);

signal channel_par_load_ds_s : std_logic_vector(channels_count_c downto 0);

signal reg_wr_procr_tms_ch_s : std_logic_vector(channels_count_c downto 0);

signal p_i_ch_s :twentythree_ch1_18_typ;--tr_aray_18_len_16_typ;

signal p_q_ch_s :twentythree_ch1_18_typ;--tr_aray_18_len_16_typ;

signal e_i_ch_s :twentythree_ch1_18_typ;--tr_aray_18_len_16_typ;

signal e_q_ch_s :twentythree_ch1_18_typ;--tr_aray_18_len_16_typ;

signal l_i_ch_s :twentythree_ch1_18_typ;--tr_aray_18_len_16_typ;

signal l_q_ch_s :twentythree_ch1_18_typ;--tr_aray_18_len_16_typ;

signal lock_status_out_s : std_logic_vector(channels_count_c downto 0);

signal lock_status_in_ds_s:  std_logic_vector(channels_count_c downto 0);

signal loop_start_ch_s :  std_logic_vector(channels_count_c downto 0);

signal fll_data_check_ch_s,fll_zero_s,Data_boundary_input_s,NAVIC_indicator_s: std_logic_vector(channels_count_c downto 0);

signal Prev_phase_error_ds_s :std_logic_vector(16 downto 0);--tr_aray_18_len_17_typ;

signal Order1_out_ds_s :std_logic_vector(48 downto 0);--tr_aray_18_len_49_typ;

signal Order2_out_ds_s :std_logic_vector(48 downto 0);--tr_aray_18_len_49_typ;

signal Order3_out_ds_s :std_logic_vector(48 downto 0);--tr_aray_18_len_49_typ;

signal DLL_Order1_out_ds_s :std_logic_vector(48 downto 0);--tr_aray_18_len_49_typ;

signal DLL_Order2_out_ds_s :std_logic_vector(48 downto 0);--tr_aray_18_len_49_typ;

signal Prev_phase_error_ch_s :seventeen_ch1_18_typ;

signal Order1_out_ch_s :fortynine_ch1_18_typ;

signal Order2_out_ch_s :fortynine_ch1_18_typ;

signal Order3_out_ch_s :fortynine_ch1_18_typ;

signal DLL_Order1_out_ch_s :fortynine_ch1_18_typ;

signal DLL_Order2_out_ch_s :fortynine_ch1_18_typ;

signal code_corr_freq_in_ds_s, car_corr_freq_in_ds_s : std_logic_vector(31 downto 0);

signal stage_control_s :five_ch1_18_typ;  -- fll_start signal, when FLL descriminator to be operated

signal Launch_or_Orbit_input_s : std_logic;

signal set1_to_set2_timer_launch_s : std_logic_vector(31 downto 0);

signal set2_to_set3_timer_launch_s : std_logic_vector(31 downto 0);

signal set3_to_set4_timer_launch_s :  std_logic_vector(31 downto 0);

signal set4_to_set5_timer_launch_s :  std_logic_vector(31 downto 0);

signal set1_to_set2_timer_launch_def_s : std_logic_vector(31 downto 0);

signal set2_to_set3_timer_launch_def_s : std_logic_vector(31 downto 0);

signal set3_to_set4_timer_launch_def_s :  std_logic_vector(31 downto 0);

signal set4_to_set5_timer_launch_def_s :  std_logic_vector(31 downto 0);

signal FLL_data_check_counter_s :  std_logic_vector(31 downto 0);

signal fll_timer_s :  std_logic_vector(31 downto 0);

signal fll_timer_def_s :  std_logic_vector(31 downto 0);

signal All_Timers_updated_s : std_logic;

signal p_cndr_out_ch_s :twentyeight_ch1_18_typ;

signal q_cndr_out_ch_s :twentyeight_ch1_18_typ;

signal car_cycle_count_ch_s :thirtytwo_ch1_18_typ;

signal car_phase_count_ch_s :thirtytwo_ch1_18_typ;

signal one_ms_counter_value_ch_s :five_ch1_18_typ;

signal twenty_ms_counter_value_ch_s :ten_ch1_18_typ;

signal code_cyle_count_value_ch_s :ten_ch1_18_typ;

signal code_phase_count_ch_s :thirtytwo_ch1_18_typ;

signal start_channel_s,nav_data_read_over_ch_s: std_logic_vector(channels_count_c downto 0);

signal order1_loop_init_ch_s, dll_order1_loop_init_ch_s : std_logic_vector(48 downto 0);

signal order1_loop_init_acq_s, dll_order1_loop_init_acq_s : fortynine_acq1_4_typ;

signal nav_data_reg_ch_s: eight_ch1_18_typ;

signal alm_ant1_gps_reg_s,alm_ant2_gps_reg_s,alm_ant3_gps_reg_s,alm_ant4_gps_reg_s: std_logic_vector(acq_modules_count_c downto 0);

signal alm_ant1_nav_reg_s,alm_ant2_nav_reg_s,alm_ant3_nav_reg_s,alm_ant4_nav_reg_s: std_logic_vector(acq_modules_count_c downto 0);

signal alm_ant1_sbas_reg_s,alm_ant2_sbas_reg_s,alm_ant3_sbas_reg_s,alm_ant4_sbas_reg_s: std_logic_vector(acq_modules_count_c downto 0);

signal car_freq_word_acq_s,code_freq_word_acq_s : thirtytwo_acq1_4_typ;

signal acq_th_MSW_sel_s,acq_th_LSW_sel_s : std_logic;

signal pps_pulse_count_data_en_s: std_logic;

signal   pll_par1_set1_launch_enable_s :  std_logic;

signal 	pll_par2_set1_launch_enable_s :  std_logic;

signal	pll_par3_set1_launch_enable_s :  std_logic;

signal   pll_par4_set1_launch_enable_s :  std_logic;

signal	fll_par1_set1_launch_enable_s :  std_logic;

signal	fll_par2_set1_launch_enable_s :  std_logic;

signal	fll_par3_set1_launch_enable_s :  std_logic;

signal	dll_par1_set1_launch_enable_s :  std_logic;

signal	dll_par2_set1_launch_enable_s :  std_logic;

signal	dll_par3_set1_launch_enable_s :  std_logic;

signal	pll_par1_set2_launch_enable_s :  std_logic;

signal	pll_par2_set2_launch_enable_s :  std_logic;

signal	pll_par3_set2_launch_enable_s :  std_logic;

signal	pll_par4_set2_launch_enable_s :  std_logic;

signal	fll_par1_set2_launch_enable_s :  std_logic;

signal	fll_par2_set2_launch_enable_s :  std_logic;

signal	fll_par3_set2_launch_enable_s :  std_logic;

signal	dll_par1_set2_launch_enable_s :  std_logic;

signal	dll_par2_set2_launch_enable_s :  std_logic;

signal	dll_par3_set2_launch_enable_s :  std_logic;

signal	pll_par1_set3_launch_enable_s :  std_logic;

signal	pll_par2_set3_launch_enable_s :  std_logic;

signal	pll_par3_set3_launch_enable_s :  std_logic;

signal	pll_par4_set3_launch_enable_s :  std_logic;

signal	fll_par1_set3_launch_enable_s :  std_logic;

signal	fll_par2_set3_launch_enable_s :  std_logic;

signal	fll_par3_set3_launch_enable_s :  std_logic;

signal	dll_par1_set3_launch_enable_s :  std_logic;

signal	dll_par2_set3_launch_enable_s :  std_logic;

signal	dll_par3_set3_launch_enable_s :  std_logic;

signal	pll_par1_set4_launch_enable_s :  std_logic;

signal	pll_par2_set4_launch_enable_s :  std_logic;

signal	pll_par3_set4_launch_enable_s :  std_logic;

signal	pll_par4_set4_launch_enable_s :  std_logic;

signal	fll_par1_set4_launch_enable_s :  std_logic;

signal	fll_par2_set4_launch_enable_s :  std_logic;

signal	fll_par3_set4_launch_enable_s :  std_logic;

signal	dll_par1_set4_launch_enable_s :  std_logic;

signal	dll_par2_set4_launch_enable_s :  std_logic;

signal	dll_par3_set4_launch_enable_s :  std_logic;

signal	pll_par1_set5_launch_enable_s :  std_logic;

signal	pll_par2_set5_launch_enable_s :  std_logic;

signal	pll_par3_set5_launch_enable_s :  std_logic;

signal	pll_par4_set5_launch_enable_s :  std_logic;

signal	fll_par1_set5_launch_enable_s :  std_logic;

signal	fll_par2_set5_launch_enable_s :  std_logic;

signal	fll_par3_set5_launch_enable_s :  std_logic;

signal	dll_par1_set5_launch_enable_s :  std_logic;

signal	dll_par2_set5_launch_enable_s :  std_logic;

signal	dll_par3_set5_launch_enable_s :  std_logic;

signal  cdll_par1_set1_launch_enable_s :  std_logic;

signal	launch_orbit_phase_sel_s      :  std_logic;

signal	carr_aided_loop_sel_s         :  std_logic;

signal	set1_to_set2_timer_launch_enable_s   :  std_logic;

signal	set2_to_set3_timer_launch_enable_s   :  std_logic;

signal	set3_to_set4_timer_launch_enable_s   :  std_logic;

signal	set4_to_set5_timer_launch_enable_s   :  std_logic;

signal	fll_timer_enable_s            :  std_logic;

signal	fll_zero_en_dis_sel_s      :  std_logic;

signal	loop_par_status1_enable_s      :  std_logic;

signal	All_Timers_updated_enable_s    :  std_logic;

signal	FLL_data_check_counter_enable_s:  std_logic;

signal  TIC_intrpt_clear_s     : std_logic;

signal  TIC_wr_enable_s        : std_logic;

signal  clear_TIC_counter_s    : std_logic;

signal  TIC_drift_wr_enable_s  : std_logic;

signal  ten_ms_reg_s: std_logic;

signal ant1_2015_upst_cntr_s,ant2_2015_upst_cntr_s:std_logic_vector(7 downto 0);

signal ant3_2015_upst_cntr_s,ant4_2015_upst_cntr_s:std_logic_vector(7 downto 0);

signal acq_threshold_LSW_s : std_logic_vector(31 downto 0);

signal acq_threshold_MSW_s : std_logic_vector(31 downto 0);

signal trk_threshold_s : std_logic_vector(31 downto 0);

signal prev_lock_detector_ant1_s ,	prev_lock_detector_ant2_s, prev_lock_detector_ant3_s,	prev_lock_detector_ant4_s : std_logic;

signal acq1_search_reg_s : std_logic_vector(63 downto 0);

signal acq2_search_reg_s : std_logic_vector(63 downto 0);

signal acq3_search_reg_s : std_logic_vector(63 downto 0);

signal acq4_search_reg_s : std_logic_vector(63 downto 0);

signal carr_aided_loop_out_s,fll_zero_en_dis_out_s: std_logic;

--signal samp_clk_enable_s : std_logic;

signal upconv1_spi_reg_s: std_logic_vector(4 downto 0);

signal prev_upconv1_sclk_s,prev_upconv2_sclk_s : std_logic;

signal upconv2_spi_reg_s: std_logic_vector(4 downto 0);

signal pll_order1_s, pll_order2_s, pll_order3_s   :  thirtytwo_ch1_18_typ;

signal lock_counter_threshold1_s, lock_counter_threshold2_s : std_logic_vector(9 downto 0);

signal lock_detector_threshold_sel_s : std_logic;

signal RT1_flag_ff1_s,RT1_flag_ff2_s,RT1_flag_ff3_s : std_logic;

signal RT2_flag_ff1_s,RT2_flag_ff2_s,RT2_flag_ff3_s : std_logic;

signal RT3_flag_ff1_s,RT3_flag_ff2_s,RT3_flag_ff3_s : std_logic;

signal mil_1553_interrupt_RT1_s: std_logic;

signal mil_1553_interrupt_RT2_s: std_logic;

signal mil_1553_interrupt_RT3_s: std_logic;

signal fft_status_s : std_logic_vector(31 downto 0);

signal antenna_combining_control_s : std_logic_vector(3 downto 0);

signal Antenna_combining_sel_enable_s : std_logic;

-- carrier mix ROM scrub signals.

signal car_mix_ROM_ena_s : STD_LOGIC;

signal car_mix_ROM_wea_s :  STD_LOGIC_VECTOR(0 DOWNTO 0);

signal car_mix_ROM_addra_s :  STD_LOGIC_VECTOR(9 DOWNTO 0);

signal car_mix_ROM_dina_s :  STD_LOGIC_VECTOR(19 DOWNTO 0);

signal one_ms_count_scrub_s : std_logic_vector(16 downto 0);

signal start_scrub_s : std_logic;

signal edac_data_out_scrub_s : std_logic_vector(13 downto 0);

signal  edac_par_out_scrub_s : std_logic_vector(5 downto 0);

signal Local_inphase_scrub_s, Local_qphase_scrub_s , incoming_inphase_scrub_s, incoming_qphase_scrub_s : std_logic_vector(3 downto 0);

signal edac_data_input_scrub_s : std_logic_vector(13 downto 0);

signal IP_mixout_scrub_s , QP_mixout_scrub_s : STD_LOGIC_VECTOR(7 DOWNTO 0);

signal local_addr_gen_scrub_s , loc1_scrub_s, loc2_scrub_s, loc3_scrub_s : std_logic_vector(9 downto 0);

signal IF_Freq_GPS_acq_sel_s,IF_Freq_NAVIC_acq_sel_s , IF_Freq_GPS_trk_sel_s,IF_Freq_NAVIC_trk_sel_s : std_logic;

signal Narrow_corr_step1_timer_s :  std_logic_vector(31 downto 0);

signal Narrow_corr_step2_timer_s :  std_logic_vector(31 downto 0);

signal Narrow_corr_step1_timer_def_s :  std_logic_vector(31 downto 0);

signal Narrow_corr_step2_timer_def_s :  std_logic_vector(31 downto 0);

signal Narrow_correlator_step1_enable_s:  std_logic;

signal Narrow_correlator_step2_enable_s:  std_logic;

signal Narrow_corr_step1_timer_sel_s  :  std_logic;

signal Narrow_corr_step2_timer_sel_s   :  std_logic;

signal Narrow_correlator_step1_enable_sel_s  :  std_logic;

signal Narrow_correlator_step2_enable_sel_s  :  std_logic;

signal start_prog_flag_SY_s, done_wr_SY_s, read_SPI_SY_s, read_error_SY_s : std_logic;

signal SY_FE_write_count_s : std_logic_vector(3 downto 0);

signal reg_val_read_s : std_logic_vector(7 downto 0);

signal   SY17_PLL_val_s : std_logic_vector (15 downto 0) ;

signal   SY17_AGC_val_s : std_logic_vector (15 downto 0) ;

signal   SYFE_PM_val_s : std_logic_vector (15 downto 0) ;

signal   SYFE_RFPLL_val_s : std_logic_vector (15 downto 0) ;

signal   SYFE_IFPLL_val_s : std_logic_vector (15 downto 0) ;

signal reprogram_SY_cmd_s  :  std_logic;

signal reprogram_SY_cmd_sel_s  :  std_logic;

signal SY17_PLL_val_sel_s , SY17_AGC_val_sel_s ,SYFE_PM_val_sel_s ,SYFE_RFPLL_val_sel_s ,SYFE_IFPLL_val_sel_s  :  std_logic;

signal incoming_inp1_countp7_s, incoming_inp1_countp5_s, incoming_inp1_countp3_s, incoming_inp1_countp1_s : std_logic_vector(25 downto 0);

signal incoming_inp1_countm7_s, incoming_inp1_countm5_s, incoming_inp1_countm3_s, incoming_inp1_countm1_s : std_logic_vector(25 downto 0);

signal incoming_qp1_countp7_s, incoming_qp1_countp5_s, incoming_qp1_countp3_s, incoming_qp1_countp1_s : std_logic_vector(25 downto 0);

signal incoming_qp1_countm7_s, incoming_qp1_countm5_s, incoming_qp1_countm3_s, incoming_qp1_countm1_s : std_logic_vector(25 downto 0);

signal latched_incoming_inp1_countp7_s, latched_incoming_inp1_countp5_s, latched_incoming_inp1_countp3_s, latched_incoming_inp1_countp1_s : std_logic_vector(25 downto 0);

signal latched_incoming_inp1_countm7_s, latched_incoming_inp1_countm5_s, latched_incoming_inp1_countm3_s, latched_incoming_inp1_countm1_s : std_logic_vector(25 downto 0);

signal latched_incoming_qp1_countp7_s, latched_incoming_qp1_countp5_s, latched_incoming_qp1_countp3_s, latched_incoming_qp1_countp1_s : std_logic_vector(25 downto 0);

signal latched_incoming_qp1_countm7_s, latched_incoming_qp1_countm5_s, latched_incoming_qp1_countm3_s, latched_incoming_qp1_countm1_s : std_logic_vector(25 downto 0);

signal incoming_inp2_countp7_s, incoming_inp2_countp5_s, incoming_inp2_countp3_s, incoming_inp2_countp1_s : std_logic_vector(25 downto 0);

signal incoming_inp2_countm7_s, incoming_inp2_countm5_s, incoming_inp2_countm3_s, incoming_inp2_countm1_s : std_logic_vector(25 downto 0);

signal incoming_qp2_countp7_s, incoming_qp2_countp5_s, incoming_qp2_countp3_s, incoming_qp2_countp1_s : std_logic_vector(25 downto 0);

signal incoming_qp2_countm7_s, incoming_qp2_countm5_s, incoming_qp2_countm3_s, incoming_qp2_countm1_s : std_logic_vector(25 downto 0);

signal latched_incoming_inp2_countp7_s, latched_incoming_inp2_countp5_s, latched_incoming_inp2_countp3_s, latched_incoming_inp2_countp1_s : std_logic_vector(25 downto 0);

signal latched_incoming_inp2_countm7_s, latched_incoming_inp2_countm5_s, latched_incoming_inp2_countm3_s, latched_incoming_inp2_countm1_s : std_logic_vector(25 downto 0);

signal latched_incoming_qp2_countp7_s, latched_incoming_qp2_countp5_s, latched_incoming_qp2_countp3_s, latched_incoming_qp2_countp1_s : std_logic_vector(25 downto 0);

signal latched_incoming_qp2_countm7_s, latched_incoming_qp2_countm5_s, latched_incoming_qp2_countm3_s, latched_incoming_qp2_countm1_s : std_logic_vector(25 downto 0);

signal incoming_inp3_countp7_s, incoming_inp3_countp5_s, incoming_inp3_countp3_s, incoming_inp3_countp1_s : std_logic_vector(25 downto 0);

signal incoming_inp3_countm7_s, incoming_inp3_countm5_s, incoming_inp3_countm3_s, incoming_inp3_countm1_s : std_logic_vector(25 downto 0);

signal incoming_qp3_countp7_s, incoming_qp3_countp5_s, incoming_qp3_countp3_s, incoming_qp3_countp1_s : std_logic_vector(25 downto 0);

signal incoming_qp3_countm7_s, incoming_qp3_countm5_s, incoming_qp3_countm3_s, incoming_qp3_countm1_s : std_logic_vector(25 downto 0);

signal latched_incoming_inp3_countp7_s, latched_incoming_inp3_countp5_s, latched_incoming_inp3_countp3_s, latched_incoming_inp3_countp1_s : std_logic_vector(25 downto 0);

signal latched_incoming_inp3_countm7_s, latched_incoming_inp3_countm5_s, latched_incoming_inp3_countm3_s, latched_incoming_inp3_countm1_s : std_logic_vector(25 downto 0);

signal latched_incoming_qp3_countp7_s, latched_incoming_qp3_countp5_s, latched_incoming_qp3_countp3_s, latched_incoming_qp3_countp1_s : std_logic_vector(25 downto 0);

signal latched_incoming_qp3_countm7_s, latched_incoming_qp3_countm5_s, latched_incoming_qp3_countm3_s, latched_incoming_qp3_countm1_s : std_logic_vector(25 downto 0);

signal incoming_inp4_countp7_s, incoming_inp4_countp5_s, incoming_inp4_countp3_s, incoming_inp4_countp1_s : std_logic_vector(25 downto 0);

signal incoming_inp4_countm7_s, incoming_inp4_countm5_s, incoming_inp4_countm3_s, incoming_inp4_countm1_s : std_logic_vector(25 downto 0);

signal incoming_qp4_countp7_s, incoming_qp4_countp5_s, incoming_qp4_countp3_s, incoming_qp4_countp1_s : std_logic_vector(25 downto 0);

signal incoming_qp4_countm7_s, incoming_qp4_countm5_s, incoming_qp4_countm3_s, incoming_qp4_countm1_s : std_logic_vector(25 downto 0);

signal latched_incoming_inp4_countp7_s, latched_incoming_inp4_countp5_s, latched_incoming_inp4_countp3_s, latched_incoming_inp4_countp1_s : std_logic_vector(25 downto 0);

signal latched_incoming_inp4_countm7_s, latched_incoming_inp4_countm5_s, latched_incoming_inp4_countm3_s, latched_incoming_inp4_countm1_s : std_logic_vector(25 downto 0);

signal latched_incoming_qp4_countp7_s, latched_incoming_qp4_countp5_s, latched_incoming_qp4_countp3_s, latched_incoming_qp4_countp1_s : std_logic_vector(25 downto 0);

signal latched_incoming_qp4_countm7_s, latched_incoming_qp4_countm5_s, latched_incoming_qp4_countm3_s, latched_incoming_qp4_countm1_s : std_logic_vector(25 downto 0);

begin

ADC_cap: process(clk_is, POR_G_is)

begin

if(POR_G_is = '1') then

incoming_inp1_countp7_s<= (OTHERS=>'0');

incoming_inp1_countp5_s<= (OTHERS=>'0');

incoming_inp1_countp3_s<= (OTHERS=>'0');

incoming_inp1_countp1_s<= (OTHERS=>'0');

incoming_inp1_countm7_s<= (OTHERS=>'0');

incoming_inp1_countm5_s<= (OTHERS=>'0');

incoming_inp1_countm3_s<= (OTHERS=>'0');

incoming_inp1_countm1_s<= (OTHERS=>'0');

incoming_qp1_countp7_s<= (OTHERS=>'0');

incoming_qp1_countp5_s<= (OTHERS=>'0');

incoming_qp1_countp3_s<= (OTHERS=>'0');

incoming_qp1_countp1_s<= (OTHERS=>'0');

incoming_qp1_countm7_s<= (OTHERS=>'0');

incoming_qp1_countm5_s<= (OTHERS=>'0');

incoming_qp1_countm3_s<= (OTHERS=>'0');

incoming_qp1_countm1_s<= (OTHERS=>'0');

latched_incoming_inp1_countp7_s<= (OTHERS=>'0');

latched_incoming_inp1_countp5_s<= (OTHERS=>'0');

latched_incoming_inp1_countp3_s<= (OTHERS=>'0');

latched_incoming_inp1_countp1_s<= (OTHERS=>'0');

latched_incoming_inp1_countm7_s<= (OTHERS=>'0');

latched_incoming_inp1_countm5_s<= (OTHERS=>'0');

latched_incoming_inp1_countm3_s<= (OTHERS=>'0');

latched_incoming_inp1_countm1_s<= (OTHERS=>'0');

latched_incoming_qp1_countp7_s<= (OTHERS=>'0');

latched_incoming_qp1_countp5_s<= (OTHERS=>'0');

latched_incoming_qp1_countp3_s<= (OTHERS=>'0');

latched_incoming_qp1_countp1_s<= (OTHERS=>'0');

latched_incoming_qp1_countm7_s<= (OTHERS=>'0');

latched_incoming_qp1_countm5_s<= (OTHERS=>'0');

latched_incoming_qp1_countm3_s<= (OTHERS=>'0');

latched_incoming_qp1_countm1_s<= (OTHERS=>'0');

incoming_inp2_countp7_s<= (OTHERS=>'0');

incoming_inp2_countp5_s<= (OTHERS=>'0');

incoming_inp2_countp3_s<= (OTHERS=>'0');

incoming_inp2_countp1_s<= (OTHERS=>'0');

incoming_inp2_countm7_s<= (OTHERS=>'0');

incoming_inp2_countm5_s<= (OTHERS=>'0');

incoming_inp2_countm3_s<= (OTHERS=>'0');

incoming_inp2_countm1_s<= (OTHERS=>'0');

incoming_qp2_countp7_s<= (OTHERS=>'0');

incoming_qp2_countp5_s<= (OTHERS=>'0');

incoming_qp2_countp3_s<= (OTHERS=>'0');

incoming_qp2_countp1_s<= (OTHERS=>'0');

incoming_qp2_countm7_s<= (OTHERS=>'0');

incoming_qp2_countm5_s<= (OTHERS=>'0');

incoming_qp2_countm3_s<= (OTHERS=>'0');

incoming_qp2_countm1_s<= (OTHERS=>'0');

latched_incoming_inp2_countp7_s<= (OTHERS=>'0');

latched_incoming_inp2_countp5_s<= (OTHERS=>'0');

latched_incoming_inp2_countp3_s<= (OTHERS=>'0');

latched_incoming_inp2_countp1_s<= (OTHERS=>'0');

latched_incoming_inp2_countm7_s<= (OTHERS=>'0');

latched_incoming_inp2_countm5_s<= (OTHERS=>'0');

latched_incoming_inp2_countm3_s<= (OTHERS=>'0');

latched_incoming_inp2_countm1_s<= (OTHERS=>'0');

latched_incoming_qp2_countp7_s<= (OTHERS=>'0');

latched_incoming_qp2_countp5_s<= (OTHERS=>'0');

latched_incoming_qp2_countp3_s<= (OTHERS=>'0');

latched_incoming_qp2_countp1_s<= (OTHERS=>'0');

latched_incoming_qp2_countm7_s<= (OTHERS=>'0');

latched_incoming_qp2_countm5_s<= (OTHERS=>'0');

latched_incoming_qp2_countm3_s<= (OTHERS=>'0');

latched_incoming_qp2_countm1_s<= (OTHERS=>'0');

incoming_inp3_countp7_s<= (OTHERS=>'0');

incoming_inp3_countp5_s<= (OTHERS=>'0');

incoming_inp3_countp3_s<= (OTHERS=>'0');

incoming_inp3_countp1_s<= (OTHERS=>'0');

incoming_inp3_countm7_s<= (OTHERS=>'0');

incoming_inp3_countm5_s<= (OTHERS=>'0');

incoming_inp3_countm3_s<= (OTHERS=>'0');

incoming_inp3_countm1_s<= (OTHERS=>'0');

incoming_qp3_countp7_s<= (OTHERS=>'0');

incoming_qp3_countp5_s<= (OTHERS=>'0');

incoming_qp3_countp3_s<= (OTHERS=>'0');

incoming_qp3_countp1_s<= (OTHERS=>'0');

incoming_qp3_countm7_s<= (OTHERS=>'0');

incoming_qp3_countm5_s<= (OTHERS=>'0');

incoming_qp3_countm3_s<= (OTHERS=>'0');

incoming_qp3_countm1_s<= (OTHERS=>'0');

latched_incoming_inp3_countp7_s<= (OTHERS=>'0');

latched_incoming_inp3_countp5_s<= (OTHERS=>'0');

latched_incoming_inp3_countp3_s<= (OTHERS=>'0');

latched_incoming_inp3_countp1_s<= (OTHERS=>'0');

latched_incoming_inp3_countm7_s<= (OTHERS=>'0');

latched_incoming_inp3_countm5_s<= (OTHERS=>'0');

latched_incoming_inp3_countm3_s<= (OTHERS=>'0');

latched_incoming_inp3_countm1_s<= (OTHERS=>'0');

latched_incoming_qp3_countp7_s<= (OTHERS=>'0');

latched_incoming_qp3_countp5_s<= (OTHERS=>'0');

latched_incoming_qp3_countp3_s<= (OTHERS=>'0');

latched_incoming_qp3_countp1_s<= (OTHERS=>'0');

latched_incoming_qp3_countm7_s<= (OTHERS=>'0');

latched_incoming_qp3_countm5_s<= (OTHERS=>'0');

latched_incoming_qp3_countm3_s<= (OTHERS=>'0');

latched_incoming_qp3_countm1_s<= (OTHERS=>'0');

incoming_inp4_countp7_s<= (OTHERS=>'0');

incoming_inp4_countp5_s<= (OTHERS=>'0');

incoming_inp4_countp3_s<= (OTHERS=>'0');

incoming_inp4_countp1_s<= (OTHERS=>'0');

incoming_inp4_countm7_s<= (OTHERS=>'0');

incoming_inp4_countm5_s<= (OTHERS=>'0');

incoming_inp4_countm3_s<= (OTHERS=>'0');

incoming_inp4_countm1_s<= (OTHERS=>'0');

incoming_qp4_countp7_s<= (OTHERS=>'0');

incoming_qp4_countp5_s<= (OTHERS=>'0');

incoming_qp4_countp3_s<= (OTHERS=>'0');

incoming_qp4_countp1_s<= (OTHERS=>'0');

incoming_qp4_countm7_s<= (OTHERS=>'0');

incoming_qp4_countm5_s<= (OTHERS=>'0');

incoming_qp4_countm3_s<= (OTHERS=>'0');

incoming_qp4_countm1_s<= (OTHERS=>'0');

latched_incoming_inp4_countp7_s<= (OTHERS=>'0');

latched_incoming_inp4_countp5_s<= (OTHERS=>'0');

latched_incoming_inp4_countp3_s<= (OTHERS=>'0');

latched_incoming_inp4_countp1_s<= (OTHERS=>'0');

latched_incoming_inp4_countm7_s<= (OTHERS=>'0');

latched_incoming_inp4_countm5_s<= (OTHERS=>'0');

latched_incoming_inp4_countm3_s<= (OTHERS=>'0');

latched_incoming_inp4_countm1_s<= (OTHERS=>'0');

latched_incoming_qp4_countp7_s<= (OTHERS=>'0');

latched_incoming_qp4_countp5_s<= (OTHERS=>'0');

latched_incoming_qp4_countp3_s<= (OTHERS=>'0');

latched_incoming_qp4_countp1_s<= (OTHERS=>'0');

latched_incoming_qp4_countm7_s<= (OTHERS=>'0');

latched_incoming_qp4_countm5_s<= (OTHERS=>'0');

latched_incoming_qp4_countm3_s<= (OTHERS=>'0');

latched_incoming_qp4_countm1_s<= (OTHERS=>'0');

elsif(clk_is='1') and (clk_is'event) then

if(epoch_latch_pulse_s='1') then

latched_incoming_inp1_countp7_s<= incoming_inp1_countp7_s;

latched_incoming_inp1_countp5_s<= incoming_inp1_countp5_s;

latched_incoming_inp1_countp3_s<= incoming_inp1_countp3_s;

latched_incoming_inp1_countp1_s<= incoming_inp1_countp1_s;

latched_incoming_inp1_countm7_s<= incoming_inp1_countm7_s;

latched_incoming_inp1_countm5_s<= incoming_inp1_countm5_s;

latched_incoming_inp1_countm3_s<= incoming_inp1_countm3_s;

latched_incoming_inp1_countm1_s<= incoming_inp1_countm1_s;

latched_incoming_qp1_countp7_s<= incoming_qp1_countp7_s;

latched_incoming_qp1_countp5_s<= incoming_qp1_countp5_s;

latched_incoming_qp1_countp3_s<= incoming_qp1_countp3_s;

latched_incoming_qp1_countp1_s<= incoming_qp1_countp1_s;

latched_incoming_qp1_countm7_s<= incoming_qp1_countm7_s;

latched_incoming_qp1_countm5_s<= incoming_qp1_countm5_s;

latched_incoming_qp1_countm3_s<= incoming_qp1_countm3_s;

latched_incoming_qp1_countm1_s<= incoming_qp1_countm1_s;

incoming_inp1_countp7_s<= (OTHERS=>'0');

incoming_inp1_countp5_s<= (OTHERS=>'0');

incoming_inp1_countp3_s<= (OTHERS=>'0');

incoming_inp1_countp1_s<= (OTHERS=>'0');

incoming_inp1_countm7_s<= (OTHERS=>'0');

incoming_inp1_countm5_s<= (OTHERS=>'0');

incoming_inp1_countm3_s<= (OTHERS=>'0');

incoming_inp1_countm1_s<= (OTHERS=>'0');

incoming_qp1_countp7_s<= (OTHERS=>'0');

incoming_qp1_countp5_s<= (OTHERS=>'0');

incoming_qp1_countp3_s<= (OTHERS=>'0');

incoming_qp1_countp1_s<= (OTHERS=>'0');

incoming_qp1_countm7_s<= (OTHERS=>'0');

incoming_qp1_countm5_s<= (OTHERS=>'0');

incoming_qp1_countm3_s<= (OTHERS=>'0');

incoming_qp1_countm1_s<= (OTHERS=>'0');

latched_incoming_inp2_countp7_s<= incoming_inp2_countp7_s;

latched_incoming_inp2_countp5_s<= incoming_inp2_countp5_s;

latched_incoming_inp2_countp3_s<= incoming_inp2_countp3_s;

latched_incoming_inp2_countp1_s<= incoming_inp2_countp1_s;

latched_incoming_inp2_countm7_s<= incoming_inp2_countm7_s;

latched_incoming_inp2_countm5_s<= incoming_inp2_countm5_s;

latched_incoming_inp2_countm3_s<= incoming_inp2_countm3_s;

latched_incoming_inp2_countm1_s<= incoming_inp2_countm1_s;

latched_incoming_qp2_countp7_s<= incoming_qp2_countp7_s;

latched_incoming_qp2_countp5_s<= incoming_qp2_countp5_s;

latched_incoming_qp2_countp3_s<= incoming_qp2_countp3_s;

latched_incoming_qp2_countp1_s<= incoming_qp2_countp1_s;

latched_incoming_qp2_countm7_s<= incoming_qp2_countm7_s;

latched_incoming_qp2_countm5_s<= incoming_qp2_countm5_s;

latched_incoming_qp2_countm3_s<= incoming_qp2_countm3_s;

latched_incoming_qp2_countm1_s<= incoming_qp2_countm1_s;

incoming_inp2_countp7_s<= (OTHERS=>'0');

incoming_inp2_countp5_s<= (OTHERS=>'0');

incoming_inp2_countp3_s<= (OTHERS=>'0');

incoming_inp2_countp1_s<= (OTHERS=>'0');

incoming_inp2_countm7_s<= (OTHERS=>'0');

incoming_inp2_countm5_s<= (OTHERS=>'0');

incoming_inp2_countm3_s<= (OTHERS=>'0');

incoming_inp2_countm1_s<= (OTHERS=>'0');

incoming_qp2_countp7_s<= (OTHERS=>'0');

incoming_qp2_countp5_s<= (OTHERS=>'0');

incoming_qp2_countp3_s<= (OTHERS=>'0');

incoming_qp2_countp1_s<= (OTHERS=>'0');

incoming_qp2_countm7_s<= (OTHERS=>'0');

incoming_qp2_countm5_s<= (OTHERS=>'0');

incoming_qp2_countm3_s<= (OTHERS=>'0');

incoming_qp2_countm1_s<= (OTHERS=>'0');

latched_incoming_inp3_countp7_s<= incoming_inp3_countp7_s;

latched_incoming_inp3_countp5_s<= incoming_inp3_countp5_s;

latched_incoming_inp3_countp3_s<= incoming_inp3_countp3_s;

latched_incoming_inp3_countp1_s<= incoming_inp3_countp1_s;

latched_incoming_inp3_countm7_s<= incoming_inp3_countm7_s;

latched_incoming_inp3_countm5_s<= incoming_inp3_countm5_s;

latched_incoming_inp3_countm3_s<= incoming_inp3_countm3_s;

latched_incoming_inp3_countm1_s<= incoming_inp3_countm1_s;

latched_incoming_qp3_countp7_s<= incoming_qp3_countp7_s;

latched_incoming_qp3_countp5_s<= incoming_qp3_countp5_s;

latched_incoming_qp3_countp3_s<= incoming_qp3_countp3_s;

latched_incoming_qp3_countp1_s<= incoming_qp3_countp1_s;

latched_incoming_qp3_countm7_s<= incoming_qp3_countm7_s;

latched_incoming_qp3_countm5_s<= incoming_qp3_countm5_s;

latched_incoming_qp3_countm3_s<= incoming_qp3_countm3_s;

latched_incoming_qp3_countm1_s<= incoming_qp3_countm1_s;

incoming_inp3_countp7_s<= (OTHERS=>'0');

incoming_inp3_countp5_s<= (OTHERS=>'0');

incoming_inp3_countp3_s<= (OTHERS=>'0');

incoming_inp3_countp1_s<= (OTHERS=>'0');

incoming_inp3_countm7_s<= (OTHERS=>'0');

incoming_inp3_countm5_s<= (OTHERS=>'0');

incoming_inp3_countm3_s<= (OTHERS=>'0');

incoming_inp3_countm1_s<= (OTHERS=>'0');

incoming_qp3_countp7_s<= (OTHERS=>'0');

incoming_qp3_countp5_s<= (OTHERS=>'0');

incoming_qp3_countp3_s<= (OTHERS=>'0');

incoming_qp3_countp1_s<= (OTHERS=>'0');

incoming_qp3_countm7_s<= (OTHERS=>'0');

incoming_qp3_countm5_s<= (OTHERS=>'0');

incoming_qp3_countm3_s<= (OTHERS=>'0');

incoming_qp3_countm1_s<= (OTHERS=>'0');

latched_incoming_inp4_countp7_s<= incoming_inp4_countp7_s;

latched_incoming_inp4_countp5_s<= incoming_inp4_countp5_s;

latched_incoming_inp4_countp3_s<= incoming_inp4_countp3_s;

latched_incoming_inp4_countp1_s<= incoming_inp4_countp1_s;

latched_incoming_inp4_countm7_s<= incoming_inp4_countm7_s;

latched_incoming_inp4_countm5_s<= incoming_inp4_countm5_s;

latched_incoming_inp4_countm3_s<= incoming_inp4_countm3_s;

latched_incoming_inp4_countm1_s<= incoming_inp4_countm1_s;

latched_incoming_qp4_countp7_s<= incoming_qp4_countp7_s;

latched_incoming_qp4_countp5_s<= incoming_qp4_countp5_s;

latched_incoming_qp4_countp3_s<= incoming_qp4_countp3_s;

latched_incoming_qp4_countp1_s<= incoming_qp4_countp1_s;

latched_incoming_qp4_countm7_s<= incoming_qp4_countm7_s;

latched_incoming_qp4_countm5_s<= incoming_qp4_countm5_s;

latched_incoming_qp4_countm3_s<= incoming_qp4_countm3_s;

latched_incoming_qp4_countm1_s<= incoming_qp4_countm1_s;

incoming_inp4_countp7_s<= (OTHERS=>'0');

incoming_inp4_countp5_s<= (OTHERS=>'0');

incoming_inp4_countp3_s<= (OTHERS=>'0');

incoming_inp4_countp1_s<= (OTHERS=>'0');

incoming_inp4_countm7_s<= (OTHERS=>'0');

incoming_inp4_countm5_s<= (OTHERS=>'0');

incoming_inp4_countm3_s<= (OTHERS=>'0');

incoming_inp4_countm1_s<= (OTHERS=>'0');

incoming_qp4_countp7_s<= (OTHERS=>'0');

incoming_qp4_countp5_s<= (OTHERS=>'0');

incoming_qp4_countp3_s<= (OTHERS=>'0');

incoming_qp4_countp1_s<= (OTHERS=>'0');

incoming_qp4_countm7_s<= (OTHERS=>'0');

incoming_qp4_countm5_s<= (OTHERS=>'0');

incoming_qp4_countm3_s<= (OTHERS=>'0');

incoming_qp4_countm1_s<= (OTHERS=>'0');

else

if(ant1_data_is(5 downto 3) = "000" ) then

incoming_inp1_countp7_s <= incoming_inp1_countp7_s + '1';

elsif(ant1_data_is(5 downto 3) = "001" ) then

incoming_inp1_countp5_s <= incoming_inp1_countp5_s + '1';

elsif(ant1_data_is(5 downto 3) = "010" ) then

incoming_inp1_countp3_s <= incoming_inp1_countp3_s + '1';

elsif(ant1_data_is(5 downto 3) = "011" ) then

incoming_inp1_countp1_s <= incoming_inp1_countp1_s + '1';

elsif(ant1_data_is(5 downto 3) = "100" ) then

incoming_inp1_countm1_s <= incoming_inp1_countm1_s + '1';

elsif(ant1_data_is(5 downto 3) = "101" ) then

incoming_inp1_countm3_s <= incoming_inp1_countm3_s + '1';

elsif(ant1_data_is(5 downto 3) = "110" ) then

incoming_inp1_countm5_s <= incoming_inp1_countm5_s + '1';

else

incoming_inp1_countm7_s <= incoming_inp1_countm7_s + '1';

end if;

if(ant1_data_is(2 downto 0)  = "000" ) then

incoming_qp1_countp7_s <= incoming_qp1_countp7_s + '1';

elsif(ant1_data_is(2 downto 0)  = "001"  ) then

incoming_qp1_countp5_s <= incoming_qp1_countp5_s + '1';

elsif(ant1_data_is(2 downto 0)  = "010"  ) then

incoming_qp1_countp3_s <= incoming_qp1_countp3_s + '1';

elsif(ant1_data_is(2 downto 0)  = "011"  ) then

incoming_qp1_countp1_s <= incoming_qp1_countp1_s + '1';

elsif(ant1_data_is(2 downto 0)  = "100"  ) then

incoming_qp1_countm1_s <= incoming_qp1_countm1_s + '1';

elsif(ant1_data_is(2 downto 0)  = "101"  ) then

incoming_qp1_countm3_s <= incoming_qp1_countm3_s + '1';

elsif(ant1_data_is(2 downto 0)  = "110"  ) then

incoming_qp1_countm5_s <= incoming_qp1_countm5_s + '1';

else

incoming_qp1_countm7_s <= incoming_qp1_countm7_s + '1';

end if;

if(ant2_data_is(5 downto 3) = "000" ) then

incoming_inp2_countp7_s <= incoming_inp2_countp7_s + '1';

elsif(ant2_data_is(5 downto 3) = "001" ) then

incoming_inp2_countp5_s <= incoming_inp2_countp5_s + '1';

elsif(ant2_data_is(5 downto 3) = "010" ) then

incoming_inp2_countp3_s <= incoming_inp2_countp3_s + '1';

elsif(ant2_data_is(5 downto 3) = "011" ) then

incoming_inp2_countp1_s <= incoming_inp2_countp1_s + '1';

elsif(ant2_data_is(5 downto 3) = "100" ) then

incoming_inp2_countm1_s <= incoming_inp2_countm1_s + '1';

elsif(ant2_data_is(5 downto 3) = "101" ) then

incoming_inp2_countm3_s <= incoming_inp2_countm3_s + '1';

elsif(ant2_data_is(5 downto 3) = "110" ) then

incoming_inp2_countm5_s <= incoming_inp2_countm5_s + '1';

else

incoming_inp2_countm7_s <= incoming_inp2_countm7_s + '1';

end if;

if(ant2_data_is(2 downto 0)  = "000" ) then

incoming_qp2_countp7_s <= incoming_qp2_countp7_s + '1';

elsif(ant2_data_is(2 downto 0)  = "001"  ) then

incoming_qp2_countp5_s <= incoming_qp2_countp5_s + '1';

elsif(ant2_data_is(2 downto 0)  = "010"  ) then

incoming_qp2_countp3_s <= incoming_qp2_countp3_s + '1';

elsif(ant2_data_is(2 downto 0)  = "011"  ) then

incoming_qp2_countp1_s <= incoming_qp2_countp1_s + '1';

elsif(ant2_data_is(2 downto 0)  = "100"  ) then

incoming_qp2_countm1_s <= incoming_qp2_countm1_s + '1';

elsif(ant2_data_is(2 downto 0)  = "101"  ) then

incoming_qp2_countm3_s <= incoming_qp2_countm3_s + '1';

elsif(ant2_data_is(2 downto 0)  = "110"  ) then

incoming_qp2_countm5_s <= incoming_qp2_countm5_s + '1';

else

incoming_qp2_countm7_s <= incoming_qp2_countm7_s + '1';

end if;

if(ant1_data_is(5 downto 3) = "000" ) then

incoming_inp3_countp7_s <= incoming_inp3_countp7_s + '1';

elsif(ant1_data_is(5 downto 3) = "001" ) then

incoming_inp3_countp5_s <= incoming_inp3_countp5_s + '1';

elsif(ant1_data_is(5 downto 3) = "010" ) then

incoming_inp3_countp3_s <= incoming_inp3_countp3_s + '1';

elsif(ant1_data_is(5 downto 3) = "011" ) then

incoming_inp3_countp1_s <= incoming_inp3_countp1_s + '1';

elsif(ant1_data_is(5 downto 3) = "100" ) then

incoming_inp3_countm1_s <= incoming_inp3_countm1_s + '1';

elsif(ant1_data_is(5 downto 3) = "101" ) then

incoming_inp3_countm3_s <= incoming_inp3_countm3_s + '1';

elsif(ant1_data_is(5 downto 3) = "110" ) then

incoming_inp3_countm5_s <= incoming_inp3_countm5_s + '1';

else

incoming_inp3_countm7_s <= incoming_inp3_countm7_s + '1';

end if;

if(ant1_data_is(2 downto 0)  = "000" ) then

incoming_qp3_countp7_s <= incoming_qp3_countp7_s + '1';

elsif(ant1_data_is(2 downto 0)  = "001"  ) then

incoming_qp3_countp5_s <= incoming_qp3_countp5_s + '1';

elsif(ant1_data_is(2 downto 0)  = "010"  ) then

incoming_qp3_countp3_s <= incoming_qp3_countp3_s + '1';

elsif(ant1_data_is(2 downto 0)  = "011"  ) then

incoming_qp3_countp1_s <= incoming_qp3_countp1_s + '1';

elsif(ant1_data_is(2 downto 0)  = "100"  ) then

incoming_qp3_countm1_s <= incoming_qp3_countm1_s + '1';

elsif(ant1_data_is(2 downto 0)  = "101"  ) then

incoming_qp3_countm3_s <= incoming_qp3_countm3_s + '1';

elsif(ant1_data_is(2 downto 0)  = "110"  ) then

incoming_qp3_countm5_s <= incoming_qp3_countm5_s + '1';

else

incoming_qp3_countm7_s <= incoming_qp3_countm7_s + '1';

end if;

if(ant3_data_is(5 downto 3) = "000" ) then

incoming_inp4_countp7_s <= incoming_inp4_countp7_s + '1';

elsif(ant3_data_is(5 downto 3) = "001" ) then

incoming_inp4_countp5_s <= incoming_inp4_countp5_s + '1';

elsif(ant3_data_is(5 downto 3) = "010" ) then

incoming_inp4_countp3_s <= incoming_inp4_countp3_s + '1';

elsif(ant3_data_is(5 downto 3) = "011" ) then

incoming_inp4_countp1_s <= incoming_inp4_countp1_s + '1';

elsif(ant3_data_is(5 downto 3) = "100" ) then

incoming_inp4_countm1_s <= incoming_inp4_countm1_s + '1';

elsif(ant3_data_is(5 downto 3) = "101" ) then

incoming_inp4_countm3_s <= incoming_inp4_countm3_s + '1';

elsif(ant3_data_is(5 downto 3) = "110" ) then

incoming_inp4_countm5_s <= incoming_inp4_countm5_s + '1';

else

incoming_inp4_countm7_s <= incoming_inp4_countm7_s + '1';

end if;

if(ant4_data_is(2 downto 0)  = "000" ) then

incoming_qp4_countp7_s <= incoming_qp4_countp7_s + '1';

elsif(ant4_data_is(2 downto 0)  = "001"  ) then

incoming_qp4_countp5_s <= incoming_qp4_countp5_s + '1';

elsif(ant4_data_is(2 downto 0)  = "010"  ) then

incoming_qp4_countp3_s <= incoming_qp4_countp3_s + '1';

elsif(ant4_data_is(2 downto 0)  = "011"  ) then

incoming_qp4_countp1_s <= incoming_qp4_countp1_s + '1';

elsif(ant4_data_is(2 downto 0)  = "100"  ) then

incoming_qp4_countm1_s <= incoming_qp4_countm1_s + '1';

elsif(ant4_data_is(2 downto 0)  = "101"  ) then

incoming_qp4_countm3_s <= incoming_qp4_countm3_s + '1';

elsif(ant4_data_is(2 downto 0)  = "110"  ) then

incoming_qp4_countm5_s <= incoming_qp4_countm5_s + '1';

else

incoming_qp4_countm7_s <= incoming_qp4_countm7_s + '1';

end if;

end if;

end if;

end process ADC_cap;

epoch_latch_pulse_s <= epoch_latch_pulse_temp_s;

data_to_processor_os <= data_to_processor_old_s when up_address_is(11) = '0' else

"00000000000000000000000000000000";

upconv1_spi_reg_os <= upconv1_spi_reg_s;

upconv2_spi_reg_os <= upconv2_spi_reg_s;

P_pps_ten_ms_gen: process(clk_is, POR_G_is)

begin

if POR_G_is = '1' then

pps_on_off_s <= '0';

elsif clk_is'event and clk_is = '1' then

if pps_on_off_sel_s = '1' then

pps_on_off_s <= data_processor_is(0);

end if;

end if;

if POR_G_is = '1' then

ten_ms_counter1_18_prev_s <= '0';

elsif clk_is'event and clk_is = '1' then

ten_ms_counter1_18_prev_s <= ten_ms_counter1_s(18);

end if;

if POR_G_is = '1' then

count_enable1_s <= '0' ;

clear_ten_ms_counter1_ceil_s <= '0';

elsif clk_is'event and clk_is = '1' then

if ten_ms_counter1_s = "1100001101010000000" then

clear_ten_ms_counter1_ceil_s <= '1';

else

clear_ten_ms_counter1_ceil_s <= '0';

end if;

if set_ce1_s = '1' then

count_enable1_s <= '1';

elsif clr_ce1_s = '1' then

count_enable1_s <= '0';

end if;

end if;

end process P_pps_ten_ms_gen;

P_ten_ms_counter_SPI: process(clk_is,POR_G_is)

begin

if POR_G_is = '1' then

ten_ms_counter1_s <= (others => '0');

elsif clk_is'event and clk_is = '1' then

if clear_ten_ms_counter1_ceil_s = '1' then

ten_ms_counter1_s <= (others => '0');

else

ten_ms_counter1_s <= ten_ms_counter1_s + 1;

end if;

end if;

if POR_G_is = '1' then

upconv1_spi_reg_s <= (others => '0');

prev_upconv1_sclk_s <=  '0';

elsif clk_is'event and clk_is = '1' then

if upconv1_spi_reg_wr_sel_s = '1' then

upconv1_spi_reg_s <= data_processor_is(4 downto 0);

end if;

prev_upconv1_sclk_s <= upconv1_spi_reg_s(3);

end if;

if POR_G_is = '1' then

upconv2_spi_reg_s <= (others => '0');

prev_upconv2_sclk_s <= '0';

elsif clk_is'event and clk_is = '1' then

if upconv2_spi_reg_wr_sel_s = '1' then

upconv2_spi_reg_s <= data_processor_is(4 downto 0);

end if;

prev_upconv2_sclk_s <= upconv2_spi_reg_s(3);

end if;

if POR_G_is = '1' then

upconv1_spi_gpio4_out_s <= '0';

upconv2_spi_gpio4_out_s <= '0';

elsif clk_is'event and clk_is = '1' then

if(upconv1_spi_reg_s(3) = '0') and (prev_upconv1_sclk_s = '1') then

upconv1_spi_gpio4_out_s <= upconv1_spi_gpio4_is;

end if;

if(upconv2_spi_reg_s(3) = '0') and (prev_upconv2_sclk_s = '1') then

upconv2_spi_gpio4_out_s <= upconv2_spi_gpio4_is;

end if;

end if;

end process P_ten_ms_counter_SPI;

ms_10_interrupt_os <= count_enable1_s;

set_ce1_s <= '1' when ten_ms_counter1_s(18) = '0' and ten_ms_counter1_18_prev_s = '1' else

'0';

clr_ce1_s <= '1' when ten_ms_counter1_s(18) = '1' else

'1' when ten_ms_reg_s='1' else

'0';

ms_500_module_e_inst: ms_500_module_e

port map(clk_is => clk_is,

POR_G_is => POR_G_is,

--		  clk_enable_is => clk_enable_s,

data_processor_is => data_processor_is,

TIC_intrpt_clear_is=> TIC_intrpt_clear_s,

TIC_wr_enable_is => TIC_wr_enable_s,

clear_TIC_counter_is => clear_TIC_counter_s,

TIC_drift_wr_enable_is => TIC_drift_wr_enable_s,

epoch_latch_pulse_os => epoch_latch_pulse_temp_s,

ms_500_interrupt_os => ms_500_interrupt_os,

--RT1

TSM_interrupt_RT1_is => TSM_interrupt_RT1_is,

reset_pps_interrupt_RT1_is => reset_pps_interrupt_RT1_s,

TSM_flag_RT1_out_os => TSM_flag_out_RT1_os,

PPS_flag_RT1_out_os => PPS_flag_out_RT1_os,

TSM_data_RT1_os => TSM_data_RT1_s,

--RT2

TSM_interrupt_RT2_is => TSM_interrupt_RT2_is,

reset_pps_interrupt_RT2_is => reset_pps_interrupt_RT2_s,

TSM_flag_RT2_out_os => TSM_flag_out_RT2_os,

PPS_flag_RT2_out_os => PPS_flag_out_RT2_os,

TSM_data_RT2_os => TSM_data_RT2_s,

--RT3

TSM_interrupt_RT3_is => TSM_interrupt_RT3_is,

reset_pps_interrupt_RT3_is => reset_pps_interrupt_RT3_s,

TSM_flag_RT3_out_os => TSM_flag_out_RT3_os,

PPS_flag_RT3_out_os => PPS_flag_out_RT3_os,

TSM_data_RT3_os => TSM_data_RT3_s,

pps_pulse_count_data_en_is => pps_pulse_count_data_en_s,

pps_on_off_is => pps_on_off_s,

pps_signal_os => pps_signal_os);

discrm :  top_discriminator_e

port map (

clk_is => clk_is,

POR_is => POR_G_is,

p_i_cndr_is => p_cndr_out_ch_s,

p_q_cndr_is => q_cndr_out_ch_s,			p_i_is => p_i_ch_s,

p_q_is => p_q_ch_s,

e_i_is => e_i_ch_s,

e_q_is => e_q_ch_s,

l_i_is => l_i_ch_s,

l_q_is => l_q_ch_s,

lock_counter_enable_os => lock_status_in_ds_s,

channel_par_load_output_os => channel_par_load_ds_s,

car_corr_freq_word_out_os => car_corr_freq_in_ds_s,

code_corr_freq_word_out_os => code_corr_freq_in_ds_s,	Prev_phase_error_input_is => Prev_phase_error_ch_s,

Order1_input_is => Order1_out_ch_s,

Order2_input_is => Order2_out_ch_s,

Order3_input_is  => Order3_out_ch_s,

DLL_Order1_input_is =>  DLL_Order1_out_ch_s,

DLL_Order2_input_is =>  DLL_Order2_out_ch_s,

Prev_phase_error_output_os => Prev_phase_error_ds_s,

Order1_output_os => Order1_out_ds_s,

Order2_output_os => Order2_out_ds_s,

Order3_output_os  => Order3_out_ds_s,

DLL_Order1_output_os => DLL_Order1_out_ds_s,

DLL_Order2_output_os => DLL_Order2_out_ds_s,

loop_start_input_is => loop_start_ch_s,

stage_control_is => stage_control_s,

fll_zero_is => fll_zero_s,

NAVIC_indicator_is => NAVIC_indicator_s,

Data_boundary_input_is => Data_boundary_input_s,

fll_data_check_input_is  => fll_data_check_ch_s,

track_th_sel_is  => track_th_sel_s,

up_data_is => data_processor_is ,

pll_par1_set1_launch_enable_is => pll_par1_set1_launch_enable_s,

pll_par2_set1_launch_enable_is => pll_par2_set1_launch_enable_s,

pll_par3_set1_launch_enable_is => pll_par3_set1_launch_enable_s,

pll_par4_set1_launch_enable_is => pll_par4_set1_launch_enable_s,

fll_par1_set1_launch_enable_is => fll_par1_set1_launch_enable_s,

fll_par2_set1_launch_enable_is => fll_par2_set1_launch_enable_s,

fll_par3_set1_launch_enable_is => fll_par3_set1_launch_enable_s,

dll_par1_set1_launch_enable_is => dll_par1_set1_launch_enable_s,

dll_par2_set1_launch_enable_is => dll_par2_set1_launch_enable_s,

dll_par3_set1_launch_enable_is => dll_par3_set1_launch_enable_s,

pll_par1_set2_launch_enable_is => pll_par1_set2_launch_enable_s,

pll_par2_set2_launch_enable_is => pll_par2_set2_launch_enable_s,

pll_par3_set2_launch_enable_is => pll_par3_set2_launch_enable_s,

pll_par4_set2_launch_enable_is => pll_par4_set2_launch_enable_s,

fll_par1_set2_launch_enable_is => fll_par1_set2_launch_enable_s,

fll_par2_set2_launch_enable_is => fll_par2_set2_launch_enable_s,

fll_par3_set2_launch_enable_is => fll_par3_set2_launch_enable_s,

dll_par1_set2_launch_enable_is => dll_par1_set2_launch_enable_s,

dll_par2_set2_launch_enable_is => dll_par2_set2_launch_enable_s,

dll_par3_set2_launch_enable_is => dll_par3_set2_launch_enable_s,

pll_par1_set3_launch_enable_is => pll_par1_set3_launch_enable_s,

pll_par2_set3_launch_enable_is => pll_par2_set3_launch_enable_s,

pll_par3_set3_launch_enable_is => pll_par3_set3_launch_enable_s,

pll_par4_set3_launch_enable_is => pll_par4_set3_launch_enable_s,

fll_par1_set3_launch_enable_is => fll_par1_set3_launch_enable_s,

fll_par2_set3_launch_enable_is => fll_par2_set3_launch_enable_s,

fll_par3_set3_launch_enable_is => fll_par3_set3_launch_enable_s,

dll_par1_set3_launch_enable_is => dll_par1_set3_launch_enable_s,

dll_par2_set3_launch_enable_is => dll_par2_set3_launch_enable_s,

dll_par3_set3_launch_enable_is => dll_par3_set3_launch_enable_s,

pll_par1_set4_launch_enable_is => pll_par1_set4_launch_enable_s,

pll_par2_set4_launch_enable_is => pll_par2_set4_launch_enable_s,

pll_par3_set4_launch_enable_is => pll_par3_set4_launch_enable_s,

pll_par4_set4_launch_enable_is => pll_par4_set4_launch_enable_s,

fll_par1_set4_launch_enable_is => fll_par1_set4_launch_enable_s,

fll_par2_set4_launch_enable_is => fll_par2_set4_launch_enable_s,

fll_par3_set4_launch_enable_is => fll_par3_set4_launch_enable_s,

dll_par1_set4_launch_enable_is => dll_par1_set4_launch_enable_s,

dll_par2_set4_launch_enable_is => dll_par2_set4_launch_enable_s,

dll_par3_set4_launch_enable_is => dll_par3_set4_launch_enable_s,

pll_par1_set5_launch_enable_is => pll_par1_set5_launch_enable_s,

pll_par2_set5_launch_enable_is => pll_par2_set5_launch_enable_s,

pll_par3_set5_launch_enable_is => pll_par3_set5_launch_enable_s,

pll_par4_set5_launch_enable_is => pll_par4_set5_launch_enable_s,

fll_par1_set5_launch_enable_is => fll_par1_set5_launch_enable_s,

fll_par2_set5_launch_enable_is => fll_par2_set5_launch_enable_s,

fll_par3_set5_launch_enable_is => fll_par3_set5_launch_enable_s,

dll_par1_set5_launch_enable_is => dll_par1_set5_launch_enable_s,

dll_par2_set5_launch_enable_is => dll_par2_set5_launch_enable_s,

dll_par3_set5_launch_enable_is => dll_par3_set5_launch_enable_s,

cdll_pr1_set1_launch_enable_is	 => cdll_par1_set1_launch_enable_s,

carr_aided_loop_sel_is          => carr_aided_loop_sel_s,

fll_zero_en_dis_sel_is          => fll_zero_en_dis_sel_s,

loop_par_status1_enable_is      => loop_par_status1_enable_s,

IF_FREQ_TRK_GPS_sel_is    => IF_Freq_GPS_trk_sel_s,

IF_FREQ_TRK_NAVIC_sel_is    => IF_Freq_NAVIC_trk_sel_s,

trk_threshold_os                => trk_threshold_s,

carr_aided_loop_os              => carr_aided_loop_out_s,

fll_zero_en_dis_os              => fll_zero_en_dis_out_s

);

gen_channel: for i in 0 to channels_count_c-1 generate			trker: trk1_e

port map(clk_is => clk_is,

POR_is => POR_G_is,

start_is => start_channel_s(i),

--		samp_clk_enable_is => clk_enable_s,

channel_par_load_is => channel_par_load_ds_s(i),

sat_id_in_is => svid_channel_in_s,

ant1_ant2_sel_ch_is => ant1_ant2_sel_ch_s,

car_freq_init_is => car_doppler_ch_s,

code_freq_init_is => code_doppler_ch_s,			order1_loop_init_is => order1_loop_init_ch_s,

dll_order1_loop_init_is => dll_order1_loop_init_ch_s,

--Doppler_bin_in_is => Doppler_bin_in_is_s(i),

indata_ant1_is => ant1_data_is,

indata_ant2_is => ant2_data_is,

indata_ant3_is => ant3_data_is,

indata_ant4_is => ant4_data_is,

car_mix_ROM_ena_is => car_mix_ROM_ena_s,

car_mix_ROM_wea_is => car_mix_ROM_wea_s,

car_mix_ROM_addra_is => car_mix_ROM_addra_s,

car_mix_ROM_dina_is => car_mix_ROM_dina_s,

sat_id_out_os => svid_channel_ch_s(i),

p_i_os => p_i_ch_s(i) ,

p_q_os => p_q_ch_s(i) ,

e_i_os => e_i_ch_s(i) ,

e_q_os => e_q_ch_s(i),

l_i_os => l_i_ch_s(i) ,

l_q_os => l_q_ch_s(i),

lock_status_out_os => lock_status_out_s(i),

lock_status_in_is => lock_status_in_ds_s(i),   -- V2_change

loop_start_os => loop_start_ch_s(i),

stage_control_os => stage_control_s(i),

FLL_data_check_counter_is => FLL_data_check_counter_s,

fll_zero_os	 => fll_zero_s(i),

NAVIC_indicator_os => NAVIC_indicator_s(i),

Data_boundary_output_os => Data_boundary_input_s(i),

fll_data_check_os => fll_data_check_ch_s(i),

Prev_phase_error_input_is => Prev_phase_error_ds_s,

Order1_input_is => Order1_out_ds_s,

Order2_input_is => Order2_out_ds_s,

Order3_input_is => Order3_out_ds_s,

DLL_Order1_input_is => DLL_Order1_out_ds_s,

DLL_Order2_input_is => DLL_Order2_out_ds_s,

Prev_phase_error_os => Prev_phase_error_ch_s(i),

Order1_out_os => Order1_out_ch_s(i),

Order2_out_os => Order2_out_ch_s(i),

Order3_out_os => Order3_out_ch_s(i),

DLL_Order1_out_os => DLL_Order1_out_ch_s(i),

DLL_Order2_out_os => DLL_Order2_out_ch_s(i),

car_corr_freq_in_is => car_corr_freq_in_ds_s,

code_corr_freq_in_is => code_corr_freq_in_ds_s,

navdata_read_mp_is => nav_data_read_over_ch_s(i),

navdata_reg_os => nav_data_reg_ch_s(i),

p_cndr_out_os => p_cndr_out_ch_s(i),

q_cndr_out_os => q_cndr_out_ch_s(i),

car_cycle_count_os => car_cycle_count_ch_s(i),

car_phase_count_os => car_phase_count_ch_s(i),

one_ms_counter_value_os => one_ms_counter_value_ch_s(i),

twenty_ms_counter_value_os => twenty_ms_counter_value_ch_s(i),

code_cyle_count_value_os => code_cyle_count_value_ch_s(i),

code_phase_count_os => code_phase_count_ch_s(i),

-- code_nco_cycle_counters_os => code_nco_cycle_counters_os_s(i),

reg_wr_processor_tms_is => reg_wr_procr_tms_ch_s(i),

epoch_latch_pulse_is => epoch_latch_pulse_s,

set1_to_set2_timer_Launch_is => set1_to_set2_timer_launch_def_s,

set2_to_set3_timer_Launch_is => set2_to_set3_timer_launch_def_s,

set3_to_set4_timer_Launch_is => set3_to_set4_timer_launch_def_s,

set4_to_set5_timer_Launch_is => set4_to_set5_timer_launch_def_s,

Narrow_corr_step1_timer_is => Narrow_corr_step1_timer_def_s,

Narrow_corr_step2_timer_is => Narrow_corr_step2_timer_def_s,

Narrow_correlator_step1_enable_is => Narrow_correlator_step1_enable_s,

Narrow_correlator_step2_enable_is => Narrow_correlator_step2_enable_s,

fll_timer_is => fll_timer_def_s,

antenna_combining_control_is => antenna_combining_control_s,

lock_counter_threshold1_is => lock_counter_threshold1_s,

lock_counter_threshold2_is => lock_counter_threshold2_s,

reset_channel_en_pr_is => reset_channel_en_s(i)

);

end generate gen_channel;

sbas_trker: trk2_e

port map(clk_is => clk_is,

POR_is => POR_G_is,

start_is => start_channel_s(channels_count_c),

--samp_clk_enable_is => clk_enable_s,

channel_par_load_is => channel_par_load_ds_s(channels_count_c),

sat_id_in_is => svid_channel_in_s,

ant1_ant2_sel_ch_is => ant1_ant2_sel_ch_s,

car_freq_init_is => car_doppler_ch_s,

code_freq_init_is => code_doppler_ch_s,			order1_loop_init_is => order1_loop_init_ch_s,

dll_order1_loop_init_is => dll_order1_loop_init_ch_s,

--Doppler_bin_in_is => Doppler_bin_in_is_s(i),

indata_ant1_is => ant1_data_is,

indata_ant2_is => ant2_data_is,

indata_ant3_is => ant3_data_is,

indata_ant4_is => ant4_data_is,

car_mix_ROM_ena_is => car_mix_ROM_ena_s,

car_mix_ROM_wea_is => car_mix_ROM_wea_s,

car_mix_ROM_addra_is => car_mix_ROM_addra_s,

car_mix_ROM_dina_is => car_mix_ROM_dina_s,

sat_id_out_os => svid_channel_ch_s(channels_count_c),

p_i_os => p_i_ch_s(channels_count_c) ,

p_q_os => p_q_ch_s(channels_count_c) ,

e_i_os => e_i_ch_s(channels_count_c) ,

e_q_os => e_q_ch_s(channels_count_c),

l_i_os => l_i_ch_s(channels_count_c) ,

l_q_os => l_q_ch_s(channels_count_c),

lock_status_out_os => lock_status_out_s(channels_count_c),

lock_status_in_is => lock_status_in_ds_s(channels_count_c),   -- V2_change

loop_start_os => loop_start_ch_s(channels_count_c),

stage_control_os => stage_control_s(channels_count_c),

FLL_data_check_counter_is => FLL_data_check_counter_s,

fll_zero_os	 => fll_zero_s(channels_count_c),

NAVIC_indicator_os => NAVIC_indicator_s(channels_count_c),

Data_boundary_output_os => Data_boundary_input_s(channels_count_c),

fll_data_check_os => fll_data_check_ch_s(channels_count_c),

Prev_phase_error_input_is => Prev_phase_error_ds_s,

Order1_input_is => Order1_out_ds_s,

Order2_input_is => Order2_out_ds_s,

Order3_input_is => Order3_out_ds_s,

DLL_Order1_input_is => DLL_Order1_out_ds_s,

DLL_Order2_input_is => DLL_Order2_out_ds_s,

Prev_phase_error_os => Prev_phase_error_ch_s(channels_count_c),

Order1_out_os => Order1_out_ch_s(channels_count_c),

Order2_out_os => Order2_out_ch_s(channels_count_c),

Order3_out_os => Order3_out_ch_s(channels_count_c),

DLL_Order1_out_os => DLL_Order1_out_ch_s(channels_count_c),

DLL_Order2_out_os => DLL_Order2_out_ch_s(channels_count_c),

car_corr_freq_in_is => car_corr_freq_in_ds_s,

code_corr_freq_in_is => code_corr_freq_in_ds_s,

navdata_read_mp_is => nav_data_read_over_ch_s(channels_count_c),

navdata_reg_os => nav_data_reg_ch_s(channels_count_c),

p_cndr_out_os => p_cndr_out_ch_s(channels_count_c),

q_cndr_out_os => q_cndr_out_ch_s(channels_count_c),

car_cycle_count_os => car_cycle_count_ch_s(channels_count_c),

car_phase_count_os => car_phase_count_ch_s(channels_count_c),

one_ms_counter_value_os => one_ms_counter_value_ch_s(channels_count_c),

twenty_ms_counter_value_os => twenty_ms_counter_value_ch_s(channels_count_c),

code_cyle_count_value_os => code_cyle_count_value_ch_s(channels_count_c),

code_phase_count_os => code_phase_count_ch_s(channels_count_c),

-- code_nco_cycle_counters_os => code_nco_cycle_counters_os_s(i),

reg_wr_processor_tms_is => reg_wr_procr_tms_ch_s(channels_count_c),

epoch_latch_pulse_is => epoch_latch_pulse_s,

set1_to_set2_timer_Launch_is => set1_to_set2_timer_launch_def_s,

set2_to_set3_timer_Launch_is => set2_to_set3_timer_launch_def_s,

set3_to_set4_timer_Launch_is => set3_to_set4_timer_launch_def_s,

set4_to_set5_timer_Launch_is => set4_to_set5_timer_launch_def_s,

Narrow_corr_step1_timer_is => Narrow_corr_step1_timer_def_s,

Narrow_corr_step2_timer_is => Narrow_corr_step2_timer_def_s,

Narrow_correlator_step1_enable_is => Narrow_correlator_step1_enable_s,

Narrow_correlator_step2_enable_is => Narrow_correlator_step2_enable_s,

fll_timer_is => fll_timer_def_s,

antenna_combining_control_is => antenna_combining_control_s,

reset_channel_en_pr_is => reset_channel_en_s(channels_count_c),

lock_counter_threshold1_is => lock_counter_threshold1_s,

lock_counter_threshold2_is => lock_counter_threshold2_s,

sbas_enable_is  => sbas_enable_s,

sbas_navdata_os => sbas_navdata_s

);

total_channel_fsm_inst: total_ch_fsm_e port map

(clk_is => clk_is,

POR_is => POR_G_is,

channel_lock_status_is => lock_status_out_s,			 acq_interrupt_is => sat_pr_interrupt_s,

ant1_ant2_sel_is => ant1_ant2_sel_s,

svid_acq_is => svid_acq_s,

car_freq_init_is => car_freq_word_acq_s,

code_freq_init_is => code_freq_word_acq_s,

order1_loop_init_is => order1_loop_init_acq_s,

dll_order1_loop_init_is => dll_order1_loop_init_acq_s,

svid_channel_is => svid_channel_ch_s,

svid_channel_load_os => svid_channel_in_s,

car_freq_init_os => car_doppler_ch_s,

code_freq_init_os => code_doppler_ch_s,

order1_loop_init_os => order1_loop_init_ch_s,

dll_order1_loop_init_os => dll_order1_loop_init_ch_s,

ant1_ant2_sel_ch_os => ant1_ant2_sel_ch_s,

channel_start_os => start_channel_s,

acq_start_os => start_aquisition_s,

channel_status_os => channel_status_s,

sat_current_status_os => sat_status_s,

sbas_enable_is => sbas_enable_s,

navic_cmd_enable_is => navic_cmd_enable_s

);

EDAC_enc_ROM: edac_14_enc1_e

port map (

wr_en_is => '1', data_in_is => edac_data_input_scrub_s  ,data_out_os =>  edac_data_out_scrub_s ,par_out_os => edac_par_out_scrub_s );

total_acq_inst : Top_acquisition_e port map

(

Sys_CLK_40M_is => clk_is,

POR_RST_is => POR_G_is,

Incoming_data_ant1_is => ant1_data_is,

Incoming_data_ant2_is => ant2_data_is,

Incoming_data_ant3_is => ant3_data_is,

Incoming_data_ant4_is => ant4_data_is,

IF_Freq_GPS_sel_is  => IF_Freq_GPS_acq_sel_s,

IF_Freq_NAVIC_sel_is  => IF_Freq_NAVIC_acq_sel_s,

car_mix_ROM_ena_is => car_mix_ROM_ena_s,

car_mix_ROM_wea_is => car_mix_ROM_wea_s,

car_mix_ROM_addra_is => car_mix_ROM_addra_s,

car_mix_ROM_dina_is => car_mix_ROM_dina_s,

FPGA_Tracking_register1_is => sat_status_s(31 downto 0),

FPGA_Tracking_register2_is => sat_status_s(63 downto 32),

data_processor_is => data_processor_is,

Acquisition_threshold_LSW_sel_is => acq_th_LSW_sel_s,

Acquisition_threshold_MSW_sel_is => acq_th_MSW_sel_s,

alm_ant1_gps_reg_is => alm_ant1_gps_reg_s,

alm_ant2_gps_reg_is => alm_ant2_gps_reg_s,

alm_ant3_gps_reg_is => alm_ant3_gps_reg_s,

alm_ant4_gps_reg_is => alm_ant4_gps_reg_s,

alm_ant1_nav_reg_is => alm_ant1_nav_reg_s,

alm_ant2_nav_reg_is => alm_ant2_nav_reg_s,

alm_ant3_nav_reg_is => alm_ant3_nav_reg_s,

alm_ant4_nav_reg_is => alm_ant4_nav_reg_s,

alm_ant1_sbas_reg_is => alm_ant1_sbas_reg_s,

alm_ant2_sbas_reg_is => alm_ant2_sbas_reg_s,

alm_ant3_sbas_reg_is => alm_ant3_sbas_reg_s,

alm_ant4_sbas_reg_is => alm_ant4_sbas_reg_s,

SVID_loading_is => start_aquisition_s,

SVID_acquired_os => svid_acq_s,

SVID_Searched_os => svid_searched_s,

Antenna_output_os => ant1_ant2_sel_s,

Track_epoch_output_pulse_os => sat_pr_interrupt_s,

car_freq_init_os => car_freq_word_acq_s,

code_freq_init_os => code_freq_word_acq_s,

order1_loop_init_os => order1_loop_init_acq_s,

dll_order1_loop_init_os => dll_order1_loop_init_acq_s,

acq_threshold_LSW_os => acq_threshold_LSW_s,

acq_threshold_MSW_os => acq_threshold_MSW_s,

fft_status_os => fft_status_s,

acq1_search_reg_os => acq1_search_reg_s,

acq2_search_reg_os => acq2_search_reg_s,

acq3_search_reg_os => acq3_search_reg_s,

acq4_search_reg_os => acq4_search_reg_s );

wdt_logic_inst : wdt_logic_e

port map(clk_is => clk_is,

POR_G_is => POR_G_is,

POR_in_is => POR_in_is,

hwdt_cmd_is => hwdt_cmd_is,

swdt_cmd_is => swdt_cmd_is,

hwdt_reset_clear_pr_wr_enable_is => hwdt_reset_clear_pr_wr_enable_s,

up_data_is => data_processor_is(0),

wdt_enable_pr_wr_enable_is => wdt_enable_pr_wr_enable_s,

clear_wdt_pr_is => clear_wdt_pr_s,

reset_processor_os => reset_processor_os,

wdt_cmd_status_os => wdt_cmd_status_s,

hwdt_reset_status_ff_os => hwdt_reset_status_ff_s,

hwdt_reset_counter_os => hdwt_reset_counter_s,

PR_to_FPGA_reset_reg_sel_is => PR_to_FPGA_reset_reg_sel_s,

soft_reset_os => soft_reset_os,

soft_reset_flag_os => soft_reset_flag_s);

corr_reg_address_decoder_e_inst: corr_reg_address_decoder_e PORT MAP(

wr_pr_is => wr_pr_is,

cs_correlator_is => cs_correlator_is,

up_address_is => up_address_is,

upconv1_spi_reg_wr_sel_os => upconv1_spi_reg_wr_sel_s,

upconv2_spi_reg_wr_sel_os => upconv2_spi_reg_wr_sel_s,

PR_to_FPGA_reset_reg_sel_os => PR_to_FPGA_reset_reg_sel_s,

sbas_enable_reg_wr_sel_os => sbas_enable_reg_wr_sel_s,

navic_cmd_enable_reg_wr_sel_os => navic_cmd_enable_reg_wr_sel_s,

wdt_enable_pr_wr_enable_os => wdt_enable_pr_wr_enable_s,

clear_wdt_pr_os => clear_wdt_pr_s,

reset_tsm_interrupt_RT1_os => reset_tsm_interrupt_RT1_os,

reset_tsm_interrupt_RT2_os => reset_tsm_interrupt_RT2_os,

reset_tsm_interrupt_RT3_os => reset_tsm_interrupt_RT3_os,

reset_pps_interrupt_RT1_os => reset_pps_interrupt_RT1_s,

reset_pps_interrupt_RT2_os => reset_pps_interrupt_RT2_s,

reset_pps_interrupt_RT3_os => reset_pps_interrupt_RT3_s,

track_th_sel_os => track_th_sel_s,

hwdt_reset_clear_pr_wr_enable_os => hwdt_reset_clear_pr_wr_enable_s,

ten_ms_reg_os => ten_ms_reg_s,

TIC_intrpt_clear_os => TIC_intrpt_clear_s,

TIC_wr_enable_os =>  TIC_wr_enable_s,

clear_TIC_counter_os => clear_TIC_counter_s,

TIC_drift_wr_enable_os => TIC_drift_wr_enable_s,

ant1_GPS_wr_enable_os   => alm_ant1_gps_reg_s,

ant1_NavIC_wr_enable_os => alm_ant1_nav_reg_s,

ant1_SBAS_wr_enable_os  => alm_ant1_sbas_reg_s,

ant2_GPS_wr_enable_os   => alm_ant2_gps_reg_s,

ant2_NavIC_wr_enable_os => alm_ant2_nav_reg_s,

ant2_SBAS_wr_enable_os  => alm_ant2_sbas_reg_s,

ant3_GPS_wr_enable_os   => alm_ant3_gps_reg_s,

ant3_NavIC_wr_enable_os => alm_ant3_nav_reg_s,

ant3_SBAS_wr_enable_os  => alm_ant3_sbas_reg_s,

ant4_GPS_wr_enable_os   => alm_ant4_gps_reg_s,

ant4_NavIC_wr_enable_os => alm_ant4_nav_reg_s,

ant4_SBAS_wr_enable_os  => alm_ant4_sbas_reg_s,

reset_channel_en_os => reset_channel_en_s,

nav_data_read_over_ch_os => nav_data_read_over_ch_s,

reg_wr_procr_tms_ch_os => reg_wr_procr_tms_ch_s,

comparator_th_lsw_sel_os => acq_th_LSW_sel_s,	comparator_th_msw_sel_os => acq_th_MSW_sel_s,

pps_pulse_count_data_en_os => pps_pulse_count_data_en_s,

pps_on_off_sel_os => pps_on_off_sel_s,

pll_par1_set1_launch_enable_os => pll_par1_set1_launch_enable_s ,

pll_par2_set1_launch_enable_os => pll_par2_set1_launch_enable_s,

pll_par3_set1_launch_enable_os => pll_par3_set1_launch_enable_s ,

pll_par4_set1_launch_enable_os => pll_par4_set1_launch_enable_s,

fll_par1_set1_launch_enable_os => fll_par1_set1_launch_enable_s,

fll_par2_set1_launch_enable_os => fll_par2_set1_launch_enable_s,

fll_par3_set1_launch_enable_os => fll_par3_set1_launch_enable_s,

dll_par1_set1_launch_enable_os => dll_par1_set1_launch_enable_s,

dll_par2_set1_launch_enable_os => dll_par2_set1_launch_enable_s,

dll_par3_set1_launch_enable_os => dll_par3_set1_launch_enable_s,

pll_par1_set2_launch_enable_os => pll_par1_set2_launch_enable_s,

pll_par2_set2_launch_enable_os => pll_par2_set2_launch_enable_s,

pll_par3_set2_launch_enable_os => pll_par3_set2_launch_enable_s,

pll_par4_set2_launch_enable_os => pll_par4_set2_launch_enable_s,

fll_par1_set2_launch_enable_os => fll_par1_set2_launch_enable_s,

fll_par2_set2_launch_enable_os => fll_par2_set2_launch_enable_s,

fll_par3_set2_launch_enable_os => fll_par3_set2_launch_enable_s,

dll_par1_set2_launch_enable_os => dll_par1_set2_launch_enable_s,

dll_par2_set2_launch_enable_os => dll_par2_set2_launch_enable_s,

dll_par3_set2_launch_enable_os => dll_par3_set2_launch_enable_s,

pll_par1_set3_launch_enable_os => pll_par1_set3_launch_enable_s,

pll_par2_set3_launch_enable_os => pll_par2_set3_launch_enable_s,

pll_par3_set3_launch_enable_os => pll_par3_set3_launch_enable_s,

pll_par4_set3_launch_enable_os => pll_par4_set3_launch_enable_s,

fll_par1_set3_launch_enable_os => fll_par1_set3_launch_enable_s,

fll_par2_set3_launch_enable_os => fll_par2_set3_launch_enable_s,

fll_par3_set3_launch_enable_os => fll_par3_set3_launch_enable_s,

dll_par1_set3_launch_enable_os => dll_par1_set3_launch_enable_s,

dll_par2_set3_launch_enable_os => dll_par2_set3_launch_enable_s,

dll_par3_set3_launch_enable_os => dll_par3_set3_launch_enable_s,

pll_par1_set4_launch_enable_os => pll_par1_set4_launch_enable_s,

pll_par2_set4_launch_enable_os => pll_par2_set4_launch_enable_s,

pll_par3_set4_launch_enable_os => pll_par3_set4_launch_enable_s,

pll_par4_set4_launch_enable_os => pll_par4_set4_launch_enable_s,

fll_par1_set4_launch_enable_os => fll_par1_set4_launch_enable_s,

fll_par2_set4_launch_enable_os => fll_par2_set4_launch_enable_s,

fll_par3_set4_launch_enable_os => fll_par3_set4_launch_enable_s,

dll_par1_set4_launch_enable_os => dll_par1_set4_launch_enable_s,

dll_par2_set4_launch_enable_os => dll_par2_set4_launch_enable_s,

dll_par3_set4_launch_enable_os => dll_par3_set4_launch_enable_s,

pll_par1_set5_launch_enable_os => pll_par1_set5_launch_enable_s,

pll_par2_set5_launch_enable_os => pll_par2_set5_launch_enable_s,

pll_par3_set5_launch_enable_os => pll_par3_set5_launch_enable_s,

pll_par4_set5_launch_enable_os => pll_par4_set5_launch_enable_s,

fll_par1_set5_launch_enable_os => fll_par1_set5_launch_enable_s,

fll_par2_set5_launch_enable_os => fll_par2_set5_launch_enable_s,

fll_par3_set5_launch_enable_os => fll_par3_set5_launch_enable_s,

dll_par1_set5_launch_enable_os => dll_par1_set5_launch_enable_s,

dll_par2_set5_launch_enable_os => dll_par2_set5_launch_enable_s,

dll_par3_set5_launch_enable_os => dll_par3_set5_launch_enable_s,

cdll_par1_set1_launch_enable_os => cdll_par1_set1_launch_enable_s,

launch_orbit_phase_sel_os      => launch_orbit_phase_sel_s ,

carr_aided_loop_sel_os         => carr_aided_loop_sel_s ,

set1_to_set2_timer_launch_enable_os   => set1_to_set2_timer_launch_enable_s ,

set2_to_set3_timer_launch_enable_os   => set2_to_set3_timer_launch_enable_s,

set3_to_set4_timer_launch_enable_os   => set3_to_set4_timer_launch_enable_s ,

set4_to_set5_timer_launch_enable_os   => set4_to_set5_timer_launch_enable_s ,

Narrow_corr_step1_timer_sel_os =>	Narrow_corr_step1_timer_sel_s  ,

Narrow_corr_step2_timer_sel_os => Narrow_corr_step2_timer_sel_s ,

Narrow_correlator_step1_enable_sel_os => Narrow_correlator_step1_enable_sel_s  ,

Narrow_correlator_step2_enable_sel_os => Narrow_correlator_step2_enable_sel_s  ,

fll_timer_enable_os             => fll_timer_enable_s ,

fll_zero_en_dis_sel_os          => fll_zero_en_dis_sel_s,

loop_par_status1_enable_os      => loop_par_status1_enable_s ,

All_Timers_updated_enable_os    => All_Timers_updated_enable_s,

lock_detector_threshold_sel_os => lock_detector_threshold_sel_s,

Antenna_combining_sel_enable_os => Antenna_combining_sel_enable_s,

FLL_data_check_counter_enable_os => FLL_data_check_counter_enable_s,

reset_mil_interrupt_RT1_os => reset_mil_interrupt_RT1_os,

reset_mil_interrupt_RT2_os => reset_mil_interrupt_RT2_os,

reset_mil_interrupt_RT3_os => reset_mil_interrupt_RT3_os,

IF_FREQ_GPS_acq_sel_os => IF_Freq_GPS_acq_sel_s,

IF_FREQ_NAVIC_acq_sel_os =>IF_Freq_NAVIC_acq_sel_s,

IF_FREQ_GPS_trk_sel_os =>IF_Freq_GPS_trk_sel_s,

IF_FREQ_NAVIC_trk_sel_os => IF_Freq_NAVIC_trk_sel_s,

-------------saphyrion spi reg--------

reprogram_SY_cmd_sel_os => reprogram_SY_cmd_sel_s,

SY17_PLL_val_sel_os => SY17_PLL_val_sel_s,

SY17_AGC_val_sel_os => SY17_AGC_val_sel_s,

SYFE_PM_val_sel_os => SYFE_PM_val_sel_s,

SYFE_RFPLL_val_sel_os => SYFE_RFPLL_val_sel_s,

SYFE_IFPLL_val_sel_os => SYFE_IFPLL_val_sel_s

);

corr_data_out_mux_e_inst: corr_data_out_mux_e PORT MAP(

svid_acq_is => svid_searched_s,

nav_data_reg_ch_is => nav_data_reg_ch_s,

sat_id_is => svid_channel_ch_s,

code_cyle_count_value_ch_is => code_cyle_count_value_ch_s,

one_ms_counter_value_ch_is => one_ms_counter_value_ch_s,

twenty_ms_counter_value_ch_is => twenty_ms_counter_value_ch_s,

code_phase_count_ch_is => code_phase_count_ch_s,

car_cycle_count_ch_is => car_cycle_count_ch_s,

car_phase_count_ch_is => car_phase_count_ch_s,

p_cndr_is => p_cndr_out_ch_s,

q_cndr_is => q_cndr_out_ch_s,

pll_order1_is => pll_order1_s,

pll_order2_is  => pll_order2_s,

pll_order3_is  => pll_order3_s,

sbas_navdata_is => sbas_navdata_s,

rt_address_RT1_is => rt_address_RT1_is,

rt_address_RT2_is => rt_address_RT2_is,

rt_address_RT3_is => rt_address_RT3_is,

lock_detector_ant1_is => lock_detector_ant1_is,

lock_detector_ant2_is => lock_detector_ant2_is,

lock_detector_ant3_is => lock_detector_ant3_is,

lock_detector_ant4_is => lock_detector_ant4_is,

wdt_cmd_status_is => wdt_cmd_status_s,

up_address_is => up_address_is,

hdwt_reset_counter_is => hdwt_reset_counter_s,

hwdt_reset_status_ff_is => hwdt_reset_status_ff_s,

channel_status_is => channel_status_s,

sat_status_is => sat_status_s,

TSM_data_RT1_is => TSM_data_RT1_s,

TSM_data_RT2_is => TSM_data_RT2_s,

TSM_data_RT3_is => TSM_data_RT3_s,

soft_reset_flag_is => soft_reset_flag_s,

upconv1_spi_gpio4_out_is => upconv1_spi_gpio4_out_s,

upconv2_spi_gpio4_out_is => upconv2_spi_gpio4_out_s,

ant1_2015_upst_cntr_is => ant1_2015_upst_cntr_s,

ant2_2015_upst_cntr_is => ant2_2015_upst_cntr_s,

ant3_2015_upst_cntr_is => ant3_2015_upst_cntr_s,

ant4_2015_upst_cntr_is => ant4_2015_upst_cntr_s,

acq_threshold_LSW_is  => acq_threshold_LSW_s,

acq_threshold_MSW_is  => acq_threshold_MSW_s,

trk_threshold_is     => trk_threshold_s,

acq1_search_reg_is  =>  acq1_search_reg_s,

acq2_search_reg_is  =>  acq2_search_reg_s,

acq3_search_reg_is  =>  acq3_search_reg_s,

acq4_search_reg_is  =>  acq4_search_reg_s,

fft_status_is => fft_status_s,

mil_1553_interrupt_RT1_is => mil_1553_interrupt_RT1_s,

mil_1553_interrupt_RT2_is => mil_1553_interrupt_RT2_s,

mil_1553_interrupt_RT3_is => mil_1553_interrupt_RT3_s,

launch_orbit_phase_is=> Launch_or_Orbit_input_s,

carr_aided_loop_is  => carr_aided_loop_out_s,

fll_zero_en_dis_is    => fll_zero_en_dis_out_s,

config_crc_err_is => config_crc_err_is,

config_ecc_err_is => config_ecc_err_is,

config_synd_valid_is => config_synd_valid_is,

icap_busy_is       =>  icap_busy_is,

syndrome_is  =>  syndrome_is ,

synd_valid_count_is => synd_valid_count_is,

rt_adddr_sel_is    => rt_adddr_sel_is,

antenna_combining_control_is => antenna_combining_control_s,

lock_detector_threshold1_is => lock_counter_threshold1_s,

lock_detector_threshold2_is => lock_counter_threshold2_s,

SY_FE_write_count_is  => SY_FE_write_count_s,

reg_val_read_is  => reg_val_read_s,

latched_incoming_inp1_countp7_is => latched_incoming_inp1_countp7_s,

latched_incoming_inp1_countp5_is => latched_incoming_inp1_countp5_s,

latched_incoming_inp1_countp3_is => latched_incoming_inp1_countp3_s,

latched_incoming_inp1_countp1_is => latched_incoming_inp1_countp1_s,

latched_incoming_inp1_countm7_is => latched_incoming_inp1_countm7_s,

latched_incoming_inp1_countm5_is => latched_incoming_inp1_countm5_s,

latched_incoming_inp1_countm3_is => latched_incoming_inp1_countm3_s,

latched_incoming_inp1_countm1_is => latched_incoming_inp1_countm1_s,

latched_incoming_qp1_countp7_is => latched_incoming_qp1_countp7_s,

latched_incoming_qp1_countp5_is => latched_incoming_qp1_countp5_s,

latched_incoming_qp1_countp3_is => latched_incoming_qp1_countp3_s,

latched_incoming_qp1_countp1_is => latched_incoming_qp1_countp1_s,

latched_incoming_qp1_countm7_is => latched_incoming_qp1_countm7_s,

latched_incoming_qp1_countm5_is => latched_incoming_qp1_countm5_s,

latched_incoming_qp1_countm3_is => latched_incoming_qp1_countm3_s,

latched_incoming_qp1_countm1_is => latched_incoming_qp1_countm1_s,

latched_incoming_inp2_countp7_is => latched_incoming_inp2_countp7_s,

latched_incoming_inp2_countp5_is => latched_incoming_inp2_countp5_s,

latched_incoming_inp2_countp3_is => latched_incoming_inp2_countp3_s,

latched_incoming_inp2_countp1_is => latched_incoming_inp2_countp1_s,

latched_incoming_inp2_countm7_is => latched_incoming_inp2_countm7_s,

latched_incoming_inp2_countm5_is => latched_incoming_inp2_countm5_s,

latched_incoming_inp2_countm3_is => latched_incoming_inp2_countm3_s,

latched_incoming_inp2_countm1_is => latched_incoming_inp2_countm1_s,

latched_incoming_qp2_countp7_is => latched_incoming_qp2_countp7_s,

latched_incoming_qp2_countp5_is => latched_incoming_qp2_countp5_s,

latched_incoming_qp2_countp3_is => latched_incoming_qp2_countp3_s,

latched_incoming_qp2_countp1_is => latched_incoming_qp2_countp1_s,

latched_incoming_qp2_countm7_is => latched_incoming_qp2_countm7_s,

latched_incoming_qp2_countm5_is => latched_incoming_qp2_countm5_s,

latched_incoming_qp2_countm3_is => latched_incoming_qp2_countm3_s,

latched_incoming_qp2_countm1_is => latched_incoming_qp2_countm1_s,

latched_incoming_inp3_countp7_is => latched_incoming_inp3_countp7_s,

latched_incoming_inp3_countp5_is => latched_incoming_inp3_countp5_s,

latched_incoming_inp3_countp3_is => latched_incoming_inp3_countp3_s,

latched_incoming_inp3_countp1_is => latched_incoming_inp3_countp1_s,

latched_incoming_inp3_countm7_is => latched_incoming_inp3_countm7_s,

latched_incoming_inp3_countm5_is => latched_incoming_inp3_countm5_s,

latched_incoming_inp3_countm3_is => latched_incoming_inp3_countm3_s,

latched_incoming_inp3_countm1_is => latched_incoming_inp3_countm1_s,

latched_incoming_qp3_countp7_is => latched_incoming_qp3_countp7_s,

latched_incoming_qp3_countp5_is => latched_incoming_qp3_countp5_s,

latched_incoming_qp3_countp3_is => latched_incoming_qp3_countp3_s,

latched_incoming_qp3_countp1_is => latched_incoming_qp3_countp1_s,

latched_incoming_qp3_countm7_is => latched_incoming_qp3_countm7_s,

latched_incoming_qp3_countm5_is => latched_incoming_qp3_countm5_s,

latched_incoming_qp3_countm3_is => latched_incoming_qp3_countm3_s,

latched_incoming_qp3_countm1_is => latched_incoming_qp3_countm1_s,

latched_incoming_inp4_countp7_is => latched_incoming_inp4_countp7_s,

latched_incoming_inp4_countp5_is => latched_incoming_inp4_countp5_s,

latched_incoming_inp4_countp3_is => latched_incoming_inp4_countp3_s,

latched_incoming_inp4_countp1_is => latched_incoming_inp4_countp1_s,

latched_incoming_inp4_countm7_is => latched_incoming_inp4_countm7_s,

latched_incoming_inp4_countm5_is => latched_incoming_inp4_countm5_s,

latched_incoming_inp4_countm3_is => latched_incoming_inp4_countm3_s,

latched_incoming_inp4_countm1_is => latched_incoming_inp4_countm1_s,

latched_incoming_qp4_countp7_is => latched_incoming_qp4_countp7_s,

latched_incoming_qp4_countp5_is => latched_incoming_qp4_countp5_s,

latched_incoming_qp4_countp3_is => latched_incoming_qp4_countp3_s,

latched_incoming_qp4_countp1_is => latched_incoming_qp4_countp1_s,

latched_incoming_qp4_countm7_is => latched_incoming_qp4_countm7_s,

latched_incoming_qp4_countm5_is => latched_incoming_qp4_countm5_s,

latched_incoming_qp4_countm3_is => latched_incoming_qp4_countm3_s,

latched_incoming_qp4_countm1_is => latched_incoming_qp4_countm1_s,

data_to_processor_os => data_to_processor_old_s,

uart_txd_is => uart_txd_is,

rwe3_is  => rwe3_is,

wdt_l_is => 	wdt_l_is  ,

bexc_l_is => bexc_l_is

);

SPI_code_sy_inst: SPI_code_sy

port map (

spi_clk_SY_is=>spi_clk_SY_is,

reset_is =>POR_G_is,

start_prog_flag_SY_is =>start_prog_flag_SY_s,

read_SPI_SY_is =>read_SPI_SY_s,

done_wr_SY_os=>done_wr_SY_s,

read_error_SY_os=>read_error_SY_s,

spi_en_SY_os=>spi_en_SY_os,

spi_mosi_SY_os=>spi_mosi_SY_os,

spi_miso_SY_is=>spi_miso_SY_is,

reg_val_read_os=>reg_val_read_s,

SY17_PLL_val_is=>SY17_PLL_val_s,

SY17_AGC_val_is=>SY17_AGC_val_s,

SYFE_PM_val_is=>SYFE_PM_val_s,

SYFE_RFPLL_val_is=>SYFE_RFPLL_val_s,

SYFE_IFPLL_val_is=>SYFE_IFPLL_val_s

);

SPI_control_SY_inst: SPI_control_SY

Port map (

clk_is =>spi_clk_SY_is,

reset_is =>POR_G_is,

--   reprogram_SY_cmd_flag_is =>'0',

reprogram_SY_cmd_flag_is =>reprogram_SY_cmd_s,

done_wr_SY_is =>done_wr_SY_s,

read_error_SY_is =>read_error_SY_s,

start_prog_flag_SY_os =>start_prog_flag_SY_s,

read_SPI_SY_os =>read_SPI_SY_s,

SY_FE_write_count_os =>SY_FE_write_count_s 	);

P_descr_timers: process(clk_is, POR_G_is)

begin

if POR_G_is = '1' then

set1_to_set2_timer_launch_s <= "00000000000000000000011111010000";-- default value 2000

set2_to_set3_timer_launch_s <= "00000000000000000000101000101000"; -- default value 2600

set3_to_set4_timer_launch_s <= "00000000000000000000110010000000"; -- default value 	3200

set4_to_set5_timer_launch_s <= "00000000000000000000111011011000"; -- default value 	3800

set1_to_set2_timer_launch_def_s <= "00000000000000000000011111010000";-- default value 2000

set2_to_set3_timer_launch_def_s <= "00000000000000000000101000101000"; -- default value 2600

set3_to_set4_timer_launch_def_s <= "00000000000000000000110010000000"; -- default value 	3200

set4_to_set5_timer_launch_def_s <= "00000000000000000000111011011000"; -- default value 	3800

Narrow_corr_step1_timer_s <= "00000000000000000001011101110000";

Narrow_corr_step2_timer_s <= "00000000000000000001111101000000";

Narrow_corr_step1_timer_def_s <= "00000000000000000001011101110000";

Narrow_corr_step2_timer_def_s <= "00000000000000000001111101000000";

Narrow_correlator_step1_enable_s <= '1';

Narrow_correlator_step2_enable_s <= '1';

fll_timer_s         <= "00000000000000000001000100110000";  -- default value 4400

fll_timer_def_s     <= "00000000000000000001000100110000";  -- default value 4400

Launch_or_Orbit_input_s <= '0';  -- most important

All_Timers_updated_s <= '0';

FLL_data_check_counter_s <= "00000000000000000000000000100111";

elsif clk_is'event and clk_is = '1' then

if(FLL_data_check_counter_enable_s = '1')  then

FLL_data_check_counter_s <= data_processor_is;

end if;

if  launch_orbit_phase_sel_s = '1' then

Launch_or_Orbit_input_s  <= data_processor_is(0);

end if;

if set1_to_set2_timer_launch_enable_s = '1' then

set1_to_set2_timer_launch_s <= data_processor_is(31 downto 0);

end if;

if set2_to_set3_timer_launch_enable_s = '1' then

set2_to_set3_timer_launch_s <= data_processor_is(31 downto 0);

end if;

if set3_to_set4_timer_launch_enable_s = '1' then

set3_to_set4_timer_launch_s <= data_processor_is(31 downto 0);

end if;

if set4_to_set5_timer_launch_enable_s = '1' then

set4_to_set5_timer_launch_s <= data_processor_is(31 downto 0);

end if;

if( Narrow_corr_step1_timer_sel_s = '1') then

Narrow_corr_step1_timer_s <= data_processor_is;

end if;

if( Narrow_corr_step2_timer_sel_s = '1') then

Narrow_corr_step2_timer_s <= data_processor_is;

end if;

if(Narrow_correlator_step1_enable_sel_s ='1') then

Narrow_correlator_step1_enable_s <= data_processor_is(0);

end if;

if(Narrow_correlator_step2_enable_sel_s ='1') then

Narrow_correlator_step2_enable_s <= data_processor_is(0);

end if;

if  fll_timer_enable_s = '1' then

fll_timer_s  <= data_processor_is(31 downto 0);

end if;

if(All_Timers_updated_enable_s = '1') then

All_Timers_updated_s <= data_processor_is(0);

end if;

if(All_Timers_updated_s = '1') then

set1_to_set2_timer_launch_def_s <= set1_to_set2_timer_launch_s ;

set2_to_set3_timer_launch_def_s <= set2_to_set3_timer_launch_s ;

set3_to_set4_timer_launch_def_s <= set3_to_set4_timer_launch_s ;

set4_to_set5_timer_launch_def_s <= set4_to_set5_timer_launch_s ;

Narrow_corr_step1_timer_def_s <= Narrow_corr_step1_timer_s;

Narrow_corr_step2_timer_def_s <= Narrow_corr_step2_timer_s;

fll_timer_def_s <= fll_timer_s ;

end if;

end if;

end process P_descr_timers;

-- NAVIC Command enable register contents from processor

Process_p1: process(clk_is, POR_G_is)

begin

if POR_G_is = '1' then

navic_cmd_enable_s<="00000001000001";

elsif clk_is'event and clk_is = '1' then

if(navic_cmd_enable_reg_wr_sel_s = '1')  then

navic_cmd_enable_s <= data_processor_is(13 downto 0);

end if;

end if;

end process Process_p1;

-- SBAS enable register contents from processor

P_lock_bit_sync_threshold_reg: process(clk_is, POR_G_is)

begin

if POR_G_is = '1' then

sbas_enable_s<='1';

antenna_combining_control_s <= "0000";

lock_counter_threshold1_s<="0000111100";

lock_counter_threshold2_s<="0000001111";

elsif clk_is'event and clk_is = '1' then

if(sbas_enable_reg_wr_sel_s = '1')  then

sbas_enable_s <= data_processor_is(0);

end if;

if(Antenna_combining_sel_enable_s = '1') then

antenna_combining_control_s <= data_processor_is(3 downto 0);

end if;

if(lock_detector_threshold_sel_s = '1') then

lock_counter_threshold1_s <= data_processor_is(9 downto 0);

lock_counter_threshold2_s <= data_processor_is(19 downto 10);

end if;

end if;

end process P_lock_bit_sync_threshold_reg;

-- GP2015 upset counter based on lock detector

P_RF_upset_gen: process(clk_is, POR_in_is)

begin

if POR_in_is = '1' then

ant1_2015_upst_cntr_s<=(others=>'0');

ant2_2015_upst_cntr_s<=(others=>'0');

ant3_2015_upst_cntr_s<=(others=>'0');

ant4_2015_upst_cntr_s<=(others=>'0');

elsif clk_is'event and clk_is = '1' then

-- GP2015 upset counter for Antenna-1 chain

if(lock_detector_ant1_is = '0') and (prev_lock_detector_ant1_s = '1') then

ant1_2015_upst_cntr_s(6 downto 0) <= ant1_2015_upst_cntr_s(6 downto 0) + 1 ;

ant1_2015_upst_cntr_s(7) <= '1';

end if;

-- GP2015 upset counter for Antenna-2 chain

if(lock_detector_ant2_is = '0') and (prev_lock_detector_ant2_s = '1')   then

ant2_2015_upst_cntr_s(6 downto 0) <= ant2_2015_upst_cntr_s(6 downto 0) + 1 ;

ant2_2015_upst_cntr_s(7) <= '1';

end if;

-- GP2015 upset counter for Antenna-3 chain

if(lock_detector_ant3_is = '0') and (prev_lock_detector_ant3_s = '1')   then

ant3_2015_upst_cntr_s(6 downto 0) <= ant3_2015_upst_cntr_s(6 downto 0) + 1 ;

ant3_2015_upst_cntr_s(7) <= '1';

end if;

-- GP2015 upset counter for Antenna-4 chain

if(lock_detector_ant4_is = '0') and (prev_lock_detector_ant4_s = '1')   then

ant4_2015_upst_cntr_s(6 downto 0) <= ant4_2015_upst_cntr_s(6 downto 0) + 1 ;

ant4_2015_upst_cntr_s(7) <= '1';

end if;

end if;

end process P_RF_upset_gen;

-- GP2015 upset counter based on lock detector

P_GP2015_lock_reg: process(clk_is, POR_G_is)

begin

if POR_G_is = '1' then

prev_lock_detector_ant1_s <= '0';

prev_lock_detector_ant2_s <= '0';

prev_lock_detector_ant3_s <= '0';

prev_lock_detector_ant4_s <= '0';

elsif clk_is'event and clk_is = '1' then

prev_lock_detector_ant1_s <= lock_detector_ant1_is;

prev_lock_detector_ant2_s <= lock_detector_ant2_is;

prev_lock_detector_ant3_s <= lock_detector_ant3_is;

prev_lock_detector_ant4_s <= lock_detector_ant4_is;

end if;

end process P_GP2015_lock_reg;

-- MIL-1553 Interrupt

Process_p2: process(clk_is,POR_G_is)

begin

if POR_G_is = '1' then

RT1_flag_ff1_s <= '0';

RT1_flag_ff2_s <= '0';

RT1_flag_ff3_s <= '0';

RT2_flag_ff1_s <= '0';

RT2_flag_ff2_s <= '0';

RT2_flag_ff3_s <= '0';

RT3_flag_ff1_s <= '0';

RT3_flag_ff2_s <= '0';

RT3_flag_ff3_s <= '0';

elsif clk_is'event and clk_is = '1' then

RT1_flag_ff1_s <= mil_1553_interrupt_RT1_is;

RT1_flag_ff2_s <= RT1_flag_ff1_s;

RT1_flag_ff3_s <= RT1_flag_ff2_s;

RT2_flag_ff1_s <= mil_1553_interrupt_RT2_is;

RT2_flag_ff2_s <= RT2_flag_ff1_s;

RT2_flag_ff3_s <= RT2_flag_ff2_s;

RT3_flag_ff1_s <= mil_1553_interrupt_RT3_is;

RT3_flag_ff2_s <= RT3_flag_ff1_s;

RT3_flag_ff3_s <= RT3_flag_ff2_s;

end if;

end process Process_p2;

mil_1553_interrupt_RT1_s  <= RT1_flag_ff3_s;

mil_1553_interrupt_RT2_s  <= RT2_flag_ff3_s;

mil_1553_interrupt_RT3_s  <= RT3_flag_ff3_s;

P_PLL_order_registers: process(clk_is, POR_G_is)

begin

if POR_G_is = '1' then

pll_order1_s(0) <= (others=>'0');

pll_order1_s(1) <= (others=>'0');

pll_order1_s(2) <= (others=>'0');

pll_order1_s(3) <= (others=>'0');

pll_order1_s(4) <= (others=>'0');

pll_order1_s(5) <= (others=>'0');

pll_order1_s(6) <= (others=>'0');

pll_order1_s(7) <= (others=>'0');

pll_order1_s(8) <= (others=>'0');

pll_order1_s(9) <= (others=>'0');

pll_order1_s(10) <= (others=>'0');

pll_order1_s(11) <= (others=>'0');

pll_order1_s(12) <= (others=>'0');

pll_order1_s(13) <= (others=>'0');

pll_order1_s(14) <= (others=>'0');

pll_order1_s(15) <= (others=>'0');

pll_order1_s(16) <= (others=>'0');

pll_order1_s(17) <= (others=>'0');

pll_order2_s(0) <= (others=>'0');

pll_order2_s(1) <= (others=>'0');

pll_order2_s(2) <= (others=>'0');

pll_order2_s(3) <= (others=>'0');

pll_order2_s(4) <= (others=>'0');

pll_order2_s(5) <= (others=>'0');

pll_order2_s(6) <= (others=>'0');

pll_order2_s(7) <= (others=>'0');

pll_order2_s(8) <= (others=>'0');

pll_order2_s(9) <= (others=>'0');

pll_order2_s(10) <= (others=>'0');

pll_order2_s(11) <= (others=>'0');

pll_order2_s(12) <= (others=>'0');

pll_order2_s(13) <= (others=>'0');

pll_order2_s(14) <= (others=>'0');

pll_order2_s(15) <= (others=>'0');

pll_order2_s(16) <= (others=>'0');

pll_order2_s(17) <= (others=>'0');

pll_order3_s(0) <= (others=>'0');

pll_order3_s(1) <= (others=>'0');

pll_order3_s(2) <= (others=>'0');

pll_order3_s(3) <= (others=>'0');

pll_order3_s(4) <= (others=>'0');

pll_order3_s(5) <= (others=>'0');

pll_order3_s(6) <= (others=>'0');

pll_order3_s(7) <= (others=>'0');

pll_order3_s(8) <= (others=>'0');

pll_order3_s(9) <= (others=>'0');

pll_order3_s(10) <= (others=>'0');

pll_order3_s(11) <= (others=>'0');

pll_order3_s(12) <= (others=>'0');

pll_order3_s(13) <= (others=>'0');

pll_order3_s(14) <= (others=>'0');

pll_order3_s(15) <= (others=>'0');

pll_order3_s(16) <= (others=>'0');

pll_order3_s(17) <= (others=>'0');

elsif clk_is'event and clk_is = '1' then

if(epoch_latch_pulse_s='1') then

pll_order1_s(0) <= Order1_out_ch_s(0)(36 downto 5);

pll_order1_s(1) <= Order1_out_ch_s(1)(36 downto 5);

pll_order1_s(2) <= Order1_out_ch_s(2)(36 downto 5);

pll_order1_s(3) <= Order1_out_ch_s(3)(36 downto 5);

pll_order1_s(4) <= Order1_out_ch_s(4)(36 downto 5);

pll_order1_s(5) <= Order1_out_ch_s(5)(36 downto 5);

pll_order1_s(6) <= Order1_out_ch_s(6)(36 downto 5);

pll_order1_s(7) <= Order1_out_ch_s(7)(36 downto 5);

pll_order1_s(8) <= Order1_out_ch_s(8)(36 downto 5);

pll_order1_s(9) <= Order1_out_ch_s(9)(36 downto 5);

pll_order1_s(10) <= Order1_out_ch_s(10)(36 downto 5);

pll_order1_s(11) <= Order1_out_ch_s(11)(36 downto 5);

pll_order1_s(12) <= Order1_out_ch_s(12)(36 downto 5);

pll_order1_s(13) <= Order1_out_ch_s(13)(36 downto 5);

pll_order1_s(14) <= Order1_out_ch_s(14)(36 downto 5);

pll_order1_s(15) <= Order1_out_ch_s(15)(36 downto 5);

pll_order1_s(16) <= Order1_out_ch_s(16)(36 downto 5);

pll_order1_s(17) <= Order1_out_ch_s(17)(36 downto 5);

pll_order2_s(0) <= Order2_out_ch_s(0)(36 downto 5);

pll_order2_s(1) <= Order2_out_ch_s(1)(36 downto 5);

pll_order2_s(2) <= Order2_out_ch_s(2)(36 downto 5);

pll_order2_s(3) <= Order2_out_ch_s(3)(36 downto 5);

pll_order2_s(4) <= Order2_out_ch_s(4)(36 downto 5);

pll_order2_s(5) <= Order2_out_ch_s(5)(36 downto 5);

pll_order2_s(6) <= Order2_out_ch_s(6)(36 downto 5);

pll_order2_s(7) <= Order2_out_ch_s(7)(36 downto 5);

pll_order2_s(8) <= Order2_out_ch_s(8)(36 downto 5);

pll_order2_s(9) <= Order2_out_ch_s(9)(36 downto 5);

pll_order2_s(10) <= Order2_out_ch_s(10)(36 downto 5);

pll_order2_s(11) <= Order2_out_ch_s(11)(36 downto 5);

pll_order2_s(12) <= Order2_out_ch_s(12)(36 downto 5);

pll_order2_s(13) <= Order2_out_ch_s(13)(36 downto 5);

pll_order2_s(14) <= Order2_out_ch_s(14)(36 downto 5);

pll_order2_s(15) <= Order2_out_ch_s(15)(36 downto 5);

pll_order2_s(16) <= Order2_out_ch_s(16)(36 downto 5);

pll_order2_s(17) <= Order2_out_ch_s(17)(36 downto 5);

pll_order3_s(0) <= Order3_out_ch_s(0)(36 downto 5);

pll_order3_s(1) <= Order3_out_ch_s(1)(36 downto 5);

pll_order3_s(2) <= Order3_out_ch_s(2)(36 downto 5);

pll_order3_s(3) <= Order3_out_ch_s(3)(36 downto 5);

pll_order3_s(4) <= Order3_out_ch_s(4)(36 downto 5);

pll_order3_s(5) <= Order3_out_ch_s(5)(36 downto 5);

pll_order3_s(6) <= Order3_out_ch_s(6)(36 downto 5);

pll_order3_s(7) <= Order3_out_ch_s(7)(36 downto 5);

pll_order3_s(8) <= Order3_out_ch_s(8)(36 downto 5);

pll_order3_s(9) <= Order3_out_ch_s(9)(36 downto 5);

pll_order3_s(10) <= Order3_out_ch_s(10)(36 downto 5);

pll_order3_s(11) <= Order3_out_ch_s(11)(36 downto 5);

pll_order3_s(12) <= Order3_out_ch_s(12)(36 downto 5);

pll_order3_s(13) <= Order3_out_ch_s(13)(36 downto 5);

pll_order3_s(14) <= Order3_out_ch_s(14)(36 downto 5);

pll_order3_s(15) <= Order3_out_ch_s(15)(36 downto 5);

pll_order3_s(16) <= Order3_out_ch_s(16)(36 downto 5);

pll_order3_s(17) <= Order3_out_ch_s(17)(36 downto 5);

end if;

end if;

end process P_PLL_order_registers;

P_scrub: process(clk_is, POR_G_is )

begin

if POR_G_is='1' then

car_mix_ROM_ena_s <= '0';

start_scrub_s <= '0';

car_mix_ROM_wea_s<= (others=>'0');

car_mix_ROM_addra_s<= (others=>'0');

car_mix_ROM_dina_s<= (others=>'0');

one_ms_count_scrub_s<= (others=>'0');

Local_inphase_scrub_s<= (others=>'0');

Local_qphase_scrub_s <= (others=>'0');

incoming_inphase_scrub_s<= (others=>'0');

incoming_qphase_scrub_s<= (others=>'0');

edac_data_input_scrub_s<= (others=>'0');

IP_mixout_scrub_s<= (others=>'0');

QP_mixout_scrub_s<= (others=>'0');

local_addr_gen_scrub_s<= (others=>'0');

loc1_scrub_s<= (others=>'0');

loc2_scrub_s<= (others=>'0');

loc3_scrub_s <= (others=>'0');

elsif clk_is='1' and clk_is'event then

if( one_ms_count_scrub_s >= "01001110000111111" ) then -- 1ms @ 40MHz

one_ms_count_scrub_s<= (others=>'0');

else

one_ms_count_scrub_s<= one_ms_count_scrub_s + 1;

end if;

if( one_ms_count_scrub_s >= "01001110000111111" ) then -- 1ms @ 40MHz

start_scrub_s <= '1';

elsif( one_ms_count_scrub_s >= "00001000000101010" ) then  -- 4138 cycles

start_scrub_s <= '0';

end if;

if(start_scrub_s = '1') then

if(one_ms_count_scrub_s(1 downto 0) = "11" ) then

local_addr_gen_scrub_s <= local_addr_gen_scrub_s  + 1;

end if;

car_mix_ROM_wea_s <= "1";

car_mix_ROM_ena_s <= '1';

car_mix_ROM_dina_s <= edac_par_out_scrub_s & edac_data_out_scrub_s ;

car_mix_ROM_addra_s <= 	loc3_scrub_s ;

else

car_mix_ROM_wea_s <= "0";

car_mix_ROM_ena_s <= '0';

car_mix_ROM_dina_s <= (others=>'0');

car_mix_ROM_addra_s<= (others=>'0');

local_addr_gen_scrub_s	<= (others=>'0');

end if;

loc1_scrub_s <= local_addr_gen_scrub_s;

loc2_scrub_s <= loc1_scrub_s;

loc3_scrub_s <= loc2_scrub_s;

--			latched_addr_gen_s <= local_addr_gen_s;

case local_addr_gen_scrub_s(3 downto 0) is

when "0000" => Local_inphase_scrub_s <= "0110";

when "0001" => Local_inphase_scrub_s <= "0101";

when "0010" => Local_inphase_scrub_s <= "0011";

when "0011" => Local_inphase_scrub_s <= "0001";

when "0100" => Local_inphase_scrub_s <= "1111";

when "0101" => Local_inphase_scrub_s <= "1101";

when "0110" => Local_inphase_scrub_s <= "1011";

when "0111" => Local_inphase_scrub_s <= "1010";

when "1000" => Local_inphase_scrub_s <= "1010";

when "1001" => Local_inphase_scrub_s <= "1011";

when "1010" => Local_inphase_scrub_s <= "1101";

when "1011" => Local_inphase_scrub_s <= "1111";

when "1100" => Local_inphase_scrub_s <= "0001";

when "1101" => Local_inphase_scrub_s <= "0011";

when "1110" => Local_inphase_scrub_s <= "0101";

when others => Local_inphase_scrub_s <= "0110";

end case;

case local_addr_gen_scrub_s(3 downto 0) is

when "0000" => Local_qphase_scrub_s <= "1111";

when "0001" => Local_qphase_scrub_s <= "1101";

when "0010" => Local_qphase_scrub_s <= "1011";

when "0011" => Local_qphase_scrub_s <= "1010";

when "0100" => Local_qphase_scrub_s <= "1010";

when "0101" => Local_qphase_scrub_s <= "1011";

when "0110" => Local_qphase_scrub_s <= "1101";

when "0111" => Local_qphase_scrub_s <= "1111";

when "1000" => Local_qphase_scrub_s <= "0001";

when "1001" => Local_qphase_scrub_s <= "0011";

when "1010" => Local_qphase_scrub_s <= "0101";

when "1011" => Local_qphase_scrub_s <= "0110";

when "1100" => Local_qphase_scrub_s <= "0110";

when "1101" => Local_qphase_scrub_s <= "0101";

when "1110" => Local_qphase_scrub_s <= "0011";

when others => Local_qphase_scrub_s <= "0001";

end case;

case local_addr_gen_scrub_s(6 downto 4) is

when "000" => incoming_qphase_scrub_s <= "0111";

when "001" => incoming_qphase_scrub_s <= "0101";

when "010" => incoming_qphase_scrub_s <= "0011";

when "011" => incoming_qphase_scrub_s <= "0001";

when "100" => incoming_qphase_scrub_s <= "1111";

when "101" => incoming_qphase_scrub_s <= "1101";

when "110" => incoming_qphase_scrub_s <= "1011";

when others => incoming_qphase_scrub_s <= "1001";

end case;

case local_addr_gen_scrub_s(9 downto 7) is

when "000" => incoming_inphase_scrub_s <= "0111";

when "001" => incoming_inphase_scrub_s <= "0101";

when "010" => incoming_inphase_scrub_s <= "0011";

when "011" => incoming_inphase_scrub_s <= "0001";

when "100" => incoming_inphase_scrub_s <= "1111";

when "101" => incoming_inphase_scrub_s <= "1101";

when "110" => incoming_inphase_scrub_s <= "1011";

when others => incoming_inphase_scrub_s <= "1001";

end case;

IP_mixout_scrub_s <=  incoming_inphase_scrub_s * Local_inphase_scrub_s - incoming_qphase_scrub_s * Local_qphase_scrub_s;

QP_mixout_scrub_s <=  incoming_qphase_scrub_s * Local_inphase_scrub_s + incoming_inphase_scrub_s * Local_qphase_scrub_s;

edac_data_input_scrub_s <= QP_mixout_scrub_s(6 downto 0) & IP_mixout_scrub_s(6 downto 0);

end if;--clk event with POR

end process P_scrub;

-- Saphyrion spi registor read

Process_saphyrion: process(clk_is, POR_G_is)

begin

if POR_G_is = '1' then

reprogram_SY_cmd_s<='1';

SY17_PLL_val_s<=X"4CEF"; ---SY1017C---PM and PLL  multiplier--8

SY17_AGC_val_s<=(OTHERS=>'0'); ---SY1017C---AGC-DAC---0--highest gain

SYFE_PM_val_s<=X"D055";----SY1008/1007--PM

SYFE_RFPLL_val_s<= X"C462" ; ---SY1008/1007--RF PLL divider--70

SYFE_IFPLL_val_s<=X"F490";  -----SY1008--VCO tuning/ --SY1007 IF PLL Divider

elsif clk_is'event and clk_is = '1' then

if(reprogram_SY_cmd_sel_s = '1')  then

reprogram_SY_cmd_s <= data_processor_is(0);

end if;

if(SY17_PLL_val_sel_s = '1') then

SY17_PLL_val_s <= data_processor_is(15 downto 0);

end if;

if(SY17_AGC_val_sel_s = '1') then

SY17_AGC_val_s <= data_processor_is(15 downto 0);

end if;

if(SYFE_PM_val_sel_s = '1') then

SYFE_PM_val_s <= data_processor_is(15 downto 0);

end if;

if(SYFE_RFPLL_val_sel_s = '1') then

SYFE_RFPLL_val_s <= data_processor_is(15 downto 0);

end if;

if(SYFE_IFPLL_val_sel_s = '1') then

SYFE_IFPLL_val_s <= data_processor_is(15 downto 0);

end if;

end if;

end process Process_saphyrion;

end gnss_correlator_top_a;

44) input_sampler_module_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

Library UNISIM;

use UNISIM.vcomponents.all;

entity input_sampler_module_e is

Port ( clk_40MHz_is : in  STD_LOGIC;

POR_is : in  STD_LOGIC;

sampling_clk_out1_os : out std_logic;

sampling_clk_en_os   : out std_logic

);

end input_sampler_module_e;

architecture input_sampler_module_a of input_sampler_module_e is

component sampling_freq_gen_e is

port( clk_40MHz_is: in std_logic;

POR_is: in std_logic;

sampling_clk_os    : out std_logic;

sampling_clk_en_os : out std_logic);

end component;

signal sampling_clk_s : std_logic;

signal sampling_clk_en_s: std_logic;

begin

sampling_clk_out1_os <= sampling_clk_s;

sampling_freq_gen_inst : sampling_freq_gen_e

port map(   clk_40MHz_is => clk_40MHz_is,

POR_is => POR_is,

sampling_clk_os => sampling_clk_s,

sampling_clk_en_os=> sampling_clk_en_s

);

-- 40MHz Sampling Clock enable connected to Global Buffer BUFG

BUFG_samp : BUFG

port map (

I => sampling_clk_en_s,     -- Clock buffer input

O => sampling_clk_en_os    -- Clock buffer output

);

end input_sampler_module_a;

45) leon_prom_module_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

Library UNISIM;

use UNISIM.vcomponents.all;

entity leon_prom_module_e is

port(clk_40MHz_is,re_is : in std_logic;

address_is: in std_logic_vector(16 downto 0);

data_out_os : out std_logic_vector(31 downto 0));

end leon_prom_module_e;

architecture leon_prom_module_a of leon_prom_module_e is

--Block RAM instantiation, to store the LEON-3 processor code--

COMPONENT code_bram_ip

PORT (

clka : IN STD_LOGIC;

--ena : IN STD_LOGIC;

addra : IN STD_LOGIC_VECTOR(16 DOWNTO 0);

douta : OUT STD_LOGIC_VECTOR(51 DOWNTO 0)

);

END COMPONENT;

--16-bit BCH Decoder component declaration

COMPONENT bch_16_dec1_e

PORT(

rd_en_is : IN std_logic;

data_in_is : IN std_logic_vector(15 downto 0);

par_in_is : IN std_logic_vector(9 downto 0);

data_out_os : OUT std_logic_vector(15 downto 0)

);

END COMPONENT;

signal data_out_s :std_logic_vector(51 DOWNTO 0);

signal bch_decoder1_out_s,bch_decoder2_out_s :std_logic_vector(15 downto 0);

begin

code_bram_ip_inst : code_bram_ip

PORT MAP (

clka => clk_40MHz_is,

-- ena => re_is,

addra => address_is,

douta => data_out_s

);

--16-bit BCH Decoder for Least Significant Word(LSW)--

bch_16_dec_inst1: bch_16_dec1_e

PORT MAP(

rd_en_is => '1' ,

data_in_is => data_out_s(25 downto 10),

par_in_is => data_out_s(9 downto 0),

data_out_os => bch_decoder1_out_s

);

--16-bit BCH Decoder for Most Significant Word(MSW)--

bch_16_dec_inst2: bch_16_dec1_e

PORT MAP(

rd_en_is => '1' ,

data_in_is => data_out_s(51 downto 36),

par_in_is => data_out_s(35 downto 26),

data_out_os => bch_decoder2_out_s

);

data_out_os <=  bch_decoder2_out_s & bch_decoder1_out_s;

end leon_prom_module_a;

46) lock_deglitch_e

library ieee;

use ieee.std_logic_1164.all;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity lock_deglitch_e is

port(clk_40MHz_is : in std_logic;

reset_is: in std_logic;

cmd_in_is: in std_logic;

cmd_deg_os : out std_logic

);

end lock_deglitch_e;

architecture lock_deglitch_a of lock_deglitch_e is

signal counter_logic1_s, counter_logic2_s : std_logic_vector(3 downto 0);

signal enable_ff_logic1_s, enable_ff_logic2_s: std_logic;

signal enable_ff_s : std_logic;

begin

P_deglitch: process(clk_40MHz_is,reset_is)

begin

if reset_is = '1' then

counter_logic1_s <= (others => '0');

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if cmd_in_is = '0' then

counter_logic1_s <= (others => '0');

else

counter_logic1_s <= counter_logic1_s + 1;

end if;  end if;

if reset_is = '1' then

counter_logic2_s <= (others => '0');

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if cmd_in_is = '1' then

counter_logic2_s <= (others => '0');

else

counter_logic2_s <= counter_logic2_s + 1;

end if;  end if;

if reset_is = '1' then

enable_ff_s <= '0';

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if enable_ff_logic1_s = '1' then

enable_ff_s <= '1';

elsif enable_ff_logic2_s = '1' then

enable_ff_s <= '0';

end if;  end if;

end process P_deglitch;

enable_ff_logic1_s <= '1' when counter_logic1_s = "1111" else '0';

enable_ff_logic2_s <= '1' when counter_logic2_s = "1111" else '0';

cmd_deg_os <= enable_ff_s;

end lock_deglitch_a;

47) lock_detector_ch_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity lock_detector_ch_e is

PORT(

clk_40MHz_is : IN std_logic;

POR_is : IN std_logic;

start_is : IN std_logic;

channel_par_load_is : IN std_logic;

lock_status_in_is : IN std_logic;

reset_channel_en_pr_is : IN std_logic;

lock_counter_threshold1_is : in std_logic_vector(9 downto 0);

lock_counter_threshold2_is : in std_logic_vector(9 downto 0);

bit_sync_ind_is : IN std_logic;

lock_status_os : OUT std_logic

);

end lock_detector_ch_e;

architecture lock_detector_ch_a of lock_detector_ch_e is

signal lock_counter_s: std_logic_vector(9 downto 0);

signal lock_enable_s,lock_enable2_s : std_logic;

signal lock_counter2_s: std_logic_vector(12 downto 0);

signal lock_state_s : std_logic;

signal reset_channel_s: std_logic;

begin

P_reset_ch_ff: process(clk_40MHz_is,POR_is)

begin

if POR_is = '1' then

reset_channel_s <= '0';

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if start_is = '1' then

reset_channel_s <= '0';

elsif reset_channel_en_pr_is = '1' then

reset_channel_s <= '1';

end if;   end if;

end process P_reset_ch_ff;

-- below code if  lock status output made according flow chart- FDD

P_lock_status_ff: process(clk_40MHz_is,POR_is)

begin

if POR_is = '1' then

lock_status_os <= '0';

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if start_is = '1' then

lock_status_os <= '1';

elsif reset_channel_s = '1' then

lock_status_os <= '0';

elsif lock_state_s = '1' then

lock_status_os <= '1';

elsif lock_enable_s = '1' then

lock_status_os <= '1' ;

else

lock_status_os <= '0';

end if;  end if;

end process P_lock_status_ff;

P_lock_counters: process(clk_40MHz_is,POR_is)

begin

if POR_is = '1'  then

lock_state_s <= '0';

lock_counter_s <= (others => '0');

lock_counter2_s <= (others => '0');

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

if  start_is = '1' then				lock_state_s <= '1';				elsif lock_enable2_s = '1' then

lock_state_s <= '0';

end if;

if  start_is = '1' then					lock_counter_s <= (others => '0');

elsif channel_par_load_is = '1' then

if lock_status_in_is = '1' then

if lock_counter_s = lock_counter_threshold1_is then

lock_counter_s <= lock_counter_threshold1_is;

else

lock_counter_s <= lock_counter_s + 1;

end if;

elsif lock_status_in_is = '0' then

if lock_counter_s = "0000000000" then

lock_counter_s <= "0000000000";

else

lock_counter_s <= lock_counter_s - 1;

end if;  end if;end if;

if  start_is = '1' then							lock_counter2_s <= (others => '0');

elsif channel_par_load_is = '1' then

lock_counter2_s <= lock_counter2_s + 1;

end if;  end if;

end process P_lock_counters;

lock_enable_s <=  '0' when bit_sync_ind_is = '0' else  -- V2_change

'1' when lock_counter_s >= lock_counter_threshold2_is else '0';

--(actual lock output, when, bit sync is zero, it is zero, when lock counter is >12 it is locked

lock_enable2_s <= lock_counter2_s(12);

-- lockenable2 maintain the lock status high till bit sync time (12th bit = 4096 seconds)

end lock_detector_ch_a;

48) entity lock_detector_descr_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_SIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity lock_detector_descr_e is

port(clk_40MHz_is,POR_is: in std_logic;

p_i_is,p_q_is: in std_logic_vector(27 downto 0);		  track_th_sel_is : in std_logic;

up_data_is : in std_logic_vector(31 downto 0);

lock_counter_enable_os: out std_logic;

trk_threshold_os: out std_logic_vector(31 downto 0)

);

end lock_detector_descr_e;

architecture lock_detector_descr_a of lock_detector_descr_e is

--	signal idata_square_s,qdata_square_s : std_logic_vector(31 downto 0);

signal threshold_value_s , result1_s: std_logic_vector(55 downto 0);

signal lock_counter_enable_temp_s : std_logic;

signal p_i_temp_s,p_q_temp_s: std_logic_vector(27 downto 0);

signal comapre_th_value_reg_s : std_logic_vector(31 downto 0);

signal result2_s : std_logic_vector(56 downto 0);

signal result3_s : std_logic_vector(57 downto 0);

begin

P_power_calc: process(clk_40MHz_is,POR_is)

begin

if POR_is = '1' then

--			idata_square_s <= (others => '0');

--			qdata_square_s <= (others => '0');

p_i_temp_s <= (others => '0');

p_q_temp_s <= (others => '0');

lock_counter_enable_os <= '0';

result1_s<= (others => '0');

result2_s<= (others => '0');

result3_s<= (others => '0');

threshold_value_s<= (others => '0');

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

p_i_temp_s <= p_i_is;

p_q_temp_s <= p_q_is;

--				idata_square_s <= p_i_temp_s * p_i_temp_s;

--		qdata_square_s <= p_q_temp_s * p_q_temp_s;

result1_s <=  p_i_temp_s * p_i_temp_s - p_q_temp_s * p_q_temp_s;

result2_s <=  p_i_temp_s * sxt(p_i_temp_s,29) -   p_q_temp_s * sxt(p_q_temp_s,29);

result3_s <=  sxt(p_i_temp_s,30) *  p_i_temp_s - sxt(p_q_temp_s,30) * p_q_temp_s;

if(result1_s = result2_s(55 downto 0) ) then

threshold_value_s <= result1_s;

elsif(result2_s(55 downto 0) = result3_s(55 downto 0)) then

threshold_value_s <= result2_s(55 downto 0);

elsif(result1_s = result3_s(55 downto 0)) then

threshold_value_s <= result3_s(55 downto 0);

else

threshold_value_s <= result1_s;

end if;

lock_counter_enable_os <= lock_counter_enable_temp_s;

end if;

if POR_is = '1' then

comapre_th_value_reg_s <=  x"00027100";  -- V2_change

elsif clk_40MHz_is'event and clk_40MHz_is='1' then

if track_th_sel_is = '1' then

comapre_th_value_reg_s <= up_data_is;

end if;

end if;

end process P_power_calc;

--	threshold_value_s <= idata_square_s - qdata_square_s;

lock_counter_enable_temp_s <= '0' when threshold_value_s(55) = '1' else

'1' when threshold_value_s >= x"00000000" & comapre_th_value_reg_s else

'0';

trk_threshold_os <= comapre_th_value_reg_s;

end lock_detector_descr_a;

49) mil_command_ram_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity mil_command_ram_e is

port(     MRST             : in    std_logic;

RCLK             : in    std_logic;

USER_WEN_TRP1    : in    std_logic;

USER_REN_TRP1    : in    std_logic;

MSG_TRP1         : in    std_logic_vector(15 downto 0);

USER_WA_TRP1     : in    std_logic_vector(8 downto 0);

USER_RA_TRP1     : in    std_logic_vector(8 downto 0);

DATA_OUT         : out   std_logic_vector(15 downto 0)

);

end mil_command_ram_e;

architecture mil_command_ram_a of mil_command_ram_e is

--------------------------------------------

--9 bit EDAC encoder component declaration

--------------------------------------------

COMPONENT edac_16_enc1_e

PORT(

wr_en_is : IN std_logic;

data_in_is : IN std_logic_vector(15 downto 0);

data_out_os : OUT std_logic_vector(15 downto 0);

par_out_os : OUT std_logic_vector(5 downto 0)

);

END COMPONENT;

------------------------------------------------

--9 bit EDAC decoder component declaration

------------------------------------------------

COMPONENT edac_16_dec1_e

PORT(

rd_en_is : IN std_logic;

data_in_is : IN std_logic_vector(15 downto 0);

par_in_is : IN std_logic_vector(5 downto 0);

data_out_os : OUT std_logic_vector(15 downto 0)

);

END COMPONENT;

--------------------------------------------------------

-- Block RAM component declaration

--------------------------------------------------------

COMPONENT command_mem_core1

PORT (

clka : IN STD_LOGIC;

ena : IN STD_LOGIC;

wea : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

addra : IN STD_LOGIC_VECTOR(8 DOWNTO 0);

dina : IN STD_LOGIC_VECTOR(21 DOWNTO 0);

clkb : IN STD_LOGIC;

enb : IN STD_LOGIC;

addrb : IN STD_LOGIC_VECTOR(8 DOWNTO 0);

doutb : OUT STD_LOGIC_VECTOR(21 DOWNTO 0)

);

END COMPONENT;

-- Scrubbing interface signals

signal scrub_wr_en_s, scrub_rd_en_s : std_logic; --Scrub read enable, Scrub write enable

signal scrub_wr_address_s,scrub_rd_address_s : std_logic_vector(8 downto 0); --Scrub write address and Scrub read address

signal scrub_count_en_s: std_logic; -- Scrub count enable

-- RAM interface signals

signal ram_wr_en_s: std_logic; -- RAM write enable

signal ram_rd_en_s: std_logic;-- RAM read enable

signal ram_rd_address_s, ram_wr_address_s: std_logic_vector(8 downto 0); --RAM write address and read address

signal ram_din_s, ram_dout_s: std_logic_vector(21 downto 0);-- Input data to RAM, Output data from RAM

-- EDAC interface signals

signal edac_enc_out_s,edac_dec_in_s: std_logic_vector(21 downto 0);

signal edac_enc_in_s: std_logic_vector(15 downto 0);

signal edac_dec_out_s:std_logic_vector(15 downto 0);

signal edac_rd_en_s,edac_wr_en_s :std_logic;

--fsm signals

type state_type is (s1,s2,s3); --FSM State variable

signal fsm_state_s: state_type;

--4 msec timer signals

signal scrub_interval_s: std_logic_vector(17 downto 0);

--State transition control signals

signal scrub_timer_en_s :std_logic;

signal scrub_timer_s:std_logic_vector(3 downto 0);

signal s1_s2_en_s:std_logic;  -- State1 to State2 enable

signal s2_s3_en_s: std_logic; -- State2 to State3 enable

signal s3_s2_en_s:std_logic;  -- State3 to State2 enable

signal USER_WEN_TRP1_REG: std_logic; --EDAC Write enable

signal USER_WA_TRP1_REG :  std_logic_vector(8 downto 0); -- Write address

signal MSG_TRP1_REG:  std_logic_vector(15 downto 0);    -- Input to Encoder

signal wea_temp :STD_LOGIC_VECTOR(0 downto  0);

attribute FSM_ENCODING : string;

attribute FSM_ENCODING of fsm_state_s: signal is "ONE-HOT";

attribute SAFE_IMPLEMENTATION :string;

attribute SAFE_IMPLEMENTATION of fsm_state_s: signal is"yes";

begin

DATA_OUT  <=  edac_dec_out_s;

wea_temp(0) <= ram_wr_en_s;

P_write_port_registers: process(RCLK,MRST)

begin

if MRST='0' then

USER_WEN_TRP1_REG <=   '0'; --Initialize write enable

USER_WA_TRP1_REG  <=  (others=>'0');--Iniitialize write address

MSG_TRP1_REG      <=  (others=>'0');--Initialize write data

elsif RCLK'EVENT and RCLK='1' then

USER_WEN_TRP1_REG <=   USER_WEN_TRP1; -- Register for user write enable

USER_WA_TRP1_REG  <=   USER_WA_TRP1 ; -- Register for user write address

MSG_TRP1_REG      <=   MSG_TRP1;      -- Register for user write data

end if;

end process P_write_port_registers;

edac16_encoder: edac_16_enc1_e

PORT MAP(

wr_en_is => edac_wr_en_s, --encoder write enable

data_in_is => edac_enc_in_s , -- encoder: input data bits

data_out_os => edac_enc_out_s(21 downto 6), -- Information bits after encoding

par_out_os => edac_enc_out_s(5 downto 0) -- Check bits

);

edac16_decoder: edac_16_dec1_e

PORT MAP(

rd_en_is => edac_rd_en_s , -- decoder read enable

data_in_is => edac_dec_in_s(21 downto 6), -- decoder: input data bits

par_in_is => edac_dec_in_s(5 downto 0), -- decoder: input check bits

data_out_os => edac_dec_out_s -- Information bits after decoding

);

command_ram : command_mem_core1

PORT MAP (        clka => RCLK,

ena => ram_wr_en_s,

wea => wea_temp,

addra => ram_wr_address_s,

dina => ram_din_s,

clkb => RCLK,

enb => ram_rd_en_s ,

addrb => ram_rd_address_s ,

doutb => ram_dout_s

);

-- User data and Scrub interface

ram_din_s        <=  edac_enc_out_s ; -- Encoder output connected to RAM write port.

edac_dec_in_s    <=  ram_dout_s;      -- RAM read port connected to decoder input.

ram_wr_address_s <=  USER_WA_TRP1_REG when fsm_state_s=s1 else --User write address in state1 for RAM.

scrub_wr_address_s when fsm_state_s=s3 else --Scrub write address in state3 for RAM.

(others=>'0');

ram_rd_address_s <=  USER_RA_TRP1 when fsm_state_s=s1 else --User read address in state1 for RAM.

scrub_rd_address_s when fsm_state_s=s3 else --Scrub read address in state3 for RAM.

(others=>'0');

edac_enc_in_s    <=  MSG_TRP1_REG when fsm_state_s=s1 else --User data connected to edac encoder input.

edac_dec_out_s when fsm_state_s=s3 else -- Decoder output to be connected to encoder input in scrub mode.

(others=>'0');

ram_wr_en_s      <=  USER_WEN_TRP1_REG when fsm_state_s=s1 else --User write enable to be coonnected to RAM read enable

scrub_wr_en_s when fsm_state_s=s3 else --Scrub write enable to be coonnected to RAM read enable

'0';

edac_wr_en_s     <=  '1' when fsm_state_s=s1 or fsm_state_s=s3  else

'0';

ram_rd_en_s      <=  USER_REN_TRP1 when fsm_state_s=s1 else  --User read enable to be coonnected to RAM read enable

scrub_rd_en_s when fsm_state_s=s3 else --Scrub read enable to be coonnected to RAM read enable

'0';

edac_rd_en_s     <=  '1' when fsm_state_s=s1 or fsm_state_s=s3  else

'0';

P_scurbbing: process(RCLK,MRST  )

begin

if MRST ='0' then

scrub_wr_address_s <=(others=>'0');

scrub_rd_address_s <=(others=>'0');

scrub_wr_en_s <='0';

s3_s2_en_s<='0';

elsif RCLK'event and RCLK='1' then

if fsm_state_s=s1 or fsm_state_s=s2  then

scrub_rd_address_s <=(others=>'0'); -- Initializer Scrub  read address to zero.

s3_s2_en_s<='0';  -- disable state3 to state2 transition

elsif fsm_state_s=s3 then

scrub_rd_address_s<=scrub_rd_address_s+1; --Scrub read address gets incremented

if scrub_wr_address_s="111111111" then

s3_s2_en_s<='1';           --Enable state3 to state2 transition after completing scrubbing.

else

s3_s2_en_s<='0';

end if ;

end if;

scrub_wr_address_s<= scrub_rd_address_s;

scrub_wr_en_s <= scrub_rd_en_s;

end if;

end process P_scurbbing;

P_fsm_state: process(RCLK,MRST)

begin

if MRST  = '0' then

fsm_state_s <= s1;

elsif RCLK'event and RCLK = '1' then

C_fsm_state_dec: case fsm_state_s is

when  s1=> if (USER_WEN_TRP1 ='0' and USER_REN_TRP1 ='0' and s1_s2_en_s ='1')  then

fsm_state_s <= s2;

end if;

when  s2=> if (USER_WEN_TRP1 ='1' or USER_REN_TRP1 ='1') then

fsm_state_s <= s1;

elsif s2_s3_en_s='1' then

fsm_state_s <= s3;

end if ;

when s3=>  if (USER_WEN_TRP1 ='1' or USER_REN_TRP1 ='1')  then

fsm_state_s <= s1;

elsif s3_s2_en_s='1' then

fsm_state_s <= s2;

end if;

when others=> fsm_state_s <= s1;

end case C_fsm_state_dec;

end if;

end process P_fsm_state;

scrub_timer_en_s<='1' when USER_WEN_TRP1 ='0' and USER_REN_TRP1 ='0' and fsm_state_s=s1 else

'0';

scrub_rd_en_s<= '1' when fsm_state_s=s3 else

'0';

P_scrub_timer_counter: process(RCLK,MRST)

begin

if MRST ='0' then

scrub_timer_s <=(others=>'0');

s1_s2_en_s <= '0';

elsif RCLK'event and RCLK='1' then

if scrub_timer_en_s='1' then

scrub_timer_s<=scrub_timer_s+1;

else

scrub_timer_s <=(others=>'0');

end if;

if scrub_timer_s ="1111"  then

s1_s2_en_s <= '1';

else

s1_s2_en_s <= '0';

end if;

end if;

end process P_scrub_timer_counter;

scrub_count_en_s<='1' when fsm_state_s = s2   else

'0';

-- 4msec counter

-- State3 to State2 transition enable signal generation

P_scrub_interval_counter: process(RCLK,MRST)

begin

if MRST ='0' then

s2_s3_en_s <='0';

scrub_interval_s <=(others=>'0');

elsif RCLK'event and RCLK='1' then

if scrub_count_en_s='1'  then

scrub_interval_s <= scrub_interval_s+1;

else

scrub_interval_s <=(others=>'0');

end if;

if scrub_interval_s >="01"& X"3880" then -- Value is hard coded and corresponds to 4msec.

s2_s3_en_s<='1';

else

s2_s3_en_s<='0';

end if;

end if;

end process P_scrub_interval_counter;

end mil_command_ram_a;

50) mil_data_ram_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity mil_data_ram_e is

port(     MRST             : in    std_logic;

RCLK             : in    std_logic;

USER_WEN_TRP1    : in    std_logic;

USER_REN_TRP1    : in    std_logic;

MSG_TRP1         : in    std_logic_vector(15 downto 0);

USER_WA_TRP1     : in    std_logic_vector(9 downto 0);

USER_RA_TRP1     : in    std_logic_vector(9 downto 0);

DATA_OUT         : out   std_logic_vector(15 downto 0)

);

end mil_data_ram_e;

architecture mil_data_ram_a of mil_data_ram_e is

--------------------------------------------

--9 bit EDAC encoder component declaration

--------------------------------------------

COMPONENT edac_16_enc1_e

PORT(

wr_en_is : IN std_logic;

data_in_is : IN std_logic_vector(15 downto 0);

data_out_os : OUT std_logic_vector(15 downto 0);

par_out_os : OUT std_logic_vector(5 downto 0)

);

END COMPONENT;

------------------------------------------------

--9 bit EDAC decoder component declaration

------------------------------------------------

COMPONENT edac_16_dec1_e

PORT(

rd_en_is : IN std_logic;

data_in_is : IN std_logic_vector(15 downto 0);

par_in_is : IN std_logic_vector(5 downto 0);

data_out_os : OUT std_logic_vector(15 downto 0)

);

END COMPONENT;

--------------------------------------------------------

-- Block RAM component declaration

--------------------------------------------------------

COMPONENT data_mem_core1

PORT (

clka : IN STD_LOGIC;

ena : IN STD_LOGIC;

wea : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

addra : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

dina : IN STD_LOGIC_VECTOR(21 DOWNTO 0);

clkb : IN STD_LOGIC;

enb : IN STD_LOGIC;

addrb : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

doutb : OUT STD_LOGIC_VECTOR(21 DOWNTO 0)

);

END COMPONENT;

-- Scrubbing interface signals

signal scrub_wr_en_s, scrub_rd_en_s : std_logic; --Scrub read enable, Scrub write enable

signal scrub_wr_address_s,scrub_rd_address_s : std_logic_vector(9 downto 0); --Scrub write address and Scrub read address

signal scrub_count_en_s: std_logic; -- Scrub count enable

-- RAM interface signals

signal ram_wr_en_s: std_logic; -- RAM write enable

signal ram_rd_en_s: std_logic;-- RAM read enable

signal ram_rd_address_s, ram_wr_address_s: std_logic_vector(9 downto 0); --RAM write address and read address

signal ram_din_s, ram_dout_s: std_logic_vector(21 downto 0);-- Input data to RAM, Output data from RAM

-- EDAC interface signals

signal edac_enc_out_s,edac_dec_in_s: std_logic_vector(21 downto 0);

signal edac_enc_in_s: std_logic_vector(15 downto 0);

signal edac_dec_out_s:std_logic_vector(15 downto 0);

signal edac_rd_en_s,edac_wr_en_s :std_logic;

--fsm signals

type state_type is (s1,s2,s3); --FSM State variable

signal fsm_state_s: state_type;

--25 msec timer signals

signal scrub_interval_s : std_logic_vector(19 downto 0);

--State transition control signals

signal scrub_timer_en_s :std_logic;

signal scrub_timer_s:std_logic_vector(3 downto 0);

signal s1_s2_en_s:std_logic;  -- State1 to State2 enable

signal s2_s3_en_s: std_logic; -- State2 to State3 enable

signal s3_s2_en_s:std_logic;  -- State3 to State2 enable

signal USER_WEN_TRP1_REG: std_logic; --EDAC Write enable

signal USER_WA_TRP1_REG :  std_logic_vector(9 downto 0); -- Write address

signal MSG_TRP1_REG:  std_logic_vector(15 downto 0);    -- Input to Encoder

signal wea_temp :STD_LOGIC_VECTOR(0 downto  0);

attribute FSM_ENCODING : string;

attribute FSM_ENCODING of fsm_state_s: signal is "ONE-HOT";

attribute SAFE_IMPLEMENTATION :string;

attribute SAFE_IMPLEMENTATION of fsm_state_s: signal is"yes";

begin

DATA_OUT  <=  edac_dec_out_s;

wea_temp(0) <= ram_wr_en_s;

P_write_port_registers: process(RCLK,MRST)

begin

if MRST='0' then

USER_WEN_TRP1_REG <=   '0'; --Initialize write enable

USER_WA_TRP1_REG  <=  (others=>'0');--Iniitialize write address

MSG_TRP1_REG      <=  (others=>'0');--Initialize write data

elsif RCLK'EVENT and RCLK='1' then

USER_WEN_TRP1_REG <=   USER_WEN_TRP1; -- Register for user write enable

USER_WA_TRP1_REG  <=   USER_WA_TRP1 ; -- Register for user write address

MSG_TRP1_REG      <=   MSG_TRP1;      -- Register for user write data

end if;

end process P_write_port_registers;

edac16_encoder: edac_16_enc1_e

PORT MAP(

wr_en_is => edac_wr_en_s, --encoder write enable

data_in_is => edac_enc_in_s , -- encoder: input data bits

data_out_os => edac_enc_out_s(21 downto 6), -- Information bits after encoding

par_out_os => edac_enc_out_s(5 downto 0) -- Check bits

);

edac16_decoder: edac_16_dec1_e

PORT MAP(

rd_en_is => edac_rd_en_s , -- decoder read enable

data_in_is => edac_dec_in_s(21 downto 6), -- decoder: input data bits

par_in_is => edac_dec_in_s(5 downto 0), -- decoder: input check bits

data_out_os => edac_dec_out_s -- Information bits after decoding

);

data_ram   : data_mem_core1

PORT MAP (        clka => RCLK,

ena => ram_wr_en_s,

wea => wea_temp,

addra => ram_wr_address_s,

dina => ram_din_s,

clkb => RCLK,

enb => ram_rd_en_s ,

addrb => ram_rd_address_s ,

doutb => ram_dout_s

);

-- User data and Scrub interface

ram_din_s        <=  edac_enc_out_s ; -- Encoder output connected to RAM write port.

edac_dec_in_s    <=  ram_dout_s;      -- RAM read port connected to decoder input.

ram_wr_address_s <=  USER_WA_TRP1_REG when fsm_state_s=s1 else --User write address in state1 for RAM.

scrub_wr_address_s when fsm_state_s=s3 else --Scrub write address in state2 for RAM.

(others=>'0');

ram_rd_address_s <=  USER_RA_TRP1 when fsm_state_s=s1 else --User read address in state1 for RAM.

scrub_rd_address_s when fsm_state_s=s3 else --Scrub read address in state2 for RAM.

(others=>'0');

edac_enc_in_s    <=  MSG_TRP1_REG when fsm_state_s=s1 else --User data connected to edac encoder input.

edac_dec_out_s when fsm_state_s=s3 else -- Decoder output to be connected to encoder input in scrub mode.

(others=>'0');

ram_wr_en_s      <=  USER_WEN_TRP1_REG when fsm_state_s=s1 else --User write enable to be coonnected to RAM read enable

scrub_wr_en_s when fsm_state_s=s3 else --Scrub write enable to be coonnected to RAM read enable

'0';

edac_wr_en_s     <=  '1' when fsm_state_s=s1 or fsm_state_s=s3  else

'0'; 	 ram_rd_en_s      <=  USER_REN_TRP1 when fsm_state_s=s1 else  --User read enable to be coonnected to RAM read enable

scrub_rd_en_s when fsm_state_s=s3 else --Scrub read enable to be coonnected to RAM read enable

'0';

edac_rd_en_s     <=  '1' when fsm_state_s=s1 or fsm_state_s=s3  else

'0';

P_scurbbing: process(RCLK,MRST  )

begin

if MRST ='0' then

scrub_wr_address_s <=(others=>'0');

scrub_rd_address_s <=(others=>'0');

scrub_wr_en_s <='0';

s3_s2_en_s<='0';

elsif RCLK'event and RCLK='1' then

if fsm_state_s=s1 or fsm_state_s=s2  then

scrub_rd_address_s <=(others=>'0'); -- Initializer Scrub  read address to zero.

s3_s2_en_s<='0';  -- disable state3 to state2 transition

elsif fsm_state_s=s3 then

scrub_rd_address_s<=scrub_rd_address_s+1; --Scrub read address gets incremented

if scrub_wr_address_s="1111111111" then

s3_s2_en_s<='1';           --Enable state3 to state2 transition after completing scrubbing.

else

s3_s2_en_s<='0';

end if ;

end if;

scrub_wr_address_s<= scrub_rd_address_s;

scrub_wr_en_s <= scrub_rd_en_s;

end if;

end process P_scurbbing;

P_fsm_state: process(RCLK,MRST )

begin

if MRST  = '0' then

fsm_state_s <= s1;

elsif RCLK'event and RCLK = '1' then

C_fsm_state_dec: case fsm_state_s is

when  s1=> if (USER_WEN_TRP1 ='0' and USER_REN_TRP1 ='0' and s1_s2_en_s ='1')  then

fsm_state_s <= s2;

end if;

when  s2=> if (USER_WEN_TRP1 ='1' or USER_REN_TRP1 ='1') then

fsm_state_s <= s1;

elsif s2_s3_en_s='1' then

fsm_state_s <= s3;

end if ;

when s3=>  if (USER_WEN_TRP1 ='1' or USER_REN_TRP1 ='1')  then

fsm_state_s <= s1;

elsif s3_s2_en_s='1' then

fsm_state_s <= s2;

end if;

when others=>  fsm_state_s <= s1;

end case C_fsm_state_dec;

end if;

end process P_fsm_state;

scrub_timer_en_s<='1' when USER_WEN_TRP1 ='0' and USER_REN_TRP1 ='0' and fsm_state_s=s1 else

'0';

scrub_rd_en_s<= '1' when fsm_state_s=s3 else

'0';

P_scrub_timer_counter: process(RCLK,MRST)

begin

if MRST ='0' then

scrub_timer_s <=(others=>'0');

s1_s2_en_s <= '0';

elsif RCLK'event and RCLK='1' then

if scrub_timer_en_s='1' then

scrub_timer_s<=scrub_timer_s+1;

else

scrub_timer_s <=(others=>'0');

end if;

if scrub_timer_s ="1111"  then

s1_s2_en_s <= '1';

else

s1_s2_en_s <= '0';

end if;

end if;

end process P_scrub_timer_counter;

scrub_count_en_s <='1' when fsm_state_s = s2   else

'0';

-- 25msec counter

-- State3 to State2 transition: enable signal generation

P_scrub_interval_counter: process(RCLK,MRST)

begin

if MRST ='0' then

s2_s3_en_s <='0';

scrub_interval_s <=(others=>'0');

elsif RCLK'event and RCLK='1' then

if scrub_count_en_s='1'  then

scrub_interval_s <= scrub_interval_s+1;

else

scrub_interval_s <=(others=>'0');

end if;

if scrub_interval_s >= X"7A120" then -- Value is hard coded and corresponds to 25msec.

s2_s3_en_s<='1';

else

s2_s3_en_s<='0';

end if;

end if;

end process P_scrub_interval_counter;

end mil_data_ram_a;

51) mil_descriptor_ram_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity mil_descriptor_ram_e is

port(     MRST             : in    std_logic;

RCLK             : in    std_logic;

USER_WEN_TRP1    : in    std_logic;

USER_REN_TRP1    : in    std_logic;

MSG_TRP1         : in    std_logic_vector(15 downto 0);

USER_WA_TRP1     : in    std_logic_vector(8 downto 0);

USER_RA_TRP1     : in    std_logic_vector(8 downto 0);

DATA_OUT         : out   std_logic_vector(15 downto 0)

);

end mil_descriptor_ram_e;

architecture mil_descriptor_ram_a of mil_descriptor_ram_e is

--------------------------------------------

--9 bit EDAC encoder component declaration

--------------------------------------------

COMPONENT edac_16_enc1_e

PORT(

wr_en_is : IN std_logic;

data_in_is : IN std_logic_vector(15 downto 0);

data_out_os : OUT std_logic_vector(15 downto 0);

par_out_os : OUT std_logic_vector(5 downto 0)

);

END COMPONENT;

------------------------------------------------

--9 bit EDAC decoder component declaration

------------------------------------------------

COMPONENT edac_16_dec1_e

PORT(

rd_en_is : IN std_logic;

data_in_is : IN std_logic_vector(15 downto 0);

par_in_is : IN std_logic_vector(5 downto 0);

data_out_os : OUT std_logic_vector(15 downto 0)

);

END COMPONENT;

--------------------------------------------------------

-- Block RAM component declaration

--------------------------------------------------------

COMPONENT descr_mem_core1

PORT (

clka : IN STD_LOGIC;

ena : IN STD_LOGIC;

wea : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

addra : IN STD_LOGIC_VECTOR(8 DOWNTO 0);

dina : IN STD_LOGIC_VECTOR(21 DOWNTO 0);

clkb : IN STD_LOGIC;

enb : IN STD_LOGIC;

addrb : IN STD_LOGIC_VECTOR(8 DOWNTO 0);

doutb : OUT STD_LOGIC_VECTOR(21 DOWNTO 0)

);

END COMPONENT;

-- Scrubbing interface signals

signal scrub_wr_en_s, scrub_rd_en_s : std_logic; --Scrub read enable, Scrub write enable

signal scrub_wr_address_s,scrub_rd_address_s : std_logic_vector(8 downto 0); --Scrub write address and Scrub read address

signal scrub_count_en_s: std_logic; -- Scrub count enable

-- RAM interface signals

signal ram_wr_en_s: std_logic; -- RAM write enable

signal ram_rd_en_s: std_logic;-- RAM read enable

signal ram_rd_address_s, ram_wr_address_s: std_logic_vector(8 downto 0); --RAM write address and read address

signal ram_din_s, ram_dout_s: std_logic_vector(21 downto 0);-- Input data to RAM, Output data from RAM

-- EDAC interface signals

signal edac_enc_out_s,edac_dec_in_s: std_logic_vector(21 downto 0);

signal edac_enc_in_s: std_logic_vector(15 downto 0);

signal edac_dec_out_s:std_logic_vector(15 downto 0);

signal edac_rd_en_s,edac_wr_en_s :std_logic;

--fsm signals

type state_type is (s1,s2,s3); --FSM State variable

signal fsm_state_s: state_type;

--25 msec timer signals

signal scrub_interval_s: std_logic_vector(19 downto 0);

--State transition control signals

signal scrub_timer_en_s :std_logic;

signal scrub_timer_s:std_logic_vector(3 downto 0);

signal s1_s2_en_s:std_logic;  -- State1 to State2 enable

signal s2_s3_en_s: std_logic; -- State2 to State3 enable

signal s3_s2_en_s:std_logic;  -- State3 to State2 enable

signal USER_WEN_TRP1_REG: std_logic; --EDAC Write enable

signal USER_WA_TRP1_REG :  std_logic_vector(8 downto 0); -- Write address

signal MSG_TRP1_REG:  std_logic_vector(15 downto 0);    -- Input to Encoder

signal wea_temp :STD_LOGIC_VECTOR(0 downto  0);

attribute FSM_ENCODING : string;

attribute FSM_ENCODING of fsm_state_s: signal is "ONE-HOT";

attribute SAFE_IMPLEMENTATION :string;

attribute SAFE_IMPLEMENTATION of fsm_state_s: signal is"yes";

begin

DATA_OUT  <=  edac_dec_out_s;

wea_temp(0) <= ram_wr_en_s;

P_write_port_registers: process(RCLK,MRST)

begin

if MRST='0' then

USER_WEN_TRP1_REG <=   '0'; --Initialize write enable

USER_WA_TRP1_REG  <=  (others=>'0');--Iniitialize write address

MSG_TRP1_REG      <=  (others=>'0');--Initialize write data

elsif RCLK'EVENT and RCLK='1' then

USER_WEN_TRP1_REG <=   USER_WEN_TRP1; -- Register for user write enable

USER_WA_TRP1_REG  <=   USER_WA_TRP1 ; -- Register for user write address

MSG_TRP1_REG      <=   MSG_TRP1;      -- Register for user write data

end if;

end process P_write_port_registers;

edac16_encoder: edac_16_enc1_e

PORT MAP(

wr_en_is => edac_wr_en_s, --encoder write enable

data_in_is => edac_enc_in_s , -- encoder: input data bits

data_out_os => edac_enc_out_s(21 downto 6), -- Information bits after encoding

par_out_os => edac_enc_out_s(5 downto 0) -- Check bits

);

edac16_decoder: edac_16_dec1_e

PORT MAP(

rd_en_is => edac_rd_en_s , -- decoder read enable

data_in_is => edac_dec_in_s(21 downto 6), -- decoder: input data bits

par_in_is => edac_dec_in_s(5 downto 0), -- decoder: input check bits

data_out_os => edac_dec_out_s -- Information bits after decoding

);

descr_ram : descr_mem_core1

PORT MAP (        clka => RCLK,

ena => ram_wr_en_s,

wea => wea_temp,

addra => ram_wr_address_s,

dina => ram_din_s,

clkb => RCLK,

enb => ram_rd_en_s ,

addrb => ram_rd_address_s ,

doutb => ram_dout_s

);

-- User data and Scrub interface

ram_din_s        <=  edac_enc_out_s ; -- Encoder output connected to RAM write port.

edac_dec_in_s    <=  ram_dout_s;      -- RAM read port connected to decoder input.

ram_wr_address_s <=  USER_WA_TRP1_REG when fsm_state_s=s1 else --User write address in state1 for RAM.

scrub_wr_address_s when fsm_state_s=s3 else --Scrub write address in state2 for RAM.

(others=>'0');

ram_rd_address_s <=  USER_RA_TRP1 when fsm_state_s=s1 else --User read address in state1 for RAM.

scrub_rd_address_s when fsm_state_s=s3 else --Scrub read address in state2 for RAM.

(others=>'0');

edac_enc_in_s    <=  MSG_TRP1_REG when fsm_state_s=s1 else --User data connected to edac encoder input.

edac_dec_out_s when fsm_state_s=s3 else -- Decoder output to be connected to encoder input in scrub mode.

(others=>'0');

ram_wr_en_s      <=  USER_WEN_TRP1_REG when fsm_state_s=s1 else --User write enable to be coonnected to RAM read enable

scrub_wr_en_s when fsm_state_s=s3 else --Scrub write enable to be coonnected to RAM read enable

'0';

edac_wr_en_s     <=  '1' when fsm_state_s=s1 or fsm_state_s=s3  else

'0'; 								 ram_rd_en_s      <=  USER_REN_TRP1 when fsm_state_s=s1 else  --User read enable to be coonnected to RAM read enable

scrub_rd_en_s when fsm_state_s=s3 else --Scrub read enable to be coonnected to RAM read enable

'0';

edac_rd_en_s     <=  '1' when fsm_state_s=s1 or fsm_state_s=s3  else

'0';

P_scurbbing: process(RCLK,MRST  )

begin

if MRST ='0' then

scrub_wr_address_s <=(others=>'0');

scrub_rd_address_s <=(others=>'0');

scrub_wr_en_s <='0';

s3_s2_en_s<='0';

elsif RCLK'event and RCLK='1' then

if fsm_state_s=s1 or fsm_state_s=s2  then

scrub_rd_address_s <=(others=>'0'); -- Initializer Scrub  read address to zero.

s3_s2_en_s<='0';  -- disable state3 to state2 transition

elsif fsm_state_s=s3 then

scrub_rd_address_s<=scrub_rd_address_s+1; --Scrub read address gets incremented

if scrub_wr_address_s="111111111" then

s3_s2_en_s<='1';           --Enable state3 to state2 transition after completing scrubbing.

else

s3_s2_en_s<='0';

end if ;

end if;

scrub_wr_address_s<= scrub_rd_address_s;

scrub_wr_en_s <= scrub_rd_en_s;

end if;

end process P_scurbbing;

P_fsm_state: process(RCLK,MRST)

begin

if MRST  = '0' then

fsm_state_s <= s1;

elsif RCLK'event and RCLK = '1' then

C_fsm_state_dec: case fsm_state_s is

when  s1=> if (USER_WEN_TRP1 ='0' and USER_REN_TRP1 ='0' and s1_s2_en_s ='1')  then

fsm_state_s <= s2;

end if;

when  s2=> if (USER_WEN_TRP1 ='1' or USER_REN_TRP1 ='1') then

fsm_state_s <= s1;

elsif s2_s3_en_s='1' then

fsm_state_s <= s3;

end if ;

when s3=>  if (USER_WEN_TRP1 ='1' or USER_REN_TRP1 ='1')  then

fsm_state_s <= s1;

elsif s3_s2_en_s='1' then

fsm_state_s <= s2;

end if;

when others=> fsm_state_s <= s1;

end case C_fsm_state_dec;

end if;

end process P_fsm_state;

scrub_timer_en_s<='1' when (USER_WEN_TRP1 ='0' and USER_REN_TRP1 ='0' and fsm_state_s=s1) else

'0';

scrub_rd_en_s<= '1' when fsm_state_s=s3 else

'0';

P_scrub_timer_counter: process(RCLK,MRST)

begin

if MRST ='0' then

scrub_timer_s <=(others=>'0');

s1_s2_en_s <= '0';

elsif RCLK'event and RCLK='1' then

if scrub_timer_en_s='1' then

scrub_timer_s<=scrub_timer_s+1;

else

scrub_timer_s <=(others=>'0');

end if;

if scrub_timer_s ="1111"  then

s1_s2_en_s <= '1';

else

s1_s2_en_s <= '0';

end if;  end if;

end process P_scrub_timer_counter;		scrub_count_en_s<='1' when fsm_state_s = s2   else

'0';

P_scrub_interval_counter: process(RCLK,MRST)

begin

if MRST ='0' then

s2_s3_en_s <='0';

scrub_interval_s <=(others=>'0');

elsif RCLK'event and RCLK='1' then

if scrub_count_en_s='1'  then

scrub_interval_s <= scrub_interval_s+1;

else

scrub_interval_s <=(others=>'0');

end if;

if scrub_interval_s >= X"7A120" then -- Value is hard coded and corresponds to 25msec.

s2_s3_en_s<='1';

else

s2_s3_en_s<='0';

end if;

end if;

end process P_scrub_interval_counter;

end mil_descriptor_ram_a;

52) mil_top_level_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

Library UNISIM;

use UNISIM.vcomponents.all;

entity mil_top_level_e is

port(

-- Input signals

clk_is     : in std_logic ;

reset_n_is : in std_logic ;

rd_pr_sync_is: in std_logic;

wr_pr_sync_is: in std_logic;

mil_command_ram_cs_is    : in std_logic;

mil_descriptor_ram_cs_is : in std_logic;

mil_data_ram_cs_is       : in std_logic;

mil_rt_ip_csn_is         : in std_logic;

pr_address_is    : in std_logic_vector(15 downto 0);

pr_write_data_is : in std_logic_vector(15 downto 0);

rt_address_is          : in std_logic_vector(5 downto 0);

reset_tsm_interrupt_is : in std_logic;

reset_mil_interrupt_is : in std_logic;

BUSAINP_rt_is    : in  std_logic;

BUSAINN_rt_is    : in  std_logic;

BUSBINP_rt_is    : in  std_logic;

BUSBINN_rt_is    : in  std_logic;

-- Output signals

BUSAINEN_rt_os       : out std_logic;

BUSAOUTIN_rt_os      : out std_logic;

BUSAOUTP_rt_out_os   : out std_logic; -- BUS-A Output Positive data

BUSAOUTN_rt_out_os   : out std_logic; -- BUS-A Output Negative data

BUSBINEN_rt_os       : out std_logic;

BUSBOUTIN_rt_os      : out std_logic;

BUSBOUTP_rt_out_os   : out std_logic; -- BUS-A Output Positive data

BUSBOUTN_rt_out_os   : out std_logic; -- BUS-B Output Negative data

pr_read_data_from_rt_ip_os,pr_read_data_from_descr_ram_os   : out std_logic_vector(15 downto 0);

pr_read_data_from_comm_ram_os,pr_read_data_from_data_ram_os : out std_logic_vector(15 downto 0);

tsm_interrupt_os : out std_logic;

mil_1553_interrupt_os: out std_logic

);

end mil_top_level_e;

architecture mil_top_level_a of mil_top_level_e is

component mil_descriptor_ram_e is

port(  MRST             : in    std_logic;

RCLK             : in    std_logic;

USER_WEN_TRP1    : in    std_logic;

USER_REN_TRP1    : in    std_logic;

MSG_TRP1         : in    std_logic_vector(15 downto 0);

USER_WA_TRP1     : in    std_logic_vector(8 downto 0);

USER_RA_TRP1     : in    std_logic_vector(8 downto 0);

DATA_OUT         : out   std_logic_vector(15 downto 0)

);

end component;

component mil_data_ram_e is

port(  MRST             : in    std_logic;

RCLK             : in    std_logic;

USER_WEN_TRP1    : in    std_logic;

USER_REN_TRP1    : in    std_logic;

MSG_TRP1         : in    std_logic_vector(15 downto 0);

USER_WA_TRP1     : in    std_logic_vector(9 downto 0);

USER_RA_TRP1     : in    std_logic_vector(9 downto 0);

DATA_OUT         : out   std_logic_vector(15 downto 0)

);

end component;

component mil_command_ram_e is

port( MRST             : in    std_logic;

RCLK             : in    std_logic;

USER_WEN_TRP1    : in    std_logic;

USER_REN_TRP1    : in    std_logic;

MSG_TRP1         : in    std_logic_vector(15 downto 0);

USER_WA_TRP1     : in    std_logic_vector(8 downto 0);

USER_RA_TRP1     : in    std_logic_vector(8 downto 0);

DATA_OUT         : out   std_logic_vector(15 downto 0)

);

end component;

component BRM is

generic ( BCENABLE : INTEGER range 0 to 1 := 1;

RTENABLE : INTEGER range 0 to 1 := 1;

MTENABLE : INTEGER range 0 to 1 := 1;

LEGREGS  : INTEGER range 0 to 5 := 1;

ENHANCED : INTEGER range 0 to 1 := 1;

INITFREQ : INTEGER range 12 to 24  := 24;

BETIMING : INTEGER range 0 to 1    := 0

);

port ( CLK        : in  std_logic;

TCLK       : in  std_logic;

RSTINn     : in  std_logic;

RSTOUTn    : out std_logic;

-- 1553B Interface

BUSAINEN   : out std_logic;

BUSAINP    : in  std_logic;

BUSAINN    : in  std_logic;

BUSBINEN   : out std_logic;

BUSBINP    : in  std_logic;

BUSBINN    : in  std_logic;

BUSAOUTIN  : out std_logic;

BUSAOUTP   : out std_logic;

BUSAOUTN   : out std_logic;

BUSBOUTIN  : out std_logic;

BUSBOUTP   : out std_logic;

BUSBOUTN   : out std_logic;

-- BRM Control Signals - should be tied high or low

CPUMEMEN   : in  std_logic;

ASYNCIF    : in  std_logic;

INTLEVEL   : in  std_logic;

-- Combined Status Signals

ACTIVE     : out std_logic;

BUSY       : out std_logic;

INTOUTH    : out std_logic;

INTOUTM    : out std_logic;

INTACKH    : in  std_logic;

INTACKM    : in  std_logic;

MEMFAIL    : out std_logic;

OPMODE     : out std_logic_vector(1 downto 0);

-- RT ADDRESS Setting

LOCKn      : in  std_logic;

RTADDRIN   : in  std_logic_vector(4 downto 0);

RTADDRPIN  : in  std_logic;

RTADERR    : out std_logic;

MSELIN     : in  std_logic_vector(1 downto 0);

SSYSFn     : in  std_logic;

ABSTDIN    : in  std_logic;

-- RT Status Signals

MSGSTART   : out std_logic;

CMDSYNC    : out std_logic;

SYNCNOW    : out std_logic;

BUSRESET   : out std_logic;

-- RT Command Legalization Interface

CMDSTB     : out std_logic;

CMDVAL     : out std_logic_vector(11 downto 0);

CMDOK      : in  std_logic;

CMDOKOUT   : out std_logic;

-- CPU Interface

CPUWRn     : in  std_logic_vector(1 downto 0);

CPURDn     : in  std_logic;

CPUCSn     : in  std_logic;

CPUMEM     : in  std_logic;

CPUWAITn   : out std_logic;

CPUDEN     : out std_logic;

CPUADDR    : in  std_logic_vector(15 downto 0);

CPUDOUT    : out std_logic_vector(15 downto 0);

CPUDIN     : in  std_logic_vector(15 downto 0);

-- Backend Memory interface

MEMREQn    : out std_logic;

MEMGNTn    : in  std_logic;

MEMACCn    : out std_logic;

MEMWRn     : out std_logic_vector(1 downto 0);

MEMRDn     : out std_logic;

MEMCSn     : out std_logic;

MEMWAITn   : in  std_logic;

MEMCEN     : out std_logic;

MEMDEN     : out std_logic;

MEMADDR    : out std_logic_vector(15 downto 0);

MEMDOUT    : out std_logic_vector(15 downto 0);

MEMDIN     : in  std_logic_vector(15 downto 0)

);

end component;

signal mil_wrn_in_s, mil_wrn_out_s, mil_wr_out_s : std_logic_vector(1 downto 0);

signal INTOUTM_rt_s,  mil_rdn_in_s, memcen_out_s, mil_rdn_out_s: std_logic;

signal mil_ADDR_in_s,mil_dataout_to_up_s,mil_datain_from_up_s : std_logic_vector(15 downto 0);

signal mil_addrout_to_ram_s, mil_dataout_to_ram_s, mil_datain_from_ram_s : std_logic_vector(15 downto 0);

signal we_descriptor_ram_s,  re_descriptor_ram_s : std_logic;

signal we_mil_dataram_s, re_mil_dataram_s : std_logic;

signal we_mil_command_ram_s, re_mil_command_ram_s : std_logic;

signal wr_data_descriptor_ram_s, rd_data_descriptor_ram_s : std_logic_vector(15 downto 0);

signal wr_data_mil_dataram_s, rd_data_mil_dataram_s : std_logic_vector(15 downto 0);

signal wr_data_mil_command_ram_s, rd_data_mil_command_ram_s : std_logic_vector(15 downto 0);

signal wr_address_descriptor_ram_s, rd_address_descriptor_ram_s : std_logic_vector(8 downto 0);

signal wr_address_mil_dataram_s, rd_address_mil_dataram_s : std_logic_vector(9 downto 0);

signal wr_address_mil_command_ram_s, rd_address_mil_command_ram_s : std_logic_vector(8 downto 0);

signal mil_rd_out_s, memwait_rt_s : std_logic;

signal tclk_counter_s: std_logic_vector(3 downto 0);

signal tclk_mil_s : std_logic;

signal tsm_rt_s,tsm_interrupt_ff0_s,mil_interrupt_ff0_s : std_logic;

signal reset_mil_interrupt_ff1_s,reset_mil_interrupt_ff2_s,reset_mil_interrupt_ff3_s: std_logic;

signal INTOUTM_rt_reg_s: std_logic;

begin

P_tclk_gen: process(clk_is,reset_n_is)

begin

if reset_n_is = '0' then

tclk_counter_s <= (others => '0');

elsif clk_is'event and clk_is = '0' then

tclk_counter_s <= tclk_counter_s + 1;

end if;

end process P_tclk_gen;

tclk_mil_s <= tclk_counter_s(3);

BRM_inst_rt: BRM

generic map ( BCENABLE => 0,  -- BC Function not implemented.

RTENABLE => 1,  -- RT Function implemented.

MTENABLE => 0,  -- MT Function not implemented.

LEGREGS  => 1,  -- RT Legalization registers implemented using FPGA registers.

ENHANCED => 0,  -- No enhanced mode of operation.

INITFREQ => 20, -- Core configured to 20MHz clock.

BETIMING => 0   -- Backend Memory timing set to default.

)

port map(

CLK     => clk_is,         -- 20MHz clock.

TCLK       => tclk_mil_s,  -- 1.25MHz clock (time base clock) for protocol controller.

RSTINn     => reset_n_is,  -- Active low reset.

RSTOUTn    => open,

BUSAINEN   => BUSAINEN_rt_os, -- BUS-A Input enable.

BUSAINP    => BUSAINP_rt_is,  -- BUS-A Data  input, positive.

BUSAINN    => BUSAINN_rt_is,  -- BUS-A Data  input, negative.

BUSBINEN   => BUSBINEN_rt_os, -- BUS-B Input enable.

BUSBINP    => BUSBINP_rt_is,  -- BUS-B Data input, positive.

BUSBINN    => BUSBINN_rt_is,  -- BUS-B Data input, negative.

BUSAOUTIN  => BUSAOUTIN_rt_os,     -- BUS-A Output inhibit.

BUSAOUTP   => BUSAOUTP_rt_out_os,  -- BUS-A Data output,positive.

BUSAOUTN   => BUSAOUTN_rt_out_os,  -- BUS-A Data output,negative.

BUSBOUTIN  => BUSBOUTIN_rt_os,     -- BUS-B Output Inhibit.

BUSBOUTP   => BUSBOUTP_rt_out_os,  -- BUS-B Data Output,positive.

BUSBOUTN   => BUSBOUTN_rt_out_os,  -- BUS-B Data Output,negative.

-- BRM Control Signals - should be tied high or low

CPUMEMEN   => '0', -- CPU can't access backend memory through core.

ASYNCIF    => '0', -- Backend memory interface in synchronous mode.

INTLEVEL   => '0', -- Interrupt Operating mode : INTOUTM & INTOUTH  high for 3 Clock cycles.

-- Combined Status Signals

ACTIVE     => open,

BUSY       => open ,

INTOUTH    => open,

INTOUTM    => INTOUTM_rt_s,

INTACKH    => '1',

INTACKM    => '1',

MEMFAIL    => open,

OPMODE     => open,

-- RT ADDRESS Setting

LOCKn      => '0',     -- To prevent overriding other core setup signals.

RTADDRIN   => rt_address_is(4 downto 0), -- RT Address.

RTADDRPIN  => rt_address_is(5),          -- Parity bit in RT Address.

RTADERR    => open,

MSELIN     => "01", -- Operating mode: Remote Terminal.

SSYSFn     => '1',  -- Control the subsystem Flag bit in the 1553B status word.

ABSTDIN    => '0',  -- Bus standard: MIL-STD-1553B.

-- RT Status Signals

MSGSTART   => open,

CMDSYNC    => open,

SYNCNOW    => tsm_rt_s,

BUSRESET   => open,

-- RT Command Legalization Interface

CMDSTB     => open,

CMDVAL     => open,

CMDOK      => '1',

CMDOKOUT   => open,

-- CPU Interface / LEON-3 interface.

CPUWRn     => mil_wrn_in_s,

CPURDn     => mil_rdn_in_s,

CPUCSn     => mil_rt_ip_csn_is,

CPUMEM     => '0',  -- CPU Accesses internal registers of IP Core.

CPUWAITn   => open,

CPUDEN     => open,

CPUADDR    => mil_ADDR_in_s,

CPUDOUT    => mil_dataout_to_up_s,

CPUDIN     => mil_datain_from_up_s,

-- Backend Memory interface

MEMREQn    => open,

MEMGNTn    => '0',

MEMACCn    => open,

MEMWRn     => mil_wrn_out_s,

MEMRDn     => mil_rdn_out_s,

MEMCSn     => open,

MEMWAITn   => memwait_rt_s, -- Active low, to extend memory read cycle.

MEMCEN     => memcen_out_s,

MEMDEN     => open,

MEMADDR    => mil_addrout_to_ram_s,

MEMDOUT    => mil_dataout_to_ram_s,

MEMDIN     => mil_datain_from_ram_s

);

mil_descriptor_ram_inst:  mil_descriptor_ram_e port map

( MRST             => reset_n_is,

RCLK             => clk_is,

USER_WEN_TRP1    => we_descriptor_ram_s,

USER_REN_TRP1    => re_descriptor_ram_s,

MSG_TRP1         => wr_data_descriptor_ram_s,

DATA_OUT         => rd_data_descriptor_ram_s,

USER_WA_TRP1     => wr_address_descriptor_ram_s,

USER_RA_TRP1     => rd_address_descriptor_ram_s

);

mil_data_ram_inst:  mil_data_ram_e port map

( MRST             => reset_n_is,

RCLK             => clk_is,

USER_WEN_TRP1    => we_mil_dataram_s,

USER_REN_TRP1    => re_mil_dataram_s,

MSG_TRP1         => wr_data_mil_dataram_s,

DATA_OUT         => rd_data_mil_dataram_s,

USER_WA_TRP1     => wr_address_mil_dataram_s,

USER_RA_TRP1     => rd_address_mil_dataram_s);

mil_command_ram_inst:  mil_command_ram_e port map

( MRST             => reset_n_is,

RCLK             => clk_is,

USER_WEN_TRP1    => we_mil_command_ram_s,

USER_REN_TRP1    => re_mil_command_ram_s,

MSG_TRP1         => wr_data_mil_command_ram_s,

DATA_OUT         => rd_data_mil_command_ram_s,

USER_WA_TRP1     => wr_address_mil_command_ram_s,

USER_RA_TRP1     => rd_address_mil_command_ram_s);

--Mil 1553 ip core address and data

mil_ADDR_in_s <=  pr_address_is;

mil_wrn_in_s  <=  (not wr_pr_sync_is) & (not wr_pr_sync_is) ;

mil_rdn_in_s  <=  not rd_pr_sync_is ;

-- Data from processor

mil_datain_from_up_s <= pr_write_data_is ;

-- Data to processor

pr_read_data_from_rt_ip_os     <= mil_dataout_to_up_s;

pr_read_data_from_descr_ram_os <= rd_data_descriptor_ram_s;

pr_read_data_from_comm_ram_os  <= rd_data_mil_command_ram_s;

pr_read_data_from_data_ram_os  <= rd_data_mil_dataram_s;

-- descriptor ram data and address

wr_data_descriptor_ram_s <=  mil_dataout_to_ram_s when memcen_out_s = '1' else

pr_write_data_is;

wr_address_descriptor_ram_s <=  mil_addrout_to_ram_s(8 downto 0) when memcen_out_s = '1' else

pr_address_is(8 downto 0) ;

rd_address_descriptor_ram_s <= mil_addrout_to_ram_s(8 downto 0) when memcen_out_s = '1' else

pr_address_is(8 downto 0) ;

we_descriptor_ram_s <= mil_wr_out_s(1) when memcen_out_s = '1' and mil_addrout_to_ram_s(15 downto 14) = "00" else

wr_pr_sync_is when mil_descriptor_ram_cs_is = '1' else

'0';

re_descriptor_ram_s <= mil_rd_out_s when memcen_out_s = '1' and mil_addrout_to_ram_s(15 downto 14) = "00" else

rd_pr_sync_is when mil_descriptor_ram_cs_is = '1' else

'0';

-- data ram data and address

wr_data_mil_dataram_s <= pr_write_data_is ;

wr_address_mil_dataram_s <=  pr_address_is(9 downto 0) ;

rd_address_mil_dataram_s <= mil_addrout_to_ram_s(9 downto 0) when memcen_out_s = '1' else

pr_address_is(9 downto 0) ;

we_mil_dataram_s <= wr_pr_sync_is when mil_data_ram_cs_is = '1' else

'0';

re_mil_dataram_s <= mil_rd_out_s when memcen_out_s = '1' and mil_addrout_to_ram_s(15 downto 14) = "01" else

rd_pr_sync_is when mil_data_ram_cs_is = '1' else

'0';

-- command ram address and data

wr_data_mil_command_ram_s <= mil_dataout_to_ram_s when memcen_out_s = '1' and mil_addrout_to_ram_s(15 downto 14) = "11" else

pr_write_data_is;

wr_address_mil_command_ram_s <= mil_addrout_to_ram_s(8 downto 0) when memcen_out_s = '1' and mil_addrout_to_ram_s(15 downto 14) = "11" else

pr_address_is(8 downto 0);

rd_address_mil_command_ram_s <= pr_address_is(8 downto 0);

we_mil_command_ram_s <= mil_wr_out_s(1) when memcen_out_s = '1' and mil_addrout_to_ram_s(15 downto 14) = "11" else

wr_pr_sync_is when mil_command_ram_cs_is = '1' else

'0';

re_mil_command_ram_s <= rd_pr_sync_is when mil_command_ram_cs_is = '1' else '0';

-- mil-1553 backend data interface

mil_datain_from_ram_s <= rd_data_descriptor_ram_s when mil_addrout_to_ram_s(15 downto 14) = "00" else

rd_data_mil_dataram_s when mil_addrout_to_ram_s(15 downto 14) = "01" else

(others => '0');

-- negating

mil_wr_out_s <= not mil_wrn_out_s;

mil_rd_out_s <= not mil_rdn_out_s;

-- Read cycle extension

P_read_cycle_ext: process(clk_is,reset_n_is)

begin

if reset_n_is = '0' then

memwait_rt_s <= '1' ;

elsif clk_is'event and clk_is = '0' then   -- Falling edge

if mil_rdn_out_s = '0' and memwait_rt_s = '1' then

memwait_rt_s <= '0';

elsif memwait_rt_s = '0' then

memwait_rt_s <= '1' ;

end if;

end if;

end process P_read_cycle_ext;

-- TSM interrupt clock domain

P_TSM_int: process(clk_is,reset_n_is)  -- Updated based on Lint report warning.

begin

if reset_n_is = '0' then

tsm_interrupt_ff0_s <= '0';

elsif clk_is'event and clk_is = '1' then   -- Raising edge

if(tsm_rt_s = '1') then

tsm_interrupt_ff0_s <= '1';

elsif reset_tsm_interrupt_is = '1' then

tsm_interrupt_ff0_s <= '0';

end if;

end if;

end process P_TSM_int;

tsm_interrupt_os <= tsm_interrupt_ff0_s;

--MIL-STD-1553 message interrupt

PROCESS_P2:process(clk_is,reset_n_is)  -- Updated based on Lint report warning.

begin

if reset_n_is = '0' then

reset_mil_interrupt_ff1_s <='0';

reset_mil_interrupt_ff2_s <='0';

reset_mil_interrupt_ff3_s <='0';

elsif clk_is'event and clk_is = '1' then   -- Raising edge

reset_mil_interrupt_ff1_s <= reset_mil_interrupt_is;

reset_mil_interrupt_ff2_s <= reset_mil_interrupt_ff1_s;

reset_mil_interrupt_ff3_s <= reset_mil_interrupt_ff2_s;

end if;

end process PROCESS_P2;

PROCESS_P3:process(clk_is,reset_n_is) -- Updated based on Lint report warning.

begin

if reset_n_is = '0' then

mil_interrupt_ff0_s <= '0';

INTOUTM_rt_reg_s    <= '0';

elsif clk_is'event and clk_is = '1' then   -- Raising edge

INTOUTM_rt_reg_s <= INTOUTM_rt_s;

if  reset_mil_interrupt_ff3_s = '1' then

mil_interrupt_ff0_s <= '0';

elsif INTOUTM_rt_s='1' and INTOUTM_rt_reg_s = '0' then

mil_interrupt_ff0_s <= '1';

end if;

end if;

end process PROCESS_P3;

mil_1553_interrupt_os <= mil_interrupt_ff0_s;

end mil_top_level_a;

53) ms_500_module_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity ms_500_module_e is

port(clk_is,POR_G_is: in std_logic;

--clk_enable_is : in std_logic;

data_processor_is: in std_logic_vector(31 downto 0);

TIC_intrpt_clear_is:in std_logic;

TIC_wr_enable_is : in std_logic;

clear_TIC_counter_is: in std_logic;

TIC_drift_wr_enable_is: in std_logic;

epoch_latch_pulse_os : out std_logic;

ms_500_interrupt_os: out std_logic;

--RT1

TSM_interrupt_RT1_is : in std_logic;

reset_pps_interrupt_RT1_is : in std_logic;

TSM_flag_RT1_out_os,PPS_flag_RT1_out_os : out std_logic;

TSM_data_RT1_os : out std_logic_vector(31 downto 0);

--RT2

TSM_interrupt_RT2_is : in std_logic;

reset_pps_interrupt_RT2_is : in std_logic;

TSM_flag_RT2_out_os,PPS_flag_RT2_out_os : out std_logic;

TSM_data_RT2_os : out std_logic_vector(31 downto 0);

--RT3

TSM_interrupt_RT3_is : in std_logic;

reset_pps_interrupt_RT3_is : in std_logic;

TSM_flag_RT3_out_os,PPS_flag_RT3_out_os : out std_logic;

TSM_data_RT3_os : out std_logic_vector(31 downto 0);

pps_pulse_count_data_en_is: in std_logic;

pps_on_off_is: in std_logic;

pps_signal_os: out std_logic_vector(5 downto 0));

end ms_500_module_e;

architecture ms_500_module_a of ms_500_module_e is

signal epoch_latch_pulse_delay_s: std_logic;

signal set_ce2_delay_s,set_ce2_delay2_s,set_ce2_delay3_s: std_logic;

signal count_enable2_s,clr_thirty_ms_counter_temp_s: std_logic;

signal set_ce2_s, clr_ce2_s : std_logic;

signal five_hundred_ms_counter_s, TIC_register_s,TIC_drift_register_s : std_logic_vector(31 downto 0);

signal thirty_ms_counter_sig_s:  std_logic_vector(20 downto 0);

signal TSM1_flag_ff1_s,TSM1_flag_ff2_s,TSM1_flag_ff3_s : std_logic;

signal PPS1_flag_ff1_s,PPS1_flag_ff2_s,PPS1_flag_ff3_s : std_logic;

signal pps_flag_RT1_s,TSM1_latch_pulse_s,pps_int_s : std_logic;

signal TSM2_flag_ff1_s,TSM2_flag_ff2_s,TSM2_flag_ff3_s : std_logic;

signal PPS2_flag_ff1_s,PPS2_flag_ff2_s,PPS2_flag_ff3_s : std_logic;

signal pps_flag_RT2_s,TSM2_latch_pulse_s : std_logic;

signal TSM3_flag_ff1_s,TSM3_flag_ff2_s,TSM3_flag_ff3_s : std_logic;

signal PPS3_flag_ff1_s,PPS3_flag_ff2_s,PPS3_flag_ff3_s : std_logic;

signal pps_flag_RT3_s,TSM3_latch_pulse_s : std_logic;

signal TIC_register_sum_s: std_logic_vector(31 downto 0);

signal one_ms_counter_reg_s: std_logic_vector(15 downto 0);

signal pps_pulse_en_s,clear_one_ms_counter_s,pps_pulse_count_s,pps_signal_temp_s: std_logic;

begin

P_TIC_counter: process(clk_is,POR_G_is)

begin

if POR_G_is = '1' then

five_hundred_ms_counter_s <= (others => '0');

elsif clk_is'event and clk_is = '1' then

if  clear_TIC_counter_is = '1' then

five_hundred_ms_counter_s <= (others => '0');

elsif five_hundred_ms_counter_s = TIC_register_sum_s then

five_hundred_ms_counter_s <= (others => '0');

--elsif clk_enable_is = '1' then

else

five_hundred_ms_counter_s <= five_hundred_ms_counter_s + 1;

end if;

end if;

if POR_G_is = '1' then

set_ce2_s <= '0';

elsif clk_is'event and clk_is = '1' then

if five_hundred_ms_counter_s = TIC_register_sum_s then

set_ce2_s <= '1';

else

set_ce2_s <= '0';

end if;

end if;

end process P_TIC_counter;

P_thirty_ms_pulse_counter: process(clk_is,POR_G_is)

begin

if POR_G_is = '1' then

thirty_ms_counter_sig_s <= (others => '0');

pps_signal_os <= "000000";

elsif clk_is'event and clk_is = '1' then

if clr_thirty_ms_counter_temp_s = '1' then

thirty_ms_counter_sig_s <= (others => '0');

elsif count_enable2_s = '1' then

thirty_ms_counter_sig_s <= thirty_ms_counter_sig_s + 1;

end if;

pps_signal_os <= pps_signal_temp_s & pps_signal_temp_s & pps_signal_temp_s & pps_signal_temp_s  & pps_signal_temp_s & pps_signal_temp_s ;

end if;

end process P_thirty_ms_pulse_counter;

clear_one_ms_counter_s <= '1' when one_ms_counter_reg_s = "1001110001000000" else

'0';

pps_signal_temp_s <= pps_pulse_en_s when pps_on_off_is = '1' else

'0';

P_PPS_gen: process(clk_is, POR_G_is)

begin

if POR_G_is = '1' then

one_ms_counter_reg_s <= (others => '0');

elsif clk_is'event and clk_is = '1' then

if clear_one_ms_counter_s = '1' then

one_ms_counter_reg_s <= (others => '0');

--elsif clk_enable_is = '1' and pps_pulse_en_s = '1' then

elsif pps_pulse_en_s = '1' then

one_ms_counter_reg_s <= one_ms_counter_reg_s + 1;

end if;

end if;

if POR_G_is = '1' then

pps_pulse_en_s <= '0';

elsif clk_is'event and clk_is = '1' then

if pps_pulse_count_s = '1' and set_ce2_s = '1' and set_ce2_delay_s = '0' then

pps_pulse_en_s <= '1';

elsif clear_one_ms_counter_s = '1' then

pps_pulse_en_s <= '0';

end if;

end if;

if POR_G_is = '1' then

pps_pulse_count_s <= '0';

elsif clk_is'event and clk_is = '1' then

if set_ce2_s = '1' and set_ce2_delay_s = '0' then

pps_pulse_count_s <= pps_pulse_count_s xor '1';

elsif pps_pulse_count_data_en_is = '1' then

pps_pulse_count_s <= data_processor_is(0);

end if;

end if;

if POR_G_is = '1' then

count_enable2_s <= '0' ;

set_ce2_delay_s <= '0';

set_ce2_delay2_s <= '0';

set_ce2_delay3_s <= '0';

elsif clk_is'event and clk_is = '1' then

set_ce2_delay_s <= set_ce2_s;

set_ce2_delay2_s <= set_ce2_delay_s;

set_ce2_delay3_s <= set_ce2_delay2_s;

if set_ce2_delay3_s = '1' then

count_enable2_s <= '1';

elsif clr_ce2_s = '1' then

count_enable2_s <= '0';

end if;

end if;

if POR_G_is = '1' then

clr_thirty_ms_counter_temp_s <= '0';

epoch_latch_pulse_os <= '0';

epoch_latch_pulse_delay_s <= '0';

elsif clk_is'event and clk_is = '1' then

clr_thirty_ms_counter_temp_s <= clr_ce2_s;

epoch_latch_pulse_delay_s <= set_ce2_s;

epoch_latch_pulse_os <= epoch_latch_pulse_delay_s ;

end if;

if POR_G_is = '1' then

TIC_register_s <= x"01312CFF";

elsif clk_is'event and clk_is = '1' then

if TIC_wr_enable_is = '1' then

TIC_register_s <= data_processor_is;

end if;

end if;

if POR_G_is = '1' then

TIC_drift_register_s <= x"00000000";

elsif clk_is'event and clk_is = '1' then

if TIC_drift_wr_enable_is = '1' then

TIC_drift_register_s <= data_processor_is;

end if;

end if;

if POR_G_is = '1' then

TIC_register_sum_s <= x"01312CFF";

elsif clk_is'event and clk_is = '1' then

TIC_register_sum_s <= TIC_register_s + TIC_drift_register_s;

end if;

end process P_PPS_gen;

ms_500_interrupt_os <= count_enable2_s;

clr_ce2_s <= '1'  when thirty_ms_counter_sig_s = "100100100111110000000" else

'1' when TIC_intrpt_clear_is='1' else

'1' when clear_TIC_counter_is='1' else

'0';

pps_int_s <= set_ce2_delay_s;

--RT1

P_TSM_PPS_flag_gen: process(clk_is,POR_G_is)

begin

if POR_G_is = '1' then

TSM1_flag_ff1_s <= '0';

TSM1_flag_ff2_s <= '0';

TSM1_flag_ff3_s <= '0';

PPS1_flag_ff1_s <= '0';

PPS1_flag_ff2_s <= '0';

PPS1_flag_ff3_s <= '0';

elsif clk_is'event and clk_is = '1' then

TSM1_flag_ff1_s <= TSM_interrupt_RT1_is;

TSM1_flag_ff2_s <= TSM1_flag_ff1_s;

TSM1_flag_ff3_s <= TSM1_flag_ff2_s;

PPS1_flag_ff1_s <= pps_flag_RT1_s;

PPS1_flag_ff2_s <= PPS1_flag_ff1_s;

PPS1_flag_ff3_s <= PPS1_flag_ff2_s;

end if;

if POR_G_is = '1' then

TSM_data_RT1_os <= (others => '0');

elsif clk_is'event and clk_is = '1' then

if TSM1_latch_pulse_s = '1' then

TSM_data_RT1_os <= five_hundred_ms_counter_s;

end if;

end if;

end process P_TSM_PPS_flag_gen;

TSM_flag_RT1_out_os <= TSM1_flag_ff3_s;

PPS_flag_RT1_out_os <= PPS1_flag_ff3_s;

TSM1_latch_pulse_s <= '1' when TSM1_flag_ff3_s = '0' and TSM1_flag_ff2_s = '1' else '0';

P_PPS_flag_RT1_latch: process(POR_G_is,clk_is)

begin

if POR_G_is = '1' then

pps_flag_RT1_s <= '0' ;

elsif clk_is'event and clk_is = '1' then

if reset_pps_interrupt_RT1_is = '1' then

pps_flag_RT1_s <= '0' ;

elsif pps_int_s = '1' then

pps_flag_RT1_s <= '1' ;

end if;

end if;

end process P_PPS_flag_RT1_latch;

--RT2

process_P1:process(clk_is,POR_G_is)

begin

if POR_G_is = '1' then

TSM2_flag_ff1_s <= '0';

TSM2_flag_ff2_s <= '0';

TSM2_flag_ff3_s <= '0';

PPS2_flag_ff1_s <= '0';

PPS2_flag_ff2_s <= '0';

PPS2_flag_ff3_s <= '0';

elsif clk_is'event and clk_is = '1' then

TSM2_flag_ff1_s <= TSM_interrupt_RT2_is;

TSM2_flag_ff2_s <= TSM2_flag_ff1_s;

TSM2_flag_ff3_s <= TSM2_flag_ff2_s;

PPS2_flag_ff1_s <= pps_flag_RT2_s;

PPS2_flag_ff2_s <= PPS2_flag_ff1_s;

PPS2_flag_ff3_s <= PPS2_flag_ff2_s;

end if;

if POR_G_is = '1' then

TSM_data_RT2_os <= (others => '0');

elsif clk_is'event and clk_is = '1' then

if TSM2_latch_pulse_s = '1' then

TSM_data_RT2_os <= five_hundred_ms_counter_s;

end if;

end if;

end process process_P1;

TSM_flag_RT2_out_os <= TSM2_flag_ff3_s;

PPS_flag_RT2_out_os <= PPS2_flag_ff3_s;

TSM2_latch_pulse_s <= '1' when TSM2_flag_ff3_s = '0' and TSM2_flag_ff2_s = '1' else '0';

process_P2:process(POR_G_is,clk_is)

begin

if POR_G_is = '1' then

pps_flag_RT2_s <= '0' ;

elsif clk_is'event and clk_is = '1' then

if reset_pps_interrupt_RT2_is = '1' then

pps_flag_RT2_s <= '0' ;

elsif pps_int_s = '1' then

pps_flag_RT2_s <= '1' ;

end if;

end if;

end process process_P2;

--RT3

process_P3:process(clk_is,POR_G_is)

begin

if POR_G_is = '1' then

TSM3_flag_ff1_s <= '0';

TSM3_flag_ff2_s <= '0';

TSM3_flag_ff3_s <= '0';

PPS3_flag_ff1_s <= '0';

PPS3_flag_ff2_s <= '0';

PPS3_flag_ff3_s <= '0';

elsif clk_is'event and clk_is = '1' then

TSM3_flag_ff1_s <= TSM_interrupt_RT3_is;

TSM3_flag_ff2_s <= TSM3_flag_ff1_s;

TSM3_flag_ff3_s <= TSM3_flag_ff2_s;

PPS3_flag_ff1_s <= pps_flag_RT3_s;

PPS3_flag_ff2_s <= PPS3_flag_ff1_s;

PPS3_flag_ff3_s <= PPS3_flag_ff2_s;

end if;

if POR_G_is = '1' then

TSM_data_RT3_os <= (others => '0');

elsif clk_is'event and clk_is = '1' then

if TSM3_latch_pulse_s = '1' then

TSM_data_RT3_os <= five_hundred_ms_counter_s;

end if;

end if;

end process process_P3;

TSM_flag_RT3_out_os <= TSM3_flag_ff3_s;

PPS_flag_RT3_out_os <= PPS3_flag_ff3_s;

TSM3_latch_pulse_s <= '1' when TSM3_flag_ff3_s = '0' and TSM3_flag_ff2_s = '1' else '0';

process_P4:process(POR_G_is,clk_is)

begin

if POR_G_is = '1' then

pps_flag_RT3_s <= '0' ;

elsif clk_is'event and clk_is = '1' then

if reset_pps_interrupt_RT3_is = '1' then

pps_flag_RT3_s <= '0' ;

elsif pps_int_s = '1' then

pps_flag_RT3_s <= '1' ;

end if;

end if;

end process process_P4;

end ms_500_module_a;

54) mult_16_by_16_sqr

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_SIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity mult_16_by_16_sqr is

Port ( Sys_CLK_40M_is : in  STD_LOGIC;

POR_RST_is : in  STD_LOGIC;

Data_in_16_bit1_is : in  STD_LOGIC_VECTOR (22 downto 0);

Data_in_16_bit2_is : in  STD_LOGIC_VECTOR (22 downto 0);

Data_out_32_bit_os : out  STD_LOGIC_VECTOR (45 downto 0)

);

end mult_16_by_16_sqr;

architecture Behavioral of mult_16_by_16_sqr is

signal result1_s  : std_logic_vector(45 downto 0);

signal result2_s : std_logic_vector(46 downto 0);

signal result3_s  : std_logic_vector(47 downto 0);

begin

P_mult: process(Sys_CLK_40M_is,POR_RST_is )

begin

if POR_RST_is='1' then

Data_out_32_bit_os <= (others => '0');

result1_s<= (others => '0');

result2_s<= (others => '0');

result3_s<= (others => '0');

elsif Sys_CLK_40M_is='1' and Sys_CLK_40M_is'event then

result1_s <= Data_in_16_bit1_is * Data_in_16_bit1_is +  Data_in_16_bit2_is * Data_in_16_bit2_is;

result2_s <=  Data_in_16_bit1_is * sxt(Data_in_16_bit1_is,24) +  Data_in_16_bit2_is * sxt(Data_in_16_bit2_is,24);

result3_s <=  sxt(Data_in_16_bit1_is,25) * Data_in_16_bit1_is +  sxt(Data_in_16_bit2_is,25) * Data_in_16_bit2_is;

if(result1_s = result2_s(45 downto 0) ) then

Data_out_32_bit_os <= result1_s;

elsif(result2_s(45 downto 0) = result3_s(45 downto 0) ) then

Data_out_32_bit_os <= result2_s(45 downto 0);

elsif(result1_s = result3_s(45 downto 0)) then

Data_out_32_bit_os <= result3_s(45 downto 0);

else

Data_out_32_bit_os <= result1_s;

end if;

end if;

end process P_mult;

end Behavioral;

55) mult_32_by_17

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_SIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity mult_32_by_17 is

Port ( Sys_CLK_40M_is : in  STD_LOGIC;

POR_RST_is : in  STD_LOGIC;

Data_in_32_bit_is : in  STD_LOGIC_VECTOR (31 downto 0);

Data_in_17_bit_is : in  STD_LOGIC_VECTOR (16 downto 0);

Data_out_49_bit_os : out  STD_LOGIC_VECTOR (48 downto 0) );

end mult_32_by_17;

architecture Behavioral of mult_32_by_17 is

signal result1_s  : std_logic_vector(48 downto 0);

signal result2_s, result3_s   : std_logic_vector(49 downto 0);

begin

P_mult: process(Sys_CLK_40M_is,POR_RST_is )

begin

if POR_RST_is='1' then

Data_out_49_bit_os <= (others => '0');

result1_s<= (others => '0');

result2_s<= (others => '0');

result3_s<= (others => '0');

elsif Sys_CLK_40M_is='1' and Sys_CLK_40M_is'event then

result1_s <= Data_in_32_bit_is * Data_in_17_bit_is;

result2_s <= Data_in_32_bit_is * sxt(Data_in_17_bit_is,18);

result3_s <= sxt(Data_in_32_bit_is,33) * Data_in_17_bit_is;

if(result1_s = result2_s(48 downto 0) ) then

Data_out_49_bit_os <= result1_s;

elsif(result2_s = result3_s) then

Data_out_49_bit_os <= result2_s(48 downto 0);

elsif(result1_s = result3_s(48 downto 0)) then

Data_out_49_bit_os <= result3_s(48 downto 0);

else

Data_out_49_bit_os <= result1_s;

end if;

end if;

end process P_mult;

end Behavioral;

56) mult_49_by_27

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_SIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity mult_49_by_27 is

Port ( Sys_CLK_40M_is : in  STD_LOGIC;

POR_RST_is : in  STD_LOGIC;

Data_in_49_bit_is : in  STD_LOGIC_VECTOR (48 downto 0);

Data_in_27_bit_is : in  STD_LOGIC_VECTOR (26 downto 0);

Data_out_76_bit_os : out  STD_LOGIC_VECTOR (75 downto 0)

);

end mult_49_by_27;

architecture Behavioral of mult_49_by_27 is

signal result1_s  : std_logic_vector(75 downto 0);

signal result2_s, result3_s   : std_logic_vector(76 downto 0);

begin

P_mult: process(Sys_CLK_40M_is,POR_RST_is )

begin

if POR_RST_is='1' then

Data_out_76_bit_os <= (others => '0');

result1_s<= (others => '0');

result2_s<= (others => '0');

result3_s<= (others => '0');

elsif Sys_CLK_40M_is='1' and Sys_CLK_40M_is'event then

result1_s <= Data_in_49_bit_is * Data_in_27_bit_is ;

result2_s <= Data_in_49_bit_is * sxt(Data_in_27_bit_is,28) ;

result3_s <= sxt(Data_in_49_bit_is,50) * Data_in_27_bit_is ;

if(result1_s = result2_s(75 downto 0) ) then

Data_out_76_bit_os <= result1_s;

elsif(result2_s = result3_s) then

Data_out_76_bit_os <= result2_s(75 downto 0);

elsif(result1_s = result3_s(75 downto 0)) then

Data_out_76_bit_os <= result3_s(75 downto 0);

else

Data_out_76_bit_os <= result1_s ;

end if;  end if;

end process P_mult;

end Behavioral;

57) pulse_deglitch_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.std_logic_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity pulse_deglitch_e  is

Port ( clk_40MHz_is : in  STD_LOGIC;  -- Clock

reset_is     : in  STD_LOGIC;      -- Reset

pulse_cmd_is : in  STD_LOGIC;      -- Input pulse command

pulse_deg_os : out  STD_LOGIC      -- Deglitched pulse output

);

end pulse_deglitch_e ;

architecture pulse_deglitch_a of pulse_deglitch_e is

signal count1_s: STD_LOGIC_VECTOR(17 downto 0); --18 bit counter

signal count2_s: STD_LOGIC_VECTOR(17 downto 0); --18 bit counter

signal count3_s: STD_LOGIC_VECTOR(1 downto 0); --6 bit counter

signal count1_en_s,count2_en_s,count3_en_s: STD_LOGIC; -- counter1,counter2 and counter3 enable signals

signal pulse_deg_s: STD_LOGIC;  -- temporary output

begin

Counter_1:process(clk_40MHz_is,reset_is)

begin

if reset_is='1' then

count1_s  <= (OTHERS=>'0');

elsif clk_40MHz_is'event and clk_40MHz_is='1' then

if pulse_cmd_is='0' or count1_en_s='0' then

count1_s<= (OTHERS=>'0');

elsif count1_en_s='1' then

count1_s <= count1_s+1;

end if;

end if;

end process Counter_1;

Counter_2:process(clk_40MHz_is,reset_is)

begin

if reset_is='1' then

count2_s  <= (OTHERS=>'0');

elsif clk_40MHz_is'event and clk_40MHz_is='1' then

if pulse_cmd_is='1' or count2_en_s='0' then

count2_s<= (OTHERS=>'0');

elsif count2_en_s='1' then

count2_s <= count2_s+1;

end if;

end if;

end process Counter_2;

--Generate Control singals for Counter1 and Counter 2

P_control_gen:   process(clk_40MHz_is,reset_is)

begin

if reset_is='1' then

count1_en_s<='1';

count2_en_s<='0';

count3_en_s<='0';

elsif clk_40MHz_is'EVENT and clk_40MHz_is='1' then

if count1_s >="11" & X"E800" then -- count1 3E800 corresponds to 6.4 msec.

count1_en_s<='0';           -- Disable counter1.

count2_en_s<='1';           -- Enable counter2.

elsif count2_s >= "11"& X"E800" then

count1_en_s<='1';           -- Enable counter1 only after detecting low level.

count2_en_s<='0';           -- Disable counter2 only after detecting low level.

end if;

if count1_s >="11" & X"E800" then -- count1 3E800 corresponds to 6.4 msec.

count3_en_s<='1';           -- Enable counter3.

elsif count3_s = "11"  then  -- FF corresponds to approx 100 nsec pulse.

count3_en_s <='0';

end if;

-- Disable counter2 after generating the pulse.

end if;

end process P_control_gen;

--2-bit counter required to generate glitch-free signal of 100 nsec duration

Counter_3: process(clk_40MHz_is,reset_is)

begin

if reset_is='1' then

count3_s<= (OTHERS=>'0');

pulse_deg_s<='0';

elsif clk_40MHz_is'EVENT and clk_40MHz_is='1' then

if count3_en_s='1' then

count3_s<= count3_s+1;

pulse_deg_s <='1';

else

count3_s <= (OTHERS=>'0');

pulse_deg_s <='0';

end if;

end if;

end process Counter_3;

pulse_deg_os <= pulse_deg_s;

end pulse_deglitch_a;

58) radix2_2048pt_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_SIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity radix2_2048pt_e is

generic(

input_width_g      : integer ;

fft_size_log_g     : integer ;

radix2_size1_g     : integer ;

radix2_size2_g     : integer

);

port (

-- Clock and reset inputs.

clk_is: in std_logic;

reset_is: in std_logic;

-- Radix-2 butterfly inputs

x0_re_is: in std_logic_vector(radix2_size1_g-1 downto 0);

x0_im_is: in std_logic_vector(radix2_size1_g-1 downto 0);

x1_re_is: in std_logic_vector(radix2_size1_g-1 downto 0);

x1_im_is: in std_logic_vector(radix2_size1_g-1 downto 0);

twiddle_re_is: in std_logic_vector(15 downto 0); --16-bit twiddle/phase factor: real input

twiddle_im_is: in std_logic_vector(15 downto 0); --16-bit twiddle/phase factor: imaginary input

-- Radix-2 butterfly outputs

Y0_RE_OS: out std_logic_vector(radix2_size1_g-1 downto 0);

Y0_IM_OS: out std_logic_vector(radix2_size1_g-1 downto 0);

Y1_RE_OS: out std_logic_vector(radix2_size1_g-1 downto 0);

Y1_IM_OS: out std_logic_vector(radix2_size1_g-1 downto 0)

);

end radix2_2048pt_e ;

architecture radix2_2048pt_a of radix2_2048pt_e is

signal X0_RE_TEMP1_S,X0_IM_TEMP1_S : std_logic_vector(radix2_size2_g-1 downto 0);

signal X1_RE_TEMP1_S,X1_IM_TEMP1_S  : std_logic_vector(radix2_size2_g-1 downto 0);

signal Y0_RE_S,Y0_IM_S,Y1_RE_S,Y1_IM_S : std_logic_vector(radix2_size2_g-1 downto 0);

signal x0_re_reg_s,x0_im_reg_s: std_logic_vector(radix2_size1_g-1 downto 0);

signal twiddle_re_reg_s , twiddle_im_reg_s: std_logic_vector(15 downto 0);

begin

P_butter_fly:	process(clk_is,reset_is )

begin

if reset_is = '1' then

-- Initialize the Radix-2 butterfly outputs with zeros.

Y0_RE_S <= (others=>'0');

Y0_IM_S <= (others=>'0');

Y1_RE_S <= (others=>'0');

Y1_IM_S <= (others=>'0');

X0_RE_TEMP1_S<= (others=>'0');

X0_IM_TEMP1_S<= (others=>'0');

X1_RE_TEMP1_S<= (others=>'0');

X1_IM_TEMP1_S<= (others=>'0');

x0_re_reg_s <= (others=>'0');

x0_im_reg_s <= (others=>'0');

twiddle_re_reg_s <= (others=>'0');

twiddle_im_reg_s <= (others=>'0');

elsif clk_is'event and clk_is='1' then

x0_re_reg_s <= x0_re_is;

x0_im_reg_s <= x0_im_is;

twiddle_re_reg_s <= twiddle_re_is;

twiddle_im_reg_s <= twiddle_im_is;

Y0_RE_S  <= ( x0_re_reg_s(radix2_size1_g-1) & x0_re_reg_s & "000" & X"000") + X0_RE_TEMP1_S + X1_RE_TEMP1_S ;

Y0_IM_S  <= ( x0_im_reg_s(radix2_size1_g-1) & x0_im_reg_s & "000" & X"000") - X0_IM_TEMP1_S + X1_IM_TEMP1_S;

Y1_RE_S  <= ( x0_re_reg_s(radix2_size1_g-1) & x0_re_reg_s & "000" & X"000") - X0_RE_TEMP1_S - X1_RE_TEMP1_S;

Y1_IM_S  <= ( x0_im_reg_s(radix2_size1_g-1) & x0_im_reg_s & "000" & X"000") + X0_IM_TEMP1_S - X1_IM_TEMP1_S;

--Radix-2 butterfly top output.

X0_RE_TEMP1_S<=  (x1_re_is*twiddle_re_reg_s);

X0_IM_TEMP1_S<=  (x1_re_is*twiddle_im_reg_s);

X1_RE_TEMP1_S<=  (x1_im_is*twiddle_im_reg_s);

X1_IM_TEMP1_S<=  (x1_im_is*twiddle_re_reg_s);

end if;

end process P_butter_fly;

Y0_RE_OS  <=  Y0_RE_S (radix2_size2_g-2 downto 15) ;

Y0_IM_OS  <=  Y0_IM_S (radix2_size2_g-2 downto 15) ;

Y1_RE_OS  <=  Y1_RE_S (radix2_size2_g-2 downto 15) ;

Y1_IM_OS  <=  Y1_IM_S (radix2_size2_g-2 downto 15) ;

end radix2_2048pt_a;

59) rd_wr_sync_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

Library UNISIM;

use UNISIM.vcomponents.all;

entity rd_wr_sync_e is

port(

--Inputs

clk_40MHz_is: in std_logic; -- 40MHz Sytem Clock

clk_20MHz_is: in std_logic; -- 20MHz MIL Clock

reset_is    : in std_logic; -- POR Global reset.

oen_is    : in std_logic;

writen_is : in std_logic;

iosn_is   : in std_logic;

correlator_cs_is: in std_logic;

mil_rt1_ip_csn_is, mil_data_ram1_cs_is,mil_command_ram1_cs_is, mil_descriptor_ram1_cs_is: in std_logic;

mil_rt2_ip_csn_is, mil_data_ram2_cs_is,mil_command_ram2_cs_is, mil_descriptor_ram2_cs_is: in std_logic;

mil_rt3_ip_csn_is, mil_data_ram3_cs_is,mil_command_ram3_cs_is, mil_descriptor_ram3_cs_is: in std_logic;

-- Outputs

fpga_we_os, mil_we_os : out std_logic;

mil_oe_os : out std_logic;

brdy_os : out std_logic; -- BRDY signal to LEON-3 Processor.

prom_cs_is : in std_logic;

prom_re_os: out std_logic

);

end rd_wr_sync_e;

architecture rd_wr_sync_a of rd_wr_sync_e is

signal fpga_we_ff_s : std_logic_vector(5 downto 0);

signal mil_we_ff_s : std_logic_vector(4 downto 0);

signal fpga_re_ff_s : std_logic_vector(4 downto 0);

signal mil_re_ff_s: std_logic_vector(3 downto 0);

signal brdy_we_fpga_s, brdy_we_mil_s : std_logic;

signal brdy_re_fpga_s, brdy_re_mil_s : std_logic;

signal fpga_reg_cs_s, mil_cs_s : std_logic;

signal mil_cs1_s, mil_cs2_s ,mil_cs3_s : std_logic;

signal write_en_fpga_s, write_en_mil_s    : std_logic;

signal read_en_fpga_s, read_en_mil_s      : std_logic;

signal prom_re_ff_s: std_logic_vector(2 downto 0);

signal prom_re_s: std_logic;

begin

-- Chip select signals for MIL and Correlator blocks.

fpga_reg_cs_s <=  correlator_cs_is ;

mil_cs1_s <=  (not mil_rt1_ip_csn_is ) or mil_data_ram1_cs_is or mil_command_ram1_cs_is or mil_descriptor_ram1_cs_is ;

mil_cs2_s <=  (not mil_rt2_ip_csn_is ) or mil_data_ram2_cs_is or mil_command_ram2_cs_is or mil_descriptor_ram2_cs_is ;

mil_cs3_s <=  (not mil_rt3_ip_csn_is ) or mil_data_ram3_cs_is or mil_command_ram3_cs_is or mil_descriptor_ram3_cs_is ;

mil_cs_s <=    mil_cs1_s or mil_cs2_s or  mil_cs3_s;

write_en_fpga_s <= not (writen_is or iosn_is) and fpga_reg_cs_s  ;

read_en_fpga_s  <= not (oen_is    or iosn_is) and fpga_reg_cs_s  ;

write_en_mil_s <=  not (writen_is or iosn_is) and mil_cs_s  ;

read_en_mil_s  <=  not (oen_is    or iosn_is) and mil_cs_s  ;

prom_re_s <= not (oen_is    or prom_cs_is);

--prom rd signal gen

process_P:process(clk_40MHz_is, reset_is)

begin

if reset_is = '1'  then

prom_re_ff_s<=(others=>'0');

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

prom_re_ff_s(0) <= prom_re_s;

prom_re_ff_s(1) <= prom_re_ff_s(0);

prom_re_ff_s(2) <= prom_re_ff_s(1);

end if;

end process process_P;

prom_re_os <= '1' when prom_re_ff_s(2) = '1' else

'0';

-- fpga we signal gen

Process_FPGA_we_gen:     process(clk_40MHz_is, reset_is)

begin

if reset_is = '1'  then

fpga_we_ff_s<=(others=>'0');

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

fpga_we_ff_s(0) <= write_en_fpga_s;

fpga_we_ff_s(1) <= fpga_we_ff_s(0);

fpga_we_ff_s(2) <= fpga_we_ff_s(1);

fpga_we_ff_s(3) <= fpga_we_ff_s(2);

fpga_we_ff_s(4) <= fpga_we_ff_s(3);

fpga_we_ff_s(5) <= fpga_we_ff_s(4);

end if;

end process Process_FPGA_we_gen;

fpga_we_os <= '1' when fpga_we_ff_s(1) = '1' and fpga_we_ff_s(3) = '0'  else

'0'; -- FPGA Write Enable asserted for two clock cycles.

brdy_we_fpga_s <= '0' when fpga_we_ff_s(3) = '1' and fpga_we_ff_s(5) = '0' else

'1'; --  BRYD asserted for two clock cycles.

--fpga rd signal gen

Process_FPGA_rd_gen: process(clk_40MHz_is, reset_is)

begin

if reset_is = '1'  then

fpga_re_ff_s<=(others=>'0');

elsif clk_40MHz_is'event and clk_40MHz_is = '1' then

fpga_re_ff_s(0) <= read_en_fpga_s;

fpga_re_ff_s(1) <= fpga_re_ff_s(0);

fpga_re_ff_s(2) <= fpga_re_ff_s(1);

fpga_re_ff_s(3) <= fpga_re_ff_s(2);

fpga_re_ff_s(4) <= fpga_re_ff_s(3);

end if;

end process Process_FPGA_rd_gen;

brdy_re_fpga_s <='0' when fpga_re_ff_s(2) = '1' and fpga_re_ff_s(4) = '0' else

'1'; --BRDY asserted for two clock cycles.

--mil rd signal gen

Process_mil_rd_gen:   process(clk_20MHz_is,reset_is)

begin

if reset_is = '1'  then

mil_re_ff_s<=(others=>'0');

elsif clk_20MHz_is'event and clk_20MHz_is = '1' then

mil_re_ff_s(0) <= read_en_mil_s;

mil_re_ff_s(1) <= mil_re_ff_s(0);

mil_re_ff_s(2) <= mil_re_ff_s(1);

mil_re_ff_s(3) <= mil_re_ff_s(2);

end if;

end process Process_mil_rd_gen;

mil_oe_os <= '1'  when mil_re_ff_s(1)='1' and mil_re_ff_s(3)='0' else

'0'; -- MIL Read Enable asserted for two 20MHz clock cycles.

brdy_re_mil_s <= '0' when mil_re_ff_s(2) = '1' and mil_re_ff_s(3) = '0' else

'1'; -- BRDY asserted for one 20 MHz clock cycle.

-- mil we signal gen

Process_Mil_we_gen:   process(clk_20MHz_is, reset_is)

begin

if reset_is = '1'  then

mil_we_ff_s<=(others=>'0');

elsif clk_20MHz_is'event and clk_20MHz_is = '1' then

mil_we_ff_s(0) <= write_en_mil_s;

mil_we_ff_s(1) <= mil_we_ff_s(0);

mil_we_ff_s(2) <= mil_we_ff_s(1);

mil_we_ff_s(3) <= mil_we_ff_s(2);

mil_we_ff_s(4) <= mil_we_ff_s(3);

end if;

end process Process_Mil_we_gen;

mil_we_os <= '1' when mil_we_ff_s(1) = '1' and mil_we_ff_s(3) = '0'  else

'0'; -- MIL Write Enable asserted for two 20MHz clock cycles.

brdy_we_mil_s <= '0' when mil_we_ff_s(3) = '1' and mil_we_ff_s(4) = '0' else

'1'; -- BRDY asserted for one 20 MHz clock cycle.

-- BRDY signal gen

brdy_os <= '0' when brdy_we_fpga_s = '0' else

'0' when brdy_re_fpga_s = '0' else

'0' when brdy_we_mil_s = '0'  else

'0' when brdy_re_mil_s = '0'  else

'1';

end rd_wr_sync_a;

60) reset_sync_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

Library UNISIM;

use UNISIM.vcomponents.all;

entity reset_sync_e is

port(

--Inputs

clk_40MHz_is: in std_logic;   -- 40 MHz System Clock.

clk_20MHz_is: in std_logic;   -- 20 MHz Clock for MIL-IP Core.

soft_reset_is : in std_logic; -- Reset from LEON-3 processor.

-- Outputs

POR_sync_os : out std_logic;

POR_G_sync_os: out std_logic;

mil_reset_os : out std_logic

);

end reset_sync_e;

architecture reset_sync_a of reset_sync_e is

signal por_ff1_s,por_ff2_s,por_ff3_s : std_logic ;

signal por_sync_s: std_logic;

signal POR_G_sync_s,POR_G_sync_temp_s : std_logic;

signal por_g_ff1_s, por_g_ff2_s,por_g_ff3_s :std_logic;

signal por_ff_n_s, por_n_s: std_logic;

signal n_por_20m_ff1_s : std_logic;

signal por_count_s: std_logic_vector(3 downto 0);

signal por_count_en_s: std_logic;

signal por_n_en_s: std_logic;

signal reset_n_s   : std_logic ;

signal mil_reset_s : std_logic;

attribute SHIFT_EXTRACT: string;

attribute SHIFT_EXTRACT of por_g_ff1_s: signal is "NO";

attribute SHIFT_EXTRACT of por_g_ff2_s: signal is "NO";

attribute SHIFT_EXTRACT of por_g_ff3_s: signal is "NO";

attribute SHIFT_EXTRACT of POR_G_sync_s: signal is "NO";

attribute SHIFT_EXTRACT of n_por_20m_ff1_s: signal is "NO";

attribute SHIFT_EXTRACT of reset_n_s: signal is "NO";

attribute SHIFT_EXTRACT of por_n_s: signal is "NO";

attribute SHIFT_EXTRACT of por_ff_n_s: signal is "NO";

begin

-- Power-on Reset Synchroniser.

-- POR_FF1 Instantiation.

por_ff1_inst : FDCE

generic map (

INIT => '1') -- Initial value of register ('0' or '1')

port map (

Q => por_ff1_s,      -- Data output

C => clk_40MHz_is,      -- Clock input

CE => '1',    -- Clock enable input

CLR => '0', -- Asynchronous clear input

D => '0'      -- Data input

);

-- POR_FF2 Instantiation.

por_ff2_inst : FDCE

generic map (

INIT => '1') -- Initial value of register ('0' or '1')

port map (

Q => por_ff2_s,      -- Data output

C => clk_40MHz_is,      -- Clock input

CE => '1',    -- Clock enable input

CLR => '0', -- Asynchronous clear input

D => por_ff1_s     -- Data input

);

-- POR_FF3 Instantiation.

por_ff3_inst : FDCE

generic map (

INIT => '1') -- Initial value of register ('0' or '1')

port map (

Q => por_ff3_s,      -- Data output

C => clk_40MHz_is,      -- Clock input

CE => '1',    -- Clock enable input

CLR => '0', -- Asynchronous clear input

D => por_ff2_s     -- Data input

);

-- POR_Sync Instantiation.

por_sync_inst : FDCE

generic map (

INIT => '1') -- Initial value of register ('0' or '1')

port map (

Q => por_sync_s,      -- Data output

C => clk_40MHz_is,      -- Clock input

CE => '1',    -- Clock enable input

CLR => '0', -- Asynchronous clear input

D => por_ff3_s     -- Data input

);

POR_sync_os <= por_sync_s;

-- Power-on Reset(Global) Synchroniser.

P_POR_sync:   process(clk_40MHz_is,por_sync_s)

begin

if por_sync_s='1' then

por_g_ff1_s <='1';

por_g_ff2_s <='1';

por_g_ff3_s <='1';

POR_G_sync_s<='1';

elsif clk_40MHz_is'event and clk_40MHz_is='1' then

if soft_reset_is='1' then

por_g_ff1_s <='1';

por_g_ff2_s <='1';

por_g_ff3_s <='1';

POR_G_sync_s<='1';

else

por_g_ff1_s <= '0';

por_g_ff2_s <= por_g_ff1_s;

por_g_ff3_s <= por_g_ff2_s;

POR_G_sync_s <= por_g_ff3_s;

end if;end if;

end process P_POR_sync;

BUFG_POR_Global : BUFG

port map (

O => POR_G_sync_temp_s,    -- Clock buffer output

I => POR_G_sync_s      -- Clock buffer input

);

POR_G_sync_os<=POR_G_sync_temp_s;

P_Mil_reset_16_cycles: process(clk_40MHz_is,POR_G_sync_temp_s)

begin

if POR_G_sync_temp_s='1' then

por_count_s <=(others=>'0');

por_count_en_s<='1';

por_n_en_s<='0';

elsif( clk_40MHz_is'event and clk_40MHz_is='1') then

if por_count_en_s='1' then

por_count_s<=por_count_s+1;

end if;

if por_count_s ="1111" then

por_count_en_s<='0';

por_n_en_s<='1';

end if;

end if;

end process P_Mil_reset_16_cycles;

P_Mil_reset_active_low: process(clk_40MHz_is,POR_G_sync_temp_s)

begin

if POR_G_sync_temp_s='1' then

por_n_s<='0';

por_ff_n_s <='0';

elsif(clk_40MHz_is'event and clk_40MHz_is='1') then

if por_n_en_s='1' then

por_ff_n_s <= '1';

por_n_s <= por_ff_n_s;

end if;

end if;

end process P_Mil_reset_active_low;

-- Active low Reset Synchronised to 20MHz rising edge.

P_Mil_reset_synchronizer: process(clk_20MHz_is,por_n_s)

begin

if por_n_s='0' then

n_por_20m_ff1_s <='0';

reset_n_s  <='0';

elsif clk_20MHz_is'event and clk_20MHz_is='1' then

n_por_20m_ff1_s <= '1';

reset_n_s  <= n_por_20m_ff1_s;

end if;

end process P_Mil_reset_synchronizer;

-- Synchronised active low reset connected to BUFG.

BUFG_mil_reset : BUFG

port map (

I => reset_n_s,     -- Clock buffer input

O => mil_reset_s    -- Clock buffer output

);

mil_reset_os <= mil_reset_s;

end reset_sync_a;

61) sampling_freq_gen_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity sampling_freq_gen_e is

port(clk_40MHz_is: in std_logic;

POR_is : in std_logic;

sampling_clk_os: out std_logic;  -- Sampling clk at 5.714MHz

sampling_clk_en_os : out std_logic --Clk enable

);

end sampling_freq_gen_e;

architecture sampling_freq_gen_a of sampling_freq_gen_e is

signal count_s:  std_logic_vector(2 downto 0);

signal en_ff1_s,en_ff2_s :std_logic; -- Clock enable signals

signal clk_r_s,clk_f_s: std_logic;

signal clk_mod7_s,clk_mod7_delay_s: std_logic;

signal sampling_clk_en_s,sampling_clk_en1_s: std_logic;

attribute TIG:string;

attribute TIG of clk_f_s: signal is "TRUE";

begin

-- Clock generation, with rising edge reference

P_clk_rising:  process(clk_40MHz_is,POR_is)

begin

if(POR_is='1') then

count_s <=(others=>'0');

elsif clk_40MHz_is'event and  clk_40MHz_is='1' then

if(count_s >="110") then

count_s <=(others=>'0');

else

count_s<= count_s + 1;

end if;

end if;

end process P_clk_rising;

en_ff1_s<='1' when count_s="001" else '0';--Generate enable when count=1

en_ff2_s<='1' when count_s="101" else '0';--Generate enable when count=5

P_toggle_ff_rising_edge: process(clk_40MHz_is,POR_is)

begin

if(POR_is='1') then

clk_r_s <='0';

elsif clk_40MHz_is'event and  clk_40MHz_is='1' then

if(en_ff1_s='1') then

clk_r_s <= not clk_r_s; --Toggle Flipflop, rising edge

end if;

end if;

end process P_toggle_ff_rising_edge;

P_toggle_ff_falling_edge: process(clk_40MHz_is,POR_is)

begin

if(POR_is='1') then

clk_f_s <='0';

elsif clk_40MHz_is'event and  clk_40MHz_is='0' then

if(en_ff2_s='1') then

clk_f_s <= not clk_f_s; --Toggle Flipflop, Falling edge

end if;

end if;

end process P_toggle_ff_falling_edge;

--divided by 7 with 50% duty cycle

sampling_clk_os<= clk_r_s xor clk_f_s;

clk_mod7_s<= clk_r_s xor clk_f_s;

--Genrate clock delayed by one cycle

P_delay: process(clk_40MHz_is,POR_is)

begin

if(POR_is='1') then

clk_mod7_delay_s<='0';

elsif clk_40MHz_is='0' and clk_40MHz_is'event then

clk_mod7_delay_s  <= clk_mod7_s;

end if;

end process P_delay;

-- Generate Clock enable at Falling edge of 40MHz Clock

sampling_clk_en_s<='1' when clk_mod7_s='0' and clk_mod7_delay_s='1' else

'0';

P_flipflop: process(clk_40MHz_is,POR_is)

begin

if(POR_is='1') then

sampling_clk_en1_s <='0';

elsif clk_40MHz_is'event and  clk_40MHz_is='1' then

sampling_clk_en1_s<=sampling_clk_en_s;

end if;

end process P_flipflop;

sampling_clk_en_os <= sampling_clk_en1_s;

end sampling_freq_gen_a;

62) signal_type_p

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

package signal_type_p is

constant channels_count_c : integer := 17;  -- 18 Trakcing channels in FPGA

constant acq_modules_count_c : integer := 3; -- 4 Acquisition modules

constant acq_parallel_mods_c : integer := 23;  -- 24 downsampler modules

type seventeen_ch1_18_typ is array (channels_count_c downto 0) of std_logic_vector(16 downto 0);

type fortynine_ch1_18_typ is array (channels_count_c downto 0) of std_logic_vector(48 downto 0);

type twentysix_ch1_18_typ is array (channels_count_c downto 0) of std_logic_vector(25 downto 0);

type twentythree_ch1_18_typ is array (channels_count_c downto 0) of std_logic_vector(22 downto 0);

type twentyeight_ch1_18_typ is array (channels_count_c downto 0) of std_logic_vector(27 downto 0);

type thirtytwo_ch1_18_typ is array (channels_count_c downto 0) of std_logic_vector(31 downto 0);

type sixteen_ch1_18_typ   is array (channels_count_c downto 0) of std_logic_vector(15 downto 0);

type four_ch1_18_typ      is array (channels_count_c downto 0) of std_logic_vector(3 downto 0);

type eight_ch1_18_typ     is array (channels_count_c downto 0) of std_logic_vector(7 downto 0);

type six_ch1_18_typ       is array (channels_count_c downto 0) of std_logic_vector(5 downto 0);

type two_ch1_18_typ       is array (channels_count_c downto 0) of std_logic_vector(1 downto 0);

type five_ch1_18_typ      is array (channels_count_c downto 0) of std_logic_vector(4 downto 0);

type six_acq1_4_typ       is array (acq_modules_count_c downto 0) of std_logic_vector(5 downto 0);

type eight_acq1_4_typ     is array (acq_modules_count_c downto 0) of std_logic_vector(7 downto 0);

type eleven_ch1_18_typ    is array (channels_count_c downto 0) of std_logic_vector(10 downto 0);

type ten_ch1_18_typ       is array (channels_count_c downto 0) of std_logic_vector(9 downto 0);

type twentysix_acq1_4_typ is array (acq_modules_count_c downto 0) of std_logic_vector(25 downto 0);

type sixteen_acq1_4_typ   is array (acq_modules_count_c downto 0) of std_logic_vector(15 downto 0);

type ten_acq1_4_typ       is array (acq_modules_count_c downto 0) of std_logic_vector(9 downto 0);

type two_acq1_4_typ       is array (acq_modules_count_c downto 0) of std_logic_vector(1 downto 0);

type one_acq1_4_typ       is array (acq_modules_count_c downto 0) of std_logic;

type thirtytwo_acq1_4_typ is array (acq_modules_count_c downto 0) of std_logic_vector(31 downto 0);

type fortynine_acq1_4_typ is array (acq_modules_count_c downto 0) of std_logic_vector(48 downto 0);

type sixtyfour_acq1_4_typ is array (acq_modules_count_c downto 0) of std_logic_vector(63 downto 0);

type one_acq_typ is array (acq_modules_count_c downto 0) of std_logic;

type fourteen_acq_typ is array (acq_modules_count_c downto 0) of std_logic_vector(13 downto 0);

type eleven_acq_typ is array (acq_modules_count_c downto 0) of std_logic_vector(10 downto 0);

type twentyfive_acq_typ is array (acq_modules_count_c downto 0) of std_logic_vector(25 downto 0);

end signal_type_p;

63) SPI_code_sy

library IEEE;

use IEEE.std_logic_1164.all;

use IEEE.std_logic_unsigned.all;

use IEEE.std_logic_arith.all;

use IEEE.numeric_std.all;

entity SPI_code_sy is

port (

reset_is : IN std_logic;

start_prog_flag_SY_is : IN std_logic;

SY17_PLL_val_is : IN std_logic_vector (15 downto 0) ;

SY17_AGC_val_is : IN std_logic_vector (15 downto 0) ;

SYFE_PM_val_is : IN std_logic_vector (15 downto 0) ;

SYFE_RFPLL_val_is : IN std_logic_vector (15 downto 0) ;

SYFE_IFPLL_val_is : IN std_logic_vector (15 downto 0) ;

read_SPI_SY_is : IN std_logic;

done_wr_SY_os: OUT std_logic;

read_error_SY_os: OUT std_logic;

spi_clk_SY_is: in std_logic;

spi_en_SY_os: OUT std_logic;

spi_mosi_SY_os: OUT std_logic;

spi_miso_SY_is: in std_logic;

reg_val_read_os: OUT std_logic_vector (7 downto 0)

);

end SPI_code_sy;

architecture Behavioral_a of SPI_code_sy is

-- signal, component etc. declarations

signal shift_reg_s,shift_reg2_s: std_logic_vector(15 downto 0);

signal cycle1_s: std_logic_vector (3 downto 0);

signal count1_s: std_logic_vector (4 downto 0) ;

signal count2_s: std_logic_vector (15 downto 0) ;

signal ce_s: std_logic  ;

signal spi_clk_s: std_logic  ;

signal spi_pre_re_s: std_logic  ;

signal spi_clk_div_s: std_logic_vector (4 downto 0) ;

signal count_s: std_logic_vector (3 downto 0) ;

signal cycle1_read_s: std_logic_vector (2 downto 0);

signal count1_read_s: std_logic_vector (4 downto 0);

signal shift_reg_read_s,shift_reg2_read_s,reg_val_read_s,reg2_val_read_s: std_logic_vector(7 downto 0);

signal spi_mosi_wr_s,spi_mosi_read_s:  std_logic  ;

signal spi_en_wr_s,spi_en_read_s,spi_miso_SY_s:  std_logic  ;

signal   SY17_PLL_val_s : std_logic_vector (15 downto 0) ;

signal   SY17_AGC_val_s : std_logic_vector (15 downto 0) ;

signal   SYFE_PM_val_s : std_logic_vector (15 downto 0) ;

signal   SYFE_RFPLL_val_s : std_logic_vector (15 downto 0) ;

signal   SYFE_IFPLL_val_s : std_logic_vector (15 downto 0) ;

begin

spi_mosi_SY_os <= spi_mosi_wr_s when spi_en_wr_s='0' else

spi_mosi_read_s;

spi_en_SY_os <= spi_en_wr_s and spi_en_read_s ;

----SPI write----------------------------------------------------------

SY_SPI_write: process(spi_clk_SY_is ,reset_is)

begin

if (reset_is='1') then

count2_s<=(OTHERS=>'0');

count1_s <= (others => '0');

cycle1_s <=(others => '0');

ce_s <='0';

shift_reg_s<= X"4000";

done_wr_SY_os<='0';

SY17_PLL_val_s<=(OTHERS=>'0');

SY17_AGC_val_s<=(OTHERS=>'0');

SYFE_PM_val_s<=(OTHERS=>'0');

SYFE_RFPLL_val_s<=(OTHERS=>'0');

SYFE_IFPLL_val_s<=(OTHERS=>'0');

elsif (spi_clk_SY_is'EVENT and spi_clk_SY_is='1') then

--	SY17_AGC_val_s<=SY17_AGC_val_is;----ONLY FOR TESTING

if (start_prog_flag_SY_is ='1')then

-------SPI reg val read-------------

SY17_PLL_val_s<=SY17_PLL_val_is;

SY17_AGC_val_s<=SY17_AGC_val_is;

SYFE_PM_val_s<=SYFE_PM_val_is;

SYFE_RFPLL_val_s<=SYFE_RFPLL_val_is;

SYFE_IFPLL_val_s<=SYFE_IFPLL_val_is;

-------------------------------------------

ce_s <='1';

done_wr_SY_os<='0';

end if;

if (ce_s = '0') then

count1_s <= (others => '0');

else

count1_s <= count1_s+1;

end if;

if (count1_s = 19 ) then

cycle1_s <= cycle1_s+1;

end if;

if (cycle1_s >=10) then

ce_s <='0';

cycle1_s <=(others => '0');

done_wr_SY_os<='1';

end if;

if (count1_s <= 1) then

shift_reg_s <= shift_reg2_s;

end if ;

if (spi_en_wr_s='0') then

shift_reg_s(15 downto 0) <=  shift_reg_s(14 downto 0) & '0';

end if;

end if;

end process SY_SPI_write;

shift_reg2_s<=SY17_PLL_val_s when cycle1_s=0 else ---SY1017C---PM and PLL  --11010111--20

SY17_PLL_val_s when cycle1_s=1 else ---SY1017C---PM and PLL-

SY17_AGC_val_s when cycle1_s=2 else ---SY1017C---AGC-DAC---0--highest gain

SY17_AGC_val_s when cycle1_s=3 else ---SY1017C---AGC-DAC

SYFE_PM_val_s when cycle1_s=4 else  ----SY1008--PM

SYFE_PM_val_s when cycle1_s=5 else  ---SY1008--PM

SYFE_RFPLL_val_s when cycle1_s=6 else  ---SY1008--RF PLL divider--70

SYFE_RFPLL_val_s when cycle1_s=7 else    ---SY1008--RF PLL divider

SYFE_IFPLL_val_s when cycle1_s=8 else     -----SY1008--VCO tuning   -----SY1007--IF PLL divider

SYFE_IFPLL_val_s when cycle1_s=9 else    -----SY1008--VCO tuning   -----SY1007--IF PLL divider

X"4000" when cycle1_s=10 else      ---reserved

X"4000" when cycle1_s=11 else  ---reserved

X"4000" when cycle1_s=12 else  ------reserved

X"4000" when cycle1_s=13 else  ------reserved

X"4000" when cycle1_s=14 else   -----reserved

X"4000" ;-- ---reserved

spi_mosi_wr_s <= shift_reg_s(15) ;

spi_en_wr_s <= '0' when count1_s > 1 and count1_s < 18 else '1';

--------------------------------------------------------------

----SPI Read----------------------------------------------------------

SY_SPI_Read : process(spi_clk_SY_is ,reset_is)

begin

if (reset_is='1') then

count1_read_s <= (others => '0');

cycle1_read_s <=(others => '0');

shift_reg_read_s<= X"04";

elsif (spi_clk_SY_is'EVENT and spi_clk_SY_is='1') then

if (read_SPI_SY_is = '0') then

count1_read_s <= (others => '0');

cycle1_read_s <= (others => '0');

--	read_error_SY_os <='0';

else

count1_read_s <= count1_read_s+1;

end if;

if (count1_read_s = 19) then

cycle1_read_s <= cycle1_read_s+1;

end if;

---- SPI Registers configuration data

if (count1_read_s <= 1) then

shift_reg_read_s <= shift_reg2_read_s;

end if ;

---- SPI Registers configuration data check

if (spi_en_read_s='0') then

if (count1_read_s <= 9) then  ---count 2 to 9

shift_reg_read_s(7 downto 0) <=  shift_reg_read_s(6 downto 0) & '0';

--    elsif (count1_read_s >= 10) and (count1_read_s < 19) then  ---count 10 to 17

--      spi_miso_SY_s <= spi_miso_SY_is;

--      reg_val_read_s(7 downto 0) <=  reg_val_read_s(6 downto 0) & spi_miso_SY_s;

--    else  ---count 18 & 19

--if (reg_val_read_s = reg2_val_read_s) then

--		read_error_SY_os <='0';

--	else

--		read_error_SY_os <='1';

--	end if;

end if;

end if;

--reg_val_read_os<=reg_val_read_s;

end if;

end process SY_SPI_Read;

SY_SPI_Read_MISO : process(spi_clk_SY_is ,reset_is)

begin

if (reset_is='1') then

reg_val_read_s<= (others => '0');

read_error_SY_os<='0';

elsif (spi_clk_SY_is'EVENT and spi_clk_SY_is='0') then

if (read_SPI_SY_is = '0') then

read_error_SY_os <='0';

end if;

---- SPI Registers configuration data check

if (spi_en_read_s='0') then

if (count1_read_s >= 10) and (count1_read_s <= 17) then  ---count 10 to 17

reg_val_read_s(7 downto 0) <=  reg_val_read_s(6 downto 0) & spi_miso_SY_is;

elsif (count1_read_s > 17) then  ---count 18

if (reg_val_read_s = reg2_val_read_s) then

read_error_SY_os <='0';

else

read_error_SY_os <='1';

end if;

end if;

end if;

reg_val_read_os<=reg_val_read_s;

end if;

end process SY_SPI_Read_MISO;

shift_reg2_read_s<=X"08" when cycle1_read_s=0 else ----SY1017C---PM and PLL

X"10" when cycle1_read_s=1 else ---SY1017C---AGC-DAC

X"94" when cycle1_read_s=2 else -----SY1008/1007--PM

X"80" when cycle1_read_s=3 else ---SY1008--RF PLL divider

X"B0" when cycle1_read_s=4 else ------SY1008--VCO tuning //-SY1007--IF PLL divider

X"BC" when cycle1_read_s=5 else ------SY1008--VCO LF read ---ONLY FOR sy1008 --COMMENT FOR SY1007

X"04" ; --spare

reg2_val_read_s<=SY17_PLL_val_s(7 downto 0) when cycle1_read_s=0 else ---val for --SY1017C---PM and PLL

SY17_AGC_val_s(7 downto 0) when cycle1_read_s=1 else ---val for -SY1017C---AGC-DAC

SYFE_PM_val_s(7 downto 0) when cycle1_read_s=2 else ---val for --SY1008--PM

SYFE_RFPLL_val_s(7 downto 0) when cycle1_read_s=3 else ---val for -SY1008/1007--RF PLL divider

SYFE_IFPLL_val_s(7 downto 0) when cycle1_read_s=4 else ---val for ---SY1008--VCO tuning//-SY1007--IF PLL divider

X"40" when cycle1_read_s=5 else ---ONLY FOR sy1008 val for -SY1008--VCO LF read--COMMENT FOR SY1007

X"00" ; --spare

spi_en_read_s <= '0' when count1_read_s >=2 and count1_read_s <= 18 else '1';

spi_mosi_read_s <= shift_reg_read_s(7);

--------------------------------------------------------------

end Behavioral_a;

64) SPI_control_SY

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_unsigned.ALL;

use IEEE.STD_LOGIC_arith.ALL;

entity SPI_control_SY is

Port (

clk_is : in  STD_LOGIC;

reset_is : in  STD_LOGIC;

reprogram_SY_cmd_flag_is : in  STD_LOGIC;

done_wr_SY_is : in  STD_LOGIC;

read_error_SY_is : in  STD_LOGIC;

start_prog_flag_SY_os : out  STD_LOGIC;

read_SPI_SY_os : out  STD_LOGIC;

SY_FE_write_count_os :out std_logic_vector(3 downto 0)

);

end SPI_control_SY;

architecture Behavioral of SPI_control_SY is

constant count_spi_SY_c : integer:=1999999 ;

signal count_spi_s:integer range count_spi_SY_c downto 0;

type SPI_state_typ is (idle, wr_SPI, read_SPI );

signal state_SPI_SY_s : SPI_state_typ;

signal	SY_FE_write_count_s : std_logic_vector(3 downto 0);

begin

--LAIS_FE_write_count_os<=LAIS_FE_write_count_s;

SY_FE_write_count_os<=SY_FE_write_count_s;

process_SPI_Control: process(reset_is,clk_is)

begin

if reset_is='1'then

SY_FE_write_count_s<=(OTHERS=>'0');

count_spi_s<=0;

--   state_SPI_LAIS_s<=idle;

state_SPI_SY_s<=idle;

elsif (clk_is='1' and clk_is'event) then

----------------SPI counter --------------

if count_spi_s <= count_spi_SY_c then

count_spi_s <= count_spi_s+1;

else

count_spi_s <= count_spi_s;

end if;

------------------------------------------------------------------------

--      Case_ctrl_SPI_LAIS: case (state_SPI_LAIS_s) is

--      when idle => ----SPI count check state and wait for wr_done

--         if count_spi_s= count_spi_LAIS_g  then

--            state_SPI_LAIS_s <= wr_SPI;

--         elsif done_wr_LAIS_is = '1' then

--            state_SPI_LAIS_s <= read_SPI;

--         else

--            state_SPI_LAIS_s <= idle;

--			end if;

--      when wr_SPI => ----write LAIS SPI prog enable flag

--         state_SPI_LAIS_s <= idle;

--         LAIS_FE_write_count_s<= LAIS_FE_write_count_s +1;

--      when read_SPI => ----SPI read check state

--         if ((read_error_LAIS_is ='1') or (reprogram_LAIS_cmd_flag_is ='1')) then

--            state_SPI_LAIS_s <= wr_SPI;

--         else

--            state_SPI_LAIS_s <= read_SPI;

--			end if;

--      when others =>

--            state_SPI_LAIS_s <= idle;

--      end case Case_ctrl_SPI_LAIS;

Case_ctrl_SPI_SY: case (state_SPI_SY_s) is

when idle => ----SPI count check state and wait for wr_done

if count_spi_s= count_spi_SY_c  then

state_SPI_SY_s <= wr_SPI;

elsif done_wr_SY_is = '1' then

state_SPI_SY_s <= read_SPI;

else

state_SPI_SY_s <= idle;

end if;

when wr_SPI => ----write LAIS SPI prog enable flag

state_SPI_SY_s <= idle;

SY_FE_write_count_s<= SY_FE_write_count_s +1;

when read_SPI => ----SPI read check state

if ((read_error_SY_is ='1') or (reprogram_SY_cmd_flag_is ='1')) then

state_SPI_SY_s <= wr_SPI;

else

state_SPI_SY_s <= read_SPI;

end if;

when others =>

state_SPI_SY_s <= idle;

end case Case_ctrl_SPI_SY;

end if;

end process process_SPI_Control;

start_prog_flag_SY_os <= '1' when state_SPI_SY_s = wr_SPI else

'0';

read_SPI_SY_os <= '1' when state_SPI_SY_s = read_SPI else

'0';

end Behavioral;

65) svid_ff_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity svid_ff_e is

port( clk_is : in std_logic;

POR_is: in std_logic;

set_svid_is: in std_logic;

clr_svid_is: in std_logic;

svid_ff_os: out std_logic

);

end svid_ff_e;

architecture svid_ff_a of svid_ff_e is

begin

P_SVID_FF: process(clk_is, POR_is)

begin

if POR_is = '1' then

svid_ff_os <= '0';

elsif clk_is'event and clk_is = '1' then

if set_svid_is = '1' then

svid_ff_os <= '1';

elsif clr_svid_is = '1' then

svid_ff_os <= '0';

end if;

end if;

end process P_SVID_FF;

end svid_ff_a;

66) svid_status_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use WORK.signal_type_p.ALL;

entity svid_status_e is

port(clk_is: in std_logic;

POR_is: in std_logic;

load_fsm_state_is: in std_logic;

clear_fsm_state_is: in std_logic;

current_svid_is: in std_logic_vector(5 downto 0);

svid_channel_is: in six_ch1_18_typ;

channel_lock_status_is: in std_logic_vector(channels_count_c downto 0);

channel_register_is: in std_logic_vector(channels_count_c downto 0);

svid_status_out_os: out std_logic_vector(63 downto 0)

);

end svid_status_e;

architecture svid_status_a of svid_status_e is

signal set_svid_s, set_svid_delay_s: std_logic_vector(63 downto 0);

signal clear_svid_s,clear_svid_delay_s : std_logic_vector(63 downto 0);

signal svid_ff_s: std_logic_vector(63 downto 0);

component svid_ff_e is

port( clk_is: in std_logic;

POR_is: in std_logic;

set_svid_is: in std_logic;

clr_svid_is: in std_logic;

svid_ff_os: out std_logic);

end component;

begin

svid_ff_gen: for i in 0 to 63 generate

svid_ff_e_inst : svid_ff_e

port map(clk_is => clk_is,

POR_is => POR_is,

set_svid_is => set_svid_s(i),

clr_svid_is => clear_svid_s(i),

svid_ff_os => svid_ff_s(i));

end generate svid_ff_gen;

P_set_clear_svid_registers: process(clk_is, POR_is)

begin

if POR_is = '1' then

set_svid_s <= (others => '0');

clear_svid_s <= (others => '0');

elsif clk_is'event and clk_is = '1' then

set_svid_s <= set_svid_delay_s;

clear_svid_s <= clear_svid_delay_s;

end if;

end process P_set_clear_svid_registers;

svid_status_out_os(63 downto 0) <=  svid_ff_s; --Used by top acquisition module.

-- When FSM in state4 i.e. load state, check current svid and set the necessar svid ff to set state.

set_svid_delay_gen: for i in 0 to 63 generate

set_svid_delay_s(i) <= '1' when current_svid_is = CONV_STD_LOGIC_VECTOR(i,6) and load_fsm_state_is = '1' else '0';

end generate set_svid_delay_gen;

-- SVID Clearing logic: Tracking channel is assigned to particular SVID, Unlock status and FSM in clear state.

clear_svid_delay_gen: for i in 0 to 63 generate

clear_svid_delay_s(i) <=  '1' when channel_lock_status_is(0) = '0' and channel_register_is(0) = '1' and svid_channel_is(0) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(1) = '0' and channel_register_is(1) = '1' and svid_channel_is(1) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(2) = '0' and channel_register_is(2) = '1' and svid_channel_is(2) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(3) = '0' and channel_register_is(3) = '1' and svid_channel_is(3) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(4) = '0' and channel_register_is(4) = '1' and svid_channel_is(4) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(5) = '0' and channel_register_is(5) = '1' and svid_channel_is(5) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(6) = '0' and channel_register_is(6) = '1' and svid_channel_is(6) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(7) = '0' and channel_register_is(7) = '1' and svid_channel_is(7) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(8) = '0' and channel_register_is(8) = '1' and svid_channel_is(8) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(9) = '0' and channel_register_is(9) = '1' and svid_channel_is(9) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(10) = '0' and channel_register_is(10) = '1' and svid_channel_is(10) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(11) = '0' and channel_register_is(11) = '1' and svid_channel_is(11) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(12) = '0' and channel_register_is(12) = '1' and svid_channel_is(12) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(13) = '0' and channel_register_is(13) = '1' and svid_channel_is(13) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(14) = '0' and channel_register_is(14) = '1' and svid_channel_is(14) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(15) = '0' and channel_register_is(15) = '1' and svid_channel_is(15) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(16) = '0' and channel_register_is(16) = '1' and svid_channel_is(16) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'1' when channel_lock_status_is(17) = '0' and channel_register_is(17) = '1' and svid_channel_is(17) = CONV_STD_LOGIC_VECTOR(i,6) and clear_fsm_state_is = '1' else

'0' ;

end generate clear_svid_delay_gen;

end svid_status_a;

67) synchronizer_cmd_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity synchronizer_cmd_e is

port(

clk_40MHz_in_is: in std_logic;

PORG_is: in std_logic;

signal_in_is: in std_logic;

signal_out_os : out std_logic

);

end synchronizer_cmd_e;

architecture synchronizer_cmd_a of synchronizer_cmd_e is

signal sync_ff1_s,sync_ff2_s,sync_ff3_s : std_logic;

begin

PROCESS_P1:process(clk_40MHz_in_is, PORG_is)

begin

if PORG_is = '1' then

sync_ff1_s <= '0';

sync_ff2_s <= '0';

sync_ff3_s <= '0';

elsif clk_40MHz_in_is'event and clk_40MHz_in_is = '1' then

sync_ff1_s <= signal_in_is;

sync_ff2_s <= sync_ff1_s;

sync_ff3_s <= sync_ff2_s;

end if;

end process PROCESS_P1;

signal_out_os <= sync_ff3_s;

end synchronizer_cmd_a;

68) Top_acquisition_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

use WORK.signal_type_p.ALL;

entity Top_acquisition_e is

port (

Sys_CLK_40M_is : in  STD_LOGIC;

POR_RST_is : in  STD_LOGIC;

Incoming_data_ant1_is : in std_logic_vector(5 downto 0);

Incoming_data_ant2_is : in std_logic_vector(5 downto 0);

Incoming_data_ant3_is : in std_logic_vector(5 downto 0);

Incoming_data_ant4_is : in std_logic_vector(5 downto 0);

IF_Freq_GPS_sel_is : in std_logic;

IF_Freq_NAVIC_sel_is : in std_logic;

car_mix_ROM_ena_is : IN STD_LOGIC;

car_mix_ROM_wea_is : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

car_mix_ROM_addra_is : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

car_mix_ROM_dina_is : IN STD_LOGIC_VECTOR(19 DOWNTO 0);

FPGA_Tracking_register1_is : in std_logic_vector(31 downto 0);

FPGA_Tracking_register2_is : in std_logic_vector(31 downto 0);

data_processor_is : in std_logic_vector(31 downto 0);

Acquisition_threshold_LSW_sel_is : in std_logic;

Acquisition_threshold_MSW_sel_is : in std_logic;

alm_ant1_gps_reg_is, alm_ant2_gps_reg_is, alm_ant3_gps_reg_is, alm_ant4_gps_reg_is : in std_logic_vector(acq_modules_count_c downto 0);

alm_ant1_nav_reg_is, alm_ant2_nav_reg_is, alm_ant3_nav_reg_is, alm_ant4_nav_reg_is : in std_logic_vector(acq_modules_count_c downto 0);

alm_ant1_sbas_reg_is, alm_ant2_sbas_reg_is, alm_ant3_sbas_reg_is, alm_ant4_sbas_reg_is : in std_logic_vector(acq_modules_count_c downto 0);

SVID_loading_is : in std_logic_vector(acq_modules_count_c downto 0);

SVID_acquired_os : out six_acq1_4_typ;

SVID_Searched_os : out six_acq1_4_typ;

Antenna_output_os : out two_acq1_4_typ;

Track_epoch_output_pulse_os : out std_logic_vector(acq_modules_count_c downto 0);

car_freq_init_os , code_freq_init_os: out thirtytwo_acq1_4_typ;

order1_loop_init_os, dll_order1_loop_init_os : out fortynine_acq1_4_typ;

acq_threshold_LSW_os : out std_logic_vector(31 downto 0);

acq_threshold_MSW_os : out std_logic_vector(31 downto 0);

fft_status_os : out std_logic_vector(31 downto 0);

acq1_search_reg_os   : out std_logic_vector(63 downto 0);

acq2_search_reg_os   : out std_logic_vector(63 downto 0);

acq3_search_reg_os   : out std_logic_vector(63 downto 0);

acq4_search_reg_os   : out std_logic_vector(63 downto 0)

);

end Top_acquisition_e;

architecture Top_acquisition_a of Top_acquisition_e is

component acq_module0_controller_gp2015_e

port (

Sys_CLK_40M_is : in  STD_LOGIC;

POR_RST_is : in  STD_LOGIC;

Incoming_data_ant1_is : in std_logic_vector(5 downto 0);

Incoming_data_ant2_is : in std_logic_vector(5 downto 0);

Incoming_data_ant3_is : in std_logic_vector(5 downto 0);

Incoming_data_ant4_is : in std_logic_vector(5 downto 0);

DSP_SV_register_ant1_is : in std_logic_vector(63 downto 0);

DSP_SV_register_ant2_is : in std_logic_vector(63 downto 0);

DSP_SV_register_ant3_is : in std_logic_vector(63 downto 0);

DSP_SV_register_ant4_is : in std_logic_vector(63 downto 0);

FPGA_SV_register_is : in std_logic_vector(63 downto 0);

Module_input_is :in std_logic_vector(1 downto 0);

g2_reg_input_is : in std_logic_vector(9 downto 0);

Acquisition_threshold_is : in std_logic_vector(51 downto 0);

IF_Freq_GPS_is : in std_logic_vector(31 downto 0);

IF_Freq_NAVIC_is : in std_logic_vector(31 downto 0);

car_mix_ROM_ena_is : IN STD_LOGIC;

car_mix_ROM_wea_is : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

car_mix_ROM_addra_is : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

car_mix_ROM_dina_is : IN STD_LOGIC_VECTOR(19 DOWNTO 0);

fft_done_is : in std_logic;

--fft_edone_is : in std_logic;

xk_re1_is : in std_logic_vector(25 downto 0);

xk_im1_is : in std_logic_vector(25 downto 0);

xk_re2_is : in std_logic_vector(25 downto 0);

xk_im2_is : in std_logic_vector(25 downto 0);

xn_index_is : in std_logic_vector(10 downto 0);

set_delay_is : in std_logic;

SVID_acquired_os : out std_logic_vector(5 downto 0);

SVID_Searched_os : out std_logic_vector(5 downto 0);

Antenna_output_os : out std_logic_vector(1 downto 0);

Track_epoch_output_os : out std_logic;

SVID_loading_is : in std_logic;

car_freq_init_os : out std_logic_vector(31 downto 0);

code_freq_init_os : out std_logic_vector(31 downto 0);

order1_loop_init_os: out std_logic_vector(48 downto 0);

dll_order1_loop_init_os:out std_logic_vector(48 downto 0);

xn_re1_os : out std_logic_vector(13 downto 0);

xn_im1_os : out std_logic_vector(13 downto 0);

xn_re2_os : out std_logic_vector(13 downto 0);

xn_im2_os : out std_logic_vector(13 downto 0);

fft_start_os : out std_logic;

fft_unload_os : out std_logic;

fsm2_enable_os: out std_logic;

slew_done_os: out std_logic

);

end component;

COMPONENT acqmod_almanc_reg_e

PORT(

clk_40M_is : IN std_logic;

reset_is : IN std_logic;

alm_ant1_gps_reg_is : IN std_logic_vector(3 downto 0);

alm_ant2_gps_reg_is : IN std_logic_vector(3 downto 0);

alm_ant3_gps_reg_is : IN std_logic_vector(3 downto 0);

alm_ant4_gps_reg_is : IN std_logic_vector(3 downto 0);

alm_ant1_nav_reg_is : IN std_logic_vector(3 downto 0);

alm_ant2_nav_reg_is : IN std_logic_vector(3 downto 0);

alm_ant3_nav_reg_is : IN std_logic_vector(3 downto 0);

alm_ant4_nav_reg_is : IN std_logic_vector(3 downto 0);

alm_ant1_sbas_reg_is : IN std_logic_vector(3 downto 0);

alm_ant2_sbas_reg_is : IN std_logic_vector(3 downto 0);

alm_ant3_sbas_reg_is : IN std_logic_vector(3 downto 0);

alm_ant4_sbas_reg_is : IN std_logic_vector(3 downto 0);

data_processor_is : IN std_logic_vector(31 downto 0);

DSP_SV_register_ant1_os : OUT sixtyfour_acq1_4_typ;

DSP_SV_register_ant2_os : OUT sixtyfour_acq1_4_typ;

DSP_SV_register_ant3_os : OUT sixtyfour_acq1_4_typ;

DSP_SV_register_ant4_os : OUT sixtyfour_acq1_4_typ

);

END COMPONENT;

COMPONENT fft_top_cont_e

PORT(

clk_40M_is : IN std_logic;

reset_is : IN std_logic;

xn_re1_is : IN std_logic_vector(13 downto 0);

xn_re2_is : IN std_logic_vector(13 downto 0);

xn_im1_is : IN std_logic_vector(13 downto 0);

xn_im2_is : IN std_logic_vector(13 downto 0);

fft_start1_is : IN std_logic;

fft_start2_is : IN std_logic;

fft_unload1_is : IN std_logic;

fft_unload2_is : IN std_logic;

sel_is : IN std_logic;

fft_done1_os : OUT std_logic;

fft_done2_os : OUT std_logic;

-- fft_edone1_os    : out  std_logic;

-- fft_edone2_os    : out  std_logic;

fft_status_os    : out  std_logic_vector(7 downto 0);

xk_re1_os : OUT std_logic_vector(25 downto 0);

xk_re2_os : OUT std_logic_vector(25 downto 0);

xk_im1_os : OUT std_logic_vector(25 downto 0);

xk_im2_os : OUT std_logic_vector(25 downto 0);

xn_index1_os : OUT std_logic_vector(10 downto 0);

xn_index2_os : OUT std_logic_vector(10 downto 0)

);

END COMPONENT;

signal DSP_SV_register_ant1_s, DSP_SV_register_ant2_s ,DSP_SV_register_ant3_s , DSP_SV_register_ant4_s : sixtyfour_acq1_4_typ;

signal FPGA_SV_register_s: std_logic_vector(63 downto 0);

constant Module_input_s : two_acq1_4_typ := ("11","10","01","00");

constant g2_reg_input_s : ten_acq1_4_typ := ("0011100000", "1000100110","1001011000","1111101100");

signal Acquisition_threshold_s : std_logic_vector(51 downto 0);

signal fft_done_s,fft_start_s,fft_unload_s,set_delay_s,slew_done_s, fsm2_enable_s: one_acq_typ;

signal xn_re1_s,xn_im1_s,xn_re2_s,xn_im2_s  : fourteen_acq_typ;

signal xk_re1_s,xk_im1_s,xk_re2_s,xk_im2_s  : twentyfive_acq_typ;

signal xn_index_s: eleven_acq_typ;

signal delay_count_s : std_logic_vector(20 downto 0);

signal sel_a_s, sel_b_s: std_logic;

signal sel_a_temp_s,sel_b_temp_s : std_logic;

signal fft1_status_s,fft2_status_s,fft3_status_s,fft4_status_s : std_logic_vector(7 downto 0);

signal IF_Freq_GPS_s, IF_Freq_NAVIC_s : std_logic_vector(31 downto 0);

begin

fft_status_os <= fft4_status_s & fft3_status_s & fft2_status_s & fft1_status_s;

-- Read interface for Acqusition threshold regiters.

acq_threshold_LSW_os <= Acquisition_threshold_s(31 downto 0);

acq_threshold_MSW_os <= "000000000000" & Acquisition_threshold_s(51 downto 32);

-- Read interface for search registers by acquisition modules.

acq1_search_reg_os  <=  DSP_SV_register_ant1_s(0)(63 downto 46)  &  DSP_SV_register_ant2_s(0)(45 downto 32) & DSP_SV_register_ant1_s(0)(31 downto 0) ;

acq2_search_reg_os  <=  DSP_SV_register_ant1_s(1)(63 downto 46)  &  DSP_SV_register_ant2_s(1)(45 downto 32) & DSP_SV_register_ant1_s(1)(31 downto 0) ;

acq3_search_reg_os  <=  DSP_SV_register_ant1_s(2)(63 downto 46)  &  DSP_SV_register_ant2_s(2)(45 downto 32) & DSP_SV_register_ant1_s(2)(31 downto 0) ;

acq4_search_reg_os  <=  DSP_SV_register_ant1_s(3)(63 downto 46)  &  DSP_SV_register_ant2_s(3)(45 downto 32) & DSP_SV_register_ant1_s(3)(31 downto 0) ;

--  4 Acquisition modules instantiated.

acq_gen: for i in 0 to acq_modules_count_c generate

acq_module1: acq_module0_controller_gp2015_e

port map( Sys_CLK_40M_is => Sys_CLK_40M_is, POR_RST_is => POR_RST_is,

Incoming_data_ant1_is => Incoming_data_ant1_is, Incoming_data_ant2_is => Incoming_data_ant2_is,

Incoming_data_ant3_is => Incoming_data_ant3_is, Incoming_data_ant4_is => Incoming_data_ant4_is,

DSP_SV_register_ant1_is => DSP_SV_register_ant1_s(i), DSP_SV_register_ant2_is => DSP_SV_register_ant2_s(i),

DSP_SV_register_ant3_is => DSP_SV_register_ant3_s(i), DSP_SV_register_ant4_is => DSP_SV_register_ant4_s(i),

FPGA_SV_register_is => FPGA_SV_register_s, Module_input_is => Module_input_s(i),g2_reg_input_is => g2_reg_input_s(i),

Acquisition_threshold_is => Acquisition_threshold_s,

IF_Freq_GPS_is  => IF_Freq_GPS_s,

IF_Freq_NAVIC_is => IF_Freq_NAVIC_s,

car_mix_ROM_ena_is => car_mix_ROM_ena_is,

car_mix_ROM_wea_is => car_mix_ROM_wea_is,

car_mix_ROM_addra_is => car_mix_ROM_addra_is,

car_mix_ROM_dina_is => car_mix_ROM_dina_is,

fft_done_is => fft_done_s(i),

--fft_edone_is => fft_edone_s(i),

xk_re1_is => xk_re1_s(i) ,

xk_im1_is => xk_im1_s(i) ,

xk_re2_is => xk_re2_s(i) ,

xk_im2_is => xk_im2_s(i) ,

xn_index_is => xn_index_s(i),

set_delay_is => set_delay_s(i),

SVID_acquired_os => SVID_acquired_os(i),

SVID_Searched_os => SVID_Searched_os(i), Antenna_output_os => Antenna_output_os(i), Track_epoch_output_os => Track_epoch_output_pulse_os(i),

SVID_loading_is => SVID_loading_is(i),car_freq_init_os => car_freq_init_os(i),

code_freq_init_os => code_freq_init_os(i),order1_loop_init_os => order1_loop_init_os(i),dll_order1_loop_init_os => dll_order1_loop_init_os(i),

xn_re1_os => xn_re1_s(i),

xn_im1_os => xn_im1_s(i),

xn_re2_os => xn_re2_s(i),

xn_im2_os => xn_im2_s(i),

fft_start_os => fft_start_s(i),

fft_unload_os =>  fft_unload_s(i),

fsm2_enable_os => fsm2_enable_s(i),

slew_done_os => slew_done_s(i)

);

end generate acq_gen;

--  Almanac register interface for the acquisiton module.

Inst_acqmod_almanc_reg_e: acqmod_almanc_reg_e PORT MAP(

clk_40M_is => Sys_CLK_40M_is ,

reset_is => POR_RST_is ,

alm_ant1_gps_reg_is => alm_ant1_gps_reg_is ,

alm_ant2_gps_reg_is => alm_ant2_gps_reg_is,

alm_ant3_gps_reg_is => alm_ant3_gps_reg_is,

alm_ant4_gps_reg_is => alm_ant4_gps_reg_is,

alm_ant1_nav_reg_is => alm_ant1_nav_reg_is,

alm_ant2_nav_reg_is => alm_ant2_nav_reg_is,

alm_ant3_nav_reg_is => alm_ant3_nav_reg_is,

alm_ant4_nav_reg_is => alm_ant4_nav_reg_is,

alm_ant1_sbas_reg_is => alm_ant1_sbas_reg_is ,

alm_ant2_sbas_reg_is => alm_ant2_sbas_reg_is,

alm_ant3_sbas_reg_is => alm_ant3_sbas_reg_is,

alm_ant4_sbas_reg_is => alm_ant4_sbas_reg_is,

data_processor_is => data_processor_is,

DSP_SV_register_ant1_os => DSP_SV_register_ant1_s ,

DSP_SV_register_ant2_os => DSP_SV_register_ant2_s,

DSP_SV_register_ant3_os => DSP_SV_register_ant3_s,

DSP_SV_register_ant4_os => DSP_SV_register_ant4_s

);

-- FFT top controller module for Acquisiton  module-1 & Acquisiton module-2.

fft_top_cont1A: fft_top_cont_e PORT MAP(

clk_40M_is => Sys_CLK_40M_is ,

reset_is =>  POR_RST_is,

xn_re1_is => xn_re1_s(0),

xn_re2_is => xn_re1_s(1),

xn_im1_is => xn_im1_s(0),

xn_im2_is => xn_im1_s(1),

fft_start1_is => fft_start_s(0),

fft_start2_is => fft_start_s(1),

fft_unload1_is => fft_unload_s(0),

fft_unload2_is => fft_unload_s(1),

sel_is => sel_a_s ,

fft_done1_os => fft_done_s(0),

fft_done2_os => fft_done_s(1),

-- fft_edone1_os => fft_edone_s(0),

-- fft_edone2_os => fft_edone_s(1),

fft_status_os => fft1_status_s,

xk_re1_os => xk_re1_s(0),

xk_re2_os => xk_re1_s(1),

xk_im1_os => xk_im1_s(0),

xk_im2_os => xk_im1_s(1),

xn_index1_os => xn_index_s(0),

xn_index2_os => xn_index_s(1)

);

fft_top_cont1B: fft_top_cont_e PORT MAP(

clk_40M_is => Sys_CLK_40M_is ,

reset_is =>  POR_RST_is,

xn_re1_is => xn_re2_s(0),

xn_re2_is => xn_re2_s(1),

xn_im1_is => xn_im2_s(0),

xn_im2_is => xn_im2_s(1),

fft_start1_is => fft_start_s(0),

fft_start2_is => fft_start_s(1),

fft_unload1_is => fft_unload_s(0),

fft_unload2_is => fft_unload_s(1),

sel_is => sel_a_s ,

fft_done1_os => open ,

fft_done2_os => open,

-- fft_edone1_os => open,

-- fft_edone2_os => open,

fft_status_os => fft2_status_s,

xk_re1_os => xk_re2_s(0),

xk_re2_os => xk_re2_s(1),

xk_im1_os => xk_im2_s(0),

xk_im2_os => xk_im2_s(1),

xn_index1_os => open ,

xn_index2_os => open

);

-- FFT top controller module for Acquisiton  module-1 & Acquisiton module-2.

fft_top_cont2A: fft_top_cont_e PORT MAP(

clk_40M_is => Sys_CLK_40M_is ,

reset_is =>  POR_RST_is,

xn_re1_is => xn_re1_s(2),

xn_re2_is => xn_re1_s(3),

xn_im1_is => xn_im1_s(2),

xn_im2_is => xn_im1_s(3),

fft_start1_is => fft_start_s(2),

fft_start2_is => fft_start_s(3),

fft_unload1_is => fft_unload_s(2),

fft_unload2_is => fft_unload_s(3),

sel_is => sel_b_s ,

fft_done1_os => fft_done_s(2),

fft_done2_os => fft_done_s(3),

-- fft_edone1_os => fft_edone_s(2),

-- fft_edone2_os => fft_edone_s(3),

fft_status_os => fft3_status_s,

xk_re1_os => xk_re1_s(2),

xk_re2_os => xk_re1_s(3),

xk_im1_os => xk_im1_s(2),

xk_im2_os => xk_im1_s(3),

xn_index1_os => xn_index_s(2),

xn_index2_os => xn_index_s(3)

);

fft_top_cont2B: fft_top_cont_e PORT MAP(

clk_40M_is => Sys_CLK_40M_is ,

reset_is =>  POR_RST_is,

xn_re1_is => xn_re2_s(2),

xn_re2_is => xn_re2_s(3),

xn_im1_is => xn_im2_s(2),

xn_im2_is => xn_im2_s(3),

fft_start1_is => fft_start_s(2),

fft_start2_is => fft_start_s(3),

fft_unload1_is => fft_unload_s(2),

fft_unload2_is => fft_unload_s(3),

sel_is => sel_b_s ,

fft_done1_os => open ,

fft_done2_os => open,

-- fft_edone1_os => open,

-- fft_edone2_os => open,

fft_status_os => fft4_status_s,

xk_re1_os => xk_re2_s(2),

xk_re2_os => xk_re2_s(3),

xk_im1_os => xk_im2_s(2),

xk_im2_os => xk_im2_s(3),

xn_index1_os => open ,

xn_index2_os => open

);

P_SVID_search_reg: process(Sys_CLK_40M_is, POR_RST_is)

begin

if(POR_RST_is = '1') then

-- Acquisition_threshold_s <= "0000000000000011011100010101100100000000";--7600

Acquisition_threshold_s <= "0000000000000000001000010000110000101000110010000000";--66600

IF_Freq_GPS_s <=  x"02B020C5"; --0.42e6 at 40M

IF_Freq_NAVIC_s <= x"0947AE15"; --1.45e6 at 40M

elsif(Sys_CLK_40M_is='1') and (Sys_CLK_40M_is'event) then

-- acquisition threshold is 40 bit input. so LSW and MSW is used.

if(Acquisition_threshold_LSW_sel_is = '1') then

Acquisition_threshold_s(31 downto 0) <= data_processor_is;

end if;

if(Acquisition_threshold_MSW_sel_is = '1') then

Acquisition_threshold_s(51 downto 32) <= data_processor_is(19 downto 0);

end if;

if(IF_Freq_GPS_sel_is = '1') then

IF_Freq_GPS_s <= data_processor_is;

end if;

if(IF_Freq_NAVIC_sel_is = '1') then

IF_Freq_NAVIC_s <= data_processor_is;

end if;

end if;

end process P_SVID_search_reg;

PROCESS_P1:process(Sys_CLK_40M_is, POR_RST_is)

begin

if(POR_RST_is = '1') then

delay_count_s  <= (others=>'0');

set_delay_s(0) <='0';

set_delay_s(1) <='1';

set_delay_s(2) <='0';

set_delay_s(3) <='1';

sel_a_s <= '0';

sel_b_s <= '0';

sel_a_temp_s <= '0';

sel_b_temp_s <= '0';

elsif(Sys_CLK_40M_is='1') and (Sys_CLK_40M_is'event) then

-- 12.4 msec delay is generated.

if delay_count_s >= '0' & X"79180" then

delay_count_s  <='0' & X"FFFFF";

set_delay_s(0) <='0';

set_delay_s(1) <='0';

set_delay_s(2) <='0';

set_delay_s(3) <='0';

else

delay_count_s <= delay_count_s+1;

set_delay_s(0) <='0';

set_delay_s(1) <='1';

set_delay_s(2) <='0';

set_delay_s(3) <='1';

end if;

-- Select signal for module 1 and module 2

if slew_done_s(0)='1'  then

sel_a_temp_s<='1';

elsif slew_done_s(1)='1'  then

sel_a_temp_s<='0';

end if;

-- Select signal for module 3 and module 4

if slew_done_s(2)='1'  then

sel_b_temp_s<='1';

elsif slew_done_s(3)='1' then

sel_b_temp_s<='0';

end if;

sel_a_s <= sel_a_temp_s and  fsm2_enable_s(1);

sel_b_s <= sel_b_temp_s and  fsm2_enable_s(3);

end if;

end process PROCESS_P1;

FPGA_SV_register_s <= FPGA_Tracking_register2_is & FPGA_Tracking_register1_is; --64 Satellites

end Top_acquisition_a;

69) top_discriminator_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_SIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

use WORK.signal_type_p.ALL;

entity top_discriminator_e is

port(

clk_is,POR_is: in std_logic;

-- all inputs from channel has 18 channel type

-- all outputs are single output

p_i_cndr_is: in twentyeight_ch1_18_typ;  -- promt inphase for cndr

p_q_cndr_is: in twentyeight_ch1_18_typ;  -- promt inphase for cndr

p_i_is: in twentythree_ch1_18_typ;  -- promt inphase

p_q_is: in twentythree_ch1_18_typ; -- promt qphase

e_i_is: in twentythree_ch1_18_typ; -- early inphase

e_q_is: in twentythree_ch1_18_typ; -- early qphase

l_i_is: in twentythree_ch1_18_typ; -- late inphase

l_q_is: in twentythree_ch1_18_typ; -- late qphase

lock_counter_enable_os : out std_logic_vector(channels_count_c downto 0);   -- output of lock indicator to each channels

channel_par_load_output_os: out std_logic_vector(channels_count_c downto 0);  -- loop filter update signal for each channel

car_corr_freq_word_out_os:  out std_logic_vector(31 downto 0);  -- carrier freq update value for the tracking channel

code_corr_freq_word_out_os : out std_logic_vector(31 downto 0);  -- code freq update value for the tracking channel

Prev_phase_error_input_is : in seventeen_ch1_18_typ;	 -- previous phase error for frequency error computation from all channels.

Order1_input_is,Order2_input_is,Order3_input_is : in fortynine_ch1_18_typ;  -- previous order1,2,3, from all channels.

DLL_Order1_input_is,DLL_Order2_input_is : in  fortynine_ch1_18_typ; -- previous dllorder1,2, from all channels.

Prev_phase_error_output_os : out std_logic_vector(16 downto 0); --output of discriminator for storing into channel;

Order1_output_os,Order2_output_os,Order3_output_os : out  std_logic_vector(48 downto 0); --output of discriminator for storing into channel;

DLL_Order1_output_os,DLL_Order2_output_os : out std_logic_vector(48 downto 0);--output of discriminator for storing into channel;

loop_start_input_is : in std_logic_vector(channels_count_c downto 0);

stage_control_is : in five_ch1_18_typ;

fll_zero_is : in std_logic_vector(channels_count_c downto 0);

NAVIC_indicator_is :  in std_logic_vector(channels_count_c downto 0);

Data_boundary_input_is  :  in std_logic_vector(channels_count_c downto 0);

fll_data_check_input_is : in std_logic_vector(channels_count_c downto 0);

-- loop start = A&D is complete start the loop

-- fll_start_input = once loop is settled no need for FLL that input decided by channels

-- fll_data_check_os = when FLL crosses 0.5*pi it is due to data transition, it shall be corrected.

track_th_sel_is : in std_logic; -- tracking threshold update input

up_data_is : in std_logic_vector(31 downto 0);

-- PLL, Set1, Launch phase, enable registers

pll_par1_set1_launch_enable_is : IN std_logic;

pll_par2_set1_launch_enable_is : IN std_logic;

pll_par3_set1_launch_enable_is : IN std_logic;

pll_par4_set1_launch_enable_is : IN std_logic;

-- FLL, Set1, Launch phase, enable registers

fll_par1_set1_launch_enable_is : IN std_logic;

fll_par2_set1_launch_enable_is : IN std_logic;

fll_par3_set1_launch_enable_is : IN std_logic;

-- DLL, Set1, Launch phase, enable registers

dll_par1_set1_launch_enable_is : IN std_logic;

dll_par2_set1_launch_enable_is : IN std_logic;

dll_par3_set1_launch_enable_is : IN std_logic;

-- PLL, Set2, Launch phase, enable registers

pll_par1_set2_launch_enable_is : IN std_logic;

pll_par2_set2_launch_enable_is : IN std_logic;

pll_par3_set2_launch_enable_is : IN std_logic;

pll_par4_set2_launch_enable_is : IN std_logic;

-- FLL, Set2, Launch phase, enable registers

fll_par1_set2_launch_enable_is : IN std_logic;

fll_par2_set2_launch_enable_is : IN std_logic;

fll_par3_set2_launch_enable_is : IN std_logic;

-- DLL, Set2, Launch phase, enable registers

dll_par1_set2_launch_enable_is : IN std_logic;

dll_par2_set2_launch_enable_is : IN std_logic;

dll_par3_set2_launch_enable_is : IN std_logic;

-- PLL, Set3, Launch phase, enable registers

pll_par1_set3_launch_enable_is : IN std_logic;

pll_par2_set3_launch_enable_is : IN std_logic;

pll_par3_set3_launch_enable_is : IN std_logic;

pll_par4_set3_launch_enable_is : IN std_logic;

-- FLL, Set3, Launch phase, enable registers

fll_par1_set3_launch_enable_is : IN std_logic;

fll_par2_set3_launch_enable_is : IN std_logic;

fll_par3_set3_launch_enable_is : IN std_logic;

-- DLL, Set3, Launch phase, enable registers

dll_par1_set3_launch_enable_is : IN std_logic;

dll_par2_set3_launch_enable_is : IN std_logic;

dll_par3_set3_launch_enable_is : IN std_logic;

-- PLL, Set4, Launch phase, enable registers

pll_par1_set4_launch_enable_is : IN std_logic;

pll_par2_set4_launch_enable_is : IN std_logic;

pll_par3_set4_launch_enable_is : IN std_logic;

pll_par4_set4_launch_enable_is : IN std_logic;

-- FLL, Set4, Launch phase, enable registers

fll_par1_set4_launch_enable_is : IN std_logic;

fll_par2_set4_launch_enable_is : IN std_logic;

fll_par3_set4_launch_enable_is : IN std_logic;

-- DLL, Set4, Launch phase, enable registers

dll_par1_set4_launch_enable_is : IN std_logic;

dll_par2_set4_launch_enable_is : IN std_logic;

dll_par3_set4_launch_enable_is : IN std_logic;

-- PLL, Set5, Launch phase, enable registers

pll_par1_set5_launch_enable_is : IN std_logic;

pll_par2_set5_launch_enable_is : IN std_logic;

pll_par3_set5_launch_enable_is : IN std_logic;

pll_par4_set5_launch_enable_is : IN std_logic;

-- FLL, Set5, Launch phase, enable registers

fll_par1_set5_launch_enable_is : IN std_logic;

fll_par2_set5_launch_enable_is : IN std_logic;

fll_par3_set5_launch_enable_is : IN std_logic;

-- DLL, Set5, Launch phase, enable registers

dll_par1_set5_launch_enable_is : IN std_logic;

dll_par2_set5_launch_enable_is : IN std_logic;

dll_par3_set5_launch_enable_is : IN std_logic;

cdll_par1_set1_launch_enable_is: IN std_logic;

carr_aided_loop_sel_is         : IN std_logic;

fll_zero_en_dis_sel_is         : IN std_logic;

loop_par_status1_enable_is      : IN std_logic;

IF_FREQ_TRK_GPS_sel_is        : IN std_logic;

IF_FREQ_TRK_NAVIC_sel_is        : IN std_logic;

trk_threshold_os              : out std_logic_vector(31 downto 0);

carr_aided_loop_os             : out std_logic ;

fll_zero_en_dis_os             : out std_logic

);  -- Tracking threshold input.

end top_discriminator_e;

architecture top_discriminator_a of top_discriminator_e is

component divsip

port (

clk: in std_logic;

ce: in std_logic;

sclr: in std_logic;

nd: in std_logic;

rdy: out std_logic;

rfd: out std_logic;

dividend: in std_logic_vector(45 downto 0);

divisor: in std_logic_vector(45 downto 0);

quotient: out std_logic_vector(45 downto 0);

fractional: out std_logic_vector(13 downto 0));

end component;

COMPONENT tanip

PORT (

x_in : IN STD_LOGIC_VECTOR(23 DOWNTO 0);

y_in : IN STD_LOGIC_VECTOR(23 DOWNTO 0);

nd : IN STD_LOGIC;

phase_out : OUT STD_LOGIC_VECTOR(16 DOWNTO 0);

rdy : OUT STD_LOGIC;

rfd : OUT STD_LOGIC;

clk : IN STD_LOGIC;

ce : IN STD_LOGIC;

sclr: in std_logic

);

END COMPONENT;

component mult_32_by_17

Port ( Sys_CLK_40M_is : in  STD_LOGIC;

POR_RST_is : in  STD_LOGIC;

Data_in_32_bit_is : in  STD_LOGIC_VECTOR (31 downto 0);

Data_in_17_bit_is : in  STD_LOGIC_VECTOR (16 downto 0);

Data_out_49_bit_os : out  STD_LOGIC_VECTOR (48 downto 0));

end component;

component mult_16_by_16_sqr

Port ( Sys_CLK_40M_is : in  STD_LOGIC;

POR_RST_is : in  STD_LOGIC;

Data_in_16_bit1_is : in  STD_LOGIC_VECTOR (22 downto 0);

Data_in_16_bit2_is : in  STD_LOGIC_VECTOR (22 downto 0);

Data_out_32_bit_os : out  STD_LOGIC_VECTOR (45 downto 0)

);

end component;

component mult_49_by_27

Port ( Sys_CLK_40M_is : in  STD_LOGIC;

POR_RST_is : in  STD_LOGIC;

Data_in_49_bit_is : in  STD_LOGIC_VECTOR (48 downto 0);

Data_in_27_bit_is : in  STD_LOGIC_VECTOR (26 downto 0);

Data_out_76_bit_os : out  STD_LOGIC_VECTOR (75 downto 0)

);

end component;

signal IF_FREQ_TRK_GPS_s, IF_FREQ_TRK_NAVIC_s : std_logic_vector(48 downto 0);

signal PhaseError1_s, PhaseError2_s, DLL_Phase_error_out_s : std_logic_vector(16 downto 0);

-- to store the phase error outputs of arc_tan ip.  and DLL divisor ip.

-- phase error1 = atan,   phase error2 = atan2 ,  dll_phase = DLL discrminator output from divisor ip.

signal Prompt_inphase_cndr_s, Prompt_qphase_cndr_s : std_logic_vector(27 downto 0);

signal Prompt_inphase_s, Prompt_qphase_s :  std_logic_vector(22 downto 0);

signal Early_inphase_s, Early_qphase_s :  std_logic_vector(22 downto 0);

signal Late_inphase_s, Late_qphase_s :  std_logic_vector(22 downto 0);

-- 6 correlators to be latched from 12 or 20 channels

signal  start_tanip_s, new_data_s, Completed_tanip_s : std_logic;

-- start_tanip = ce for atanip.

-- new data s = new data flag for atanip

-- Completed_tanip_s = flag to indicate tanip data is ready and proceed for Loop filter.

-- Compute_loop1_s = flag to indicate, atan for PLL is also complete and proceed for loop filter

--phase_out_ready_s = flag to indicate PLL atan also complete (1 cycle prior to compute_loop1_s

--signal phase_out_ready_s : std_logic;

signal Tan_inputX_s,Tan_inputY_s: std_logic_vector(23 downto 0);

signal  Phase_out_s : std_logic_vector(16 downto 0);

-- inputs and outputs of Tan_ip

signal Previous_phase2_err_s,Freq_error_s : std_logic_vector(16 downto 0);

-- local storage of previous phase error to find frequency error, Freq error variable.

signal Order3_s, Order2_s, Order1_s, CarrNCO_s,Code_NCO_s,Rx_freq_s,Rx_code_freq_s,Rx_code_freq1_s : std_logic_vector(48 downto 0);

signal DLL_order1_local_s, DLL_order2_local_s  : std_logic_vector(48 downto 0);

-- all are local storage variables to store the respective loop filter accumulator values.

-- CarrNCO_s,Code_NCO_s,Rx_freq_s,Rx_code_freq_s,Rx_code_freq1_s = intermediate or final multiplied values for NCO.

-- all factors are stored above signals, these have 15 bit fractional part so they are 25 bit effectively. 25 bit resolution is checked in

-- matlab, it is 100 times more than what is required for actual storage.

signal Rx_freq_word_s ,Rx_code_freq_word_s,Rx_code_freq_word1_s: std_logic_vector(75 downto 0);

-- once PLL output genrates required freq with IF so the result is multiplied  with NCO factor (16bit fractional) final result storage this variable is used

-- for programming nco, actual frequency shall be multiplied with NCO factor.  So then we need 76 bits = 49 + 17 (NCO factor) bits

signal completed_accum_s: std_logic;

-- completed_accum_s = signal to indicate start the discrminator activities.

-- busy_sig_s = presently a channel is using the discrminator.

-- e_minus_l_prior_step_s = flag to do the E-L prior step of E = ei* ei* + eq * eq;

--signal Doppler_bin_input_s : std_logic_vector(9 downto 0);

--signal NCO_update_s : std_logic;

signal C1PLL_s, C2PLL_s, C3PLL_s, C4PLL_s, C1FLL_s , C2FLL_s , C3FLL_s , C1DLL_s, C2DLL_s, C3DLL_s, C1DLL1_s : std_logic_vector(31 downto 0);

-- default values are stored in these signals.

signal temp_pll_mul_order1_s, temp_pll_mul_order2_s,temp_pll_mul_order3_s : std_logic_vector(48 downto 0);

signal temp_dll_mul_order1_s, temp_dll_mul_order2_s,temp_dll_mul_order11_s: std_logic_vector(48 downto 0);

signal temp_dll_mul_order0_s,	temp_pll_mul_order0_s : std_logic_vector(48 downto 0);

signal temp_fll_mul_order1_s,	temp_fll_mul_order2_s , temp_fll_mul_order3_s: std_logic_vector(48 downto 0);

signal temp_ratio_conversion_s : std_logic_vector(69 downto 0);

-- temporary loop multiplications are stored here.

-- flag to maintain the loop constants updated or not.

signal 	E_arm_s ,	L_arm_s , E_plus_L_s, E_minus_L_s : std_logic_vector(45 downto 0);

-- Earm = ei * ei + eq * eq --  L_arm = li* li + lq * lq (late inphase, late qphase). E+L ,  E-L

signal 				 start_divisor_ip_s  : std_logic;

-- flags for different steps of DLL.  Start_divisor_ip = CE for divisor IP.

signal  fractional_s : std_logic_vector(13 downto 0);

-- division ip outputs.

signal new_data_divsip_s : std_logic;

--signal tracking_threshold_default_s : std_logic_vector(31 downto 0);

signal channel_trk_integer_s,trk_integer_s : integer range 0 to 63;

-- 12 or 20 channels outputs are available here.  these variable to used to select a particular channel

signal Latched_loop_start_s, Processed_the_channel_s : std_logic_vector(channels_count_c downto 0);

-- to indicate a particular channel is already processed or not.

signal stage_control_local_s : std_logic_vector(4 downto 0);

signal fll_zero_local_s,NAVIC_indicator_local_s ,Data_boundary_input_local_s: std_logic;

--constant IF_FREQ_c : std_logic_vector(48 downto 0) :=   "0000000000010100101011001111100000000000000000000";

-- 15 bit fractional format (IF freq = 1.405e6 * 2^15)

constant CODE_FREQ_c : std_logic_vector(48 downto 0) := "0000000000000011111001110000011000000000000000000";

-- 15 bit fractional format (CODE freq = 1.023e6 * 2^15)

constant Code_to_carr_ratio_GPS_c : std_logic_vector(26 downto 0) :=   "000000010101010001110010011";

constant Code_to_carr_ratio_NavIC_c : std_logic_vector(26 downto 0) := "000000011100011111100111000";

-- 1/1540 x 2^30

constant NCO_factor_code_c : std_logic_vector(26 downto 0) := "000011010110101111111001010";

--16bit fractional format (107.37 * 2^16)

constant NCO_factor_c : std_logic_vector(26 downto 0) := "000011010110101111111001010";

--16 bit fractional format (751.6 * 2^16)

--constant NCO_factor1_c : std_logic_vector(29 downto 0) := "000000010101110011000101101110";

-- this variable used for direct multiplication of doppler_bin * bin_resolution * NCO factor

-- so = 174.38 * 751.6 = 5714286 , here there is no decimal. this value used to initialize the order1 of loops doppler bin

-- when working directly order1 loaded with bin * nco_factor1  to get actual frequency.

--constant Doppler_bin_input_s : std_logic_vector(9 downto 0):= "1000111110";--"0111000010";--"1000111110";--"1000000100";

-- this need to be part of the input for simulation. (after acquisition this number also comes in part of track loading

signal PLL_C1_set1_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000110011";

signal PLL_C2_set1_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000010011111100";

signal PLL_C3_set1_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000111110111110111";

signal PLL_C4_set1_launch_s : std_logic_vector(31 downto 0);-- := "00000000000001100011011100011100";

signal PLL_C1_set2_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000101010";

signal PLL_C2_set2_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000011011101101";

signal PLL_C3_set2_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000100100011100101";

signal PLL_C4_set2_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000101111111101010001";

signal PLL_C1_set3_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000011110";

signal PLL_C2_set3_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000001110010111";

signal PLL_C3_set3_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000001101100110101";

signal PLL_C4_set3_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000001100111000110101";

signal PLL_C1_set4_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000011110";

signal PLL_C2_set4_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000001110010111";

signal PLL_C3_set4_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000001101100110101";

signal PLL_C4_set4_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000001100111000110101";

signal PLL_C1_set5_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000011110";

signal PLL_C2_set5_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000001110010111";

signal PLL_C3_set5_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000001101100110101";

signal PLL_C4_set5_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000001100111000110101";

-- FLL settings

signal FLL_C1_set1_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000001001100";

signal FLL_C2_set1_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000010001011101";

signal FLL_C3_set1_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000111111001101010";

signal FLL_C1_set2_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000111101";

signal FLL_C2_set2_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000001011001011";

signal FLL_C3_set2_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000001000000010111010";

signal FLL_C1_set3_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000011111";

signal FLL_C2_set3_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000010110011";

signal FLL_C3_set3_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000100000010111";

signal FLL_C1_set4_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000001001";

signal FLL_C2_set4_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000010000";

signal FLL_C3_set4_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000111000";

signal FLL_C1_set5_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000001001";

signal FLL_C2_set5_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000010000";

signal FLL_C3_set5_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000111000";

-- DLL settings

signal DLL_C1_set1_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000001100";

signal DLL_C2_set1_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000011101";

signal DLL_C3_set1_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000010000100";

signal DLL_C1_set2_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000001100";

signal DLL_C2_set2_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000011101";

signal DLL_C3_set2_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000010000100";

signal DLL_C1_set3_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000001100";

signal DLL_C2_set3_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000011101";

signal DLL_C3_set3_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000010000100";

signal DLL_C1_set4_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000001100";

signal DLL_C2_set4_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000011101";

signal DLL_C3_set4_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000010000100";

signal DLL_C1_set5_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000001100";

signal DLL_C2_set5_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000011101";

signal DLL_C3_set5_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000010000100";

signal DLL1_C1_set_launch_s : std_logic_vector(31 downto 0);-- := "00000000000000000000000000000100";

--signal PLL_C4_set1_launch

-- Additional signals for GAGANYAN Tracking

-- PLL, Set1 Launch phase, parameters

signal   pll_par1_set1_launch_s : std_logic_vector(31 downto 0);

signal 	pll_par2_set1_launch_s : std_logic_vector(31 downto 0);

signal	pll_par3_set1_launch_s : std_logic_vector(31 downto 0);

signal   pll_par4_set1_launch_s : std_logic_vector(31 downto 0);

-- FLL, Set1 Launch phase, parameters

signal	fll_par1_set1_launch_s : std_logic_vector(31 downto 0);

signal	fll_par2_set1_launch_s : std_logic_vector(31 downto 0);

signal	fll_par3_set1_launch_s : std_logic_vector(31 downto 0);

-- DLL, Set1 Launch phase, parameters

signal	dll_par1_set1_launch_s : std_logic_vector(31 downto 0);

signal	dll_par2_set1_launch_s : std_logic_vector(31 downto 0);

signal	dll_par3_set1_launch_s : std_logic_vector(31 downto 0);

-- PLL, Set2 Launch phase, parameters

signal	pll_par1_set2_launch_s : std_logic_vector(31 downto 0);

signal	pll_par2_set2_launch_s : std_logic_vector(31 downto 0);

signal	pll_par3_set2_launch_s : std_logic_vector(31 downto 0);

signal	pll_par4_set2_launch_s : std_logic_vector(31 downto 0);

-- FLL, Set2 Launch phase, parameters

signal	fll_par1_set2_launch_s : std_logic_vector(31 downto 0);

signal	fll_par2_set2_launch_s : std_logic_vector(31 downto 0);

signal	fll_par3_set2_launch_s : std_logic_vector(31 downto 0);

-- DLL, Set2 Launch phase, parameters

signal	dll_par1_set2_launch_s : std_logic_vector(31 downto 0);

signal	dll_par2_set2_launch_s : std_logic_vector(31 downto 0);

signal	dll_par3_set2_launch_s : std_logic_vector(31 downto 0);

-- PLL, Set3 Launch phase, parameters

signal	pll_par1_set3_launch_s : std_logic_vector(31 downto 0);

signal	pll_par2_set3_launch_s : std_logic_vector(31 downto 0);

signal	pll_par3_set3_launch_s : std_logic_vector(31 downto 0);

signal	pll_par4_set3_launch_s : std_logic_vector(31 downto 0);

-- FLL, Set3 Launch phase, parameters

signal	fll_par1_set3_launch_s : std_logic_vector(31 downto 0);

signal	fll_par2_set3_launch_s : std_logic_vector(31 downto 0);

signal	fll_par3_set3_launch_s : std_logic_vector(31 downto 0);

-- DLL, Set3 Launch phase, parameters

signal	dll_par1_set3_launch_s : std_logic_vector(31 downto 0);

signal	dll_par2_set3_launch_s : std_logic_vector(31 downto 0);

signal	dll_par3_set3_launch_s : std_logic_vector(31 downto 0);

-- PLL, Set4 Launch phase, parameters

signal	pll_par1_set4_launch_s : std_logic_vector(31 downto 0);

signal	pll_par2_set4_launch_s : std_logic_vector(31 downto 0);

signal	pll_par3_set4_launch_s : std_logic_vector(31 downto 0);

signal	pll_par4_set4_launch_s : std_logic_vector(31 downto 0);

-- FLL, Set4 Launch phase, parameters

signal	fll_par1_set4_launch_s : std_logic_vector(31 downto 0);

signal	fll_par2_set4_launch_s : std_logic_vector(31 downto 0);

signal	fll_par3_set4_launch_s : std_logic_vector(31 downto 0);

-- DLL, Set4 Launch phase, parameters

signal	dll_par1_set4_launch_s : std_logic_vector(31 downto 0);

signal	dll_par2_set4_launch_s : std_logic_vector(31 downto 0);

signal	dll_par3_set4_launch_s : std_logic_vector(31 downto 0);

-- PLL, Set5 Launch phase, parameters

signal	pll_par1_set5_launch_s : std_logic_vector(31 downto 0);

signal	pll_par2_set5_launch_s : std_logic_vector(31 downto 0);

signal	pll_par3_set5_launch_s : std_logic_vector(31 downto 0);

signal	pll_par4_set5_launch_s : std_logic_vector(31 downto 0);

-- FLL, Set5 Launch phase, parameters

signal	fll_par1_set5_launch_s : std_logic_vector(31 downto 0);

signal	fll_par2_set5_launch_s : std_logic_vector(31 downto 0);

signal	fll_par3_set5_launch_s : std_logic_vector(31 downto 0);

-- DLL, Set5 Launch phase, parameters

signal	dll_par1_set5_launch_s : std_logic_vector(31 downto 0);

signal	dll_par2_set5_launch_s : std_logic_vector(31 downto 0);

signal	dll_par3_set5_launch_s : std_logic_vector(31 downto 0);

signal	cdll_par1_set1_launch_s : std_logic_vector(31 downto 0);

signal	carr_aided_loop_s :  std_logic;--

signal	fll_zero_en_dis_s :  std_logic;--

signal	loop_par_status_s :  std_logic_vector(15 downto 0);--

signal completed_the_loop_s : std_logic;

signal fsm_counter_s : std_logic_vector(8 downto 0);

type state_type1 is (s1,s2,s3,s4); --FSM State type

signal fsm1_state_s: state_type1; --FSM State variable

attribute FSM_ENCODING : string;

attribute FSM_ENCODING of fsm1_state_s: signal is "ONE-HOT";

attribute SAFE_IMPLEMENTATION :string;

attribute SAFE_IMPLEMENTATION of fsm1_state_s: signal is"yes";

signal data_in32_multiplier_32_17_s : std_logic_vector(31 downto 0);

signal data_in17_multiplier_32_17_s : std_logic_vector(16 downto 0);

signal  data_out49_multiplier_32_17_s : std_logic_vector(48 downto 0);

signal  data1_in16_multiplier_16_16_s, data2_in16_multiplier_16_16_s : std_logic_vector(22 downto 0);

signal data_out32_multiplier_16_16_s: std_logic_vector(45 downto 0);

signal  data_in49_multiplier_49_27_s: std_logic_vector(48 downto 0);

signal  data_in27_multiplier_49_27_s: std_logic_vector(26 downto 0);

signal data_out76_multiplier_49_27_s : std_logic_vector(75 downto 0);

signal rdy_divip_s,rfd_divip_s: std_logic;

COMPONENT lock_detector_descr_e

PORT(

clk_40MHz_is : IN std_logic;

POR_is : IN std_logic;

p_i_is : IN std_logic_vector(27 downto 0);

p_q_is : IN std_logic_vector(27 downto 0);

track_th_sel_is : IN std_logic;

up_data_is : IN std_logic_vector(31 downto 0);

lock_counter_enable_os : OUT std_logic;

trk_threshold_os: out std_logic_vector(31 downto 0)

);

END COMPONENT;

signal lock_counter_enable_ch_s: std_logic;

begin

Inst_lock_detector_descr_e: lock_detector_descr_e PORT MAP(

clk_40MHz_is => clk_is,

POR_is => POR_is,

p_i_is => Prompt_inphase_cndr_s,

p_q_is => Prompt_qphase_cndr_s,

track_th_sel_is => track_th_sel_is,

up_data_is => up_data_is,

lock_counter_enable_os => lock_counter_enable_ch_s,

trk_threshold_os =>  trk_threshold_os

);

tan1: tanip

PORT MAP (     x_in => Tan_inputX_s,     y_in => Tan_inputY_s,     nd => new_data_s,     phase_out => Phase_out_s,

rdy => open,     rfd => open,     clk => clk_is ,     ce => start_tanip_s , sclr => completed_accum_s  );

div1: divsip

port map( clk=> clk_is , ce => start_divisor_ip_s, sclr => completed_accum_s, nd => new_data_divsip_s, rdy => rdy_divip_s,

rfd => open,  dividend => E_minus_L_s,  divisor => E_plus_L_s,  quotient => open,  fractional =>  fractional_s);

multply_32_17:  mult_32_by_17  port map(

Sys_CLK_40M_is => clk_is,            POR_RST_is => POR_is,

Data_in_32_bit_is => data_in32_multiplier_32_17_s,             Data_in_17_bit_is  => data_in17_multiplier_32_17_s,

Data_out_49_bit_os => data_out49_multiplier_32_17_s );

multply_16_16:  mult_16_by_16_sqr  port map(

Sys_CLK_40M_is => clk_is,            POR_RST_is => POR_is,

Data_in_16_bit1_is => data1_in16_multiplier_16_16_s,             Data_in_16_bit2_is  => data2_in16_multiplier_16_16_s,

Data_out_32_bit_os => data_out32_multiplier_16_16_s );

multply_49_27:  mult_49_by_27  port map(

Sys_CLK_40M_is => clk_is,            POR_RST_is => POR_is,

Data_in_49_bit_is => data_in49_multiplier_49_27_s,             Data_in_27_bit_is  => data_in27_multiplier_49_27_s,

Data_out_76_bit_os => data_out76_multiplier_49_27_s );

carr_aided_loop_os    <=  carr_aided_loop_s;

fll_zero_en_dis_os     <= fll_zero_en_dis_s;

P_fsm1_state: process(clk_is, POR_is)

begin

if(POR_is = '1') then

fsm1_state_s <= s1;

elsif(clk_is = '1' and clk_is'event ) then

C_fsm_state_dec: case fsm1_state_s is

when s1=>  fsm1_state_s <= s2;

when s2=>  if Latched_loop_start_s(trk_integer_s)='1' then

fsm1_state_s<=s3;

end if;

when s3=>  if completed_the_loop_s='1' then

fsm1_state_s <= s4;

end if;

when s4=>  fsm1_state_s <= s1;

when others=> fsm1_state_s <=s1;

end case C_fsm_state_dec;

end if;

end process P_fsm1_state;

P_channel_process_gen: process(clk_is, POR_is)

begin

if(POR_is = '1') then

completed_the_loop_s <= '0';

fsm_counter_s <= "000000000";

trk_integer_s <= 0;

completed_accum_s <= '0';

channel_trk_integer_s <= 0;

Processed_the_channel_s<= (others => '0');

elsif(clk_is = '1' and clk_is'event ) then

if fsm1_state_s=s2 and Latched_loop_start_s(trk_integer_s)='0'  then

if trk_integer_s=17 then

trk_integer_s<=0;

else

trk_integer_s<=trk_integer_s+1;

end if;

end if;

if fsm1_state_s=s3 and (fsm_counter_s = "000000000") then

channel_trk_integer_s <= trk_integer_s;

Processed_the_channel_s(trk_integer_s) <= '1';

elsif fsm1_state_s=s4 then

Processed_the_channel_s(channel_trk_integer_s) <= '0';

end if;

if(fsm1_state_s = s3) and (fsm_counter_s = "000000001")  then

completed_accum_s <= '1';

else

completed_accum_s <= '0';

end if;

if(fsm1_state_s =s3) then

fsm_counter_s <= fsm_counter_s + 1;

if(fsm_counter_s >= "001111110") then

completed_the_loop_s <= '1';

end if;

else

completed_the_loop_s <= '0';

fsm_counter_s <= "000000000";

end if;

end if;

end process P_channel_process_gen;

P_loop_filter: process(clk_is, POR_is)

begin

if(POR_is = '1') then

Prompt_inphase_s <= (others => '0');

Prompt_qphase_s <= (others => '0');

Prompt_inphase_cndr_s <= (others => '0');

Prompt_qphase_cndr_s <= (others => '0');

Early_inphase_s <=(others => '0');

Early_qphase_s <=(others => '0');

Late_inphase_s <= (others => '0');

Late_qphase_s <= (others => '0');

DLL_Phase_error_out_s <= "00000000000000000";

code_corr_freq_word_out_os <= "00000000000000000000000000000000";

car_corr_freq_word_out_os  <= "00000000000000000000000000000000";

Prev_phase_error_output_os <=	"00000000000000000";

Order1_output_os <=	"0000000000000000000000000000000000000000000000000";

Order2_output_os <=	"0000000000000000000000000000000000000000000000000";

Order3_output_os <=	"0000000000000000000000000000000000000000000000000";

DLL_Order1_output_os <=	"0000000000000000000000000000000000000000000000000";

DLL_Order2_output_os <=	"0000000000000000000000000000000000000000000000000";

lock_counter_enable_os <= (others => '0');  --"00000000000000000000000000";  -- 6 channels change_channels

channel_par_load_output_os <= (others => '0'); --"00000000000000000000000000";--"000000";  -- 6 channels change_channels

Latched_loop_start_s <= (others => '0'); --"00000000000000000000000000";--"000000";  -- 6 channels change_channels

Tan_inputX_s <= "011111111001110000000000"; -- to avoid divide by zero

Tan_inputY_s <= "011111111001110000000000"; -- to avoid divide by zero

start_tanip_s <= '0';

new_data_s <= '0';

Completed_tanip_s <= '0';

PhaseError1_s <= "00000000000000000";

PhaseError2_s <= "00000000000000000";

Previous_phase2_err_s <= "00000000000000000";

Freq_error_s <= "00000000000000000";

Rx_freq_s <= "0000000000000000000000000000000000000000000000000";

Rx_code_freq_s <= "0000000000000000000000000000000000000000000000000";

Rx_code_freq1_s <= "0000000000000000000000000000000000000000000000000";

CarrNCO_s <= "0000000000000000000000000000000000000000000000000";

Code_NCO_s <= "0000000000000000000000000000000000000000000000000";

Order3_s <= "0000000000000000000000000000000000000000000000000";

Order2_s <= "0000000000000000000000000000000000000000000000000";

Order1_s <= "0000000000000000000000000000000000000000000000000";

DLL_order1_local_s  <= "0000000000000000000000000000000000000000000000000";

DLL_order2_local_s  <= "0000000000000000000000000000000000000000000000000";

E_arm_s <= (others => '0');

L_arm_s <= (others => '0');

E_plus_L_s <= (others => '0');

E_minus_L_s <= (others => '0');

start_divisor_ip_s <= '0';

new_data_divsip_s <= '0';

-- the doppler input is loaded here for proper maintainance of input. (order1 holds the frequency offset, so the best place

-- to store the doppler offset.

Rx_freq_word_s <= "0000000000000000000000000000000000000000000000000000000000000000000000000000";

Rx_code_freq_word_s <= "0000000000000000000000000000000000000000000000000000000000000000000000000000";

Rx_code_freq_word1_s <= "0000000000000000000000000000000000000000000000000000000000000000000000000000";

temp_ratio_conversion_s <= "0000000000000000000000000000000000000000000000000000000000000000000000";

temp_pll_mul_order1_s <= "0000000000000000000000000000000000000000000000000";

temp_pll_mul_order2_s <= "0000000000000000000000000000000000000000000000000";

temp_pll_mul_order3_s <= "0000000000000000000000000000000000000000000000000";

temp_dll_mul_order1_s <= "0000000000000000000000000000000000000000000000000";

temp_dll_mul_order2_s <= "0000000000000000000000000000000000000000000000000";

temp_dll_mul_order11_s<= "0000000000000000000000000000000000000000000000000";

temp_fll_mul_order1_s <= "0000000000000000000000000000000000000000000000000";

temp_fll_mul_order2_s <= "0000000000000000000000000000000000000000000000000";

temp_fll_mul_order3_s <= "0000000000000000000000000000000000000000000000000";

temp_pll_mul_order0_s <= "0000000000000000000000000000000000000000000000000";

temp_dll_mul_order0_s <= "0000000000000000000000000000000000000000000000000";

--actual C4PLL/1000 , C3PLL/1000, C2PLL/1000 , C1PLL as it is due to direct multiplication as per PLL equations

--FLL constants are maintained as it is becuase, Freq_error shall be multiplied by 1000 (due divide by T_int) finaly the result shall be multiplied with T_int

-- so both factors cancel each other so, the FLL constants are maintained as it is.

C4PLL_s <= "00000000000001100011011100011100";

C3PLL_s <= "00000000000000000111110111110111";

C2PLL_s <= "00000000000000000000010011111100";

C1PLL_s <= "00000000000000000000000000110011";

C3FLL_s <= "00000000000000000111111001101010";

C2FLL_s <= "00000000000000000000010001011101";

C1FLL_s <= "00000000000000000000000001001100";

C3DLL_s <= "00000000000000000000000010000100";

C2DLL_s <= "00000000000000000000000000011101";

C1DLL_s <= "00000000000000000000000000001100";

C1DLL1_s <= "00000000000000000000000000000100";

stage_control_local_s <= "00001";

fll_zero_local_s  <= '0';

NAVIC_indicator_local_s <= '0';

Data_boundary_input_local_s <= '0';

data_in32_multiplier_32_17_s <=  (others => '0');

data_in17_multiplier_32_17_s <=  (others => '0');

data1_in16_multiplier_16_16_s <=  (others => '0');

data2_in16_multiplier_16_16_s <=  (others => '0');

data_in49_multiplier_49_27_s <=  (others => '0');

data_in27_multiplier_49_27_s <=  (others => '0');

elsif(clk_is = '1' and clk_is'event ) then

if(stage_control_local_s = "00001")  then

C4PLL_s <= PLL_C4_set1_launch_s;

C3PLL_s <= PLL_C3_set1_launch_s;

C2PLL_s <= PLL_C2_set1_launch_s;

C1PLL_s <= PLL_C1_set1_launch_s;

C3FLL_s <= FLL_C3_set1_launch_s;

C2FLL_s <= FLL_C2_set1_launch_s;

C1FLL_s <= FLL_C1_set1_launch_s;

C3DLL_s <= DLL_C3_set1_launch_s;

C2DLL_s <= DLL_C2_set1_launch_s;

C1DLL_s <= DLL_C1_set1_launch_s;

C1DLL1_s <= DLL1_C1_set_launch_s;

elsif(stage_control_local_s = "00010") then

C4PLL_s <= PLL_C4_set2_launch_s;

C3PLL_s <= PLL_C3_set2_launch_s;

C2PLL_s <= PLL_C2_set2_launch_s;

C1PLL_s <= PLL_C1_set2_launch_s;

C3FLL_s <= FLL_C3_set2_launch_s;

C2FLL_s <= FLL_C2_set2_launch_s;

C1FLL_s <= FLL_C1_set2_launch_s;

C3DLL_s <= DLL_C3_set2_launch_s;

C2DLL_s <= DLL_C2_set2_launch_s;

C1DLL_s <= DLL_C1_set2_launch_s;

C1DLL1_s <= DLL1_C1_set_launch_s;

elsif(stage_control_local_s = "00100") then

C4PLL_s <= PLL_C4_set3_launch_s;

C3PLL_s <= PLL_C3_set3_launch_s;

C2PLL_s <= PLL_C2_set3_launch_s;

C1PLL_s <= PLL_C1_set3_launch_s;

C3FLL_s <= FLL_C3_set3_launch_s;

C2FLL_s <= FLL_C2_set3_launch_s;

C1FLL_s <= FLL_C1_set3_launch_s;

C3DLL_s <= DLL_C3_set3_launch_s;

C2DLL_s <= DLL_C2_set3_launch_s;

C1DLL_s <= DLL_C1_set3_launch_s;

C1DLL1_s <= DLL1_C1_set_launch_s;

elsif(stage_control_local_s = "01000")  then

C4PLL_s <= PLL_C4_set4_launch_s;

C3PLL_s <= PLL_C3_set4_launch_s;

C2PLL_s <= PLL_C2_set4_launch_s;

C1PLL_s <= PLL_C1_set4_launch_s;

C3FLL_s <= FLL_C3_set4_launch_s;

C2FLL_s <= FLL_C2_set4_launch_s;

C1FLL_s <= FLL_C1_set4_launch_s;

C3DLL_s <= DLL_C3_set4_launch_s;

C2DLL_s <= DLL_C2_set4_launch_s;

C1DLL_s <= DLL_C1_set4_launch_s;

C1DLL1_s <= DLL1_C1_set_launch_s;

elsif(stage_control_local_s = "10000") and (fll_zero_local_s = '1') and(fll_zero_en_dis_s= '1') then

C4PLL_s <= PLL_C4_set5_launch_s;

C3PLL_s <= PLL_C3_set5_launch_s;

C2PLL_s <= PLL_C2_set5_launch_s;

C1PLL_s <= PLL_C1_set5_launch_s;

C3FLL_s <= "00000000000000000000000000000000";

C2FLL_s <= "00000000000000000000000000000000";

C1FLL_s <= "00000000000000000000000000000000";

C3DLL_s <= DLL_C3_set5_launch_s;

C2DLL_s <= DLL_C2_set5_launch_s;

C1DLL_s <= DLL_C1_set5_launch_s;

C1DLL1_s <= DLL1_C1_set_launch_s;

elsif(stage_control_local_s = "10000")  then

C4PLL_s <= PLL_C4_set5_launch_s;

C3PLL_s <= PLL_C3_set5_launch_s;

C2PLL_s <= PLL_C2_set5_launch_s;

C1PLL_s <= PLL_C1_set5_launch_s;

C3FLL_s <= FLL_C3_set5_launch_s;

C2FLL_s <= FLL_C2_set5_launch_s;

C1FLL_s <= FLL_C1_set5_launch_s;

C3DLL_s <= DLL_C3_set5_launch_s;

C2DLL_s <= DLL_C2_set5_launch_s;

C1DLL_s <= DLL_C1_set5_launch_s;

C1DLL1_s <= DLL1_C1_set_launch_s;

end if;

if(loop_start_input_is(0) = '1') then

Latched_loop_start_s(0) <= '1';

elsif(Latched_loop_start_s(0) = '1') and (Processed_the_channel_s(0) = '1') then

Latched_loop_start_s(0) <= '0';

end if;

if(loop_start_input_is(1) = '1') then

Latched_loop_start_s(1) <= '1';

elsif(Latched_loop_start_s(1) = '1') and (Processed_the_channel_s(1) = '1') then

Latched_loop_start_s(1) <= '0';

end if;

if(loop_start_input_is(2) = '1') then

Latched_loop_start_s(2) <= '1';

elsif(Latched_loop_start_s(2) = '1') and (Processed_the_channel_s(2) = '1') then

Latched_loop_start_s(2) <= '0';

end if;

if(loop_start_input_is(3) = '1') then

Latched_loop_start_s(3) <= '1';

elsif(Latched_loop_start_s(3) = '1') and (Processed_the_channel_s(3) = '1') then

Latched_loop_start_s(3) <= '0';

end if;

if(loop_start_input_is(4) = '1') then

Latched_loop_start_s(4) <= '1';

elsif(Latched_loop_start_s(4) = '1') and (Processed_the_channel_s(4) = '1') then

Latched_loop_start_s(4) <= '0';

end if;

if(loop_start_input_is(5) = '1') then

Latched_loop_start_s(5) <= '1';

elsif(Latched_loop_start_s(5) = '1') and (Processed_the_channel_s(5) = '1') then

Latched_loop_start_s(5) <= '0';

end if;

if(loop_start_input_is(6) = '1') then

Latched_loop_start_s(6) <= '1';

elsif(Latched_loop_start_s(6) = '1') and (Processed_the_channel_s(6) = '1') then

Latched_loop_start_s(6) <= '0';

end if;

if(loop_start_input_is(7) = '1') then

Latched_loop_start_s(7) <= '1';

elsif(Latched_loop_start_s(7) = '1') and (Processed_the_channel_s(7) = '1') then

Latched_loop_start_s(7) <= '0';

end if;

if(loop_start_input_is(8) = '1') then

Latched_loop_start_s(8) <= '1';

elsif(Latched_loop_start_s(8) = '1') and (Processed_the_channel_s(8) = '1') then

Latched_loop_start_s(8) <= '0';

end if;

if(loop_start_input_is(9) = '1') then

Latched_loop_start_s(9) <= '1';

elsif(Latched_loop_start_s(9) = '1') and (Processed_the_channel_s(9) = '1') then

Latched_loop_start_s(9) <= '0';

end if;

if(loop_start_input_is(10) = '1') then

Latched_loop_start_s(10) <= '1';

elsif(Latched_loop_start_s(10) = '1') and (Processed_the_channel_s(10) = '1') then

Latched_loop_start_s(10) <= '0';

end if;

if(loop_start_input_is(11) = '1') then

Latched_loop_start_s(11) <= '1';

elsif(Latched_loop_start_s(11) = '1') and (Processed_the_channel_s(11) = '1') then

Latched_loop_start_s(11) <= '0';

end if;

if(loop_start_input_is(12) = '1') then

Latched_loop_start_s(12) <= '1';

elsif(Latched_loop_start_s(12) = '1') and (Processed_the_channel_s(12) = '1') then

Latched_loop_start_s(12) <= '0';

end if;

if(loop_start_input_is(13) = '1') then

Latched_loop_start_s(13) <= '1';

elsif(Latched_loop_start_s(13) = '1') and (Processed_the_channel_s(13) = '1') then

Latched_loop_start_s(13) <= '0';

end if;

if(loop_start_input_is(14) = '1') then

Latched_loop_start_s(14) <= '1';

elsif(Latched_loop_start_s(14) = '1') and (Processed_the_channel_s(14) = '1') then

Latched_loop_start_s(14) <= '0';

end if;

if(loop_start_input_is(15) = '1') then

Latched_loop_start_s(15) <= '1';

elsif(Latched_loop_start_s(15) = '1') and (Processed_the_channel_s(15) = '1') then

Latched_loop_start_s(15) <= '0';

end if;

if(loop_start_input_is(16) = '1') then

Latched_loop_start_s(16) <= '1';

elsif(Latched_loop_start_s(16) = '1') and (Processed_the_channel_s(16) = '1') then

Latched_loop_start_s(16) <= '0';

end if;

if(loop_start_input_is(17) = '1') then

Latched_loop_start_s(17) <= '1';

elsif(Latched_loop_start_s(17) = '1') and (Processed_the_channel_s(17) = '1') then

Latched_loop_start_s(17) <= '0';

end if;

if(completed_accum_s = '1') then

Prompt_inphase_cndr_s <= p_i_cndr_is(channel_trk_integer_s);

Prompt_qphase_cndr_s <= p_q_cndr_is(channel_trk_integer_s);

Prompt_inphase_s <= p_i_is(channel_trk_integer_s);

Prompt_qphase_s  <= p_q_is(channel_trk_integer_s);

Early_inphase_s <= e_i_is(channel_trk_integer_s);

Early_qphase_s <= e_q_is(channel_trk_integer_s);

Late_inphase_s <= l_i_is(channel_trk_integer_s);

Late_qphase_s <= l_q_is(channel_trk_integer_s);

Previous_phase2_err_s <= Prev_phase_error_input_is(channel_trk_integer_s);

stage_control_local_s <= stage_control_is(channel_trk_integer_s);

fll_zero_local_s <= fll_zero_is(channel_trk_integer_s);

NAVIC_indicator_local_s <= NAVIC_indicator_is(channel_trk_integer_s);

Data_boundary_input_local_s	<= Data_boundary_input_is(channel_trk_integer_s);

end if;

--			E_arm_s <= Early_inphase_s * Early_inphase_s + Early_qphase_s * Early_qphase_s;

--			L_arm_s <= Late_inphase_s * Late_inphase_s + Late_qphase_s * Late_qphase_s;

--	input for DLL division discrminator is prepared here.

if(fsm_counter_s = "000000100") then

data1_in16_multiplier_16_16_s <= Early_inphase_s;

data2_in16_multiplier_16_16_s <= Early_qphase_s;

end if;

if(fsm_counter_s = "000001000") then

E_arm_s <= data_out32_multiplier_16_16_s;

end if;

if(fsm_counter_s = "000001001") then

data1_in16_multiplier_16_16_s <= Late_inphase_s;

data2_in16_multiplier_16_16_s <= Late_qphase_s;

end if;

if(fsm_counter_s = "000001101") then

L_arm_s<= data_out32_multiplier_16_16_s;

end if;

E_plus_L_s <= E_arm_s + L_arm_s;

E_minus_L_s <= E_arm_s  - L_arm_s;

-- one step prior to get the Earm and Larm

if(fsm_counter_s = "000000100") then

-- next step for DLL.

-- completed_accum indication of new data is available for tracking

--below codes checks different boundary conditions, as the ATAN ip needs 0/0 boundary checks

-- basically avoiding divide by zero.

if( (Prompt_inphase_s = "00000000000000000000000") and (Prompt_qphase_s > "00000000000000000000000" ) ) then

Completed_tanip_s <= '1';

PhaseError2_s <= "00010000000000000";

elsif( (Prompt_inphase_s = "00000000000000000000000") and (Prompt_qphase_s < "00000000000000000000000" ) ) then

Completed_tanip_s <= '1';

PhaseError2_s <= "11110000000000000";

elsif( (Prompt_inphase_s = "00000000000000000000000") and (Prompt_qphase_s = "00000000000000000000000" ) ) then

Completed_tanip_s <= '1';

PhaseError2_s <= "00000000000000000";

else

start_tanip_s <= '1';  -- ce for tanip is enabled

new_data_s <= '1';	 -- new data flag for tanip is enabled

Tan_inputX_s <= sxt(Prompt_inphase_s,24);  -- input for the tanip

Tan_inputY_s <= sxt(Prompt_qphase_s,24);    -- input for the tanip

--if all boundary checks are over, come here to feed the input to aractan ip

-- arc_tan ip has format of 2.15 (15 bit fractional). as a relative factor q and I are 16 bit data.

-- just extend sign and relatively they both will look same format if we just extend the sign bit by 1 time.

end if;

elsif(start_tanip_s = '1') then

new_data_s <= '0';

-- after 25 cycles we can take the tanip outputs.

if(fsm_counter_s = "000011100") then

-- this is the latency for tanip output. after 20 cycles the output is available. (for divider ip = 50 cycles)

PhaseError2_s <= Phase_out_s;

Completed_tanip_s <= '1';

-- this flag used subsequently

start_tanip_s <= '0';

-- ce for tanip can be made zero.

-- counter reset.

end if;

else

new_data_s <= '0';

Completed_tanip_s <= '0';

end if;

-- check for divide by zero.

if(fsm_counter_s = "000010001") then

if(E_minus_L_s =  "0000000000000000000000000000000000000000000000" ) then

-- if numerator is zero, then make output as zero.

DLL_Phase_error_out_s  <= "00000000000000000";

elsif(E_plus_L_s =  "0000000000000000000000000000000000000000000000" ) and (E_minus_L_s <  "0000000000000000000000000000000000000000000000" ) then

-- if denominator is zero and numerator is negative, then saturate to -0.5

DLL_Phase_error_out_s  <= "11100000000000000";

elsif(E_plus_L_s =  "0000000000000000000000000000000000000000000000" ) and (E_minus_L_s >  "0000000000000000000000000000000000000000000000" ) then

-- if denominator is zero and numerator is positive, then saturate to 0.5

DLL_Phase_error_out_s  <= "00100000000000000";

else

start_divisor_ip_s <= '1';  -- ce for division ip.

end if;

elsif(start_divisor_ip_s = '1') then

--- division ip takes 56 cycles for division completion.

if (rdy_divip_s ='1')  then

start_divisor_ip_s <= '0';

-- ce can be made zero.

-- output of division ip is latched here.

if (E_minus_L_s(45) = '1') then

DLL_Phase_error_out_s <= "111" & fractional_s;

else

DLL_Phase_error_out_s <= "000" & fractional_s;

end if;

end if;

end if;

if(fsm_counter_s = "000010011") then

new_data_divsip_s <= '1';

else

new_data_divsip_s <= '0';

end if;

-- once output is available below code is to compute atan 2 quadrant output also.

-- based on quadrant +/- pi (+/- 1) is added.

if(Completed_tanip_s = '1') then

--based on quadrant this output is chosen and verified in matlab also.

if  (Prompt_inphase_s < "0000000000000000") and (Prompt_qphase_s > "0000000000000000" )  then

PhaseError1_s <= PhaseError2_s - "00100000000000000";

elsif(Prompt_inphase_s < "0000000000000000") and (Prompt_qphase_s < "0000000000000000" )  then

PhaseError1_s <= PhaseError2_s + "00100000000000000";

elsif (Phase_out_s >= "00011000000000000") then

PhaseError1_s <= PhaseError2_s - "00100000000000000";

elsif(Phase_out_s <= "11101000000000000")	 then

PhaseError1_s <= PhaseError2_s + "00100000000000000";

else

PhaseError1_s <= PhaseError2_s;

end if;

end if;

if(fsm_counter_s = "000100000") then

data_in32_multiplier_32_17_s <= C4PLL_s;

data_in17_multiplier_32_17_s <= PhaseError1_s;

end if;

if(fsm_counter_s = "000100100") then

temp_pll_mul_order3_s <= data_out49_multiplier_32_17_s;

end if;

if(fsm_counter_s = "000100101") then

data_in32_multiplier_32_17_s <= C3PLL_s;

data_in17_multiplier_32_17_s <= PhaseError1_s;

end if;

if(fsm_counter_s = "000101001") then

temp_pll_mul_order2_s <= data_out49_multiplier_32_17_s;

end if;

if(fsm_counter_s = "000101010") then

data_in32_multiplier_32_17_s <= C2PLL_s;

data_in17_multiplier_32_17_s <= PhaseError1_s;

end if;

if(fsm_counter_s = "000101110") then

temp_pll_mul_order1_s <= data_out49_multiplier_32_17_s;

end if;

if(fsm_counter_s = "000101111") then

data_in32_multiplier_32_17_s <= C1PLL_s;

data_in17_multiplier_32_17_s <= PhaseError1_s;

end if;

if(fsm_counter_s = "000110011") then

temp_pll_mul_order0_s <= data_out49_multiplier_32_17_s;

end if;

if(fsm_counter_s = "000110100") then

data_in32_multiplier_32_17_s <= C3FLL_s;

data_in17_multiplier_32_17_s <= Freq_error_s;

end if;

if(fsm_counter_s = "000111000") then

temp_fll_mul_order3_s <= data_out49_multiplier_32_17_s;

end if;

if(fsm_counter_s = "000111001") then

data_in32_multiplier_32_17_s <= C2FLL_s;

data_in17_multiplier_32_17_s <= Freq_error_s;

end if;

if(fsm_counter_s = "000111101") then

temp_fll_mul_order2_s <= data_out49_multiplier_32_17_s;

end if;

if(fsm_counter_s = "000111110") then

data_in32_multiplier_32_17_s <= C1FLL_s;

data_in17_multiplier_32_17_s <= Freq_error_s;

end if;

if(fsm_counter_s = "001000010") then

temp_fll_mul_order1_s <= data_out49_multiplier_32_17_s;

end if;

if(fsm_counter_s = "001001010") then

data_in32_multiplier_32_17_s <= C1DLL1_s;

data_in17_multiplier_32_17_s <= DLL_Phase_error_out_s;

end if;

if(fsm_counter_s = "001001110") then

temp_dll_mul_order11_s <= data_out49_multiplier_32_17_s;

end if;

if(fsm_counter_s = "001001111") then

data_in32_multiplier_32_17_s <= C2DLL_s;

data_in17_multiplier_32_17_s <= DLL_Phase_error_out_s;

end if;

if(fsm_counter_s = "001010011") then

temp_dll_mul_order1_s <= data_out49_multiplier_32_17_s;

end if;

if(fsm_counter_s = "001010100") then

data_in32_multiplier_32_17_s <= C3DLL_s;

data_in17_multiplier_32_17_s <= DLL_Phase_error_out_s;

end if;

if(fsm_counter_s = "001011000") then

temp_dll_mul_order2_s <= data_out49_multiplier_32_17_s;

end if;

if(fsm_counter_s = "001011001") then

data_in32_multiplier_32_17_s <= C1DLL_s;

data_in17_multiplier_32_17_s <= DLL_Phase_error_out_s;

end if;

if(fsm_counter_s = "001011101") then

temp_dll_mul_order0_s <= data_out49_multiplier_32_17_s;

end if;

--			temp_pll_mul_order3_s <= C4PLL_s * PhaseError1_s;

--			temp_pll_mul_order2_s <= C3PLL_s * PhaseError1_s;

--			temp_pll_mul_order1_s <= C2PLL_s * PhaseError1_s;

--			temp_dll_mul_order11_s <= C1DLL1_s * DLL_Phase_error_out_s  ;

--			temp_dll_mul_order1_s <= C2DLL_s * DLL_Phase_error_out_s  ;

--			temp_dll_mul_order2_s <= C3DLL_s * DLL_Phase_error_out_s  ;

CarrNCO_s <= Order1_s + temp_pll_mul_order0_s ;

Code_NCO_s <= DLL_order1_local_s + temp_dll_mul_order0_s ;

if(NAVIC_indicator_local_s = '0')then

Rx_freq_s <= IF_FREQ_TRK_GPS_s + CarrNCO_s;

else

Rx_freq_s <= IF_FREQ_TRK_NAVIC_s + CarrNCO_s;

end if;

Rx_code_freq_s <= CODE_FREQ_c + Code_NCO_s;

-- code frequency for 2nd order DLL.

if(fsm_counter_s = "001100100") then

data_in49_multiplier_49_27_s <= Order1_s;

if(NAVIC_indicator_local_s ='0') then

data_in27_multiplier_49_27_s <= Code_to_carr_ratio_GPS_c;

else

data_in27_multiplier_49_27_s <= Code_to_carr_ratio_NavIC_c;

end if;

end if;

if(fsm_counter_s = "001101000") then

temp_ratio_conversion_s <= data_out76_multiplier_49_27_s(69 downto 0);

end if;

--if(NAVIC_indicator_local_s ='0') then

--temp_ratio_conversion_s <= -Order1_s * Code_to_carr_ratio_GPS_c;

--else

--temp_ratio_conversion_s <= Order1_s * Code_to_carr_ratio_NavIC_c;

--end if;

--Rx_freq_word_s <= Rx_freq_s * NCO_factor_c;

if(fsm_counter_s = "001101001") then

data_in49_multiplier_49_27_s <= Rx_freq_s;

data_in27_multiplier_49_27_s <= NCO_factor_c;

end if;

if(fsm_counter_s = "001101101") then

Rx_freq_word_s <= data_out76_multiplier_49_27_s;

end if;

Rx_code_freq1_s <= CODE_FREQ_c + temp_ratio_conversion_s(69 downto 30) + temp_dll_mul_order11_s;

-- this computation for carirer aided code lock loop. (1st order DLL )

--Rx_code_freq_word_s <= Rx_code_freq_s * NCO_factor_code_c;

-- this is for 2nd order DLL computation.

if(fsm_counter_s = "001101110") then

data_in49_multiplier_49_27_s <= Rx_code_freq_s;

data_in27_multiplier_49_27_s <= NCO_factor_code_c;

end if;

if(fsm_counter_s = "001110010") then

Rx_code_freq_word_s <= data_out76_multiplier_49_27_s;

end if;

--Rx_code_freq_word1_s <= 	Rx_code_freq1_s * 	NCO_factor_code_c;

-- this is 1st order DLL carrier aided.

if(fsm_counter_s = "001110011") then

data_in49_multiplier_49_27_s <= Rx_code_freq1_s;

data_in27_multiplier_49_27_s <= NCO_factor_code_c;

end if;

if(fsm_counter_s = "001110111") then

Rx_code_freq_word1_s <= data_out76_multiplier_49_27_s;

end if;

Order1_output_os <= Order1_s;

Order2_output_os <= Order2_s;

Order3_output_os <= Order3_s;

Prev_phase_error_output_os <= PhaseError2_s;

DLL_Order1_output_os <= DLL_order1_local_s ;

DLL_Order2_output_os <= DLL_order2_local_s ;

-- all integrator outputs are outputed for channel latching.

car_corr_freq_word_out_os  <= Rx_freq_word_s(62 downto 31);

if(carr_aided_loop_s = '1') and (stage_control_local_s="10000") then

code_corr_freq_word_out_os  <= Rx_code_freq_word1_s(62 downto 31);

else

code_corr_freq_word_out_os  <= Rx_code_freq_word_s(62 downto 31);

end if;

-- first if all the previous signals, orders are latched for loop filter calculations.

if(completed_accum_s = '1' ) then

Order3_s <= Order3_input_is(channel_trk_integer_s);

Order2_s <= Order2_input_is(channel_trk_integer_s);

Order1_s <= Order1_input_is(channel_trk_integer_s);

DLL_order1_local_s <= DLL_Order1_input_is(channel_trk_integer_s);

DLL_order2_local_s <= DLL_Order2_input_is(channel_trk_integer_s);

end if;

-- at first step the freq error is calculated.

if(fsm_counter_s = "000100000") then

Freq_error_s <= (PhaseError2_s - Previous_phase2_err_s);

-- compute frequency error with previous phase error.

-- next step the freq error is adjusted for +/- 0.5  correction.

elsif(fsm_counter_s = "000100001") then

if(Freq_error_s >= "00100000000000000") then

Freq_error_s <= Freq_error_s - "01000000000000000";

elsif(Freq_error_s <= "11100000000000000") then

Freq_error_s <= Freq_error_s + "01000000000000000";

end if;

-- check if the phase are more than 0.5 add -1, less than -0.5 add +1

-- compute the neccesary pll multiplcations aprior

-- below step if data transition happens the freq error shall be further corrected (+/ 0.25 then correct them)

elsif(fsm_counter_s = "000100010") then

if(Freq_error_s >= "00010000000000000") and (fll_data_check_input_is(channel_trk_integer_s) = '1')  and (Data_boundary_input_local_s = '1') then

Freq_error_s <= Freq_error_s - "00100000000000000";

elsif(Freq_error_s <= "11110000000000000") and (fll_data_check_input_is(channel_trk_integer_s) = '1')  and (Data_boundary_input_local_s = '1') then

Freq_error_s <= Freq_error_s + "00100000000000000";

end if;

-- compute the carrier loop order3 and code loop order2

elsif(fsm_counter_s = "001100000") then

Order3_s <= Order3_s + temp_fll_mul_order3_s  + temp_pll_mul_order3_s(48 downto 10);

-- C4pll is already divided by 1000, so T_int factor is not required.

DLL_order2_local_s <= DLL_order2_local_s + temp_dll_mul_order2_s(48 downto 10);

-- as per kaplan book, the Frequency eror also generated by dividing by T, then inside loop it is multiplied by T_int

-- so freq error is used as it is in loop.

-- but pll when multiplied it shall be multiplied with T so 10 bit shift in pll multiplications

-- similarly old order also multiplied by T so 10 bit shift.

elsif(fsm_counter_s = "001100001")then

Order2_s <= Order2_s + temp_fll_mul_order2_s + temp_pll_mul_order2_s(48 downto 10) +  Order3_s(48 downto 10) ;

--order3 shall be divided here with 1000, so 10 bit shift.

DLL_order1_local_s <= DLL_order1_local_s + temp_dll_mul_order1_s(48 downto 10) + DLL_order2_local_s(48 downto 10) 	;

elsif(fsm_counter_s = "001100010") then

Order1_s <= Order1_s + temp_fll_mul_order1_s + temp_pll_mul_order1_s(48 downto 10) + Order2_s(48 downto 10) ;

--order2 shall be divided here with 1000, so 10 bit shift.

elsif(fsm_counter_s = "001111011") then

-- at this step pulse generated for the channels to latch the NCO updates.

channel_par_load_output_os(channel_trk_integer_s) <= '1';

lock_counter_enable_os(channel_trk_integer_s) <= lock_counter_enable_ch_s;   -- V2_change

else

channel_par_load_output_os <= (others => '0');--; --"00000000000000000000000000";--"000000"; -- 6 channels  -- change_channels

end if;

end if;

end process P_loop_filter;

P_loop_filter_reg: process(clk_is, POR_is)

begin

--Launch phase, Set 1

if POR_is = '1' then

pll_par1_set1_launch_s <= (OTHERS=>'0');		-- default value

pll_par2_set1_launch_s <= (OTHERS=>'0');		-- default value

pll_par3_set1_launch_s <= (OTHERS=>'0');		-- default value

pll_par4_set1_launch_s <= (OTHERS=>'0');		-- default value

fll_par1_set1_launch_s <= (OTHERS=>'0');		-- default value

fll_par2_set1_launch_s <= (OTHERS=>'0');		-- default value

fll_par3_set1_launch_s <= (OTHERS=>'0');		-- default value

dll_par1_set1_launch_s <= (OTHERS=>'0');		-- default value

dll_par2_set1_launch_s <= (OTHERS=>'0');		-- default value

dll_par3_set1_launch_s <= (OTHERS=>'0');		-- default value

cdll_par1_set1_launch_s   <= (OTHERS=>'0');		-- default value

elsif clk_is'event and clk_is = '1' then

if pll_par1_set1_launch_enable_is = '1' then

pll_par1_set1_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par2_set1_launch_enable_is = '1' then

pll_par2_set1_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par3_set1_launch_enable_is = '1' then

pll_par3_set1_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par4_set1_launch_enable_is = '1' then

pll_par4_set1_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par1_set1_launch_enable_is = '1' then

fll_par1_set1_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par2_set1_launch_enable_is = '1' then

fll_par2_set1_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par3_set1_launch_enable_is = '1' then

fll_par3_set1_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par1_set1_launch_enable_is = '1' then

dll_par1_set1_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par2_set1_launch_enable_is = '1' then

dll_par2_set1_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par3_set1_launch_enable_is = '1' then

dll_par3_set1_launch_s <= up_data_is(31 downto 0);

end if;

if(cdll_par1_set1_launch_enable_is = '1') then

cdll_par1_set1_launch_s <= up_data_is(31 downto 0);

end if;

end if;

--Launch phase, Set2

if POR_is = '1' then

pll_par1_set2_launch_s <= (OTHERS=>'0');		-- default value

pll_par2_set2_launch_s <= (OTHERS=>'0');		-- default value

pll_par3_set2_launch_s <= (OTHERS=>'0');		-- default value

pll_par4_set2_launch_s <= (OTHERS=>'0');		-- default value

fll_par1_set2_launch_s <= (OTHERS=>'0');		-- default value

fll_par2_set2_launch_s <= (OTHERS=>'0');		-- default value

fll_par3_set2_launch_s <= (OTHERS=>'0');		-- default value

dll_par1_set2_launch_s <= (OTHERS=>'0');		-- default value

dll_par2_set2_launch_s <= (OTHERS=>'0');		-- default value

dll_par3_set2_launch_s <= (OTHERS=>'0');		-- default value

elsif clk_is'event and clk_is = '1' then

if pll_par1_set2_launch_enable_is = '1' then

pll_par1_set2_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par2_set2_launch_enable_is = '1' then

pll_par2_set2_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par3_set2_launch_enable_is = '1' then

pll_par3_set2_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par4_set2_launch_enable_is = '1' then

pll_par4_set2_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par1_set2_launch_enable_is = '1' then

fll_par1_set2_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par2_set2_launch_enable_is = '1' then

fll_par2_set2_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par3_set2_launch_enable_is = '1' then

fll_par3_set2_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par1_set2_launch_enable_is = '1' then

dll_par1_set2_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par2_set2_launch_enable_is = '1' then

dll_par2_set2_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par3_set2_launch_enable_is = '1' then

dll_par3_set2_launch_s <= up_data_is(31 downto 0);

end if;

end if;

--Launch phase, Set3

if POR_is = '1' then

pll_par1_set3_launch_s <= (OTHERS=>'0');		-- default value

pll_par2_set3_launch_s <= (OTHERS=>'0');		-- default value

pll_par3_set3_launch_s <= (OTHERS=>'0');		-- default value

pll_par4_set3_launch_s <= (OTHERS=>'0');		-- default value

fll_par1_set3_launch_s <= (OTHERS=>'0');		-- default value

fll_par2_set3_launch_s <= (OTHERS=>'0');		-- default value

fll_par3_set3_launch_s <= (OTHERS=>'0');		-- default value

dll_par1_set3_launch_s <= (OTHERS=>'0');		-- default value

dll_par2_set3_launch_s <= (OTHERS=>'0');		-- default value

dll_par3_set3_launch_s <= (OTHERS=>'0');		-- default value

elsif clk_is'event and clk_is = '1' then

if pll_par1_set3_launch_enable_is = '1' then

pll_par1_set3_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par2_set3_launch_enable_is = '1' then

pll_par2_set3_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par3_set3_launch_enable_is = '1' then

pll_par3_set3_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par4_set3_launch_enable_is = '1' then

pll_par4_set3_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par1_set3_launch_enable_is = '1' then

fll_par1_set3_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par2_set3_launch_enable_is = '1' then

fll_par2_set3_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par3_set3_launch_enable_is = '1' then

fll_par3_set3_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par1_set3_launch_enable_is = '1' then

dll_par1_set3_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par2_set3_launch_enable_is = '1' then

dll_par2_set3_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par3_set3_launch_enable_is = '1' then

dll_par3_set3_launch_s <= up_data_is(31 downto 0);

end if;

end if;

--Launch phase, Set4

if POR_is ='1' then

pll_par1_set4_launch_s <= (OTHERS=>'0');		-- default value

pll_par2_set4_launch_s <= (OTHERS=>'0');		-- default value

pll_par3_set4_launch_s <= (OTHERS=>'0');		-- default value

pll_par4_set4_launch_s <= (OTHERS=>'0');		-- default value

fll_par1_set4_launch_s <= (OTHERS=>'0');		-- default value

fll_par2_set4_launch_s <= (OTHERS=>'0');		-- default value

fll_par3_set4_launch_s <= (OTHERS=>'0');		-- default value

dll_par1_set4_launch_s <= (OTHERS=>'0');		-- default value

dll_par2_set4_launch_s <= (OTHERS=>'0');		-- default value

dll_par3_set4_launch_s <= (OTHERS=>'0');		-- default value

elsif clk_is'event and clk_is = '1' then

if pll_par1_set4_launch_enable_is = '1' then

pll_par1_set4_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par2_set4_launch_enable_is = '1' then

pll_par2_set4_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par3_set4_launch_enable_is = '1' then

pll_par3_set4_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par4_set4_launch_enable_is = '1' then

pll_par4_set4_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par1_set4_launch_enable_is = '1' then

fll_par1_set4_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par2_set4_launch_enable_is = '1' then

fll_par2_set4_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par3_set4_launch_enable_is = '1' then

fll_par3_set4_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par1_set4_launch_enable_is = '1' then

dll_par1_set4_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par2_set4_launch_enable_is = '1' then

dll_par2_set4_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par3_set4_launch_enable_is = '1' then

dll_par3_set4_launch_s <= up_data_is(31 downto 0);

end if;

end if;

--Launch phase, Set5

if POR_is ='1' then

pll_par1_set5_launch_s <= (OTHERS=>'0');		-- default value

pll_par2_set5_launch_s <= (OTHERS=>'0');		-- default value

pll_par3_set5_launch_s <= (OTHERS=>'0');		-- default value

pll_par4_set5_launch_s <= (OTHERS=>'0');		-- default value

fll_par1_set5_launch_s <= (OTHERS=>'0');		-- default value

fll_par2_set5_launch_s <= (OTHERS=>'0');		-- default value

fll_par3_set5_launch_s <= (OTHERS=>'0');		-- default value

dll_par1_set5_launch_s <= (OTHERS=>'0');		-- default value

dll_par2_set5_launch_s <= (OTHERS=>'0');		-- default value

dll_par3_set5_launch_s <= (OTHERS=>'0');		-- default value

elsif clk_is'event and clk_is = '1' then

if pll_par1_set5_launch_enable_is = '1' then

pll_par1_set5_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par2_set5_launch_enable_is = '1' then

pll_par2_set5_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par3_set5_launch_enable_is = '1' then

pll_par3_set5_launch_s <= up_data_is(31 downto 0);

end if;

if pll_par4_set5_launch_enable_is = '1' then

pll_par4_set5_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par1_set5_launch_enable_is = '1' then

fll_par1_set5_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par2_set5_launch_enable_is = '1' then

fll_par2_set5_launch_s <= up_data_is(31 downto 0);

end if;

if fll_par3_set5_launch_enable_is = '1' then

fll_par3_set5_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par1_set5_launch_enable_is = '1' then

dll_par1_set5_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par2_set5_launch_enable_is = '1' then

dll_par2_set5_launch_s <= up_data_is(31 downto 0);

end if;

if dll_par3_set5_launch_enable_is = '1' then

dll_par3_set5_launch_s <= up_data_is(31 downto 0);

end if;

end if;

if POR_is = '1' then

carr_aided_loop_s      <= '1';		      -- default value

fll_zero_en_dis_s    <= '1';           -- default value

loop_par_status_s    <= (OTHERS=>'0');	-- default value

IF_FREQ_TRK_GPS_s   <= "0000000000000001100110100010100000000000000000000";  --0.42e6 x 2^15

IF_FREQ_TRK_NAVIC_s <= "0000000000011000100110101101010000000000000000000"; --6.45e6 x 2^15

-- PLL settings

PLL_C1_set1_launch_s <= "00000000000000000000000001001001";

PLL_C2_set1_launch_s <= "00000000000000000000101000001000";

PLL_C3_set1_launch_s <= "00000000000000001010111010100100";

PLL_C4_set1_launch_s <= "00000000000001001000111001100011";

PLL_C1_set2_launch_s <= "00000000000000000000000000111100";

PLL_C2_set2_launch_s <= "00000000000000000000011011111000";

PLL_C3_set2_launch_s <= "00000000000000000110010100010000";

PLL_C4_set2_launch_s <= "00000000000000100011001001111110";

PLL_C1_set3_launch_s <= "00000000000000000000000000110000";

PLL_C2_set3_launch_s <= "00000000000000000000010001110110";

PLL_C3_set3_launch_s <= "00000000000000000011001110111111";

PLL_C4_set3_launch_s <= "00000000000000001110011001100110";

PLL_C1_set4_launch_s <= "00000000000000000000000000101011";

PLL_C2_set4_launch_s <= "00000000000000000000001110011101";

PLL_C3_set4_launch_s <= "00000000000000000010010110111001";

PLL_C4_set4_launch_s <= "00000000000000001001011100101010";

PLL_C1_set5_launch_s <= "00000000000000000000000000101011";

PLL_C2_set5_launch_s <= "00000000000000000000001110011101";

PLL_C3_set5_launch_s <= "00000000000000000010010110111001";

PLL_C4_set5_launch_s <= "00000000000000001001011100101010";

-- FLL settings

FLL_C1_set1_launch_s <= "00000000000000000000000000100101";

FLL_C2_set1_launch_s <= "00000000000000000000000100000001";

FLL_C3_set1_launch_s <= "00000000000000000000110111111011";

FLL_C1_set2_launch_s <= "00000000000000000000000000011111";

FLL_C2_set2_launch_s <= "00000000000000000000000010110011";

FLL_C3_set2_launch_s <= "00000000000000000000100000010111";

FLL_C1_set3_launch_s <= "00000000000000000000000000011001";

FLL_C2_set3_launch_s <= "00000000000000000000000001110010";

FLL_C3_set3_launch_s <= "00000000000000000000010000100101";

FLL_C1_set4_launch_s <= "00000000000000000000000000001111";

FLL_C2_set4_launch_s <= "00000000000000000000000000101101";

FLL_C3_set4_launch_s <= "00000000000000000000000100000011";

FLL_C1_set5_launch_s <= "00000000000000000000000000001001";

FLL_C2_set5_launch_s <= "00000000000000000000000000010000";

FLL_C3_set5_launch_s <= "00000000000000000000000000111000";

-- DLL settings

DLL_C1_set1_launch_s <= "00000000000000000000000000010010";

DLL_C2_set1_launch_s <= "00000000000000000000000001000000";

DLL_C3_set1_launch_s <= "00000000000000000000000110111111";

DLL_C1_set2_launch_s <= "00000000000000000000000000001111";

DLL_C2_set2_launch_s <= "00000000000000000000000000101101";

DLL_C3_set2_launch_s <= "00000000000000000000000100000011";

DLL_C1_set3_launch_s <= "00000000000000000000000000001100";

DLL_C2_set3_launch_s <= "00000000000000000000000000011101";

DLL_C3_set3_launch_s <= "00000000000000000000000010000100";

DLL_C1_set4_launch_s <= "00000000000000000000000000001001";

DLL_C2_set4_launch_s <= "00000000000000000000000000010000";

DLL_C3_set4_launch_s <= "00000000000000000000000000111000";

DLL_C1_set5_launch_s <= "00000000000000000000000000000110";

DLL_C2_set5_launch_s <= "00000000000000000000000000000111";

DLL_C3_set5_launch_s <= "00000000000000000000000000010001";

DLL1_C1_set_launch_s <= "00000000000000000000000000000100";

elsif clk_is'event and clk_is = '1' then

if(IF_FREQ_TRK_GPS_sel_is = '1') then

IF_FREQ_TRK_GPS_s(46 downto 15) <= up_data_is;

end if;

if(IF_FREQ_TRK_NAVIC_sel_is = '1') then

IF_FREQ_TRK_NAVIC_s(46 downto 15) <= up_data_is;

end if;

if carr_aided_loop_sel_is= '1' then

carr_aided_loop_s <= up_data_is(0);

end if;

if fll_zero_en_dis_sel_is= '1' then

fll_zero_en_dis_s <= up_data_is(0);

end if;

if loop_par_status1_enable_is= '1' then

loop_par_status_s(15 downto 0) <= up_data_is(15 downto 0);

end if;

if(loop_par_status_s(0) = '1') then

PLL_C4_set1_launch_s <= pll_par4_set1_launch_s;

PLL_C3_set1_launch_s <= pll_par3_set1_launch_s;

PLL_C2_set1_launch_s <= pll_par2_set1_launch_s;

PLL_C1_set1_launch_s <= pll_par1_set1_launch_s;

end if;

if(loop_par_status_s(1) = '1') then

PLL_C4_set2_launch_s <= pll_par4_set2_launch_s;

PLL_C3_set2_launch_s <= pll_par3_set2_launch_s;

PLL_C2_set2_launch_s <= pll_par2_set2_launch_s;

PLL_C1_set2_launch_s <= pll_par1_set2_launch_s;

end if;

if(loop_par_status_s(2) = '1') then

PLL_C4_set3_launch_s <= pll_par4_set3_launch_s;

PLL_C3_set3_launch_s <= pll_par3_set3_launch_s;

PLL_C2_set3_launch_s <= pll_par2_set3_launch_s;

PLL_C1_set3_launch_s <= pll_par1_set3_launch_s;

end if;

if(loop_par_status_s(3) = '1') then

PLL_C4_set4_launch_s <= pll_par4_set4_launch_s;

PLL_C3_set4_launch_s <= pll_par3_set4_launch_s;

PLL_C2_set4_launch_s <= pll_par2_set4_launch_s;

PLL_C1_set4_launch_s <= pll_par1_set4_launch_s;

end if;

if(loop_par_status_s(4) = '1') then

PLL_C4_set5_launch_s <= pll_par4_set5_launch_s;

PLL_C3_set5_launch_s <= pll_par3_set5_launch_s;

PLL_C2_set5_launch_s <= pll_par2_set5_launch_s;

PLL_C1_set5_launch_s <= pll_par1_set5_launch_s;

end if;

if(loop_par_status_s(5) = '1') then

FLL_C3_set1_launch_s <= fll_par3_set1_launch_s;

FLL_C2_set1_launch_s <= fll_par2_set1_launch_s;

FLL_C1_set1_launch_s <= fll_par1_set1_launch_s;

end if;

if(loop_par_status_s(6) = '1') then

FLL_C3_set2_launch_s <= fll_par3_set2_launch_s;

FLL_C2_set2_launch_s <= fll_par2_set2_launch_s;

FLL_C1_set2_launch_s <= fll_par1_set2_launch_s;

end if;

if(loop_par_status_s(7) = '1') then

FLL_C3_set3_launch_s <= fll_par3_set3_launch_s;

FLL_C2_set3_launch_s <= fll_par2_set3_launch_s;

FLL_C1_set3_launch_s <= fll_par1_set3_launch_s;

end if;

if(loop_par_status_s(8) = '1') then

FLL_C3_set4_launch_s <= fll_par3_set4_launch_s;

FLL_C2_set4_launch_s <= fll_par2_set4_launch_s;

FLL_C1_set4_launch_s <= fll_par1_set4_launch_s;

end if;

if(loop_par_status_s(9) = '1') then

FLL_C3_set5_launch_s <= fll_par3_set5_launch_s;

FLL_C2_set5_launch_s <= fll_par2_set5_launch_s;

FLL_C1_set5_launch_s <= fll_par1_set5_launch_s;

end if;

if(loop_par_status_s(10) = '1') then

DLL_C3_set1_launch_s <= dll_par3_set1_launch_s;

DLL_C2_set1_launch_s <= dll_par2_set1_launch_s;

DLL_C1_set1_launch_s <= dll_par1_set1_launch_s;

end if;

if(loop_par_status_s(11) = '1') then

DLL_C3_set2_launch_s <= dll_par3_set2_launch_s;

DLL_C2_set2_launch_s <= dll_par2_set2_launch_s;

DLL_C1_set2_launch_s <= dll_par1_set2_launch_s;

end if;

if(loop_par_status_s(12) = '1') then

DLL_C3_set3_launch_s <= dll_par3_set3_launch_s;

DLL_C2_set3_launch_s <= dll_par2_set3_launch_s;

DLL_C1_set3_launch_s <= dll_par1_set3_launch_s;

end if;

if(loop_par_status_s(13) = '1') then

DLL_C3_set4_launch_s <= dll_par3_set4_launch_s;

DLL_C2_set4_launch_s <= dll_par2_set4_launch_s;

DLL_C1_set4_launch_s <= dll_par1_set4_launch_s;

end if;

if(loop_par_status_s(14) = '1') then

DLL_C3_set5_launch_s <= dll_par3_set5_launch_s;

DLL_C2_set5_launch_s <= dll_par2_set5_launch_s;

DLL_C1_set5_launch_s <= dll_par1_set5_launch_s;

end if;

if(loop_par_status_s(15) = '1') then

DLL1_C1_set_launch_s <= cdll_par1_set1_launch_s;

end if; end if;

end process P_loop_filter_reg;

end top_discriminator_a;

71) top_module_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use WORK.signal_type_p.ALL;

Library UNISIM;

use UNISIM.vcomponents.all;

entity top_module_e is

port(

-- 40MHz Differential Clock

clk_40MHz_in_p_is: in std_logic;

clk_40MHz_in_n_is: in std_logic;

A1_I0_p_is : IN  std_logic;

A1_I0_n_is : IN  std_logic;

A1_I1_p_is : IN  std_logic;

A1_I1_n_is : IN  std_logic;

A1_I2_p_is : IN  std_logic;

A1_I2_n_is : IN  std_logic;

A1_Q0_p_is : IN  std_logic;

A1_Q0_n_is : IN  std_logic;

A1_Q1_p_is : IN  std_logic;

A1_Q1_n_is : IN  std_logic;

A1_Q2_p_is : IN  std_logic;

A1_Q2_n_is : IN  std_logic;

A2_I0_p_is : IN  std_logic;

A2_I0_n_is : IN  std_logic;

A2_I1_p_is : IN  std_logic;

A2_I1_n_is : IN  std_logic;

A2_I2_p_is : IN  std_logic;

A2_I2_n_is : IN  std_logic;

A2_Q0_p_is : IN  std_logic;

A2_Q0_n_is : IN  std_logic;

A2_Q1_p_is : IN  std_logic;

A2_Q1_n_is : IN  std_logic;

A2_Q2_p_is : IN  std_logic;

A2_Q2_n_is : IN  std_logic;

A3_I0_p_is : IN  std_logic;

A3_I0_n_is : IN  std_logic;

A3_I1_p_is : IN  std_logic;

A3_I1_n_is : IN  std_logic;

A3_I2_p_is : IN  std_logic;

A3_I2_n_is : IN  std_logic;

A3_Q0_p_is : IN  std_logic;

A3_Q0_n_is : IN  std_logic;

A3_Q1_p_is : IN  std_logic;

A3_Q1_n_is : IN  std_logic;

A3_Q2_p_is : IN  std_logic;

A3_Q2_n_is : IN  std_logic;

A4_I0_p_is : IN  std_logic;

A4_I0_n_is : IN  std_logic;

A4_I1_p_is : IN  std_logic;

A4_I1_n_is : IN  std_logic;

A4_I2_p_is : IN  std_logic;

A4_I2_n_is : IN  std_logic;

A4_Q0_p_is : IN  std_logic;

A4_Q0_n_is : IN  std_logic;

A4_Q1_p_is : IN  std_logic;

A4_Q1_n_is : IN  std_logic;

A4_Q2_p_is : IN  std_logic;

A4_Q2_n_is : IN  std_logic;

-- ADC interface/ correlator interface

--          ant1_data_is : in std_logic_vector(5 downto 0);

--		ant2_data_is : in std_logic_vector(5 downto 0);

--		ant3_data_is : in std_logic_vector(5 downto 0);

--	 ant4_data_is : in std_logic_vector(5 downto 0);

ms_10_interrupt_os, ms_500_interrupt_os : out std_logic;

PPS_flag_out_RT1_os, PPS_flag_out_RT2_os , PPS_flag_out_RT3_os: out std_logic;                --gpio

pps_signal_os: out std_logic_vector(5 downto 0);

--processor data address bus

clk_40MHz_pr_out_os : out std_logic;

reset_pr_os : out std_logic;

up_address_is: in std_logic_vector(27 downto 2);

data_bus_up_bi : inout std_logic_vector(31 downto 0);

brdy_os: out std_logic;

prom_cs_is : in std_logic;

-- IO cs/read/write

iosn_is, oen_is, writen_is: in std_logic;

-- command interface

hwdt_enable_is, swdt_enable_is: in std_logic;

hwdt_disable_is, swdt_disable_is: in std_logic;

-- mil-1553

--RT1

rt_address_is: in std_logic_vector(1 downto 0);

RXENA_TX_R_RT1_os   : out std_logic;

RXOUTA_TX_RT1_is    : in  std_logic;

RXOUTA1_TX_RT1_is    : in  std_logic;

RXENB_TX_R_RT1_os   : out std_logic;

RXOUTB_TX_RT1_is    : in  std_logic;

RXOUTB1_TX_RT1_is    : in  std_logic;

TXINHA_TX_RT1_os  : out std_logic;

TXINA_TX_RT1_os   : out std_logic;

TXINA1_TX_RT1_os   : out std_logic;

TXINHB_TX_RT1_os  : out std_logic;

TXINB_TX_RT1_os   : out std_logic;

TXINB1_TX_RT1_os: out std_logic;

TSM_flag_out_RT1_os : out std_logic;  --gpio

--RT2

RXENA_TX_R_RT2_os   : out std_logic;

RXOUTA_TX_RT2_is    : in  std_logic;

RXOUTA1_TX_RT2_is    : in  std_logic;

RXENB_TX_R_RT2_os   : out std_logic;

RXOUTB_TX_RT2_is    : in  std_logic;

RXOUTB1_TX_RT2_is    : in  std_logic;

TXINHA_TX_RT2_os  : out std_logic;

TXINA_TX_RT2_os   : out std_logic;

TXINA1_TX_RT2_os   : out std_logic;

TXINHB_TX_RT2_os  : out std_logic;

TXINB_TX_RT2_os   : out std_logic;

TXINB1_TX_RT2_os: out std_logic;

TSM_flag_out_RT2_os : out std_logic;  --gpio

--RT3

RXENA_TX_R_RT3_os   : out std_logic;

RXOUTA_TX_RT3_is    : in  std_logic;

RXOUTA1_TX_RT3_is    : in  std_logic;

RXENB_TX_R_RT3_os   : out std_logic;

RXOUTB_TX_RT3_is    : in  std_logic;

RXOUTB1_TX_RT3_is    : in  std_logic;

TXINHA_TX_RT3_os  : out std_logic;

TXINA_TX_RT3_os   : out std_logic;

TXINA1_TX_RT3_os   : out std_logic;

TXINHB_TX_RT3_os  : out std_logic;

TXINB_TX_RT3_os   : out std_logic;

TXINB1_TX_RT3_os: out std_logic;

TSM_flag_out_RT3_os : out std_logic;  --gpio

--GP2015 interface

lock_detector_in_ant1_is,lock_detector_in_ant2_is : in std_logic;

lock_detector_in_ant3_is,lock_detector_in_ant4_is : in std_logic;

--RFFC5071 interface

upconv1_spi_reg_os: out std_logic_vector(4 downto 0);

upconv2_spi_reg_os: out std_logic_vector(4 downto 0);

upconv1_spi_gpio4_is: in std_logic;

upconv2_spi_gpio4_is: in std_logic;

--FPGA Configuration memory CRC status

configt_crc_error_to_up_os: out std_logic;

uart_txd_is: in std_logic;

rwe3_is    : in std_logic;

wdt_l_is : in std_logic;

bexc_l_is: in std_logic;

spi_clk_SY_is: in std_logic;

spi_en_SY_os: OUT std_logic;

spi_mosi_SY_os: OUT std_logic;

spi_miso_SY_is: in std_logic

);

end top_module_e;

architecture top_module_a of top_module_e is

signal ant1_data_is,ant2_data_is,ant3_data_is,ant4_data_is:  std_logic_vector(5 downto 0);

COMPONENT clk_gen_e

PORT(

clk_40MHz_in_p_is : in std_logic;

clk_40MHz_in_n_is : in std_logic;

reset_is: in std_logic;

clk_40MHz_os: out std_logic;

clk_40MHz_pr_out_os: out std_logic;

clk_20MHz_os: out std_logic

);

END COMPONENT;

signal clk_20MHz_s,clk_40MHz_s:  std_logic ;

signal rt_address_RT1_s,rt_address_RT2_s ,rt_address_RT3_s: std_logic_vector(5 downto 0);

COMPONENT reset_sync_e

PORT(

clk_40MHz_is : IN std_logic;

clk_20MHz_is : IN std_logic;

soft_reset_is : IN std_logic;

POR_sync_os : OUT std_logic;

POR_G_sync_os : OUT std_logic;

mil_reset_os : OUT std_logic

);

END COMPONENT;

signal POR_G_sync_s : std_logic;

signal POR_sync_s: std_logic;

COMPONENT config_crc_e

PORT(

clk_40MHz_is : in std_logic;

POR_in_is    : in std_logic;

crc_error_os: out std_logic;

ecc_error_os: out std_logic;

syndrome_valid_os: out std_logic;

icap_busy_os: out std_logic;

syndrome_os  : out std_logic_vector(11 downto 0);

synd_valid_count_os: out std_logic_vector(7 downto 0)

);

END COMPONENT;

signal crc_error_s,ecc_error_s,syndrome_valid_s : std_logic;

signal icap_busy_s : std_logic;

signal syndrome_s : std_logic_vector(11 downto 0);

signal synd_valid_count_s: std_logic_vector(7 downto 0);

COMPONENT rd_wr_sync_e

PORT(

clk_40MHz_is : IN std_logic;

clk_20MHz_is : IN std_logic;

reset_is : IN std_logic;

correlator_cs_is : IN std_logic;

mil_rt1_ip_csn_is : IN std_logic;

mil_data_ram1_cs_is : IN std_logic;

mil_command_ram1_cs_is : IN std_logic;

mil_descriptor_ram1_cs_is : IN std_logic;

mil_rt2_ip_csn_is : IN std_logic;

mil_data_ram2_cs_is : IN std_logic;

mil_command_ram2_cs_is : IN std_logic;

mil_descriptor_ram2_cs_is : IN std_logic;

mil_rt3_ip_csn_is : IN std_logic;

mil_data_ram3_cs_is : IN std_logic;

mil_command_ram3_cs_is : IN std_logic;

mil_descriptor_ram3_cs_is : IN std_logic;

oen_is : IN std_logic;

writen_is : IN std_logic;

iosn_is : IN std_logic;

fpga_we_os : OUT std_logic;

mil_we_os : OUT std_logic;

mil_oe_os : OUT std_logic;

brdy_os : OUT std_logic;

prom_cs_is : in std_logic;

prom_re_os: out std_logic

);

END COMPONENT;

signal mil_block_reset_s,reset_n_s,correlator_cs_s, mil_rt1_ip_csn_s, mil_data_ram1_cs_s : std_logic;

signal mil_rt2_ip_csn_s, mil_data_ram2_cs_s,mil_command_ram2_cs_s, mil_descriptor_ram2_cs_s: std_logic;

signal mil_rt3_ip_csn_s, mil_data_ram3_cs_s,mil_command_ram3_cs_s, mil_descriptor_ram3_cs_s: std_logic;

signal mil_command_ram1_cs_s, mil_descriptor_ram1_cs_s    : std_logic;

signal fpga_we_s, mil_we_s, mil_oe_s : std_logic;

signal ms_500_interrupt_temp_s: std_logic;

component bus_interface_e is

port(   data_bus_up_bi: inout std_logic_vector(31 downto 0);

fpga_data_out_to_up_is: in std_logic_vector(31 downto 0);

fpga_data_in_from_up_os: out std_logic_vector(31 downto 0);

oen_is : in std_logic ;

iosn_is : in std_logic;

prom_cs_is : in std_logic

);

end component;

signal fpga_data_out_to_up_s:  std_logic_vector(31 downto 0);

signal fpga_data_in_from_up_s:  std_logic_vector(31 downto 0);

component address_decoder_e is

port(

clk_40M_is : in std_logic;

reset_is : in std_logic;

up_address_is: in std_logic_vector(27 downto 2);

iosn_is : in std_logic;

data_from_correlator_is: in std_logic_vector(31 downto 0);

rt_address_is: in std_logic_vector(1 downto 0);

pr_read_data_from_rt1_ip_is: in std_logic_vector(15 downto 0);

pr_read_data_from_descr_ram1_is: in std_logic_vector(15 downto 0);

pr_read_data_from_comm_ram1_is:  in std_logic_vector(15 downto 0);

pr_read_data_from_data_ram1_is:  in std_logic_vector(15 downto 0);

pr_read_data_from_rt2_ip_is: in std_logic_vector(15 downto 0);

pr_read_data_from_descr_ram2_is: in std_logic_vector(15 downto 0);

pr_read_data_from_comm_ram2_is:  in std_logic_vector(15 downto 0);

pr_read_data_from_data_ram2_is:  in std_logic_vector(15 downto 0);

pr_read_data_from_rt3_ip_is: in std_logic_vector(15 downto 0);

pr_read_data_from_descr_ram3_is: in std_logic_vector(15 downto 0);

pr_read_data_from_comm_ram3_is:  in std_logic_vector(15 downto 0);

pr_read_data_from_data_ram3_is:  in std_logic_vector(15 downto 0);

correlator_cs_os: out std_logic;

mil_rt1_ip_csn_os,mil_rt2_ip_csn_os,mil_rt3_ip_csn_os : out std_logic;

mil_data_ram1_cs_os,mil_data_ram2_cs_os,mil_data_ram3_cs_os: out std_logic;

mil_command_ram1_cs_os,mil_command_ram2_cs_os,mil_command_ram3_cs_os: out std_logic;

mil_descriptor_ram1_cs_os,mil_descriptor_ram2_cs_os,mil_descriptor_ram3_cs_os : out std_logic;

fpga_data_out_to_up_os:out  std_logic_vector(31 downto 0);

rt_address_RT1_os: out std_logic_vector(5 downto 0);

rt_address_RT2_os: out std_logic_vector(5 downto 0);

rt_address_RT3_os: out std_logic_vector(5 downto 0);

prom_cs_is: in std_logic;

prom_data_to_up_is: in std_logic_vector(31 downto 0)

);

end component;

component mil_top_level_e is

port(-- Input signals

clk_is     : in std_logic ;

reset_n_is : in std_logic ;

rd_pr_sync_is: in std_logic;

wr_pr_sync_is: in std_logic;

mil_command_ram_cs_is    : in std_logic;

mil_descriptor_ram_cs_is : in std_logic;

mil_data_ram_cs_is       : in std_logic;

mil_rt_ip_csn_is         : in std_logic;

pr_address_is    : in std_logic_vector(15 downto 0);

pr_write_data_is : in std_logic_vector(15 downto 0);

rt_address_is          : in std_logic_vector(5 downto 0);

reset_tsm_interrupt_is : in std_logic;

reset_mil_interrupt_is : in std_logic;

BUSAINP_rt_is    : in  std_logic;

BUSAINN_rt_is    : in  std_logic;

BUSBINP_rt_is    : in  std_logic;

BUSBINN_rt_is    : in  std_logic;

-- Output signals

BUSAINEN_rt_os       : out std_logic;

BUSAOUTIN_rt_os      : out std_logic;

BUSAOUTP_rt_out_os   : out std_logic; -- BUS-A Output Positive data

BUSAOUTN_rt_out_os   : out std_logic; -- BUS-A Output Negative data

BUSBINEN_rt_os       : out std_logic;

BUSBOUTIN_rt_os      : out std_logic;

BUSBOUTP_rt_out_os   : out std_logic; -- BUS-A Output Positive data

BUSBOUTN_rt_out_os   : out std_logic; -- BUS-B Output Negative data

pr_read_data_from_rt_ip_os,pr_read_data_from_descr_ram_os   : out std_logic_vector(15 downto 0);

pr_read_data_from_comm_ram_os,pr_read_data_from_data_ram_os : out std_logic_vector(15 downto 0);

tsm_interrupt_os : out std_logic;

mil_1553_interrupt_os: out std_logic

);

end component;

signal pr_read_data_from_rt1_ip_s, pr_read_data_from_descr_ram1_s,pr_read_data_from_comm_ram1_s,pr_read_data_from_data_ram1_s : std_logic_vector(15 downto 0);

signal pr_read_data_from_rt2_ip_s, pr_read_data_from_descr_ram2_s,pr_read_data_from_comm_ram2_s,pr_read_data_from_data_ram2_s : std_logic_vector(15 downto 0);

signal pr_read_data_from_rt3_ip_s, pr_read_data_from_descr_ram3_s,pr_read_data_from_comm_ram3_s,pr_read_data_from_data_ram3_s : std_logic_vector(15 downto 0);

component gnss_correlator_top_e is

port(

clk_is,POR_G_is,POR_in_is: in std_logic;

ant1_data_is : in std_logic_vector(5 downto 0);

ant2_data_is : in std_logic_vector(5 downto 0);

ant3_data_is : in std_logic_vector(5 downto 0);

ant4_data_is : in std_logic_vector(5 downto 0);

soft_reset_os: out std_logic;

rt_address_RT1_is: in std_logic_vector(5 downto 0);

rt_address_RT2_is: in std_logic_vector(5 downto 0);

rt_address_RT3_is: in std_logic_vector(5 downto 0);

hwdt_cmd_is, swdt_cmd_is: in std_logic;

data_processor_is : in std_logic_vector(31 downto 0);

wr_pr_is: in std_logic;

cs_correlator_is : in std_logic;

up_address_is: in std_logic_vector(11 downto 0);

ms_10_interrupt_os, ms_500_interrupt_os,reset_processor_os : out std_logic;

data_to_processor_os: out std_logic_vector(31 downto 0);

reset_tsm_interrupt_RT1_os : out std_logic;

TSM_interrupt_RT1_is : in std_logic;

TSM_flag_out_RT1_os,PPS_flag_out_RT1_os : out std_logic;

reset_tsm_interrupt_RT2_os : out std_logic;

TSM_interrupt_RT2_is : in std_logic;

TSM_flag_out_RT2_os,PPS_flag_out_RT2_os : out std_logic;

reset_tsm_interrupt_RT3_os : out std_logic;

TSM_interrupt_RT3_is : in std_logic;

TSM_flag_out_RT3_os,PPS_flag_out_RT3_os : out std_logic;

reset_mil_interrupt_RT1_os: out std_logic;

reset_mil_interrupt_RT2_os: out std_logic;

reset_mil_interrupt_RT3_os: out std_logic;

mil_1553_interrupt_RT1_is: in std_logic;

mil_1553_interrupt_RT2_is: in std_logic;

mil_1553_interrupt_RT3_is: in std_logic;

upconv1_spi_reg_os, upconv2_spi_reg_os: out std_logic_vector(4 downto 0);

upconv1_spi_gpio4_is, upconv2_spi_gpio4_is:  in std_logic;

lock_detector_ant1_is,lock_detector_ant2_is : in std_logic;

lock_detector_ant3_is,lock_detector_ant4_is : in std_logic;

pps_signal_os: out std_logic_vector(5 downto 0);

config_crc_err_is: in std_logic;

config_ecc_err_is: in std_logic;

config_synd_valid_is: in std_logic;

icap_busy_is : in std_logic;

syndrome_is: in std_logic_vector(11 downto 0);

synd_valid_count_is: in std_logic_vector(7 downto 0);

rt_adddr_sel_is : in std_logic_vector(1 downto 0);

uart_txd_is: in std_logic;

rwe3_is    : in std_logic;

wdt_l_is : in std_logic;

bexc_l_is: in std_logic;

spi_clk_SY_is: in std_logic;

spi_en_SY_os: OUT std_logic;

spi_mosi_SY_os: OUT std_logic;

spi_miso_SY_is: in std_logic

);

end component;

signal data_from_correlator_s: std_logic_vector(31 downto 0);

signal reset_tsm_interrupt_RT1_s,tsm_interrupt_RT1_s: std_logic;

signal reset_tsm_interrupt_RT2_s,tsm_interrupt_RT2_s: std_logic;

signal reset_tsm_interrupt_RT3_s,tsm_interrupt_RT3_s: std_logic;

signal reset_mil_interrupt_RT1_s,mil_1553_interrupt_RT1_s : std_logic;

signal reset_mil_interrupt_RT2_s,mil_1553_interrupt_RT2_s : std_logic;

signal reset_mil_interrupt_RT3_s,mil_1553_interrupt_RT3_s : std_logic;

component lock_deglitch_e is

port(clk_40MHz_is: in std_logic;

reset_is: in std_logic;

cmd_in_is: in std_logic;

cmd_deg_os : out std_logic);

end component;

component pulse_deglitch_e

port(

clk_40MHz_is : in std_logic;

reset_is : in std_logic;

pulse_cmd_is : in std_logic;

pulse_deg_os : out std_logic

);

end component ;

signal hwdt_enable_deg_s, swdt_enable_deg_s : std_logic;

signal hwdt_disable_deg_s, swdt_disable_deg_s : std_logic;

component wdt_pulse_to_level_e

port(  clk_40MHz_is : IN std_logic;

reset_is : IN std_logic;

wdt_en_is : IN std_logic;

wdt_dis_is : IN std_logic;

wdt_level_os : OUT std_logic);

end component;

signal hwdt_level_out_s,swdt_level_out_s : std_logic;

component cmd_synchronizer_e is

port(clk_40MHz_is: in std_logic;

POR_G_is: in std_logic;

signal_in_is: in std_logic;

signal_sync_os : out std_logic);

end component;

signal hwdt_enable_sync_s, hwdt_disable_sync_s  : std_logic;

signal swdt_enable_sync_s, swdt_disable_sync_s  : std_logic;

signal soft_reset_s: std_logic;

signal lock_detector_ant1_out_s,lock_detector_ant2_out_s : std_logic;

signal lock_detector_ant3_out_s,lock_detector_ant4_out_s : std_logic;

signal lock_detector_ant1_sync_s,lock_detector_ant2_sync_s : std_logic;

signal lock_detector_ant3_sync_s,lock_detector_ant4_sync_s : std_logic;

signal upconv1_spi_gpio4_sync_s, upconv2_spi_gpio4_sync_s  : std_logic;

component leon_prom_module_e is

port(clk_40MHz_is, re_is : in std_logic;

address_is: in std_logic_vector(16 downto 0);

data_out_os : out std_logic_vector(31 downto 0));

end component;

signal prom_re_s: std_logic;

signal data_out_prom_s : std_logic_vector(31 downto 0);

signal syn1_adc1_s, syn2_adc1_s : std_logic_vector(5 downto 0);

begin

A1_I0_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant1_data_is(3),  -- Buffer output

I => A1_I0_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A1_I0_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A1_I1_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant1_data_is(4),  -- Buffer output

I => A1_I1_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A1_I1_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A1_I2_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant1_data_is(5),  -- Buffer output

I => A1_I2_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A1_I2_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A1_Q0_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant1_data_is(0),  -- Buffer output

I => A1_Q0_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A1_Q0_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A1_Q1_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant1_data_is(1),  -- Buffer output

I => A1_Q1_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A1_Q1_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A1_Q2_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant1_data_is(2),  -- Buffer output

I => A1_Q2_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A1_Q2_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A2_I0_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant2_data_is(3),  -- Buffer output

I => A2_I0_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A2_I0_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A2_I1_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant2_data_is(4),  -- Buffer output

I => A2_I1_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A2_I1_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A2_I2_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant2_data_is(5),  -- Buffer output

I => A2_I2_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A2_I2_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A2_Q0_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant2_data_is(0),  -- Buffer output

I => A2_Q0_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A2_Q0_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A2_Q1_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant2_data_is(1),  -- Buffer output

I => A2_Q1_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A2_Q1_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A2_Q2_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant2_data_is(2),  -- Buffer output

I => A2_Q2_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A2_Q2_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A3_I0_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant3_data_is(3),  -- Buffer output

I => A3_I0_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A3_I0_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A3_I1_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant3_data_is(4),  -- Buffer output

I => A3_I1_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A3_I1_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A3_I2_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant3_data_is(5),  -- Buffer output

I => A3_I2_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A3_I2_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A3_Q0_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant3_data_is(0),  -- Buffer output

I => A3_Q0_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A3_Q0_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A3_Q1_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant3_data_is(1),  -- Buffer output

I => A3_Q1_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A3_Q1_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A3_Q2_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant3_data_is(2),  -- Buffer output

I => A3_Q2_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A3_Q2_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A4_I0_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant4_data_is(3),  -- Buffer output

I => A4_I0_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A4_I0_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A4_I1_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant4_data_is(4),  -- Buffer output

I => A4_I1_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A4_I1_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A4_I2_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant4_data_is(5),  -- Buffer output

I => A4_I2_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A4_I2_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A4_Q0_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant4_data_is(0),  -- Buffer output

I => A4_Q0_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A4_Q0_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A4_Q1_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant4_data_is(1),  -- Buffer output

I => A4_Q1_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A4_Q1_n_is -- Diff_n buffer input (connect directly to top-level port)

);

A4_Q2_inst : IBUFDS

generic map (

DIFF_TERM => TRUE, -- Differential Termination

IOSTANDARD => "LVDS_25")

port map (

O => ant4_data_is(2),  -- Buffer output

I => A4_Q2_p_is,  -- Diff_p buffer input (connect directly to top-level port)

IB => A4_Q2_n_is -- Diff_n buffer input (connect directly to top-level port)

);

ds : process(clk_40MHz_s ,POR_G_sync_s)

begin

if (POR_G_sync_s='1') then

syn1_adc1_s <= (others => '0');

syn2_adc1_s <= (others => '0');

ant2_data_is <= (others => '0');

ant3_data_is <= (others => '0');

ant4_data_is <= (others => '0');

elsif (clk_40MHz_s'EVENT and clk_40MHz_s='1') then

syn1_adc1_s <= ant1_data_is;

syn2_adc1_s <= syn1_adc1_s;

ant2_data_is <= syn2_adc1_s;

ant3_data_is <= ant2_data_is;

ant4_data_is <= ant3_data_is;

end if;

end process ds;

clk_gen_inst: clk_gen_e

PORT MAP(

clk_40MHz_in_p_is   =>  clk_40MHz_in_p_is,

clk_40MHz_in_n_is   =>  clk_40MHz_in_n_is,

reset_is            => POR_G_sync_s,

clk_40MHz_os        => clk_40MHz_s,

clk_20MHz_os        => clk_20MHz_s,

clk_40MHz_pr_out_os => clk_40MHz_pr_out_os

);

reset_sync_inst: reset_sync_e

PORT MAP(

clk_40MHz_is => clk_40MHz_s,

clk_20MHz_is => clk_20MHz_s,

soft_reset_is => soft_reset_s ,

POR_sync_os => POR_sync_s ,

POR_G_sync_os => POR_G_sync_s,

mil_reset_os => mil_block_reset_s

);

config_crc_inst: config_crc_e

PORT MAP(

clk_40MHz_is => clk_40MHz_s,

POR_in_is => POR_G_sync_s,

crc_error_os => crc_error_s,

ecc_error_os => ecc_error_s,

syndrome_valid_os=> syndrome_valid_s,

icap_busy_os => icap_busy_s,

syndrome_os  => syndrome_s,

synd_valid_count_os=>synd_valid_count_s

);

configt_crc_error_to_up_os <= crc_error_s;

synchronizer_hwdt_enable : cmd_synchronizer_e

port map (   clk_40MHz_is => clk_40MHz_s,

POR_G_is => POR_G_sync_s,

signal_in_is => hwdt_enable_is,

signal_sync_os => hwdt_enable_sync_s);

synchronizer_hwdt_disable : cmd_synchronizer_e

port map (   clk_40MHz_is => clk_40MHz_s,

POR_G_is => POR_G_sync_s,

signal_in_is => hwdt_disable_is,

signal_sync_os => hwdt_disable_sync_s);

synchronizer_swdt_enable : cmd_synchronizer_e

port map (   clk_40MHz_is => clk_40MHz_s,

POR_G_is => POR_G_sync_s,

signal_in_is => swdt_enable_is,

signal_sync_os => swdt_enable_sync_s);

synchronizer_swdt_disable : cmd_synchronizer_e

port map (   clk_40MHz_is => clk_40MHz_s,

POR_G_is => POR_G_sync_s,

signal_in_is => swdt_disable_is,

signal_sync_os => swdt_disable_sync_s);

synchronizer_ld_ant1_enable : cmd_synchronizer_e

port map (   clk_40MHz_is => clk_40MHz_s,

POR_G_is => POR_G_sync_s,

signal_in_is => lock_detector_in_ant1_is,

signal_sync_os => lock_detector_ant1_sync_s);

synchronizer_ld_ant2_enable : cmd_synchronizer_e

port map (   clk_40MHz_is => clk_40MHz_s,

POR_G_is => POR_G_sync_s,

signal_in_is => lock_detector_in_ant2_is,

signal_sync_os => lock_detector_ant2_sync_s);

synchronizer_ld_ant3_enable : cmd_synchronizer_e

port map (   clk_40MHz_is => clk_40MHz_s,

POR_G_is => POR_G_sync_s,

signal_in_is => lock_detector_in_ant3_is,

signal_sync_os => lock_detector_ant3_sync_s);

synchronizer_ld_ant4_enable : cmd_synchronizer_e

port map (   clk_40MHz_is => clk_40MHz_s,

POR_G_is => POR_G_sync_s,

signal_in_is => lock_detector_in_ant4_is,

signal_sync_os => lock_detector_ant4_sync_s);

synchronizer_upconv1_spi_reg : cmd_synchronizer_e

port map (   clk_40MHz_is => clk_40MHz_s,

POR_G_is => POR_G_sync_s,

signal_in_is => upconv1_spi_gpio4_is,

signal_sync_os => upconv1_spi_gpio4_sync_s);

synchronizer_upconv2_spi_reg : cmd_synchronizer_e

port map (   clk_40MHz_is => clk_40MHz_s,

POR_G_is => POR_G_sync_s,

signal_in_is => upconv2_spi_gpio4_is,

signal_sync_os => upconv2_spi_gpio4_sync_s);

deglitch_ld_ant1 : lock_deglitch_e

port map(    clk_40MHz_is => clk_40MHz_s,

reset_is => POR_G_sync_s,

cmd_in_is => lock_detector_ant1_sync_s,

cmd_deg_os => lock_detector_ant1_out_s );

deglitch_ld_ant2 : lock_deglitch_e

port map( clk_40MHz_is => clk_40MHz_s,

reset_is => POR_G_sync_s,

cmd_in_is => lock_detector_ant2_sync_s,

cmd_deg_os => lock_detector_ant2_out_s );

deglitch_ld_ant3 : lock_deglitch_e

port map( clk_40MHz_is => clk_40MHz_s,

reset_is => POR_G_sync_s,

cmd_in_is => lock_detector_ant3_sync_s,

cmd_deg_os => lock_detector_ant3_out_s );

deglitch_ld_ant4 : lock_deglitch_e

port map( clk_40MHz_is => clk_40MHz_s,

reset_is => POR_G_sync_s,

cmd_in_is => lock_detector_ant4_sync_s,

cmd_deg_os => lock_detector_ant4_out_s );

hdwt_enable_deglitch_inst: pulse_deglitch_e

port map(        clk_40MHz_is => clk_40MHz_s,

reset_is => POR_G_sync_s,

pulse_cmd_is => hwdt_enable_sync_s ,

pulse_deg_os => hwdt_enable_deg_s);

hdwt_disable_deglitch_inst: pulse_deglitch_e

port map(        clk_40MHz_is => clk_40MHz_s,

reset_is => POR_G_sync_s,

pulse_cmd_is => hwdt_disable_sync_s ,

pulse_deg_os => hwdt_disable_deg_s);

swdt_enable_deglitch_inst: pulse_deglitch_e

port map(        clk_40MHz_is => clk_40MHz_s,

reset_is => POR_G_sync_s,

pulse_cmd_is => swdt_enable_sync_s ,

pulse_deg_os => swdt_enable_deg_s);

swdt_disable_deglitch_inst: pulse_deglitch_e

port map(        clk_40MHz_is => clk_40MHz_s,

reset_is => POR_G_sync_s,

pulse_cmd_is => swdt_disable_sync_s ,

pulse_deg_os => swdt_disable_deg_s);

hwdt_pulse_to_level_inst:  wdt_pulse_to_level_e

port map(        clk_40MHz_is => clk_40MHz_s,

reset_is =>  POR_G_sync_s ,

wdt_en_is => hwdt_enable_deg_s ,

wdt_dis_is => hwdt_disable_deg_s ,

wdt_level_os => hwdt_level_out_s );

swdt_pulse_to_level_inst:  wdt_pulse_to_level_e

port map(        clk_40MHz_is => clk_40MHz_s,

reset_is =>  POR_G_sync_s ,

wdt_en_is => swdt_enable_deg_s ,

wdt_dis_is => swdt_disable_deg_s ,

wdt_level_os => swdt_level_out_s );

bus_interface_inst : bus_interface_e

port map(

data_bus_up_bi => data_bus_up_bi,

fpga_data_out_to_up_is => fpga_data_out_to_up_s,

fpga_data_in_from_up_os => fpga_data_in_from_up_s,

oen_is => oen_is,

iosn_is => iosn_is,

prom_cs_is => prom_cs_is

);

rd_wr_sync_inst: rd_wr_sync_e PORT MAP(

clk_40MHz_is => clk_40MHz_s,

clk_20MHz_is => clk_20MHz_s,

reset_is => POR_G_sync_s,

correlator_cs_is => correlator_cs_s ,

mil_rt1_ip_csn_is => mil_rt1_ip_csn_s,

mil_data_ram1_cs_is => mil_data_ram1_cs_s,

mil_command_ram1_cs_is => mil_command_ram1_cs_s,

mil_descriptor_ram1_cs_is => mil_descriptor_ram1_cs_s,

mil_rt2_ip_csn_is => mil_rt2_ip_csn_s,

mil_data_ram2_cs_is => mil_data_ram2_cs_s,

mil_command_ram2_cs_is => mil_command_ram2_cs_s,

mil_descriptor_ram2_cs_is => mil_descriptor_ram2_cs_s,

mil_rt3_ip_csn_is => mil_rt3_ip_csn_s,

mil_data_ram3_cs_is => mil_data_ram3_cs_s,

mil_command_ram3_cs_is => mil_command_ram3_cs_s,

mil_descriptor_ram3_cs_is => mil_descriptor_ram3_cs_s,

oen_is => oen_is,

writen_is => writen_is,

iosn_is => iosn_is,

fpga_we_os => fpga_we_s,

mil_we_os => mil_we_s,

mil_oe_os => mil_oe_s,

brdy_os => brdy_os,

prom_cs_is => prom_cs_is,

prom_re_os => prom_re_s

);

--      leon3_prom_module_inst :  leon_prom_module_e

--		port map(clk_40MHz_is => clk_40MHz_s,

--					re_is => prom_re_s,

--					address_is => up_address_is(18 downto 2),

--					data_out_os => data_out_prom_s);

address_decoder_inst : address_decoder_e

port map (

clk_40M_is => clk_40MHz_s,

reset_is => POR_G_sync_s,

up_address_is => up_address_is,

iosn_is => iosn_is ,

data_from_correlator_is => data_from_correlator_s,

rt_address_is=>  rt_address_is,

pr_read_data_from_rt1_ip_is =>  pr_read_data_from_rt1_ip_s,

pr_read_data_from_descr_ram1_is => pr_read_data_from_descr_ram1_s,

pr_read_data_from_comm_ram1_is  => pr_read_data_from_comm_ram1_s,

pr_read_data_from_data_ram1_is  => pr_read_data_from_data_ram1_s,

pr_read_data_from_rt2_ip_is => pr_read_data_from_rt2_ip_s,

pr_read_data_from_descr_ram2_is => pr_read_data_from_descr_ram2_s,

pr_read_data_from_comm_ram2_is  => pr_read_data_from_comm_ram2_s,

pr_read_data_from_data_ram2_is  => pr_read_data_from_data_ram2_s,

pr_read_data_from_rt3_ip_is => pr_read_data_from_rt3_ip_s,

pr_read_data_from_descr_ram3_is => pr_read_data_from_descr_ram3_s,

pr_read_data_from_comm_ram3_is  => pr_read_data_from_comm_ram3_s,

pr_read_data_from_data_ram3_is  => pr_read_data_from_data_ram3_s,

correlator_cs_os => correlator_cs_s,

mil_rt1_ip_csn_os => mil_rt1_ip_csn_s,

mil_rt2_ip_csn_os => mil_rt2_ip_csn_s,

mil_rt3_ip_csn_os => mil_rt3_ip_csn_s,

mil_data_ram1_cs_os => mil_data_ram1_cs_s,

mil_data_ram2_cs_os => mil_data_ram2_cs_s,

mil_data_ram3_cs_os => mil_data_ram3_cs_s,

mil_command_ram1_cs_os => mil_command_ram1_cs_s,

mil_command_ram2_cs_os => mil_command_ram2_cs_s,

mil_command_ram3_cs_os => mil_command_ram3_cs_s,

mil_descriptor_ram1_cs_os => mil_descriptor_ram1_cs_s,

mil_descriptor_ram2_cs_os => mil_descriptor_ram2_cs_s,

mil_descriptor_ram3_cs_os => mil_descriptor_ram3_cs_s,

fpga_data_out_to_up_os=> fpga_data_out_to_up_s,

rt_address_RT1_os=> rt_address_RT1_s,

rt_address_RT2_os=> rt_address_RT2_s,

rt_address_RT3_os=> rt_address_RT3_s,

prom_cs_is => prom_cs_is,

prom_data_to_up_is => data_out_prom_s

);

reset_n_s <= mil_block_reset_s;

-- Mil RT1 instantiaton

mil_top_level_RT1_inst : mil_top_level_e

port map (clk_is => clk_20MHz_s,

reset_n_is => reset_n_s,

rd_pr_sync_is => mil_oe_s,

wr_pr_sync_is => mil_we_s,

mil_command_ram_cs_is => mil_command_ram1_cs_s,

mil_descriptor_ram_cs_is => mil_descriptor_ram1_cs_s,

mil_rt_ip_csn_is => mil_rt1_ip_csn_s,

mil_data_ram_cs_is => mil_data_ram1_cs_s,

pr_address_is => up_address_is(17 downto 2),

pr_write_data_is =>  fpga_data_in_from_up_s(15 downto 0),

rt_address_is => rt_address_RT1_s,

BUSAINEN_rt_os   => RXENA_TX_R_RT1_os,

BUSAINP_rt_is => RXOUTA_TX_RT1_is,

BUSAINN_rt_is => RXOUTA1_TX_RT1_is,

BUSBINEN_rt_os => RXENB_TX_R_RT1_os,

BUSBINP_rt_is => RXOUTB_TX_RT1_is,

BUSBINN_rt_is => RXOUTB1_TX_RT1_is,

BUSAOUTIN_rt_os => TXINHA_TX_RT1_os,

BUSAOUTP_rt_out_os => TXINA_TX_RT1_os,

BUSAOUTN_rt_out_os => TXINA1_TX_RT1_os,

BUSBOUTIN_rt_os => TXINHB_TX_RT1_os,

BUSBOUTP_rt_out_os => TXINB_TX_RT1_os,

BUSBOUTN_rt_out_os => TXINB1_TX_RT1_os,

pr_read_data_from_rt_ip_os => pr_read_data_from_rt1_ip_s,

pr_read_data_from_descr_ram_os => pr_read_data_from_descr_ram1_s,

pr_read_data_from_comm_ram_os => pr_read_data_from_comm_ram1_s,

pr_read_data_from_data_ram_os => pr_read_data_from_data_ram1_s,

tsm_interrupt_os => tsm_interrupt_RT1_s,

mil_1553_interrupt_os => mil_1553_interrupt_RT1_s,

reset_tsm_interrupt_is => reset_tsm_interrupt_RT1_s,

reset_mil_interrupt_is => reset_mil_interrupt_RT1_s

);

-- Mil RT2 instantiaton

mil_top_level_RT2_inst : mil_top_level_e

port map (clk_is => clk_20MHz_s,

reset_n_is => reset_n_s,

rd_pr_sync_is => mil_oe_s,

wr_pr_sync_is => mil_we_s,

mil_command_ram_cs_is => mil_command_ram2_cs_s,

mil_descriptor_ram_cs_is => mil_descriptor_ram2_cs_s,

mil_rt_ip_csn_is => mil_rt2_ip_csn_s,

mil_data_ram_cs_is => mil_data_ram2_cs_s,

pr_address_is => up_address_is(17 downto 2),

pr_write_data_is =>  fpga_data_in_from_up_s(15 downto 0),

rt_address_is => rt_address_RT2_s,

BUSAINEN_rt_os   => RXENA_TX_R_RT2_os,

BUSAINP_rt_is => RXOUTA_TX_RT2_is,

BUSAINN_rt_is => RXOUTA1_TX_RT2_is,

BUSBINEN_rt_os => RXENB_TX_R_RT2_os,

BUSBINP_rt_is => RXOUTB_TX_RT2_is,

BUSBINN_rt_is => RXOUTB1_TX_RT2_is,

BUSAOUTIN_rt_os => TXINHA_TX_RT2_os,

BUSAOUTP_rt_out_os => TXINA_TX_RT2_os,

BUSAOUTN_rt_out_os => TXINA1_TX_RT2_os,

BUSBOUTIN_rt_os => TXINHB_TX_RT2_os,

BUSBOUTP_rt_out_os => TXINB_TX_RT2_os,

BUSBOUTN_rt_out_os => TXINB1_TX_RT2_os,

pr_read_data_from_rt_ip_os => pr_read_data_from_rt2_ip_s,

pr_read_data_from_descr_ram_os => pr_read_data_from_descr_ram2_s,

pr_read_data_from_comm_ram_os => pr_read_data_from_comm_ram2_s,

pr_read_data_from_data_ram_os => pr_read_data_from_data_ram2_s,

tsm_interrupt_os => tsm_interrupt_RT2_s,

mil_1553_interrupt_os => mil_1553_interrupt_RT2_s,

reset_tsm_interrupt_is => reset_tsm_interrupt_RT2_s,

reset_mil_interrupt_is => reset_mil_interrupt_RT2_s

);

-- Mil RT3 instantiaton

mil_top_level_RT3_inst : mil_top_level_e

port map (clk_is => clk_20MHz_s,

reset_n_is => reset_n_s,

rd_pr_sync_is => mil_oe_s,

wr_pr_sync_is => mil_we_s,

mil_command_ram_cs_is => mil_command_ram3_cs_s,

mil_descriptor_ram_cs_is => mil_descriptor_ram3_cs_s,

mil_rt_ip_csn_is => mil_rt3_ip_csn_s,

mil_data_ram_cs_is => mil_data_ram3_cs_s,

pr_address_is => up_address_is(17 downto 2),

pr_write_data_is =>  fpga_data_in_from_up_s(15 downto 0),

rt_address_is => rt_address_RT3_s,

BUSAINEN_rt_os => RXENA_TX_R_RT3_os,

BUSAINP_rt_is  => RXOUTA_TX_RT3_is,

BUSAINN_rt_is  => RXOUTA1_TX_RT3_is,

BUSBINEN_rt_os => RXENB_TX_R_RT3_os,

BUSBINP_rt_is => RXOUTB_TX_RT3_is,

BUSBINN_rt_is => RXOUTB1_TX_RT3_is,

BUSAOUTIN_rt_os => TXINHA_TX_RT3_os,

BUSAOUTP_rt_out_os => TXINA_TX_RT3_os,

BUSAOUTN_rt_out_os => TXINA1_TX_RT3_os,

BUSBOUTIN_rt_os => TXINHB_TX_RT3_os,

BUSBOUTP_rt_out_os => TXINB_TX_RT3_os,

BUSBOUTN_rt_out_os => TXINB1_TX_RT3_os,

pr_read_data_from_rt_ip_os => pr_read_data_from_rt3_ip_s,

pr_read_data_from_descr_ram_os => pr_read_data_from_descr_ram3_s,

pr_read_data_from_comm_ram_os => pr_read_data_from_comm_ram3_s,

pr_read_data_from_data_ram_os => pr_read_data_from_data_ram3_s,

tsm_interrupt_os => tsm_interrupt_RT3_s,

mil_1553_interrupt_os => mil_1553_interrupt_RT3_s,

reset_tsm_interrupt_is => reset_tsm_interrupt_RT3_s,

reset_mil_interrupt_is => reset_mil_interrupt_RT3_s

);

ms_500_interrupt_os <= ms_500_interrupt_temp_s;

gnss_correlator_top_inst : gnss_correlator_top_e

port map(clk_is => clk_40MHz_s,

POR_G_is => POR_G_sync_s,

POR_in_is => POR_sync_s,

ant1_data_is => syn2_adc1_s,

ant2_data_is => ant2_data_is,

ant3_data_is => ant3_data_is,

ant4_data_is => ant4_data_is,

soft_reset_os => soft_reset_s,

rt_address_RT1_is => rt_address_RT1_s,

rt_address_RT2_is => rt_address_RT2_s,

rt_address_RT3_is => rt_address_RT3_s,

hwdt_cmd_is => hwdt_level_out_s,

swdt_cmd_is => swdt_level_out_s,

data_processor_is => fpga_data_in_from_up_s,

wr_pr_is => fpga_we_s,

cs_correlator_is => correlator_cs_s,

up_address_is => up_address_is(13 downto 2),

ms_10_interrupt_os => ms_10_interrupt_os,

ms_500_interrupt_os => ms_500_interrupt_temp_s,

reset_processor_os => reset_pr_os,

data_to_processor_os => data_from_correlator_s,

reset_tsm_interrupt_RT1_os => reset_tsm_interrupt_RT1_s,

TSM_interrupt_RT1_is => tsm_interrupt_RT1_s,

TSM_flag_out_RT1_os => TSM_flag_out_RT1_os,

PPS_flag_out_RT1_os => PPS_flag_out_RT1_os,

reset_tsm_interrupt_RT2_os => reset_tsm_interrupt_RT2_s,

TSM_interrupt_RT2_is => tsm_interrupt_RT2_s,

TSM_flag_out_RT2_os => TSM_flag_out_RT2_os,

PPS_flag_out_RT2_os => PPS_flag_out_RT2_os,

reset_tsm_interrupt_RT3_os => reset_tsm_interrupt_RT3_s,

TSM_interrupt_RT3_is => tsm_interrupt_RT3_s,

TSM_flag_out_RT3_os => TSM_flag_out_RT3_os,

PPS_flag_out_RT3_os => PPS_flag_out_RT3_os,

reset_mil_interrupt_RT1_os => reset_mil_interrupt_RT1_s,

reset_mil_interrupt_RT2_os => reset_mil_interrupt_RT2_s,

reset_mil_interrupt_RT3_os => reset_mil_interrupt_RT3_s,

mil_1553_interrupt_RT1_is => mil_1553_interrupt_RT1_s,

mil_1553_interrupt_RT2_is => mil_1553_interrupt_RT2_s,

mil_1553_interrupt_RT3_is => mil_1553_interrupt_RT3_s,

upconv1_spi_reg_os => upconv1_spi_reg_os,

upconv2_spi_reg_os => upconv2_spi_reg_os,

upconv1_spi_gpio4_is => upconv1_spi_gpio4_sync_s,

upconv2_spi_gpio4_is => upconv2_spi_gpio4_sync_s,

lock_detector_ant1_is => lock_detector_ant1_out_s,

lock_detector_ant2_is => lock_detector_ant2_out_s,

lock_detector_ant3_is => lock_detector_ant3_out_s,

lock_detector_ant4_is => lock_detector_ant4_out_s,

pps_signal_os => pps_signal_os,

config_crc_err_is =>crc_error_s,

config_ecc_err_is => ecc_error_s,

config_synd_valid_is=>syndrome_valid_s,

icap_busy_is => icap_busy_s,

syndrome_is  => syndrome_s,

synd_valid_count_is=>synd_valid_count_s,

rt_adddr_sel_is => rt_address_is,

uart_txd_is => uart_txd_is,

rwe3_is => rwe3_is,

wdt_l_is => 	wdt_l_is ,

bexc_l_is => bexc_l_is,

spi_clk_SY_is => spi_clk_SY_is,

spi_en_SY_os => spi_en_SY_os,

spi_mosi_SY_os => spi_mosi_SY_os,

spi_miso_SY_is => spi_miso_SY_is

);

end top_module_a;

72) total_ch_fsm_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use WORK.signal_type_p.ALL;

entity total_ch_fsm_e is

port(

--Input signals

clk_is,POR_is: in std_logic;

channel_lock_status_is: in std_logic_vector(channels_count_c downto 0);

acq_interrupt_is  : in std_logic_vector(acq_modules_count_c downto 0);

ant1_ant2_sel_is : in two_acq1_4_typ;

svid_acq_is : in six_acq1_4_typ;

car_freq_init_is,code_freq_init_is : in thirtytwo_acq1_4_typ;

order1_loop_init_is, dll_order1_loop_init_is : in fortynine_acq1_4_typ;

svid_channel_is: in six_ch1_18_typ;

sbas_enable_is: in std_logic;

navic_cmd_enable_is : in std_logic_vector(13 downto 0);

-- Output signals

svid_channel_load_os: out std_logic_vector(5 downto 0);

car_freq_init_os,code_freq_init_os: out std_logic_vector(31 downto 0);

order1_loop_init_os, dll_order1_loop_init_os : out std_logic_vector(48 downto 0);

ant1_ant2_sel_ch_os: out std_logic_vector(1 downto 0);

channel_start_os : out std_logic_vector(channels_count_c downto 0);

acq_start_os : out std_logic_vector(acq_modules_count_c downto 0);

channel_status_os: out std_logic_vector(channels_count_c downto 0);

sat_current_status_os: out std_logic_vector(63 downto 0)

);

end total_ch_fsm_e;

architecture total_ch_fsm_a of total_ch_fsm_e is

type state_type is (s1,s2,s3,s4,s5,s6);

signal fsm_state_s: state_type;

type    fourteen_typ is array (13 downto 0) of std_logic_vector(5 downto 0);

signal  navic_cmd_svid_s: fourteen_typ;

signal  current_channel_s,current_channel_temp_s: std_logic_vector(channels_count_c downto 0);

signal  channel_load_s,channel_clear_s,channel_register_s,n_channel_register_s: std_logic_vector(channels_count_c downto 0);

signal  channel_empty_s, sat_pr_s : std_logic ;

signal  navic_cmd_channel_status_s: std_logic_vector(13 downto 0);

signal channel_ff_s: std_logic_vector(channels_count_c downto 0);

signal acq_module_s,acq_module_temp_s : std_logic_vector(2 downto 0);

signal current_svid_s: std_logic_vector(5 downto 0);

signal sbas_current_svid_s: std_logic;

signal navic_current_svid_s, navic_cmd_enable_sts_s: std_logic;

component svid_status_e is

port(clk_is: in std_logic;

POR_is: in std_logic;

load_fsm_state_is: in std_logic;

clear_fsm_state_is: in std_logic;

current_svid_is: in std_logic_vector(5 downto 0);

svid_channel_is: in six_ch1_18_typ;

channel_lock_status_is: in std_logic_vector(channels_count_c downto 0);

channel_register_is: in std_logic_vector(channels_count_c downto 0);

svid_status_out_os: out std_logic_vector(63 downto 0));

end component;

signal channel_start_temp_s : std_logic_vector(channels_count_c downto 0);

signal load_fsm_state_s, clear_fsm_state_s : std_logic;

signal channel_empty_temp_s: std_logic_vector(3 downto 0);

component channel_ff_e is

port( clk_is : in std_logic;

POR_is : in std_logic;

channel_load_is: in std_logic;

channel_clear_is: in std_logic;

channel_ff_os: out std_logic);

end component;

attribute FSM_ENCODING : string;

attribute FSM_ENCODING of fsm_state_s: signal is "ONE-HOT";

attribute SAFE_IMPLEMENTATION :string;

attribute SAFE_IMPLEMENTATION of fsm_state_s: signal is"yes";

begin

svid_status_inst: svid_status_e port map

(clk_is => clk_is,

POR_is => POR_is,

load_fsm_state_is => load_fsm_state_s,

clear_fsm_state_is => clear_fsm_state_s,

current_svid_is  => current_svid_s,

svid_channel_is => svid_channel_is,

channel_lock_status_is => channel_lock_status_is,

channel_register_is => channel_register_s,

svid_status_out_os => sat_current_status_os

);

channel_ff_gen: for i in 0 to channels_count_c  generate

channel_ff_e_inst : channel_ff_e

port map(   clk_is => clk_is,

POR_is => POR_is,

channel_load_is => channel_load_s(i),

channel_clear_is => channel_clear_s(i),

channel_ff_os => channel_ff_s(i));

end generate channel_ff_gen;

--------------------------------------------------

-- FSM initialization and state transition -----

--------------------------------------------------

P_channel_allottment_FSM: process(clk_is, POR_is)

begin

if POR_is = '1' then

fsm_state_s <= s1; --Initial state/ Clear state

elsif clk_is'event and clk_is = '1' then

case fsm_state_s is

when s1=> if sat_pr_s = '1' then--At least 1 acquistion module generated

fsm_state_s <= s2;                                         -- the interrupt:state1 to state 2.

end if;

when s2=> if channel_empty_s = '1' then -- At least one channel is vacant: state2 to state3.

fsm_state_s <= s3;                       -- Start state

elsif channel_empty_s = '0' then

fsm_state_s <= s5;

end if;

when s3=> fsm_state_s <= s4;

when s4=> fsm_state_s <= s5;

when s5=> fsm_state_s <= s6;

when s6=> fsm_state_s <= s1;   -- Initial state/Clear state

when others=> fsm_state_s <= s1;   -- Initial state/Clear state

end case;

end if;

end process P_channel_allottment_FSM;

-------------------------------------------------------------

-- Ouputs( Synchornous) based on the current fsm state ------

-------------------------------------------------------------

P_acq_start_gen:	process(clk_is, POR_is)

begin

if POR_is = '1' then

acq_module_s <= (others => '0'); --initilaization

current_channel_s <= (others => '0');--initialization, All channels are vacant

channel_start_os <= (others => '0');

elsif clk_is'event and clk_is = '1' then

channel_start_os <= channel_start_temp_s; --Start signal for tracking modules

if fsm_state_s = s1 and sat_pr_s = '1'then

acq_module_s <= acq_module_temp_s;

end if;

if fsm_state_s = s2 and channel_empty_s = '1' then

if sbas_enable_is='1' and sbas_current_svid_s='1' then --SBAS enable and acquired svid belongs to SBAS.

current_channel_s <="100000000000000000"; --Assign Channel 18 to SBAS.

elsif navic_cmd_enable_sts_s='1' and navic_current_svid_s ='1' then  -- NAVIC Commanding enable and acquired svid belongs to NAVIC.

current_channel_s <="010000000000000000"; --Assign Channel 17 to NAVIC.

else

current_channel_s <= current_channel_temp_s; -- Assign to Channel 1 to 16.

end if;

end if;

end if;

end process;

---- when fsm in state5: it gives the start acquisiton signal,

---- which is used as control signals by 4 acquistion modules.

acq_start_gen: for i in 0 to acq_modules_count_c generate

acq_start_os(i) <= '1' when acq_module_s = CONV_STD_LOGIC_VECTOR(i+1,3) and fsm_state_s = s5 else '0';

end generate acq_start_gen;

channel_start_temp_s <= current_channel_s when fsm_state_s = s4 or fsm_state_s = s3 else

(others => '0');

channel_load_s <= current_channel_s when fsm_state_s = s4 else

(others => '0');

load_fsm_state_s <= '1' when fsm_state_s = s4 else '0'; -- Load the acquired svid to tracking channel in state4.

clear_fsm_state_s <= '1' when fsm_state_s = s1 else '0'; -- Clear state

current_svid_s <=    svid_acq_is(0) when acq_module_s = "001" else

svid_acq_is(1) when acq_module_s = "010" else

svid_acq_is(2) when acq_module_s = "011" else

svid_acq_is(3) when acq_module_s = "100" else

(others => '0');

car_freq_init_os <=     car_freq_init_is(0) when acq_module_s = "001" else

car_freq_init_is(1) when acq_module_s = "010" else

car_freq_init_is(2) when acq_module_s = "011" else

car_freq_init_is(3) when acq_module_s = "100" else

(others => '0');

code_freq_init_os <=    code_freq_init_is(0) when acq_module_s = "001" else

code_freq_init_is(1) when acq_module_s = "010" else

code_freq_init_is(2) when acq_module_s = "011" else

code_freq_init_is(3) when acq_module_s = "100" else

(others => '0');

order1_loop_init_os <= 	order1_loop_init_is(0) when acq_module_s = "001" else

order1_loop_init_is(1) when acq_module_s = "010" else

order1_loop_init_is(2) when acq_module_s = "011" else

order1_loop_init_is(3) when acq_module_s = "100" else

(others => '0');

dll_order1_loop_init_os <= 	dll_order1_loop_init_is(0) when acq_module_s = "001" else

dll_order1_loop_init_is(1) when acq_module_s = "010" else

dll_order1_loop_init_is(2) when acq_module_s = "011" else

dll_order1_loop_init_is(3) when acq_module_s = "100" else

(others => '0');

ant1_ant2_sel_ch_os <=   ant1_ant2_sel_is(0) when acq_module_s = "001" else

ant1_ant2_sel_is(1) when acq_module_s = "010" else

ant1_ant2_sel_is(2) when acq_module_s = "011" else

ant1_ant2_sel_is(3) when acq_module_s = "100" else

(others => '0');

sat_pr_s <= acq_interrupt_is(0) or acq_interrupt_is(1) or acq_interrupt_is (2) or acq_interrupt_is(3);

channel_register_s(channels_count_c downto 0) <= channel_ff_s;

channel_status_os <= channel_register_s;-- Current track status of each channel

n_channel_register_s <= not channel_register_s; --negate channel register contents

navic_cmd_svid_s (0)     <= "001000" when navic_cmd_enable_is(0) ='1' else  -- 1A, NAVIC SVID 1

"000000";

navic_cmd_svid_s (1)     <= "001001" when navic_cmd_enable_is(1) ='1' else  -- 1B, NAVIC SVID 2

"000000";

navic_cmd_svid_s (2)     <= "001010" when navic_cmd_enable_is(2) ='1' else  -- 1C, NAVIC SVID 3

"000000";

navic_cmd_svid_s (3)	 <= "001011" when navic_cmd_enable_is(3) ='1' else  -- 1D, NAVIC SVID 4

"000000";

navic_cmd_svid_s (4)	 <= "011000" when navic_cmd_enable_is(4) ='1' else  -- 1E, NAVIC SVID 5

"000000";

navic_cmd_svid_s (5)	 <= "011001" when navic_cmd_enable_is(5) ='1' else  -- 1F, NAVIC SVID 6

"000000";

navic_cmd_svid_s (6)	 <= "011010" when navic_cmd_enable_is(6) ='1' else  -- 1G, NAVIC SVID 7

"000000";

navic_cmd_svid_s (7)	 <= "011011" when navic_cmd_enable_is(7) ='1' else  -- 1I, NAVIC SVID 8

"000000";

navic_cmd_svid_s (8)	 <= "101000" when navic_cmd_enable_is(8) ='1' else  -- 1J, NAVIC SVID 9

"000000";

navic_cmd_svid_s (9)	 <= "101001" when navic_cmd_enable_is(9) ='1' else  -- 1K, NAVIC SVID 10

"000000";

navic_cmd_svid_s (10)	 <= "101010" when navic_cmd_enable_is(10) ='1' else -- 1L, NAVIC SVID 11

"000000";

navic_cmd_svid_s (11)	 <= "101011" when navic_cmd_enable_is(11) ='1' else -- 1M, NAVIC SVID 12

"000000";

navic_cmd_svid_s (12)    <= "111000" when navic_cmd_enable_is(12) ='1' else -- 1N, NAVIC SVID 13

"000000";

navic_cmd_svid_s (13)	 <= "111001" when navic_cmd_enable_is(13) ='1' else -- 1O, NAVIC SVID 14

"000000";

navic_cmd_channel_status_gen: for i in 0 to 13 generate

navic_cmd_channel_status_s(i) <= '1' when navic_cmd_enable_is(i)='1'  and  current_svid_s = navic_cmd_svid_s (i) else

'0';

end generate;

navic_current_svid_s <= navic_cmd_channel_status_s(0) or navic_cmd_channel_status_s(1) or navic_cmd_channel_status_s(2) or navic_cmd_channel_status_s(3) or navic_cmd_channel_status_s(4) or navic_cmd_channel_status_s(5) or navic_cmd_channel_status_s(6)or

navic_cmd_channel_status_s(7) or navic_cmd_channel_status_s(8) or navic_cmd_channel_status_s(9) or navic_cmd_channel_status_s(10) or navic_cmd_channel_status_s(11) or navic_cmd_channel_status_s(12) or navic_cmd_channel_status_s(13) ;

navic_cmd_enable_sts_s<= navic_cmd_enable_is(0) or  navic_cmd_enable_is(1) or  navic_cmd_enable_is(2) or  navic_cmd_enable_is(3) or navic_cmd_enable_is(4) or  navic_cmd_enable_is(5) or  navic_cmd_enable_is(6) or  navic_cmd_enable_is(7) or

navic_cmd_enable_is(8) or  navic_cmd_enable_is(9) or  navic_cmd_enable_is(10) or  navic_cmd_enable_is(11) or navic_cmd_enable_is(12) or  navic_cmd_enable_is(13) ;

sbas_current_svid_s<='1' when  current_svid_s= "001100" or current_svid_s="001101" or current_svid_s="001110" or current_svid_s="001111" or

current_svid_s= "011100" or current_svid_s="011101" or current_svid_s="011110" or current_svid_s="011111" or

current_svid_s= "101100" or current_svid_s="101101" or current_svid_s="101110" or current_svid_s="101111" or

current_svid_s= "111010" or current_svid_s="111011" or current_svid_s= "111100" or current_svid_s="111101" or current_svid_s="111110" or current_svid_s="111111"

else '0';

channel_empty_temp_s(0) <= n_channel_register_s(0) or n_channel_register_s(1) or n_channel_register_s(2) or n_channel_register_s(3) or n_channel_register_s(4) or n_channel_register_s(5) or n_channel_register_s(6) or n_channel_register_s(7) or n_channel_register_s(8) or n_channel_register_s(9) or n_channel_register_s(10) or

n_channel_register_s(11) or n_channel_register_s(12) or n_channel_register_s(13) or n_channel_register_s(14) or n_channel_register_s(15) or n_channel_register_s(16) or n_channel_register_s(17);

channel_empty_temp_s(1) <= n_channel_register_s(0) or n_channel_register_s(1) or n_channel_register_s(2) or n_channel_register_s(3) or n_channel_register_s(4) or n_channel_register_s(5) or n_channel_register_s(6) or n_channel_register_s(7) or n_channel_register_s(8) or n_channel_register_s(9) or n_channel_register_s(10) or --or ch_17_empty;

n_channel_register_s(11) or n_channel_register_s(12) or n_channel_register_s(13) or n_channel_register_s(14) or n_channel_register_s(15) or n_channel_register_s(16);

channel_empty_temp_s(2) <= n_channel_register_s(0) or n_channel_register_s(1) or n_channel_register_s(2) or n_channel_register_s(3) or n_channel_register_s(4) or n_channel_register_s(5) or n_channel_register_s(6) or n_channel_register_s(7) or n_channel_register_s(8) or n_channel_register_s(9) or n_channel_register_s(10) or --or ch_17_empty;

n_channel_register_s(11) or n_channel_register_s(12) or n_channel_register_s(13) or n_channel_register_s(14) or n_channel_register_s(15) or n_channel_register_s(17);

channel_empty_temp_s(3) <= n_channel_register_s(0) or n_channel_register_s(1) or n_channel_register_s(2) or n_channel_register_s(3) or n_channel_register_s(4) or n_channel_register_s(5) or n_channel_register_s(6) or n_channel_register_s(7) or n_channel_register_s(8) or n_channel_register_s(9) or n_channel_register_s(10) or --or ch_17_empty;

n_channel_register_s(11) or n_channel_register_s(12) or n_channel_register_s(13) or n_channel_register_s(14) or n_channel_register_s(15);

channel_empty_s <=    channel_empty_temp_s(0)    when  sbas_enable_is = '0'       and  navic_cmd_enable_sts_s='0' else -- SBAS disable & NAVIC Telecommand  disable

n_channel_register_s(16)   when  navic_cmd_enable_sts_s='1' and  navic_current_svid_s ='1' else -- Channel 17 for NAVIC Telecommand

n_channel_register_s(17)   when  sbas_enable_is = '1'       and  sbas_current_svid_s='1'  else

channel_empty_temp_s(1)    when  sbas_enable_is = '1'       and  navic_cmd_enable_sts_s = '0' else -- SBAS enable  & 1A disable

channel_empty_temp_s(2)    when  sbas_enable_is = '0'       and  navic_cmd_enable_sts_s = '1' else -- SBAS disable & 1A enable

channel_empty_temp_s(3);

acq_module_temp_s <=   "001" when acq_interrupt_is(0) = '1' else

"010" when acq_interrupt_is(1) = '1' else

"011" when acq_interrupt_is(2) = '1' else

"100" when acq_interrupt_is(3) = '1' else

"000" ;

current_channel_temp_s <= "000000000000000001" when n_channel_register_s(0) = '1'  else -- Channel 1 vacant,new sat can be alloted

"000000000000000010" when n_channel_register_s(1) = '1'  else -- Channel 2 vacant,new sat can be alloted

"000000000000000100" when n_channel_register_s(2) = '1'  else -- Channel 3 vacant,new sat can be alloted

"000000000000001000" when n_channel_register_s(3) = '1'  else -- Channel 4 vacant,new sat can be alloted

"000000000000010000" when n_channel_register_s(4) = '1'  else -- Channel 5 vacant,new sat can be alloted

"000000000000100000" when n_channel_register_s(5) = '1'  else -- Channel 6 vacant,new sat can be alloted

"000000000001000000" when n_channel_register_s(6) = '1'  else -- Channel 7 vacant,new sat can be alloted

"000000000010000000" when n_channel_register_s(7) = '1'  else -- Channel 8 vacant,new sat can be alloted

"000000000100000000" when n_channel_register_s(8) = '1'  else -- Channel 9 vacant,new sat can be alloted

"000000001000000000" when n_channel_register_s(9) = '1'  else -- Channel 10 vacant,new sat can be alloted

"000000010000000000" when n_channel_register_s(10)= '1'  else -- Channel 11 vacant,new sat can be alloted

"000000100000000000" when n_channel_register_s(11)= '1'  else -- Channel 12 vacant,new sat can be alloted

"000001000000000000" when n_channel_register_s(12) = '1' else-- Channel 13 vacant,new sat can be alloted

"000010000000000000" when n_channel_register_s(13) = '1' else-- Channel 14 vacant,new sat can be alloted

"000100000000000000" when n_channel_register_s(14) = '1' else-- Channel 15 vacant,new sat can be alloted

"001000000000000000" when n_channel_register_s(15) = '1' else-- Channel 16 vacant,new sat can be alloted

"010000000000000000" when n_channel_register_s(16) = '1' else-- Channel 17 vacant,new sat can be alloted

"100000000000000000" when n_channel_register_s(17) = '1' else-- Channel 18 vacant,new sat can be alloted

"000000000000000000" ;

svid_channel_load_os <= current_svid_s;

--Logic to clear Channel registers in unlock condition.

channel_clear_gen: for i in 0 to channels_count_c generate

channel_clear_s(i) <= '1' when channel_lock_status_is(i) = '0' and channel_register_s(i) = '1' and fsm_state_s = s1 else '0';

end generate channel_clear_gen;

end total_ch_fsm_a;

73) trk_code_gen_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

-- variables and its meaning

-- clk_is = 40 MHz input system clock from GP2015

--POR_is = POR async reset.

-- start_is =  loading from Acquisition module to start exactly at same time, when incoming signal is expected.

-- sat_id_in_is = is the input SVID for which the code to be generated.

-- channel_par_load_is is a pulse input when to latch the loop filter output for code frequency

-- code_corr_freq_in_is : loop filter output of code freq.

--code_freq_init_is : comes from acquisition module corrected with doppler.

-- sat_id_out_os : output of what SVID is tracking.

--one_ms_pulse_os : output of when code boundary completes. (1ms)

--one_ms_counter_value_os, twenty_ms_counter_value_os, = counter values during measurement latch

--code_cyle_count_value_os : slew count (within 1023) value during measurement latch

--code_phase_count_os : code phase value during measurement latch.

--reg_wr_processor_tms_is : to reset 20ms counter (when processor finds sync)

-- epoch_latch_pulse_is : measurement latching pulse.

--Data_boundary_pulse_is : when this pulse from bit synchronizer module, the 1ms counter to be reset.

--Codeout_os : E P L code outputs.

-- Incoming SVID assumption

--											GPS = 1 to 8  NavIC 1 to 4 as 9 to 12 in same module SBAS 1 to 4 as 13 to 16

--											GPS = 9 to 16 as 17 to 24 NavIC 5 to 8 as 25 to 28 in same module SBAS 5 to 8 as 29 to 32

--											GPS = 17 to 24 as 33 to 40 NavIC 9 to 12 as 41 to 44 in same module SBAS 9 to 12 as 45 to 48

--											GPS = 25 to 32 as 49 to 56 NavIC 13 to 14 as 57 to 58 in same module SBAS 13 to 18 as 59 to 64

--											Whichever non-working SVID's of the NavIC shall be disabled during almanac writing and at the same time

--											Even default reset init also it shall be disabled

--											SBAS 120,123 and 130 are removed as they are not broadcasting.

entity trk_code_gen_e is

Port ( clk_is : in  STD_LOGIC;

POR_is : in  STD_LOGIC;

start_is : in std_logic;

channel_par_load_is : in std_logic; -- this signal for latching loppfilter outputs

sat_id_in_is: in std_logic_vector(5 downto 0); -- comes from acquisition module

code_corr_freq_in_is: in std_logic_vector(31 downto 0);  ----comes from descriminator module

code_freq_init_is : in std_logic_vector(31 downto 0); -- comes from acquisition module for initial loading

sat_id_out_os : out std_logic_vector(5 downto 0);  -- satellite id of this channel

one_ms_pulse_os : out std_logic;  -- one ms pulse for monitering

one_ms_counter_value_os: out std_logic_vector(4 downto 0); -- at epoch latched

twenty_ms_counter_value_os: out std_logic_vector(9 downto 0); -- at epoch latched

code_cyle_count_value_os: out std_logic_vector(9 downto 0); -- at epoch latched

code_phase_count_os : out std_logic_vector(31 downto 0); -- at epoch latched

reg_wr_processor_tms_is : in std_logic; -- to clear twenty ms counter by processor

epoch_latch_pulse_is: in std_logic;	-- epoch pulse to latch the measurements

Data_boundary_pulse_is : in std_logic; -- this is required for clearing the 1ms signals.

Narrow_corr_step_12_enable_is : in std_logic_vector(1 downto 0);

Data_boundary_output_every20ms_os : out std_logic;

Codeout_os : out  std_logic_vector(2 downto 0)  -- epl out for the main tracking channel

);

end trk_code_gen_e;

architecture trk_code_gen_a of trk_code_gen_e is

signal g1_reg_s : std_logic_vector(9 downto 0);

signal g2_reg_s : std_logic_vector(9 downto 0);

-- For gold code generation G1 G2 registers

signal Codeclk_1p023M_s,Codeclk_2p046M_s: std_logic;

-- For generating 1.023MHz signal

signal Codeclk_1p023M_latched_s: std_logic;

-- this code clock shall be used as Clock enable to shift the G1 G2 registers running at 40MHz clock

signal Code_phase_s,code_Nco_add_s : std_logic_vector(31 downto 0);

-- For NCO this register is required to maintain the phase.  31st bit will give 1.023MHz

signal Codeclk_2p046M_latched_s, Codeout_temp_s : std_logic;

signal Codeout_epl_s : std_logic_vector(8 downto 0);

signal Slew_count_s : std_logic_vector(9 downto 0);

signal one_ms_cnt_s : std_logic_vector(4 downto 0);

signal twenty_ms_cnt_s : std_logic_vector(9 downto 0);

signal Bit_sync_found_s : std_logic;

begin

--Codeout_os <= Codeout_epl_s;

P_code_gen: process(clk_is,POR_is)

begin

if POR_is ='1' then

sat_id_out_os <= "000000";

one_ms_pulse_os <= '0';

one_ms_counter_value_os <= "00000";

twenty_ms_counter_value_os <= "0000000000";

code_cyle_count_value_os <= "0000000000";

code_phase_count_os <= "00000000000000000000000000000000";

g1_reg_s <= "0000000000";

g2_reg_s <= "0000000000";

Codeclk_1p023M_s <= '0';

Codeclk_2p046M_s <= '0';

Codeclk_1p023M_latched_s <= '0';

Codeclk_2p046M_latched_s <= '0';

Code_phase_s <= "00000000000000000000000000000000";

code_Nco_add_s <= "00000000000000000000000000000000";

Codeout_temp_s <= '0';

Codeout_epl_s <= (others => '0');

Slew_count_s <= "0000000000";

one_ms_cnt_s <= "00000";

twenty_ms_cnt_s <= "0000000000";

Data_boundary_output_every20ms_os <= '1';

Bit_sync_found_s <= '0';

elsif clk_is='1' and clk_is'event then

-- start signal comes from channel assignment FSM module.

if(start_is = '1') then

one_ms_pulse_os <= '0';

Codeclk_1p023M_s <= '0';

Codeclk_2p046M_s <= '0';

Codeclk_1p023M_latched_s <= '0';

Codeclk_2p046M_latched_s <= '0';

Code_phase_s <= "00000000000000000000000000000000";

-- to output what svid is tracked.

sat_id_out_os <= sat_id_in_is;

g1_reg_s <= (others=>'1');

-- upon start pulse from acquisition module, here the direct value of code nco is loaded which comes from acquisition module

Slew_count_s <= (others=>'1');

-- to see that when incrementing starts with 0 slew count.

-- otherwise 1 slew count difference will be there.

code_Nco_add_s <= code_freq_init_is;

-- from acquisition module this input corrected with code doppler comes.

Data_boundary_output_every20ms_os <= '1';

Bit_sync_found_s <= '0';

-- G2 registers are initialized based on SVID.

C_SVID_decoder: case sat_id_in_is is

-- first 8 are GPS 1- 8  next 4 NavIC 1-4  next SBAS 1-4

when "000000" => g2_reg_s <= "1111101100";

when "000001" => g2_reg_s <= "1111011000";

when "000010" => g2_reg_s <= "1110110000";

when "000011" => g2_reg_s <= "1101100000";

when "000100" => g2_reg_s <= "0010010110";

when "000101" => g2_reg_s <= "0100101100";

when "000110" => g2_reg_s <= "0110010110";

when "000111" => g2_reg_s <= "1100101100";

when "001000" => g2_reg_s <= "1110010111";

when "001001" => g2_reg_s <= "0110010000";

when "001010" => g2_reg_s <= "0010110001";

when "001011" => g2_reg_s <= "0100111010";

when "001100" => g2_reg_s <= "0001010010";--0450  PRN-138

when "001101" => g2_reg_s <= "1000010101";--1241  PRN-121

when "001110" => g2_reg_s <= "1110110100";--0267  PRN-122

when "001111" => g2_reg_s <= "0101100100";--0232  PRN-123

-- first 8 are GPS 9- 16  next 4 NavIC 5-8  next SBAS 5-8

when "010000" => g2_reg_s <= "1001011000";

when "010001" => g2_reg_s <= "1101110100";

when "010010" => g2_reg_s <= "1011101000";

when "010011" => g2_reg_s <= "1110100000";

when "010100" => g2_reg_s <= "1101000000";

when "010101" => g2_reg_s <= "1010000000";

when "010110" => g2_reg_s <= "0100000000";

when "010111" => g2_reg_s <= "1000000000";

when "011000" => g2_reg_s <= "0000110111";

when "011001" => g2_reg_s <= "1101011000";

when "011010" => g2_reg_s <= "0010100000";

when "011011" => g2_reg_s <= "0000110010";

when "011100" => g2_reg_s <= "1011011100"; -- 0355  PRN 147

when "011101" => g2_reg_s <= "0111110001"; -- 1076  PRN 125

when "011110" => g2_reg_s <= "0010111111"; -- 1764  PRN 126

when "011111" => g2_reg_s <= "1111001110"; -- 0717  PRN 127

-- first 8 are GPS 17- 24 next 4 NavIC 9-12  next SBAS 9 - 12

when "100000" => g2_reg_s <= "1000100110";

when "100001" => g2_reg_s <= "0001001100";

when "100010" => g2_reg_s <= "0010011000";

when "100011" => g2_reg_s <= "0100110000";

when "100100" => g2_reg_s <= "1001100000";

when "100101" => g2_reg_s <= "0011000000";

when "100110" => g2_reg_s <= "0011001110";

when "100111" => g2_reg_s <= "1001110000";

when "101000" => g2_reg_s <= "0001100100";

when "101001" => g2_reg_s <= "0010011011";

when "101010" => g2_reg_s <= "0011001000";

when "101011" => g2_reg_s <= "0011111011";

when "101100" => g2_reg_s <= "0101101011";  --1532  PRN 128

when "101101" => g2_reg_s <= "0001010101";  --1250  PRN 129

when "101110" => g2_reg_s <= "1000011100";  --0341  PRN 130

when "101111" => g2_reg_s <= "1001011010";  --0551  PRN 131

-- first 8 are GPS 25- 32 next 4 NavIC 13-14  next SBAS 13 - 18

when "110000" => g2_reg_s <= "0011100000";

when "110001" => g2_reg_s <= "0111000000";

when "110010" => g2_reg_s <= "1110000000";

when "110011" => g2_reg_s <= "1100000000";

when "110100" => g2_reg_s <= "0001010110";

when "110101" => g2_reg_s <= "0010101100";

when "110110" => g2_reg_s <= "0101011000";

when "110111" => g2_reg_s <= "1010110000";

when "111000" => g2_reg_s <= "0100101101";

when "111001" => g2_reg_s <= "0101011110";

when "111010" => g2_reg_s <= "0000101010";  --0520  PRN 132

when "111011" => g2_reg_s <= "1001101111";  --1731  PRN 133

when "111100" => g2_reg_s <= "0110001110";  --0706  PRN 134

when "111101" => g2_reg_s <= "0111000101";  --1216  PRN 135

when "111110" => g2_reg_s <= "0000011110";  --0740  PRN 136

when others =>   g2_reg_s <= "1110000001";  --1007  PRN 137

end case C_SVID_decoder;

else

if(Narrow_corr_step_12_enable_is = "00") then

Codeout_os <= Codeout_epl_s(8) & Codeout_epl_s(4) & Codeout_epl_s(0);

elsif(Narrow_corr_step_12_enable_is = "01") then

Codeout_os <= Codeout_epl_s(6) & Codeout_epl_s(4) & Codeout_epl_s(2);

elsif(Narrow_corr_step_12_enable_is = "11") then

Codeout_os <= Codeout_epl_s(5) & Codeout_epl_s(4) & Codeout_epl_s(3);

else

Codeout_os <= Codeout_epl_s(8) & Codeout_epl_s(4) & Codeout_epl_s(0);

end if;

-- when processor writes for clearing the measurements, only 20ms counter is cleared

-- this is required for psuedo range measurements

if(reg_wr_processor_tms_is = '1') then

twenty_ms_cnt_s <= (others=>'0');

end if;

-- when bit-sync from top module, this one ms counter has to be cleared.

if(Data_boundary_pulse_is = '1') then

one_ms_cnt_s <= "00001";

--						twenty_ms_cnt_s <= (others=>'0');

Bit_sync_found_s <= '1';

end if;

if(one_ms_cnt_s = "00001" ) and (Bit_sync_found_s = '1') then

Data_boundary_output_every20ms_os <= '1';

elsif(Bit_sync_found_s = '1') then

Data_boundary_output_every20ms_os <= '0';

else

Data_boundary_output_every20ms_os <= '1';

end if;

-- when 500ms latch pulse comes all the counters are latched at a short.

if(epoch_latch_pulse_is = '1') then

one_ms_counter_value_os <= one_ms_cnt_s;

twenty_ms_counter_value_os <= twenty_ms_cnt_s;

code_cyle_count_value_os <= Slew_count_s;

code_phase_count_os <= Code_phase_s;

end if;

-- when discriminator gives the pulse for latching into code nco the values are latched here

if(channel_par_load_is = '1') then

code_Nco_add_s <= code_corr_freq_in_is;

end if;

Code_phase_s <= Code_phase_s + code_Nco_add_s ;

-- above code NCO actions take place.

Codeclk_1p023M_s <= not Code_phase_s(31);

--1.023MHz is generated here

Codeclk_2p046M_s <= not Code_phase_s(28);

--2.046MHz is generated here

-- upon 1.023MHz clock shift the G1,G2 shift registers and calculate their next values based on XOR

if(Codeclk_1p023M_s= '1') and (Codeclk_1p023M_latched_s = '0')  then

--						Codeout_temp_s <= g1_reg_s(9) xor g2_reg_s(9);

--

--						g1_reg_s(1 to 9) <= ( g1_reg_s(0 to 8) );

--						g2_reg_s(1 to 9) <= ( g2_reg_s(0 to 8) );

--						g1_reg_s(0) <= g1_reg_s(2) xor g1_reg_s(9);

--						g2_reg_s(0) <= g2_reg_s(1) xor g2_reg_s(2) xor g2_reg_s(5) xor g2_reg_s(7) xor g2_reg_s(8) xor g2_reg_s(9) ;

Codeout_temp_s <= g1_reg_s(0) xor g2_reg_s(0);

g1_reg_s(8 downto 0) <= ( g1_reg_s(9 downto 1) );

g2_reg_s(8 downto 0) <= ( g2_reg_s(9 downto 1) );

g1_reg_s(9) <= g1_reg_s(7) xor g1_reg_s(0);

g2_reg_s(9) <= g2_reg_s(8) xor g2_reg_s(7) xor g2_reg_s(4) xor g2_reg_s(2) xor g2_reg_s(1) xor g2_reg_s(0) ;

-- this 0th value for G1 and G2 are generated based ICD.

Codeclk_1p023M_latched_s <= '1' ;

-- this setting of this signal ensures multiple times with same code clock period it does not come inside this loop

Slew_count_s <= Slew_count_s + 1;

if(Slew_count_s = "1111111110") then

Slew_count_s <= (others=>'0');

one_ms_cnt_s <= one_ms_cnt_s + 1;

one_ms_pulse_os <= '1';

if(one_ms_cnt_s = "10011")then --19

one_ms_cnt_s <= (others=>'0');

twenty_ms_cnt_s <= twenty_ms_cnt_s + 1;

if(twenty_ms_cnt_s =  "0000110001") then --0100101011") then  --299

twenty_ms_cnt_s <= (others=>'0');

end if;

end if;

end if;

else

one_ms_pulse_os <= '0';

end if;

-- when code clok is zero, latched also made zero so that next time high comes it goes to the shifting loop.

if(Codeclk_1p023M_s = '0') then

Codeclk_1p023M_latched_s <= '0';

end if;

-- in this below code Eearly Prompt, Late codes are generated

if(Codeclk_2p046M_s= '1') and (Codeclk_2p046M_latched_s = '0')  then

Codeout_epl_s(8 downto 0) <= Codeout_epl_s(7 downto 0) & Codeout_temp_s;

Codeclk_2p046M_latched_s <= '1';

end if;

if(Codeclk_2p046M_s = '0') then

Codeclk_2p046M_latched_s <= '0';

end if;

end if;  -- sync reset

end if;-- clock event

end process P_code_gen;

end trk_code_gen_a;

74) trk_code_gen_sbas_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

-- variables and its meaning

-- clk_is = 40 MHz input system clock from GP2015

--POR_is = POR async reset.

-- start_is =  loading from Acquisition module to start exactly at same time, when incoming signal is expected.

-- sat_id_in_is = is the input SVID for which the code to be generated.

-- channel_par_load_is is a pulse input when to latch the loop filter output for code frequency

-- code_corr_freq_in_is : loop filter output of code freq.

--code_freq_init_is : comes from acquisition module corrected with doppler.

-- sat_id_out_os : output of what SVID is tracking.

--one_ms_pulse_os : output of when code boundary completes. (1ms)

--one_ms_counter_value_os, twenty_ms_counter_value_os, = counter values during measurement latch

--code_cyle_count_value_os : slew count (within 1023) value during measurement latch

--code_phase_count_os : code phase value during measurement latch.

--reg_wr_processor_tms_is : to reset 20ms counter (when processor finds sync)

-- epoch_latch_pulse_is : measurement latching pulse.

--Data_boundary_pulse_is : when this pulse from bit synchronizer module, the 1ms counter to be reset.

--Codeout_os : E P L code outputs.

-- Incoming SVID assumption

--											GPS = 1 to 8  NavIC 1 to 4 as 9 to 12 in same module SBAS 1 to 4 as 13 to 16

--											GPS = 9 to 16 as 17 to 24 NavIC 5 to 8 as 25 to 28 in same module SBAS 5 to 8 as 29 to 32

--											GPS = 17 to 24 as 33 to 40 NavIC 9 to 12 as 41 to 44 in same module SBAS 9 to 12 as 45 to 48

--											GPS = 25 to 32 as 49 to 56 NavIC 13 to 14 as 57 to 58 in same module SBAS 13 to 18 as 59 to 64

--											Whichever non-working SVID's of the NavIC shall be disabled during almanac writing and at the same time

--											Even default reset init also it shall be disabled

--											SBAS 120,123 and 130 are removed as they are not broadcasting.

entity trk_code_gen_sbas_e is

Port ( clk_is : in  STD_LOGIC;

POR_is : in  STD_LOGIC;

start_is : in std_logic;

channel_par_load_is : in std_logic; -- this signal for latching loppfilter outputs

sat_id_in_is: in std_logic_vector(5 downto 0); -- comes from acquisition module

code_corr_freq_in_is: in std_logic_vector(31 downto 0);  ----comes from descriminator module

code_freq_init_is : in std_logic_vector(31 downto 0); -- comes from acquisition module for initial loading

sbas_enable_is : in std_logic;

sat_id_out_os : out std_logic_vector(5 downto 0);  -- satellite id of this channel

one_ms_pulse_os : out std_logic;  -- one ms pulse for monitering

one_ms_counter_value_os: out std_logic_vector(4 downto 0); -- at epoch latched

twenty_ms_counter_value_os: out std_logic_vector(9 downto 0); -- at epoch latched

code_cyle_count_value_os: out std_logic_vector(9 downto 0); -- at epoch latched

code_phase_count_os : out std_logic_vector(31 downto 0); -- at epoch latched

reg_wr_processor_tms_is : in std_logic; -- to clear twenty ms counter by processor

epoch_latch_pulse_is: in std_logic;	-- epoch pulse to latch the measurements

Data_boundary_pulse_is : in std_logic; -- this is required for clearing the 1ms signals.

Narrow_corr_step_12_enable_is : in std_logic_vector(1 downto 0);

Data_boundary_output_every20ms_os : out std_logic;

Codeout_os : out  std_logic_vector(2 downto 0)  -- epl out for the main tracking channel

);

end trk_code_gen_sbas_e;

architecture trk_code_gen_sbas_a of trk_code_gen_sbas_e is

signal g1_reg_s : std_logic_vector(9 downto 0);

signal g2_reg_s : std_logic_vector(9 downto 0);

-- For gold code generation G1 G2 registers

signal Codeclk_1p023M_s,Codeclk_2p046M_s: std_logic;

-- For generating 1.023MHz signal

signal Codeclk_1p023M_latched_s: std_logic;

-- this code clock shall be used as Clock enable to shift the G1 G2 registers running at 40MHz clock

signal Code_phase_s,code_Nco_add_s : std_logic_vector(31 downto 0);

-- For NCO this register is required to maintain the phase.  31st bit will give 1.023MHz

signal Codeclk_2p046M_latched_s, Codeout_temp_s : std_logic;

signal Codeout_epl_s : std_logic_vector(8 downto 0);

signal Slew_count_s : std_logic_vector(9 downto 0);

signal one_ms_cnt_s, one_ms_cnt_temp_s : std_logic_vector(4 downto 0);

signal twenty_ms_cnt_s : std_logic_vector(9 downto 0);

signal Bit_sync_found_s : std_logic;

begin

--Codeout_os <= Codeout_epl_s;

one_ms_cnt_temp_s<= "10011" when sbas_enable_is='0' else

"00001";

PROCESS_P1:process(clk_is,POR_is)

begin

if POR_is ='1' then

sat_id_out_os <= "000000";

one_ms_pulse_os <= '0';

one_ms_counter_value_os <= "00000";

twenty_ms_counter_value_os <= "0000000000";

code_cyle_count_value_os <= "0000000000";

code_phase_count_os <= "00000000000000000000000000000000";

g1_reg_s <= "0000000000";

g2_reg_s <= "0000000000";

Codeclk_1p023M_s <= '0';

Codeclk_2p046M_s <= '0';

Codeclk_1p023M_latched_s <= '0';

Codeclk_2p046M_latched_s <= '0';

Code_phase_s <= "00000000000000000000000000000000";

code_Nco_add_s <= "00000000000000000000000000000000";

Codeout_temp_s <= '0';

Codeout_epl_s <= (others => '0');

Slew_count_s <= "0000000000";

one_ms_cnt_s <= "00000";

twenty_ms_cnt_s <= "0000000000";

Data_boundary_output_every20ms_os <= '1';

Bit_sync_found_s <= '0';

elsif clk_is='1' and clk_is'event then

-- start signal comes from channel assignment FSM module.

if(start_is = '1') then

one_ms_pulse_os <= '0';

Codeclk_1p023M_s <= '0';

Codeclk_2p046M_s <= '0';

Codeclk_1p023M_latched_s <= '0';

Codeclk_2p046M_latched_s <= '0';

Code_phase_s <= (others=>'0');

-- to output what svid is tracked.

sat_id_out_os <= sat_id_in_is;

g1_reg_s <= (others=>'1');

-- upon start pulse from acquisition module, here the direct value of code nco is loaded which comes from acquisition module

Slew_count_s <= (others=>'1');

-- to see that when incrementing starts with 0 slew count.

-- otherwise 1 slew count difference will be there.

code_Nco_add_s <= code_freq_init_is;

-- from acquisition module this input corrected with code doppler comes.

Data_boundary_output_every20ms_os <= '1';

Bit_sync_found_s <= '0';

-- G2 registers are initialized based on SVID.

CASE_1:case sat_id_in_is is

-- first 8 are GPS 1- 8  next 4 NavIC 1-4  next SBAS 1-4

when "000000" => g2_reg_s <= "1111101100";

when "000001" => g2_reg_s <= "1111011000";

when "000010" => g2_reg_s <= "1110110000";

when "000011" => g2_reg_s <= "1101100000";

when "000100" => g2_reg_s <= "0010010110";

when "000101" => g2_reg_s <= "0100101100";

when "000110" => g2_reg_s <= "0110010110";

when "000111" => g2_reg_s <= "1100101100";

when "001000" => g2_reg_s <= "1110010111";

when "001001" => g2_reg_s <= "0110010000";

when "001010" => g2_reg_s <= "0010110001";

when "001011" => g2_reg_s <= "0100111010";

when "001100" => g2_reg_s <= "0001010010";--0450  PRN-138

when "001101" => g2_reg_s <= "1000010101";--1241  PRN-121

when "001110" => g2_reg_s <= "1110110100";--0267  PRN-122

when "001111" => g2_reg_s <= "0101100100";--0232  PRN-123

-- first 8 are GPS 9- 16  next 4 NavIC 5-8  next SBAS 5-8

when "010000" => g2_reg_s <= "1001011000";

when "010001" => g2_reg_s <= "1101110100";

when "010010" => g2_reg_s <= "1011101000";

when "010011" => g2_reg_s <= "1110100000";

when "010100" => g2_reg_s <= "1101000000";

when "010101" => g2_reg_s <= "1010000000";

when "010110" => g2_reg_s <= "0100000000";

when "010111" => g2_reg_s <= "1000000000";

when "011000" => g2_reg_s <= "0000110111";

when "011001" => g2_reg_s <= "1101011000";

when "011010" => g2_reg_s <= "0010100000";

when "011011" => g2_reg_s <= "0000110010";

when "011100" => g2_reg_s <= "1011011100"; -- 0355  PRN 147

when "011101" => g2_reg_s <= "0111110001"; -- 1076  PRN 125

when "011110" => g2_reg_s <= "0010111111"; -- 1764  PRN 126

when "011111" => g2_reg_s <= "1111001110"; -- 0717  PRN 127

-- first 8 are GPS 17- 24 next 4 NavIC 9-12  next SBAS 9 - 12

when "100000" => g2_reg_s <= "1000100110";

when "100001" => g2_reg_s <= "0001001100";

when "100010" => g2_reg_s <= "0010011000";

when "100011" => g2_reg_s <= "0100110000";

when "100100" => g2_reg_s <= "1001100000";

when "100101" => g2_reg_s <= "0011000000";

when "100110" => g2_reg_s <= "0011001110";

when "100111" => g2_reg_s <= "1001110000";

when "101000" => g2_reg_s <= "0001100100";

when "101001" => g2_reg_s <= "0010011011";

when "101010" => g2_reg_s <= "0011001000";

when "101011" => g2_reg_s <= "0011111011";

when "101100" => g2_reg_s <= "0101101011";  --1532  PRN 128

when "101101" => g2_reg_s <= "0001010101";  --1250  PRN 129

when "101110" => g2_reg_s <= "1000011100";  --0341  PRN 130

when "101111" => g2_reg_s <= "1001011010";  --0551  PRN 131

-- first 8 are GPS 25- 32 next 4 NavIC 13-14  next SBAS 13 - 18

when "110000" => g2_reg_s <= "0011100000";

when "110001" => g2_reg_s <= "0111000000";

when "110010" => g2_reg_s <= "1110000000";

when "110011" => g2_reg_s <= "1100000000";

when "110100" => g2_reg_s <= "0001010110";

when "110101" => g2_reg_s <= "0010101100";

when "110110" => g2_reg_s <= "0101011000";

when "110111" => g2_reg_s <= "1010110000";

when "111000" => g2_reg_s <= "0100101101";

when "111001" => g2_reg_s <= "0101011110";

when "111010" => g2_reg_s <= "0000101010";  --0520  PRN 132

when "111011" => g2_reg_s <= "1001101111";  --1731  PRN 133

when "111100" => g2_reg_s <= "0110001110";  --0706  PRN 134

when "111101" => g2_reg_s <= "0111000101";  --1216  PRN 135

when "111110" => g2_reg_s <= "0000011110";  --0740  PRN 136

when others =>   g2_reg_s <= "1110000001";  --1007  PRN 137

end case CASE_1;

else

if(Narrow_corr_step_12_enable_is = "00") then

Codeout_os <= Codeout_epl_s(8) & Codeout_epl_s(4) & Codeout_epl_s(0);

elsif(Narrow_corr_step_12_enable_is = "01") then

Codeout_os <= Codeout_epl_s(6) & Codeout_epl_s(4) & Codeout_epl_s(2);

elsif(Narrow_corr_step_12_enable_is = "11") then

Codeout_os <= Codeout_epl_s(5) & Codeout_epl_s(4) & Codeout_epl_s(3);

else

Codeout_os <= Codeout_epl_s(8) & Codeout_epl_s(4) & Codeout_epl_s(0);

end if;

-- when processor writes for clearing the measurements, only 20ms counter is cleared

-- this is required for psuedo range measurements

if(reg_wr_processor_tms_is = '1') then

twenty_ms_cnt_s <= (others=>'0');

end if;

-- when bit-sync from top module, this one ms counter has to be cleared.

if(Data_boundary_pulse_is = '1') then

one_ms_cnt_s <= "00001";

--						twenty_ms_cnt_s <= "0000000000";

Bit_sync_found_s <= '1';

end if;

if(one_ms_cnt_s = "00001" ) and (Bit_sync_found_s = '1') then

Data_boundary_output_every20ms_os <= '1';

elsif(Bit_sync_found_s = '1') then

Data_boundary_output_every20ms_os <= '0';

else

Data_boundary_output_every20ms_os <= '1';

end if;

-- when 500ms latch pulse comes all the counters are latched at a short.

if(epoch_latch_pulse_is = '1') then

one_ms_counter_value_os <= one_ms_cnt_s;

twenty_ms_counter_value_os <= twenty_ms_cnt_s;

code_cyle_count_value_os <= Slew_count_s;

code_phase_count_os <= Code_phase_s;

end if;

-- when discriminator gives the pulse for latching into code nco the values are latched here

if(channel_par_load_is = '1') then

code_Nco_add_s <= code_corr_freq_in_is;

end if;

Code_phase_s <= Code_phase_s + code_Nco_add_s ;

-- above code NCO actions take place.

Codeclk_1p023M_s <= not Code_phase_s(31);

--1.023MHz is generated here

Codeclk_2p046M_s <= not Code_phase_s(30);

--2.046MHz is generated here

-- upon 1.023MHz clock shift the G1,G2 shift registers and calculate their next values based on XOR

if(Codeclk_1p023M_s= '1') and (Codeclk_1p023M_latched_s = '0')  then

--						Codeout_temp_s <= g1_reg_s(9) xor g2_reg_s(9);

--

--						g1_reg_s(1 to 9) <= ( g1_reg_s(0 to 8) );

--						g2_reg_s(1 to 9) <= ( g2_reg_s(0 to 8) );

--						g1_reg_s(0) <= g1_reg_s(2) xor g1_reg_s(9);

--						g2_reg_s(0) <= g2_reg_s(1) xor g2_reg_s(2) xor g2_reg_s(5) xor g2_reg_s(7) xor g2_reg_s(8) xor g2_reg_s(9) ;

Codeout_temp_s <= g1_reg_s(0) xor g2_reg_s(0);

g1_reg_s(8 downto 0) <= ( g1_reg_s(9 downto 1) );

g2_reg_s(8 downto 0) <= ( g2_reg_s(9 downto 1) );

g1_reg_s(9) <= g1_reg_s(7) xor g1_reg_s(0);

g2_reg_s(9) <= g2_reg_s(8) xor g2_reg_s(7) xor g2_reg_s(4) xor g2_reg_s(2) xor g2_reg_s(1) xor g2_reg_s(0) ;

Codeclk_1p023M_latched_s <= '1' ;

-- this setting of this signal ensures multiple times with same code clock period it does not come inside this loop

Slew_count_s <= Slew_count_s + 1;

if(Slew_count_s = "1111111110") then

Slew_count_s <= (others=>'0');

one_ms_cnt_s <= one_ms_cnt_s + 1;

one_ms_pulse_os <= '1';

if(one_ms_cnt_s = one_ms_cnt_temp_s)then --19 for GPS/NAVIC,1 for SBAS

one_ms_cnt_s <= (others=>'0');

twenty_ms_cnt_s <= twenty_ms_cnt_s + 1;

if(twenty_ms_cnt_s =  "0000110001") then --0100101011") then  --299

twenty_ms_cnt_s <= (others=>'0');

end if;

end if;

end if;

else

one_ms_pulse_os <= '0';

end if;

-- when code clok is zero, latched also made zero so that next time high comes it goes to the shifting loop.

if(Codeclk_1p023M_s = '0') then

Codeclk_1p023M_latched_s <= '0';

end if;

-- in this below code Eearly Prompt, Late codes are generated

if(Codeclk_2p046M_s= '1') and (Codeclk_2p046M_latched_s = '0')  then

Codeout_epl_s(8 downto 0) <= Codeout_epl_s(7 downto 0) & Codeout_temp_s;

Codeclk_2p046M_latched_s <= '1';

end if;

if(Codeclk_2p046M_s = '0') then

Codeclk_2p046M_latched_s <= '0';

end if;

end if;  -- sync reset

end if;-- clock event

end process PROCESS_P1;

end trk_code_gen_sbas_a;

75)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_SIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity trk1_e is

Port (    clk_is: in std_logic; -- 40 MHz clock

POR_is : in std_logic; -- POR and BMU reset combined

start_is: in std_logic;  -- to start the channel comes from top fsm when a satellite is found and this channel is free

--			    samp_clk_enable_is : in std_logic;  -- sampling clk signal

channel_par_load_is : in std_logic;	  -- comes from descriminator module to update the nco values and store the loop register values

sat_id_in_is: in std_logic_vector(5 downto 0); -- comes from acquisition module

ant1_ant2_sel_ch_is: in std_logic_vector(1 downto 0);  -- Antenna selection comes from Acquistion module

car_freq_init_is,code_freq_init_is : in std_logic_vector(31 downto 0); -- comes from acquistion module for initializing the NCOs

order1_loop_init_is, dll_order1_loop_init_is : in std_logic_vector(48 downto 0); -- comes from acquisition module for initializing the loops

indata_ant1_is,indata_ant2_is,indata_ant3_is,indata_ant4_is: in std_logic_vector(5 downto 0);  -- data from different antennas

car_mix_ROM_ena_is : IN STD_LOGIC;

car_mix_ROM_wea_is : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

car_mix_ROM_addra_is : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

car_mix_ROM_dina_is : IN STD_LOGIC_VECTOR(19 DOWNTO 0);

sat_id_out_os : out std_logic_vector(5 downto 0);  -- satellite id of this channel

p_i_os,p_q_os,e_i_os,e_q_os,l_i_os,l_q_os : out std_logic_vector(22 downto 0);  -- A& D outputs

lock_status_out_os : out std_logic; -- goes to descriminator modulefor the top lock detector module..

lock_status_in_is: in std_logic; -- comes from descriminator module  for individual storage

loop_start_os: out std_logic;  -- mentioning A&D are ready and descriminator has to work and update

stage_control_os : out std_logic_vector(4 downto 0);  -- fll_start signal, when FLL descriminator to be operated

FLL_data_check_counter_is : in std_logic_vector(31 downto 0);

fll_zero_os : out std_logic;

NAVIC_indicator_os : out std_logic;

Data_boundary_output_os : out std_logic;

fll_data_check_os : out std_logic; -- fll data check for +/- 0.25 rad jump.

-- input to the tracking channels regarding the previous Loop filter values

Prev_phase_error_input_is : in std_logic_vector(16 downto 0);

Order1_input_is,Order2_input_is,Order3_input_is : in std_logic_vector(48 downto 0);

DLL_Order1_input_is,DLL_Order2_input_is : in std_logic_vector(48 downto 0);

-- output to the tracking channels regarding the previous Loop filter values

Prev_phase_error_os : out std_logic_vector(16 downto 0);

Order1_out_os,Order2_out_os,Order3_out_os : out std_logic_vector(48 downto 0);

DLL_Order1_out_os,DLL_Order2_out_os : out std_logic_vector(48 downto 0);

car_corr_freq_in_is: in std_logic_vector(31 downto 0); -- comes from descriminator module after loop filter, the required NCO value.

code_corr_freq_in_is: in std_logic_vector(31 downto 0);  ----comes from descriminator module after loop filter, the required NCO value.

navdata_read_mp_is : in std_logic; -- new data bit is resetted, comes from processor

navdata_reg_os : out std_logic_vector(7 downto 0);  -- register 0 contains navdata, new bit etc..

p_cndr_out_os,q_cndr_out_os : out std_logic_vector(27 downto 0); -- this channel p and q cndr register to compute cndr by processor

car_cycle_count_os: out std_logic_vector(31 downto 0); -- at meas epoch latched

car_phase_count_os : out std_logic_vector(31 downto 0); -- at  meas epoch latched

one_ms_counter_value_os: out std_logic_vector(4 downto 0); -- at meas  epoch latched

twenty_ms_counter_value_os: out std_logic_vector(9 downto 0); -- at meas  epoch latched

code_cyle_count_value_os: out std_logic_vector(9 downto 0); -- at meas  epoch latched

code_phase_count_os : out std_logic_vector(31 downto 0); -- at meas  epoch latched

reg_wr_processor_tms_is : in std_logic; -- to clear twenty ms counter by processor

epoch_latch_pulse_is: in std_logic;	-- epoch pulse to latch the measurements

set1_to_set2_timer_Launch_is : in std_logic_vector(31 downto 0);

set2_to_set3_timer_Launch_is : in std_logic_vector(31 downto 0);

set3_to_set4_timer_Launch_is : in std_logic_vector(31 downto 0);

set4_to_set5_timer_Launch_is : in std_logic_vector(31 downto 0);

Narrow_corr_step1_timer_is : in std_logic_vector(31 downto 0);

Narrow_corr_step2_timer_is : in std_logic_vector(31 downto 0);

Narrow_correlator_step1_enable_is: in std_logic;

Narrow_correlator_step2_enable_is: in std_logic;

fll_timer_is :  in std_logic_vector(31 downto 0);

antenna_combining_control_is : in std_logic_vector(3 downto 0);

lock_counter_threshold1_is : in std_logic_vector(9 downto 0);

lock_counter_threshold2_is : in std_logic_vector(9 downto 0);

reset_channel_en_pr_is : in std_logic -- force reset the channel by processor

);

end trk1_e;

architecture trk1_a of trk1_e is

component trk_code_gen_e

Port ( clk_is : in  STD_LOGIC;

POR_is : in  STD_LOGIC;

start_is : in std_logic;

channel_par_load_is : in std_logic;

sat_id_in_is: in std_logic_vector(5 downto 0); -- comes from acquisition module

code_corr_freq_in_is: in std_logic_vector(31 downto 0);  ----comes from descriminator module

code_freq_init_is : in std_logic_vector(31 downto 0);

sat_id_out_os : out std_logic_vector(5 downto 0);  -- satellite id of this channel

one_ms_pulse_os : out std_logic;  -- one ms pulse for monitering

one_ms_counter_value_os: out std_logic_vector(4 downto 0); -- at epoch latched

twenty_ms_counter_value_os: out std_logic_vector(9 downto 0); -- at epoch latched

code_cyle_count_value_os: out std_logic_vector(9 downto 0); -- at epoch latched

code_phase_count_os : out std_logic_vector(31 downto 0); -- at epoch latched

reg_wr_processor_tms_is : in std_logic; -- to clear twenty ms counter by processor

epoch_latch_pulse_is: in std_logic;	-- epoch pulse to latch the measurements

Data_boundary_pulse_is : in std_logic;  -- to check the data boundary and generate p_cndr_out, q_cndr_out  -- kishore

Narrow_corr_step_12_enable_is : in std_logic_vector(1 downto 0);

Data_boundary_output_every20ms_os : out std_logic;

Codeout_os : out  std_logic_vector(2 downto 0)  -- epl out for the main tracking channel

);

end component trk_code_gen_e;

signal cycle_counted_s,prior_loop_start_s,prior1_loop_start_s, loop_start_one_pulse_s : std_logic;

-- for counting the cycles of carrier

signal trk_accum_s,trk_freq_word_s: std_logic_vector(31 downto 0);

-- tracking NCO registers.

signal IP_mixout_s, QP_mixout_s : std_logic_vector(7 downto 0);

signal IP_mixout1_s, QP_mixout1_s : std_logic_vector(6 downto 0);

signal IP_mixout2_s, QP_mixout2_s : std_logic_vector(6 downto 0);

-- this signal takes the input from incoming data and generates the carrier mixout results

signal ei_mix_s, eq_mix_s, pi_mix_s, pq_mix_s, li_mix_s, lq_mix_s : std_logic_vector(7 downto 0);

signal sig_accum_pinphase_s,sig_accum_pqphase_s : std_logic_vector(22 downto 0);

signal sig_accum_einphase_s,sig_accum_eqphase_s : std_logic_vector(22 downto 0);

signal sig_accum_linphase_s,sig_accum_lqphase_s : std_logic_vector(22 downto 0);

--signal local_p_cndr_s, local_q_cndr_s : std_logic_vector(31 downto 0);

-- above signal will have intermediate accumulating results, once at 1ms epoch these results will be latched with 6 outputs.

signal trking_counter_s : std_logic_vector(31 downto 0);

-- this signal is maintained to maintain how much millisecond into track.

signal one_ms_tic_s: std_logic;

-- this signal to latch all 6 correlator dumps.

signal one_ms_latching_sig_s : std_logic;

signal incoming_sig_s: std_logic_vector(11 downto 0);

-- this signal to select the required antenna based on antenna selection.

signal antenna_port_selection_s : std_logic_vector(1 downto 0);

-- this signal latches from which antenna inputs to be taken for tracking

signal Rx_car_cycle_counter_s : std_logic_vector(31 downto 0);

-- free running cycle counters

signal Code_input_epl_s : std_logic_vector(2 downto 0);

-- for early promt late code signals

signal Data_boundary_pulse_s : std_logic;

-- lock detector module for each channel.  major maintanance is maintained in top module.

component lock_detector_ch_e is

PORT(

clk_40MHz_is : IN std_logic;

POR_is : IN std_logic;

start_is : IN std_logic;

channel_par_load_is : IN std_logic;

lock_status_in_is : IN std_logic;

reset_channel_en_pr_is : IN std_logic;

lock_counter_threshold1_is : in std_logic_vector(9 downto 0);

lock_counter_threshold2_is : in std_logic_vector(9 downto 0);

bit_sync_ind_is : IN std_logic;

lock_status_os : OUT std_logic

);

end component lock_detector_ch_e;

-- bit synchrnoizer module, this gives data boundary os signal for reseting 1ms counter.

COMPONENT bit_sync_ch_e

PORT(

clk_40MHz_is : IN std_logic;

POR_is : IN std_logic;

start_is : IN std_logic;

one_ms_pulse_is: IN std_logic;

I_data_is, Q_data_is : IN std_logic_vector(22 downto 0);

lock_status_is : IN std_logic;

navdata_read_mp_is : IN std_logic;

navdata_os : OUT std_logic;

newdata_indicator_os : OUT std_logic;

bit_sync_indicator_os : OUT std_logic;

bit_sync_found_pulse_os: OUT std_logic;

p_cndr_os : OUT std_logic_vector(27 downto 0);

q_cndr_os : OUT std_logic_vector(27 downto 0)

);

END COMPONENT bit_sync_ch_e;

COMPONENT carrier_mix_ROM

PORT (

clka : IN STD_LOGIC;

ena : IN STD_LOGIC;

wea : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

addra : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

dina : IN STD_LOGIC_VECTOR(19 DOWNTO 0);

clkb : IN STD_LOGIC;

addrb : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

doutb : OUT STD_LOGIC_VECTOR(19 DOWNTO 0)

);

END COMPONENT carrier_mix_ROM;

COMPONENT edac_14_dec1_e

Port (

-- Input signals

rd_en_is       : in  STD_LOGIC;  -- EDAC Decoder  Read enable

data_in_is     : in  STD_LOGIC_VECTOR (13 downto 0);   -- Input Information bits to EDAC Decoder

par_in_is      : in  STD_LOGIC_VECTOR (5 downto 0);   -- Input Parity bits to EDAC Decoder

-- Output signals

data_out_os    : out  STD_LOGIC_VECTOR (13 downto 0)  -- Output information bits after correction

);

end COMPONENT edac_14_dec1_e;

signal lock_status_out_s: std_logic;

signal bit_sync_indicator_s: std_logic;

signal p_i_s, p_q_s , abs_pq_s: std_logic_vector(22 downto 0);

signal navdata_s, newdata_indicator_s : std_logic;

signal Narrow_corr_step_12_enable_s : std_logic_vector(1 downto 0);

signal addrb_s : std_logic_vector(9 downto 0);

signal doutb_s : std_logic_vector(19 downto 0);

signal edac_out_s : std_logic_vector(13 downto 0);

signal addrb1_s : std_logic_vector(9 downto 0);

signal doutb1_s : std_logic_vector(19 downto 0);

signal edac_out1_s : std_logic_vector(13 downto 0);

begin

carmixrom1 : carrier_mix_ROM

PORT MAP (

clka => clk_is,     ena => car_mix_ROM_ena_is,     wea => car_mix_ROM_wea_is,     addra => car_mix_ROM_addra_is,     dina => car_mix_ROM_dina_is,

clkb => clk_is,     addrb => addrb_s,      doutb => doutb_s   );

EDAC_mixROM1: edac_14_dec1_e

port map (

rd_en_is => '1', data_in_is => doutb_s(13 downto 0)  ,par_in_is =>  doutb_s(19 downto 14) ,data_out_os => edac_out_s );

carmixrom2 : carrier_mix_ROM

PORT MAP (

clka => clk_is,     ena => car_mix_ROM_ena_is,     wea => car_mix_ROM_wea_is,     addra => car_mix_ROM_addra_is,     dina => car_mix_ROM_dina_is,

clkb => clk_is,     addrb => addrb1_s,      doutb => doutb1_s   );

EDAC_mixROM2: edac_14_dec1_e

port map (

rd_en_is => '1', data_in_is => doutb1_s(13 downto 0)  ,par_in_is =>  doutb1_s(19 downto 14) ,data_out_os => edac_out1_s );

Inst_lock_detector_ch_e: lock_detector_ch_e PORT MAP(

clk_40MHz_is => clk_is,

POR_is => POR_is,

start_is => start_is,

channel_par_load_is => channel_par_load_is,

lock_status_in_is => lock_status_in_is,

reset_channel_en_pr_is => reset_channel_en_pr_is,

lock_counter_threshold1_is => lock_counter_threshold1_is,

lock_counter_threshold2_is => lock_counter_threshold2_is,

bit_sync_ind_is => bit_sync_indicator_s,

lock_status_os => lock_status_out_s

);

lock_status_out_os <= lock_status_out_s;

Inst_bit_sync_ch_e: bit_sync_ch_e PORT MAP(

clk_40MHz_is => clk_is,

POR_is => POR_is,

start_is => start_is,

one_ms_pulse_is => loop_start_one_pulse_s,

I_data_is => p_i_s,

Q_data_is => abs_pq_s,

lock_status_is => lock_status_out_s,

navdata_read_mp_is => navdata_read_mp_is,

navdata_os => navdata_s,

newdata_indicator_os => newdata_indicator_s,

bit_sync_indicator_os => bit_sync_indicator_s,

bit_sync_found_pulse_os => Data_boundary_pulse_s,

p_cndr_os => p_cndr_out_os,

q_cndr_os => q_cndr_out_os

);

p_i_os  <= p_i_s;

p_q_os  <= p_q_s;

navdata_reg_os <= '0' & '0' & antenna_port_selection_s & bit_sync_indicator_s & lock_status_out_s & newdata_indicator_s & navdata_s;

loop_start_os <= loop_start_one_pulse_s;

codegen: trk_code_gen_e

Port map ( clk_is => clk_is,            POR_is => POR_is,            start_is => start_is,  	channel_par_load_is => channel_par_load_is,

sat_id_in_is =>  sat_id_in_is,

code_corr_freq_in_is => code_corr_freq_in_is,

code_freq_init_is => code_freq_init_is,

sat_id_out_os => sat_id_out_os ,one_ms_pulse_os   => one_ms_tic_s, one_ms_counter_value_os => one_ms_counter_value_os,

twenty_ms_counter_value_os => twenty_ms_counter_value_os, code_cyle_count_value_os => code_cyle_count_value_os,

code_phase_count_os=> code_phase_count_os,

reg_wr_processor_tms_is	=> reg_wr_processor_tms_is,

epoch_latch_pulse_is => epoch_latch_pulse_is,

Data_boundary_pulse_is => Data_boundary_pulse_s,

Narrow_corr_step_12_enable_is => Narrow_corr_step_12_enable_s,

Data_boundary_output_every20ms_os => Data_boundary_output_os, Codeout_os =>Code_input_epl_s );

-- antenna mux selection based which input

P_data_selection:process(antenna_port_selection_s,indata_ant1_is,indata_ant2_is,indata_ant3_is,indata_ant4_is)

begin

c_ant_port_decoder: case antenna_port_selection_s is

when "00" => incoming_sig_s <= indata_ant3_is & indata_ant1_is  ;

when "01" => incoming_sig_s <= indata_ant4_is & indata_ant2_is  ;

when "10" => incoming_sig_s <= indata_ant1_is & indata_ant3_is  ;

when "11" => incoming_sig_s <= indata_ant2_is & indata_ant4_is  ;

when others => incoming_sig_s <= indata_ant3_is & indata_ant1_is  ;

end case c_ant_port_decoder;

end process P_data_selection;

P_ch_reg: process(clk_is,POR_is)

begin

if (POR_is = '1') then

p_i_s  <= "00000000000000000000000";

p_q_s  <= "00000000000000000000000";

abs_pq_s<= "00000000000000000000000";

e_i_os  <= "00000000000000000000000";

e_q_os  <= "00000000000000000000000";

l_i_os  <= "00000000000000000000000";

l_q_os  <= "00000000000000000000000";

loop_start_one_pulse_s <= '0';

fll_data_check_os <= '0';

Prev_phase_error_os  <= "00000000000000000";

Order1_out_os  <= "0000000000000000000000000000000000000000000000000";

Order2_out_os  <= "0000000000000000000000000000000000000000000000000";

Order3_out_os  <= "0000000000000000000000000000000000000000000000000";

DLL_Order1_out_os  <= "0000000000000000000000000000000000000000000000000";

DLL_Order2_out_os  <= "0000000000000000000000000000000000000000000000000";

car_cycle_count_os  <= "00000000000000000000000000000000";

car_phase_count_os  <= "00000000000000000000000000000000";

cycle_counted_s <= '0';

trk_accum_s     <= "00000000000000000000000000000000";

trk_freq_word_s <= "00000000000000000000000000000000";

IP_mixout_s <= "00000000";

QP_mixout_s <= "00000000";

IP_mixout1_s <= "0000000";

QP_mixout1_s <= "0000000";

IP_mixout2_s <= "0000000";

QP_mixout2_s <= "0000000";

ei_mix_s<= "00000000";

eq_mix_s<= "00000000";

pi_mix_s<= "00000000";

pq_mix_s<= "00000000";

li_mix_s<= "00000000";

lq_mix_s <= "00000000";

sig_accum_pinphase_s <= "00000000000000000000000";

sig_accum_pqphase_s <=  "00000000000000000000000";

sig_accum_einphase_s <= "00000000000000000000000";

sig_accum_eqphase_s <=  "00000000000000000000000";

sig_accum_linphase_s <= "00000000000000000000000";

sig_accum_lqphase_s <=  "00000000000000000000000";

trking_counter_s <= "00000000000000000000000000000000";

Rx_car_cycle_counter_s  <= "00000000000000000000000000000000";

antenna_port_selection_s <= "00";

one_ms_latching_sig_s <= '0';

prior_loop_start_s <= '0';

prior1_loop_start_s <= '0';

stage_control_os <= "00001";

fll_zero_os <= '0';

NAVIC_indicator_os <= '0';

addrb_s<= (others=>'0');

addrb1_s<= (others=>'0');

elsif clk_is='1' and clk_is'event then

-- when acquisition module tells us to start, we will be starting.

if(start_is='1') then  -- synchrnous reset input

p_i_s  <= "00000000000000000000000";

p_q_s  <= "00000000000000000000000";

abs_pq_s<= "00000000000000000000000";

e_i_os  <= "00000000000000000000000";

e_q_os  <= "00000000000000000000000";

l_i_os  <= "00000000000000000000000";

l_q_os  <= "00000000000000000000000";

sig_accum_pinphase_s <= "00000000000000000000000";

sig_accum_pqphase_s <=  "00000000000000000000000";

sig_accum_einphase_s <= "00000000000000000000000";

sig_accum_eqphase_s <=  "00000000000000000000000";

sig_accum_linphase_s <= "00000000000000000000000";

sig_accum_lqphase_s <=  "00000000000000000000000";

IP_mixout_s <= "00000000";

QP_mixout_s <= "00000000";

IP_mixout1_s <= "0000000";

QP_mixout1_s <= "0000000";

IP_mixout2_s <= "0000000";

QP_mixout2_s <= "0000000";

ei_mix_s<= "00000000";

eq_mix_s<= "00000000";

pi_mix_s<= "00000000";

pq_mix_s<= "00000000";

li_mix_s<= "00000000";

lq_mix_s <= "00000000";

trking_counter_s <= "00000000000000000000000000000000";

trk_accum_s     <= "00000000000000000000000000000000";

trk_freq_word_s <= car_freq_init_is;

Order1_out_os <= order1_loop_init_is;

DLL_Order1_out_os <= dll_order1_loop_init_is;

antenna_port_selection_s <= ant1_ant2_sel_ch_is;

fll_data_check_os <= '0';

-- at acquisition loading the antenna input is latched

Prev_phase_error_os  <= "00000000000000000";

Order2_out_os  <= "0000000000000000000000000000000000000000000000000";

Order3_out_os  <= "0000000000000000000000000000000000000000000000000";

DLL_Order2_out_os  <= "0000000000000000000000000000000000000000000000000";

cycle_counted_s <= '0'; -- V2_change

one_ms_latching_sig_s <= '0'; -- V2_change

prior_loop_start_s <= '0'; -- V2_change

prior1_loop_start_s <= '0'; -- V2_change

loop_start_one_pulse_s <= '0';

stage_control_os <= "00001";

fll_zero_os <= '0';

addrb_s<= (others=>'0');

addrb1_s<= (others=>'0');

-- if( sat_id_in_is(5 downto 0) >= "001000" ) and ( sat_id_in_is(5 downto 0) <= "001011" ) then

-- NAVIC_indicator_os <= '1';

-- elsif( sat_id_in_is(5 downto 0) >= "011000" ) and ( sat_id_in_is(5 downto 0) <= "011011" ) then

-- NAVIC_indicator_os <= '1';

-- elsif( sat_id_in_is(5 downto 0) >= "101000" ) and ( sat_id_in_is(5 downto 0) <= "101011" )then

-- NAVIC_indicator_os <= '1';

-- elsif( sat_id_in_is(5 downto 0) >= "111000" ) and ( sat_id_in_is(5 downto 0) <= "111001" ) then

-- NAVIC_indicator_os <= '1';

-- else

-- NAVIC_indicator_os <= '0';

-- end if;

C_Navid_idetifcation_decoder: case sat_id_in_is is

when "001000" => NAVIC_indicator_os <='1';

when "001001" => NAVIC_indicator_os <='1';

when "001010" => NAVIC_indicator_os <='1';

when "001011" => NAVIC_indicator_os <='1';

when "011000" => NAVIC_indicator_os <='1';

when "011001" => NAVIC_indicator_os <='1';

when "011010" => NAVIC_indicator_os <='1';

when "011011" => NAVIC_indicator_os <='1';

when "101000" => NAVIC_indicator_os <='1';

when "101001" => NAVIC_indicator_os <='1';

when "101010" => NAVIC_indicator_os <='1';

when "101011" => NAVIC_indicator_os <='1';

when "111000" => NAVIC_indicator_os <='1';

when "111001" => NAVIC_indicator_os <='1';

when  OTHERS  => NAVIC_indicator_os<='0';

end case C_Navid_idetifcation_decoder;

else

-- load the corrected frequency from loop filter and store all values here.

if(channel_par_load_is = '1') then

trk_freq_word_s <= car_corr_freq_in_is;

Prev_phase_error_os <= Prev_phase_error_input_is;

Order1_out_os <= Order1_input_is ;

Order2_out_os <= Order2_input_is ;

Order3_out_os <= Order3_input_is ;

DLL_Order1_out_os <=DLL_Order1_input_is;

DLL_Order2_out_os <= DLL_Order2_input_is;

end if;

-- latching when 500ms measurement latch

if(epoch_latch_pulse_is = '1') then

car_cycle_count_os <=  Rx_car_cycle_counter_s;

car_phase_count_os <= 	trk_accum_s;

end if;

if( (trking_counter_s >= Narrow_corr_step2_timer_is) and (Narrow_correlator_step2_enable_is = '1')  ) then

Narrow_corr_step_12_enable_s <= "11";

elsif( (trking_counter_s >= Narrow_corr_step1_timer_is) and (Narrow_correlator_step1_enable_is = '1') ) then

Narrow_corr_step_12_enable_s <= "01";

else

Narrow_corr_step_12_enable_s <= "00";

end if;

-- in fll there is datacheck for overflow of pi/2 condition for this it is put

if(trking_counter_s(31 downto 0) = FLL_data_check_counter_is) then		-- if tracking counter is more than 10ms, the data detection should happen in fll

fll_data_check_os <= '1';

end if;

if(trking_counter_s(31 downto 0) >= fll_timer_is) then

fll_zero_os <= '1';

end if;

if(trking_counter_s(31 downto 0) <= set1_to_set2_timer_Launch_is )  then

stage_control_os <= "00001";

elsif(trking_counter_s(31 downto 0) <= set2_to_set3_timer_Launch_is ) then

stage_control_os <= "00010";

elsif(trking_counter_s(31 downto 0) <= set3_to_set4_timer_Launch_is ) then

stage_control_os <= "00100";

elsif(trking_counter_s(31 downto 0) <= set4_to_set5_timer_Launch_is ) then

stage_control_os <= "01000";

elsif(trking_counter_s(31 downto 0) > set4_to_set5_timer_Launch_is )  then

stage_control_os <= "10000";

end if;

-- when output from code gen module indicating 1ms boundary it is latched into a signal.

if(one_ms_tic_s = '1') then

one_ms_latching_sig_s <= '1';

else

one_ms_latching_sig_s <= '0';

end if;

--					if(samp_clk_enable_is = '1')  and (one_ms_latching_sig_s = '1') then

--						one_ms_latching_sig_s <= '0';

--					end if;

-- below code for cycle counters storage and phase storage.

if(trk_accum_s(31)='0') and (cycle_counted_s = '0') then

Rx_car_cycle_counter_s <= Rx_car_cycle_counter_s + 1;

cycle_counted_s <= '1';

end if;

if(trk_accum_s(31)='1') then

cycle_counted_s <= '0';

end if;

if(Code_input_epl_s(0) = '1') then

ei_mix_s <= IP_mixout_s;

eq_mix_s <= QP_mixout_s;

else

ei_mix_s <= -IP_mixout_s;

eq_mix_s <= -QP_mixout_s;

end if;

if(Code_input_epl_s(1) = '1') then

pi_mix_s <= IP_mixout_s;

pq_mix_s <= QP_mixout_s;

else

pi_mix_s <= -IP_mixout_s;

pq_mix_s <= -QP_mixout_s;

end if;

if(Code_input_epl_s(2) = '1') then

li_mix_s <= IP_mixout_s;

lq_mix_s <= QP_mixout_s;

else

li_mix_s <= -IP_mixout_s;

lq_mix_s <= -QP_mixout_s;

end if;

if( (antenna_port_selection_s & antenna_combining_control_is(0) = "001") or (antenna_port_selection_s & antenna_combining_control_is(1) = "011") or (antenna_port_selection_s & antenna_combining_control_is(2) = "101") or (antenna_port_selection_s & antenna_combining_control_is(3) = "111") ) then

IP_mixout_s <= sxt(IP_mixout1_s,8) + sxt(IP_mixout2_s,8);

QP_mixout_s <= sxt(QP_mixout1_s,8) + sxt(QP_mixout2_s,8);

else

IP_mixout_s <=  sxt(IP_mixout1_s,8);

QP_mixout_s <=  sxt(QP_mixout1_s,8);

end if;

-- when 1ms boundary detected by codegen module, the pulse comes.

if(one_ms_latching_sig_s = '1') then

prior1_loop_start_s <= '1';

-- a pulse set high to make the loop start output.

-- latching 6 arms into a signal for discriminator

p_i_s   <= sig_accum_pinphase_s;

p_q_s    <= sig_accum_pqphase_s;

abs_pq_s <= abs(sig_accum_pqphase_s);

e_i_os   <= sig_accum_einphase_s;

e_q_os    <= sig_accum_eqphase_s;

l_i_os   <= sig_accum_linphase_s;

l_q_os    <= sig_accum_lqphase_s;

trking_counter_s <= trking_counter_s + 1;

sig_accum_pinphase_s <= (others => '0');

sig_accum_pqphase_s <= (others => '0');

sig_accum_einphase_s <= (others => '0');

sig_accum_eqphase_s <= (others => '0');

sig_accum_linphase_s <= (others => '0');

sig_accum_lqphase_s <= (others => '0');

else

prior1_loop_start_s <= '0';

--accumulation of 6 arms are happening below

sig_accum_pinphase_s <= sig_accum_pinphase_s + pi_mix_s;

sig_accum_pqphase_s <= sig_accum_pqphase_s + pq_mix_s;

sig_accum_einphase_s <= sig_accum_einphase_s + ei_mix_s;

sig_accum_eqphase_s <= sig_accum_eqphase_s + eq_mix_s;

sig_accum_linphase_s <= sig_accum_linphase_s + li_mix_s;

sig_accum_lqphase_s <= sig_accum_lqphase_s + lq_mix_s;

-- accumulation of local carrier mix is happening.

end if;

if(prior1_loop_start_s = '1') then

prior_loop_start_s <= '1';

else

prior_loop_start_s <= '0';

end if;

if(prior_loop_start_s = '1') and (lock_status_out_s = '1') then

loop_start_one_pulse_s <= '1';

else

loop_start_one_pulse_s	<= '0';

-- loop start os set high based on prior loop start signals.

end if;

--nco action

trk_accum_s <= trk_accum_s + trk_freq_word_s;

addrb_s <= incoming_sig_s(5 downto 0) & trk_accum_s(31 downto 28) ;

addrb1_s <= incoming_sig_s(11 downto 6) & trk_accum_s(31 downto 28) ;

IP_mixout1_s <=   edac_out_s(6 downto 0);

QP_mixout1_s <=   edac_out_s(13 downto 7);

IP_mixout2_s <=   edac_out1_s(6 downto 0);

QP_mixout2_s <=   edac_out1_s(13 downto 7);

end if; -- sync reset

end if; -- if of clock event

end process P_ch_reg;

end trk1_a;

76) trk2_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_SIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity trk2_e is

Port (    clk_is: in std_logic; -- 40 MHz clock

POR_is : in std_logic; -- POR and BMU reset combined

start_is: in std_logic;  -- to start the channel comes from top fsm when a satellite is found and this channel is free

--			    samp_clk_enable_is : in std_logic;  -- sampling clk signal

channel_par_load_is : in std_logic;	  -- comes from descriminator module to update the nco values and store the loop register values

sat_id_in_is: in std_logic_vector(5 downto 0); -- comes from acquisition module

ant1_ant2_sel_ch_is: in std_logic_vector(1 downto 0);  -- Antenna selection comes from Acquistion module

car_freq_init_is,code_freq_init_is : in std_logic_vector(31 downto 0); -- comes from acquistion module for initializing the NCOs

order1_loop_init_is, dll_order1_loop_init_is : in std_logic_vector(48 downto 0); -- comes from acquisition module for initializing the loops

indata_ant1_is,indata_ant2_is,indata_ant3_is,indata_ant4_is: in std_logic_vector(5 downto 0);  -- data from different antennas

car_mix_ROM_ena_is : IN STD_LOGIC;

car_mix_ROM_wea_is : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

car_mix_ROM_addra_is : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

car_mix_ROM_dina_is : IN STD_LOGIC_VECTOR(19 DOWNTO 0);

sat_id_out_os : out std_logic_vector(5 downto 0);  -- satellite id of this channel

p_i_os,p_q_os,e_i_os,e_q_os,l_i_os,l_q_os : out std_logic_vector(22 downto 0);  -- A& D outputs

lock_status_out_os : out std_logic; -- goes to descriminator modulefor the top lock detector module..

lock_status_in_is: in std_logic; -- comes from descriminator module  for individual storage

loop_start_os: out std_logic;  -- mentioning A&D are ready and descriminator has to work and update

stage_control_os : out std_logic_vector(4 downto 0);  -- fll_start signal, when FLL descriminator to be operated

FLL_data_check_counter_is : in std_logic_vector(31 downto 0);

fll_zero_os : out std_logic;

NAVIC_indicator_os : out std_logic;

Data_boundary_output_os : out std_logic;

fll_data_check_os : out std_logic; -- fll data check for +/- 0.25 rad jump.

-- input to the tracking channels regarding the previous Loop filter values

Prev_phase_error_input_is : in std_logic_vector(16 downto 0);

Order1_input_is,Order2_input_is,Order3_input_is : in std_logic_vector(48 downto 0);

DLL_Order1_input_is,DLL_Order2_input_is : in std_logic_vector(48 downto 0);

-- output to the tracking channels regarding the previous Loop filter values

Prev_phase_error_os : out std_logic_vector(16 downto 0);

Order1_out_os,Order2_out_os,Order3_out_os : out std_logic_vector(48 downto 0);

DLL_Order1_out_os,DLL_Order2_out_os : out std_logic_vector(48 downto 0);

car_corr_freq_in_is: in std_logic_vector(31 downto 0); -- comes from descriminator module after loop filter, the required NCO value.

code_corr_freq_in_is: in std_logic_vector(31 downto 0);  ----comes from descriminator module after loop filter, the required NCO value.

navdata_read_mp_is : in std_logic; -- new data bit is resetted, comes from processor

navdata_reg_os : out std_logic_vector(7 downto 0);  -- register 0 contains navdata, new bit etc..

p_cndr_out_os,q_cndr_out_os : out std_logic_vector(27 downto 0); -- this channel p and q cndr register to compute cndr by processor

car_cycle_count_os: out std_logic_vector(31 downto 0); -- at meas epoch latched

car_phase_count_os : out std_logic_vector(31 downto 0); -- at  meas epoch latched

one_ms_counter_value_os: out std_logic_vector(4 downto 0); -- at meas  epoch latched

twenty_ms_counter_value_os: out std_logic_vector(9 downto 0); -- at meas  epoch latched

code_cyle_count_value_os: out std_logic_vector(9 downto 0); -- at meas  epoch latched

code_phase_count_os : out std_logic_vector(31 downto 0); -- at meas  epoch latched

reg_wr_processor_tms_is : in std_logic; -- to clear twenty ms counter by processor

epoch_latch_pulse_is: in std_logic;	-- epoch pulse to latch the measurements

set1_to_set2_timer_Launch_is : in std_logic_vector(31 downto 0);

set2_to_set3_timer_Launch_is : in std_logic_vector(31 downto 0);

set3_to_set4_timer_Launch_is : in std_logic_vector(31 downto 0);

set4_to_set5_timer_Launch_is : in std_logic_vector(31 downto 0);

Narrow_corr_step1_timer_is : in std_logic_vector(31 downto 0);

Narrow_corr_step2_timer_is : in std_logic_vector(31 downto 0);

Narrow_correlator_step1_enable_is: in std_logic;

Narrow_correlator_step2_enable_is: in std_logic;

fll_timer_is :  in std_logic_vector(31 downto 0);

reset_channel_en_pr_is : in std_logic; -- force reset the channel by processor

antenna_combining_control_is : in std_logic_vector(3 downto 0);

lock_counter_threshold1_is : in std_logic_vector(9 downto 0);

lock_counter_threshold2_is : in std_logic_vector(9 downto 0);

-- these are for channel 18 only

sbas_enable_is : in std_logic; -- comes from processor if this channel needs to be tracked for SBAS

sbas_navdata_os : out std_logic_vector(5 downto 0) -- This channel i.e 18th channel SBAS data

);

end trk2_e;

architecture trk2_a of trk2_e is

component trk_code_gen_sbas_e

Port ( clk_is : in  STD_LOGIC;

POR_is : in  STD_LOGIC;

start_is : in std_logic;

channel_par_load_is : in std_logic;

sat_id_in_is: in std_logic_vector(5 downto 0);

code_corr_freq_in_is: in std_logic_vector(31 downto 0);  ----comes from descriminator module

code_freq_init_is : in std_logic_vector(31 downto 0);

sbas_enable_is : in std_logic;

sat_id_out_os : out std_logic_vector(5 downto 0);  -- satellite id of this channel

one_ms_pulse_os : out std_logic;  -- one ms pulse for monitering

one_ms_counter_value_os: out std_logic_vector(4 downto 0); -- at epoch latched

twenty_ms_counter_value_os: out std_logic_vector(9 downto 0); -- at epoch latched

code_cyle_count_value_os: out std_logic_vector(9 downto 0); -- at epoch latched

code_phase_count_os : out std_logic_vector(31 downto 0); -- at epoch latched

reg_wr_processor_tms_is : in std_logic; -- to clear twenty ms counter by processor

epoch_latch_pulse_is: in std_logic;	-- epoch pulse to latch the measurements

Data_boundary_pulse_is : in std_logic;  -- to check the data boundary and generate p_cndr_out, q_cndr_out  -- kishore

Narrow_corr_step_12_enable_is : in std_logic_vector(1 downto 0);

Data_boundary_output_every20ms_os : out std_logic;

Codeout_os : out  std_logic_vector(2 downto 0)  -- epl out for the main tracking channel

);

end component;

signal cycle_counted_s,prior_loop_start_s,prior1_loop_start_s, loop_start_one_pulse_s : std_logic;

-- for counting the cycles of carrier

signal trk_accum_s,trk_freq_word_s: std_logic_vector(31 downto 0);

-- tracking NCO registers.

signal IP_mixout_s, QP_mixout_s : std_logic_vector(7 downto 0);

signal IP_mixout1_s, QP_mixout1_s : std_logic_vector(6 downto 0);

signal IP_mixout2_s, QP_mixout2_s : std_logic_vector(6 downto 0);

-- this signal takes the input from incoming data and generates the carrier mixout results

signal ei_mix_s, eq_mix_s, pi_mix_s, pq_mix_s, li_mix_s, lq_mix_s : std_logic_vector(7 downto 0);

signal sig_accum_pinphase_s,sig_accum_pqphase_s : std_logic_vector(22 downto 0);

signal sig_accum_einphase_s,sig_accum_eqphase_s : std_logic_vector(22 downto 0);

signal sig_accum_linphase_s,sig_accum_lqphase_s : std_logic_vector(22 downto 0);

--signal local_p_cndr_s, local_q_cndr_s : std_logic_vector(31 downto 0);

-- above signal will have intermediate accumulating results, once at 1ms epoch these results will be latched with 6 outputs.

signal trking_counter_s : std_logic_vector(31 downto 0);

-- this signal is maintained to maintain how much millisecond into track.

signal one_ms_tic_s: std_logic;

-- this signal to latch all 6 correlator dumps.

signal one_ms_latching_sig_s : std_logic;

signal incoming_sig_s: std_logic_vector(11 downto 0);

-- this signal to select the required antenna based on antenna selection.

signal antenna_port_selection_s : std_logic_vector(1 downto 0);

-- this signal latches from which antenna inputs to be taken for tracking

signal Rx_car_cycle_counter_s : std_logic_vector(31 downto 0);

-- free running cycle counters

signal Code_input_epl_s : std_logic_vector(2 downto 0);

-- for early promt late code signals

signal Data_boundary_pulse_s : std_logic;

-- lock detector module for each channel.  major maintanance is maintained in top module.

component lock_detector_ch_e is

PORT(

clk_40MHz_is : IN std_logic;

POR_is : IN std_logic;

start_is : IN std_logic;

channel_par_load_is : IN std_logic;

lock_status_in_is : IN std_logic;

reset_channel_en_pr_is : IN std_logic;

lock_counter_threshold1_is : in std_logic_vector(9 downto 0);

lock_counter_threshold2_is : in std_logic_vector(9 downto 0);

bit_sync_ind_is : IN std_logic;

lock_status_os : OUT std_logic

);

end component lock_detector_ch_e;

-- bit synchrnoizer module, this gives data boundary os signal for reseting 1ms counter.

COMPONENT bit_sync_sbas_e

PORT(

clk_40MHz_is : IN std_logic;

POR_is : IN std_logic;

start_is : IN std_logic;

one_ms_pulse_is: IN std_logic;

I_data_is, Q_data_is : IN std_logic_vector(22 downto 0);

lock_status_is : IN std_logic;

navdata_read_mp_is : IN std_logic;

sbas_enable_is: IN std_logic ;

navdata_os : OUT std_logic;

newdata_indicator_os : OUT std_logic;

bit_sync_indicator_os : OUT std_logic;

bit_sync_found_pulse_os: OUT std_logic;

p_cndr_os : OUT std_logic_vector(27 downto 0);

q_cndr_os : OUT std_logic_vector(27 downto 0);

sbas_navdata_os : out std_logic_vector(5 downto 0)

);

END COMPONENT bit_sync_sbas_e;

COMPONENT carrier_mix_ROM

PORT (

clka : IN STD_LOGIC;

ena : IN STD_LOGIC;

wea : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

addra : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

dina : IN STD_LOGIC_VECTOR(19 DOWNTO 0);

clkb : IN STD_LOGIC;

addrb : IN STD_LOGIC_VECTOR(9 DOWNTO 0);

doutb : OUT STD_LOGIC_VECTOR(19 DOWNTO 0)

);

END COMPONENT carrier_mix_ROM;

COMPONENT edac_14_dec1_e

Port (

-- Input signals

rd_en_is       : in  STD_LOGIC;  -- EDAC Decoder  Read enable

data_in_is     : in  STD_LOGIC_VECTOR (13 downto 0);   -- Input Information bits to EDAC Decoder

par_in_is      : in  STD_LOGIC_VECTOR (5 downto 0);   -- Input Parity bits to EDAC Decoder

-- Output signals

data_out_os    : out  STD_LOGIC_VECTOR (13 downto 0)  -- Output information bits after correction

);

end COMPONENT edac_14_dec1_e;

signal lock_status_out_s: std_logic;

signal bit_sync_indicator_s: std_logic;

signal p_i_s, p_q_s , abs_pq_s: std_logic_vector(22 downto 0);

signal navdata_s, newdata_indicator_s : std_logic;

signal sat_id_out_temp_s : std_logic_vector(5 downto 0);

signal addrb_s : std_logic_vector(9 downto 0);

signal doutb_s : std_logic_vector(19 downto 0);

signal edac_out_s : std_logic_vector(13 downto 0);

signal addrb1_s : std_logic_vector(9 downto 0);

signal doutb1_s : std_logic_vector(19 downto 0);

signal edac_out1_s : std_logic_vector(13 downto 0);

signal Narrow_corr_step_12_enable_s : std_logic_vector(1 downto 0);

begin

carmixrom1 : carrier_mix_ROM

PORT MAP (

clka => clk_is,     ena => car_mix_ROM_ena_is,     wea => car_mix_ROM_wea_is,     addra => car_mix_ROM_addra_is,     dina => car_mix_ROM_dina_is,

clkb => clk_is,     addrb => addrb_s,      doutb => doutb_s   );

EDAC_mixROM1: edac_14_dec1_e

port map (

rd_en_is => '1', data_in_is => doutb_s(13 downto 0)  ,par_in_is =>  doutb_s(19 downto 14) ,data_out_os => edac_out_s );

carmixrom2 : carrier_mix_ROM

PORT MAP (

clka => clk_is,     ena => car_mix_ROM_ena_is,     wea => car_mix_ROM_wea_is,     addra => car_mix_ROM_addra_is,     dina => car_mix_ROM_dina_is,

clkb => clk_is,     addrb => addrb1_s,      doutb => doutb1_s   );

EDAC_mixROM2: edac_14_dec1_e

port map (

rd_en_is => '1', data_in_is => doutb1_s(13 downto 0)  ,par_in_is =>  doutb1_s(19 downto 14) ,data_out_os => edac_out1_s );

Inst_lock_detector_ch_e: lock_detector_ch_e PORT MAP(

clk_40MHz_is => clk_is,

POR_is => POR_is,

start_is => start_is,

channel_par_load_is => channel_par_load_is,

lock_status_in_is => lock_status_in_is,

reset_channel_en_pr_is => reset_channel_en_pr_is,

lock_counter_threshold1_is => lock_counter_threshold1_is,

lock_counter_threshold2_is => lock_counter_threshold2_is,

bit_sync_ind_is => bit_sync_indicator_s,

lock_status_os => lock_status_out_s

);

lock_status_out_os <= lock_status_out_s;

Inst_bit_sync_sbas_e: bit_sync_sbas_e PORT MAP(

clk_40MHz_is => clk_is,

POR_is => POR_is,

start_is => start_is,

one_ms_pulse_is => loop_start_one_pulse_s,

I_data_is => p_i_s,

Q_data_is => abs_pq_s,

lock_status_is => lock_status_out_s,

navdata_read_mp_is => navdata_read_mp_is,

sbas_enable_is => sbas_enable_is,

navdata_os => navdata_s,

newdata_indicator_os => newdata_indicator_s,

bit_sync_indicator_os => bit_sync_indicator_s,

bit_sync_found_pulse_os => Data_boundary_pulse_s,

p_cndr_os => p_cndr_out_os,

q_cndr_os => q_cndr_out_os,

sbas_navdata_os=> sbas_navdata_os

);

p_i_os  <= p_i_s;

p_q_os  <= p_q_s;

navdata_reg_os <= '0' & '0' & antenna_port_selection_s & bit_sync_indicator_s & lock_status_out_s & newdata_indicator_s & navdata_s;

loop_start_os <= loop_start_one_pulse_s;

sat_id_out_os <= sat_id_out_temp_s;

codegen: trk_code_gen_sbas_e

Port map ( clk_is => clk_is,            POR_is => POR_is,            start_is => start_is,  	channel_par_load_is => channel_par_load_is,

sat_id_in_is =>  sat_id_in_is,

code_corr_freq_in_is => code_corr_freq_in_is,

code_freq_init_is => code_freq_init_is,

sbas_enable_is => sbas_enable_is,

sat_id_out_os => sat_id_out_temp_s ,one_ms_pulse_os   => one_ms_tic_s, one_ms_counter_value_os => one_ms_counter_value_os,

twenty_ms_counter_value_os => twenty_ms_counter_value_os, code_cyle_count_value_os => code_cyle_count_value_os,

code_phase_count_os=> code_phase_count_os,

reg_wr_processor_tms_is	=> reg_wr_processor_tms_is,

epoch_latch_pulse_is => epoch_latch_pulse_is,

Data_boundary_pulse_is => Data_boundary_pulse_s,

Narrow_corr_step_12_enable_is => Narrow_corr_step_12_enable_s,

Data_boundary_output_every20ms_os => Data_boundary_output_os, Codeout_os =>Code_input_epl_s );

-- antenna mux selection based which input

PROCESS_P1:process(antenna_port_selection_s,indata_ant1_is,indata_ant2_is,indata_ant3_is,indata_ant4_is)

begin

c_ant_port_decoder: case antenna_port_selection_s is

when "00" => incoming_sig_s <= indata_ant3_is & indata_ant1_is  ;

when "01" => incoming_sig_s <= indata_ant4_is & indata_ant2_is  ;

when "10" => incoming_sig_s <= indata_ant1_is & indata_ant3_is  ;

when "11" => incoming_sig_s <= indata_ant2_is & indata_ant4_is  ;

when others => incoming_sig_s <= indata_ant3_is & indata_ant1_is  ;

end case c_ant_port_decoder;

end process PROCESS_P1;

PROCESS_P2:process(clk_is,POR_is)

begin

if (POR_is = '1') then

p_i_s  <= "00000000000000000000000";

p_q_s  <= "00000000000000000000000";

abs_pq_s<= "00000000000000000000000";

e_i_os  <= "00000000000000000000000";

e_q_os  <= "00000000000000000000000";

l_i_os  <= "00000000000000000000000";

l_q_os  <= "00000000000000000000000";

loop_start_one_pulse_s <= '0';

fll_data_check_os <= '0';

Prev_phase_error_os  <= "00000000000000000";

Order1_out_os  <= "0000000000000000000000000000000000000000000000000";

Order2_out_os  <= "0000000000000000000000000000000000000000000000000";

Order3_out_os  <= "0000000000000000000000000000000000000000000000000";

DLL_Order1_out_os  <= "0000000000000000000000000000000000000000000000000";

DLL_Order2_out_os  <= "0000000000000000000000000000000000000000000000000";

car_cycle_count_os  <= "00000000000000000000000000000000";

car_phase_count_os  <= "00000000000000000000000000000000";

cycle_counted_s <= '0';

trk_accum_s     <= "00000000000000000000000000000000";

trk_freq_word_s <= "00000000000000000000000000000000";

IP_mixout_s <= "00000000";

QP_mixout_s <= "00000000";

IP_mixout1_s <= "0000000";

QP_mixout1_s <= "0000000";

IP_mixout2_s <= "0000000";

QP_mixout2_s <= "0000000";

ei_mix_s<= "00000000";

eq_mix_s<= "00000000";

pi_mix_s<= "00000000";

pq_mix_s<= "00000000";

li_mix_s<= "00000000";

lq_mix_s <= "00000000";

sig_accum_pinphase_s <= "00000000000000000000000";

sig_accum_pqphase_s <=  "00000000000000000000000";

sig_accum_einphase_s <= "00000000000000000000000";

sig_accum_eqphase_s <=  "00000000000000000000000";

sig_accum_linphase_s <= "00000000000000000000000";

sig_accum_lqphase_s <=  "00000000000000000000000";

trking_counter_s <= "00000000000000000000000000000000";

Rx_car_cycle_counter_s  <= "00000000000000000000000000000000";

antenna_port_selection_s <= "00";

one_ms_latching_sig_s <= '0';

prior_loop_start_s <= '0';

prior1_loop_start_s <= '0';

stage_control_os <= "00001";

fll_zero_os <= '0';

NAVIC_indicator_os <= '0';

addrb_s<= (others=>'0');

addrb1_s<= (others=>'0');

elsif clk_is='1' and clk_is'event then

-- when acquisition module tells us to start, we will be starting.

if(start_is='1') then  -- synchrnous reset input

p_i_s  <= "00000000000000000000000";

p_q_s  <= "00000000000000000000000";

abs_pq_s<= "00000000000000000000000";

e_i_os  <= "00000000000000000000000";

e_q_os  <= "00000000000000000000000";

l_i_os  <= "00000000000000000000000";

l_q_os  <= "00000000000000000000000";

sig_accum_pinphase_s <= "00000000000000000000000";

sig_accum_pqphase_s <=  "00000000000000000000000";

sig_accum_einphase_s <= "00000000000000000000000";

sig_accum_eqphase_s <=  "00000000000000000000000";

sig_accum_linphase_s <= "00000000000000000000000";

sig_accum_lqphase_s <=  "00000000000000000000000";

IP_mixout_s <= "00000000";

QP_mixout_s <= "00000000";

IP_mixout1_s <= "0000000";

QP_mixout1_s <= "0000000";

IP_mixout2_s <= "0000000";

QP_mixout2_s <= "0000000";

ei_mix_s<= "00000000";

eq_mix_s<= "00000000";

pi_mix_s<= "00000000";

pq_mix_s<= "00000000";

li_mix_s<= "00000000";

lq_mix_s <= "00000000";

trking_counter_s <= "00000000000000000000000000000000";

trk_accum_s     <= "00000000000000000000000000000000";

trk_freq_word_s <= car_freq_init_is;

Order1_out_os <= order1_loop_init_is;

DLL_Order1_out_os <= dll_order1_loop_init_is;

antenna_port_selection_s <= ant1_ant2_sel_ch_is;

fll_data_check_os <= '0';

-- at acquisition loading the antenna input is latched

Prev_phase_error_os  <= "00000000000000000";

Order2_out_os  <= "0000000000000000000000000000000000000000000000000";

Order3_out_os  <= "0000000000000000000000000000000000000000000000000";

DLL_Order2_out_os  <= "0000000000000000000000000000000000000000000000000";

cycle_counted_s <= '0'; -- V2_change

one_ms_latching_sig_s <= '0'; -- V2_change

prior_loop_start_s <= '0'; -- V2_change

prior1_loop_start_s <= '0'; -- V2_change

loop_start_one_pulse_s <= '0';

stage_control_os <= "00001";

fll_zero_os <= '0';

addrb_s<= (others=>'0');

addrb1_s<= (others=>'0');

-- if( sat_id_in_is(5 downto 0) >= "001000" ) and ( sat_id_in_is(5 downto 0) <= "001011" ) then

-- NAVIC_indicator_os <= '1';

-- elsif( sat_id_in_is(5 downto 0) >= "011000" ) and ( sat_id_in_is(5 downto 0) <= "011011" ) then

-- NAVIC_indicator_os <= '1';

-- elsif( sat_id_in_is(5 downto 0) >= "101000" ) and ( sat_id_in_is(5 downto 0) <= "101011" )then

-- NAVIC_indicator_os <= '1';

-- elsif( sat_id_in_is(5 downto 0) >= "111000" ) and ( sat_id_in_is(5 downto 0) <= "111001" ) then

-- NAVIC_indicator_os <= '1';

-- else

-- NAVIC_indicator_os <= '0';

-- end if;

CASE_1:case sat_id_in_is is

when "001000" => NAVIC_indicator_os <='1';

when "001001" => NAVIC_indicator_os <='1';

when "001010" => NAVIC_indicator_os <='1';

when "001011" => NAVIC_indicator_os <='1';

when "011000" => NAVIC_indicator_os <='1';

when "011001" => NAVIC_indicator_os <='1';

when "011010" => NAVIC_indicator_os <='1';

when "011011" => NAVIC_indicator_os <='1';

when "101000" => NAVIC_indicator_os <='1';

when "101001" => NAVIC_indicator_os <='1';

when "101010" => NAVIC_indicator_os <='1';

when "101011" => NAVIC_indicator_os <='1';

when "111000" => NAVIC_indicator_os <='1';

when "111001" => NAVIC_indicator_os <='1';

when  OTHERS  => NAVIC_indicator_os<='0';

end case CASE_1;

else

if( (trking_counter_s >= Narrow_corr_step2_timer_is) and (Narrow_correlator_step2_enable_is = '1')  ) then

Narrow_corr_step_12_enable_s <= "11";

elsif( (trking_counter_s >= Narrow_corr_step1_timer_is) and (Narrow_correlator_step1_enable_is = '1') ) then

Narrow_corr_step_12_enable_s <= "01";

else

Narrow_corr_step_12_enable_s <= "00";

end if;

-- load the corrected frequency from loop filter and store all values here.

if(channel_par_load_is = '1') then

trk_freq_word_s <= car_corr_freq_in_is;

Prev_phase_error_os <= Prev_phase_error_input_is;

Order1_out_os <= Order1_input_is ;

Order2_out_os <= Order2_input_is ;

Order3_out_os <= Order3_input_is ;

DLL_Order1_out_os <=DLL_Order1_input_is;

DLL_Order2_out_os <= DLL_Order2_input_is;

end if;

-- latching when 500ms measurement latch

if(epoch_latch_pulse_is = '1') then

car_cycle_count_os <=  Rx_car_cycle_counter_s;

car_phase_count_os <= 	trk_accum_s;

end if;

-- in fll there is datacheck for overflow of pi/2 condition for this it is put

if(trking_counter_s(31 downto 0) = FLL_data_check_counter_is) then		-- if tracking counter is more than 10ms, the data detection should happen in fll

fll_data_check_os <= '1';

end if;

if(trking_counter_s(31 downto 0) >= fll_timer_is) then

fll_zero_os <= '1';

end if;

if(trking_counter_s(31 downto 0) <= set1_to_set2_timer_Launch_is )  then

stage_control_os <= "00001";

elsif(trking_counter_s(31 downto 0) <= set2_to_set3_timer_Launch_is ) then

stage_control_os <= "00010";

elsif(trking_counter_s(31 downto 0) <= set3_to_set4_timer_Launch_is ) then

stage_control_os <= "00100";

elsif(trking_counter_s(31 downto 0) <= set4_to_set5_timer_Launch_is ) then

stage_control_os <= "01000";

elsif(trking_counter_s(31 downto 0) > set4_to_set5_timer_Launch_is ) then

stage_control_os <= "10000";

end if;

-- when output from code gen module indicating 1ms boundary it is latched into a signal.

if(one_ms_tic_s = '1') then

one_ms_latching_sig_s <= '1';

else

one_ms_latching_sig_s <= '0';

end if;

--					if(samp_clk_enable_is = '1')  and (one_ms_latching_sig_s = '1') then

--						one_ms_latching_sig_s <= '0';

--					end if;

-- below code for cycle counters storage and phase storage.

if(trk_accum_s(31)='0') and (cycle_counted_s = '0') then

Rx_car_cycle_counter_s <= Rx_car_cycle_counter_s + 1;

cycle_counted_s <= '1';

end if;

if(trk_accum_s(31)='1') then

cycle_counted_s <= '0';

end if;

if(Code_input_epl_s(0) = '1') then

ei_mix_s <= IP_mixout_s;

eq_mix_s <= QP_mixout_s;

else

ei_mix_s <= -IP_mixout_s;

eq_mix_s <= -QP_mixout_s;

end if;

if(Code_input_epl_s(1) = '1') then

pi_mix_s <= IP_mixout_s;

pq_mix_s <= QP_mixout_s;

else

pi_mix_s <= -IP_mixout_s;

pq_mix_s <= -QP_mixout_s;

end if;

if(Code_input_epl_s(2) = '1') then

li_mix_s <= IP_mixout_s;

lq_mix_s <= QP_mixout_s;

else

li_mix_s <= -IP_mixout_s;

lq_mix_s <= -QP_mixout_s;

end if;

if( (antenna_port_selection_s & antenna_combining_control_is(0) = "001") or (antenna_port_selection_s & antenna_combining_control_is(1) = "011") or (antenna_port_selection_s & antenna_combining_control_is(2) = "101") or (antenna_port_selection_s & antenna_combining_control_is(3) = "111") ) then

IP_mixout_s <= sxt(IP_mixout1_s,8) + sxt(IP_mixout2_s,8);

QP_mixout_s <= sxt(QP_mixout1_s,8) + sxt(QP_mixout2_s,8);

else

IP_mixout_s <=  sxt(IP_mixout1_s,8);

QP_mixout_s <=  sxt(QP_mixout1_s,8);

end if;

-- when 1ms boundary detected by codegen module, the pulse comes.

if(one_ms_latching_sig_s = '1') then

prior1_loop_start_s <= '1';

-- a pulse set high to make the loop start output.

-- latching 6 arms into a signal for discriminator

p_i_s   <= sig_accum_pinphase_s;

p_q_s    <= sig_accum_pqphase_s;

abs_pq_s   <= abs(sig_accum_pqphase_s);

e_i_os   <= sig_accum_einphase_s;

e_q_os    <= sig_accum_eqphase_s;

l_i_os   <= sig_accum_linphase_s;

l_q_os    <= sig_accum_lqphase_s;

trking_counter_s <= trking_counter_s + 1;

sig_accum_pinphase_s <= (others => '0');

sig_accum_pqphase_s <= (others => '0');

sig_accum_einphase_s <= (others => '0');

sig_accum_eqphase_s <= (others => '0');

sig_accum_linphase_s <= (others => '0');

sig_accum_lqphase_s <= (others => '0');

else

prior1_loop_start_s <= '0';

--accumulation of 6 arms are happening below

sig_accum_pinphase_s <= sig_accum_pinphase_s + pi_mix_s;

sig_accum_pqphase_s <= sig_accum_pqphase_s + pq_mix_s;

sig_accum_einphase_s <= sig_accum_einphase_s + ei_mix_s;

sig_accum_eqphase_s <= sig_accum_eqphase_s + eq_mix_s;

sig_accum_linphase_s <= sig_accum_linphase_s + li_mix_s;

sig_accum_lqphase_s <= sig_accum_lqphase_s + lq_mix_s;

-- accumulation of local carrier mix is happening.

end if;

if(prior1_loop_start_s = '1') then

prior_loop_start_s <= '1';

else

prior_loop_start_s <= '0';

end if;

if(prior_loop_start_s = '1') and (lock_status_out_s = '1') then

loop_start_one_pulse_s <= '1';

else

loop_start_one_pulse_s	<= '0';

-- loop start os set high based on prior loop start signals.

end if;

--nco action

trk_accum_s <= trk_accum_s + trk_freq_word_s;

addrb_s <= incoming_sig_s(5 downto 0) & trk_accum_s(31 downto 28) ;

addrb1_s <= incoming_sig_s(11 downto 6) & trk_accum_s(31 downto 28) ;

IP_mixout1_s <=   edac_out_s(6 downto 0);

QP_mixout1_s <=   edac_out_s(13 downto 7);

IP_mixout2_s <=   edac_out1_s(6 downto 0);

QP_mixout2_s <=   edac_out1_s(13 downto 7);

end if; -- sync reset

end if; -- if of clock event

end process PROCESS_P2;

end trk2_a;

77) twid_ram_2048pt_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity twid_ram_2048pt_e  is

generic(

fft_size_log_g     : integer  := 11 -- default value : 11.);

port(clk_is: in std_logic;

reset_is : in std_logic;

fft_rom_re_is : in std_logic;

twid_refresh_is : in std_logic;

fft_rom_addr_is : in std_logic_vector(fft_size_log_g-2 downto 0);

twiddle_re_os  : out std_logic_vector(15 downto 0);

twiddle_im_os  : out std_logic_vector(15 downto 0));

end twid_ram_2048pt_e;

architecture twid_ram_2048pt_a of twid_ram_2048pt_e is

signal twid_re_ram_out_s, twid_im_ram_out_s : std_logic_vector(21 downto 0);

signal twid_re_ram_in_s,twid_im_ram_in_s    : std_logic_vector(21 downto 0);

signal twiddle_re_out_s,twiddle_im_out_s    : std_logic_vector(15 downto 0);

signal twid_wr_addr_s, twid_rd_addr_s, twid_rd_addr1_s : std_logic_vector(fft_size_log_g-2 downto 0);

signal edac_wr_en_s,edac_rd_en_s : std_logic;

signal twid_rd_en1_s,twid_rd_en_s, twid_wr_en_s : std_logic;

signal twid_wea_s: std_logic_vector(0 downto 0);

COMPONENT twid_re_bram_2048

PORT (

clka : IN STD_LOGIC;

ena : IN STD_LOGIC;

wea : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

addra : IN STD_LOGIC_VECTOR(fft_size_log_g-2 DOWNTO 0);

dina : IN STD_LOGIC_VECTOR (21 downto 0);

clkb : IN STD_LOGIC;

enb : IN STD_LOGIC;

addrb : IN STD_LOGIC_VECTOR (fft_size_log_g-2 DOWNTO 0);

doutb : OUT STD_LOGIC_VECTOR(21 downto 0)

);

END COMPONENT;

COMPONENT twid_im_bram_2048

PORT (

clka : IN STD_LOGIC;

ena : IN STD_LOGIC;

wea : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

addra : IN STD_LOGIC_VECTOR(fft_size_log_g-2 DOWNTO 0);

dina : IN STD_LOGIC_VECTOR (21 downto 0);

clkb : IN STD_LOGIC;

enb : IN STD_LOGIC;

addrb : IN STD_LOGIC_VECTOR (fft_size_log_g-2 DOWNTO 0);

doutb : OUT STD_LOGIC_VECTOR(21 downto 0)

);

END COMPONENT;

--16 bit EDAC encoder component declaration

COMPONENT edac_16_enc1_e

PORT(

wr_en_is : IN std_logic;

data_in_is : IN std_logic_vector(15 downto 0);

data_out_os : OUT std_logic_vector(15 downto 0);

par_out_os : OUT std_logic_vector(5 downto 0));

END COMPONENT;

--16 bit EDAC decoder component declaration

COMPONENT edac_16_dec1_e

PORT(

rd_en_is : IN std_logic;

data_in_is : IN std_logic_vector(15 downto 0);

par_in_is : IN std_logic_vector(5 downto 0);

data_out_os : OUT std_logic_vector(15 downto 0));

END COMPONENT;

begin

twiddle_re_os <= twiddle_re_out_s;

twiddle_im_os <= twiddle_im_out_s;

twid_wea_s(0) <= twid_wr_en_s;

twid_real : twid_re_bram_2048

PORT MAP (

clka    => clk_is,

ena     => twid_wr_en_s ,

wea     => twid_wea_s,

addra   => twid_wr_addr_s,

dina    => twid_re_ram_in_s,

clkb    => clk_is,

enb     => twid_rd_en1_s,

addrb   => twid_rd_addr1_s,

doutb   => twid_re_ram_out_s );

twid_imag : twid_im_bram_2048

PORT MAP (

clka    => clk_is,

ena      => twid_wr_en_s ,

wea     => twid_wea_s,

addra   => twid_wr_addr_s,

dina    => twid_im_ram_in_s,

clkb    => clk_is,

enb     => twid_rd_en1_s,

addrb   => twid_rd_addr1_s,

doutb   => twid_im_ram_out_s );

twid_re_encoder: edac_16_enc1_e

PORT MAP(

wr_en_is    => edac_wr_en_s, --encoder write enable

data_in_is  => twiddle_re_out_s , -- encoder: input data bits

data_out_os => twid_re_ram_in_s(21 downto 6), -- Information bits after encoding

par_out_os  => twid_re_ram_in_s(5 downto 0) -- Check bits

);

twid_re_decoder: edac_16_dec1_e

PORT MAP(

rd_en_is     => edac_rd_en_s , -- decoder read enable

data_in_is   => twid_re_ram_out_s(21 downto 6), -- decoder: input data bits

par_in_is    => twid_re_ram_out_s(5 downto 0), -- decoder: input check bits

data_out_os  => twiddle_re_out_s -- Information bits after decoding

);

twid_im_encoder: edac_16_enc1_e

PORT MAP(

wr_en_is    => edac_wr_en_s, --encoder write enable

data_in_is  => twiddle_im_out_s , -- encoder: input data bits

data_out_os => twid_im_ram_in_s(21 downto 6), -- Information bits after encoding

par_out_os  => twid_im_ram_in_s(5 downto 0) -- Check bits

);

twid_im_decoder: edac_16_dec1_e

PORT MAP(

rd_en_is     => edac_rd_en_s , -- decoder read enable

data_in_is   => twid_im_ram_out_s(21 downto 6), -- decoder: input data bits

par_in_is    => twid_im_ram_out_s(5 downto 0), -- decoder: input check bits

data_out_os  => twiddle_im_out_s -- Information bits after decoding

);

edac_rd_en_s  <= '1';

--edac_wr_en_s  <=  '1' when twid_rd_en_s='1' else

--                  '0';

edac_wr_en_s  <= '1';

twid_rd_en1_s <=  '1' when twid_rd_en_s='1' or fft_rom_re_is='1' else

'0';

twid_rd_addr1_s <= twid_rd_addr_s	when twid_rd_en_s='1' else

fft_rom_addr_is  when fft_rom_re_is='1'else

(others=>'0');

P_twid_addr_gen: process(clk_is,reset_is)

begin

if reset_is ='1' then

twid_rd_addr_s <= (others=>'0');

twid_wr_addr_s <= (others=>'0');

twid_rd_en_s   <= '0';

twid_wr_en_s   <= '0';

elsif  clk_is'event and  clk_is='1'  then

if twid_refresh_is='1' then

twid_rd_en_s <='1';

elsif twid_wr_addr_s= "1111111111" then

twid_rd_en_s <='0';

end if;

if twid_rd_en_s='0' then

twid_rd_addr_s <= (others=>'0');

else

twid_rd_addr_s <= twid_rd_addr_s+1;

end if;

if twid_wr_addr_s= "1111111111" then

twid_wr_en_s <='0';

else

twid_wr_en_s  <= twid_rd_en_s;

end if;

twid_wr_addr_s <= twid_rd_addr_s;

end if;

end process P_twid_addr_gen;

end twid_ram_2048pt_a;

78) wdt_logic_e

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity wdt_logic_e is

port(--Inputs

clk_is   : in std_logic;

POR_G_is : in std_logic;

POR_in_is: in std_logic;

hwdt_cmd_is, swdt_cmd_is: in std_logic;

hwdt_reset_clear_pr_wr_enable_is: in std_logic;

up_data_is: in std_logic;

wdt_enable_pr_wr_enable_is, clear_wdt_pr_is: in std_logic;

PR_to_FPGA_reset_reg_sel_is: in std_logic;

-- Outputs

reset_processor_os: out std_logic;

wdt_cmd_status_os: out std_logic_vector(1 downto 0);

hwdt_reset_status_ff_os : out std_logic;

hwdt_reset_counter_os: out std_logic_vector(7 downto 0);

soft_reset_os: out std_logic;

soft_reset_flag_os : out std_logic );

end wdt_logic_e;

architecture wdt_logic_a of wdt_logic_e is

signal wdt_counter_reg_s: std_logic_vector(25 downto 0); --26-bit counter for watch-dog timer.

signal up_wdt_en_reg_s, clear_wdt_s: std_logic;

signal count_enable_s,wdt_counter_reset_s: std_logic;

signal hwdt_reset_counter_temp_s: std_logic_vector(7 downto 0);

signal soft_reset_s,sw_reg_s: std_logic;

signal sw_reg_prev_s,change_in_sw_reg_s : std_logic;

signal up_por_reset_s,up_rst_count_en_s: std_logic;

signal up_por_rst_count_s: std_logic_vector(3 downto 0);

begin

-- 26-bit WDT Counter

P_wdt_counter_reg: process(clk_is,POR_G_is)

begin

if POR_G_is = '1' then

wdt_counter_reg_s <= (others => '0');

elsif clk_is'event and clk_is = '1' then

if wdt_counter_reset_s = '1' then -- Counter reset.

wdt_counter_reg_s <= (others => '0');

elsif count_enable_s = '1' then

wdt_counter_reg_s <= wdt_counter_reg_s + 1;

end if;

end if;

end process P_wdt_counter_reg;

wdt_counter_reset_s <= clear_wdt_pr_is or clear_wdt_s or (not count_enable_s);

wdt_cmd_status_os   <= swdt_cmd_is & hwdt_cmd_is ; -- Two-bit WDT Command status.

P_WDT_enable_reg: process(clk_is, POR_G_is)

begin

--Read the WDT enable reg status from processor.

if POR_G_is = '1' then

up_wdt_en_reg_s <= '0' ;

elsif clk_is'event and clk_is = '1' then

if wdt_enable_pr_wr_enable_is = '1' then

up_wdt_en_reg_s <= up_data_is;

end if;

end if;

-- Hardware Reset status

if POR_G_is = '1' then

hwdt_reset_status_ff_os <= '0' ;

elsif clk_is'event and clk_is = '1' then

if clear_wdt_s = '1' then

hwdt_reset_status_ff_os <= '1';

elsif hwdt_reset_clear_pr_wr_enable_is = '1' then

hwdt_reset_status_ff_os <= '0';

end if;

end if;

end process P_WDT_enable_reg;

-- Hardware Reset Counter: To count number of WDT resets issued to processor.

-- Processor soft-reset shall not clear the counter, hence POR_in_is is used.

P_hwdt_counter: process(clk_is, POR_in_is)

begin

if POR_in_is = '1' then

hwdt_reset_counter_temp_s <= (others => '0');

elsif clk_is'event and clk_is = '1' then

if clear_wdt_s = '1' then

hwdt_reset_counter_temp_s <= hwdt_reset_counter_temp_s + 1;

end if;

end if;

end process P_hwdt_counter;

hwdt_reset_counter_os <= hwdt_reset_counter_temp_s;

count_enable_s <= '1' when  hwdt_cmd_is = '0' and up_wdt_en_reg_s = '1' else

'0';

clear_wdt_s <= '1' when  wdt_counter_reg_s(25) = '1' and wdt_counter_reg_s(23) = '1' else

'0';

--- 16 cycles delay is generated.

--Generate Control singals

P_process_reset_gen_at_wdt: process(clk_is,POR_in_is)

begin

if POR_in_is='1' then

up_rst_count_en_s<='1';

elsif clk_is'event and clk_is = '1' then

if clear_wdt_s='1' then

up_rst_count_en_s<='1';

elsif up_por_rst_count_s = "1111" then --- 1111 corresponds to 400nsec pulse.

up_rst_count_en_s <='0';

end if;                                 end if;

end process P_process_reset_gen_at_wdt;

-- Generate 400nsec pulse to reset processor at POR.

P_process_reset_gen_at_POR: process(clk_is,POR_in_is)

begin

if POR_in_is='1' then

up_por_rst_count_s  <= (OTHERS=>'0');

up_por_reset_s <= '0';

elsif clk_is'event and clk_is='1' then

if up_rst_count_en_s='0' then

up_por_rst_count_s<= (OTHERS=>'0');

up_por_reset_s <= '1';

else

up_por_rst_count_s <= up_por_rst_count_s+1;

up_por_reset_s <= '0';

end if;            end if;

end process P_process_reset_gen_at_POR;

-- 400nsec pulse to processor.

reset_processor_os <= up_por_reset_s ;

-- Soft reset Logic

P_sw_reset_gen: process(clk_is,POR_G_is)

begin

--Soft reset register staus: read from processor

if POR_G_is = '1' then

sw_reg_s <= '0';

elsif clk_is'event and clk_is = '1' then

if PR_to_FPGA_reset_reg_sel_is = '1' then

sw_reg_s <= up_data_is;

end if;

end if;

-- One cycle delay to read the previous value in the register.

if POR_G_is = '1' then

sw_reg_prev_s <= '0';

elsif clk_is'event and clk_is = '1' then

sw_reg_prev_s <= sw_reg_s;

end if;

if POR_G_is = '1' then

soft_reset_s <= '0';

elsif clk_is'event and clk_is = '1' then

if change_in_sw_reg_s = '1' then

soft_reset_s <= '1';

end if;

end if;

end process P_sw_reset_gen;

-- Detect the change in sw_reg status from 0 to 1.

change_in_sw_reg_s <= '1' when sw_reg_prev_s = '0' and sw_reg_s = '1'  else

'0';

-- Soft-reset is completely controlleed by LEON-3 processor.

-- SWDT is used for PROM or EEPROM code selection hence it's dependency is avoided w.rto prvious version of the code.

soft_reset_os <= soft_reset_s;

soft_reset_flag_os <= sw_reg_s;

end wdt_logic_a;

79)

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

use IEEE.std_logic_UNSIGNED.ALL;

use IEEE.NUMERIC_STD.ALL;

entity wdt_pulse_to_level_e is

Port ( clk_40MHz_is  : in  STD_LOGIC;

reset_is         : in  STD_LOGIC;

wdt_en_is        : in  STD_LOGIC;

wdt_dis_is       : in  STD_LOGIC;

wdt_level_os     : out  STD_LOGIC

);

end wdt_pulse_to_level_e;

architecture wdt_pulse_to_level_a of wdt_pulse_to_level_e is

signal wdt_level_s: STD_LOGIC;

begin

P1: process(clk_40MHz_is, reset_is)

begin

if reset_is='1' then

wdt_level_s<='0';  -- Default WDT disable condition.

elsif clk_40MHz_is'event and clk_40MHz_is='1' then

if wdt_en_is='1' then

wdt_level_s<='0';   -- WDT enable condition: Level- Low

elsif wdt_dis_is='1' then

wdt_level_s<='1';   -- WDT disable condition: Leve- High

end if;  end if;

end process P1;

wdt_level_os<=wdt_level_s;

end wdt_pulse_to_level_a;